3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240424

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-5 | CR | 0115 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to the request URI template |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160, UPV/EHU |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, MCImp-UEConTest |  |  |  |  |  | Date: |  |  | 2024-06-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the last RAN5#103 meeting, R5-242398 introduced both alternatives for the Request Uri: http and https. The current implementation builds incorrect Request Uri. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The expected requested URI has been corrected. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE will fail the MCX test cases. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | All MCX test cases |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	3

2.3	Change 3	4

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D24wk24 related to the title of this CR.

Contact:	Olivier Genoud
	olivier.genoud@etsi.org

# Corrections required

## Change 1

| Function name | fl_MCX_HTTP_HttpHttpsRequestUriTemplate |
| --- | --- |
| Reason for change | New helper function to build the URI template |
| Summary of change | New helper function to build the URI template |
| TTCN module | MCX_HTTP_CommonFunctions.ttcn |

Before Change:

After Change:

function fl_MCX_HTTP_HttpHttpsRequestUriTemplate(charstring p_Path) return template (present) charstring { return ("http://" & p_Path, "https://" & p_Path); }

## Change 2

| Function name | f_MCX_HTTP_UserAuthentication_Step11_12 |
| --- | --- |
| Reason for change | Incorrect expected request Uri. |
| Summary of change | Corrected expected request Uri. |
| TTCN module | MCX_HTTP_CommonFunctions.ttcn |

Before Change:

function f_MCX_HTTP_UserAuthentication_Step11_12(MCX_ServiceType_Type p_ServiceType, MCX_SecurityParameters_Type p_SecurityParameters) runs on MCX_HTTP_PTC return MCX_SecurityParameters_Type { var charstring v_MCServiceID := f_MCX_MCService_ID_User_A(p_ServiceType); var MCX_SecurityParameters_Type v_SecurityParameters := p_SecurityParameters; var charstring v_AccessToken := p_SecurityParameters.Tokens.AccessToken; var template (present) charstring v_ExpectedRequestUri := ("http" & tsc_MCX_KMS_Init_ReqUri_HostnamePath, "https" & tsc_tsc_MCX_KMS_Init_ReqUri_HostnamePath); /* @sic R5-242398: "http" or "https" sic@ */ var universal charstring v_HttpBody; var IP_Connection_Type v_HttpRoutingInfo; var HTTP_DATA_IND v_HTTP_DATA_IND; var charstring v_RequestUri; … }

After Change:

function f_MCX_HTTP_UserAuthentication_Step11_12(MCX_ServiceType_Type p_ServiceType, MCX_SecurityParameters_Type p_SecurityParameters) runs on MCX_HTTP_PTC return MCX_SecurityParameters_Type { var charstring v_MCServiceID := f_MCX_MCService_ID_User_A(p_ServiceType); var MCX_SecurityParameters_Type v_SecurityParameters := p_SecurityParameters; var charstring v_AccessToken := p_SecurityParameters.Tokens.AccessToken; var template (present) charstring v_ExpectedRequestUri := fl_MCX_HTTP_HttpHttpsRequestUriTemplate (tsc_MCX_KMS_Init_ReqUri_HostnamePath); /* @sic R5-242398: "http" or "https" sic@ */ var universal charstring v_HttpBody; var IP_Connection_Type v_HttpRoutingInfo; var HTTP_DATA_IND v_HTTP_DATA_IND; var charstring v_RequestUri; … }

## Change 3

| Function name | f_MCX_HTTP_UserAuthentication_Step13_14 |
| --- | --- |
| Reason for change | Incorrect expected request Uri. |
| Summary of change | Corrected expected request Uri. |
| TTCN module | MCX_HTTP_CommonFunctions.ttcn |

Before Change:

function f_MCX_HTTP_UserAuthentication_Step13_14(MCX_ServiceType_Type p_ServiceType, MCX_SecurityParameters_Type p_SecurityParameters) runs on MCX_HTTP_PTC return MCX_SecurityParameters_Type { var charstring v_MCServiceID := f_MCX_MCService_ID_User_A(p_ServiceType); var MCX_SecurityParameters_Type v_SecurityParameters := p_SecurityParameters; var charstring v_AccessToken := p_SecurityParameters.Tokens.AccessToken; var template (present) charstring v_ExpectedRequestUri := ("http" & tsc_MCX_KMS_KeyProv_ReqUri_HostnamePath, "https" & tsc_MCX_KMS_KeyProv_ReqUri_HostnamePath); /* @sic R5-242398: "http" or "https" sic@ */ var IP_Connection_Type v_HttpRoutingInfo; var HTTP_DATA_IND v_HTTP_DATA_IND; var universal charstring v_HttpBody; var octetstring v_MCPTT_UID; var charstring v_RequestUri; … }

After Change:

function f_MCX_HTTP_UserAuthentication_Step13_14(MCX_ServiceType_Type p_ServiceType, MCX_SecurityParameters_Type p_SecurityParameters) runs on MCX_HTTP_PTC return MCX_SecurityParameters_Type { var charstring v_MCServiceID := f_MCX_MCService_ID_User_A(p_ServiceType); var MCX_SecurityParameters_Type v_SecurityParameters := p_SecurityParameters; var charstring v_AccessToken := p_SecurityParameters.Tokens.AccessToken; var template (present) charstring v_ExpectedRequestUri := fl_MCX_HTTP_HttpHttpsRequestUriTemplate (tsc_MCX_KMS_KeyProv_ReqUri_HostnamePath); /* @sic R5-242398: "http" or "https" sic@ */ var IP_Connection_Type v_HttpRoutingInfo; var HTTP_DATA_IND v_HTTP_DATA_IND; var universal charstring v_HttpBody; var octetstring v_MCPTT_UID; var charstring v_RequestUri; … }