3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240430

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4834 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NBIOT-NTN testcase 22.3.1.5a |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, LTE_NBIOT_eMTC_NTN_plus_EPS-UEConTest |  |  |  |  |  | Date: |  |  | 2024-06-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - Invoke f_NBIOT_MobileInfo_GetNTNSatelliteType() to retrieve NTN_SatelliteType before it is initialized. - At Step 2 The grant is sent with 2 entries in the v_TransRetransmissionList and as a result an additional grant will be sent after the Harq response, since Step 4 uplink grant is now introduced in the code, the v_TransRetransmissionList shall be updated with just one entry - The timing calculation for Step 4 grant needs to take into account the NTN offset delay, as the ULRetransmissionTimer on the UE side would have started when the data when the data was scheduled to be sent. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - called f_NBIOT_Init(c8), which initialize the NTN_SatelliteType, before invoking the f_NBIOT_MobileInfo_GetNTNSatelliteType(). - v_TransRetransmissionList at Step 2 is with one entry (new transmission) - v_NpdcchPeriod calculation is updated to take into account f_NBIOT_NTN_Get_T_NTNoffset(nbiot_Cell1) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A Conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.3.1.5a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	3

2	Corrections required	3

2.1	Change to the f_TC_22_3_1_5a_NBIOT	3

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D24wk24 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change to the f_TC_22_3_1_5a_NBIOT

| Function name | f_TC_22_3_1_5a_NBIOT () |
| --- | --- |
| Reason for change | - Invoke f_NBIOT_MobileInfo_GetNTNSatelliteType() to retrieve NTN_SatelliteType before it is initialized. - At Step 2 The grant is sent with 2 entries in the v_TransRetransmissionList and as a result an additional grant will be sent after the Harq response, since Step 4 uplink grant is now introduced in the code, the v_TransRetransmissionList shall be updated with just one entry - The timing calculation for Step 4 grant needs to take into account the NTN offset delay, as the ULRetransmissionTimer on the UE side would have started when the data when the data was scheduled to be sent. |
| Summary of change | - called f_NBIOT_Init(c8), which initialize the NTN_SatelliteType, before invoking the f_NBIOT_MobileInfo_GetNTNSatelliteType(). - v_TransRetransmissionList at Step 2 is with one entry (new transmission) - v_NpdcchPeriod calculation is updated to take into account f_NBIOT_NTN_Get_T_NTNoffset(nbiot_Cell1) |
| TTCN module | ttcn\develop\NBIOT\22_3\NBIOT_MAC_NTN.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_22_3_1_5a_NBIOT() runs on NBIOT_PTC { var template (value) NPDCCH_ConfigDedicated_NB_r13 v_NPDCCH_ConfigDedicated; var NPdcchSearchSpaceCandidateIndex_Type v_SearchSpaceCandidateForULGrant; var template (value) NPdcchUESpecificSearchSpace_Type v_NPdcchUESpecificSearchSpace; var template (value) MAC_MainConfig_NB_r13 v_MAC_MainConfig := cs_MAC_MainConfig_NB_SRB; var template (value) DL_CCCH_Message_NB v_RrcConnSetup; var NB_RLC_CountsInfoList_Type v_RLC_CountsInfoListByRef; // RLC status to be maintained by Send/Receive functions var template (value) TimingInfo_Type v_TimingInfo; var template (value) MAC_PDU_Type v_MAC_PDU; var template (value) RLC_SDU_Type v_RlcSdu; var MAC_SDUList_Type v_MAC_SDUList; var SubFrameTiming_Type v_NpdcchPeriod; var NBIOT_SearchSpaceParameters_Type v_SearchSpaceParameters := tsc_NBIOT_SearchSpaceParameters_Def; var template (present) MAC_PDU_Type v_MAC_PDU_RX; var template (value) NB_UL_TransRetransmissionList_Type v_TransRetransmissionList; var template (value) NB_DciUlInfo_Type v_DciUlInfo; var integer v_IncrementVRR; var NB_SYSTEM_IND v_SYSTEM_IND; var NB_L2_DATA_IND v_NB_L2_DATA_IND; var integer v_UL_HARQ_RTT; var integer v_LegacyTA := 0; var integer v_CommonTA; var integer v_UeSpecificTA; var integer v_TA; var integer v_Drx_ULRetransmissionTimer := 6144; if (f_NBIOT_MobileInfo_GetNTNSatelliteType()==ntn_GSO) {//@sic R5s240411 sic@ v_CommonTA := 8243100; v_UeSpecificTA := 7476932; } else { v_CommonTA := 0; v_UeSpecificTA := 124805; } v_TA:= (v_LegacyTA + v_CommonTA + v_UeSpecificTA)* 1/(15000 * 2048); //TS 36.523-3 cl 7A.14.2.2 //System information combination 8 as defined in TS 36.508 [18] clause 8.1.4.3.1.1 is used. f_NBIOT_Init(c8); //Create and configure all cells f_NBIOT_CellConfig_Def(nbiot_Cell1, CONTROL_PLANE); // configure RRC Connection Setup being used during RACH procedure: Message modified according to Table 22.3.1.5a.3.3-1 v_MAC_MainConfig := cs_MAC_MainConfig_NB_SRB; v_MAC_MainConfig.ul_SCH_Config_r13.periodicBSR_Timer_r13 := infinity_; v_MAC_MainConfig.drx_Config_r13 := cs_DRX_Config_NB_Setup; v_NPDCCH_ConfigDedicated := cs_NPDCCH_ConfigDedicated_NB_DEFAULT; v_NPDCCH_ConfigDedicated.npdcch_NumRepetitions_r13 := r64; v_SearchSpaceParameters.Rmax := 64; // to be used for timing calculations v_RrcConnSetup := f_NBIOT_508_RRCConnectionSetup(-, -, v_MAC_MainConfig, v_NPDCCH_ConfigDedicated); f_NBIOT_SS_ConfigRachProcedureMsg4(nbiot_Cell1, v_RrcConnSetup); // Modify cell configuration for UE specific search space according to Table 22.3.1.5a.3.3-1 and UL grants to be scheduled in the last search space candidate as needed for step 2: v_SearchSpaceCandidateForULGrant := tsc_LastSearchSpaceCandidate; /* as we have a NPDCCH period of 256ms (instead of 64ms), there is no issue always to use the last search space candidate during the test case */ v_NPdcchUESpecificSearchSpace := cs_NPdcchUESpecificSearchSpace(v_NPDCCH_ConfigDedicated.npdcch_NumRepetitions_r13, v_NPDCCH_ConfigDedicated.npdcch_StartSF_USS_r13, v_NPDCCH_ConfigDedicated.npdcch_Offset_USS_r13, -, v_SearchSpaceCandidateForULGrant); f_NBIOT_SS_CommonCellConfig(nbiot_Cell1, cas_NB_CellConfig_REQ(nbiot_Cell1, -, -, cds_NB_BasicCellConfig_NPDCCH(cs_NPdcchConfig(v_NPdcchUESpecificSearchSpace)))); f_NBIOT_SS_DrxCtrlConfig(nbiot_Cell1, cs_NB_DrxCtrl(cs_DRX_Config_NB_Setup)); //@sic R5s230746 Ch2.1.2 sic@ //The UE is in state 2B-NB according to TS 36.508 [18] clause 8.1.5.2B with test loop mode G closed f_NBIOT_SetCellPower(nbiot_Cell1, tsc_ServingCellRS_EPRE); // Switch on UE f_UT_SwitchOnUE(UT, false); //f_UT_AutomaticPLMN_Select(UT); //@sic R5s240281 FFS Prose CR, R5s2390746 Ch.2.1.1 sic@ //The preconfigured UE location is defined in TS 36.508 [18] Clause 4.13. f_NBIOT_PreconfigureNTN_UE(); // @sic R5w240103 sic@ // Now continue with the procedure f_NBIOT_InitialRegistration(nbiot_Cell1, CONTROL_PLANE, PREAMBLE, STATE2A_NB_TESTLOOP_ModeG); f_NBIOT_CloseUE_TestLoopModeGH(nbiot_Cell1, // Bring UE to state 2B-NB f_NBIOT_UE_TestLoopMode_Type(STATE2A_NB_TESTLOOP_ModeG), tsc_TestLoopModeGH_UplinkMode_RLC); v_RLC_CountsInfoListByRef := f_NBIOT_L2_CPMode_StopULGrant_EnableSrbTestMode(nbiot_Cell1, TransparentMode_RLC_MAC); //@sic R5s2390746 Ch.2.1.3 sic@ f_NBIOT_TestBody_Set(true, layer2); f_NBIOT_SS_DrxCtrlConfig(nbiot_Cell1, cs_NB_DrxCtrl_None); //@sic R5s2390746 Ch.2.1.4 sic@ f_Delay(1.0); //@sic R5s2390746 Ch.2.1.5 sic@ f_NBIOT_SS_ConfigUL_HARQ_IndMode(nbiot_Cell1, enable); //@siclog "Step 1" siclog@ //In the first NPDCCH period of the next DRX On Duration the SS schedules the DL transmission of a MAC PDU in the first search space candidate of the search space. v_NpdcchPeriod := fl_NBIOT_DrxCycle_NpdcchPeriod(nbiot_Cell1, tsc_FirstNpdcchPeriod, v_SearchSpaceParameters); v_TimingInfo := cs_TimingInfo_SubFrameFull(v_NpdcchPeriod); v_RlcSdu := crs_PDCP_SDU_16B; //v_RlcSduList := { v_RlcSdu }; v_MAC_SDUList := { f_NBIOT_MAC_CPMode_EncodeMacSdu(STATE2A_NB_TESTLOOP_ModeG, v_RlcSdu, v_RLC_CountsInfoListByRef.AM_VTS) }; // NAS/RRC PDU gets encoded by NAS emulator and packed into RLC PDU v_MAC_PDU := f_NBIOT_MAC_PDU_WithSDUs_TX(tsc_LchId_SIB1bis, v_MAC_SDUList); f_NBIOT_MAC_CPMode_SendMacPdu(nbiot_Cell1, v_TimingInfo, v_MAC_PDU, tsc_FirstSearchSpaceCandidate); v_RLC_CountsInfoListByRef.AM_VTS := v_RLC_CountsInfoListByRef.AM_VTS + 1; //@siclog "Step 1A" siclog@ //The UE transmit a HARQ ACK for the DL MAC PDU in Step 1. SYSIND.receive(car_NB_UL_HARQ_IND(nbiot_Cell1, ack)); //@siclog "Step 2" siclog@ //In the last NPDCCH period of the next DRX On Duration the SS allocates an UL grant for UE in the last search space candidate of the search space. v_NpdcchPeriod := fl_NBIOT_DrxCycle_NpdcchPeriod(nbiot_Cell1, tsc_LastNpdcchPeriod, v_SearchSpaceParameters); v_TimingInfo := cs_TimingInfo_SubFrameFull(v_NpdcchPeriod); v_TransRetransmissionList := {cs_NB_UL_TransRetransmission_NewTransmission, cs_NB_UL_TransRetransmission_ReTransmission(1)}; //@sic R5s230746 Ch2.1.6 sic@ v_DciUlInfo := cs_NB_DciInfo_CcchDcchDtchUL(2, 4, v_TransRetransmissionList); f_NBIOT_ULGrantTransmission(nbiot_Cell1, cs_NB_UL_GrantScheduling_Start(cs_NB_SingleGrant(v_DciUlInfo)), v_TimingInfo); //@siclog "Step 3" siclog@ //The UE loops back the MAC PDU. v_MAC_PDU_RX := f_NBIOT_MacPdu_SingleRlcAmdPdu_RX(-, v_RlcSdu); //same as in Step 1 v_NB_L2_DATA_IND := f_NBIOT_MAC_CPMode_MacPdu_Receive(v_RLC_CountsInfoListByRef, nbiot_Cell1, v_MAC_PDU_RX); //@sic R5s240411 sic@ v_TimingInfo := v_NB_L2_DATA_IND.Common.TimingInfo; // reference time for calculation of UE timer start @sic R5s240411 sic@ //For NB-IoT, when single TB is scheduled by PDCCH the UL HARQ RTT timer length is set to 4 subframes plus RTToffset +deltaPDCCH, //where deltaPDCCH is the interval starting from the subframe following the last subframe of the PUSCH transmission plus 3 subframes plus RTToffset to the first subframe of the next PDCCH occasion v_UL_HARQ_RTT := 3 + (v_TA + f_NBIOT_CellInfo_Get_Kmac(nbiot_Cell1))/2; //3 subframes + half of RTT @sic R5s240411 sic@ //@siclog "Step 4" siclog@ //In the NPDCCH sub frame when the drx-uLRetransmissionTimer-r13 for MAC PDU in Step 3 is still running, the SS transmits UL grant for the retransmission of the MAC PDU in step 3, and NDI is the same with which in step 2, and redundancy version = 1 //i.e. UL HARQ RTT + drx-uLRetransmissionTimer-r13 - x subframes after step 3 (x is between 0 and 20 the difference between On duration and UL Retransmission timer) (Note 5) v_Drx_ULRetransmissionTimer := v_Drx_ULRetransmissionTimer - 10; // x = 10 @sic R5s240411 sic@ v_NpdcchPeriod := f_SubFrameTiming_AddMilliSeconds(valueof(v_TimingInfo.SubFrame), v_UL_HARQ_RTT + v_Drx_ULRetransmissionTimer); // @sic R5s240411 sic@ v_NpdcchPeriod := f_SubFrameTiming_CalculateCycleStartTime(v_NpdcchPeriod, f_NBIOT_SearchSpace_GetPdcchPeriod(v_SearchSpaceParameters), v_SearchSpaceParameters.Alpha); // + delatPDCCH @sic R5s240411 sic@ v_TimingInfo := cs_TimingInfo_SubFrameFull(v_NpdcchPeriod); v_TransRetransmissionList := {cs_NB_UL_TransRetransmission_ReTransmission(1)}; //RV=1 v_DciUlInfo := cs_NB_DciInfo_CcchDcchDtchUL(2, 4, v_TransRetransmissionList); f_NBIOT_ULGrantTransmission(nbiot_Cell1, cs_NB_UL_GrantScheduling_Start(cs_NB_SingleGrant(v_DciUlInfo)), v_TimingInfo); //@sic R5s240411 sic@ //@siclog "Step 5" siclog@ //Void.

After Change:

function f_TC_22_3_1_5a_NBIOT() runs on NBIOT_PTC { var template (value) NPDCCH_ConfigDedicated_NB_r13 v_NPDCCH_ConfigDedicated; var NPdcchSearchSpaceCandidateIndex_Type v_SearchSpaceCandidateForULGrant; var template (value) NPdcchUESpecificSearchSpace_Type v_NPdcchUESpecificSearchSpace; var template (value) MAC_MainConfig_NB_r13 v_MAC_MainConfig := cs_MAC_MainConfig_NB_SRB; var template (value) DL_CCCH_Message_NB v_RrcConnSetup; var NB_RLC_CountsInfoList_Type v_RLC_CountsInfoListByRef; // RLC status to be maintained by Send/Receive functions var template (value) TimingInfo_Type v_TimingInfo; var template (value) MAC_PDU_Type v_MAC_PDU; var template (value) RLC_SDU_Type v_RlcSdu; var MAC_SDUList_Type v_MAC_SDUList; var SubFrameTiming_Type v_NpdcchPeriod; var NBIOT_SearchSpaceParameters_Type v_SearchSpaceParameters := tsc_NBIOT_SearchSpaceParameters_Def; var template (present) MAC_PDU_Type v_MAC_PDU_RX; var template (value) NB_UL_TransRetransmissionList_Type v_TransRetransmissionList; var template (value) NB_DciUlInfo_Type v_DciUlInfo; var integer v_IncrementVRR; var NB_SYSTEM_IND v_SYSTEM_IND; var NB_L2_DATA_IND v_NB_L2_DATA_IND; var integer v_UL_HARQ_RTT; var integer v_LegacyTA := 0; var integer v_CommonTA; var integer v_UeSpecificTA; var integer v_TA; var integer v_Drx_ULRetransmissionTimer := 6144; //System information combination 8 as defined in TS 36.508 [18] clause 8.1.4.3.1.1 is used. f_NBIOT_Init(c8); // if (f_NBIOT_MobileInfo_GetNTNSatelliteType()==ntn_GSO) {//@sic R5s240411 sic@ v_CommonTA := 8243100; v_UeSpecificTA := 7476932; } else { v_CommonTA := 0; v_UeSpecificTA := 124805; } v_TA:= (v_LegacyTA + v_CommonTA + v_UeSpecificTA)* 1/(15000 * 2048); //TS 36.523-3 cl 7A.14.2.2 //Create and configure all cells f_NBIOT_CellConfig_Def(nbiot_Cell1, CONTROL_PLANE); // configure RRC Connection Setup being used during RACH procedure: Message modified according to Table 22.3.1.5a.3.3-1 v_MAC_MainConfig := cs_MAC_MainConfig_NB_SRB; v_MAC_MainConfig.ul_SCH_Config_r13.periodicBSR_Timer_r13 := infinity_; v_MAC_MainConfig.drx_Config_r13 := cs_DRX_Config_NB_Setup; v_NPDCCH_ConfigDedicated := cs_NPDCCH_ConfigDedicated_NB_DEFAULT; v_NPDCCH_ConfigDedicated.npdcch_NumRepetitions_r13 := r64; v_SearchSpaceParameters.Rmax := 64; // to be used for timing calculations v_RrcConnSetup := f_NBIOT_508_RRCConnectionSetup(-, -, v_MAC_MainConfig, v_NPDCCH_ConfigDedicated); f_NBIOT_SS_ConfigRachProcedureMsg4(nbiot_Cell1, v_RrcConnSetup); // Modify cell configuration for UE specific search space according to Table 22.3.1.5a.3.3-1 and UL grants to be scheduled in the last search space candidate as needed for step 2: v_SearchSpaceCandidateForULGrant := tsc_LastSearchSpaceCandidate; /* as we have a NPDCCH period of 256ms (instead of 64ms), there is no issue always to use the last search space candidate during the test case */ v_NPdcchUESpecificSearchSpace := cs_NPdcchUESpecificSearchSpace(v_NPDCCH_ConfigDedicated.npdcch_NumRepetitions_r13, v_NPDCCH_ConfigDedicated.npdcch_StartSF_USS_r13, v_NPDCCH_ConfigDedicated.npdcch_Offset_USS_r13, -, v_SearchSpaceCandidateForULGrant); f_NBIOT_SS_CommonCellConfig(nbiot_Cell1, cas_NB_CellConfig_REQ(nbiot_Cell1, -, -, cds_NB_BasicCellConfig_NPDCCH(cs_NPdcchConfig(v_NPdcchUESpecificSearchSpace)))); f_NBIOT_SS_DrxCtrlConfig(nbiot_Cell1, cs_NB_DrxCtrl(cs_DRX_Config_NB_Setup)); //@sic R5s230746 Ch2.1.2 sic@ //The UE is in state 2B-NB according to TS 36.508 [18] clause 8.1.5.2B with test loop mode G closed f_NBIOT_SetCellPower(nbiot_Cell1, tsc_ServingCellRS_EPRE); // Switch on UE f_UT_SwitchOnUE(UT, false); //f_UT_AutomaticPLMN_Select(UT); //@sic R5s240281 FFS Prose CR, R5s2390746 Ch.2.1.1 sic@ //The preconfigured UE location is defined in TS 36.508 [18] Clause 4.13. f_NBIOT_PreconfigureNTN_UE(); // @sic R5w240103 sic@ // Now continue with the procedure f_NBIOT_InitialRegistration(nbiot_Cell1, CONTROL_PLANE, PREAMBLE, STATE2A_NB_TESTLOOP_ModeG); f_NBIOT_CloseUE_TestLoopModeGH(nbiot_Cell1, // Bring UE to state 2B-NB f_NBIOT_UE_TestLoopMode_Type(STATE2A_NB_TESTLOOP_ModeG), tsc_TestLoopModeGH_UplinkMode_RLC); v_RLC_CountsInfoListByRef := f_NBIOT_L2_CPMode_StopULGrant_EnableSrbTestMode(nbiot_Cell1, TransparentMode_RLC_MAC); //@sic R5s2390746 Ch.2.1.3 sic@ f_NBIOT_TestBody_Set(true, layer2); f_NBIOT_SS_DrxCtrlConfig(nbiot_Cell1, cs_NB_DrxCtrl_None); //@sic R5s2390746 Ch.2.1.4 sic@ f_Delay(1.0); //@sic R5s2390746 Ch.2.1.5 sic@ f_NBIOT_SS_ConfigUL_HARQ_IndMode(nbiot_Cell1, enable); //@siclog "Step 1" siclog@ //In the first NPDCCH period of the next DRX On Duration the SS schedules the DL transmission of a MAC PDU in the first search space candidate of the search space. v_NpdcchPeriod := fl_NBIOT_DrxCycle_NpdcchPeriod(nbiot_Cell1, tsc_FirstNpdcchPeriod, v_SearchSpaceParameters); v_TimingInfo := cs_TimingInfo_SubFrameFull(v_NpdcchPeriod); v_RlcSdu := crs_PDCP_SDU_16B; //v_RlcSduList := { v_RlcSdu }; v_MAC_SDUList := { f_NBIOT_MAC_CPMode_EncodeMacSdu(STATE2A_NB_TESTLOOP_ModeG, v_RlcSdu, v_RLC_CountsInfoListByRef.AM_VTS) }; // NAS/RRC PDU gets encoded by NAS emulator and packed into RLC PDU v_MAC_PDU := f_NBIOT_MAC_PDU_WithSDUs_TX(tsc_LchId_SIB1bis, v_MAC_SDUList); f_NBIOT_MAC_CPMode_SendMacPdu(nbiot_Cell1, v_TimingInfo, v_MAC_PDU, tsc_FirstSearchSpaceCandidate); v_RLC_CountsInfoListByRef.AM_VTS := v_RLC_CountsInfoListByRef.AM_VTS + 1; //@siclog "Step 1A" siclog@ //The UE transmit a HARQ ACK for the DL MAC PDU in Step 1. SYSIND.receive(car_NB_UL_HARQ_IND(nbiot_Cell1, ack)); //@siclog "Step 2" siclog@ //In the last NPDCCH period of the next DRX On Duration the SS allocates an UL grant for UE in the last search space candidate of the search space. v_NpdcchPeriod := fl_NBIOT_DrxCycle_NpdcchPeriod(nbiot_Cell1, tsc_LastNpdcchPeriod, v_SearchSpaceParameters); v_TimingInfo := cs_TimingInfo_SubFrameFull(v_NpdcchPeriod); v_TransRetransmissionList := {cs_NB_UL_TransRetransmission_NewTransmission}; //cs_NB_UL_TransRetransmission_ReTransmission(1)}; //@sic R5s230746 Ch2.1.6 sic@ //// removed the retransmission if UL grant is sent at Step 4 v_DciUlInfo := cs_NB_DciInfo_CcchDcchDtchUL(2, 4, v_TransRetransmissionList); f_NBIOT_ULGrantTransmission(nbiot_Cell1, cs_NB_UL_GrantScheduling_Start(cs_NB_SingleGrant(v_DciUlInfo)), v_TimingInfo); //@siclog "Step 3" siclog@ //The UE loops back the MAC PDU. v_MAC_PDU_RX := f_NBIOT_MacPdu_SingleRlcAmdPdu_RX(-, v_RlcSdu); //same as in Step 1 v_NB_L2_DATA_IND := f_NBIOT_MAC_CPMode_MacPdu_Receive(v_RLC_CountsInfoListByRef, nbiot_Cell1, v_MAC_PDU_RX); //@sic R5s240411 sic@ v_TimingInfo := v_NB_L2_DATA_IND.Common.TimingInfo; // reference time for calculation of UE timer start @sic R5s240411 sic@ //For NB-IoT, when single TB is scheduled by PDCCH the UL HARQ RTT timer length is set to 4 subframes plus RTToffset +deltaPDCCH, //where deltaPDCCH is the interval starting from the subframe following the last subframe of the PUSCH transmission plus 3 subframes plus RTToffset to the first subframe of the next PDCCH occasion v_UL_HARQ_RTT := 3 + (v_TA + f_NBIOT_CellInfo_Get_Kmac(nbiot_Cell1))/2; //3 subframes + half of RTT @sic R5s240411 sic@ //@siclog "Step 4" siclog@ //In the NPDCCH sub frame when the drx-uLRetransmissionTimer-r13 for MAC PDU in Step 3 is still running, the SS transmits UL grant for the retransmission of the MAC PDU in step 3, and NDI is the same with which in step 2, and redundancy version = 1 //i.e. UL HARQ RTT + drx-uLRetransmissionTimer-r13 - x subframes after step 3 (x is between 0 and 20 the difference between On duration and UL Retransmission timer) (Note 5) v_Drx_ULRetransmissionTimer := v_Drx_ULRetransmissionTimer - 10 ; // x = 10 @sic R5s240411 sic@ v_NpdcchPeriod := f_SubFrameTiming_AddMilliSeconds(valueof(v_TimingInfo.SubFrame), v_UL_HARQ_RTT + v_Drx_ULRetransmissionTimer - f_NBIOT_NTN_Get_T_NTNoffset(nbiot_Cell1)); // @sic R5s240411 sic@ //WA#WI=1300194 reduced the timing by removing the NTNoffset delay v_NpdcchPeriod := f_SubFrameTiming_CalculateCycleStartTime(v_NpdcchPeriod, f_NBIOT_SearchSpace_GetPdcchPeriod(v_SearchSpaceParameters), v_SearchSpaceParameters.Alpha); // + delatPDCCH @sic R5s240411 sic@ v_TimingInfo := cs_TimingInfo_SubFrameFull(v_NpdcchPeriod); v_TransRetransmissionList := {cs_NB_UL_TransRetransmission_ReTransmission(1)}; //RV=1 v_DciUlInfo := cs_NB_DciInfo_CcchDcchDtchUL(2, 4, v_TransRetransmissionList); f_NBIOT_ULGrantTransmission(nbiot_Cell1, cs_NB_UL_GrantScheduling_Start(cs_NB_SingleGrant(v_DciUlInfo)), v_TimingInfo); //@sic R5s240411 sic@ //@siclog "Step 5" siclog@ //Void.