3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240432

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3468 | rev | - | Current version: | 17.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR5GC Test case 7.1.1.6.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-06-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - For RedCap and specific MAC test cases using MAC test mode, PDCP header is using SN=12bits instead of 18bit function f_NR_ReturnDefaultPDCP_PDU at step3. - Downlink SPS is not configured for this test case, so DRB_COMMON_REQ needs C-RNTI grant. Incorrect template is used in current TTCN. - As prose expecting from SS transmits a UL grant addressed to UE’s stored CS-RNTI with NDI set as 1 in Slot ‘p0’of PDCCH at step 11, so step11 should be configured before step 8, else SS may not have enough time to process it. Current TTCN timing is providing Ul grants to SS with CS-RNTI tranmissoin with NDI=0. In order to achive retranmission or NDI=1 for step 11 UL grants, timing of step 11 should be before step10 timing for SS stability. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Added 1 as 3rd argument on f_NR_ReturnDefaultPDCP_PDU. - At step 3 and 7A change cads_NR_DRB_COMMON_REQ_MAC_PDUList_NoCRNTI_Grant into cas_NR_DRB_COMMON_REQ_MAC_PDUList - Updated UL timing for step11 according to timedomainoffset 19 for TDD and FDD before step 7A using DL transmission time of step 7A. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.6.2.NR5GC and 7.1.1.6.2.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D24wk24 related to the title of this CR.

Contact:	Kalahasti, Narendra

Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | function f_TC_7_1_1_6_2_NR_TestBody1 |
| --- | --- |
| Reason for change | - For RedCap and specific MAC test cases using MAC test mode, PDCP header is using SN=12bits instead of 18bit function f_NR_ReturnDefaultPDCP_PDU at step3. - Downlink SPS is not configured for this test case, so DRB_COMMON_REQ needs C-RNTI grant. Incorrect template is used in current TTCN at step3. |
| Summary of change | - Added 1 as 3rd argument on f_NR_ReturnDefaultPDCP_PDU. - At step 3 change cads_NR_DRB_COMMON_REQ_MAC_PDUList_NoCRNTI_Grant into cas_NR_DRB_COMMON_REQ_MAC_PDUList |
| TTCN module | MAC_SPS_TC_Common_NR |
| MCC160 Comment |  |

Before Change

function f_TC_7_1_1_6_2_NR_TestBody1(DRB_Identity p_NR_DRB_Id ) runs on NR_BASE_PTC { //@sic R5s230287 sic@ const integer const_Cfg_Period :=40; // period in milli seconds const integer const_Cfg_TOffset := 9; // In slots var SubFrameTiming_Type v_TimingDL; //@sic R5s230287 sic@ var NR_MAC_SDU_Type v_EncodedRlcPdu1; var NR_MAC_SDU_Type v_EncodedRlcPdu2; var template (value) NR_MAC_SDU_SubPDU_List_Type v_SDU_SubPDUListDL ; var template (value) NR_MAC_PDUList_Type v_MAC_PDUListDL; var template (present) NR_MAC_SDU_SubPDU_List_Type v_SDU_SubPDUListUL; var template (present) NR_MAC_PDUList_Type v_MAC_PDUListUL1; var SubFrameTiming_Type v_TimingUL_1; var SubFrameTiming_Type v_TimingUL_2; var NR_DRB_COMMON_IND v_NR_DRB_COMMON_IND; var SubcarrierSpacing v_SCS := f_NR_CellInfo_GetSCS(nr_Cell1); var UInt_Type v_SlotsPerSubframe := f_NR_CellInfo_GetSCS_SlotsNumberPerSubframe(v_SCS); var UInt_Type v_SlotsPerCfg_Period := const_Cfg_Period *v_SlotsPerSubframe; var integer v_SlotsT1; timer t_Watchdog := 11.0; // @siclog "Step 3" siclog@ //@sic R5s240269 sic@ v_EncodedRlcPdu1 := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU(0, crs_NR_PDCP_SDU_96B))); v_EncodedRlcPdu2 := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU(1, crs_NR_PDCP_SDU_96B1))); v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1), cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu2)}; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL,cs_NR_MAC_Padding_SubPDU(tsc_NR_MAC_Padding_2B)) }; v_TimingDL:= f_NR_GetCurrentTiming(nr_Cell1); v_TimingDL.SFN.Number := 1022; v_TimingDL.Subframe.Number := 2; v_TimingDL.Slot.SlotOffset.Numerology1 := 0; v_TimingDL.Symbol := valueof (cs_SymbolTimingInfo_FirstSlot); //@sic R5s230287 sic@ DRB.send(cads_NR_DRB_COMMON_REQ_MAC_PDUList_NoCRNTI_Grant(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR(v_TimingDL), v_MAC_PDUListDL)); t_Watchdog.start; // @siclog "Step 4" siclog@ v_SDU_SubPDUListUL := {cr_NR_MAC_SDU_SubPDU(cr_NR_MAC_SubHeaderWithLI(tsc_LCID_SCG_Bearer, int2bit(100, 8)), v_EncodedRlcPdu1)}; v_MAC_PDUListUL1 := { cr_NR_UL_MAC_PDU(v_SDU_SubPDUListUL, *, *) }; DRB.receive (car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, -, v_MAC_PDUListUL1))-> value v_NR_DRB_COMMON_IND; v_TimingUL_1 := v_NR_DRB_COMMON_IND.Common.TimingInfo.SubFrame; v_SlotsT1 := f_NR_SubFrameTiming_NumSlots(nr_Cell1, v_TimingUL_1); if (v_SlotsT1 mod v_SlotsPerCfg_Period == const_Cfg_TOffset) {f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4"); } else {f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 4"); } v_TimingUL_2 :=f_SubFrameTiming_AddMilliSeconds(v_TimingUL_1,const_Cfg_Period); //Step 5 Timing // @siclog "Step 5" siclog@ v_SDU_SubPDUListUL := {cr_NR_MAC_SDU_SubPDU(cr_NR_MAC_SubHeaderWithLI(tsc_LCID_SCG_Bearer, int2bit(100, 8)), v_EncodedRlcPdu2)}; v_MAC_PDUListUL1 := { cr_NR_UL_MAC_PDU(v_SDU_SubPDUListUL, *, *) }; DRB.receive (car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cr_TimingInfo_NR( v_TimingUL_2), v_MAC_PDUListUL1)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 5"); t_Watchdog.stop; }

After Change

function f_TC_7_1_1_6_2_NR_TestBody1(DRB_Identity p_NR_DRB_Id ) runs on NR_BASE_PTC { //@sic R5s230287 sic@ const integer const_Cfg_Period :=40; // period in milli seconds const integer const_Cfg_TOffset := 9; // In slots var SubFrameTiming_Type v_TimingDL; //@sic R5s230287 sic@ var NR_MAC_SDU_Type v_EncodedRlcPdu1; var NR_MAC_SDU_Type v_EncodedRlcPdu2; var template (value) NR_MAC_SDU_SubPDU_List_Type v_SDU_SubPDUListDL ; var template (value) NR_MAC_PDUList_Type v_MAC_PDUListDL; var template (present) NR_MAC_SDU_SubPDU_List_Type v_SDU_SubPDUListUL; var template (present) NR_MAC_PDUList_Type v_MAC_PDUListUL1; var SubFrameTiming_Type v_TimingUL_1; var SubFrameTiming_Type v_TimingUL_2; var NR_DRB_COMMON_IND v_NR_DRB_COMMON_IND; var SubcarrierSpacing v_SCS := f_NR_CellInfo_GetSCS(nr_Cell1); var UInt_Type v_SlotsPerSubframe := f_NR_CellInfo_GetSCS_SlotsNumberPerSubframe(v_SCS); var UInt_Type v_SlotsPerCfg_Period := const_Cfg_Period *v_SlotsPerSubframe; var integer v_SlotsT1; timer t_Watchdog := 11.0; // @siclog "Step 3" siclog@ //@sic R5s240269 sic@ v_EncodedRlcPdu1 := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU(0, crs_NR_PDCP_SDU_96B,1))); v_EncodedRlcPdu2 := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU(1, crs_NR_PDCP_SDU_96B1,1))); v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1), cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu2)}; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL,cs_NR_MAC_Padding_SubPDU(tsc_NR_MAC_Padding_2B)) }; v_TimingDL:= f_NR_GetCurrentTiming(nr_Cell1); v_TimingDL.SFN.Number := 1022; v_TimingDL.Subframe.Number := 2; v_TimingDL.Slot.SlotOffset.Numerology1 := 0; v_TimingDL.Symbol := valueof (cs_SymbolTimingInfo_FirstSlot); //@sic R5s230287 sic@ DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR(v_TimingDL), v_MAC_PDUListDL)); t_Watchdog.start; // @siclog "Step 4" siclog@ v_SDU_SubPDUListUL := {cr_NR_MAC_SDU_SubPDU(cr_NR_MAC_SubHeaderWithLI(tsc_LCID_SCG_Bearer, int2bit(100, 8)), v_EncodedRlcPdu1)}; v_MAC_PDUListUL1 := { cr_NR_UL_MAC_PDU(v_SDU_SubPDUListUL, *, *) }; DRB.receive (car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, -, v_MAC_PDUListUL1))-> value v_NR_DRB_COMMON_IND; v_TimingUL_1 := v_NR_DRB_COMMON_IND.Common.TimingInfo.SubFrame; v_SlotsT1 := f_NR_SubFrameTiming_NumSlots(nr_Cell1, v_TimingUL_1); if (v_SlotsT1 mod v_SlotsPerCfg_Period == const_Cfg_TOffset) {f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4"); } else {f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 4"); } v_TimingUL_2 :=f_SubFrameTiming_AddMilliSeconds(v_TimingUL_1,const_Cfg_Period); //Step 5 Timing // @siclog "Step 5" siclog@ v_SDU_SubPDUListUL := {cr_NR_MAC_SDU_SubPDU(cr_NR_MAC_SubHeaderWithLI(tsc_LCID_SCG_Bearer, int2bit(100, 8)), v_EncodedRlcPdu2)}; v_MAC_PDUListUL1 := { cr_NR_UL_MAC_PDU(v_SDU_SubPDUListUL, *, *) }; DRB.receive (car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cr_TimingInfo_NR( v_TimingUL_2), v_MAC_PDUListUL1)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 5"); t_Watchdog.stop; }

## Change 2

| Function name | function f_TC_7_1_1_6_2_NR_TestBody2 |
| --- | --- |
| Reason for change | - As prose expecting from SS transmits a UL grant addressed to UE’s stored CS-RNTI with NDI set as 1 in Slot ‘p0’of PDCCH at step 11, so step11 should be configured before step 8, else SS may not have enough time to process it. Current TTCN timing is providing Ul grants to SS with CS-RNTI tranmissoin with NDI=0. In order to achive retranmission or NDI=1 for step 11 UL grants, timing of step 11 should be before step10 timing for SS stability. - Downlink SPS is not configured for this test case, so DRB_COMMON_REQ needs C-RNTI grant. Incorrect template is used in current TTCN. |
| Summary of change | - Updated UL timing for step11 according to timedomainoffset 19 for TDD and FDD before step 7A using DL transmission time of step 7A. - At step 7A change cads_NR_DRB_COMMON_REQ_MAC_PDUList_NoCRNTI_Grant into cas_NR_DRB_COMMON_REQ_MAC_PDUList |
| TTCN module | MAC_SPS_TC_Common_NR |
| MCC160 Comment |  |

Before Change

function f_TC_7_1_1_6_2_NR_TestBody2(DRB_Identity p_NR_DRB_Id, NR_ResourceAllocation_Type p_NR_ResourceAllocation_CSRNTI ) runs on NR_BASE_PTC {//@sic R5s230287 sic@ const integer const_Cfg_Period :=40; // period in milli seconds //@sic R5-236417 sic@ const integer const_Cfg_TOffset := 19; // In slots const integer const_UL_TBS_CRNTI := 848;// to allow 2 PDCP SDU's var SubFrameTiming_Type v_TimingDL; var SubFrameTiming_Type v_TimingDL1; var SubFrameTiming_Type v_TimingUL_1; var SubFrameTiming_Type v_TimingUL_2; var SubFrameTiming_Type v_TimingUL_3; var SubFrameTiming_Type v_TimingUL_4; var SubFrameTiming_Type v_TimingUL_5; var SubFrameTiming_Type v_TimingUL_6; var NR_DRB_COMMON_IND v_NR_DRB_COMMON_IND; var SubcarrierSpacing v_SCS := f_NR_CellInfo_GetSCS(nr_Cell1); var NR_UplinkBWP_Type v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(nr_Cell1, tsc_NR_BWP_Id); var UInt_Type v_SlotsPerSubframe := f_NR_CellInfo_GetSCS_SlotsNumberPerSubframe(v_SCS); var UInt_Type v_SlotsPerCfg_Period := const_Cfg_Period *v_SlotsPerSubframe; var integer v_SlotsT1; var template (value) NR_MAC_SDU_SubPDU_List_Type v_SDU_SubPDUListDL ; var template (value) NR_MAC_PDUList_Type v_MAC_PDUListDL; var template (present) NR_MAC_SDU_SubPDU_List_Type v_SDU_SubPDUListUL; var template (present) NR_MAC_PDUList_Type v_MAC_PDUListUL1; var template (present) NR_MAC_PDUList_Type v_MAC_PDUListUL2; var template (present) NR_MAC_PDUList_Type v_MAC_PDUListUL3; var template (present) NR_MAC_PDUList_Type v_MAC_PDUListUL4; var template (present) NR_MAC_PDUList_Type v_MAC_PDUListUL5; //var NR_MAC_SDU_Type v_EncodedRlcPdu1; //var NR_MAC_SDU_Type v_EncodedRlcPdu2; var NR_MAC_SDU_Type v_EncodedRlcPdu3; var NR_MAC_SDU_Type v_EncodedRlcPdu4; var NR_MAC_SDU_Type v_EncodedRlcPdu5; var NR_MAC_SDU_Type v_EncodedRlcPdu6; var NR_MAC_SDU_Type v_EncodedRlcPdu7; var NR_ResourceAllocation_Type v_NR_ResourceAllocation_CRNTI := f_NR_ResourceAllocation_UplinkGrant(const_UL_TBS_CRNTI, v_NR_UplinkBWP); var NR_ResourceAllocation_Type v_NR_ResourceAllocation_CSRNTI := p_NR_ResourceAllocation_CSRNTI; timer t_Watchdog := 11.0; // Initialise v_SubFrameInfo, v_SlotTimingInfo for UL grant on PDCCH K2 =4 slots ahead; // @siclog "Step 7A" siclog@ // PDCP SDU =12B +3B PDCP Header+ 1B RLC UM Header + 2B MAC Header =18 byte =144 bits // 6 MAC Sub PDU's 6*144 = 864. 888 is next allowed downlink TBS in all tables, 24 bit =3 byte padding in DL. //@sic R5-227518 sic@ v_EncodedRlcPdu3 := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (2, crs_NR_PDCP_SDU_96B2,1))); v_EncodedRlcPdu4 := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (3, crs_NR_PDCP_SDU_96B3,1))); v_EncodedRlcPdu5 := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (4, crs_NR_PDCP_SDU_96B4,1))); v_EncodedRlcPdu6 := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (5, crs_NR_PDCP_SDU_96B5,1))); v_EncodedRlcPdu7 := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (6, crs_NR_PDCP_SDU_96B6,1))); v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu3), cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu4), cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu5), cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu6), cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu7)}; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL,cs_NR_MAC_Padding_SubPDU(tsc_NR_MAC_Padding_2B)) }; //Prepare expected UL MAC PDU's //Step 9 v_SDU_SubPDUListUL := {cr_NR_MAC_SDU_SubPDU(cr_NR_MAC_SubHeaderWithLI(tsc_LCID_SCG_Bearer, int2bit(100, 8)), v_EncodedRlcPdu3)}; v_MAC_PDUListUL1 := { cr_NR_UL_MAC_PDU(v_SDU_SubPDUListUL, *, *) }; //Step 10 & 12 v_SDU_SubPDUListUL := {cr_NR_MAC_SDU_SubPDU(cr_NR_MAC_SubHeaderWithLI(tsc_LCID_SCG_Bearer, int2bit(100, 8)), v_EncodedRlcPdu4)}; v_MAC_PDUListUL2 := { cr_NR_UL_MAC_PDU(v_SDU_SubPDUListUL, *, *) }; //Step 13 v_SDU_SubPDUListUL := {cr_NR_MAC_SDU_SubPDU(cr_NR_MAC_SubHeaderWithLI(tsc_LCID_SCG_Bearer, int2bit(100, 8)), v_EncodedRlcPdu5)}; v_MAC_PDUListUL3 := { cr_NR_UL_MAC_PDU(v_SDU_SubPDUListUL, *, *) }; //Step 15 v_SDU_SubPDUListUL := {cr_NR_MAC_SDU_SubPDU(cr_NR_MAC_SubHeaderWithLI(tsc_LCID_SCG_Bearer, int2bit(100, 8)), v_EncodedRlcPdu6)}; v_MAC_PDUListUL4 := { cr_NR_UL_MAC_PDU(v_SDU_SubPDUListUL, *, *) }; //Step 16 v_SDU_SubPDUListUL := {cr_NR_MAC_SDU_SubPDU(cr_NR_MAC_SubHeaderWithLI(tsc_LCID_SCG_Bearer, int2bit(100, 8)), v_EncodedRlcPdu7)}; v_MAC_PDUListUL5 := { cr_NR_UL_MAC_PDU(v_SDU_SubPDUListUL, *, *) }; v_TimingDL:= f_NR_GetCurrentTiming(nr_Cell1); v_TimingDL.SFN.Number := 1022; v_TimingDL.Subframe.Number := 2; v_TimingDL.Slot.SlotOffset.Numerology1 := 0; v_TimingDL.Symbol := valueof (cs_SymbolTimingInfo_FirstSlot); DRB.send(cads_NR_DRB_COMMON_REQ_MAC_PDUList_NoCRNTI_Grant(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR(v_TimingDL), v_MAC_PDUListDL)); // @siclog "Step 8 and 9" siclog@ t_Watchdog.start; DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, -, v_MAC_PDUListUL1))-> value v_NR_DRB_COMMON_IND; v_TimingUL_1 := v_NR_DRB_COMMON_IND.Common.TimingInfo.SubFrame; v_SlotsT1 := f_NR_SubFrameTiming_NumSlots(nr_Cell1, v_TimingUL_1); if (v_SlotsT1 mod v_SlotsPerCfg_Period == const_Cfg_TOffset) {f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 9"); } else {f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 8"); } v_TimingUL_2 :=f_SubFrameTiming_AddMilliSeconds(v_TimingUL_1,const_Cfg_Period); //Step 10 Timing v_TimingUL_3 :=f_SubFrameTiming_AddMilliSeconds(v_TimingUL_2,const_Cfg_Period-10); //Step 12 Timing v_TimingDL := f_NR_PDCCHSendOccasion_UL(nr_Cell1, v_TimingUL_3 );//Step 11 Timing v_TimingDL.Symbol := valueof(cs_SymbolTimingInfo_FirstSlot); v_TimingUL_4 :=f_SubFrameTiming_AddMilliSeconds(v_TimingUL_2,const_Cfg_Period); //Step 13 Timing v_TimingUL_5 :=f_SubFrameTiming_AddMilliSeconds(v_TimingUL_4,const_Cfg_Period); //Step 15 Timing v_TimingDL1 := f_NR_PDCCHSendOccasion_UL(nr_Cell1, v_TimingUL_5 );//Step 14 Timing v_TimingDL1.Symbol := valueof (cs_SymbolTimingInfo_FirstSlot); v_TimingUL_6 :=f_SubFrameTiming_AddMilliSeconds(v_TimingUL_5,const_Cfg_Period); //Step 16 Timing // @siclog "Step 10" siclog@ DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id,cr_TimingInfo_NR( v_TimingUL_2), v_MAC_PDUListUL2)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 10"); // @siclog "Step 11" siclog@ f_NR_UL_CgGrantConfiguration(nr_Cell1,cs_TimingInfo_NR( v_TimingDL), v_NR_ResourceAllocation_CSRNTI, true, -, -, true); //NDI= 1 //@siclog "Step 12" siclog@ DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id,cr_TimingInfo_NR( v_TimingUL_3), v_MAC_PDUListUL2)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 12"); // @siclog "Step 13" siclog@ DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id,cr_TimingInfo_NR( v_TimingUL_4), v_MAC_PDUListUL3)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 13"); // @siclog "Step 14" siclog@ f_NR_OneULGrantTransmission(nr_Cell1, cs_TimingInfo_NR(v_TimingDL1), v_NR_ResourceAllocation_CRNTI); // @siclog "Step 15" siclog@ DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id,cr_TimingInfo_NR( v_TimingUL_5), v_MAC_PDUListUL4)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 15"); // @siclog "Step 16" siclog@ DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id,cr_TimingInfo_NR( v_TimingUL_6), v_MAC_PDUListUL5)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 16"); t_Watchdog.stop }

After Change

function f_TC_7_1_1_6_2_NR_TestBody2(DRB_Identity p_NR_DRB_Id, NR_ResourceAllocation_Type p_NR_ResourceAllocation_CSRNTI ) runs on NR_BASE_PTC {//@sic R5s230287 sic@ const integer const_Cfg_Period :=40; // period in milli seconds //@sic R5-236417 sic@ const integer const_Cfg_TOffset := 19; // In slots const integer const_UL_TBS_CRNTI := 848;// to allow 2 PDCP SDU's var SubFrameTiming_Type v_TimingDL; var SubFrameTiming_Type v_TimingDL_step11; var SubFrameTiming_Type v_TimingDL1; var SubFrameTiming_Type v_TimingUL_1; var SubFrameTiming_Type v_TimingUL_2; var SubFrameTiming_Type v_TimingUL_3; var SubFrameTiming_Type v_TimingUL_4; var SubFrameTiming_Type v_TimingUL_5; var SubFrameTiming_Type v_TimingUL_6; var NR_DRB_COMMON_IND v_NR_DRB_COMMON_IND; var SubcarrierSpacing v_SCS := f_NR_CellInfo_GetSCS(nr_Cell1); var NR_UplinkBWP_Type v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(nr_Cell1, tsc_NR_BWP_Id); var UInt_Type v_SlotsPerSubframe := f_NR_CellInfo_GetSCS_SlotsNumberPerSubframe(v_SCS); var UInt_Type v_SlotsPerCfg_Period := const_Cfg_Period *v_SlotsPerSubframe; var integer v_SlotsT1; var template (value) NR_MAC_SDU_SubPDU_List_Type v_SDU_SubPDUListDL ; var template (value) NR_MAC_PDUList_Type v_MAC_PDUListDL; var template (present) NR_MAC_SDU_SubPDU_List_Type v_SDU_SubPDUListUL; var template (present) NR_MAC_PDUList_Type v_MAC_PDUListUL1; var template (present) NR_MAC_PDUList_Type v_MAC_PDUListUL2; var template (present) NR_MAC_PDUList_Type v_MAC_PDUListUL3; var template (present) NR_MAC_PDUList_Type v_MAC_PDUListUL4; var template (present) NR_MAC_PDUList_Type v_MAC_PDUListUL5; //var NR_MAC_SDU_Type v_EncodedRlcPdu1; //var NR_MAC_SDU_Type v_EncodedRlcPdu2; var NR_MAC_SDU_Type v_EncodedRlcPdu3; var NR_MAC_SDU_Type v_EncodedRlcPdu4; var NR_MAC_SDU_Type v_EncodedRlcPdu5; var NR_MAC_SDU_Type v_EncodedRlcPdu6; var NR_MAC_SDU_Type v_EncodedRlcPdu7; var NR_ResourceAllocation_Type v_NR_ResourceAllocation_CRNTI := f_NR_ResourceAllocation_UplinkGrant(const_UL_TBS_CRNTI, v_NR_UplinkBWP); var NR_ResourceAllocation_Type v_NR_ResourceAllocation_CSRNTI := p_NR_ResourceAllocation_CSRNTI; timer t_Watchdog := 11.0; // Initialise v_SubFrameInfo, v_SlotTimingInfo for UL grant on PDCCH K2 =4 slots ahead; // PDCP SDU =12B +3B PDCP Header+ 1B RLC UM Header + 2B MAC Header =18 byte =144 bits // 6 MAC Sub PDU's 6*144 = 864. 888 is next allowed downlink TBS in all tables, 24 bit =3 byte padding in DL. //@sic R5-227518 sic@ v_EncodedRlcPdu3 := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (2, crs_NR_PDCP_SDU_96B2,1))); v_EncodedRlcPdu4 := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (3, crs_NR_PDCP_SDU_96B3,1))); v_EncodedRlcPdu5 := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (4, crs_NR_PDCP_SDU_96B4,1))); v_EncodedRlcPdu6 := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (5, crs_NR_PDCP_SDU_96B5,1))); v_EncodedRlcPdu7 := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (6, crs_NR_PDCP_SDU_96B6,1))); v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu3), cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu4), cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu5), cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu6), cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu7)}; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL,cs_NR_MAC_Padding_SubPDU(tsc_NR_MAC_Padding_2B)) }; //Prepare expected UL MAC PDU's //Step 9 v_SDU_SubPDUListUL := {cr_NR_MAC_SDU_SubPDU(cr_NR_MAC_SubHeaderWithLI(tsc_LCID_SCG_Bearer, int2bit(100, 8)), v_EncodedRlcPdu3)}; v_MAC_PDUListUL1 := { cr_NR_UL_MAC_PDU(v_SDU_SubPDUListUL, *, *) }; //Step 10 & 12 v_SDU_SubPDUListUL := {cr_NR_MAC_SDU_SubPDU(cr_NR_MAC_SubHeaderWithLI(tsc_LCID_SCG_Bearer, int2bit(100, 8)), v_EncodedRlcPdu4)}; v_MAC_PDUListUL2 := { cr_NR_UL_MAC_PDU(v_SDU_SubPDUListUL, *, *) }; //Step 13 v_SDU_SubPDUListUL := {cr_NR_MAC_SDU_SubPDU(cr_NR_MAC_SubHeaderWithLI(tsc_LCID_SCG_Bearer, int2bit(100, 8)), v_EncodedRlcPdu5)}; v_MAC_PDUListUL3 := { cr_NR_UL_MAC_PDU(v_SDU_SubPDUListUL, *, *) }; //Step 15 v_SDU_SubPDUListUL := {cr_NR_MAC_SDU_SubPDU(cr_NR_MAC_SubHeaderWithLI(tsc_LCID_SCG_Bearer, int2bit(100, 8)), v_EncodedRlcPdu6)}; v_MAC_PDUListUL4 := { cr_NR_UL_MAC_PDU(v_SDU_SubPDUListUL, *, *) }; //Step 16 v_SDU_SubPDUListUL := {cr_NR_MAC_SDU_SubPDU(cr_NR_MAC_SubHeaderWithLI(tsc_LCID_SCG_Bearer, int2bit(100, 8)), v_EncodedRlcPdu7)}; v_MAC_PDUListUL5 := { cr_NR_UL_MAC_PDU(v_SDU_SubPDUListUL, *, *) }; v_TimingDL:= f_NR_GetCurrentTiming(nr_Cell1); v_TimingDL.SFN.Number := 1022; v_TimingDL.Subframe.Number := 2; v_TimingDL.Slot.SlotOffset.Numerology1 := 0; v_TimingDL.Symbol := valueof (cs_SymbolTimingInfo_FirstSlot); v_TimingUL_1.HSFN.Number := v_TimingDL.HSFN.Number + 1; if (v_SCS == kHz15) { v_TimingUL_1.SFN.Number := 1; } // timedomainoffset set to 19 for FDD else {v_TimingUL_1.SFN.Number := 0;} v_TimingUL_1.Subframe.Number := 9; if (v_SCS == kHz15) { v_TimingUL_1.Slot.SlotOffset.Numerology0 := true; } else {v_TimingUL_1.Slot.SlotOffset.Numerology1 := 1;} v_TimingUL_1.Symbol := valueof(cs_SymbolTimingInfo_Any); v_TimingUL_2 :=f_SubFrameTiming_AddMilliSeconds(v_TimingUL_1,const_Cfg_Period); //Step 10 Timing v_TimingUL_3 :=f_SubFrameTiming_AddMilliSeconds(v_TimingUL_2,const_Cfg_Period-10); //Step 12 Timing v_TimingDL_step11 := f_NR_PDCCHSendOccasion_UL(nr_Cell1, v_TimingUL_3 );//Step 11 Timing v_TimingDL_step11.Symbol := valueof(cs_SymbolTimingInfo_FirstSlot); f_NR_UL_CgGrantConfiguration(nr_Cell1,cs_TimingInfo_NR( v_TimingDL_step11), v_NR_ResourceAllocation_CSRNTI, true, -, -, true); //NDI= 1 // step 11 configuration // @siclog "Step 7A" siclog@ DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR(v_TimingDL), v_MAC_PDUListDL)); // @siclog "Step 8 and 9" siclog@ t_Watchdog.start; DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, -, v_MAC_PDUListUL1))-> value v_NR_DRB_COMMON_IND; v_TimingUL_1 := v_NR_DRB_COMMON_IND.Common.TimingInfo.SubFrame; v_SlotsT1 := f_NR_SubFrameTiming_NumSlots(nr_Cell1, v_TimingUL_1); if (v_SlotsT1 mod v_SlotsPerCfg_Period == const_Cfg_TOffset) {f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 9"); } else {f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 8"); } //v_TimingUL_2 :=f_SubFrameTiming_AddMilliSeconds(v_TimingUL_1,const_Cfg_Period); //Step 10 Timing //v_TimingUL_3 :=f_SubFrameTiming_AddMilliSeconds(v_TimingUL_2,const_Cfg_Period-10); //Step 12 Timing //v_TimingDL := f_NR_PDCCHSendOccasion_UL(nr_Cell1, v_TimingUL_3 );//Step 11 Timing //v_TimingDL.Symbol := valueof(cs_SymbolTimingInfo_FirstSlot); v_TimingUL_4 :=f_SubFrameTiming_AddMilliSeconds(v_TimingUL_2,const_Cfg_Period); //Step 13 Timing v_TimingUL_5 :=f_SubFrameTiming_AddMilliSeconds(v_TimingUL_4,const_Cfg_Period); //Step 15 Timing v_TimingDL1 := f_NR_PDCCHSendOccasion_UL(nr_Cell1, v_TimingUL_5 );//Step 14 Timing v_TimingDL1.Symbol := valueof (cs_SymbolTimingInfo_FirstSlot); v_TimingUL_6 :=f_SubFrameTiming_AddMilliSeconds(v_TimingUL_5,const_Cfg_Period); //Step 16 Timing // @siclog "Step 10" siclog@ DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id,cr_TimingInfo_NR( v_TimingUL_2), v_MAC_PDUListUL2)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 10"); // @siclog "Step 11" siclog@ //f_NR_UL_CgGrantConfiguration(nr_Cell1,cs_TimingInfo_NR( v_TimingDL), v_NR_ResourceAllocation_CSRNTI, true, -, -, true); //NDI= 1 //Moved before step7A //@siclog "Step 12" siclog@ DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id,cr_TimingInfo_NR( v_TimingUL_3), v_MAC_PDUListUL2)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 12"); // @siclog "Step 13" siclog@ DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id,cr_TimingInfo_NR( v_TimingUL_4), v_MAC_PDUListUL3)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 13"); // @siclog "Step 14" siclog@ f_NR_OneULGrantTransmission(nr_Cell1, cs_TimingInfo_NR(v_TimingDL1), v_NR_ResourceAllocation_CRNTI); // @siclog "Step 15" siclog@ DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id,cr_TimingInfo_NR( v_TimingUL_5), v_MAC_PDUListUL4)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 15"); // @siclog "Step 16" siclog@ DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id,cr_TimingInfo_NR( v_TimingUL_6), v_MAC_PDUListUL5)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 16"); t_Watchdog.stop }