3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240438

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3471 | rev | - | Current version: | 17.10.0 |  |
| For HELPon using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed changeaffects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function f_NR_GetFrequencyMFBI_f1tof4 (for RedCap) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Starpoint |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-07-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According to TS38.508-1 Table 6.2.3.5-1A, Test frequencies for MFBI NR bands in FR1 for RedCap UE should be same values as for Low, Mid and High range in clause 4.3.1.1.1.78 for bandwidth=20 MHz and SCS=30 kHz. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Implemented test frequencies for MFBI signalling of RedCap according the 38508 Table 4.3.1.1.1.78-2. using the pic pc_supportOfRedCap_r17. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test cases. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.2.23.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

- Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D24wk24 related to the title of this CR.

Contact:	Ting Gao
	gaoting@starpointcomm.com

- Corrections required

  - Change 1

| Function name | f_NR_GetFrequencyMFBI_f1tof4 () |
| --- | --- |
| Reason for change | According to TS38.508-1 Table 6.2.3.5-1A, Test frequencies for MFBI NR bands in FR1 for RedCap UE should be same values as for Low, Mid and High range in clause 4.3.1.1.1.78 for bandwidth=20 MHz and SCS=30 kHz. |
| Summary of change | Implemented test frequencies for MFBI signalling of RedCap according the 38508 Table 4.3.1.1.1.78-2. using the pic pc_supportOfRedCap_r17. |
| TTCN module | NR_FrequencyInit_MFBI.ttcn |
| MCC160 Comment |  |

Before Change:

function f_NR_GetFrequencyMFBI_f1tof4 ( FreqBandIndicatorNR p_FreqBandIndicatorNR, FreqBandIndicatorNR p_FreqOverlappingBandIndicatorNR ) return template (value) NR_Frequencyf1tof4_Type { var template (value) NR_Frequencyf1tof4_Type v_NR_Frequencyf1tof4 := cs_NR_Frequencyf1tof4_Dummy; var P_Max v_P_Max := f_NR_InitP_Max(); … case( 77 ){ select (p_FreqOverlappingBandIndicatorNR){ case( 78 ){ //@sic R5-201022 R5s200826 R5s230519 R5s230882 sic@ if ( pc_supportOfRedCap_r17 ){ v_NR_Frequencyf1tof4.FrequencyInfoDL_f1 := cs_SS_FrequencyInfoDL(cs_FrequencyInfoDL( 620352 , 77 , 620040 , 0 , kHz30 , 51 ), 20 , kHz30 , 0 , 1 , 6 , 0 , 6 ); v_NR_Frequencyf1tof4.FrequencyInfoUL_f1 := cs_SS_FrequencyInfoUL(cs_FrequencyInfoUL( -, -, 0 , kHz30 , 51 ,v_P_Max), 20 ); v_NR_Frequencyf1tof4.FrequencyInfoDL_f2 := cs_SS_FrequencyInfoDL(cs_FrequencyInfoDL( 636000 , 77 , 634482 , 102 , kHz30 , 51 ), 20 , kHz30 , 6 , 0 , 106 , 2 , 2 ); v_NR_Frequencyf1tof4.FrequencyInfoUL_f2 := cs_SS_FrequencyInfoUL(cs_FrequencyInfoUL( -, -, 102 , kHz30 , 51 ,v_P_Max), 20 ); v_NR_Frequencyf1tof4.FrequencyInfoDL_f3 := cs_SS_FrequencyInfoDL(cs_FrequencyInfoDL( 651648 , 77 , 645324 , 504 , kHz30 , 51 ), 20 , kHz30 , 0 , 0 , 507 , 1 , 2 ); v_NR_Frequencyf1tof4.FrequencyInfoUL_f3 := cs_SS_FrequencyInfoUL(cs_FrequencyInfoUL( -, -, 504 , kHz30 , 51 ,v_P_Max), 20 ); } else { //@sic R5-201022 R5s200826 sic@ v_NR_Frequencyf1tof4.FrequencyInfoDL_f1 := cs_SS_FrequencyInfoDL(cs_FrequencyInfoDL( 620352 , 77 , 620058 , 0 , kHz30 , 273 ), 100 , kHz30 , 6 , 2 , 4 , 0 , 2 ); v_NR_Frequencyf1tof4.FrequencyInfoUL_f1 := cs_SS_FrequencyInfoUL(cs_FrequencyInfoUL( -, -, 0 , kHz30 , 273 ,v_P_Max), 100 ); v_NR_Frequencyf1tof4.FrequencyInfoDL_f2 := cs_SS_FrequencyInfoDL(cs_FrequencyInfoDL( 633696 , 77 , 630942 , 102 , kHz30 , 273 ), 100 , kHz30 , 18 , 2 , 208 , 0 , 2 ); v_NR_Frequencyf1tof4.FrequencyInfoUL_f2 := cs_SS_FrequencyInfoUL(cs_FrequencyInfoUL( -, -, 102 , kHz30 , 273 ,v_P_Max), 100 ); v_NR_Frequencyf1tof4.FrequencyInfoDL_f3 := cs_SS_FrequencyInfoDL(cs_FrequencyInfoDL( 647040 , 77 , 634628 , 504 , kHz30 , 273 ), 100 , kHz30 , 4 , 3 , 1014 , 0 , 3 ); v_NR_Frequencyf1tof4.FrequencyInfoUL_f3 := cs_SS_FrequencyInfoUL(cs_FrequencyInfoUL( -, -, 504 , kHz30 , 273 ,v_P_Max), 100 ); } } case else { FatalError(__FILE__, __LINE__, "Invalid NR MFBI band combination")} } } case( 78 ){ select (p_FreqOverlappingBandIndicatorNR){ case( 77 ){ //@sic R5-201023 R5s230519 R5s230882 sic@ if ( pc_supportOfRedCap_r17 ){ v_NR_Frequencyf1tof4.FrequencyInfoDL_f1 := cs_SS_FrequencyInfoDL(cs_FrequencyInfoDL( 620352 , 78 , 620040 , 0 , kHz30 , 51 ), 20 , kHz30 , 0 , 1 , 6 , 0 , 6 ); v_NR_Frequencyf1tof4.FrequencyInfoUL_f1 := cs_SS_FrequencyInfoUL(cs_FrequencyInfoUL( -, -, 0 , kHz30 , 51 ,v_P_Max), 20 ); v_NR_Frequencyf1tof4.FrequencyInfoDL_f2 := cs_SS_FrequencyInfoDL(cs_FrequencyInfoDL( 636000 , 78 , 634482 , 102 , kHz30 , 51 ), 20 , kHz30 , 6 , 0 , 106 , 2 , 2 ); v_NR_Frequencyf1tof4.FrequencyInfoUL_f2 := cs_SS_FrequencyInfoUL(cs_FrequencyInfoUL( -, -, 102 , kHz30 , 51 ,v_P_Max), 20 ); v_NR_Frequencyf1tof4.FrequencyInfoDL_f3 := cs_SS_FrequencyInfoDL(cs_FrequencyInfoDL( 651648 , 78 , 645324 , 504 , kHz30 , 51 ), 20 , kHz30 , 0 , 0 , 507 , 1 , 2 ); v_NR_Frequencyf1tof4.FrequencyInfoUL_f3 := cs_SS_FrequencyInfoUL(cs_FrequencyInfoUL( -, -, 504 , kHz30 , 51 ,v_P_Max), 20 ); } else { //@sic R5-201023 sic@ v_NR_Frequencyf1tof4.FrequencyInfoDL_f1 := cs_SS_FrequencyInfoDL(cs_FrequencyInfoDL( 620352 , 78 , 620058 , 0 , kHz30 , 273 ), 100 , kHz30 , 6 , 2 , 4 , 0 , 2 ); v_NR_Frequencyf1tof4.FrequencyInfoUL_f1 := cs_SS_FrequencyInfoUL(cs_FrequencyInfoUL( -, -, 0 , kHz30 , 273 ,v_P_Max), 100 ); v_NR_Frequencyf1tof4.FrequencyInfoDL_f2 := cs_SS_FrequencyInfoDL(cs_FrequencyInfoDL( 633696 , 78 , 630942 , 102 , kHz30 , 273 ), 100 , kHz30 , 18 , 2 , 208 , 0 , 2 ); v_NR_Frequencyf1tof4.FrequencyInfoUL_f2 := cs_SS_FrequencyInfoUL(cs_FrequencyInfoUL( -, -, 102 , kHz30 , 273 ,v_P_Max), 100 ); v_NR_Frequencyf1tof4.FrequencyInfoDL_f3 := cs_SS_FrequencyInfoDL(cs_FrequencyInfoDL( 647040 , 78 , 634628 , 504 , kHz30 , 273 ), 100 , kHz30 , 4 , 3 , 1014 , 0 , 3 ); v_NR_Frequencyf1tof4.FrequencyInfoUL_f3 := cs_SS_FrequencyInfoUL(cs_FrequencyInfoUL( -, -, 504 , kHz30 , 273 ,v_P_Max), 100 ); } } case else { FatalError(__FILE__, __LINE__, "Invalid NR MFBI band combination")} } } … return v_NR_Frequencyf1tof4; }

After Change:

function f_NR_GetFrequencyMFBI_f1tof4 ( FreqBandIndicatorNR p_FreqBandIndicatorNR, FreqBandIndicatorNR p_FreqOverlappingBandIndicatorNR ) return template (value) NR_Frequencyf1tof4_Type { var template (value) NR_Frequencyf1tof4_Type v_NR_Frequencyf1tof4 := cs_NR_Frequencyf1tof4_Dummy; var P_Max v_P_Max := f_NR_InitP_Max(); … case( 77 ){ select (p_FreqOverlappingBandIndicatorNR){ case( 78 ){ //@sic R5-201022 R5s200826 R5s230519 R5s230882 sic@ if ( pc_supportOfRedCap_r17 ){ v_NR_Frequencyf1tof4.FrequencyInfoDL_f1 := cs_SS_FrequencyInfoDL(cs_FrequencyInfoDL( 620352 , 77 , 620056 , 0 , kHz30 , 51 ), 20, kHz30 , 8 , 2 , 4 , 0 , 2 ); v_NR_Frequencyf1tof4.FrequencyInfoUL_f1 := cs_SS_FrequencyInfoUL(cs_FrequencyInfoUL( -, -, 0 , kHz30 , 51 ,v_P_Max), 20 ); v_NR_Frequencyf1tof4.FrequencyInfoDL_f2 := cs_SS_FrequencyInfoDL(cs_FrequencyInfoDL( 636384 , 77 , 633606 , 102 , kHz30 , 51 ), 20, kHz30 , 18 , 3 , 210 , 0 , 3 ); v_NR_Frequencyf1tof4.FrequencyInfoUL_f2 := cs_SS_FrequencyInfoUL(cs_FrequencyInfoUL( -, -, 102 , kHz30 , 51 ,v_P_Max), 20 ); v_NR_Frequencyf1tof4.FrequencyInfoDL_f3 := cs_SS_FrequencyInfoDL(cs_FrequencyInfoDL( 652320 , 77 , 639958 , 504 , kHz30 , 51 ), 20 , kHz30 , 2 , 1 , 1010 , 0 , 1 ); v_NR_Frequencyf1tof4.FrequencyInfoUL_f3 := cs_SS_FrequencyInfoUL(cs_FrequencyInfoUL( -, -, 504 , kHz30 , 51 ,v_P_Max), 20 ); } else { //@sic R5-201022 R5s200826 sic@ v_NR_Frequencyf1tof4.FrequencyInfoDL_f1 := cs_SS_FrequencyInfoDL(cs_FrequencyInfoDL( 620352 , 77 , 620058 , 0 , kHz30 , 273 ), 100 , kHz30 , 6 , 2 , 4 , 0 , 2 ); v_NR_Frequencyf1tof4.FrequencyInfoUL_f1 := cs_SS_FrequencyInfoUL(cs_FrequencyInfoUL( -, -, 0 , kHz30 , 273 ,v_P_Max), 100 ); v_NR_Frequencyf1tof4.FrequencyInfoDL_f2 := cs_SS_FrequencyInfoDL(cs_FrequencyInfoDL( 633696 , 77 , 630942 , 102 , kHz30 , 273 ), 100 , kHz30 , 18 , 2 , 208 , 0 , 2 ); v_NR_Frequencyf1tof4.FrequencyInfoUL_f2 := cs_SS_FrequencyInfoUL(cs_FrequencyInfoUL( -, -, 102 , kHz30 , 273 ,v_P_Max), 100 ); v_NR_Frequencyf1tof4.FrequencyInfoDL_f3 := cs_SS_FrequencyInfoDL(cs_FrequencyInfoDL( 647040 , 77 , 634628 , 504 , kHz30 , 273 ), 100 , kHz30 , 4 , 3 , 1014 , 0 , 3 ); v_NR_Frequencyf1tof4.FrequencyInfoUL_f3 := cs_SS_FrequencyInfoUL(cs_FrequencyInfoUL( -, -, 504 , kHz30 , 273 ,v_P_Max), 100 ); } } case else { FatalError(__FILE__, __LINE__, "Invalid NR MFBI band combination")} } } case( 78 ){ select (p_FreqOverlappingBandIndicatorNR){ case( 77 ){ //@sic R5-201023 R5s230519 R5s230882 sic@ if ( pc_supportOfRedCap_r17 ){ v_NR_Frequencyf1tof4.FrequencyInfoDL_f1 := cs_SS_FrequencyInfoDL(cs_FrequencyInfoDL( 620352 , 78 , 620056 , 0 , kHz30 , 51 ), 20, kHz30 , 8 , 2 , 4 , 0 , 2 ); v_NR_Frequencyf1tof4.FrequencyInfoUL_f1 := cs_SS_FrequencyInfoUL(cs_FrequencyInfoUL( -, -, 0 , kHz30 , 51 ,v_P_Max), 20 ); v_NR_Frequencyf1tof4.FrequencyInfoDL_f2 := cs_SS_FrequencyInfoDL(cs_FrequencyInfoDL( 636384 , 78 , 633606 , 102 , kHz30 , 51 ), 20, kHz30 , 18 , 3 , 210 , 0 , 3 ); v_NR_Frequencyf1tof4.FrequencyInfoUL_f2 := cs_SS_FrequencyInfoUL(cs_FrequencyInfoUL( -, -, 102 , kHz30 , 51 ,v_P_Max), 20 ); v_NR_Frequencyf1tof4.FrequencyInfoDL_f3 := cs_SS_FrequencyInfoDL(cs_FrequencyInfoDL( 652320 , 78 , 639958 , 504 , kHz30 , 51 ), 20 , kHz30 , 2 , 1 , 1010 , 0 , 1 ); v_NR_Frequencyf1tof4.FrequencyInfoUL_f3 := cs_SS_FrequencyInfoUL(cs_FrequencyInfoUL( -, -, 504 , kHz30 , 51 ,v_P_Max), 20 ); } else { //@sic R5-201023 sic@ v_NR_Frequencyf1tof4.FrequencyInfoDL_f1 := cs_SS_FrequencyInfoDL(cs_FrequencyInfoDL( 620352 , 78 , 620058 , 0 , kHz30 , 273 ), 100 , kHz30 , 6 , 2 , 4 , 0 , 2 ); v_NR_Frequencyf1tof4.FrequencyInfoUL_f1 := cs_SS_FrequencyInfoUL(cs_FrequencyInfoUL( -, -, 0 , kHz30 , 273 ,v_P_Max), 100 ); v_NR_Frequencyf1tof4.FrequencyInfoDL_f2 := cs_SS_FrequencyInfoDL(cs_FrequencyInfoDL( 633696 , 78 , 630942 , 102 , kHz30 , 273 ), 100 , kHz30 , 18 , 2 , 208 , 0 , 2 ); v_NR_Frequencyf1tof4.FrequencyInfoUL_f2 := cs_SS_FrequencyInfoUL(cs_FrequencyInfoUL( -, -, 102 , kHz30 , 273 ,v_P_Max), 100 ); v_NR_Frequencyf1tof4.FrequencyInfoDL_f3 := cs_SS_FrequencyInfoDL(cs_FrequencyInfoDL( 647040 , 78 , 634628 , 504 , kHz30 , 273 ), 100 , kHz30 , 4 , 3 , 1014 , 0 , 3 ); v_NR_Frequencyf1tof4.FrequencyInfoUL_f3 := cs_SS_FrequencyInfoUL(cs_FrequencyInfoUL( -, -, 504 , kHz30 , 273 ,v_P_Max), 100 ); } } case else { FatalError(__FILE__, __LINE__, "Invalid NR MFBI band combination")} } } … return v_NR_Frequencyf1tof4; }