3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240444

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4840 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 Jun'24 baseline upgrade for LTE & NB-IoT Test Suites |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18_Test |  |  |  |  |  | Date: |  |  | 2024-07-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In order to develop the Rel-18 test cases in TTCN-3, the current baseline shall be upgraded. RAN5 agreed to move baseline to June 2024 Rel-18 to all RAN5 TTCN-3 test suites. This CR is implementing this decision on 36.523-3 Test Suites. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The CR lists all changes affecting the messages and IEs type definitions caused by the baseline moving to the June 2024 Rel-18. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Rel-18 test cases cannot be implemented and verified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | All LTE & NB-IoT Test Suites |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

Overview	3

1	Baseline Moving – 36.331 (RRC ASN.1)	4

1.1	EUTRA_RRC_ASN1_Definitions	4

1.1.1	Type change of nonCriticalExtension	4

1.1.2	Name changes	5

1.1.3	Fields added with extension marker (ellipsis)	5

1.1.4	Further ASN.1 changes	6

1.2	NBIOT_RRC_ASN1_Definitions	6

1.2.1	Type change of nonCriticalExtension	6

1.2.2	Name changes	6

1.2.3	Fields added with extension marker (ellipsis)	6

1.2.4	Further ASN.1 changes	7

1.3	EUTRA_Sidelink_Preconf	7

1.4	EUTRA_PC5_RRC_Definitions	7

2	Baseline Moving – 24.334 (ProSe)	7

3	Baseline Moving – NAS/PDCP/RLC/MAC/SMS/TC	7

3.1	Baseline Moving – 24.301 (NAS)	7

3.1.1	Modified Message Definitions	7

3.1.2	Modified Information Elements	8

3.1.3	New Information Elements	8

3.1.4	New Constant	8

3.2	Baseline Moving – 36.323 (PDCP)	8

3.3	Baseline Moving – 36.322 (RLC)	9

3.4	Baseline Moving – 36.321 (MAC)	9

3.5	Baseline Moving – 23.038, 23.040, 24.011 (SMS)	9

3.6	Baseline Moving – 36.509 (TC)	9

4	Other changes	9

4.1	Refactoring of templates	9

4.2	Changes to Capability Checking (Test Case 8.5.4.1)	12

4.2.1	Updated function fl_EUTRA_FullCapabilityCheck to account for Rel-18	12

4.2.2	New function f_EUTRA_MatchEutraCapMsgs_r18 for Rel-18 checking	14

4.2.3	New templates	14

4.2.4	Updated templates	15

4.3	Changes to Capability Checking (Test Cases 22.4.13 & 22.4.13a)	15

4.3.1	Updated function f_TC_22_4_13_NBIOT ()	15

4.3.2	Updated function f_TC_22_4_13a_NBIOT()	16

4.3.3	New function f_NBIOT_MatchCapMsgs_r18 () for Rel-18 checking	17

4.4	Further Changes to Capability Checking	17

4.4.1	f_EUTRA_Capability	17

# Overview

This document lists all the changes needed for baseline moving based on iwd-TTCN3-B2022-09_D24wk24.

Contact:	Olivier Genoud
	olivier.genoud@etsi.org

# Baseline Moving – 36.331 (RRC ASN.1)

ASN.1 modules are upgraded to 36.331-i20.

## EUTRA_RRC_ASN1_Definitions

### Type change of nonCriticalExtension

| ASN.1 type affected | New nonCriticalExtension type |
| --- | --- |
| LoggedMeasurementConfiguration-v1700-IEs | LoggedMeasurementConfiguration-v1800-IEs |
| RRCConnectionReconfigurationComplete-v1710-IEs | RRCConnectionReconfigurationComplete-v1800-IEs |
| RRCConnectionReestablishmentComplete-v1710-IEs | RRCConnectionReestablishmentComplete-v1800-IEs |
| RRCConnectionResumeComplete-v1710-IEs | RRCConnectionResumeComplete-v1800-IEs |
| RRCConnectionSetupComplete-v1710-IEs | RRCConnectionSetupComplete-v1800-IEs |
| SystemInformationBlockType1-v1700-IEs | SystemInformationBlockType1-v1800-IEs |
| UEInformationRequest-v1710-IEs | UEInformationRequest-v1800-IEs |
| UEInformationResponse-v1710-IEs | UEInformationResponse-v1800-IEs |
| UE-EUTRA-Capability-v1730-IEs | UE-EUTRA-Capability-v1770-IEs |

no changes in TTCN

### Name changes

| ASN.1 type affected | Renaming | Impact on TTCN |
| --- | --- | --- |
| MasterInformationBlock | partEARFCN-17 renamed to partEARFCN-r17 | cs_508_MasterInformationBlock_Init f_TC_8_1_2_15_EUTRA |
| RRCConnectionResume-r13-IEs | rrcConnectionResume-v1430-IEs renamed to nonCriticalExtension | cs_508_RRCConnectionResume |
| RRCConnectionResume-v1430-IEs | rrcConnectionResume-v1510-IEs renamed to nonCriticalExtension | No impact |
| SIB-Type-v12j0 | Spare3 renamed to sibType33-v1800 | No impact |
| ServingSatelliteInfo-r17 | nta-CommonParameters-17 renamed to nta-CommonParameters-r17 | Renamed in ASP type NTN_T_TA_Type EUTRA_ASN1_Nta_Common_r17_Type cs_508_SystemInformationBlockType31_NB_Def cs_508_SystemInformationBlockType31_NB_Def |

### Fields added with extension marker (ellipsis)

| ASN.1 type affected | New fields | Impact on TTCN |
| --- | --- | --- |
| RLF-Report-r9 | voiceFallbackHO-r18 | Added as * cr_RLF_Report_1Entry |
| SystemInformationBlockType2 | gnss-PositionFixDurationReporting-r18 freqBandIndicatorAerial-r18 freqInfoAerial-r18 multiBandInfoListAerial-r18 | Added as omit in cs_508_SystemInformationBlockType2_Def |
| SystemInformationBlockType3 | satelliteAssistanceInfoList-r18 freqBandInfoAerial-r18 multiBandInfoListAerial-r18 | Added as omit in cs_508_SystemInformationBlockType3_Def cs_508_SystemInformationBlockType3_Common |
| SystemInformationBlockType5 | interFreqCarrierFreqList-v1800 interFreqCarrierFreqListExt-v1800 | Added as omit in cs_508_SystemInformationBlockType5_Def |
| SystemInformationBlockType21-r14 | sl-A2X-ConfigCommon-r18 | Added as omit in cs_SystemInformationBlockType21_Empty cs_SystemInformationBlockType21_Def |
| SystemInformationBlockType24-r15 | carrierFreqListNR-v1810 | Added as omit in cs_508_SystemInformationBlockType24_Def |
| CarrierFreqNR-r15 | ns-PmaxListNR-v1760 multiBandNsPmaxListNR-v1760 multiBandNsPmaxListNR-SUL-v1760 | Added as omit in cs_508_CarrierFreqNR_Def |
| SystemInformationBlockType31-r17 | servingSatelliteInfo-v1820 | No impact |
| SystemInformationBlockType32-r17 | satelliteInfoList-v1800 | No impact |
| LogicalChannelConfig | allowedHARQ-Mode-r18 | Added as omit in cs_LogicalChannelConfig cs_508_SRB4_ToAddMod_DEFAULT |
| PhysicalConfigDedicated | pdsch-ConfigDedicated-v1800 pusch-ConfigDedicated-v1800 | Added as omit in cs_PhysicalConfigDedicated_Omit |
| RadioResourceConfigDedicated | harq-FeedbackEnablingforSPSactive-r18 gnss-AutonomousEnabled-r18 ul-TransmissionExtensionEnabled-r18 ul-TransmissionExtensionValue-r18 | Added as omit in cs_RadioResourceConfigDedicated_AllOmit |
| MobilityControlInfo | gnss-PositionFixDurationReporting-r18 | Added as omit in cs_MobilityControlInfo |
| MeasConfig | timeMeasConfig-r18 locationMeasConfig-r18 | Added as omit in cs_MeasConfig |
| MeasObjectEUTRA | cellsToAddModList-v1810 | Added as omit in cs_MeasObject_measObjectEUTRA_Common |
| SL-CommResourcePoolV2X-r14 | sl-A2X-Service-r18 | Added as omit in cs_SL_CommResourcePoolV2X |

### Further ASN.1 changes

| ASN.1 type affected | Type change | Impact on TTCN |
| --- | --- | --- |
| SystemInformation-r8-IEs | New choice value | No impact |
| PosSystemInformation-r15-IEs | New choice value | No impact |
| PosSIB-Type-r15 | New enumerated | No impact |
| ReportConfigEUTRA | New choice value | No impact |
| CondReconfigurationTriggerEUTRA-r16 | New choice value |  |
| AccessStratumRelease | Added new choice rel18 | New case added for rel-18 in f_ConvertAccessStratumReleaseToInteger f_EUTRAN_CheckReleaseIndicator |

## NBIOT_RRC_ASN1_Definitions

### Type change of nonCriticalExtension

| ASN.1 type affected | New nonCriticalExtension type |
| --- | --- |
| RRCConnectionReconfiguration-NB-r13-IEs | RRCConnectionReconfiguration-NB-v16f0-IEs |
| RRCConnectionReestablishmentComplete-NB-v1710-IEs | RRCConnectionReestablishmentComplete-NB-v1800-IEs |
| RRCConnectionResume-NB-v1610-IEs | RRCConnectionResume-NB-v16f0-IEs |
| RRCConnectionResumeComplete-NB-v1710-IEs | RRCConnectionResumeComplete-NB-v1800-IEs |
| RRCConnectionSetupComplete-NB-v1710-IEs | RRCConnectionSetupComplete-NB-v1800-IEs |
| UE-Capability-NB-v1720-IEs | UE-Capability-NB-v1800-IEs |

no changes in TTCN

### Name changes

| ASN.1 type affected | Renaming | Impact on TTCN |
| --- | --- | --- |
| MasterInformationBlock-NB | Field partEARFCN-17renamed to partEARFCN-r17 | cs_508_MasterInformationBlock_Init f_TC_22_4_26_NBIOT f_NBIOT_InitSystemInformation |
| SIB-Type-NB-v1530 | spare4 renamed to sibType33-NB-r18 | No impact |
| AccessStratumRelease-NB-r13 | spare3 renamed to rel18 | Added rel-18 branch in f_NBIOT_CheckReleaseIndicator f_NBIOT_ConvertAccessStratumReleaseToInteger |

### Fields added with extension marker (ellipsis)

| ASN.1 type affected | New fields | Impact on TTCN |
| --- | --- | --- |
| SystemInformationBlockType2-NB-r13 | gnss-PositionFixDurationReporting-r18 | Added as omit in cs_508_SystemInformationBlockType2_NB_Def |
| SystemInformationBlockType3-NB-r13 | satelliteAssistanceInfo-r18 | Added as omit in cs_508_SystemInformationBlockType3_NB_Def |
| SystemInformationBlockType5-NB-r13 | interFreqCarrierFreqList-v1820 | Added as omit in cs_508_SystemInformationBlockType5_NB_Def |
| SystemInformationBlockType31-NB-r17 | servingSatelliteInfo-v1820 | Added as omit in cs_508_SystemInformationBlockType31_NB_Def |
| SystemInformationBlockType32-NB-r17 | satelliteInfoList-v1800 | Added as omit in cs_NB_SystemInformationBlockType32 |
| PhysicalConfigDedicated-NB-r13 | npdsch-ConfigDedicated-v1800 npusch-ConfigDedicated-v1800 | Added as omit in cs_PhysicalConfigDedicated_NB_DEFAULT |
| RadioResourceConfigDedicated-NB-r13 | gnss-AutonomousEnabled-r18 ul-TransmissionExtensionEnabled-r18 ul-TransmissionExtensionValue-r18 | Added as omit in cs_RadioResourceConfigDedicated_NB_SRBRECONFIG cs_RadioResourceConfigDedicated_NB_SRB cs_RadioResourceConfigDedicated_NB_DrbConfig cs_RadioResourceConfigDedicated_NB_DrbRelease |

### Further ASN.1 changes

| ASN.1 type affected | Type change | Impact on TTCN |
| --- | --- | --- |
| SystemInformation-NB-r13-IEs | New choice added: sib33-v1800 | No impact |
| UE-Capability-NB-v16x0-IEs | Type of lateNonCriticalExtension changed to OCTET STRING (CONTAINING UE-EUTRA-Capability-v16f0-IEs) | No impact |

## EUTRA_Sidelink_Preconf

No technical change – no impact

## EUTRA_PC5_RRC_Definitions

No technical change – no impact

# Baseline Moving – 24.334 (ProSe)

24.334 is upgraded to 24.334-i00 – no impact

# Baseline Moving – NAS/PDCP/RLC/MAC/SMS/TC

## Baseline Moving – 24.301 (NAS)

24.301 is upgraded to 24.301-i70

### Modified Message Definitions

| ASN.1 Definition | New optional fields | Ref: 3GPP 24.301 clause | TTCN-3 impact |
| --- | --- | --- | --- |
| Attach Accept | Unavailability configuration | 8.2.1 | Added as omitted in - cs_ATTACH_ACCEPT_Common |
| Security Mode Command | UE coarse location information request | 8.2.20 | Added as omitted in - cs_508_SECURITY_MODE_COMMAND - cds_SECURITY_MODE_COMMAND_MappedContext - cds_SECURITY_MODE_COMMAND_IMEISV |
| Security Mode Complete | UE coarse location information | 8.2.21 | Added as * in - cr_508_SECURITY_MODE_COMPLETE |
| Service Accept | Forbidden TAI(s) for the list of "forbidden tracking areas for roaming", Forbidden TAI(s) for the list of "forbidden tracking areas for regional provision of service" | 8.2.34 | Added as omitted in - cs_SERVICE_ACCEPT |
| TAU Accept | Maximum time offset, Unavailability configuration | 8.2.26 | Added as omitted in - cs_TAU_Accept_Common |
| TAU Request | Unavailability information | 8.2.29 | Added as cr_UnavailabilityInformation in - cr_508_TAU_Request: - cr_TAU_RequestIdleMobilityDiffRAT - cr_TAU_Request_S1ToN1 - cr_TAU_Request_N1ToS1 |

### Modified Information Elements

| ASN.1 Definition | New fields | Ref: 3GPP 24.301 clause | TTCN-3 Impact |
| --- | --- | --- | --- |
| EPS Network Feature Support | Optional: EDC (1 bit), Spare field has been changed from 3 bits to 2 bits | 9.9.3.12A | Added as omitted in - cs_EPS_NwkFtSup_CommonWithOctet5 - cs_EPS_NwkFtSup_Common |
| RemoteUEContext | Mandatory: upri4, tpri4 Optional: UDP port range for IPv4, TCP port range for IPv4 | 9.9.4.20 | No Impact |

### New Information Elements

| ASN.1 Definition | Ref: 3GPP 24.301 clause 9.9.3 | TTCN-3 impact |
| --- | --- | --- |
| Time Duration | 9.9.3.68 | No Impact |
| Unavailability information | 9.9.3.69 | New Template: - cr_UnavailabilityInformation |
| Unavailability configuration | 9.9.3.70 | No Impact |
| UE information request | 9.9.3.71 | No Impact |
| UE coarse location information | 9.9.3.72 | No Impact |

### New Constant

const NAS_CauseValue_Type tsc_EmmCause_Iab_NodeOperationNotAuth := '00100100'B

## Baseline Moving – 36.323 (PDCP)

36.323 is upgraded to 36.323-i00.

No changes required in TTCN.

## Baseline Moving – 36.322 (RLC)

36.322 is upgraded to 36.322-i00.

No changes required in TTCN.

## Baseline Moving – 36.321 (MAC)

36.321 is upgraded to 36.321-i20.

No changes required in TTCN.

## Baseline Moving – 23.038, 23.040, 24.011 (SMS)

23.038 is upgraded to 23.038-i00.

23.040 is upgraded to 23.040-i00.

24.011 is upgraded to 24.011-i00

No changes required in TTCN.

## Baseline Moving – 36.509 (TC)

36.509 is upgraded to 36.509-h40.

No changes required in TTCN.

# Other changes

## Refactoring of templates

Module EUTRA_CellCfg_Templates

template (value) CellConfigInfo_Type.Active.CcchDcchDtchConfig cs_CellConfig_CcchDcchDtchConfig (template (value) AntennaInfoDedicated p_AntennaInfoDedicated) :=

{ //@sic R5s240444 sic@

MeasGapCtrl := omit,

DL := {

DciInfo := omit,

AntennaInfo := cs_AntennaInfoDedicated_r8andLater( p_AntennaInfoDedicated ),

HarqProcessConfig := omit

},

UL := omit,

DrxCtrl := omit,

TtiBundling := omit,

CifPresence := omit

};

template (value) SYSTEM_CTRL_REQ cas_CellConfig_CQI_ReportConfig_r10(EUTRA_CellId_Type p_CellId,

template (value) TimingInfo_Type p_TimingInfo,

template (value) CQI_ReportConfig_r10 p_CQI_ReportConfig_r10,

template (omit)  CQI_ReportConfig_v1130 p_CQI_ReportConfig_v1130 := omit,

template (omit)  CQI_ReportConfig_v1250 p_CQI_ReportConfig_v1250 := omit

,

template (omit)  CellConfigInfo_Type.Active.CcchDcchDtchConfig p_CcchDcchDtchConfig := omit) :=

{ //@sic R5s180593 R5s240444 sic@

{ /* @status    APPROVED (LTE_A_PRO, LTE_A_R10_R11, LTE_A_R12) */

Common := cs_ReqAspCommonPart_CellCfg(p_CellId, p_TimingInfo),

Request := {

Cell := {

AddOrReconfigure := {

Basic := omit,

Active := {

C_RNTI := omit,

PhysicalLayerConfigUL := {

Prach := omit,

Pucch := omit,

Pusch := omit,

TimingAdvance := omit,

SRS_UL_Config := omit,

SR_Config := omit,

CQI_ReportConfig := {

R10andLater := {

cqi_ReportConfig_r10   :=  p_CQI_ReportConfig_r10,

cqi_ReportConfig_v1130 :=  p_CQI_ReportConfig_v1130,

cqi_ReportConfig_v1250 :=  p_CQI_ReportConfig_v1250      /* @sic BASELINE MOVING 2015 R5-153398 sic@ */

}

},

UplinkPowerControlCommon := omit,

UplinkPowerControlDedicated := omit

},

RachProcedureConfig := omit,

CcchDcchDtchConfig := p_CcchDcchDtchConfig,

ServingCellConfig := omit,

SciSchedulingConfig := omit, //@sic R5w160202 sic@

EIMTA_Config := omit, //@sic R5w160203r1 sic@

CE_Mode := omit //@sic R5w160205 sic@

}

}

}

}

};

template (value) SYSTEM_CTRL_REQ cas_CellConfig_CQI_ReportConfig_r10(EUTRA_CellId_Type p_CellId,

template (value) TimingInfo_Type p_TimingInfo,

template (value) CQI_ReportConfig_r10 p_CQI_ReportConfig_r10,

template (omit)  CQI_ReportConfig_v1130 p_CQI_ReportConfig_v1130 := omit,

template (omit)  CQI_ReportConfig_v1250 p_CQI_ReportConfig_v1250 := omit) :=

/* @status    APPROVED (LTE_A_PRO, LTE_A_R10_R11, LTE_A_R12) */

cas_CellConfig_CQI_ReportConfig(p_CellId, p_TimingInfo, p_CQI_ReportConfig_r10, p_CQI_ReportConfig_v1130, p_CQI_ReportConfig_v1250); //@sic R5s240444 sic@

template (value) SYSTEM_CTRL_REQ cads_CellConfig_CQI_ReportConfig_AntennaInfo(EUTRA_CellId_Type p_CellId,

template (value) TimingInfo_Type p_TimingInfo,

template (value) CQI_ReportConfig_r10 p_CQI_ReportConfig_r10,

template (omit)  CQI_ReportConfig_v1130 p_CQI_ReportConfig_v1130 := omit,

template (omit)  CQI_ReportConfig_v1250 p_CQI_ReportConfig_v1250 := omit,

template (value) AntennaInfoDedicated p_AntennaInfoDedicated)

:=

cas_CellConfig_CQI_ReportConfig(p_CellId, p_TimingInfo, p_CQI_ReportConfig_r10, p_CQI_ReportConfig_v1130, p_CQI_ReportConfig_v1250, cs_CellConfig_CcchDcchDtchConfig( p_AntennaInfoDedicated )); //@sic R5s240444 sic@

}

}

}

}

};

Module EUTRA_MAC_Common

template (value) DciUlInfo_Type cs_DciInfo_Imcs_FreqDomain_REQ(ImcsValue_Type p_Imcs,

integer p_FirstRbIndex,

integer p_Nprb):=

{ /* @status    APPROVED (LTE) */

//@sic R5s240444 sic@

Imcs := {Value := p_Imcs},

TransRetransmissionList := cs_TransRetransmissionListUL_Def, //@sic R5w100214 slide 3/4 sic@

FreqDomainSchedul := cs_FreqDomainSchedulExplicit(p_FirstRbIndex, p_Nprb),

Format := omit,                    // @sic R5w160205 sic@

HarqProcessId := omit,             // @sic R5w160205 sic@

RepetitionNumber := omit,          // @sic R5w160205 sic@

DCISubframeRepetitionNumber:= omit, // @sic R5w160205 sic@

PUSCH_TriggerA := omit,

TimingOffset := omit,

NumberOfScheduledSubframes := omit

};

template (value) SYSTEM_CTRL_REQ cas_CellConfig_DciInfo_REQ(EUTRA_CellId_Type p_CellId,

template (value) TimingInfo_Type p_TimingInfo,

ImcsValue_Type p_Imcs,

integer p_FirstRbIndex,

integer p_Nprb) :=

{ /* @status    APPROVED (LTE) */

Common := cs_ReqAspCommonPart_CellCfg(p_CellId, p_TimingInfo),

Request := {

Cell := {

AddOrReconfigure := {

Basic := omit,

Active := {

C_RNTI := omit,

PhysicalLayerConfigUL := omit,

RachProcedureConfig := omit,

CcchDcchDtchConfig := {

MeasGapCtrl := omit,

DL := omit,

UL := {

DciInfo := cs_DciInfo_Imcs_FreqDomain_REQ(p_Imcs,p_FirstRbIndex,p_Nprb), //@sic R5s240444 sic@

Hopping := omit,

PUCCH_Synch := omit,

UL_GrantConfig :=  cs_UL_GrantConfig_OneTime //@sic R5-115620 sic@

},

DrxCtrl := omit,

TtiBundling := omit,

CifPresence := omit

},

ServingCellConfig := omit,

SciSchedulingConfig := omit, //@sic R5w160202 sic@

EIMTA_Config := omit, //@sic R5w160203r1 sic@

CE_Mode := omit //@sic R5w160205 sic@

}

}

}

}

};

Modules MAC_717 and MAC_A_PRO

Calls to cas_CellConfig_7172_REQ replaced with cas_CellConfig_DciInfo_REQ

Modules RRC_ConnEst and RRC_ConnRel_MPS

Refactoring of templates of type AccessBarringInformation_Type in modules RRC_ConnEst and RRC_ConnRel_MPS using a variable of type cs_AccessBarringInformation_Def instead,

cs_AccessBarringInformation_TC_8_1_2_6

cs_AccessBarringInformation_TC_8_1_2_7

cs_AccessBarringInformation_TC_8_1_2_9_Cell11

cs_AccessBarringInformation_TC_8_1_2_13_Cell1

cs_AccessBarringInformation_TC_8_1_2_13_Cell11

cs_AccessBarringInformation_TC_8_1_2_8

cs_AccessBarringInformation_TC_8_1_3_17

Example of function f_TC_8_1_3_17_EUTRA in module RRC_ConnRel_MPS:

var template (value) AccessBarringInformation_Type v_AccessBarringInformation :=

cs_AccessBarringInformation_Def(true,

cs_AC_BarringConfig(p00, s4, '10001'B),

cs_AC_BarringConfig(p00, s4, '10001'B)); // @sic R5s240444 sic@

f_EUTRA_CellInfo_SetSysInfo_AccessBarring (eutra_Cell3, v_AccessBarringInformation); // @sic R5s240444 sic@

## Changes to Capability Checking (Test Case 8.5.4.1)

Capability checking is updated to cope with Rel-18 UEs.

### Updated function fl_EUTRA_FullCapabilityCheck to account for Rel-18

…

function fl_EUTRA_FullCapabilityCheck(EUTRA_CellId_Type p_CellId,

RRC_TransactionIdentifier p_RRC_TI) runs on EUTRA_PTC

{

var UE_EUTRA_Capability v_ReceivedCapabilityMsg;

var template UE_EUTRA_Capability v_EutraCapability;

var template UE_EUTRA_Capability_v9a0_IEs v_Expected_UE_EUTRA_Capability_v9a0_IEs := omit;

// Transmit UE Capability Enquiry message to ask for EUTRA Capability information

SRB.send(cas_SRB1_RrcPdu_REQ(p_CellId, cs_TimingInfo_Now, cs_508_UeCapabilityEnquiry(p_RRC_TI,-,-,-,-,enabled))); // @sic R5-217903 sic@

// Receive UE Capability Information message for EUTRA capabilities and decode

// Either a container is received or segments from which the container is to be established (only Rel-16 onwards)

v_ReceivedCapabilityMsg := f_EUTRA_GetCaps(p_CellId, p_RRC_TI);  // @sic R5-217903 sic@

select (v_ReceivedCapabilityMsg.accessStratumRelease) { // @sic R5s120250 sic@

case (rel8) {

v_EutraCapability := f_EUTRA_SetExpectedUeEutraCap(cr_UeCapabilitiesRAT_Container_r8);

// Check Rel 8 vs. IEs provided

if (isvalue(v_ReceivedCapabilityMsg.nonCriticalExtension)) { // @sic R5-123750 sic@

// if mixed releases then Rel 9 IEs may be povided

v_EutraCapability.nonCriticalExtension := ?;

}

else {

v_EutraCapability.nonCriticalExtension := omit;

}

// Match the ueCapabilitiesRAT-Container

f_EUTRA_MatchEutraCapMsgs_r8(v_ReceivedCapabilityMsg);  // @sic R5-130706, R5s170597 sic@

}

case (rel9) {

v_EutraCapability := f_EUTRA_SetExpectedUeEutraCap(cr_UeCapabilitiesRAT_Container_r9); // @sic R5s230759 sic@

// Match the ueCapabilitiesRAT-Container

f_EUTRA_MatchEutraCapMsgs_r9(v_ReceivedCapabilityMsg); // @sic R5-130706, R5s170597 sic@

}

case (rel10) {

v_EutraCapability := f_EUTRA_SetExpectedUeEutraCap(cr_UeCapabilitiesRAT_Container_r10); // @sic R5s230759 sic@

if (not isvalue (v_ReceivedCapabilityMsg.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension)) { // @sic R5s130088 sic@

// no Rel 10 IEs provided although Rel 10 FGIs must be provided (36.331 Annex C.1)

f_SetVerdict(fail, __FILE__, __LINE__, "UE-EUTRA-Capability-v1020-IEs must be provided by Rel 10 UE");  // @sic R5-133612 sic@

}

if (not isvalue(v_ReceivedCapabilityMsg.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.featureGroupIndRel10_r10)) { // @sic R5s130269 sic@ @sic R5-133612 sic@

f_SetVerdict(fail, __FILE__, __LINE__, "UE-EUTRA-Capability-v1020-IEs FGIs must be provided by Rel 10 UE");

}

// Match the ueCapabilitiesRAT-Container

f_EUTRA_MatchEutraCapMsgs_r10(v_ReceivedCapabilityMsg);  // @sic R5-130706, R5s170597 sic@

}

case (rel11) { // @sic R5s130195 sic@

v_EutraCapability := f_EUTRA_SetExpectedUeEutraCap(cr_UeCapabilitiesRAT_Container_r11); // @sic R5s160666, R5-166324 sic@

v_Expected_UE_EUTRA_Capability_v9a0_IEs := cr_UE_EUTRA_Capability_v9a0_IEs_r11;

// Match the ueCapabilitiesRAT-Container

f_EUTRA_MatchEutraCapMsgs_r11(v_ReceivedCapabilityMsg);  // @sic R5-130706, R5s170597 sic@

}

case (rel12) { // @sic R5s150329 - Rel-12 Baseline move sic@

v_EutraCapability := f_EUTRA_SetExpectedUeEutraCap(cr_UeCapabilitiesRAT_Container_r12); // @sic R5s160666, R5-166324 sic@

v_Expected_UE_EUTRA_Capability_v9a0_IEs := cr_UE_EUTRA_Capability_v9a0_IEs_r11;

// Match the ueCapabilitiesRAT-Container

f_EUTRA_MatchEutraCapMsgs_r12(v_ReceivedCapabilityMsg); // @sic R5s170597 sic@

}

case (rel13) { // @sic R5s160711 BASELINE MOVING 2016 sic@

v_EutraCapability := f_EUTRA_SetExpectedUeEutraCap(cr_UeCapabilitiesRAT_Container_r13); // @sic R5s160666, R5-166324 sic@

v_Expected_UE_EUTRA_Capability_v9a0_IEs := cr_UE_EUTRA_Capability_v9a0_IEs_r11;

// Match the ueCapabilitiesRAT-Container

f_EUTRA_MatchEutraCapMsgs_r13(v_ReceivedCapabilityMsg); // @sic R5s170597 sic@

}

case (rel14) { // @sic R5s170597 BASELINE MOVING 2017 sic@

v_EutraCapability := f_EUTRA_SetExpectedUeEutraCap(cr_UeCapabilitiesRAT_Container_r14);

v_Expected_UE_EUTRA_Capability_v9a0_IEs := cr_UE_EUTRA_Capability_v9a0_IEs_r11;

// Match the ueCapabilitiesRAT-Container

f_EUTRA_MatchEutraCapMsgs_r14(v_ReceivedCapabilityMsg); // @sic R5s170597 sic@

}

case (rel15) { // @sic R5s180552 BASELINE MOVING 2018 sic@

v_EutraCapability := f_EUTRA_SetExpectedUeEutraCap(cr_UeCapabilitiesRAT_Container_r15);

v_Expected_UE_EUTRA_Capability_v9a0_IEs := cr_UE_EUTRA_Capability_v9a0_IEs_r11;

// Match the ueCapabilitiesRAT-Container

f_EUTRA_MatchEutraCapMsgs_r15(v_ReceivedCapabilityMsg); // @sic R5s180552 BASELINE MOVING 2018 sic@

}

case (rel16) { // @sic R5s201386 BASELINE MOVING 2020 sic@

v_EutraCapability := f_EUTRA_SetExpectedUeEutraCap(cr_UeCapabilitiesRAT_Container_r16);

v_Expected_UE_EUTRA_Capability_v9a0_IEs := cr_UE_EUTRA_Capability_v9a0_IEs_r11;

// Match the ueCapabilitiesRAT-Container

f_EUTRA_MatchEutraCapMsgs_r16(v_ReceivedCapabilityMsg); // @sic R5s201386 BASELINE MOVING 2020 sic@

}

case (rel17) { // @sic R5s221178 BASELINE MOVING 2022 sic@

v_EutraCapability := f_EUTRA_SetExpectedUeEutraCap(cr_UeCapabilitiesRAT_Container_r17);

v_Expected_UE_EUTRA_Capability_v9a0_IEs := cr_UE_EUTRA_Capability_v9a0_IEs_r11;

// Match the ueCapabilitiesRAT-Container

f_EUTRA_MatchEutraCapMsgs_r17(v_ReceivedCapabilityMsg); // @sic R5s221178 BASELINE MOVING 2022 sic@

}

case (rel18) { // @sic R5s240444 BASELINE MOVING 2024 sic@

v_EutraCapability := f_EUTRA_SetExpectedUeEutraCap(cr_UeCapabilitiesRAT_Container_r18);

v_Expected_UE_EUTRA_Capability_v9a0_IEs := cr_UE_EUTRA_Capability_v9a0_IEs_r11;

// Match the ueCapabilitiesRAT-Container

f_EUTRA_MatchEutraCapMsgs_r18(v_ReceivedCapabilityMsg); // @sic R5s240444 BASELINE MOVING 2024 rel-18 sic@

}

case else  {

f_SetVerdictInconc (__FILE__, __LINE__, "AccessStratumRelease not testable"); // a UE implementing an unsupported release must not be failed !

}

}

// Match the received UECapabilityInformation with the expected UE_EUTRA_Capability template and, if applicable, the UE_EUTRA_Capability_v9a0_IEs template.

f_EUTRA_UE_Capability_MatchTemplates(v_ReceivedCapabilityMsg, v_EutraCapability, v_Expected_UE_EUTRA_Capability_v9a0_IEs);

} // fl_EUTRA_FullCapabilityCheck

### New function f_EUTRA_MatchEutraCapMsgs_r18 for Rel-18 checking

function f_EUTRA_MatchEutraCapMsgs_r18(UE_EUTRA_Capability p_ReceivedCapabilityMsg)

{ /* @sic R5s240444 BASELINE MOVING 2024 rel-18 sic@ */

// to check some Rel 8 + 9 + 10 + 11 + 12 + 13 + 14 + 15 + 16 + 17 UE capabilities

fl_EUTRA_CommonChecks_r17(p_ReceivedCapabilityMsg);

// further rel 18 specific checks may be needed (e.g. FGIs per TS 36.331 or UE categories per TS 36.306)

}

### New templates

- cr_UE_EUTRA_Capability_v1730_IEs

- cr_UE_EUTRA_Capability_Baseline_2024

- cr_UeCapabilitiesRAT_Container_r18

template UE_EUTRA_Capability_v1730_IEs cr_UE_EUTRA_Capability_v1730_IEs :=

{ /* @sic R5s240444 BASELINE MOVING 2024 rel-18 sic@ */

/* @status     */

phyLayerParameters_v1730 := ?,

nonCriticalExtension := {

measParameters_v1770 := ?,

nonCriticalExtension := {

measParameters_v1800 := *,

rf_Parameters_v1800 := *,

ntn_Parameters_v1800 := *,

sl_Parameters_v1800 := *,

son_Parameters_v1800 := ?,

ue_BasedNetwPerfMeasParameters_v1800 := ?,

nonCriticalExtension := *

} ifpresent // UE-EUTRA-Capability-v1800-IEs

} ifpresent // UE-EUTRA-Capability-v1770-IEs

} ifpresent; // UE-EUTRA-Capability-v1730-IEs

template UE_EUTRA_Capability_v920_IEs cr_UE_EUTRA_Capability_Baseline_2024

/* @status     */

:= cr_UE_EUTRA_Capability_Baseline_2022 (cr_UE_EUTRA_Capability_v1730_IEs); /* @sic R5s240444 BASELINE MOVING 2024 rel-18 sic@ */

template (present) UE_EUTRA_Capability cr_UeCapabilitiesRAT_Container_r18 /* @sic R5s240444 BASELINE MOVING 2024 rel-18 sic@ */

/* @status     */

:= cr_UE_EUTRA_Capability(?, -, cr_UE_EUTRA_Capability_Baseline_2024);

### Updated templates

- cr_UE_EUTRA_Capability_v1630_IEs

template UE_EUTRA_Capability_v1630_IEs cr_UE_EUTRA_Capability_v1630_IEs (template UE_EUTRA_Capability_v1730_IEs p_UE_EUTRA_Capability_v1730_IEs := *):=

{ /* @sic R5s221178 BASELINE MOVING 2022 sic@ */

/* @status    APPROVED (ENDC, LTE) */

rf_Parameters_v1630 := *,

sl_Parameters_v1630 := *,

earlySecurityReactivation_r16 := *,

mac_Parameters_v1630 := ?,

measParameters_v1630 := *,

fdd_Add_UE_EUTRA_Capabilities_v1630 := ?,

tdd_Add_UE_EUTRA_Capabilities_v1630 := ?,

nonCriticalExtension := {

otherParameters_v1650 := *,

nonCriticalExtension := {

irat_ParametersNR_v1660 := ?,

nonCriticalExtension := {

other_Parameters_v1690 := ?,

nonCriticalExtension := {

measParameters_v1700 := *,

ue_BasedNetwPerfMeasParameters_v1700 := *,

phyLayerParameters_v1700 := ?,

ntn_Parameters_r17 := *,

irat_ParametersNR_v1700 := *,

mbms_Parameters_v1700 := ?,

nonCriticalExtension := {

irat_ParametersNR_v1710 := ?,

neighCellSI_AcquisitionParameters_v1710 := *,

sl_Parameters_v1710 := *,

sidelinkRequested_r17 := *,

nonCriticalExtension := {

ntn_Parameters_v1720 := ?,

nonCriticalExtension := p_UE_EUTRA_Capability_v1730_IEs // @sic R5s240444 BASELINE MOVING 2024 rel-18 sic@

} ifpresent // UE-EUTRA-Capability-v1720-IEs

} ifpresent // UE-EUTRA-Capability-v1710-IEs

} ifpresent // UE-EUTRA-Capability-v1700-IEs

} ifpresent // UE-EUTRA-Capability-v1690-IEs

} ifpresent // UE-EUTRA-Capability-v1660-IEs

} ifpresent // UE-EUTRA-Capability-v1650-IEs

} ifpresent; // UE-EUTRA-Capability-v1630-IEs

## Changes to Capability Checking (Test Cases 22.4.13 & 22.4.13a)

Capability checking is updated to cope with Rel-18 UEs.

### Updated function f_TC_22_4_13_NBIOT ()

function f_TC_22_4_13_NBIOT() runs on NBIOT_PTC

{

…

select (v_UECapability.accessStratumRelease_r13) { // @sic R5-181277 sic@

case (rel13) {

//do nothing

}

case (rel14) {

f_NBIOT_MatchCapMsgs_r14(v_UECapabilityInformation_r13); // @sic R5-217957 sic@

}

case (rel15, rel16, rel17, rel18) { // @sic R5s180552 BASELINE MOVING 2018, R5s201386 BASELINE MOVING 2020, R5s221178 BASELINE MOVING 2022, R5s240444 BASELINE MOVING 2024 sic@

f_NBIOT_MatchCapMsgs_r15(v_UECapabilityInformation_r13); // @sic R5-217957 sic@

}

case else  {

f_SetVerdictInconc (__FILE__, __LINE__, "AccessStratumRelease not testable"); // a UE implementing an unsupported release must not be failed !

}

}

f_NBIOT_TestBody_Set(false);

//Switch/power off UE

f_NBIOT_Postamble ( nbiot_Cell1, CONTROL_PLANE, N2_CONNECTED );

}

### Updated function f_TC_22_4_13a_NBIOT()

function f_TC_22_4_13a_NBIOT() runs on NBIOT_PTC

{//22.4.13a   NB-IoT / NTN / UE capability transfer / Success

var NB_SRB_COMMON_IND v_ReceivedAspForUeCapabilityInfo;

var UECapabilityInformation_NB_r13_IEs v_UECapabilityInformation_r13;

var UE_Capability_NB_r13 v_UECapability;

//System information combination 8 as defined in TS 36.508 [18] clause 8.1.4.3.1.1 is used

f_NBIOT_Init(c8);

//Create and configure all cells

f_NBIOT_CellConfig_Def(nbiot_Cell1, CONTROL_PLANE);

//Preconfigure UE supporting TN and NTN access in NB-IoT with UE position

if(pc_NB_ntn_Connectivity_EPC and not pc_NB_ntn_only_Connectivity_EPC)  //@sic R5s240092,R5w240103 sic@

{

f_NBIOT_PreconfigureNTN_UE();

}

//Bring UE to initial state

f_NBIOT_Preamble(nbiot_Cell1, CONTROL_PLANE);

f_NBIOT_TestBody_Set(true);

//@siclog "Step 1" siclog@

//The SS transmits a UECapabilityEnquiry-NB message to request UE radio access capability information.

SRB.send(cas_NB_SRB_RrcPdu_REQ(nbiot_Cell1, tsc_SRB1bis, cs_TimingInfo_Now, cs_508_UeCapabilityEnquiry_NB(tsc_RRC_TI_Def)));

//@siclog "Step 2" siclog@

//Check: Does the UE transmit a UECapabilityInformation-NB message?

SRB.receive(car_NB_SRB_RrcPdu_IND(nbiot_Cell1, tsc_SRB1bis, cr_508_UeCapabilityInformation_NB(tsc_RRC_TI_Def,

cr_UE_Capability_NB,

cr_UE_RadioPagingInfo_NB(nb1))))

-> value v_ReceivedAspForUeCapabilityInfo;

v_UECapabilityInformation_r13 := v_ReceivedAspForUeCapabilityInfo.Signalling.Rrc.Dcch.message_.c1.ueCapabilityInformation_r13.criticalExtensions.ueCapabilityInformation_r13;

v_UECapability := v_UECapabilityInformation_r13.ue_Capability_r13;

f_NBIOT_CheckReleaseIndicator (v_UECapabilityInformation_r13.ue_Capability_r13.accessStratumRelease_r13);

fl_NBIOT_CheckMultipleDRB(v_UECapability);

fl_NBIOT_CheckPhyLayerParameters(v_UECapability.phyLayerParameters_r13);

fl_NBIOT_CheckSupportedBands(v_UECapability.rf_Parameters_r13.supportedBandList_r13, tsc_NBIOT_SupportedNtnBandList); //@sic R5s240092 sic@

if (v_UECapability.ue_Category_NB_r13 != nb1) {

f_NBIOT_SetVerdictFailOrInconc (__FILE__, __LINE__, "UE Category does not equal nb1");

} else {

f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 2");

}

select (v_UECapability.accessStratumRelease_r13) {

case (rel17){

f_NBIOT_MatchCapMsgs_r17(v_UECapabilityInformation_r13);

}

case (rel18) {//@sic R5s240444 BASELINE MOVING 2024 sic@

f_NBIOT_MatchCapMsgs_r18(v_UECapabilityInformation_r13);

}

case else  {

f_SetVerdictInconc (__FILE__, __LINE__, "AccessStratumRelease not testable"); // a UE implementing an unsupported release must not be failed !

}

}

f_NBIOT_TestBody_Set(false);

//Switch/power off UE

f_NBIOT_Postamble ( nbiot_Cell1, CONTROL_PLANE, N2_CONNECTED );

}

### New function f_NBIOT_MatchCapMsgs_r18 () for Rel-18 checking

/*

* @desc      To check r18 capabilities

* @param     p_UECapabilityInformation_r13

* @status

*/

function f_NBIOT_MatchCapMsgs_r18(UECapabilityInformation_NB_r13_IEs p_UECapabilityInformation_r13) runs on NBIOT_PTC

{//@sic R5s240444 BASELINE MOVING 2024 sic@

f_NBIOT_MatchCapMsgs_r17(p_UECapabilityInformation_r13);

// further rel 18 specific checks may be needed

}

## Further Changes to Capability Checking

### f_EUTRA_Capability

function f_EUTRA_Capability(EUTRA_CellId_Type p_CellId,

RRC_TransactionIdentifier p_RRC_TI) runs on EUTRA_PTC

{

var SRB_COMMON_IND v_ReceivedAspForUeCapabilityInfo;

var UE_EUTRA_Capability v_EutraCapability;

SRB.send(cas_SRB1_RrcPdu_REQ(p_CellId, cs_TimingInfo_Now, cs_508_UeCapabilityEnquiry(p_RRC_TI)));

SRB.receive(car_SRB1_RrcPdu_IND(p_CellId, cr_UeCapabilityInformation_Any(p_RRC_TI)))

-> value v_ReceivedAspForUeCapabilityInfo;

// Decode and store the ueCapabilitiesRAT-Container

v_EutraCapability := f_EUTRA_DecodeEutraCapMsg(v_ReceivedAspForUeCapabilityInfo.Signalling.Rrc.Dcch.message_.c1.ueCapabilityInformation.criticalExtensions.c1.ueCapabilityInformation_r8.ue_CapabilityRAT_ContainerList[0].ueCapabilityRAT_Container);

f_EUTRA_MobileInfo_SetUECapability (v_EutraCapability);

// Check the UE release

// @sic R5-123750 sic@

f_EUTRAN_CheckReleaseIndicator (v_EutraCapability.accessStratumRelease);

select (v_EutraCapability.accessStratumRelease) { // @sic R5-121847 sic@ @sic R5-123750 sic@

case (rel8) {

if (f_Bitstring_BitIsSet(v_EutraCapability.featureGroupIndicators, 30)) {

f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "FGI bit 31 not set to 0");

}

if (f_Bitstring_BitIsSet(v_EutraCapability.featureGroupIndicators, 31)) {

f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "FGI bit 32 not set to 0");

}

}

case (rel9) { // @sic R5-123750 sic@

/* @sic R5s120653 sic@

if (not f_Bitstring_BitIsSet(v_EutraCapability.featureGroupIndicators, 4)) {

f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "FGI bit 5 not set to 1");

}

if (not f_Bitstring_BitIsSet(v_EutraCapability.featureGroupIndicators, 5)) {

f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "FGI bit 6 not set to 1");

}

if (not f_Bitstring_BitIsSet(v_EutraCapability.featureGroupIndicators, 15)) {

f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "FGI bit 16 not set to 1");

}

if (not f_Bitstring_BitIsSet(v_EutraCapability.featureGroupIndicators, 16)) {

f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "FGI bit 17 not set to 1");

}

if (not f_Bitstring_BitIsSet(v_EutraCapability.featureGroupIndicators, 17)) {

f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "FGI bit 18 not set to 1");

}

if (not f_Bitstring_BitIsSet(v_EutraCapability.featureGroupIndicators, 19)) {

f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "FGI bit 20 not set to 1");

}

if (f_Bitstring_BitIsSet(v_EutraCapability.featureGroupIndicators, 30)) {

f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "FGI bit 31 not set to 0");

}

if (f_Bitstring_BitIsSet(v_EutraCapability.featureGroupIndicators, 31)) {

f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "FGI bit 32 not set to 0");

}

*/

}

case (rel10) { // @sic R5s130064 sic@

}

case (rel11) { // @sic R5s130195 sic@

}

case (rel12) {

}

case (rel13) { // @sic R5s160975 sic@

}

case (rel14) { // @sic R5s170597 BASELINE MOVING 2017 sic@

}

case (rel15) {   /* @sic R5s180271 BASELINE MOVING 2018 Phase 1 sic@ */

}

case (rel16) {   /* @sic R5s201386 BASELINE MOVING 2020 sic@ */

}

case (rel17) {   /* @sic R5s221178 BASELINE MOVING 2022 sic@ */

}

case (rel18) {   /* @sic R5s240444 BASELINE MOVING 2024 rel-18 sic@ */

}

case else{

FatalError (__FILE__, __LINE__, "accessStratumRelease not yet considered");

}

}

}