3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240450

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3476 | rev | - | Current version: | 17.11.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR MAC BWP test case 7.1.1.8.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-07-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - At step 9. f_NR_SS_ConfigureBWPs_DL() is called with parameter value for p_BwpId_Data set as v_MaxBWP. This will lead to BWP Indicator value being set as 1 in case of TDD and v_BWP_SameNumerology_upto4 set as false. This will lead to a BWP Switch. As per test spec there is no BWP Switch at step 9. - At step 10b, f_NR_SS_ConfigureBWPs() is called with parameter value for p_NoPfBWPs set as value of v_MaxBWP. In the current implementation variable v_MaxBWP is set as 3 when device supports upto 4 BWPs and v_MaxBWP shall be set as 1 when device supports upto 2 BWPs. Hence it is required to increment the parameter value v_MaxBWP by one when passed as parameter to f_NR_SS_ConfigureBWPs() |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Modified Step 9 to update the parameter value for p_BwpId_Data in function call f_NR_SS_ConfigureBWPs_DL() with v_BwpId_UL_DCI_IndicatorData instead of v_MaxBWP. - Modified Step 10b to update the parameter value to f_NR_SS_ConfigureBWPs_DL() to increment the value for p_NoPfBWPs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.8.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D24wk24 related to the title of this CR.

Contact:	Mohit Kanchan
	mohit.kanchan@keysight.com

# Corrections required

## Change 1

| Function name | f_TC_7_1_1_8_1_NR_CommonTestBody |
| --- | --- |
| Reason for change | - At step 9. f_NR_SS_ConfigureBWPs_DL() is called with parameter value for p_BwpId_Data set as v_MaxBWP. This will lead to BWP Indicator value being set as 1 in case of TDD and v_BWP_SameNumerology_upto4 set as false. This will lead to a BWP Switch. As per test spec there is no BWP Switch at step 9. - At step 10b, f_NR_SS_ConfigureBWPs() is called with parameter value for p_NoPfBWPs set as value of v_MaxBWP. In the current implementation variable v_MaxBWP is set as 3 when device supports upto 4 BWPs and v_MaxBWP shall be set as 1 when device supports upto 2 BWPs. Hence it is required to increment the parameter value v_MaxBWP by one when passed as parameter to f_NR_SS_ConfigureBWPs() |
| Summary of change | - Modified Step 9 to update the parameter value for p_BwpId_Data in function call f_NR_SS_ConfigureBWPs_DL() with v_BwpId_UL_DCI_IndicatorData instead of v_MaxBWP. - Modified Step 10b to update the parameter value to f_NR_SS_ConfigureBWPs_DL() to increment the value for p_NoPfBWPs. |
| TTCN module | MAC_BWP_NR5GC |
| MCC160 Comment |  |

Before Change:

function f_TC_7_1_1_8_1_NR_CommonTestBody(NR_CellId_Type p_NR_CellId, DRB_Identity p_DrbId, template (value) NR_ResourceAllocation_Type p_ResourceAllocationUL) runs on NR_BASE_PTC { //@sic R5-213459 R5-227417 R5-244020 sic@ var NR_PhysicalParameters_Type v_NR_PhysicalParameters; var template (value) NR_RachProcedureConfig_Type v_NR_RachProcedure; var template (value) NR_PDCCH_Order_Type v_PDCCH_Order; var BWP_Id v_BWP_Id_PDCCH_Order; var SubFrameTiming_Type v_TimingInfoDL; var SubFrameTiming_Type v_TimingInfoSS; //@sic R5s240283 sic@ var SubFrameTiming_Type v_TimingInfoUL; var SubFrameTiming_Type v_TimingInfoConfig; var octetstring v_RLC_SDU1; var octetstring v_RLC_SDU2; var integer v_RA_PreambleIndex; var integer v_SN := 0; var boolean v_BWP_SameNumerology_upto4 := fl_NR_ReturnBWP_SameNumerologyUpto4_Capability(p_NR_CellId); //@sic R5-231578 sic@ var SubFrameTiming_Type v_TimingInfoUL_SS; var BWP_Id v_BwpId_DL_DCI; //BWP on which the DL DCI is transmitted var BWP_Id v_BwpId_DL_DCI_IndicatorData; // Indicator inside DL BWP = BWP on which DL data is tranmsitted var BWP_Id v_BwpId_UL_DCI; //BWP on which the UL DCI is transmitted var BWP_Id v_BwpId_UL_DCI_IndicatorData; // Indicator inside UL BWP = BWP on which UL data is tranmsitted var boolean v_IsFDD := f_NR_CellInfo_GetIsFDD(p_NR_CellId); var integer v_MaxBWP := 3; if (not v_BWP_SameNumerology_upto4) { v_MaxBWP := 1; } // @siclog "Step 1" siclog@ v_TimingInfoDL := f_TC_7_1_1_8_1_NR_GetNextSendOccasion_DL(p_NR_CellId); v_RLC_SDU1 := fl_TC_7_1_1_8_1_SendData(p_NR_CellId, p_DrbId, cs_TimingInfo_NR(v_TimingInfoDL), v_SN); // @siclog "Step 2" siclog@ v_TimingInfoUL := f_NR_GetNextSendOccasion_ULafterDL(p_NR_CellId, v_TimingInfoDL, 100); // 100ms after step 1 f_NR_OneULGrantTransmission(p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoUL), p_ResourceAllocationUL); // @siclog "Steps 3-3A" siclog@ fl_TC_7_1_1_8_1_ReceiveData(p_NR_CellId, p_DrbId, {v_RLC_SDU1}, true, v_SN); //@sic R5-231578 R5s230220 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 3"); v_SN := v_SN + 1; //@sic R5s240283 sic@ if (v_BWP_SameNumerology_upto4) { if (not(v_IsFDD)) { //step 5a - 6a - 7a //IF (pc_bwp_sameNumerology_upto4_FR1_TDD and band under test is FR1 TDD) or (pc_bwp_sameNumerology_upto4_FR2 and band under test is FR2) //THEN The SS indicates on PDCCH (transmitted in Downlink BWP#1) DL DCI format 1_1 with new BWP Id BWP#2 and transmits a MAC PDU containing RLC PDU on the newly configured BWP (BWP#2) v_BwpId_DL_DCI := tsc_NR_BWP_Id_1; v_BwpId_DL_DCI_IndicatorData := tsc_NR_BWP_Id_2; v_BwpId_UL_DCI := tsc_NR_BWP_Id_2; v_BwpId_UL_DCI_IndicatorData := tsc_NR_BWP_Id_2; } else { //Step 5b - 6b - 7b //IF pc_bwp_sameNumerology_upto4_FR1_FDD and band under test is FR1 FDD //THEN The SS indicates on PDCCH (transmitted in Downlink BWP#1) DL DCI format 1_1 with new BWP Id BWP #2 and transmits a MAC PDU containing RLC PDU on the newly configured BWP (BWP#2) v_BwpId_DL_DCI := tsc_NR_BWP_Id_1; v_BwpId_DL_DCI_IndicatorData := tsc_NR_BWP_Id_2; v_BwpId_UL_DCI := tsc_NR_BWP_Id_2; v_BwpId_UL_DCI_IndicatorData := tsc_NR_BWP_Id_1; } } else { if (not(v_IsFDD)) { //Step 5c - 6c - 7c //IF (pc_bwp_sameNumerology_upto2_FR1_TDD and band under test is FR1 TDD ) or (pc_bwp_sameNumerology_upto2_FR2 and band under test is FR2) //THEN The SS indicates on PDCCH (transmitted in Downlink BWP#1) DL DCI format 1_1 with new BWP Id BWP#0 and transmits a MAC PDU containing RLC PDU on the newly configured BWP (BWP#0) v_BwpId_DL_DCI := tsc_NR_BWP_Id_1; v_BwpId_DL_DCI_IndicatorData := tsc_NR_BWP_Id_Initial; v_BwpId_UL_DCI := tsc_NR_BWP_Id_Initial; v_BwpId_UL_DCI_IndicatorData := tsc_NR_BWP_Id_Initial; } else { //Step 5d - 6d - 7d //IF pc_bwp_sameNumerology_upto2_FR1_FDD and band under test is FR1 FDD //THEN The SS indicates on PDCCH (transmitted in Downlink BWP#1) DL DCI format 1_1 with new BWP Id BWP #0 and transmits a MAC PDU containing RLC PDU on the newly configured BWP (BWP#0) v_BwpId_DL_DCI := tsc_NR_BWP_Id_1; v_BwpId_DL_DCI_IndicatorData := tsc_NR_BWP_Id_Initial; v_BwpId_UL_DCI := tsc_NR_BWP_Id_Initial; v_BwpId_UL_DCI_IndicatorData := tsc_NR_BWP_Id_1; } } v_TimingInfoSS := f_NR_GetNextSendOccasion_DL(p_NR_CellId); f_NR_SS_ConfigureBWPs(p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoSS), v_BwpId_DL_DCI, v_BwpId_DL_DCI_IndicatorData, v_BwpId_UL_DCI, v_BwpId_UL_DCI_IndicatorData, v_MaxBWP); // @siclog "Step 5a/b/c/d" siclog@ v_TimingInfoDL := f_TC_7_1_1_8_1_NR_GetNextSendOccasion_DL(p_NR_CellId); v_RLC_SDU1 := fl_TC_7_1_1_8_1_SendData(p_NR_CellId, p_DrbId, cs_TimingInfo_NR(v_TimingInfoDL), v_SN); // @siclog "Step 6a/b/c/d" siclog@ v_TimingInfoUL := f_NR_GetNextSendOccasion_ULafterDL(p_NR_CellId, v_TimingInfoDL, 100); // 100ms after step 5 f_NR_OneULGrantTransmission(p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoUL), p_ResourceAllocationUL); // @siclog "Steps 7a/b/c/d-7A" siclog@ fl_TC_7_1_1_8_1_ReceiveData(p_NR_CellId, p_DrbId, {v_RLC_SDU1}, true, v_SN); //@sic R5-231578 R5s230220 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 7"); v_SN := v_SN + 1; //@sic R5s240283 sic@ v_TimingInfoSS := f_NR_GetNextSendOccasion_DL(p_NR_CellId); //@sic R5s230220 change 9.2 R5-2134599 sic@ // Reconfigure DL DCI to be transmitted on v_BwpId_DL_DCI_IndicatorData f_NR_SS_ConfigureBWPs_DL(p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoSS), v_BwpId_DL_DCI_IndicatorData, v_MaxBWP); // @siclog "Step 9a/b/c/d" siclog@ v_TimingInfoDL := f_TC_7_1_1_8_1_NR_GetNextSendOccasion_DL(p_NR_CellId, 300); v_RLC_SDU1 := fl_TC_7_1_1_8_1_SendData(p_NR_CellId, p_DrbId, cs_TimingInfo_NR(v_TimingInfoDL), v_SN); //@sic R5s230220 change 9.2 sic@ //Initialise variable for step 10a/b/c/d if (v_BWP_SameNumerology_upto4) { if (not(v_IsFDD)) { //step 10a - 11a //IF pc_bwp_sameNumerology_upto4_FR1_TDD and band under test is FR1 TDD) or (pc_bwp_sameNumerology_upto4_FR2 and band under test is FR2) //THEN after 100ms from step 9 the SS indicates on PDCCH (transmitted in Downlink BWP#2) an UL DCI format 0_1 with new BWP Id BWP #3 and allocates an UL Grant, sufficient for loopback of the RLC SDU from step 9. v_BwpId_DL_DCI := tsc_NR_BWP_Id_3; v_BwpId_DL_DCI_IndicatorData := tsc_NR_BWP_Id_3; v_BwpId_UL_DCI := tsc_NR_BWP_Id_2; v_BwpId_UL_DCI_IndicatorData := tsc_NR_BWP_Id_3; } else { //Step 10b - 11b //IF pc_bwp_sameNumerology_upto4_FR1_FDD and band under test is FR1 FDD //THEN After 100ms from step 9 the SS indicates on PDCCH (transmitted in Downlink BWP#2) an UL DCI format 0_1 with new BWP Id BWP #3 and allocates an UL Grant, sufficient for loopback of the RLC SDU from step 9. v_BwpId_DL_DCI := tsc_NR_BWP_Id_2; v_BwpId_DL_DCI_IndicatorData := tsc_NR_BWP_Id_2; v_BwpId_UL_DCI := tsc_NR_BWP_Id_2; v_BwpId_UL_DCI_IndicatorData := tsc_NR_BWP_Id_3; } } else { if (not(v_IsFDD)) { //Step 10c - 11c //IF (pc_bwp_sameNumerology_upto2_FR1_TDD and band under test is FR1 TDD) or (pc_bwp_sameNumerology_upto2_FR2 and band under test is FR2) //THEN After 100ms from step 9 the SS indicates on PDCCH (transmitted in Downlink BWP#0 an UL DCI format 0_1 with new BWP Id BWP#1 and allocates an UL Grant, sufficient for loopback of the RLC SDU from step 9. v_BwpId_DL_DCI := tsc_NR_BWP_Id_1; v_BwpId_DL_DCI_IndicatorData := tsc_NR_BWP_Id_1; v_BwpId_UL_DCI := tsc_NR_BWP_Id_Initial; v_BwpId_UL_DCI_IndicatorData := tsc_NR_BWP_Id_1; } else { // Step 10d - 11d v_BwpId_DL_DCI := tsc_NR_BWP_Id_Initial; v_BwpId_DL_DCI_IndicatorData := tsc_NR_BWP_Id_Initial; v_BwpId_UL_DCI := tsc_NR_BWP_Id_Initial; v_BwpId_UL_DCI_IndicatorData := tsc_NR_BWP_Id_Initial; } } v_TimingInfoUL_SS := f_NR_GetNextSendOccasion_ULafterDL(p_NR_CellId, v_TimingInfoDL, 90); // 90 ms after step 9 - reconfigure UL/DL BWP in SS v_TimingInfoUL := f_NR_GetNextSendOccasion_ULafterDL(p_NR_CellId, v_TimingInfoDL, 100); // 100ms after step 9 if (not(v_IsFDD)) { f_NR_SS_ConfigureBWPs( p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoUL_SS), v_BwpId_DL_DCI, v_BwpId_DL_DCI_IndicatorData, v_BwpId_UL_DCI, v_BwpId_UL_DCI_IndicatorData, v_MaxBWP); } else { // For FDD reconfigure only UL BWP f_NR_SS_ConfigureBWPs_UL( p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoUL_SS), v_BwpId_UL_DCI, v_BwpId_UL_DCI_IndicatorData, -, v_MaxBWP); } // @siclog "Step 10a/b/c/d" siclog@ f_NR_OneULGrantTransmission(p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoUL), p_ResourceAllocationUL); // @siclog "Steps 11a/b/c/d-11AA" siclog@ fl_TC_7_1_1_8_1_ReceiveData(p_NR_CellId, p_DrbId, {v_RLC_SDU1}, true, v_SN); //@sic R5-231578 R5s230220 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 11"); if (v_IsFDD and not(v_BWP_SameNumerology_upto4)) { // for FDD and pc_bwp_sameNumerology_upto2_FR1_FDD switch DL and UL BWP from BWP#0 to (BWP#1) v_BwpId_DL_DCI := tsc_NR_BWP_Id_Initial; v_BwpId_DL_DCI_IndicatorData := tsc_NR_BWP_Id_1; v_BwpId_UL_DCI := tsc_NR_BWP_Id_1; v_BwpId_UL_DCI_IndicatorData := tsc_NR_BWP_Id_1; v_SN := v_SN + 1; //@siclog 11AABa1 siclog@ f_NR_SS_ConfigureBWPs( p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoUL_SS), v_BwpId_DL_DCI, v_BwpId_DL_DCI_IndicatorData, v_BwpId_UL_DCI, v_BwpId_UL_DCI_IndicatorData, v_MaxBWP); v_TimingInfoDL := f_TC_7_1_1_8_1_NR_GetNextSendOccasion_DL(p_NR_CellId); v_RLC_SDU1 := fl_TC_7_1_1_8_1_SendData(p_NR_CellId, p_DrbId, cs_TimingInfo_NR(v_TimingInfoDL), v_SN); // @siclog "Step 11AABa2" siclog@ //After 100ms from step 9 the SS indicates on PDCCH (transmitted in Downlink BWP#1 an UL DCI format 0_1 with new BWP Id BWP#1 and allocates an UL Grant, sufficient for loopback of the RLC SDU from step 11AABa1. v_TimingInfoUL := f_NR_GetNextSendOccasion_ULafterDL(p_NR_CellId, v_TimingInfoDL, 100); f_NR_OneULGrantTransmission(p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoUL), p_ResourceAllocationUL); // @siclog "Steps 11AABa3 - 11AABa4" siclog@ fl_TC_7_1_1_8_1_ReceiveData(p_NR_CellId, p_DrbId, {v_RLC_SDU1}, true, v_SN); f_NR_SS_ConfigureBWPs_DL(p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoSS), v_BwpId_DL_DCI_IndicatorData, v_MaxBWP); } // configure CBRA CRNTI Based RACH procedure //@sic R5s230220 change 10 sic@ v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(p_NR_CellId); v_NR_RachProcedure := cs_NR_RachProcedureConfig_CBRA_CrntiBased(v_NR_PhysicalParameters, p_ResourceAllocationUL, -, tsc_NR_ENDC_Msg3Grant_Def, cs_NR_AssignedBWPs_InitialBWP); f_NR_SS_RachProcedureConfig(p_NR_CellId, v_NR_RachProcedure); v_SN := v_SN + 1; //@sic R5-2134599 R5s230634 sic@ if (v_BWP_SameNumerology_upto4) { v_BWP_Id_PDCCH_Order := tsc_NR_BWP_Id_3; } else { v_BWP_Id_PDCCH_Order := tsc_NR_BWP_Id_1; } // @sic R5s230733 sic@ // @siclog "Step 11Aa/b/c/d" siclog@ v_TimingInfoDL := f_TC_7_1_1_8_1_NR_GetNextSendOccasion_DL(p_NR_CellId); v_RLC_SDU1 := fl_TC_7_1_1_8_1_SendData(p_NR_CellId, p_DrbId, cs_TimingInfo_NR(v_TimingInfoDL), v_SN); // Enable PRACH reception indication f_NR_SS_SystemIndCtrlConfig(p_NR_CellId, cds_NR_SystemIndCtrl_RachPreamble(enable)); // @siclog "Step 12" siclog@ v_TimingInfoDL := f_SubFrameTiming_AddMilliSeconds(v_TimingInfoDL, 100); // 100ms after step 9 v_RA_PreambleIndex := 0; // => contention-based random access v_PDCCH_Order := cs_NR_PDCCH_Order(f_NR_CellInfo_GetSSB_Index(p_NR_CellId), v_RA_PreambleIndex); f_NR_SS_CommonCellConfig(p_NR_CellId, cads_NR_DciTriggerPDCCHOrder_REQ(p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoDL), v_PDCCH_Order, cs_NR_AssignedBWPs_DedicatedBWP(v_BWP_Id_PDCCH_Order))); // @sic R5s230733 sic@ v_TimingInfoConfig := f_SubFrameTiming_AddMilliSeconds(v_TimingInfoDL, 10); f_NR_SS_ConfigureBWPs(p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoConfig), tsc_NR_BWP_Id_Initial, tsc_NR_BWP_Id_Initial, tsc_NR_BWP_Id_Initial, tsc_NR_BWP_Id_Initial); // @siclog "Step 13" siclog@ SYSIND.receive(car_NR_PRACH_Preamble_IND(p_NR_CellId)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 13"); // @siclog "Step 13A - 13C" siclog@ // steps are done automatically according to configuration of the RACH procedure // @siclog "Steps 13D-13E" siclog@ fl_TC_7_1_1_8_1_ReceiveData(p_NR_CellId, p_DrbId, {v_RLC_SDU1}, true, v_SN); //@sic R5-231578 R5s230220 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 13D"); f_NR_SS_SystemIndCtrlConfig(p_NR_CellId, cds_NR_SystemIndCtrl_RachPreamble(disable)); //@sic R5s230634 sic@ v_SN := v_SN + 1; if (not(v_IsFDD)) { //Step 16 - 18a/18c - 19a/c //IF (pc_bwp_sameNumerology_upto2_FR1_TDD and band under test is FR1 TDD) or (pc_bwp_sameNumerology_upto2_FR2 and band under test is FR2) //THEN After 100ms from step 9 the SS indicates on PDCCH (transmitted in Downlink BWP#0 an UL DCI format 0_1 with new BWP Id BWP#1 and allocates an UL Grant, sufficient for loopback of the RLC SDU from step 9. v_BwpId_DL_DCI := tsc_NR_BWP_Id_Initial; v_BwpId_DL_DCI_IndicatorData := tsc_NR_BWP_Id_1; v_BwpId_UL_DCI := tsc_NR_BWP_Id_1; v_BwpId_UL_DCI_IndicatorData := tsc_NR_BWP_Id_1; } else { // Step 16 - 18b/d - 19b/d v_BwpId_DL_DCI := tsc_NR_BWP_Id_Initial; v_BwpId_DL_DCI_IndicatorData := tsc_NR_BWP_Id_1; v_BwpId_UL_DCI := tsc_NR_BWP_Id_Initial; v_BwpId_UL_DCI_IndicatorData := tsc_NR_BWP_Id_Initial; } //@sic R5s240283 sic@ v_TimingInfoSS := f_NR_GetNextSendOccasion_DL(p_NR_CellId); //@sic R5s230634 sic@ f_NR_SS_ConfigureBWPs(p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoSS), v_BwpId_DL_DCI, v_BwpId_DL_DCI_IndicatorData, v_BwpId_UL_DCI, v_BwpId_UL_DCI_IndicatorData); // @siclog "Step 16" siclog@ v_TimingInfoDL := f_TC_7_1_1_8_1_NR_GetNextSendOccasion_DL(p_NR_CellId); //@sic R5s230220 change 10 sic@ v_TimingInfoConfig := f_SubFrameTiming_AddMilliSeconds(v_TimingInfoDL, 100); // 100ms after step 16 v_RLC_SDU1 := fl_TC_7_1_1_8_1_SendData(p_NR_CellId, p_DrbId, cs_TimingInfo_NR(v_TimingInfoDL), v_SN); v_SN := v_SN + 1; //@sic R5s230220 change 10 sic@ f_NR_SS_ConfigureBWPs_DL(p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoConfig), tsc_NR_BWP_Id_1); if (v_IsFDD) { // For FDD reconfigure only UL DCI BWP f_NR_SS_ConfigureBWPs_UL( p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoUL_SS), tsc_NR_BWP_Id_1, v_BwpId_UL_DCI_IndicatorData, -, v_MaxBWP); } // @siclog "Step 17" siclog@ v_TimingInfoDL := f_SubFrameTiming_AddMilliSeconds(v_TimingInfoDL, 200); // 200ms after step 16 @sic R5s230220 R5-231578 sic@ v_RLC_SDU2 := fl_TC_7_1_1_8_1_SendData(p_NR_CellId, p_DrbId, cs_TimingInfo_NR(v_TimingInfoDL), v_SN); // @siclog "Step 18a/b/c/d" siclog@ v_TimingInfoUL := f_NR_GetNextSendOccasion_ULafterDL(p_NR_CellId, v_TimingInfoDL, 200); // 200ms after step 17 @sic R5s230220 R5-231578 sic@ f_NR_OneULGrantTransmission(p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoUL), p_ResourceAllocationUL); // @siclog "Steps 19a/b/c/d-19A" siclog@ fl_TC_7_1_1_8_1_ReceiveData(p_NR_CellId, p_DrbId, {v_RLC_SDU1, v_RLC_SDU2}, true, v_SN); //@sic R5-231578 R5s230220 sic@ v_SN := v_SN + 1; //@sic R5-216169 R5s230220 R5s230634 R5s240283 sic@ v_TimingInfoSS := f_NR_GetNextSendOccasion_DL(p_NR_CellId); if (v_BWP_SameNumerology_upto4 ) { f_NR_SS_ConfigureBWPs(p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoSS), tsc_NR_BWP_Id_2, tsc_NR_BWP_Id_2, tsc_NR_BWP_Id_2, tsc_NR_BWP_Id_2); } else { f_NR_SS_ConfigureBWPs(p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoSS), tsc_NR_BWP_Id_Initial, tsc_NR_BWP_Id_Initial, tsc_NR_BWP_Id_Initial, tsc_NR_BWP_Id_Initial); } // @siclog "Step 20a/b/c/d" siclog@ //The SS waits 1000 ms from step 18 to ensure that the bwp-InactivityTimer expired and then transmits a valid MAC PDU containing RLC PDU in the BWP with defaultDownlinkBWP-Id (= Downlink BWP#2 IF pc_bwp_sameNumerology_upto4_FR1_TDD and band under test is FR1 TDD ELSE BWP#0 IF pc_bwp_sameNumerology_upto2_FR1_TDD and band under test is FR1 TDD). v_TimingInfoDL := f_SubFrameTiming_AddMilliSeconds(v_TimingInfoDL, 1200); // 1200ms after step 17 => 1000ms after step 18 @sic R5s240283 sic@ v_RLC_SDU1 := fl_TC_7_1_1_8_1_SendData(p_NR_CellId, p_DrbId, cs_TimingInfo_NR(v_TimingInfoDL), v_SN); // @siclog "Step 21" siclog@ v_TimingInfoUL := f_NR_GetNextSendOccasion_ULafterDL(p_NR_CellId, v_TimingInfoDL, 100); // 100ms after step 20 f_NR_OneULGrantTransmission(p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoUL), p_ResourceAllocationUL); // @siclog "Steps 22-22A" siclog@ fl_TC_7_1_1_8_1_ReceiveData(p_NR_CellId, p_DrbId, {v_RLC_SDU1}, true, v_SN); //@sic R5-231578 R5s230220 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 22"); //DL and UL BWP already configured on tsc_NR_BWP_Id_2/tsc_NR_BWP_Id_Initial @sic R5s230733 sic@ }

After Change:

function f_TC_7_1_1_8_1_NR_CommonTestBody(NR_CellId_Type p_NR_CellId, DRB_Identity p_DrbId, template (value) NR_ResourceAllocation_Type p_ResourceAllocationUL) runs on NR_BASE_PTC { //@sic R5-213459 R5-227417 R5-244020 sic@ var NR_PhysicalParameters_Type v_NR_PhysicalParameters; var template (value) NR_RachProcedureConfig_Type v_NR_RachProcedure; var template (value) NR_PDCCH_Order_Type v_PDCCH_Order; var BWP_Id v_BWP_Id_PDCCH_Order; var SubFrameTiming_Type v_TimingInfoDL; var SubFrameTiming_Type v_TimingInfoSS; //@sic R5s240283 sic@ var SubFrameTiming_Type v_TimingInfoUL; var SubFrameTiming_Type v_TimingInfoConfig; var octetstring v_RLC_SDU1; var octetstring v_RLC_SDU2; var integer v_RA_PreambleIndex; var integer v_SN := 0; var boolean v_BWP_SameNumerology_upto4 := fl_NR_ReturnBWP_SameNumerologyUpto4_Capability(p_NR_CellId); //@sic R5-231578 sic@ var SubFrameTiming_Type v_TimingInfoUL_SS; var BWP_Id v_BwpId_DL_DCI; //BWP on which the DL DCI is transmitted var BWP_Id v_BwpId_DL_DCI_IndicatorData; // Indicator inside DL BWP = BWP on which DL data is tranmsitted var BWP_Id v_BwpId_UL_DCI; //BWP on which the UL DCI is transmitted var BWP_Id v_BwpId_UL_DCI_IndicatorData; // Indicator inside UL BWP = BWP on which UL data is tranmsitted var boolean v_IsFDD := f_NR_CellInfo_GetIsFDD(p_NR_CellId); var integer v_MaxBWP := 3; if (not v_BWP_SameNumerology_upto4) { v_MaxBWP := 1; } // @siclog "Step 1" siclog@ v_TimingInfoDL := f_TC_7_1_1_8_1_NR_GetNextSendOccasion_DL(p_NR_CellId); v_RLC_SDU1 := fl_TC_7_1_1_8_1_SendData(p_NR_CellId, p_DrbId, cs_TimingInfo_NR(v_TimingInfoDL), v_SN); // @siclog "Step 2" siclog@ v_TimingInfoUL := f_NR_GetNextSendOccasion_ULafterDL(p_NR_CellId, v_TimingInfoDL, 100); // 100ms after step 1 f_NR_OneULGrantTransmission(p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoUL), p_ResourceAllocationUL); // @siclog "Steps 3-3A" siclog@ fl_TC_7_1_1_8_1_ReceiveData(p_NR_CellId, p_DrbId, {v_RLC_SDU1}, true, v_SN); //@sic R5-231578 R5s230220 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 3"); v_SN := v_SN + 1; //@sic R5s240283 sic@ if (v_BWP_SameNumerology_upto4) { if (not(v_IsFDD)) { //step 5a - 6a - 7a //IF (pc_bwp_sameNumerology_upto4_FR1_TDD and band under test is FR1 TDD) or (pc_bwp_sameNumerology_upto4_FR2 and band under test is FR2) //THEN The SS indicates on PDCCH (transmitted in Downlink BWP#1) DL DCI format 1_1 with new BWP Id BWP#2 and transmits a MAC PDU containing RLC PDU on the newly configured BWP (BWP#2) v_BwpId_DL_DCI := tsc_NR_BWP_Id_1; v_BwpId_DL_DCI_IndicatorData := tsc_NR_BWP_Id_2; v_BwpId_UL_DCI := tsc_NR_BWP_Id_2; v_BwpId_UL_DCI_IndicatorData := tsc_NR_BWP_Id_2; } else { //Step 5b - 6b - 7b //IF pc_bwp_sameNumerology_upto4_FR1_FDD and band under test is FR1 FDD //THEN The SS indicates on PDCCH (transmitted in Downlink BWP#1) DL DCI format 1_1 with new BWP Id BWP #2 and transmits a MAC PDU containing RLC PDU on the newly configured BWP (BWP#2) v_BwpId_DL_DCI := tsc_NR_BWP_Id_1; v_BwpId_DL_DCI_IndicatorData := tsc_NR_BWP_Id_2; v_BwpId_UL_DCI := tsc_NR_BWP_Id_2; v_BwpId_UL_DCI_IndicatorData := tsc_NR_BWP_Id_1; } } else { if (not(v_IsFDD)) { //Step 5c - 6c - 7c //IF (pc_bwp_sameNumerology_upto2_FR1_TDD and band under test is FR1 TDD ) or (pc_bwp_sameNumerology_upto2_FR2 and band under test is FR2) //THEN The SS indicates on PDCCH (transmitted in Downlink BWP#1) DL DCI format 1_1 with new BWP Id BWP#0 and transmits a MAC PDU containing RLC PDU on the newly configured BWP (BWP#0) v_BwpId_DL_DCI := tsc_NR_BWP_Id_1; v_BwpId_DL_DCI_IndicatorData := tsc_NR_BWP_Id_Initial; v_BwpId_UL_DCI := tsc_NR_BWP_Id_Initial; v_BwpId_UL_DCI_IndicatorData := tsc_NR_BWP_Id_Initial; } else { //Step 5d - 6d - 7d //IF pc_bwp_sameNumerology_upto2_FR1_FDD and band under test is FR1 FDD //THEN The SS indicates on PDCCH (transmitted in Downlink BWP#1) DL DCI format 1_1 with new BWP Id BWP #0 and transmits a MAC PDU containing RLC PDU on the newly configured BWP (BWP#0) v_BwpId_DL_DCI := tsc_NR_BWP_Id_1; v_BwpId_DL_DCI_IndicatorData := tsc_NR_BWP_Id_Initial; v_BwpId_UL_DCI := tsc_NR_BWP_Id_Initial; v_BwpId_UL_DCI_IndicatorData := tsc_NR_BWP_Id_1; } } v_TimingInfoSS := f_NR_GetNextSendOccasion_DL(p_NR_CellId); f_NR_SS_ConfigureBWPs(p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoSS), v_BwpId_DL_DCI, v_BwpId_DL_DCI_IndicatorData, v_BwpId_UL_DCI, v_BwpId_UL_DCI_IndicatorData, v_MaxBWP); // @siclog "Step 5a/b/c/d" siclog@ v_TimingInfoDL := f_TC_7_1_1_8_1_NR_GetNextSendOccasion_DL(p_NR_CellId); v_RLC_SDU1 := fl_TC_7_1_1_8_1_SendData(p_NR_CellId, p_DrbId, cs_TimingInfo_NR(v_TimingInfoDL), v_SN); // @siclog "Step 6a/b/c/d" siclog@ v_TimingInfoUL := f_NR_GetNextSendOccasion_ULafterDL(p_NR_CellId, v_TimingInfoDL, 100); // 100ms after step 5 f_NR_OneULGrantTransmission(p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoUL), p_ResourceAllocationUL); // @siclog "Steps 7a/b/c/d-7A" siclog@ fl_TC_7_1_1_8_1_ReceiveData(p_NR_CellId, p_DrbId, {v_RLC_SDU1}, true, v_SN); //@sic R5-231578 R5s230220 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 7"); v_SN := v_SN + 1; //@sic R5s240283 sic@ v_TimingInfoSS := f_NR_GetNextSendOccasion_DL(p_NR_CellId); //@sic R5s230220 change 9.2 R5-2134599 sic@ // Reconfigure DL DCI to be transmitted on v_BwpId_DL_DCI_IndicatorData f_NR_SS_ConfigureBWPs_DL(p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoSS), v_BwpId_DL_DCI_IndicatorData, v_BwpId_DL_DCI_IndicatorData); //v_MaxBWP // @siclog "Step 9a/b/c/d" siclog@ v_TimingInfoDL := f_TC_7_1_1_8_1_NR_GetNextSendOccasion_DL(p_NR_CellId, 300); v_RLC_SDU1 := fl_TC_7_1_1_8_1_SendData(p_NR_CellId, p_DrbId, cs_TimingInfo_NR(v_TimingInfoDL), v_SN); //@sic R5s230220 change 9.2 sic@ //Initialise variable for step 10a/b/c/d if (v_BWP_SameNumerology_upto4) { if (not(v_IsFDD)) { //step 10a - 11a //IF pc_bwp_sameNumerology_upto4_FR1_TDD and band under test is FR1 TDD) or (pc_bwp_sameNumerology_upto4_FR2 and band under test is FR2) //THEN after 100ms from step 9 the SS indicates on PDCCH (transmitted in Downlink BWP#2) an UL DCI format 0_1 with new BWP Id BWP #3 and allocates an UL Grant, sufficient for loopback of the RLC SDU from step 9. v_BwpId_DL_DCI := tsc_NR_BWP_Id_3; v_BwpId_DL_DCI_IndicatorData := tsc_NR_BWP_Id_3; v_BwpId_UL_DCI := tsc_NR_BWP_Id_2; v_BwpId_UL_DCI_IndicatorData := tsc_NR_BWP_Id_3; } else { //Step 10b - 11b //IF pc_bwp_sameNumerology_upto4_FR1_FDD and band under test is FR1 FDD //THEN After 100ms from step 9 the SS indicates on PDCCH (transmitted in Downlink BWP#2) an UL DCI format 0_1 with new BWP Id BWP #3 and allocates an UL Grant, sufficient for loopback of the RLC SDU from step 9. v_BwpId_DL_DCI := tsc_NR_BWP_Id_2; v_BwpId_DL_DCI_IndicatorData := tsc_NR_BWP_Id_2; v_BwpId_UL_DCI := tsc_NR_BWP_Id_2; v_BwpId_UL_DCI_IndicatorData := tsc_NR_BWP_Id_3; } } else { if (not(v_IsFDD)) { //Step 10c - 11c //IF (pc_bwp_sameNumerology_upto2_FR1_TDD and band under test is FR1 TDD) or (pc_bwp_sameNumerology_upto2_FR2 and band under test is FR2) //THEN After 100ms from step 9 the SS indicates on PDCCH (transmitted in Downlink BWP#0 an UL DCI format 0_1 with new BWP Id BWP#1 and allocates an UL Grant, sufficient for loopback of the RLC SDU from step 9. v_BwpId_DL_DCI := tsc_NR_BWP_Id_1; v_BwpId_DL_DCI_IndicatorData := tsc_NR_BWP_Id_1; v_BwpId_UL_DCI := tsc_NR_BWP_Id_Initial; v_BwpId_UL_DCI_IndicatorData := tsc_NR_BWP_Id_1; } else { // Step 10d - 11d v_BwpId_DL_DCI := tsc_NR_BWP_Id_Initial; v_BwpId_DL_DCI_IndicatorData := tsc_NR_BWP_Id_Initial; v_BwpId_UL_DCI := tsc_NR_BWP_Id_Initial; v_BwpId_UL_DCI_IndicatorData := tsc_NR_BWP_Id_Initial; } } v_TimingInfoUL_SS := f_NR_GetNextSendOccasion_ULafterDL(p_NR_CellId, v_TimingInfoDL, 90); // 90 ms after step 9 - reconfigure UL/DL BWP in SS v_TimingInfoUL := f_NR_GetNextSendOccasion_ULafterDL(p_NR_CellId, v_TimingInfoDL, 100); // 100ms after step 9 if (not(v_IsFDD)) { f_NR_SS_ConfigureBWPs( p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoUL_SS), v_BwpId_DL_DCI, v_BwpId_DL_DCI_IndicatorData, v_BwpId_UL_DCI, v_BwpId_UL_DCI_IndicatorData, v_MaxBWP+1); } else { // For FDD reconfigure only UL BWP f_NR_SS_ConfigureBWPs_UL( p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoUL_SS), v_BwpId_UL_DCI, v_BwpId_UL_DCI_IndicatorData, -, v_MaxBWP); } // @siclog "Step 10a/b/c/d" siclog@ f_NR_OneULGrantTransmission(p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoUL), p_ResourceAllocationUL); // @siclog "Steps 11a/b/c/d-11AA" siclog@ fl_TC_7_1_1_8_1_ReceiveData(p_NR_CellId, p_DrbId, {v_RLC_SDU1}, true, v_SN); //@sic R5-231578 R5s230220 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 11"); if (v_IsFDD and not(v_BWP_SameNumerology_upto4)) { // for FDD and pc_bwp_sameNumerology_upto2_FR1_FDD switch DL and UL BWP from BWP#0 to (BWP#1) v_BwpId_DL_DCI := tsc_NR_BWP_Id_Initial; v_BwpId_DL_DCI_IndicatorData := tsc_NR_BWP_Id_1; v_BwpId_UL_DCI := tsc_NR_BWP_Id_1; v_BwpId_UL_DCI_IndicatorData := tsc_NR_BWP_Id_1; v_SN := v_SN + 1; //@siclog 11AABa1 siclog@ f_NR_SS_ConfigureBWPs( p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoUL_SS), v_BwpId_DL_DCI, v_BwpId_DL_DCI_IndicatorData, v_BwpId_UL_DCI, v_BwpId_UL_DCI_IndicatorData, v_MaxBWP); v_TimingInfoDL := f_TC_7_1_1_8_1_NR_GetNextSendOccasion_DL(p_NR_CellId); v_RLC_SDU1 := fl_TC_7_1_1_8_1_SendData(p_NR_CellId, p_DrbId, cs_TimingInfo_NR(v_TimingInfoDL), v_SN); // @siclog "Step 11AABa2" siclog@ //After 100ms from step 9 the SS indicates on PDCCH (transmitted in Downlink BWP#1 an UL DCI format 0_1 with new BWP Id BWP#1 and allocates an UL Grant, sufficient for loopback of the RLC SDU from step 11AABa1. v_TimingInfoUL := f_NR_GetNextSendOccasion_ULafterDL(p_NR_CellId, v_TimingInfoDL, 100); f_NR_OneULGrantTransmission(p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoUL), p_ResourceAllocationUL); // @siclog "Steps 11AABa3 - 11AABa4" siclog@ fl_TC_7_1_1_8_1_ReceiveData(p_NR_CellId, p_DrbId, {v_RLC_SDU1}, true, v_SN); f_NR_SS_ConfigureBWPs_DL(p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoSS), v_BwpId_DL_DCI_IndicatorData, v_MaxBWP); } // configure CBRA CRNTI Based RACH procedure //@sic R5s230220 change 10 sic@ v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(p_NR_CellId); v_NR_RachProcedure := cs_NR_RachProcedureConfig_CBRA_CrntiBased(v_NR_PhysicalParameters, p_ResourceAllocationUL, -, tsc_NR_ENDC_Msg3Grant_Def, cs_NR_AssignedBWPs_InitialBWP); f_NR_SS_RachProcedureConfig(p_NR_CellId, v_NR_RachProcedure); v_SN := v_SN + 1; //@sic R5-2134599 R5s230634 sic@ if (v_BWP_SameNumerology_upto4) { v_BWP_Id_PDCCH_Order := tsc_NR_BWP_Id_3; } else { v_BWP_Id_PDCCH_Order := tsc_NR_BWP_Id_1; } // @sic R5s230733 sic@ // @siclog "Step 11Aa/b/c/d" siclog@ v_TimingInfoDL := f_TC_7_1_1_8_1_NR_GetNextSendOccasion_DL(p_NR_CellId); v_RLC_SDU1 := fl_TC_7_1_1_8_1_SendData(p_NR_CellId, p_DrbId, cs_TimingInfo_NR(v_TimingInfoDL), v_SN); // Enable PRACH reception indication f_NR_SS_SystemIndCtrlConfig(p_NR_CellId, cds_NR_SystemIndCtrl_RachPreamble(enable)); // @siclog "Step 12" siclog@ v_TimingInfoDL := f_SubFrameTiming_AddMilliSeconds(v_TimingInfoDL, 100); // 100ms after step 9 v_RA_PreambleIndex := 0; // => contention-based random access v_PDCCH_Order := cs_NR_PDCCH_Order(f_NR_CellInfo_GetSSB_Index(p_NR_CellId), v_RA_PreambleIndex); f_NR_SS_CommonCellConfig(p_NR_CellId, cads_NR_DciTriggerPDCCHOrder_REQ(p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoDL), v_PDCCH_Order, cs_NR_AssignedBWPs_DedicatedBWP(v_BWP_Id_PDCCH_Order))); // @sic R5s230733 sic@ v_TimingInfoConfig := f_SubFrameTiming_AddMilliSeconds(v_TimingInfoDL, 10); f_NR_SS_ConfigureBWPs(p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoConfig), tsc_NR_BWP_Id_Initial, tsc_NR_BWP_Id_Initial, tsc_NR_BWP_Id_Initial, tsc_NR_BWP_Id_Initial); // @siclog "Step 13" siclog@ SYSIND.receive(car_NR_PRACH_Preamble_IND(p_NR_CellId)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 13"); // @siclog "Step 13A - 13C" siclog@ // steps are done automatically according to configuration of the RACH procedure // @siclog "Steps 13D-13E" siclog@ fl_TC_7_1_1_8_1_ReceiveData(p_NR_CellId, p_DrbId, {v_RLC_SDU1}, true, v_SN); //@sic R5-231578 R5s230220 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 13D"); f_NR_SS_SystemIndCtrlConfig(p_NR_CellId, cds_NR_SystemIndCtrl_RachPreamble(disable)); //@sic R5s230634 sic@ v_SN := v_SN + 1; if (not(v_IsFDD)) { //Step 16 - 18a/18c - 19a/c //IF (pc_bwp_sameNumerology_upto2_FR1_TDD and band under test is FR1 TDD) or (pc_bwp_sameNumerology_upto2_FR2 and band under test is FR2) //THEN After 100ms from step 9 the SS indicates on PDCCH (transmitted in Downlink BWP#0 an UL DCI format 0_1 with new BWP Id BWP#1 and allocates an UL Grant, sufficient for loopback of the RLC SDU from step 9. v_BwpId_DL_DCI := tsc_NR_BWP_Id_Initial; v_BwpId_DL_DCI_IndicatorData := tsc_NR_BWP_Id_1; v_BwpId_UL_DCI := tsc_NR_BWP_Id_1; v_BwpId_UL_DCI_IndicatorData := tsc_NR_BWP_Id_1; } else { // Step 16 - 18b/d - 19b/d v_BwpId_DL_DCI := tsc_NR_BWP_Id_Initial; v_BwpId_DL_DCI_IndicatorData := tsc_NR_BWP_Id_1; v_BwpId_UL_DCI := tsc_NR_BWP_Id_Initial; v_BwpId_UL_DCI_IndicatorData := tsc_NR_BWP_Id_Initial; } //@sic R5s240283 sic@ v_TimingInfoSS := f_NR_GetNextSendOccasion_DL(p_NR_CellId); //@sic R5s230634 sic@ f_NR_SS_ConfigureBWPs(p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoSS), v_BwpId_DL_DCI, v_BwpId_DL_DCI_IndicatorData, v_BwpId_UL_DCI, v_BwpId_UL_DCI_IndicatorData); // @siclog "Step 16" siclog@ v_TimingInfoDL := f_TC_7_1_1_8_1_NR_GetNextSendOccasion_DL(p_NR_CellId); //@sic R5s230220 change 10 sic@ v_TimingInfoConfig := f_SubFrameTiming_AddMilliSeconds(v_TimingInfoDL, 100); // 100ms after step 16 v_RLC_SDU1 := fl_TC_7_1_1_8_1_SendData(p_NR_CellId, p_DrbId, cs_TimingInfo_NR(v_TimingInfoDL), v_SN); v_SN := v_SN + 1; //@sic R5s230220 change 10 sic@ f_NR_SS_ConfigureBWPs_DL(p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoConfig), tsc_NR_BWP_Id_1); if (v_IsFDD) { // For FDD reconfigure only UL DCI BWP f_NR_SS_ConfigureBWPs_UL( p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoUL_SS), tsc_NR_BWP_Id_1, v_BwpId_UL_DCI_IndicatorData, -, v_MaxBWP); } // @siclog "Step 17" siclog@ v_TimingInfoDL := f_SubFrameTiming_AddMilliSeconds(v_TimingInfoDL, 200); // 200ms after step 16 @sic R5s230220 R5-231578 sic@ v_RLC_SDU2 := fl_TC_7_1_1_8_1_SendData(p_NR_CellId, p_DrbId, cs_TimingInfo_NR(v_TimingInfoDL), v_SN); // @siclog "Step 18a/b/c/d" siclog@ v_TimingInfoUL := f_NR_GetNextSendOccasion_ULafterDL(p_NR_CellId, v_TimingInfoDL, 200); // 200ms after step 17 @sic R5s230220 R5-231578 sic@ f_NR_OneULGrantTransmission(p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoUL), p_ResourceAllocationUL); // @siclog "Steps 19a/b/c/d-19A" siclog@ fl_TC_7_1_1_8_1_ReceiveData(p_NR_CellId, p_DrbId, {v_RLC_SDU1, v_RLC_SDU2}, true, v_SN); //@sic R5-231578 R5s230220 sic@ v_SN := v_SN + 1; //@sic R5-216169 R5s230220 R5s230634 R5s240283 sic@ v_TimingInfoSS := f_NR_GetNextSendOccasion_DL(p_NR_CellId); if (v_BWP_SameNumerology_upto4 ) { f_NR_SS_ConfigureBWPs(p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoSS), tsc_NR_BWP_Id_2, tsc_NR_BWP_Id_2, tsc_NR_BWP_Id_2, tsc_NR_BWP_Id_2); } else { f_NR_SS_ConfigureBWPs(p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoSS), tsc_NR_BWP_Id_Initial, tsc_NR_BWP_Id_Initial, tsc_NR_BWP_Id_Initial, tsc_NR_BWP_Id_Initial); } // @siclog "Step 20a/b/c/d" siclog@ //The SS waits 1000 ms from step 18 to ensure that the bwp-InactivityTimer expired and then transmits a valid MAC PDU containing RLC PDU in the BWP with defaultDownlinkBWP-Id (= Downlink BWP#2 IF pc_bwp_sameNumerology_upto4_FR1_TDD and band under test is FR1 TDD ELSE BWP#0 IF pc_bwp_sameNumerology_upto2_FR1_TDD and band under test is FR1 TDD). v_TimingInfoDL := f_SubFrameTiming_AddMilliSeconds(v_TimingInfoDL, 1200); // 1200ms after step 17 => 1000ms after step 18 @sic R5s240283 sic@ v_RLC_SDU1 := fl_TC_7_1_1_8_1_SendData(p_NR_CellId, p_DrbId, cs_TimingInfo_NR(v_TimingInfoDL), v_SN); // @siclog "Step 21" siclog@ v_TimingInfoUL := f_NR_GetNextSendOccasion_ULafterDL(p_NR_CellId, v_TimingInfoDL, 100); // 100ms after step 20 f_NR_OneULGrantTransmission(p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoUL), p_ResourceAllocationUL); // @siclog "Steps 22-22A" siclog@ fl_TC_7_1_1_8_1_ReceiveData(p_NR_CellId, p_DrbId, {v_RLC_SDU1}, true, v_SN); //@sic R5-231578 R5s230220 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 22"); //DL and UL BWP already configured on tsc_NR_BWP_Id_2/tsc_NR_BWP_Id_Initial @sic R5s230733 sic@ }