3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240459

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3483 | rev | - | Current version: | 17.11.0 |  |
| For HELPon using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed changeaffects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcase 6.4.2.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Starpoint |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-07-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | After msg4 sended, SS begin to use DCI1_1. But DCI1_1 can not be detected by UE as UE is in inactive state. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Set DCI1-0 in SS before sending RRCrelease message in step4,step8 and step12. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test cases. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.2.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

- Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D24wk24 related to the title of this CR.

Contact:	Ting Gao
	gaoting@starpointcomm.com

- Corrections required

  - Change 1

| Function name | f_TC_6_4_2_2_NR5GC_TestBody() |
| --- | --- |
| Reason for change | After msg4 sended, SS begin to use DCI1_1. But DCI1_1 can not be detected by UE as UE is in inactive state. |
| Summary of change | Set DCI1-0 in SS before sending RRCrelease message in step4,step8 and step12. |
| TTCN module | Inactive_CellReSelection_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_6_4_2_2_NR5GC_TestBody() runs on NR5GC_PTC { // @sic R5s200403 R5-203502 R5-233462 sic@ var integer v_T1_Cell3_RS_EPRE_FR1 := -78; // @sic R5-198879 sic@ var integer v_T2_Cell1_RS_EPRE_FR1 := tsc_NR_NonSuitableOffCellSSS_EPRE; var integer v_T2_Cell6_RS_EPRE_FR1 := -78; var integer v_T3_Cell6_RS_EPRE_FR1 := -98; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT1; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT2; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT3; v_CellPowerList_AtT1:={ cs_NR_CellPower(nr_Cell3, v_T1_Cell3_RS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2) }; v_CellPowerList_AtT2:={ cs_NR_CellPower(nr_Cell1, v_T2_Cell1_RS_EPRE_FR1, v_T2_Cell1_RS_EPRE_FR1), cs_NR_CellPower(nr_Cell6, v_T2_Cell6_RS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2) }; v_CellPowerList_AtT3:={ cs_NR_CellPower(nr_Cell6, v_T3_Cell6_RS_EPRE_FR1, tsc_NR_NonSuitableCellSSS_EPRE_FR2) }; // @siclog "Step 1" siclog@ // The SS changes NR Cell 3 SSS level according to the row "T1" in table 6.4.2.2.3.2-1/2. f_NR_SetCellPowerList(v_CellPowerList_AtT1); f_NR_SS_AS_ActivateSecurity_Current(nr_Cell3); // @sic R5s220486 sic@ // @siclog "Step 2 - Void" siclog@ // @siclog "Step 3" siclog@ // Check: Does the UE transmit an RRCResumeRequest message on the cell 3? SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell3, cr_38508_RRCResumeRequest(tsc_NR_ShortI_RNTI_Value1, rna_Update))); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 3"); // @siclog "Step 4" siclog@ // The SS transmits an RRCRelease message including suspendConfig with NR_RRC_INACTIVE condition. f_NR_RRC_InactiveDef(nr_Cell3, -, false); // @siclog "Step 5" siclog@ // The SS changes NR Cell 1 and NR Cell 6 SSS levels according to the row "T2" in table 6.4.2.2.3.2-1/2. f_NR_SetCellPowerList(v_CellPowerList_AtT2); f_NR_SS_AS_ActivateSecurity_Current(nr_Cell6); // @sic R5s220486 sic@ // @siclog "Step 6 - Void" siclog@ // @siclog "Step 7" siclog@ // Check: Does the UE transmit an RRCResumeRequest message on the cell 6? SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell6, cr_38508_RRCResumeRequest(tsc_NR_ShortI_RNTI_Value1, rna_Update))); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 6"); // @siclog "Step 8" siclog@ // The SS transmits an RRCRelease message including suspendConfig with NR_RRC_INACTIVE condition. f_NR_RRC_InactiveDef(nr_Cell6,-,false); f_NR_SS_AS_ActivateSecurity_Current(nr_Cell3); // @siclog "Step 9" siclog@ // The SS changes NR Cell 6 SSS level according to the row "T3" in table 6.4.2.2.3.2-1/2. f_NR_SetCellPowerList(v_CellPowerList_AtT3); // @siclog "Step 10 - Void" siclog@ // @siclog "Step 11" siclog@ // Check: Does the UE transmit an RRCResumeRequest message on the cell 3? SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell3, cr_38508_RRCResumeRequest(tsc_NR_ShortI_RNTI_Value1, rna_Update))); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 11"); // @siclog "Step 12" siclog@ // The SS transmits an RRCRelease message including suspendConfig with NR_RRC_INACTIVE condition. f_NR_RRC_InactiveDef(nr_Cell3); }//end of f_TC_6_4_2_2_NR5GC_TestBody

After Change:

function f_TC_6_4_2_2_NR5GC_TestBody() runs on NR5GC_PTC { // @sic R5s200403 R5-203502 R5-233462 sic@ var integer v_T1_Cell3_RS_EPRE_FR1 := -78; // @sic R5-198879 sic@ var integer v_T2_Cell1_RS_EPRE_FR1 := tsc_NR_NonSuitableOffCellSSS_EPRE; var integer v_T2_Cell6_RS_EPRE_FR1 := -78; var integer v_T3_Cell6_RS_EPRE_FR1 := -98; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT1; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT2; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT3; var NR_PhysicalParameters_Type v_NR_PhysicalParameters; v_CellPowerList_AtT1:={ cs_NR_CellPower(nr_Cell3, v_T1_Cell3_RS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2) }; v_CellPowerList_AtT2:={ cs_NR_CellPower(nr_Cell1, v_T2_Cell1_RS_EPRE_FR1, v_T2_Cell1_RS_EPRE_FR1), cs_NR_CellPower(nr_Cell6, v_T2_Cell6_RS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2) }; v_CellPowerList_AtT3:={ cs_NR_CellPower(nr_Cell6, v_T3_Cell6_RS_EPRE_FR1, tsc_NR_NonSuitableCellSSS_EPRE_FR2) }; // @siclog "Step 1" siclog@ // The SS changes NR Cell 3 SSS level according to the row "T1" in table 6.4.2.2.3.2-1/2. f_NR_SetCellPowerList(v_CellPowerList_AtT1); f_NR_SS_AS_ActivateSecurity_Current(nr_Cell3); // @sic R5s220486 sic@ // @siclog "Step 2 - Void" siclog@ // @siclog "Step 3" siclog@ // Check: Does the UE transmit an RRCResumeRequest message on the cell 3? SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell3, cr_38508_RRCResumeRequest(tsc_NR_ShortI_RNTI_Value1, rna_Update))); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 3"); v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(nr_Cell3); f_NR_SS_CommonCellConfig(nr_Cell3, cads_NR_RachProcedure_DcchDtchToCSS_Config_REQ(nr_Cell3, cs_TimingInfo_Now, tsc_CnfReq, -, cs_NR_SearchSpaceDlDciAssignment_CSSType1(v_NR_PhysicalParameters) )); // @siclog "Step 4" siclog@ // The SS transmits an RRCRelease message including suspendConfig with NR_RRC_INACTIVE condition. f_NR_RRC_InactiveDef(nr_Cell3, -, false); // @siclog "Step 5" siclog@ // The SS changes NR Cell 1 and NR Cell 6 SSS levels according to the row "T2" in table 6.4.2.2.3.2-1/2. f_NR_SetCellPowerList(v_CellPowerList_AtT2); f_NR_SS_AS_ActivateSecurity_Current(nr_Cell6); // @sic R5s220486 sic@ // @siclog "Step 6 - Void" siclog@ // @siclog "Step 7" siclog@ // Check: Does the UE transmit an RRCResumeRequest message on the cell 6? SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell6, cr_38508_RRCResumeRequest(tsc_NR_ShortI_RNTI_Value1, rna_Update))); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 6"); v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(nr_Cell6); f_NR_SS_CommonCellConfig(nr_Cell6, cads_NR_RachProcedure_DcchDtchToCSS_Config_REQ(nr_Cell6, cs_TimingInfo_Now, tsc_CnfReq, -, cs_NR_SearchSpaceDlDciAssignment_CSSType1(v_NR_PhysicalParameters) )); // @siclog "Step 8" siclog@ // The SS transmits an RRCRelease message including suspendConfig with NR_RRC_INACTIVE condition. f_NR_RRC_InactiveDef(nr_Cell6,-,false); f_NR_SS_AS_ActivateSecurity_Current(nr_Cell3); // @siclog "Step 9" siclog@ // The SS changes NR Cell 6 SSS level according to the row "T3" in table 6.4.2.2.3.2-1/2. f_NR_SetCellPowerList(v_CellPowerList_AtT3); // @siclog "Step 10 - Void" siclog@ // @siclog "Step 11" siclog@ // Check: Does the UE transmit an RRCResumeRequest message on the cell 3? SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell3, cr_38508_RRCResumeRequest(tsc_NR_ShortI_RNTI_Value1, rna_Update))); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 11"); v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(nr_Cell3); f_NR_SS_CommonCellConfig(nr_Cell3, cads_NR_RachProcedure_DcchDtchToCSS_Config_REQ(nr_Cell3, cs_TimingInfo_Now, tsc_CnfReq, -, cs_NR_SearchSpaceDlDciAssignment_CSSType1(v_NR_PhysicalParameters) )); // @siclog "Step 12" siclog@ // The SS transmits an RRCRelease message including suspendConfig with NR_RRC_INACTIVE condition. f_NR_RRC_InactiveDef(nr_Cell3); }//end of f_TC_6_4_2_2_NR5GC_TestBody