3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240465

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4842 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NBIOT testcase 22.4.21 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI13_Test, NB_IOT-UEConTest |  |  |  |  |  | Date: |  |  | 2024-07-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The current TTCN implementation for this testcase is for USER_PLANE state. As a result, with current TTCN implementation, Step 10a1 of 36.508 Table 8.1.5.2.3-1 gets executed even if condition S1_U_DataTransfer is set as FALSE. This is incorrect. Also, the 36.523-2 Table 4.1 does not indicate any dependence on USER_PLANE for this testcase. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Instead of using the state USER_PLANE, used CONTROL_PLANE instead. Please see below. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A Conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.4.21 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

2.1	Change 1	2

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D24wk24 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change 1

| Function name | f_TC_22_4_21_NBIOT |
| --- | --- |
| Reason for change | The current TTCN implementation for this testcase is for USER_PLANE state. As a result, with current TTCN implementation, Step 10a1 of 36.508 Table 8.1.5.2.3-1 gets executed even if condition S1_U_DataTransfer is set as FALSE. This is incorrect. Also, the 36.523-2 Table 4.1 does not indicate any dependence on USER_PLANE for this testcase. |
| Summary of change | Instead of using the state USER_PLANE, used CONTROL_PLANE instead. Please see below. |
| TTCN module | MBIOT_RRC_UserPlane.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_22_4_21_NBIOT() runs on NBIOT_PTC { var SystemInformationBlockType2_NB_r13 v_SIB2; //@sic R5-235472, R5-235331, R5-237126 sic@ f_NBIOT_Init(c1, USER_PLANE); //Set the value of T310 timer in SIB2 v_SIB2 := f_NBIOT_CellInfo_GetSIB2(nbiot_Cell1); v_SIB2.ue_TimersAndConstants_r13.t310_r13 := ms2000; f_NBIOT_CellInfo_SetSIB2(nbiot_Cell1, v_SIB2); //Create and configure all cells f_NBIOT_CellConfig_Def(nbiot_Cell1, USER_PLANE); //Bring UE to initial state f_NBIOT_Preamble(nbiot_Cell1, USER_PLANE); f_NBIOT_TestBody_Set(true); //@siclog "Step 1" siclog@ //The SS changes Ncell 1 levels according to row "T1" in Table 22.4.21.3.2-1. f_NBIOT_SetCellPower(nbiot_Cell1, tsc_NonSuitableOffNBIOTCellRS_EPRE); //@siclog "Step 2" siclog@ //Wait for 1.5 s. f_Delay(1.5); //@siclog "Step 3" siclog@ //The SS changes Ncell 1 levels according to row "T0" in Table 22.4.21.3.2-1. f_NBIOT_SetCellPower(nbiot_Cell1, -85); //@siclog "Step 4" siclog@ //Wait for 5s. // If the UE transmits any signalling message, this will be picked up in the Default Behaviour f_Delay(5.0); //@siclog "Step 5" siclog@ f_NBIOT_Capability(nbiot_Cell1, USER_PLANE); f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 5"); f_NBIOT_TestBody_Set(false); //Switch/power off UE f_NBIOT_Postamble(nbiot_Cell1, USER_PLANE, N2_CONNECTED); }

After Change:

function f_TC_22_4_21_NBIOT() runs on NBIOT_PTC { var SystemInformationBlockType2_NB_r13 v_SIB2; //@sic R5-235472, R5-235331, R5-237126 sic@ f_NBIOT_Init(c1, CONTROL_PLANE); //WA#22_4_21 USER_PLANE --> CONTROL_PLANE //Set the value of T310 timer in SIB2 v_SIB2 := f_NBIOT_CellInfo_GetSIB2(nbiot_Cell1); v_SIB2.ue_TimersAndConstants_r13.t310_r13 := ms2000; f_NBIOT_CellInfo_SetSIB2(nbiot_Cell1, v_SIB2); //Create and configure all cells f_NBIOT_CellConfig_Def(nbiot_Cell1, CONTROL_PLANE); //WA#22_4_21 USER_PLANE --> CONTROL_PLANE //Bring UE to initial state f_NBIOT_Preamble(nbiot_Cell1, CONTROL_PLANE); //WA#22_4_21 USER_PLANE --> CONTROL_PLANE f_NBIOT_TestBody_Set(true); //@siclog "Step 1" siclog@ //The SS changes Ncell 1 levels according to row "T1" in Table 22.4.21.3.2-1. f_NBIOT_SetCellPower(nbiot_Cell1, tsc_NonSuitableOffNBIOTCellRS_EPRE); //@siclog "Step 2" siclog@ //Wait for 1.5 s. f_Delay(1.5); //@siclog "Step 3" siclog@ //The SS changes Ncell 1 levels according to row "T0" in Table 22.4.21.3.2-1. f_NBIOT_SetCellPower(nbiot_Cell1, -85); //@siclog "Step 4" siclog@ //Wait for 5s. // If the UE transmits any signalling message, this will be picked up in the Default Behaviour f_Delay(5.0); //@siclog "Step 5" siclog@ f_NBIOT_Capability(nbiot_Cell1, CONTROL_PLANE); //WA#22_4_21 USER_PLANE --> CONTROL_PLANE f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 5"); f_NBIOT_TestBody_Set(false); //Switch/power off UE f_NBIOT_Postamble(nbiot_Cell1, CONTROL_PLANE, N2_CONNECTED); //WA#22_4_21 USER_PLANE --> CONTROL_PLANE }