3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240466

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4843 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function f_TC_13_4_3_2_UTRAN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2024-07-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the current TTCN implementation of the function f_TC_13_4_3_2_UTRAN, after Step 31 an Alt mechanism is implemented to handle the optional Deactivate PDP Context REQ/ACC and the coordination messge received from IMS PTC. The execution exits the Alt as soon as the coordination message is received, but has a “repeat” if the UE sends the Deactivate PDP Context before the coordination message is received. However, if a UE is slightly slower, then a situation occurs when the IMS PTC has sent the coordination message, and then the UE transmits the Deactivate PDP Context REQ. In such a situation, the testcase fails, because the Deactivate PDP Context REQ is unhandled. This needs to be accommodated. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to change the implementation of the “Alt” mechanism, and make it timer based. Please see below. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail the test cases |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 13.4.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

2.1	Change 1	2

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D24wk24 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change 1

| Function name | f_TC_13_4_3_2_UTRAN |
| --- | --- |
| Reason for change | In the current TTCN implementation of the function f_TC_13_4_3_2_UTRAN, after Step 31 an Alt mechanism is implemented to handle the optional Deactivate PDP Context REQ/ACC and the coordination messge received from IMS PTC. The execution exits the Alt as soon as the coordination message is received, but has a “repeat” if the UE sends the Deactivate PDP Context before the coordination message is received. However, if a UE is slightly slower, then a situation occurs when the IMS PTC has sent the coordination message, and then the UE transmits the Deactivate PDP Context REQ. In such a situation, the testcase fails, because the Deactivate PDP Context REQ is unhandled. This needs to be accommodated. |
| Summary of change | It is proposed to change the implementation of the “Alt” mechanism, and make it timer based. Please see below. |
| TTCN module | SRVCC_EUTRA_UTRAN.ttcn |
| MCC160 Comments |  |

Before Change:

function f_TC_13_4_3_2_UTRAN() runs on UTRAN_IRAT_PTC { /* Inter-system mobility / E-UTRA PS voice + PS data to UTRA CS voice + PS data / SRVCC */ //@sic R5-120667 sic@ var integer v_T0_CPICH_Cell5; var integer v_T1_CPICH_Cell5; var integer v_T0_PCCPCH_Cell5; var integer v_T1_PCCPCH_Cell5; v_T0_CPICH_Cell5 := -88; v_T1_CPICH_Cell5 := -64;//@sic R5-125742 sic@ v_T0_PCCPCH_Cell5 := -88; v_T1_PCCPCH_Cell5 := -64;//@sic R5-125742 sic@ // Initialise all cells f_UTRAN_Init(EUTRA_UTRAN); // Create and configure Cell 5 // Cell started switched on with default power level, -60 and max attenuation f_UTRAN_SS_CreateCellDCH(utran_Cell5); // Start sending System Information f_UTRAN_SendDefSysInfo(utran_Cell5); // Send relevant UTRAN cell info to EUTRA PTC f_UTRAN_Send_IR_SysInfoToEUTRA(utran_Cell5); f_UTRAN_TestBody_Set(true); //@siclog "Step 1" siclog@ //Configure a dedicated channel in the UTRAN FDD with configuration // according to 34.108 cl. f_SS_Config_ConvSpeech_64kPS_RAB(utran_Cell5); // wait for trigger from EUTRAN side f_IRAT_WaitForCoOrd_Trigger(EUTRA); // Set Cell Power level as per T0 f_UTRAN_SetCellPower(utran_Cell5, v_T0_CPICH_Cell5, v_T0_PCCPCH_Cell5); f_IRAT_WaitForCoOrd_Trigger(EUTRA); //@siclog "Step 31" siclog@ // Set Cell Power level as per T1 f_UTRAN_SetCellPower(utran_Cell5, v_T1_CPICH_Cell5, v_T1_PCCPCH_Cell5); // UE starts registered in EUTRA, wait its Auth Params f_UTRAN_InterRAT_InitialiseAuthParams(); // Set up the ciphering ready for when the UE hands over f_UTRAN_ConfigureCipheringBeforeHO_SRVCC(utran_Cell5); f_IRAT_SendCoOrd(EUTRA, cms_IRAT_Trigger); // Tell SS to transmit a MobilityFromEUTRACommand message f_IRAT_WaitForCoOrd_Trigger(EUTRA); // @sic R5s160330 sic@ f_UTRAN_IP_Handling_Change_Configuration (utran_CellDedicated, cell_DCH_64kPS_RAB_SRB); // @sic R5s140572, R5s140161 sic@ //according to 36.508 cl. 6.4.3.9.5, f_UTRAN_SRVCC_WithHandover(utran_Cell5, cs_PDP_ContextStatusNSAPI5, true, '0'B); //@sic R5s140096, R5-144667, R5-150397 sic@ alt { // @sic R5s170028 sic@ [] a_UTRAN_DeactivatePDPContext_MO () { repeat; } [] IMS[tsc_Index_IMS1].receive(cmr_IMS_IPCAN_Trigger){ // @sic R5s170198 sic@ // Ok, we can stop waiting now } } f_IRAT_SendCoOrd(EUTRA, cms_IRAT_Trigger); // Tell to EUTRA PTC Test case is now finished on E-UTRA side f_UTRAN_TestBody_Set(false); // Switch off UE and release cell f_UTRAN_Postamble(utran_Cell5, U5_CSCALL,-,cs_TI_MT); }

After Change:

| function f_TC_13_4_3_2_UTRAN() runs on UTRAN_IRAT_PTC { /* Inter-system mobility / E-UTRA PS voice + PS data to UTRA CS voice + PS data / SRVCC */ //@sic R5-120667 sic@ var integer v_T0_CPICH_Cell5; var integer v_T1_CPICH_Cell5; var integer v_T0_PCCPCH_Cell5; var integer v_T1_PCCPCH_Cell5; timer t_Timer := 5.0; v_T0_CPICH_Cell5 := -88; v_T1_CPICH_Cell5 := -64;//@sic R5-125742 sic@ v_T0_PCCPCH_Cell5 := -88; v_T1_PCCPCH_Cell5 := -64;//@sic R5-125742 sic@ // Initialise all cells f_UTRAN_Init(EUTRA_UTRAN); // Create and configure Cell 5 // Cell started switched on with default power level, -60 and max attenuation f_UTRAN_SS_CreateCellDCH(utran_Cell5); // Start sending System Information f_UTRAN_SendDefSysInfo(utran_Cell5); // Send relevant UTRAN cell info to EUTRA PTC f_UTRAN_Send_IR_SysInfoToEUTRA(utran_Cell5); f_UTRAN_TestBody_Set(true); //@siclog "Step 1" siclog@ //Configure a dedicated channel in the UTRAN FDD with configuration // according to 34.108 cl. f_SS_Config_ConvSpeech_64kPS_RAB(utran_Cell5); // wait for trigger from EUTRAN side f_IRAT_WaitForCoOrd_Trigger(EUTRA); // Set Cell Power level as per T0 f_UTRAN_SetCellPower(utran_Cell5, v_T0_CPICH_Cell5, v_T0_PCCPCH_Cell5); f_IRAT_WaitForCoOrd_Trigger(EUTRA); //@siclog "Step 31" siclog@ // Set Cell Power level as per T1 f_UTRAN_SetCellPower(utran_Cell5, v_T1_CPICH_Cell5, v_T1_PCCPCH_Cell5); // UE starts registered in EUTRA, wait its Auth Params f_UTRAN_InterRAT_InitialiseAuthParams(); // Set up the ciphering ready for when the UE hands over f_UTRAN_ConfigureCipheringBeforeHO_SRVCC(utran_Cell5); f_IRAT_SendCoOrd(EUTRA, cms_IRAT_Trigger); // Tell SS to transmit a MobilityFromEUTRACommand message f_IRAT_WaitForCoOrd_Trigger(EUTRA); // @sic R5s160330 sic@ f_UTRAN_IP_Handling_Change_Configuration (utran_CellDedicated, cell_DCH_64kPS_RAB_SRB); // @sic R5s140572, R5s140161 sic@ //according to 36.508 cl. 6.4.3.9.5, f_UTRAN_SRVCC_WithHandover(utran_Cell5, cs_PDP_ContextStatusNSAPI5, true, '0'B); //@sic R5s140096, R5-144667, R5-150397 sic@ t_Timer.start alt { // @sic R5s170028 sic@ [] a_UTRAN_DeactivatePDPContext_MO () { repeat; } [] IMS[tsc_Index_IMS1].receive(cmr_IMS_IPCAN_Trigger){ // @sic R5s170198 sic@ // Ok, we can stop waiting now repeat; } []t_Timer.timeout{ } } f_IRAT_SendCoOrd(EUTRA, cms_IRAT_Trigger); // Tell to EUTRA PTC Test case is now finished on E-UTRA side f_UTRAN_TestBody_Set(false); // Switch off UE and release cell f_UTRAN_Postamble(utran_Cell5, U5_CSCALL,-,cs_TI_MT); } |  |
| --- | --- |