3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240468

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0951 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to implementation of MTSI Conference Creation/5GS procedure for test 8.34 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-07-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To correct the implementation of MTSI conference creation / 5GS procedure as specified in 34.229-5 Annex A.19. Correcting implementation of optional behavior/steps. #draft ProseCR would be raised during RAN5#104 meeting |  |  |  |  |  |  |  |  |
| Summary of change: |  | The current implementation of the function for MTSI Conference Creation/5GS has optional steps 10-13 (reception of SUBSCRIBE for the conference event) implemented as mandatory. Therefore, the affected function was corrected such that both cases wherein UE follows this optional procedure and for a UE which does not, the test case will still proceed as expected. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.34.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS34.229-5 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D24wk24 related to the title of this CR.

Contact:	Kalahasti, Narendra

Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | function f_IMS_ThreeWaySession_SubscribeInvite() |
| --- | --- |
| Reason for change | To correct the implementation of MTSI conference creation / 5GS procedure as specified in 34.229-5 Annex A.19. Correcting implementation of optional behavior/steps. #draft ProseCR would be raised during RAN5#104 meeting |
| Summary of change | Modified function f_IMS_ThreeWaySession_SubscribeInvite to allow reception of SUBSCRIBE as part of optional Steps 10-13 of procedure from Annex A.19 of 34.229-5 at any time or not at all. |
| TTCN module | IMS_Procedures_Conferencing.ttcn |
| MCC160 Comment |  |

Before Change

function f_IMS_ThreeWaySession_SubscribeInvite(ThreeWaySession_SessionInfo_Type p_SessionInfo_Dialog1, ThreeWaySession_SessionInfo_Type p_SessionInfo_Dialog2, InviteUserToConference_StateInfo_Type p_Dialog1_StateInfo, InviteUserToConference_StateInfo_Type p_Dialog2_StateInfo, charstring p_FinalConferenceUri, CharStringList_Type p_MediaTypeList) runs on IMS_PTC return template (omit) SUBSCRIBE_Request { /* @sic R5-214877: p_MediaTypeList sic@ */ var ConferenceEventPackage_SubscribeInfo_Type v_ConferenceEventPackage_SubscribeInfo_ByRef := valueof(cs_ConferenceEventPackage_SubscribeInfo_Init); var InviteUserToConference_StateInfo_Type v_Dialog1_StateInfo_ByRef := p_Dialog1_StateInfo; var InviteUserToConference_StateInfo_Type v_Dialog2_StateInfo_ByRef := p_Dialog2_StateInfo; var template (present) MessageHeader v_MessageHeader_Bye_Dialog1 := f_IMS_ByeRequest_MessageHeaderRX(p_SessionInfo_Dialog1.InviteRequestWithSdp.Invite, -, firstDialog); var template (present) MessageHeader v_MessageHeader_Bye_Dialog2 := f_IMS_ByeRequest_MessageHeaderRX(p_SessionInfo_Dialog2.InviteRequestWithSdp.Invite, -, secondDialog); var template (omit) SUBSCRIBE_Request v_SubscribeRequest := omit; var boolean v_WaitForBYE_Dialog1 := false; var boolean v_WaitForBYE_Dialog2 := false; var boolean v_Dialog1_Terminated := false; var boolean v_Dialog2_Terminated := false; var boolean v_Continue; var integer v_ConferenceEventPackage_Version := 1; // version used in notifications sent to the UE to inform about remote user has joined the conference timer t_WaitForSubscribe := tsc_WaitForConferenceSubscription; // @sic R5s221272: 3s according to 34.229-1 C.10, 34.229-5 A.19 sic@ t_WaitForSubscribe.start; do { alt { // 34.229-1 C.10/38 Step 10 - 13, 34.229-5 A.19/25 Step 10 - 13 (UE subscribes the conference event) [v_ConferenceEventPackage_SubscribeInfo_ByRef.State != subscribed] a_ConferenceEventPackage_Subscribe(v_ConferenceEventPackage_SubscribeInfo_ByRef, p_FinalConferenceUri) { v_SubscribeRequest := v_ConferenceEventPackage_SubscribeInfo_ByRef.SubscribeRequest; t_WaitForSubscribe.stop; } // 34.229-1 C.19/37, 34.229-5 A.20/26 (Invite user A) [v_Dialog1_StateInfo_ByRef.State != finished] a_IMS_InviteUserToConference(v_Dialog1_StateInfo_ByRef, p_FinalConferenceUri, firstDialog, // @sic R5s150909: dialog index instead of boolean sic@ v_SubscribeRequest, p_MediaTypeList, // @sic R5-214877: p_MediaTypeList instead of tsc_MediaTypeList_Audio sic@ v_ConferenceEventPackage_Version) { v_WaitForBYE_Dialog1 := fl_IMS_ThreeWaySession_StateInfo_WaitForBYE(v_Dialog1_StateInfo_ByRef) and not v_Dialog1_Terminated; /* determine whether to wait for BYE for dialog 1; @sic R5s150910: v_Dialog1_Terminated sic@ */ if (v_Dialog1_StateInfo_ByRef.State == waitFor200OK_Step8) { /* => when then UE has subscribed to the conference event package it gets notify that user 1 has joined the conference ... */ v_ConferenceEventPackage_Version := v_ConferenceEventPackage_Version + 1; /* ... and the version needs to incremented for next notification */ } } // 34.229-1 C.19/37, 34.229-5 A.20/26 (Invite user A) [v_Dialog2_StateInfo_ByRef.State != finished] a_IMS_InviteUserToConference(v_Dialog2_StateInfo_ByRef, p_FinalConferenceUri, secondDialog, // @sic R5s150909: dialog index instead of boolean sic@ v_SubscribeRequest, p_MediaTypeList, // @sic R5-214877: p_MediaTypeList instead of tsc_MediaTypeList_Audio sic@ v_ConferenceEventPackage_Version) { v_WaitForBYE_Dialog2 := fl_IMS_ThreeWaySession_StateInfo_WaitForBYE(v_Dialog2_StateInfo_ByRef) and not v_Dialog2_Terminated; /* determine whether to wait for BYE for dialog 2; @sic R5s150910: v_Dialog2_Terminated sic@ */ if (v_Dialog2_StateInfo_ByRef.State == waitFor200OK_Step8) { /* => when then UE has subscribed to the conference event package it gets notify that user 2 has joined the conference ... */ v_ConferenceEventPackage_Version := v_ConferenceEventPackage_Version + 1; /* ... and the version needs to incremented for next notification */ } } //-------------------------------------------- // DIALOG 1 //-------------------------------------------- // Terminate dialog 1 [v_WaitForBYE_Dialog1] a_IMS_ReceiveByeSendOK(firstDialog, p_SessionInfo_Dialog1.ContactUrl, v_MessageHeader_Bye_Dialog1) { v_WaitForBYE_Dialog1 := false; v_Dialog1_Terminated := true; /* @sic R5s150910 sic@ */ } //-------------------------------------------- // DIALOG 2 //-------------------------------------------- // Terminate dialog 2 [v_WaitForBYE_Dialog2] a_IMS_ReceiveByeSendOK(secondDialog, p_SessionInfo_Dialog2.ContactUrl, v_MessageHeader_Bye_Dialog2) { v_WaitForBYE_Dialog2 := false; v_Dialog2_Terminated := true; /* @sic R5s150910 sic@ */ } [] t_WaitForSubscribe.timeout {} /* @sic R5s230598 sic@ */ } v_Continue := v_WaitForBYE_Dialog1 or v_WaitForBYE_Dialog2 or (v_Dialog1_StateInfo_ByRef.State != finished) or (v_Dialog2_StateInfo_ByRef.State != finished) or (isvalue(v_SubscribeRequest) and (v_ConferenceEventPackage_SubscribeInfo_ByRef.State != subscribed)) // isvalue(v_SubscribeRequest): UE has sent the (optional) SUBSCRIBE or t_WaitForSubscribe.running; // @sic R5s221272: t_WaitForSubscribe sic@ } while (v_Continue); return v_SubscribeRequest; }

After Change

function f_IMS_ThreeWaySession_SubscribeInvite(ThreeWaySession_SessionInfo_Type p_SessionInfo_Dialog1, ThreeWaySession_SessionInfo_Type p_SessionInfo_Dialog2, InviteUserToConference_StateInfo_Type p_Dialog1_StateInfo, InviteUserToConference_StateInfo_Type p_Dialog2_StateInfo, charstring p_FinalConferenceUri, CharStringList_Type p_MediaTypeList) runs on IMS_PTC return template (omit) SUBSCRIBE_Request { /* @sic R5-214877: p_MediaTypeList sic@ */ var ConferenceEventPackage_SubscribeInfo_Type v_ConferenceEventPackage_SubscribeInfo_ByRef := valueof(cs_ConferenceEventPackage_SubscribeInfo_Init); var InviteUserToConference_StateInfo_Type v_Dialog1_StateInfo_ByRef := p_Dialog1_StateInfo; var InviteUserToConference_StateInfo_Type v_Dialog2_StateInfo_ByRef := p_Dialog2_StateInfo; var template (present) MessageHeader v_MessageHeader_Bye_Dialog1 := f_IMS_ByeRequest_MessageHeaderRX(p_SessionInfo_Dialog1.InviteRequestWithSdp.Invite, -, firstDialog); var template (present) MessageHeader v_MessageHeader_Bye_Dialog2 := f_IMS_ByeRequest_MessageHeaderRX(p_SessionInfo_Dialog2.InviteRequestWithSdp.Invite, -, secondDialog); var template (omit) SUBSCRIBE_Request v_SubscribeRequest := omit; var boolean v_WaitForBYE_Dialog1 := false; var boolean v_WaitForBYE_Dialog2 := false; var boolean v_Dialog1_Terminated := false; var boolean v_Dialog2_Terminated := false; var boolean v_Continue; var integer v_ConferenceEventPackage_Version := 1; // version used in notifications sent to the UE to inform about remote user has joined the conference timer t_WaitForSubscribe := tsc_WaitForConferenceSubscription; // @sic R5s221272: 3s according to 34.229-1 C.10, 34.229-5 A.19 sic@ t_WaitForSubscribe.start; do { alt { // 34.229-1 C.19/37, 34.229-5 A.20/26 (Invite user A) [v_Dialog1_StateInfo_ByRef.State != finished] a_IMS_InviteUserToConference(v_Dialog1_StateInfo_ByRef, p_FinalConferenceUri, firstDialog, // @sic R5s150909: dialog index instead of boolean sic@ v_SubscribeRequest, p_MediaTypeList, // @sic R5-214877: p_MediaTypeList instead of tsc_MediaTypeList_Audio sic@ v_ConferenceEventPackage_Version) { v_WaitForBYE_Dialog1 := fl_IMS_ThreeWaySession_StateInfo_WaitForBYE(v_Dialog1_StateInfo_ByRef) and not v_Dialog1_Terminated; /* determine whether to wait for BYE for dialog 1; @sic R5s150910: v_Dialog1_Terminated sic@ */ if (v_Dialog1_StateInfo_ByRef.State == waitFor200OK_Step8) { /* => when then UE has subscribed to the conference event package it gets notify that user 1 has joined the conference ... */ v_ConferenceEventPackage_Version := v_ConferenceEventPackage_Version + 1; /* ... and the version needs to incremented for next notification */ } } // 34.229-1 C.19/37, 34.229-5 A.20/26 (Invite user A) [v_Dialog2_StateInfo_ByRef.State != finished] a_IMS_InviteUserToConference(v_Dialog2_StateInfo_ByRef, p_FinalConferenceUri, secondDialog, // @sic R5s150909: dialog index instead of boolean sic@ v_SubscribeRequest, p_MediaTypeList, // @sic R5-214877: p_MediaTypeList instead of tsc_MediaTypeList_Audio sic@ v_ConferenceEventPackage_Version) { v_WaitForBYE_Dialog2 := fl_IMS_ThreeWaySession_StateInfo_WaitForBYE(v_Dialog2_StateInfo_ByRef) and not v_Dialog2_Terminated; /* determine whether to wait for BYE for dialog 2; @sic R5s150910: v_Dialog2_Terminated sic@ */ if (v_Dialog2_StateInfo_ByRef.State == waitFor200OK_Step8) { /* => when then UE has subscribed to the conference event package it gets notify that user 2 has joined the conference ... */ v_ConferenceEventPackage_Version := v_ConferenceEventPackage_Version + 1; /* ... and the version needs to incremented for next notification */ } } //-------------------------------------------- // DIALOG 1 //-------------------------------------------- // Terminate dialog 1 [v_WaitForBYE_Dialog1] a_IMS_ReceiveByeSendOK(firstDialog, p_SessionInfo_Dialog1.ContactUrl, v_MessageHeader_Bye_Dialog1) { v_WaitForBYE_Dialog1 := false; v_Dialog1_Terminated := true; /* @sic R5s150910 sic@ */ } //-------------------------------------------- // DIALOG 2 //-------------------------------------------- // Terminate dialog 2 [v_WaitForBYE_Dialog2] a_IMS_ReceiveByeSendOK(secondDialog, p_SessionInfo_Dialog2.ContactUrl, v_MessageHeader_Bye_Dialog2) { v_WaitForBYE_Dialog2 := false; v_Dialog2_Terminated := true; /* @sic R5s150910 sic@ */ } // 34.229-1 C.10/38 Step 10 - 13, 34.229-5 A.19/25 Step 10 - 13 (UE subscribes the conference event) [] a_ConferenceEventPackage_Subscribe(v_ConferenceEventPackage_SubscribeInfo_ByRef, p_FinalConferenceUri) { v_SubscribeRequest := v_ConferenceEventPackage_SubscribeInfo_ByRef.SubscribeRequest; t_WaitForSubscribe.stop; } [] t_WaitForSubscribe.timeout {} /* @sic R5s230598 sic@ */ } v_Continue := v_WaitForBYE_Dialog1 or v_WaitForBYE_Dialog2 or (v_Dialog1_StateInfo_ByRef.State != finished) or (v_Dialog2_StateInfo_ByRef.State != finished) or (isvalue(v_SubscribeRequest) and (v_ConferenceEventPackage_SubscribeInfo_ByRef.State != subscribed)) // isvalue(v_SubscribeRequest): UE has sent the (optional) SUBSCRIBE or t_WaitForSubscribe.running; // @sic R5s221272: t_WaitForSubscribe sic@ } while (v_Continue); return v_SubscribeRequest; }