3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240469

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4844 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NBIOT testcase 22.5.20 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, NB_IOTenh-UEConTest |  |  |  |  |  | Date: |  |  | 2024-07-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the current TTCN implementation, at Step 11 the SS transmits the rrcRelese with re-direction to frequency of Ncell 1 alongwith extended wait time set to 30s. However, at this time the Ncell 1 is not set to serving cell power, and a FAST UE can retry to camp on Ncell 23 again by sending a rrcConnectionREQ. This causes TC failure and needs to be addressed. R5s230342 was initially raised for this, but still the instability remains. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Before sending the rrcConnection Release on Ncell23, called the function f_NBIOT_SS_ConfigRachProcedureWithNoMsg4(nbiot_Cell23); This ensures that the UE never gets RAR, thus it cannot send rrcConnectionREQ on Ncell 23. Once the power level change is effective and the UE detects that, it moves to Ncell 1 as expected by the testcase. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE will fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.5.20 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

2.1	Change 1	2

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D24wk24 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change 1

| Function name | fl_TC_22_5_20_Body |
| --- | --- |
| Reason for change | In the current TTCN implementation, at Step 11 the SS transmits the rrcRelese with re-direction to frequency of Ncell 1 alongwith extended wait time set to 30s. However, at this time the Ncell 1 is not set to serving cell power, and a FAST UE can retry to camp on Ncell 23 again by sending a rrcConnectionREQ. R5s230342 was initially raised for this, but still the instability remains. This causes TC failure and needs to be addressed. |
| Summary of change | Before sending the rrcConnection Release on Ncell23, called the function f_NBIOT_SS_ConfigRachProcedureWithNoMsg4(nbiot_Cell23); This ensures that the UE never gets RAR, thus it cannot send rrcConnectionREQ on Ncell 23. Once the power level change is effective and the UE detects that, it moves to Ncell 1 as expected by the testcase. |
| TTCN module | ENBIOT_NAS.ttcn |
| MCC160 Comments |  |

Before Change:

<<SKIPPED CODE>> //@siclog "Step 10" siclog@ v_NAS_Ind := f_NBIOT_RRC_ConnEst_DefWithNas(nbiot_Cell23, cr_EstablishmentCause_NB_Any, cr_NAS_Indication(tsc_SHT_IntegrityProtected, cr_508_TAU_Request (tsc_EpsUpdate_TaUpdate, f_NBIOT_SecurityKSIasme_Get(), cr_DRXparameter_Any ('5C'O) ifpresent, cr_AdditionalUpdateType_CIOT ifpresent))); if (not f_Check_UE_NetworkCap_CIOT (v_NAS_Ind.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.ueNetworkCapability, CP_Backoff)) { f_NBIOT_SetVerdictFailOrInconc (__FILE__, __LINE__, "UE Network Cap failed CIOT bit check"); } //@siclog "Step 11" siclog@ //SS transmits an RRCConnectionRelease-NB message (IE redirectedCarrierInfo including carrierFreq-r13 of Ncell 1). v_TimingAtT := f_NBIOT_GetNextSearchSpace(nbiot_Cell23, 300); // @sic R5s230342 sic@ v_TimingInfoNCell1 := f_NBIOT_TimingOtherCell(nbiot_Cell23, cs_TimingInfo_EUTRA_NB(v_TimingAtT), nbiot_Cell1); v_CellPowerList := { cs_NB_CellPower(nbiot_Cell1, tsc_ServingCellRS_EPRE) }; f_NBIOT_SetCellPowerList(v_CellPowerList, v_TimingInfoNCell1, nbiot_Cell1 ); // @sic R5s230342 sic@ // @sic R5s23525 sic@ f_NBIOT_RRC_ConnectionRelease_Common(nbiot_Cell23, CONTROL_PLANE, cs_RRCConnectionRelease_NB_r14(tsc_RRC_TI_Def, other, omit , omit, 30, v_CarrierFreq_Cell1.DL), cs_TimingInfo_EUTRA_NB(v_TimingAtT)); // @sic R5-206436, R5s210634, R5s230342 sic@ //@siclog "Step 12" siclog@ //SS configures Ncell 23 as non-suitable cell and Ncell 1 as serving cell. v_CellPowerList := { cs_NB_CellPower(nbiot_Cell23, tsc_NonSuitableNBIOTCellRS_EPRE) }; f_NBIOT_SetCellPowerList(v_CellPowerList); // @sic R5s190574 sic@ //@siclog "Step 13" siclog@ <<SKIPPED CODE>>

After Change:

| <<SKIPPED CODE>> //@siclog "Step 10" siclog@ v_NAS_Ind := f_NBIOT_RRC_ConnEst_DefWithNas(nbiot_Cell23, cr_EstablishmentCause_NB_Any, cr_NAS_Indication(tsc_SHT_IntegrityProtected, cr_508_TAU_Request (tsc_EpsUpdate_TaUpdate, f_NBIOT_SecurityKSIasme_Get(), cr_DRXparameter_Any ('5C'O) ifpresent, cr_AdditionalUpdateType_CIOT ifpresent))); if (not f_Check_UE_NetworkCap_CIOT (v_NAS_Ind.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.ueNetworkCapability, CP_Backoff)) { f_NBIOT_SetVerdictFailOrInconc (__FILE__, __LINE__, "UE Network Cap failed CIOT bit check"); } //@siclog "Step 11" siclog@ //SS transmits an RRCConnectionRelease-NB message (IE redirectedCarrierInfo including carrierFreq-r13 of Ncell 1). v_TimingAtT := f_NBIOT_GetNextSearchSpace(nbiot_Cell23, 300); // @sic R5s230342 sic@ v_TimingInfoNCell1 := f_NBIOT_TimingOtherCell(nbiot_Cell23, cs_TimingInfo_EUTRA_NB(v_TimingAtT), nbiot_Cell1); v_CellPowerList := { cs_NB_CellPower(nbiot_Cell1, tsc_ServingCellRS_EPRE) }; f_NBIOT_SetCellPowerList(v_CellPowerList, v_TimingInfoNCell1, nbiot_Cell1 ); // @sic R5s230342 sic@ // @sic R5s23525 sic@ //WA#22_5_20 This is put in place to handle a FAST UE trying to camp again on Ncell 23 before the power level change is detected f_NBIOT_SS_ConfigRachProcedureWithNoMsg4(nbiot_Cell23); f_NBIOT_RRC_ConnectionRelease_Common(nbiot_Cell23, CONTROL_PLANE, cs_RRCConnectionRelease_NB_r14(tsc_RRC_TI_Def, other, omit , omit, 30, v_CarrierFreq_Cell1.DL), cs_TimingInfo_EUTRA_NB(v_TimingAtT)); // @sic R5-206436, R5s210634, R5s230342 sic@ //@siclog "Step 12" siclog@ //SS configures Ncell 23 as non-suitable cell and Ncell 1 as serving cell. v_CellPowerList := { cs_NB_CellPower(nbiot_Cell23, tsc_NonSuitableNBIOTCellRS_EPRE) }; f_NBIOT_SetCellPowerList(v_CellPowerList); // @sic R5s190574 sic@ //@siclog "Step 13" siclog@ <<SKIPPED CODE>> |  |
| --- | --- |