3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240470

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0952 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC IMS TC 6.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-07-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Ref 3GPP TS 24.229, section 5.1.1.2.1 on receiving a 500 server internal error response for a reregistration, the UE shall initiate a new initial registration.For the new registration the UE must select a different P-CSCF. As per NG.114 and IR 92, the UE needs to send an initial REGISTER message with the next pcscf or the UE has to reestablish a new PDU session if there is no next pcscf. |  |  |  |  |  |  |  |  |
| Summary of change: |  | UE is provided with a second P-CSCF address by SS |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE will fail this case unfairly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS34.229-5 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the essential changes needed to correct issues in the iwd-TTCN3-B2022-09_D24wk24 of IMS_NR5GC_RegistarationTestcases.ttcn

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | f_TC_6_3_NR5GC_IMS1() |
| --- | --- |
| Reason for change | Ref 3GPP TS 24.229, section 5.1.1.2.1 on receiving a 500 server internal error response for a reregistration, the UE shall initiate a new initial registration.For the new registration the UE must select a different P-CSCF. #draft proseCR would be raised during RAN5#104 meeting |
| Summary of change | UE is provided with a second P-CSCF address by SS |
| TTCN module | IMS_NR5GC_RegistrationTestcases.ttcn |
| MCC160 Comment |  |

Before Change:

/* * @desc REFERENCE TS 34.229-5 clause 6.3.NR5GC * @status APPROVED (IMS) */ function f_TC_6_3_NR5GC_IMS1() runs on IMS_PTC { var SecurityClientParams_Type v_SecurityClientParams; var charstring v_Expires; var float v_TimerVal; var integer v_NonceCount; var integer v_StatusCode; var integer v_NumOfIPAddressesToAllocate := 2;//Anritsu changes for 6.3 timer t_TRegExpireMin; timer t_TRegExpireMax; f_IMS_CC_Preamble(IPCAN_SignallingOnly, IMS_NULL, IMS_Security); f_IMS_TestBody_Set(true); // @siclog "Step 1 - 9" siclog@ v_Expires := "120"; v_TimerVal := 60.0; fl_TC_6_3_InitialRegistration(IPCAN_InitialRegistration, t_TRegExpireMin, t_TRegExpireMax, v_Expires, v_TimerVal); // @siclog "Step 10 - 11" siclog@ v_NonceCount := 2; v_StatusCode := 500; v_SecurityClientParams := f_IMS_PTC_ReRegistration_SecurityClientParamsGet(); fl_TC_6_3_ReRegistration(t_TRegExpireMin, t_TRegExpireMax, v_NonceCount, v_StatusCode, v_SecurityClientParams); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 10"); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Patch 4-7-1"); f_IMS_GlobalRelease(); // @siclog "Step 12 - 19" siclog@ v_Expires := "360"; v_TimerVal := 180.0; fl_TC_6_3_InitialRegistration(IPCAN_MO_IMS_Signalling, t_TRegExpireMin, t_TRegExpireMax, v_Expires, v_TimerVal); // @sic R5s201611 Change 2: IPCAN_MO_IMS_Signalling sic@ f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 12"); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Patch 4-7-2"); // @siclog "Step 20 - 21" siclog@ v_NonceCount := 2; v_StatusCode := 200; v_SecurityClientParams := f_IMS_PTC_ReRegistration_SecurityClientParamsGet(); fl_TC_6_3_ReRegistration(t_TRegExpireMin, t_TRegExpireMax, v_NonceCount, v_StatusCode, v_SecurityClientParams); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Patch 4-7-3"); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 20"); // @siclog "Step 22 - 27" siclog@ v_NonceCount := 3; v_StatusCode := 423; v_SecurityClientParams := f_IMS_PTC_ReRegistration_SecurityClientParamsGet(); fl_TC_6_3_ReRegistration(t_TRegExpireMin, t_TRegExpireMax, v_NonceCount, v_StatusCode, v_SecurityClientParams); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 22"); // @siclog "Step 28 - 31" siclog@ v_NonceCount := 5; v_StatusCode := 401; v_Expires := "800000"; fl_TC_6_3_ReRegistration(t_TRegExpireMin, t_TRegExpireMax, v_NonceCount, v_StatusCode, -, v_Expires); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 28"); f_IMS_TestBody_Set(false); f_IMS_CC_Postamble(); // @sic R5s201611: no test procedure as connection is already released in fl_TC_6_3_ReRegistration sic@ }

After Change:

/* * @desc REFERENCE TS 34.229-5 clause 6.3.NR5GC * @status APPROVED (IMS) */ function f_TC_6_3_NR5GC_IMS1() runs on IMS_PTC { var SecurityClientParams_Type v_SecurityClientParams; var charstring v_Expires; var float v_TimerVal; var integer v_NonceCount; var integer v_StatusCode; var integer v_NumOfIPAddressesToAllocate := 2; timer t_TRegExpireMin; timer t_TRegExpireMax; f_IMS_CC_Preamble(IPCAN_SignallingOnly, IMS_NULL, IMS_Security, -, -, v_NumOfIPAddressesToAllocate); f_IMS_TestBody_Set(true); // @siclog "Step 1 - 9" siclog@ v_Expires := "120"; v_TimerVal := 60.0; fl_TC_6_3_InitialRegistration(IPCAN_InitialRegistration, t_TRegExpireMin, t_TRegExpireMax, v_Expires, v_TimerVal); // @siclog "Step 10 - 11" siclog@ v_NonceCount := 2; v_StatusCode := 500; v_SecurityClientParams := f_IMS_PTC_ReRegistration_SecurityClientParamsGet(); fl_TC_6_3_ReRegistration(t_TRegExpireMin, t_TRegExpireMax, v_NonceCount, v_StatusCode, v_SecurityClientParams); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 10"); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Patch 4-7-1"); f_IMS_GlobalRelease(); f_IMS_RestartPCSCF(PDN_1, px_IPv4_Address_Other, px_IPv6_Address_Other); // @siclog "Step 12 - 19" siclog@ v_Expires := "360"; v_TimerVal := 180.0; fl_TC_6_3_InitialRegistration(IPCAN_MO_IMS_Signalling, t_TRegExpireMin, t_TRegExpireMax, v_Expires, v_TimerVal); // @sic R5s201611 Change 2: IPCAN_MO_IMS_Signalling sic@ f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 12"); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Patch 4-7-2"); // @siclog "Step 20 - 21" siclog@ v_NonceCount := 2; v_StatusCode := 200; v_SecurityClientParams := f_IMS_PTC_ReRegistration_SecurityClientParamsGet(); fl_TC_6_3_ReRegistration(t_TRegExpireMin, t_TRegExpireMax, v_NonceCount, v_StatusCode, v_SecurityClientParams); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Patch 4-7-3"); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 20"); // @siclog "Step 22 - 27" siclog@ v_NonceCount := 3; v_StatusCode := 423; v_SecurityClientParams := f_IMS_PTC_ReRegistration_SecurityClientParamsGet(); fl_TC_6_3_ReRegistration(t_TRegExpireMin, t_TRegExpireMax, v_NonceCount, v_StatusCode, v_SecurityClientParams); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 22"); // @siclog "Step 28 - 31" siclog@ v_NonceCount := 5; v_StatusCode := 401; v_Expires := "800000"; fl_TC_6_3_ReRegistration(t_TRegExpireMin, t_TRegExpireMax, v_NonceCount, v_StatusCode, -, v_Expires); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 28"); f_IMS_TestBody_Set(false); f_IMS_CC_Postamble(); // @sic R5s201611: no test procedure as connection is already released in fl_TC_6_3_ReRegistration sic@ }