3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240471

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3403 | rev | 1 | Current version: | 17.11.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR MAC test case 7.1.1.6.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-03-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | During testing on 24wk12 IWD for TC 7.1.1.6.1 with the MCC160 proposed implementation to test TP4, it is found that the TC is failing both for ENDC and NR5GC. Specifically, the MCC160 implementation does not follow the SPS pattern and multiple chipsets are sending a DTX at Step 11 |  |  |  |  |  |  |  |  |
| Summary of change: |  | To properly test TP4, the following changes are required as was originally highlighted in TTCN CR R5s230810: - Introduce Steps 11a1 – 11a3: If pc_dl_SchedulingOffset_PDSCH_TypeA is TRUE, adjust the timing of v_TimingDL_CRNTI to make PDSCH coincide with the next SPS transmission slot and satisfy TP4. Since k1 is being changed to 8, the timing of v_TimingUL_5_CRNTI is also adjusted to be the same as the SPS HARQ slot. - Introduce Steps 11b1 – 11b2: If pc_dl_SchedulingOffset_PDSCH_TypeA is FALSE, then a normal DL SPS transmission is made instead of a C-RNTI transmission. This is done so that the timing of succeeding steps do not need to be modified. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.6.1.ENDC, 7.1.1.6.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	4

2	Corrections required	4

2.1	Change 1	4

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D24wk12 related to the title of this CR.

Contact:  Bhise Parikshit
parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change 1

| Function name | function f_TC_7_1_1_6_1_NR_TestBody1 |
| --- | --- |
| Reason for change | During testing on 24wk12 IWD for TC 7.1.1.6.1 with the MCC160 proposed implementation to test TP4, it is found that the TC is failing both for ENDC and NR5GC. Specifically, the MCC160 implementation does not follow the SPS pattern and multiple chipsets are sending a DTX at Step 11 |
| Summary of change | To properly test TP4, the following changes are required as was originally highlighted in TTCN CR R5s230810 Change 4: - Introduce Steps 11a1 – 11a3: If pc_dl_SchedulingOffset_PDSCH_TypeA is TRUE, adjust the timing of v_TimingDL_CRNTI to make PDSCH coincide with the next SPS transmission slot and satisfy TP4. Since k1 is being changed to 8, the timing of v_TimingUL_5_CRNTI is also adjusted to be the same as the SPS HARQ slot. - Introduce Steps 11b1 – 11b2: If pc_dl_SchedulingOffset_PDSCH_TypeA is FALSE, then a normal DL SPS transmission is made instead of a C-RNTI transmission. This is done so that the timing of succeeding steps do not need to be modified. |
| TTCN module | MAC_SPS_TC_Common_NR.ttcn |

Before Change:

function f_TC_7_1_1_6_1_NR_TestBody1(DRB_Identity p_NR_DRB_Id) runs on NR_BASE_PTC return SubFrameTiming_Type { const integer const_SPS_Period :=80; // period in milli seconds const integer const_HARQ_Id := 1; //arbitrarily selected in steps 17 and 20 var NR_MAC_SDU_Type v_EncodedRlcPdu1; var template (value) NR_MAC_SDU_SubPDU_List_Type v_SDU_SubPDUListDL; var template (value) NR_MAC_PDUList_Type v_MAC_PDUListDL; var SubFrameTiming_Type v_TimingDL; var SubFrameTiming_Type v_TimingDL_CRNTI; var SubFrameTiming_Type v_TimingUL_1; var SubFrameTiming_Type v_TimingUL_2; var SubFrameTiming_Type v_TimingUL_3; var SubFrameTiming_Type v_TimingUL_4; var SubFrameTiming_Type v_TimingUL_5_CRNTI; var SubFrameTiming_Type v_TimingUL_6; var SubFrameTiming_Type v_TimingUL_7; var SubFrameTiming_Type v_TimingUL_8; var SubFrameTiming_Type v_TimingUL_9; var SubFrameTiming_Type v_TimingUL_10; var SubFrameTiming_Type v_TimingUL_11; var integer v_Step11_Time; <<SKIPPED CODE>> //@siclog "Step 11a1, 11a2 11b1" siclog@ //C-RNTI grant auto transferred by SS //@sic R5s221197 R5s230088 R5-237339 sic@ if (pc_dl_SchedulingOffset_PDSCH_TypeA) { v_Step11_Time :=const_SPS_Period-4; //Set K1=8 v_DciDlInfo.ResoureAssignment.TimeDomain.Index := int2bit(1, 4); //@sic R5s240155 sic@ f_NR_SS_CommonCellConfig(nr_Cell1, cas_NR_DciDlInfo_REQ(nr_Cell1, cs_TimingInfo_Now, v_DciDlInfo)); } else { v_Step11_Time :=const_SPS_Period; } v_TimingDL_CRNTI := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,v_Step11_Time); v_TimingUL_5_CRNTI := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_4,const_SPS_Period); <<SKIPPED CODE>> // REMOVED PDCP SDU =36B +3B PDCP Header+ 1B RLC UM Header + 2B MAC Header =42B=336 bits (an allowed TB size as per 38.523-3 Annex B) v_EncodedRlcPdu1 := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (3, crs_NR_PDCP_SDU_36B, 1)));//@sic R5-227518 sic@ v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1) }; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR( v_TimingDL_CRNTI), v_MAC_PDUListDL)); //@siclog "Step 12-13 void " siclog@ //@siclog "Step 14 " siclog@ //@sic R5-237339 sic@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,2*const_SPS_Period); v_TimingUL_6 := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_4,2*const_SPS_Period); // PDCP SDU =38B +3B PDCP Header+ 1B RLC UM Header + 2B MAC Header =44B=352 bits (an allowed TB size as per 38.523-3 Annex B) v_EncodedRlcPdu1 := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (4, crs_NR_PDCP_SDU_38B, 1)));//@sic R5-227518 sic@ v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1) }; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; DRB.send(cads_NR_DRB_COMMON_REQ_MAC_PDUList_NoCRNTI_Grant(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR( v_TimingDL), v_MAC_PDUListDL)); <<SKIPPED CODE>>

After Change:

function f_TC_7_1_1_6_1_NR_TestBody1(DRB_Identity p_NR_DRB_Id) runs on NR_BASE_PTC return SubFrameTiming_Type { const integer const_SPS_Period :=80; // period in milli seconds const integer const_HARQ_Id := 1; //arbitrarily selected in steps 17 and 20 var NR_MAC_SDU_Type v_EncodedRlcPdu1; var template (value) NR_MAC_SDU_SubPDU_List_Type v_SDU_SubPDUListDL; var template (value) NR_MAC_PDUList_Type v_MAC_PDUListDL; var SubFrameTiming_Type v_TimingDL; var SubFrameTiming_Type v_TimingDL_CRNTI; var SubFrameTiming_Type v_TimingDL_Step11b; //WA# var SubFrameTiming_Type v_TimingUL_1; var SubFrameTiming_Type v_TimingUL_2; var SubFrameTiming_Type v_TimingUL_3; var SubFrameTiming_Type v_TimingUL_4; var SubFrameTiming_Type v_TimingUL_5_CRNTI; var SubFrameTiming_Type v_TimingUL_6; var SubFrameTiming_Type v_TimingUL_7; var SubFrameTiming_Type v_TimingUL_8; var SubFrameTiming_Type v_TimingUL_9; var SubFrameTiming_Type v_TimingUL_10; var SubFrameTiming_Type v_TimingUL_11; var integer v_Step11_Time; <<SKIPPED CODE>> //@siclog "Step 11a1, 11a2 11b1" siclog@ //C-RNTI grant auto transferred by SS //@sic R5s221197 R5s230088 R5-237339 sic@ //WA# if (pc_dl_SchedulingOffset_PDSCH_TypeA) { //@siclog "Step 11a1-11a2" siclog@ select(v_SCS) { case(kHz15) { v_TimingDL_CRNTI := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period/2 - 4); // C-RNTI ActTime with k0=4 -- k1 needs to be adjusted to 8 v_TimingUL_5_CRNTI := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_4,const_SPS_Period/2); // Receive C-RNTI HARQ with same timing as SPS HARQ //v_TimingUL_5_CRNTI.Symbol := valueof(cs_SymbolTimingInfo_Any); // REMOVED //v_TimingUL_5_CRNTI.Subframe.Number := v_TimingDL_CRNTI.Subframe.Number + 7; //FDD K1 =7 // REMOVED } case(kHz30) { v_TimingDL_CRNTI := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period/2 - 2); // C-RNTI ActTime with k0=4 -- k1 needs to be adjusted to 8 v_TimingUL_5_CRNTI := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_4,const_SPS_Period/2); // Receive C-RNTI HARQ with same timing as SPS HARQ //v_TimingUL_5_CRNTI.Symbol := valueof(cs_SymbolTimingInfo_Any); // REMOVED //v_TimingUL_5_CRNTI.Subframe.Number := v_TimingDL_CRNTI.Subframe.Number + 2; //TDD K1 =5 // REMOVED //v_TimingUL_5_CRNTI.Slot.SlotOffset.Numerology1 := 1; // REMOVED } case(kHz120) { v_TimingDL_CRNTI := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period/2 - 1); // C-RNTI ActTime with k0=4 -- k1 needs to be adjusted to 8 v_TimingDL_CRNTI.Slot.SlotOffset.Numerology3 := 5; v_TimingUL_5_CRNTI := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_4,const_SPS_Period/2); // Receive C-RNTI HARQ with same timing as SPS HARQ //v_TimingUL_5_CRNTI.Symbol := valueof(cs_SymbolTimingInfo_Any); // REMOVED //v_TimingUL_5_CRNTI.Subframe.Number := v_TimingDL_CRNTI.Subframe.Number + 1; //FR2 K1 =8 // REMOVED } case else { FatalError(__FILE__, __LINE__, "Invalid SubCarrierSapcing"); } } //Set K1=8 v_DciDlInfo.ResoureAssignment.TimeDomain.Index := int2bit(1, 4); //@sic R5s240155 sic@ f_NR_SS_CommonCellConfig(nr_Cell1, cas_NR_DciDlInfo_REQ(nr_Cell1, cs_TimingInfo_Now, v_DciDlInfo)); // PDCP SDU =36B +3B PDCP Header+ 1B RLC UM Header + 2B MAC Header =42B=336 bits (an allowed TB size as per 38.523-3 Annex B) v_EncodedRlcPdu1 := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (3, crs_NR_PDCP_SDU_36B, 1)));//@sic R5-227518 sic@ v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1) }; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR( v_TimingDL_CRNTI), v_MAC_PDUListDL)); } else { //@siclog "Step 11b1" siclog@ v_TimingDL_Step11b := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period/2); // Normal SPS transmission v_TimingUL_5_CRNTI := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_4,const_SPS_Period/2); // Receive HARQ with same timing as SPS HARQ // PDCP SDU =36B +3B PDCP Header+ 1B RLC UM Header + 2B MAC Header =42B=336 bits (an allowed TB size as per 38.523-3 Annex B) v_EncodedRlcPdu1 := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (3, crs_NR_PDCP_SDU_36B, 1)));//@sic R5-227518 sic@ v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1) }; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; DRB.send(cads_NR_DRB_COMMON_REQ_MAC_PDUList_NoCRNTI_Grant(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR( v_TimingDL_Step11b), v_MAC_PDUListDL)); } <<SKIPPED CODE>> //@siclog "Step 12-13 void " siclog@ //@siclog "Step 14 " siclog@ //@sic R5-237339 sic@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period+const_SPS_Period/2); //WA# v_TimingUL_6 := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_4,const_SPS_Period+const_SPS_Period/2); //WA# // PDCP SDU =38B +3B PDCP Header+ 1B RLC UM Header + 2B MAC Header =44B=352 bits (an allowed TB size as per 38.523-3 Annex B) v_EncodedRlcPdu1 := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (4, crs_NR_PDCP_SDU_38B, 1)));//@sic R5-227518 sic@ v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1) }; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; DRB.send(cads_NR_DRB_COMMON_REQ_MAC_PDUList_NoCRNTI_Grant(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR( v_TimingDL), v_MAC_PDUListDL)); <<SKIPPED CODE>>