3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240488

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3460 | rev | 1 | Current version: | 17.11.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for Test case 8.1.4.1.7.X |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-06-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | TS 38.523-3 cl. 7.3.5.3.4, step 9 enquires about PDCP count but only SRB count is received but not DRB. This is because that DRB is configured but not yet activated. As a result of having receieved only count for SRB, cipehing is not applied for DRB. For this reason, we have proposed to apply the ciphering on DRB otherwise this leads to test failure due to data not being looped back. Therefore, the 8.1.4.1.7.x test body is modified to reactivate security for all RBs after handover is complete to ensure that all added RBs are included in the process. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Modified f_TC_8_1_4_1_7_x_NR5GC_TestBody reactivate security for all RBs without key refresh after handover is complete to ensure that all configured RBs are included and ciphered properly. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.4.1.7.1, 8.1.4.1.7.2, 8.1.4.1.7.3.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D24wk12 related to the title of this CR.

Contact:	Kalahasti, Narendra

Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | function f_TC_8_1_4_1_7_x_NR5GC_TestBody |
| --- | --- |
| Reason for change | TS 38.523-3 cl. 7.3.5.3.4, step 9 enquires about PDCP count but only SRB count is received but not DRB. This is because that DRB is configured but not yet activated. As a result of having receieved only count for SRB, cipehing is not applied for DRB. For this reason, we have proposed to apply the ciphering on DRB otherwise this leads to test failure due to data not being looped back. Therefore, the 8.1.4.1.7.x test body is modified to reactivate security for all RBs after handover is complete to ensure that all added RBs are included in the process. |
| Summary of change | Modified f_TC_8_1_4_1_7_x_NR5GC_TestBody reactivate security for all RBs without key refresh after handover is complete to ensure that all configured RBs are included and ciphered properly. |
| TTCN module | RRC_NR_CA_IntraNR_Handover_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change

function f_TC_8_1_4_1_7_x_NR5GC_TestBody(NR_CellId_Type p_SourcePCell, NR_CellId_Type p_SourceSCell, NR_CellId_Type p_TargetPCell, NR_CellId_Type p_TargetSCell) runs on NR5GC_PTC { var template (value) NR_CellPowerList_Type v_CellPowerList_AtT0 :={cs_NR_CellPower(p_SourcePCell, -88, -91), //@sic R5-237355 sic@ cs_NR_CellPower(p_SourceSCell, -88, -91), //@sic R5-237355 sic@ cs_NR_CellPower(p_TargetPCell, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE), //@sic R5s210079 sic@ cs_NR_CellPower(p_TargetSCell, -88, -91) //@sic R5s210079, R5-237355 sic@ }; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT1 :={cs_NR_CellPower(p_SourcePCell, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE), cs_NR_CellPower(p_SourceSCell, -88, -91), //@sic R5-237355 sic@ cs_NR_CellPower(p_TargetPCell, -82, -82), //@sic R5s211333, R5-237355 sic@ cs_NR_CellPower(p_TargetSCell, -88, -91) //@sic R5-237355 sic@ }; var NR_CellId_Type v_SourcePCell; var NR_CellId_Type v_SourceSCell; var NR_CellId_Type v_TargetPCell; var integer v_DRBId; var PDUSessionInfoList_Type v_PDUSessionInfoList; //@siclog "Step 1" siclog@ //The SS changes Cell parameters according to the row "T0" in table 8.1.4.1.7.1.3.2-1/2. f_NR_SetCellPowerList(v_CellPowerList_AtT0); //@siclog "Steps 2-3" siclog@ //The SS transmits an RRCReconfiguration message including a sCellToAddModList to add NR Cell 3 as a SCell. //The UE transmits the RRCReconfigurationComplete message? f_NR5GC_AddMod_1SCell(p_SourcePCell, p_SourceSCell); // NR Cell1, NR Cell3/10 //@siclog "Steps 4-5" siclog@ //The SS transmits an RRCReconfiguration message including a reconfigurationWithSync for handover to NR Cell 3 and sCellToAddModList to add NR Cell 2 as a SCell. //Check: Does UE transmit the RRCReconfigurationComplete message? v_TargetPCell := p_SourceSCell; // NR Cell3/10 f_NR5GC_508RRC_IntraNR_CA_HO_InterCell(p_SourcePCell, p_SourceSCell, v_TargetPCell, p_TargetSCell, -, -, cs_MasterKeyUpdate(false, tsc_NR_38508_NextHopChainingCount));// NR Cell1, NR Cell3/10, NR Cell3/10, NR Cell2 @sic R5s201249 sic@ //@siclog "Step 5A" siclog@ //Check: Does the test result of generic test procedure in TS 38.508-1 Table 4.9.1-1A indicate that the UE is capable of exchanging IP data on DRB #n associated with the first PDU session on Cell2? f_NR5GC_CheckDataPath_CA(v_TargetPCell, p_TargetSCell, "Step 5A", true); //@sic R5-203665 sic@ //@siclog "Step 6" siclog@ //The SS changes Cell parameters according to the row "T1" in table 8.2.4.2.1.1.3.2-1/2. f_NR_SetCellPowerList(v_CellPowerList_AtT1); //@siclog "Steps 7-8" siclog@ //The SS transmits an RRCReconfiguration message including a reconfigurationWithSync to change PCell to NR Cell 23 and including sCellToReleaseList with a sCellIndex of NR Cell 2. //Check: Does the UE transmit the RRCReconfigurationComplete message? v_SourcePCell := p_SourceSCell; // NR Cell3/10 v_SourceSCell := p_TargetSCell; // NR Cell2 f_NR5GC_508RRC_IntraNR_CA_HO_InterCell(v_SourcePCell, v_SourceSCell, p_TargetPCell, omit, -, -, cs_MasterKeyUpdate(false, tsc_NR_38508_NextHopChainingCount)); // NR Cell3/10, NR Cell2, NR Cell23/30 @sic R5s201249 sic@ //@siclog "Step 9" siclog@ //Check: Does the test result of generic test procedure in TS 38.508-1 subclause 4.9.1 indicate that the UE is capable of exchanging IP data on DRB #n associated with the first PDU session on Cell23/30? v_DRBId := f_NR_GetDefaultDRB_ForFirstPDUSession(); v_PDUSessionInfoList := f_NR5GC_MobileInfo_GetSessionInfoList(); f_NR5GC_CheckDataPath(p_TargetPCell, //NR Cell23/30 f_NR5GC_GetPdnIndex(v_PDUSessionInfoList[0]), dedicatedBearer, v_DRBId, "Step 9"); }

After Change

function f_TC_8_1_4_1_7_x_NR5GC_TestBody(NR_CellId_Type p_SourcePCell, NR_CellId_Type p_SourceSCell, NR_CellId_Type p_TargetPCell, NR_CellId_Type p_TargetSCell) runs on NR5GC_PTC { var template (value) NR_CellPowerList_Type v_CellPowerList_AtT0 :={cs_NR_CellPower(p_SourcePCell, -88, -91), //@sic R5-237355 sic@ cs_NR_CellPower(p_SourceSCell, -88, -91), //@sic R5-237355 sic@ cs_NR_CellPower(p_TargetPCell, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE), //@sic R5s210079 sic@ cs_NR_CellPower(p_TargetSCell, -88, -91) //@sic R5s210079, R5-237355 sic@ }; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT1 :={cs_NR_CellPower(p_SourcePCell, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE), cs_NR_CellPower(p_SourceSCell, -88, -91), //@sic R5-237355 sic@ cs_NR_CellPower(p_TargetPCell, -82, -82), //@sic R5s211333, R5-237355 sic@ cs_NR_CellPower(p_TargetSCell, -88, -91) //@sic R5-237355 sic@ }; var NR_CellId_Type v_SourcePCell; var NR_CellId_Type v_SourceSCell; var NR_CellId_Type v_TargetPCell; var integer v_DRBId; var PDUSessionInfoList_Type v_PDUSessionInfoList; //@siclog "Step 1" siclog@ //The SS changes Cell parameters according to the row "T0" in table 8.1.4.1.7.1.3.2-1/2. f_NR_SetCellPowerList(v_CellPowerList_AtT0); //@siclog "Steps 2-3" siclog@ //The SS transmits an RRCReconfiguration message including a sCellToAddModList to add NR Cell 3 as a SCell. //The UE transmits the RRCReconfigurationComplete message? f_NR5GC_AddMod_1SCell(p_SourcePCell, p_SourceSCell); // NR Cell1, NR Cell3/10 //@siclog "Steps 4-5" siclog@ //The SS transmits an RRCReconfiguration message including a reconfigurationWithSync for handover to NR Cell 3 and sCellToAddModList to add NR Cell 2 as a SCell. //Check: Does UE transmit the RRCReconfigurationComplete message? v_TargetPCell := p_SourceSCell; // NR Cell3/10 f_NR5GC_508RRC_IntraNR_CA_HO_InterCell(p_SourcePCell, p_SourceSCell, v_TargetPCell, p_TargetSCell, -, -, cs_MasterKeyUpdate(false, tsc_NR_38508_NextHopChainingCount));// NR Cell1, NR Cell3/10, NR Cell3/10, NR Cell2 @sic R5s201249 sic@ f_NR_SS_AS_ActivateSecurity_NoRefresh(v_TargetPCell,-); //@siclog "Step 5A" siclog@ //Check: Does the test result of generic test procedure in TS 38.508-1 Table 4.9.1-1A indicate that the UE is capable of exchanging IP data on DRB #n associated with the first PDU session on Cell2? f_NR5GC_CheckDataPath_CA(v_TargetPCell, p_TargetSCell, "Step 5A", true); //@sic R5-203665 sic@ //@siclog "Step 6" siclog@ //The SS changes Cell parameters according to the row "T1" in table 8.2.4.2.1.1.3.2-1/2. f_NR_SetCellPowerList(v_CellPowerList_AtT1); //@siclog "Steps 7-8" siclog@ //The SS transmits an RRCReconfiguration message including a reconfigurationWithSync to change PCell to NR Cell 23 and including sCellToReleaseList with a sCellIndex of NR Cell 2. //Check: Does the UE transmit the RRCReconfigurationComplete message? v_SourcePCell := p_SourceSCell; // NR Cell3/10 v_SourceSCell := p_TargetSCell; // NR Cell2 f_NR5GC_508RRC_IntraNR_CA_HO_InterCell(v_SourcePCell, v_SourceSCell, p_TargetPCell, omit, -, -, cs_MasterKeyUpdate(false, tsc_NR_38508_NextHopChainingCount)); // NR Cell3/10, NR Cell2, NR Cell23/30 @sic R5s201249 sic@ //@siclog "Step 9" siclog@ //Check: Does the test result of generic test procedure in TS 38.508-1 subclause 4.9.1 indicate that the UE is capable of exchanging IP data on DRB #n associated with the first PDU session on Cell23/30? v_DRBId := f_NR_GetDefaultDRB_ForFirstPDUSession(); v_PDUSessionInfoList := f_NR5GC_MobileInfo_GetSessionInfoList(); f_NR5GC_CheckDataPath(p_TargetPCell, //NR Cell23/30 f_NR5GC_GetPdnIndex(v_PDUSessionInfoList[0]), dedicatedBearer, v_DRBId, "Step 9"); }