3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240496

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4833 | rev | 1 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to the NBIOT-NTN testcase 22.3.2.7a |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, LTE_NBIOT_eMTC_NTN_plus_EPS-UEConTest |  |  |  |  |  | Date: |  |  | 2024-06-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This f_NBIOT_PreconfigureNTN_UE() is already invoked in the f_NBIOT_RLC_Preamble_State2B_NB(). So remove the duplicated one. For NTN mode, the value of v_NTN_delay is not initialized correctly before used |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove f_NBIOT_PreconfigureNTN_UE(). Initialize the v_NTN_delay with correct value. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A Conformant UE may fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.3.2.7a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

2.1	Change to the f_TC_22_3_2_7a_NBIOT	2

2.2	Change to the fl_TC_22_3_2_7_and7a_TestBody	4

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D24wk24 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change to the f_TC_22_3_2_7a_NBIOT

| Function name | f_TC_22_3_2_7a_NBIOT () |
| --- | --- |
| Reason for change | This f_NBIOT_PreconfigureNTN_UE() is already invoked in the f_NBIOT_RLC_Preamble_State2B_NB(). So remove the duplicated one. |
| Summary of change | Remove f_NBIOT_PreconfigureNTN_UE(). |
| TTCN module | ttcn\develop\NBIOT\22_3\NBIOT_RLC_NTN.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_22_3_2_7a_NBIOT() runs on NBIOT_PTC { …… // RRCConnectionSetup-NB with twoHARQ-ProcessesConfig-r14 configured v_RrcConnSetup := f_NBIOT_508_RRCConnectionSetup(-, v_SRB_ToAddModList_NB_r13, -, -, true_); // perform NBIOT specific preamble for RLC if(pc_NB_ntn_Connectivity_EPC) // @sic R5s240065 sic@ { f_NBIOT_PreconfigureNTN_UE(); //@sic R5w240103 sic@ } f_NBIOT_RLC_Preamble_State2B_NB(v_RLC_Rec, nbiot_Cell1, STATE2A_NB_TESTLOOP_ModeG, // @sic R5-174405 sic@ v_RrcConnSetup, v_HARQ_ProcessNumber); f_NBIOT_TestBody_Set(true, layer2); f_NBIOT_HarqErrorHandling_Set(raiseInconc); fl_TC_22_3_2_7a_TestBody (v_RLC_Rec); f_NBIOT_HarqErrorHandling_Set(ignore); f_NBIOT_TestBody_Set(false); f_NBIOT_L2_Postamble_State2B_NB(nbiot_Cell1, v_RLC_Rec.AM_VTS, v_RLC_Rec.AM_VRR); }

After Change:

| function f_TC_22_3_2_7a_NBIOT() runs on NBIOT_PTC { …… // RRCConnectionSetup-NB with twoHARQ-ProcessesConfig-r14 configured v_RrcConnSetup := f_NBIOT_508_RRCConnectionSetup(-, v_SRB_ToAddModList_NB_r13, -, -, true_); // perform NBIOT specific preamble for RLC f_NBIOT_RLC_Preamble_State2B_NB(v_RLC_Rec, nbiot_Cell1, STATE2A_NB_TESTLOOP_ModeG, // @sic R5-174405 sic@ v_RrcConnSetup, v_HARQ_ProcessNumber); f_NBIOT_TestBody_Set(true, layer2); f_NBIOT_HarqErrorHandling_Set(raiseInconc); fl_TC_22_3_2_7a_TestBody (v_RLC_Rec); f_NBIOT_HarqErrorHandling_Set(ignore); f_NBIOT_TestBody_Set(false); f_NBIOT_L2_Postamble_State2B_NB(nbiot_Cell1, v_RLC_Rec.AM_VTS, v_RLC_Rec.AM_VRR); } |  |
| --- | --- |

## Change to the fl_TC_22_3_2_7_and7a_TestBody

| Function name | fl_TC_22_3_2_7_and7a_TestBody () |
| --- | --- |
| Reason for change | For NTN mode, the value of v_NTN_delay is not initialized correctly before used |
| Summary of change | Initialize the v_NTN_delay with correct value. |
| TTCN module | ttcn\develop\NBIOT\22_3\NBIOT_RLC_Common.ttcn |
| MCC160 Comment |  |

Before Change:

function fl_TC_22_3_2_7_and7a_TestBody(inout RLC_SS_State p_RLC_Rec, boolean p_NTN := false) runs on NBIOT_PTC { var integer v_RLC_DL_SDU_Length; var integer v_RLC_UL_SDU_Length := 36; var integer v_L; var integer v_Duration; var template (value) TimingInfo_Type v_TimingInfo; var template (value) TimingInfo_Type v_TimingInfo1; var template (value) TimingInfo_Type v_TimingInfo2; var template (value) TimingInfo_Type v_TimingInfo3; var template (value) TimingInfo_Type v_TimingInfo4; var template (value) TimingInfo_Type v_TimingInfo5; var template (value) TimingInfo_Type v_TimingInfo6; var template (value) TimingInfo_Type v_TimingInfo7; var template (present) RLC_Status_NACK_List_Type v_NACK_List; var template (present) RLC_Status_Padding_Type v_Padding_Any := ?; var template (present) RLC_Status_Padding_Type v_Padding_3bits_Any := '???'B; var template (value) NB_DciUlInfo_Type v_DciUlInfo; var RLC_DataField_Type v_RLC_Data1; var integer v_VTS_Offset := p_RLC_Rec.AM_VTS; var SubFrameTiming_Type v_TimingStart; var SubFrameTiming_Type v_SubFrameTimingTa; var SubFrameTiming_Type v_SubFrameTimingTb; var boolean v_NTN := p_NTN; var IntegerList_Type v_SearchSpaceIncr; var IntegerList_Type v_SearchSpaceIncr_NTN; var integer v_NTN_delay := 0; var integer v_SearchSpaceInc := 0; var float v_Watch_T_Reordering := f_NBIOT_SetTimerToleranceMax(l2Timer, p_RLC_Rec.t_Reordering_r14); ...... //@siclog "Step 9" siclog@ //Check 1: Does the UE transmit a Status Report with NACK_SN=3 and ACK_SN=5? //Record time TB // Check 2: (TB - TA) = t-Reordering? p_RLC_Rec.AM_VTA := v_VTS_Offset + 5; v_NACK_List := cr_NACK_List(v_VTS_Offset + 3, tsc_E2_NoSO_Follows, omit); f_NB_RxSTATUS_PDU_NACK(p_RLC_Rec, v_NACK_List, v_Padding_Any); f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 9 Check 1"); v_SubFrameTimingTb := p_RLC_Rec.TimeStampLastReceivedPDU; v_Duration := f_SubFrameTiming_Duration( v_SubFrameTimingTa, v_SubFrameTimingTb); if (v_NTN) { //22.3.2.7a if ( f_NBIOT_NTN_SubFrameTimingCheckDuration( l2Timer, v_Duration, float2int(p_RLC_Rec.t_Reordering_r17 * 1000.0), v_NTN_delay ))// @sic R5s230710 sic@ { f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 9 Check 2"); } else { f_NBIOT_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 9 Check 2"); }; } else { //22.3.2.7 if ( f_NBIOT_SubFrameTimingCheckDuration( l2Timer, v_Duration, float2int(p_RLC_Rec.t_Reordering_r14 * 1000.0))) { f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 9 Check 2"); } else { f_NBIOT_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 9 Check 2"); }; } //@siclog "Step 10" siclog@ ...... }

After Change:

function fl_TC_22_3_2_7_and7a_TestBody(inout RLC_SS_State p_RLC_Rec, boolean p_NTN := false) runs on NBIOT_PTC { var integer v_RLC_DL_SDU_Length; var integer v_RLC_UL_SDU_Length := 36; var integer v_L; var integer v_Duration; var template (value) TimingInfo_Type v_TimingInfo; var template (value) TimingInfo_Type v_TimingInfo1; var template (value) TimingInfo_Type v_TimingInfo2; var template (value) TimingInfo_Type v_TimingInfo3; var template (value) TimingInfo_Type v_TimingInfo4; var template (value) TimingInfo_Type v_TimingInfo5; var template (value) TimingInfo_Type v_TimingInfo6; var template (value) TimingInfo_Type v_TimingInfo7; var template (present) RLC_Status_NACK_List_Type v_NACK_List; var template (present) RLC_Status_Padding_Type v_Padding_Any := ?; var template (present) RLC_Status_Padding_Type v_Padding_3bits_Any := '???'B; var template (value) NB_DciUlInfo_Type v_DciUlInfo; var RLC_DataField_Type v_RLC_Data1; var integer v_VTS_Offset := p_RLC_Rec.AM_VTS; var SubFrameTiming_Type v_TimingStart; var SubFrameTiming_Type v_SubFrameTimingTa; var SubFrameTiming_Type v_SubFrameTimingTb; var boolean v_NTN := p_NTN; var IntegerList_Type v_SearchSpaceIncr; var IntegerList_Type v_SearchSpaceIncr_NTN; var integer v_NTN_delay := 0; var integer v_SearchSpaceInc := 0; var float v_Watch_T_Reordering := f_NBIOT_SetTimerToleranceMax(l2Timer, p_RLC_Rec.t_Reordering_r14); ...... //@siclog "Step 9" siclog@ //Check 1: Does the UE transmit a Status Report with NACK_SN=3 and ACK_SN=5? //Record time TB // Check 2: (TB - TA) = t-Reordering? p_RLC_Rec.AM_VTA := v_VTS_Offset + 5; v_NACK_List := cr_NACK_List(v_VTS_Offset + 3, tsc_E2_NoSO_Follows, omit); f_NB_RxSTATUS_PDU_NACK(p_RLC_Rec, v_NACK_List, v_Padding_Any); f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 9 Check 1"); v_SubFrameTimingTb := p_RLC_Rec.TimeStampLastReceivedPDU; v_Duration := f_SubFrameTiming_Duration( v_SubFrameTimingTa, v_SubFrameTimingTb); if (v_NTN) { //22.3.2.7a v_NTN_delay := f_NBIOT_NTN_Get_T_NTNoffset(nbiot_Cell1); // if ( f_NBIOT_NTN_SubFrameTimingCheckDuration( l2Timer, v_Duration, float2int(p_RLC_Rec.t_Reordering_r17 * 1000.0), v_NTN_delay ))// @sic R5s230710 sic@ { f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 9 Check 2"); } else { f_NBIOT_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 9 Check 2"); }; } else { //22.3.2.7 if ( f_NBIOT_SubFrameTimingCheckDuration( l2Timer, v_Duration, float2int(p_RLC_Rec.t_Reordering_r14 * 1000.0))) { f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 9 Check 2"); } else { f_NBIOT_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 9 Check 2"); }; } //@siclog "Step 10" siclog@ ...... }