3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240502

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3489 | rev | - | Current version: | 17.11.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcase 11.4.12 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-07-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the current TTCN implementation, the variable v_UENWCap is assigned cdr_NR5GC_UENetworkCap('58'O);, which initializes the IEs cIoT_proSeCap and cIoT_V2X. The variable v_UENWCap is then passed to function f_EUTRA38_TAUReqFromN1, and within this function the variable v_UENWCap is further initialized with following code: if (isvalue(p_UE_NetworkCap)) { // @sic R5-243593 sic@ v_UENWCap := p_UE_NetworkCap; } else { v_UENWCap := cdr_NR5GC_UENetworkCap('58'O); if (valueof(p_EstablishmentCause) == emergency) { // @sic R5-217714 sic@ v_UENWCap.cIoT_proSeCap := ?; // @sic R5-220644 sic@ v_UENWCap.cIoT_V2X := ? } } For testcase 11.4.12, the highlighted code is executed as a result of highlighted. This results in the testcase 11.4.12 failing, if the UE does not include the Octet 9 of the IE ueNetworkCapability. This needs to be addressed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to initialize v_UENWCap := cr_UENetworkCap_Any ('58'O);, and then pass v_UENWCap to function f_EUTRA38_TAUReqFromN1. Within function f_EUTRA38_TAUReqFromN1, it is proposed to implement following code: if (isvalue(p_UE_NetworkCap)) { // @sic R5-243593 sic@ v_UENWCap := p_UE_NetworkCap; } else { if(testcasename() == "TC_11_4_12_NR5GC"){ v_UENWCap := cr_UENetworkCap_Any('58'O); }else { v_UENWCap := cdr_NR5GC_UENetworkCap('58'O); } //WA#WI1306226 11_4_12 v_UENWCap := cdr_NR5GC_UENetworkCap('58'O); if (valueof(p_EstablishmentCause) == emergency and (testcasename() != "TC_11_4_12_NR5GC")) { // @sic R5-217714 sic@ //WA#WI1306226 11_4_12 v_UENWCap.cIoT_proSeCap := ?; // @sic R5-220644 sic@ v_UENWCap.cIoT_V2X := ? } } In the function fl_TC_11_4_12_NR5GC_EUTRA_TestBody, it is proposed to implement following code to check that the support for N1 mode is set to ‘0’B : if (ispresent (v_NasInd.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.ueNetworkCapability.cIoT_V2X)) { if ( substr(bit2str(v_NasInd.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.ueNetworkCapability.cIoT_V2X),2,1) == "0"){ f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 4"); } else { f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 4 : N1 mode supported"); } } else { f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 4"); } Please see below. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail the test cases |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.4.12.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	3

2	Corrections required	3

2.1 	Change 1	3

2.1	Change 2	9

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D24wk24 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## 2.1 Change 1

| Function name | fl_TC_11_4_12_NR5GC_EUTRA_TestBody |
| --- | --- |
| Reason for change | In the current TTCN implementation, the variable v_UENWCap is assigned cdr_NR5GC_UENetworkCap('58'O);, which initializes the IEs cIoT_proSeCap and cIoT_V2X. The variable v_UENWCap is then passed to function f_EUTRA38_TAUReqFromN1, and within this function the variable v_UENWCap is further initialized with following code: if (isvalue(p_UE_NetworkCap)) { // @sic R5-243593 sic@ v_UENWCap := p_UE_NetworkCap; } else { v_UENWCap := cdr_NR5GC_UENetworkCap('58'O); if (valueof(p_EstablishmentCause) == emergency) { // @sic R5-217714 sic@ v_UENWCap.cIoT_proSeCap := ?; // @sic R5-220644 sic@ v_UENWCap.cIoT_V2X := ? } } For testcase 11.4.12, the highlighted code is executed as a result of highlighted. This results in the testcase 11.4.12 failing, if the UE does not include the Octet 9 of the IE ueNetworkCapability. This needs to be addressed. |
| Summary of change | In the function fl_TC_11_4_12_NR5GC_EUTRA_TestBody, it is proposed to initialize v_UENWCap := cr_UENetworkCap_Any ('58'O);, and then pass v_UENWCap to function f_EUTRA38_TAUReqFromN1. In the function fl_TC_11_4_12_NR5GC_EUTRA_TestBody, it is proposed to implement following code to check that the support for N1 mode is set to ‘0’B : if (ispresent (v_NasInd.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.ueNetworkCapability.cIoT_V2X)) { if ( substr(bit2str(v_NasInd.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.ueNetworkCapability.cIoT_V2X),2,1) == "0"){ f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 4"); } else { f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 4 : N1 mode supported"); } } else { f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 4"); } Please see below. |
| TTCN module | Emergency_IRAT_NR5GC_EUTRA.ttcn |
| MCC160 Comments |  |

Before Change:

| function fl_TC_11_4_12_NR5GC_EUTRA_TestBody() runs on EUTRA_Multi5GS_PTC { var NAS_MSG_Indication_Type v_NasInd; var NasCount_Type v_NasCount; var template UE_AdditionalSecurityCapability v_UEAddSecurityCap; // @sic R5s230881 R5-241528 sic@ var template(present) UE_NetworkCap v_UENWCap := cdr_NR5GC_UENetworkCap('58'O); // @sic R5-243593 sic@ timer t_T1 := f_EUTRA_SetTimerToleranceMin (eutra_Cell1, nasTimer, 60.0); //@siclog "Step 0" siclog@ //@sic R5-223438 sic@ //Set the power levels according to T0 as per Table 11.4.12.3.1-1/2 f_EUTRA38_SetCellPower(eutra_Cell1, -91, -91); //@siclog "Step 1" siclog@ //Make the UE initiate an Emergency call. f_UT_RequestIMSEmergencyCall(UT); //@siclog "Step 2 - 4" siclog@ //Check: Does the UE transmit an RRCConnectionRequest message with 'establishmentCause' set to 'emergency'? //SS transmits an RRCConnectionSetup message. //Check: Does the UE transmit an RRCConnectionSetupComplete message to confirm the successful completion of the connection establishment and a TRACKING AREA UPDATE REQUEST message is sent to update the registration of the actual tracking area. //For some consequences related to the content of the TRACKING AREA UPDATE REQUEST message see the Notes in TS.36.508 [7], Table 4.5A.2.1-1. if (pc_EN_DC == true ) { // @sic R5s230881 R5-241528 sic@ v_UEAddSecurityCap := cr_UE_AddSecurityCap_Any; } else { v_UEAddSecurityCap := omit; } v_UENWCap.cIoT_V2X := (? length (2) & '1'B & ? length (5)) ifpresent; // @sic R5-243593 sic@ v_NasInd := f_EUTRA38_TAUReqFromN1(eutra_Cell1, tsc_RRC_TI_Def, -, MAPPED_CXT, -, emergency, -, v_UEAddSecurityCap,// @sic R5s230881 R5-241528 sic@ v_UENWCap //WA#11_4_12 valueof(v_UENWCap) // @sic R5-243593 sic@ ); f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 2"); f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 4"); v_NasCount := v_NasInd.SecurityProtection.NasCount; //@siclog "Steps 5-8" siclog@ //Steps 5-8 from the Generic Test Procedure for IMS Emergency call establishment in EUTRA: Normal Service as specified in TS 36.508 [7], Table 4.5A.4.3-1 are performed. f_EUTRA38_MultiPDN_Activate_SRB2_DRB_SendRrcMsg(eutra_Cell1, tsc_RRC_TI_Def, v_NasCount); f_EUTRA_Activate_SRB2_DRB_ReceiveRrcAndNasMsg(eutra_Cell1, tsc_RRC_TI_Def); //@siclog "Step 9" siclog@ //SS responds with TRACKING AREA UPDATE ACCEPT message. //@siclog "Step 10" siclog@ //The UE transmits a TRACKING AREA UPDATE COMPLETE message. f_EUTRA38_TrackingAreaUpdateFromN1_WithoutReq(eutra_Cell1, v_NasInd, MAPPED_CXT, noRrcConnectionRelease, -, true, -, true); //@siclog "Steps 11-17" siclog@ //Steps 9-15 from the Generic Test Procedure for IMS Emergency call establishment in EUTRA: Normal Service as specified in TS 36.508 [7], Table 4.5A.4.3-1 are performed. f_EUTRA38_MultiPDN_EstablishAdditionalPDNs(eutra_Cell1, 1, -, -, -, '100'B, omit, true); // @sic R5s211357 R5-231416 sic@ f_EUTRA38_MultiPDN_ActivateDedicatedEpsBearer_Speech(eutra_Cell1, Emergency_PDN); // @siclog "Step 18" siclog@ // The SS waits 1 second. f_Delay(1.0); // @siclog "Step 18A" siclog@ // Set the power levels according to "T1" as per Table 11.4.12.3.1-1/2. f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger("Change Power Level")); // @sic R5-241692 sic@ // @siclog "Step 19" siclog@ // Release IMS Call as specified in the generic procedure in TS 34.229-1 [35] subclause C.32. // Tell IMS PTC to release the call f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS2]); // @sic R5s221058 sic@ // Emergency call release in IMS f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS2], "Call Released"); // The SS deactivates the EPS bearer. [This is the rest of C.32] f_EUTRA_DeactivateEPS_BearerContext (eutra_Cell1, int2hex(valueof(f_Get_EPSBearerIdForAPNType(Emergency_PDN)), 1)); // @sic R5s211357 R5s221058 sic@ // @siclog "Step 20" siclog@ // The SS releases the RRC connection. f_EUTRA_RRC_ConnectionRelease(eutra_Cell1);// @sic R5-241692 sic@ // @siclog "Step 21" siclog@ t_T1.start; // @sic R5-241692 sic@ alt {// @sic R5-241692 sic@ // @siclog "Step 22a1" siclog@ // The UE transmits an RRCConnectionRequest. [] SRB.check(receive(car_SRB0_RrcPdu_IND(eutra_Cell1, cr_RRCConnectionRequest(cr_EstablishmentCause_Any)))) { // @siclog "Step 22a2" siclog@ // Stop Timer T1 t_T1.stop; // @siclog "Steps 22a3-22a7" siclog@ // Steps 2-6 of 36.508 Table 6.4.2.7-1 are executed.// @sic R5-243593 sic@ v_NasInd := f_EUTRA_RRC_ConnEst_DefWithNas(eutra_Cell1, tsc_RRC_TI_Def, ?, cr_NAS_Indication(tsc_SHT_IntegrityProtected, cr_TAU_Request_11_4_12(('000'B, '001'B, '010'B), //WA#11_4_12 ?, cr_DRXparameter_Any ('5C'O) ifpresent, f_GetAdditionalUpdateType(NORMAL), -, -, -, ?, -, -, cr_UeRadioCap ifpresent, -, -, cr_UE_AddSecurityCap_Any ifpresent))); f_EUTRA_TrackingAreaUpdate_Accept_Complete(eutra_Cell1, f_EUTRA_CellInfo_GetGuti(eutra_Cell1), f_GetEPSTAUType(NORMAL), v_NasInd.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.epsBearerContextStatus, v_NasInd.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.addUpdateType, NORMAL); f_EUTRA_RRC_ConnectionRelease(eutra_Cell1); //WA#11_4_12 } // @siclog "Step 22b1" siclog@ // Timer T1 expires. [] t_T1.timeout {} } } |  |
| --- | --- |

After Change:

function fl_TC_11_4_12_NR5GC_EUTRA_TestBody() runs on EUTRA_Multi5GS_PTC { var NAS_MSG_Indication_Type v_NasInd; var NasCount_Type v_NasCount; var template UE_AdditionalSecurityCapability v_UEAddSecurityCap; // @sic R5s230881 R5-241528 sic@ //WA#WI1306226 11_4_12 COMMENTED OUT var template(present) UE_NetworkCap v_UENWCap := cdr_NR5GC_UENetworkCap('58'O); // @sic R5-243593 sic@ var template(present) UE_NetworkCap v_UENWCap := cr_UENetworkCap_Any ('58'O); //WA#WI1306226 11_4_12 timer t_T1 := f_EUTRA_SetTimerToleranceMin (eutra_Cell1, nasTimer, 60.0); //@siclog "Step 0" siclog@ //@sic R5-223438 sic@ //Set the power levels according to T0 as per Table 11.4.12.3.1-1/2 f_EUTRA38_SetCellPower(eutra_Cell1, -91, -91); //@siclog "Step 1" siclog@ //Make the UE initiate an Emergency call. f_UT_RequestIMSEmergencyCall(UT); //@siclog "Step 2 - 4" siclog@ //Check: Does the UE transmit an RRCConnectionRequest message with 'establishmentCause' set to 'emergency'? //SS transmits an RRCConnectionSetup message. //Check: Does the UE transmit an RRCConnectionSetupComplete message to confirm the successful completion of the connection establishment and a TRACKING AREA UPDATE REQUEST message is sent to update the registration of the actual tracking area. //For some consequences related to the content of the TRACKING AREA UPDATE REQUEST message see the Notes in TS.36.508 [7], Table 4.5A.2.1-1. if (pc_EN_DC == true ) { // @sic R5s230881 R5-241528 sic@ v_UEAddSecurityCap := cr_UE_AddSecurityCap_Any; } else { v_UEAddSecurityCap := omit; } log("Parikshit Patch 4 for 11.4.12 "); //WA#WI1306226 11_4_12 v_UENWCap.cIoT_V2X := (? length (2) & '1'B & ? length (5)) ifpresent; // @sic R5-243593 sic@ v_NasInd := f_EUTRA38_TAUReqFromN1(eutra_Cell1, tsc_RRC_TI_Def, -, MAPPED_CXT, -, emergency, -, v_UEAddSecurityCap,// @sic R5s230881 R5-241528 sic@ v_UENWCap //WA#WI1306226 11_4_12 valueof(v_UENWCap) // @sic R5-243593 sic@ ); f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 2"); //WA#WI1306226 11_4_12 : Start if (ispresent (v_NasInd.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.ueNetworkCapability.cIoT_V2X)) { if ( substr(bit2str(v_NasInd.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.ueNetworkCapability.cIoT_V2X),2,1) == "0"){ f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 4"); } else { f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 4 : N1 mode supported"); } } else { f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 4"); } //WA#WI1306226 11_4_12 : Stop v_NasCount := v_NasInd.SecurityProtection.NasCount; //@siclog "Steps 5-8" siclog@ //Steps 5-8 from the Generic Test Procedure for IMS Emergency call establishment in EUTRA: Normal Service as specified in TS 36.508 [7], Table 4.5A.4.3-1 are performed. f_EUTRA38_MultiPDN_Activate_SRB2_DRB_SendRrcMsg(eutra_Cell1, tsc_RRC_TI_Def, v_NasCount); f_EUTRA_Activate_SRB2_DRB_ReceiveRrcAndNasMsg(eutra_Cell1, tsc_RRC_TI_Def); //@siclog "Step 9" siclog@ //SS responds with TRACKING AREA UPDATE ACCEPT message. //@siclog "Step 10" siclog@ //The UE transmits a TRACKING AREA UPDATE COMPLETE message. f_EUTRA38_TrackingAreaUpdateFromN1_WithoutReq(eutra_Cell1, v_NasInd, MAPPED_CXT, noRrcConnectionRelease, -, true, -, true); //@siclog "Steps 11-17" siclog@ //Steps 9-15 from the Generic Test Procedure for IMS Emergency call establishment in EUTRA: Normal Service as specified in TS 36.508 [7], Table 4.5A.4.3-1 are performed. f_EUTRA38_MultiPDN_EstablishAdditionalPDNs(eutra_Cell1, 1, -, -, -, '100'B, omit, true); // @sic R5s211357 R5-231416 sic@ f_EUTRA38_MultiPDN_ActivateDedicatedEpsBearer_Speech(eutra_Cell1, Emergency_PDN); // @siclog "Step 18" siclog@ // The SS waits 1 second. f_Delay(1.0); // @siclog "Step 18A" siclog@ // Set the power levels according to "T1" as per Table 11.4.12.3.1-1/2. f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger("Change Power Level")); // @sic R5-241692 sic@ // @siclog "Step 19" siclog@ // Release IMS Call as specified in the generic procedure in TS 34.229-1 [35] subclause C.32. // Tell IMS PTC to release the call f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS2]); // @sic R5s221058 sic@ // Emergency call release in IMS f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS2], "Call Released"); // The SS deactivates the EPS bearer. [This is the rest of C.32] f_EUTRA_DeactivateEPS_BearerContext (eutra_Cell1, int2hex(valueof(f_Get_EPSBearerIdForAPNType(Emergency_PDN)), 1)); // @sic R5s211357 R5s221058 sic@ // @siclog "Step 20" siclog@ // The SS releases the RRC connection. f_EUTRA_RRC_ConnectionRelease(eutra_Cell1);// @sic R5-241692 sic@ // @siclog "Step 21" siclog@ t_T1.start; // @sic R5-241692 sic@ alt {// @sic R5-241692 sic@ // @siclog "Step 22a1" siclog@ // The UE transmits an RRCConnectionRequest. [] SRB.check(receive(car_SRB0_RrcPdu_IND(eutra_Cell1, cr_RRCConnectionRequest(cr_EstablishmentCause_Any)))) { // @siclog "Step 22a2" siclog@ // Stop Timer T1 t_T1.stop; // @siclog "Steps 22a3-22a7" siclog@ // Steps 2-6 of 36.508 Table 6.4.2.7-1 are executed.// @sic R5-243593 sic@ v_NasInd := f_EUTRA_RRC_ConnEst_DefWithNas(eutra_Cell1, tsc_RRC_TI_Def, ?, cr_NAS_Indication(tsc_SHT_IntegrityProtected, cr_TAU_Request_UEStatus_N1(('000'B, '001'B, '010'B), //WA#WI1306226 11_4_12 ?, cr_DRXparameter_Any ('5C'O) ifpresent, f_GetAdditionalUpdateType(NORMAL), -, -, -, ?, -, -, cr_UeRadioCap ifpresent, -, -, cr_UE_AddSecurityCap_Any ifpresent))); f_EUTRA_TrackingAreaUpdate_Accept_Complete(eutra_Cell1, f_EUTRA_CellInfo_GetGuti(eutra_Cell1), f_GetEPSTAUType(NORMAL), v_NasInd.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.epsBearerContextStatus, v_NasInd.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.addUpdateType, NORMAL); f_EUTRA_RRC_ConnectionRelease(eutra_Cell1); //WA#WI1306226 11_4_12 } // @siclog "Step 22b1" siclog@ // Timer T1 expires. [] t_T1.timeout {} } }

## Change 2

| Function name | f_EUTRA38_TAUReqFromN1 |
| --- | --- |
| Reason for change | In the current TTCN implementation, the variable v_UENWCap is assigned cdr_NR5GC_UENetworkCap('58'O);, which initializes the IEs cIoT_proSeCap and cIoT_V2X. The variable v_UENWCap is then passed to function f_EUTRA38_TAUReqFromN1, and within this function the variable v_UENWCap is further initialized with following code: if (isvalue(p_UE_NetworkCap)) { // @sic R5-243593 sic@ v_UENWCap := p_UE_NetworkCap; } else { v_UENWCap := cdr_NR5GC_UENetworkCap('58'O); if (valueof(p_EstablishmentCause) == emergency) { // @sic R5-217714 sic@ v_UENWCap.cIoT_proSeCap := ?; // @sic R5-220644 sic@ v_UENWCap.cIoT_V2X := ? } } For testcase 11.4.12, the highlighted code is executed as a result of highlighted. This results in the testcase 11.4.12 failing, if the UE does not include the Octet 9 of the IE ueNetworkCapability. This needs to be addressed. |
| Summary of change | Within function f_EUTRA38_TAUReqFromN1, it is proposed to implement following code: if (isvalue(p_UE_NetworkCap)) { // @sic R5-243593 sic@ v_UENWCap := p_UE_NetworkCap; } else { if(testcasename() == "TC_11_4_12_NR5GC"){ v_UENWCap := cr_UENetworkCap_Any('58'O); }else { v_UENWCap := cdr_NR5GC_UENetworkCap('58'O); } //WA#WI1306226 11_4_12 v_UENWCap := cdr_NR5GC_UENetworkCap('58'O); if (valueof(p_EstablishmentCause) == emergency and (testcasename() != "TC_11_4_12_NR5GC")) { // @sic R5-217714 sic@ //WA#WI1306226 11_4_12 v_UENWCap.cIoT_proSeCap := ?; // @sic R5-220644 sic@ v_UENWCap.cIoT_V2X := ? } } |
| TTCN module | Eutra38_NR5GCProcedures.ttcn |
| MCC160 Comments |  |

Before Change:

<<SKIPPED CODE>> // If we need a new algorithm, have to store the new values before calling this // tell NAS EMU to check next message with 5G integrity f_EUTRA_NB_SS_NAS_SecurityConfig(NASCTRL, cas_NasSecurity_5GIntegrity_REQ(true)); if (not p_FirstN1_S1Transition) { // @sic R5s200583 sic@ v_UeRadioCap := cr_UeRadioCap ifpresent; } if (isvalue(p_UE_NetworkCap)) { // @sic R5-243593 sic@ v_UENWCap := p_UE_NetworkCap; } else { v_UENWCap := cdr_NR5GC_UENetworkCap('58'O); if (valueof(p_EstablishmentCause) == emergency) { // @sic R5-217714 sic@ v_UENWCap.cIoT_proSeCap := ?; // @sic R5-220644 sic@ v_UENWCap.cIoT_V2X := ? } } /* Steps 1-3 */ //Receive RRC Connection Setup Complete with piggy-backed NAS message on SRB1 (DCCH): <<SKIPPED CODE>>

After Change:

## <<SKIPPED CODE>> // If we need a new algorithm, have to store the new values before calling this // tell NAS EMU to check next message with 5G integrity f_EUTRA_NB_SS_NAS_SecurityConfig(NASCTRL, cas_NasSecurity_5GIntegrity_REQ(true)); if (not p_FirstN1_S1Transition) { // @sic R5s200583 sic@ v_UeRadioCap := cr_UeRadioCap ifpresent; } if (isvalue(p_UE_NetworkCap)) { // @sic R5-243593 sic@ v_UENWCap := p_UE_NetworkCap; } else { if(testcasename() == "TC_11_4_12_NR5GC"){ v_UENWCap := cr_UENetworkCap_Any('58'O); }else { v_UENWCap := cdr_NR5GC_UENetworkCap('58'O); } //WA#WI1306226 11_4_12 v_UENWCap := cdr_NR5GC_UENetworkCap('58'O); if (valueof(p_EstablishmentCause) == emergency and (testcasename() != "TC_11_4_12_NR5GC")) { // @sic R5-217714 sic@ //WA#WI1306226 11_4_12 v_UENWCap.cIoT_proSeCap := ?; // @sic R5-220644 sic@ v_UENWCap.cIoT_V2X := ? } } /* Steps 1-3 */ //Receive RRC Connection Setup Complete with piggy-backed NAS message on SRB1 (DCCH): <<SKIPPED CODE>>