3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240506

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3493 | rev | - | Current version: | 17.11.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to ENDC RRC Test case 8.2.3.11.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-07-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Correction required in TTCN to prepare measconfig based on UE Measurement Capabilities |  |  |  |  |  |  |  |  |
| Summary of change: |  | TC was failing at step5-7 as TTCN was not constructing measconfig configuration according to PICS settings. So, pc_gp2_gp3_en_dc pics defined in TTCN and corrected fl_TC_8_2_3_11_1_ENDC_EUTRA_TestBody() fl_TC_8_2_3_11_1_ENDC_NR_TestBody() |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.3.11.1.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D24wk24 related to the title of this CR.

Contact:	Kalahasti, Narendra

Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | Module EUTRA_NR_Parameters |
| --- | --- |
| Reason for change | Added missing pc_gp2_gp3_en_dc PICS as per spec 38.508-02 Table A.4.3.6-1: UE Measurement Capabilities |
| Summary of change | pc_gp2_gp3_en_dc PICS are not available in PCT UUT as TTCN has not define in EUTRA_NR_Parameters module. |
| TTCN module | EUTRA_NR_Parameters |
| MCC160 Comment |  |

Before Change

module EUTRA_NR_Parameters { import from Common4G5G all; modulepar integer pc_noOf_PDNsNewConnection; // @desc Number of UE requested PDN connection establishments after ATTACH in a new signaling connection // Reference to other Spec: 38.508-2 Table A.4.4-2/6 modulepar integer pc_noOf_PDNsSameConnection; . . . . modulepar boolean pc_interRAT_NR_ToENDC := false; // @desc Support inter-RAT Handover from NR to EN-DC // Reference: 38.508-2 Table A.4.3.8-1/10 modulepar boolean pc_ra_Type0_PUSCH := false; // @desc Support resource allocation Type 0 for PUSCH // Reference: 38.508-2 Table A.4.3.2-1/7 modulepar boolean pc_srb3 := false; // @desc Support direct SRB between the SN and the UE // Reference: 38.508-2 Table A.4.3.7-1/3 modulepar boolean pc_ss_SINR_Meas := false; // @desc Support SS-SINR measurements // Reference: 38.508-2 Table A.4.3.6-1/6 }

After Change

module EUTRA_NR_Parameters { import from Common4G5G all; modulepar integer pc_noOf_PDNsNewConnection; // @desc Number of UE requested PDN connection establishments after ATTACH in a new signaling connection // Reference to other Spec: 38.508-2 Table A.4.4-2/6 modulepar integer pc_noOf_PDNsSameConnection; . . . . modulepar boolean pc_gp2_gp3_en_dc := false; modulepar boolean pc_interRAT_NR_ToENDC := false; // @desc Support inter-RAT Handover from NR to EN-DC // Reference: 38.508-2 Table A.4.3.8-1/10 modulepar boolean pc_ra_Type0_PUSCH := false; // @desc Support resource allocation Type 0 for PUSCH // Reference: 38.508-2 Table A.4.3.2-1/7 modulepar boolean pc_srb3 := false; // @desc Support direct SRB between the SN and the UE // Reference: 38.508-2 Table A.4.3.7-1/3 modulepar boolean pc_ss_SINR_Meas := false; // @desc Support SS-SINR measurements // Reference: 38.508-2 Table A.4.3.6-1/6 }

## 2.2 Change 2

| Function name | f_EUTRA_ENDC_DeriveSupportedGapsFromPics() |
| --- | --- |
| Reason for change | To modify the implementation |
| Summary of change | Modified and added required PICS. |
| TTCN module | RRCMeasurement_ENDC_EUTRA |
| MCC160 Comment |  |

Before Change

function f_EUTRA_ENDC_DeriveSupportedGapsFromPics() return B8_Type // @sic R5s230787 sic@ { var B8_Type v_SupportedGapPatterns := f_ConvertBoolToBit(pc_gp11_en_dc) & f_ConvertBoolToBit(pc_gp10_en_dc) & f_ConvertBoolToBit(pc_gp9_en_dc) & f_ConvertBoolToBit(pc_gp8_en_dc) & f_ConvertBoolToBit(pc_gp7_en_dc) & f_ConvertBoolToBit(pc_gp6_en_dc) & f_ConvertBoolToBit(pc_gp5_en_dc) & f_ConvertBoolToBit(pc_gp4_en_dc); // @sic R5s230787 sic@ return v_SupportedGapPatterns; }

After Change

function f_EUTRA_ENDC_DeriveSupportedGapsFromPics() return B9_Type // @sic R5s230787 sic@ { var B9_Type v_SupportedGapPatterns := f_ConvertBoolToBit(pc_gp11_en_dc) & f_ConvertBoolToBit(pc_gp10_en_dc) & f_ConvertBoolToBit(pc_gp9_en_dc) & f_ConvertBoolToBit(pc_gp8_en_dc) & f_ConvertBoolToBit(pc_gp7_en_dc) & f_ConvertBoolToBit(pc_gp6_en_dc) & f_ConvertBoolToBit(pc_gp5_en_dc) & f_ConvertBoolToBit(pc_gp4_en_dc)& f_ConvertBoolToBit(pc_gp2_gp3_en_dc); // @sic R5s230787 sic@ return v_SupportedGapPatterns; }

## 2.3 Change 3

| Function name | fl_TC_8_2_3_11_1_ENDC_EUTRA_TestBody() |
| --- | --- |
| Reason for change | To modify the implementation |
| Summary of change | Corrected variables, modified step5-7 & step15-17 to include counter 11 in loop |
| TTCN module | RRCMeasurement_ENDC_EUTRA |
| MCC160 Comment |  |

Before Change

function fl_TC_8_2_3_11_1_ENDC_EUTRA_TestBody() runs on EUTRA_5GS_PTC { var RRC_TransactionIdentifier v_RRC_TI := tsc_RRC_TI_Def; var Frequency_IE_Type v_Frequency_IE_f1; var ChannelBandwidthDependency_Type v_ChBandDependency_f1; var EUTRA_NR_CoOrd_SysInfo_Type v_EUTRA_NR_SysInfo; var integer v_NR_PhysChId_Cell3; var integer v_NR_ARFCN_f2; var RS_ConfigSSB_NR_r15.subcarrierSpacingSSB_r15 v_SCS; var FreqBandIndicatorNR_r15 v_FreqBandIndicatorNR_r15; //@sic R5-194875 sic@ var ReportConfigInterRAT.reportAmount v_ReportAmount := infinity_; var FilterCoefficient v_FilterCoefficientRSRP := fc4; var FilterCoefficient v_FilterCoefficientRSRQ := fc4; var FilterCoefficient v_FilterCoefficientSINR := fc4; var AbsoluteCellPower_Type v_CellPower_T0 := -85; var default v_DefaultRef; var B8_Type v_SupportedGaps := substr(f_EUTRA_ENDC_DeriveSupportedGapsFromPics(), 0, 8); // @sic R5s230787 sic@ var B8_Type v_SupportedGaps_step5to7 := substr(f_EUTRA_ENDC_DeriveSupportedGapsFromPics(), 0, 8); // @sic R5s230787 sic@ var B8_Type v_GapMask := int2bit(1, 8); //@sic R5s240058 sic@ var integer v_Counter; var template (omit) RRCConnectionReconfiguration_v890_IEs v_RRCConnectionReconfiguration_v890_IEs := omit; // @sic R5s230787 sic@ var float v_TimerValue_8s; var float v_TimerValue_10s; var float v_TimerValue_30s; var template (value) DL_DCCH_Message v_RRCConnectionReconfiguration; var template (omit) octetstring v_NR_SCGConfig1 := omit; // @sic R5s230787 sic@ var boolean v_ENDC_ReleaseAndAdd_r15 := false; // @sic R5s230787 sic@ var float v_TimerValue_480ms; // @sic R5s230787 sic@ var boolean v_FirstReportReceived := false; // @sic R5s230787 sic@ var boolean v_MeasReport_Flag := false; // @sic R5s230787 sic@ var boolean v_RRC_flag:= false; // @sic R5s230787 sic@ var boolean v_Timeout:= true; // @sic R5s230787 sic@ var EUTRA38_SecurityParamsExtd_Type v_Security_Params; var SRB_COMMON_IND v_ReceivedMsg; // @sic R5s230787 sic@ timer t_Watchdog5s := 5.0; //5s // @sic R5s230787 sic@ timer t_Watchdog := 30.0; //30s // @sic R5s230787 sic@ timer t_TransmissionInterval; // @sic R5s230787 sic@ v_TimerValue_30s := f_EUTRA_SetTimerToleranceMax(eutra_Cell1, nonProtocolTimer, 30.0); v_TimerValue_10s := f_EUTRA_SetTimerToleranceMax(eutra_Cell1, nonProtocolTimer, 10.0); v_TimerValue_8s := f_EUTRA_SetTimerToleranceMax(eutra_Cell1, nonProtocolTimer, 8.0); v_Frequency_IE_f1 := f_EUTRA_CellInfo_GetFrequencyIEs(eutra_Cell1); v_ChBandDependency_f1 := f_EUTRA_BandDependentParam(v_Frequency_IE_f1.DL_ChBandwidth, v_Frequency_IE_f1.UL_ChBandwidth); //Acquire information of NR cells (Cell1 and Cell3) v_EUTRA_NR_SysInfo := f_EUTRA_NR_WaitForCoOrd_SysInfo(NR); v_NR_PhysChId_Cell3 := v_EUTRA_NR_SysInfo.NR[0].PhysCellId; // @sic R5s230787 sic@ v_NR_ARFCN_f2 := v_EUTRA_NR_SysInfo.NR[0].Arfcn; // @sic R5s230787 sic@ v_SCS := f_NR_ConvertIntToSCS (v_EUTRA_NR_SysInfo.NR[0].SCS); v_FreqBandIndicatorNR_r15 := v_EUTRA_NR_SysInfo.NR[0].Band; //@sic R5-194875 sic@ // Set eutra_Cell1 power level "T0" in Table 8.2.3.11.1.3.2-1 f_EUTRA38_SetCellPower(eutra_Cell1, v_CellPower_T0, v_CellPower_T0); //@siclog "Step 1-2" siclog@ //The SS transmits an RRCConnectionReconfiguration message to setup fr1-Gap to report periodical measurements for //for EUTRA serving Cell 1 and NR neighbour Cell 3 on FR1 frequency and receives an RRCConnectionReconfigurationComplete //message to confirm the setup of fr1-Gap and report periodical measurements for EUTRA serving Cell 1 and //NR neighbour Cell 3 on FR1 frequency f_EUTRA_RRCConnectionReconfiguration_MeasConfig(eutra_Cell1, cs_RRCConnectionReconfiguration_NR_2Entries_Gap(v_RRC_TI, v_Frequency_IE_f1.UL_DL_Earfcn.dl_CarrierFreq_r9, v_ChBandDependency_f1.AllowedMeasBandwidth, v_NR_ARFCN_f2, v_SCS, v_FreqBandIndicatorNR_r15, v_ReportAmount, cs_QuantityConfig_NR(cs_QuantityConfig_NRList(v_FilterCoefficientRSRP, v_FilterCoefficientRSRQ, v_FilterCoefficientSINR)), cs_MeasGapConfig_setup_gp0(9), //@sic R5-203403 sic@ cs_ReportQuantityNR_r15(true, true, false), true, true)); //@siclog "Step 3" siclog@ //Wait and ignore Measurement Report messages for 8s to allow UE to measure the neighbouring cells f_EUTRA_WaitAndIgnoreMeasurementReport(eutra_Cell1, v_TimerValue_8s, cr_MeasurementReport_CheckAny); //@siclog "Step 4" siclog@ //Wait for 30 s to ensure that the UE performs a periodical inter frequency reporting. //EXCEPTION: In parallel to events described in step 4 the steps specified in table 8.2.3.11.1.3.2-3 shall take place f_Check_EUTRA_HighFreq_PeriodicalMeasurmentReport(eutra_Cell1, v_TimerValue_30s, cr_MeasurementReport(1, ?, ?, cr_MeasResultNeighCells_measResultListNR_1Entry(v_NR_PhysChId_Cell3, ?, omit, omit, omit, ?)), //@sic R5s230787 sic@ 1.024); v_DefaultRef := activate(a_EUTRA_AddDefault_PeriodicalMeasurmentReport(eutra_Cell1, cr_MeasurementReport_CheckAny)); //@siclog "Step 5-6" siclog@ //The SS transmits an RRCConnectionReconfiguration message including measConfig to change fr1-Gap //and receives an RRCConnectionReconfigurationComplete message to confirm the change of fr1-Gap f_EUTRA_RRCConnectionReconfiguration_MeasConfig(eutra_Cell1, cs_508_RRCConnectionReconfiguration_ConditionMEAS(v_RRC_TI, cs_MeasConfig(omit, omit, omit, omit, omit, omit, omit, cs_MeasGapConfig_setup_gp1(9), //@sic R5-203403 sic@ omit, omit, omit, omit, true, true))); //@siclog "Step 7" siclog@ //Wait for 30 s to ensure that the UE performs a periodical inter frequency reporting. //EXCEPTION: In parallel to events described in step 7 the steps specified in table 8.2.3.11.1.3.2-3 shall take place f_Check_EUTRA_HighFreq_PeriodicalMeasurmentReport(eutra_Cell1, v_TimerValue_30s, cr_MeasurementReport(1, ?, ?, cr_MeasResultNeighCells_measResultListNR_1Entry(v_NR_PhysChId_Cell3, ?, omit, omit, omit, ?)), //@sic R5s230787 sic@ 1.024); v_DefaultRef := activate(a_EUTRA_AddDefault_PeriodicalMeasurmentReport(eutra_Cell1, cr_MeasurementReport_CheckAny)); //EXCEPTION: Steps 5 to 7 shall be repeated for k=1 to 11 (increment=1) v_RRCConnectionReconfiguration := cs_508_RRCConnectionReconfiguration_ConditionMEAS(v_RRC_TI, cs_MeasConfig(omit, omit, omit, omit, omit, omit, omit, cs_MeasGapConfig_setup_gp1(9), //@sic R5-203403 sic@ omit, omit, omit, omit, true, true)); //@siclog "Step 5-7" siclog@ //EXCEPTION: Steps 5 to 7 shall be repeated for k=1 to 11 (increment=1) v_Counter := 1; while (v_Counter < 11) { if(v_Counter != 1) { if (bit2int(v_SupportedGaps_step5to7 and4b v_GapMask) == 1) // @sic R5s230787 sic@ { fl_ReconfigFr1GapAndGetMeasResults(v_Counter, v_NR_PhysChId_Cell3, v_RRCConnectionReconfiguration); } v_SupportedGaps_step5to7 := v_SupportedGaps_step5to7 @> 1; // @sic R5s230787 sic@ } v_Counter := v_Counter + 1; } //@siclog "Step 8-9" siclog@ //The SS transmits an RRCConnectionReconfiguration message including measConfig to release fr1-Gap //and measid of periodical measurements and receives an RRCConnectionReconfigurationComplete message f_EUTRA_RRCConnectionReconfiguration_MeasConfig(eutra_Cell1, cs_508_RRCConnectionReconfiguration_ConditionMEAS(v_RRC_TI, cs_MeasConfig(omit, omit, omit, omit, cs_MeasIdToRemoveList_1Entry(1), omit, omit, cs_MeasGapConfig_release))); //@siclog "Step 10" siclog@ //Wait 10s f_Delay(v_TimerValue_10s); //@siclog "Step 11-12" siclog@ //The SS transmits an RRCConnectionReconfiguration including measConfig to setup fr1-Gap and //nr-Config IE containing measConfig to report periodical measurements for NR serving Cell 1 and //NR neighbor cell 3 on FR1 frequency //The UE transmits an RRCConnectionReconfigurationComplete message to confirm the setup of fr1-Gap //and report periodical measurements for NR serving Cell 1 and NR neighbor cell 3 on FR1 frequency v_Security_Params := f_EUTRA38_ENDC_InitiateAndSendS_gNBKeys(NR); f_EUTRA38_508_ENDC_Reconfiguration_MeasGap(eutra_Cell1, -, v_Security_Params.SK_Counter); //@siclog "Step 13" siclog@ //Wait and ignore MeasurementReport messages for 8 s to allow for UE to measure the neighbouring cells. v_DefaultRef := activate(a_EUTRA_AddDefault_PeriodicalMeasurmentReport(eutra_Cell1, cr_ULInformationTransferMRDC)); f_Delay(v_TimerValue_8s); //@sic R5s230787 sic@ //Deactivate the default to stop receiving periodical Measurement Report deactivate(v_DefaultRef); //@sic R5s230787 sic@ //v_TimerValue_480ms := f_EUTRA_SetTimerToleranceMax(eutra_Cell1, rrcTimer, 0.480); //@sic R5s230787 sic@ v_TimerValue_480ms := f_EUTRA_SetTimerToleranceMin(eutra_Cell1, rrcTimer, 0.480); //@sic R5s240058, R5s230787 sic@ f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger("StartTimer")); //@sic R5s230787 sic@ t_Watchdog.start; //@sic R5s230787 sic@ alt { //@sic R5s230787 sic@ [not(v_FirstReportReceived)] a_EUTRA_ReceiveULInformationTransferMRDC(eutra_Cell1) { v_FirstReportReceived := true; t_TransmissionInterval.start(v_TimerValue_480ms); repeat; } [v_FirstReportReceived] a_EUTRA_ReceiveULInformationTransferMRDC(eutra_Cell1) { if(t_TransmissionInterval.running) { f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 14. Measurement report is received while transmission interval has not expired"); t_TransmissionInterval.stop; } t_TransmissionInterval.start(v_TimerValue_480ms); repeat; } [] t_Watchdog.timeout { t_TransmissionInterval.stop; //continue TC execution } [] t_TransmissionInterval.timeout //@sic R5s240058 sic@ { repeat; } } //@sic R5s230787 sic@ v_DefaultRef := activate(a_EUTRA_AddDefault_PeriodicalMeasurmentReport(eutra_Cell1, cr_ULInformationTransferMRDC)); //@sic R5s240058 sic@ //@siclog "Step 15-17" siclog@ //EXCEPTION: Steps 15 to 17 shall be repeated for k=1 to 11 (increment=1) v_RRCConnectionReconfiguration := cs_508_RRCConnectionReconfiguration_ConditionMEAS(v_RRC_TI, cs_MeasConfig(omit, omit, omit, omit, omit, omit, omit, cs_MeasGapConfig_setup_gp1(9), //@sic R5-203403 sic@ omit, omit, omit, omit, true, true)); v_Counter := 1; while (v_Counter < 11) { if(v_Counter != 1) { if (bit2int(v_SupportedGaps and4b v_GapMask) == 1) { deactivate(v_DefaultRef); //@sic R5s240058 sic@ fl_ReconfigFr1GapAndGetMeasResults_steps15_17(v_Counter, v_RRCConnectionReconfiguration); //@sic R5s230787 sic@ } v_SupportedGaps := v_SupportedGaps @> 1; } v_Counter := v_Counter + 1; } //@siclog "Step 18-19" siclog@ //SS transmits an RRCConnectionReconfiguration message including measConfig to release fr1-Gap and measid of periodical measurements. //The UE transmits an RCConnectionReconfigrationComplete message v_NR_SCGConfig1 := f_EUTRA_NR_WaitForCoOrd_Data(NR,"Reconfig"); //@sic R5s230787 sic@ v_RRCConnectionReconfiguration_v890_IEs := f_EUTRA38_508_BuildRRCConnReconfig_NR_Config_Setup (v_NR_SCGConfig1, omit, -, -, omit, v_ENDC_ReleaseAndAdd_r15); //@sic R5s230787 sic@ SRB.send(cas_SRB1_RrcNasPduList_REQ(eutra_Cell1, cs_TimingInfo_Now, cs_RRCConnectionReconfiguration_Common(v_RRC_TI, cs_MeasConfig(omit, omit, omit, omit, cs_MeasIdToRemoveList_1Entry(2), omit, omit, cs_MeasGapConfig_release), omit, omit, omit, v_RRCConnectionReconfiguration_v890_IEs), omit)); //@sic R5s230787 sic@ deactivate(v_DefaultRef); //@sic R5s240058 sic@ t_Watchdog5s.start; //@sic R5s230787 sic@ do { //@sic R5s230787 sic@ alt { [] SRB.receive(car_SRB1_RrcPdu_IND(eutra_Cell1, cr_RRCConnectionReconfigurationComplete_r15))-> value v_ReceivedMsg { f_EUTRA_NR_SendCoOrd(NR, cms_EUTRA_NR_OctetData (v_ReceivedMsg.Signalling.Rrc.Dcch.message_.c1.rrcConnectionReconfigurationComplete.criticalExtensions.rrcConnectionReconfigurationComplete_r8.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.scg_ConfigResponseNR_r15, "Complete")); v_RRC_flag := true; if (v_MeasReport_Flag == true) {t_Watchdog5s.stop; v_Timeout := false; } //rrcCompleteisReceive } [] SRB.receive(car_SRB1_RrcPdu_IND(eutra_Cell1, cr_ULInformationTransferMRDC)) { v_MeasReport_Flag := true; if (v_RRC_flag==true) {t_Watchdog5s.stop; v_Timeout :=false; } //received report } [] t_Watchdog5s.timeout { t_Watchdog5s.stop; v_Timeout := false; //continue TC execution } } } while(v_Timeout); //@sic R5s230787 sic@ //End TestBody }

After Change

function fl_TC_8_2_3_11_1_ENDC_EUTRA_TestBody() runs on EUTRA_5GS_PTC { var RRC_TransactionIdentifier v_RRC_TI := tsc_RRC_TI_Def; var Frequency_IE_Type v_Frequency_IE_f1; var ChannelBandwidthDependency_Type v_ChBandDependency_f1; var EUTRA_NR_CoOrd_SysInfo_Type v_EUTRA_NR_SysInfo; var integer v_NR_PhysChId_Cell3; var integer v_NR_ARFCN_f2; var RS_ConfigSSB_NR_r15.subcarrierSpacingSSB_r15 v_SCS; var FreqBandIndicatorNR_r15 v_FreqBandIndicatorNR_r15; //@sic R5-194875 sic@ var ReportConfigInterRAT.reportAmount v_ReportAmount := infinity_; var FilterCoefficient v_FilterCoefficientRSRP := fc4; var FilterCoefficient v_FilterCoefficientRSRQ := fc4; var FilterCoefficient v_FilterCoefficientSINR := fc4; var AbsoluteCellPower_Type v_CellPower_T0 := -85; var default v_DefaultRef; var B9_Type v_SupportedGaps := substr(f_EUTRA_ENDC_DeriveSupportedGapsFromPics(), 0, 9); // @sic R5s230787 sic@ var B9_Type v_SupportedGaps_step5to7 := substr(f_EUTRA_ENDC_DeriveSupportedGapsFromPics(), 0, 9); // @sic R5s230787 sic@ var B9_Type v_GapMask := int2bit(1, 9); //@sic R5s240058 sic@ var integer v_Counter; var template (omit) RRCConnectionReconfiguration_v890_IEs v_RRCConnectionReconfiguration_v890_IEs := omit; // @sic R5s230787 sic@ var float v_TimerValue_8s; var float v_TimerValue_10s; var float v_TimerValue_30s; var template (value) DL_DCCH_Message v_RRCConnectionReconfiguration; var template (omit) octetstring v_NR_SCGConfig1 := omit; // @sic R5s230787 sic@ var boolean v_ENDC_ReleaseAndAdd_r15 := false; // @sic R5s230787 sic@ var float v_TimerValue_480ms; // @sic R5s230787 sic@ var boolean v_FirstReportReceived := false; // @sic R5s230787 sic@ var boolean v_MeasReport_Flag := false; // @sic R5s230787 sic@ var boolean v_RRC_flag:= false; // @sic R5s230787 sic@ var boolean v_Timeout:= true; // @sic R5s230787 sic@ var EUTRA38_SecurityParamsExtd_Type v_Security_Params; var SRB_COMMON_IND v_ReceivedMsg; // @sic R5s230787 sic@ timer t_Watchdog5s := 5.0; //5s // @sic R5s230787 sic@ timer t_Watchdog := 30.0; //30s // @sic R5s230787 sic@ timer t_TransmissionInterval; // @sic R5s230787 sic@ v_TimerValue_30s := f_EUTRA_SetTimerToleranceMax(eutra_Cell1, nonProtocolTimer, 30.0); v_TimerValue_10s := f_EUTRA_SetTimerToleranceMax(eutra_Cell1, nonProtocolTimer, 10.0); v_TimerValue_8s := f_EUTRA_SetTimerToleranceMax(eutra_Cell1, nonProtocolTimer, 8.0); v_Frequency_IE_f1 := f_EUTRA_CellInfo_GetFrequencyIEs(eutra_Cell1); v_ChBandDependency_f1 := f_EUTRA_BandDependentParam(v_Frequency_IE_f1.DL_ChBandwidth, v_Frequency_IE_f1.UL_ChBandwidth); //Acquire information of NR cells (Cell1 and Cell3) v_EUTRA_NR_SysInfo := f_EUTRA_NR_WaitForCoOrd_SysInfo(NR); v_NR_PhysChId_Cell3 := v_EUTRA_NR_SysInfo.NR[0].PhysCellId; // @sic R5s230787 sic@ v_NR_ARFCN_f2 := v_EUTRA_NR_SysInfo.NR[0].Arfcn; // @sic R5s230787 sic@ v_SCS := f_NR_ConvertIntToSCS (v_EUTRA_NR_SysInfo.NR[0].SCS); v_FreqBandIndicatorNR_r15 := v_EUTRA_NR_SysInfo.NR[0].Band; //@sic R5-194875 sic@ // Set eutra_Cell1 power level "T0" in Table 8.2.3.11.1.3.2-1 f_EUTRA38_SetCellPower(eutra_Cell1, v_CellPower_T0, v_CellPower_T0); //@siclog "Step 1-2" siclog@ //The SS transmits an RRCConnectionReconfiguration message to setup fr1-Gap to report periodical measurements for //for EUTRA serving Cell 1 and NR neighbour Cell 3 on FR1 frequency and receives an RRCConnectionReconfigurationComplete //message to confirm the setup of fr1-Gap and report periodical measurements for EUTRA serving Cell 1 and //NR neighbour Cell 3 on FR1 frequency f_EUTRA_RRCConnectionReconfiguration_MeasConfig(eutra_Cell1, cs_RRCConnectionReconfiguration_NR_2Entries_Gap(v_RRC_TI, v_Frequency_IE_f1.UL_DL_Earfcn.dl_CarrierFreq_r9, v_ChBandDependency_f1.AllowedMeasBandwidth, v_NR_ARFCN_f2, v_SCS, v_FreqBandIndicatorNR_r15, v_ReportAmount, cs_QuantityConfig_NR(cs_QuantityConfig_NRList(v_FilterCoefficientRSRP, v_FilterCoefficientRSRQ, v_FilterCoefficientSINR)), cs_MeasGapConfig_setup_gp0(9), //@sic R5-203403 sic@ cs_ReportQuantityNR_r15(true, true, false), true, true)); //@siclog "Step 3" siclog@ //Wait and ignore Measurement Report messages for 8s to allow UE to measure the neighbouring cells f_EUTRA_WaitAndIgnoreMeasurementReport(eutra_Cell1, v_TimerValue_8s, cr_MeasurementReport_CheckAny); //@siclog "Step 4" siclog@ //Wait for 30 s to ensure that the UE performs a periodical inter frequency reporting. //EXCEPTION: In parallel to events described in step 4 the steps specified in table 8.2.3.11.1.3.2-3 shall take place f_Check_EUTRA_HighFreq_PeriodicalMeasurmentReport(eutra_Cell1, v_TimerValue_30s, cr_MeasurementReport(1, ?, ?, cr_MeasResultNeighCells_measResultListNR_1Entry(v_NR_PhysChId_Cell3, ?, omit, omit, omit, ?)), //@sic R5s230787 sic@ 1.024); v_DefaultRef := activate(a_EUTRA_AddDefault_PeriodicalMeasurmentReport(eutra_Cell1, cr_MeasurementReport_CheckAny)); //@siclog "Step 5-6" siclog@ //The SS transmits an RRCConnectionReconfiguration message including measConfig to change fr1-Gap //and receives an RRCConnectionReconfigurationComplete message to confirm the change of fr1-Gap f_EUTRA_RRCConnectionReconfiguration_MeasConfig(eutra_Cell1, cs_508_RRCConnectionReconfiguration_ConditionMEAS(v_RRC_TI, cs_MeasConfig(omit, omit, omit, omit, omit, omit, omit, cs_MeasGapConfig_setup_gp1(9), //@sic R5-203403 sic@ omit, omit, omit, omit, true, true))); //@siclog "Step 7" siclog@ //Wait for 30 s to ensure that the UE performs a periodical inter frequency reporting. //EXCEPTION: In parallel to events described in step 7 the steps specified in table 8.2.3.11.1.3.2-3 shall take place f_Check_EUTRA_HighFreq_PeriodicalMeasurmentReport(eutra_Cell1, v_TimerValue_30s, cr_MeasurementReport(1, ?, ?, cr_MeasResultNeighCells_measResultListNR_1Entry(v_NR_PhysChId_Cell3, ?, omit, omit, omit, ?)), //@sic R5s230787 sic@ 1.024); v_DefaultRef := activate(a_EUTRA_AddDefault_PeriodicalMeasurmentReport(eutra_Cell1, cr_MeasurementReport_CheckAny)); //EXCEPTION: Steps 5 to 7 shall be repeated for k=1 to 11 (increment=1) v_RRCConnectionReconfiguration := cs_508_RRCConnectionReconfiguration_ConditionMEAS(v_RRC_TI, cs_MeasConfig(omit, omit, omit, omit, omit, omit, omit, cs_MeasGapConfig_setup_gp1(9), //@sic R5-203403 sic@ omit, omit, omit, omit, true, true)); //@siclog "Step 5-7" siclog@ //EXCEPTION: Steps 5 to 7 shall be repeated for k=1 to 11 (increment=1) v_Counter := 1; while (v_Counter <= 11) { if(v_Counter != 1) { if (bit2int(v_SupportedGaps_step5to7 and4b v_GapMask) == 1) // @sic R5s230787 sic@ { fl_ReconfigFr1GapAndGetMeasResults(v_Counter, v_NR_PhysChId_Cell3, v_RRCConnectionReconfiguration); } v_SupportedGaps_step5to7 := v_SupportedGaps_step5to7 @> 1; // @sic R5s230787 sic@ } if(v_Counter == 2) {v_Counter := v_Counter + 1; } v_Counter := v_Counter + 1; } //@siclog "Step 8-9" siclog@ //The SS transmits an RRCConnectionReconfiguration message including measConfig to release fr1-Gap //and measid of periodical measurements and receives an RRCConnectionReconfigurationComplete message f_EUTRA_RRCConnectionReconfiguration_MeasConfig(eutra_Cell1, cs_508_RRCConnectionReconfiguration_ConditionMEAS(v_RRC_TI, cs_MeasConfig(omit, omit, omit, omit, cs_MeasIdToRemoveList_1Entry(1), omit, omit, cs_MeasGapConfig_release))); //@siclog "Step 10" siclog@ //Wait 10s f_Delay(v_TimerValue_10s); //@siclog "Step 11-12" siclog@ //The SS transmits an RRCConnectionReconfiguration including measConfig to setup fr1-Gap and //nr-Config IE containing measConfig to report periodical measurements for NR serving Cell 1 and //NR neighbor cell 3 on FR1 frequency //The UE transmits an RRCConnectionReconfigurationComplete message to confirm the setup of fr1-Gap //and report periodical measurements for NR serving Cell 1 and NR neighbor cell 3 on FR1 frequency v_Security_Params := f_EUTRA38_ENDC_InitiateAndSendS_gNBKeys(NR); f_EUTRA38_508_ENDC_Reconfiguration_MeasGap(eutra_Cell1, -, v_Security_Params.SK_Counter); //@siclog "Step 13" siclog@ //Wait and ignore MeasurementReport messages for 8 s to allow for UE to measure the neighbouring cells. v_DefaultRef := activate(a_EUTRA_AddDefault_PeriodicalMeasurmentReport(eutra_Cell1, cr_ULInformationTransferMRDC)); f_Delay(v_TimerValue_8s); //@sic R5s230787 sic@ //Deactivate the default to stop receiving periodical Measurement Report deactivate(v_DefaultRef); //@sic R5s230787 sic@ //v_TimerValue_480ms := f_EUTRA_SetTimerToleranceMax(eutra_Cell1, rrcTimer, 0.480); //@sic R5s230787 sic@ v_TimerValue_480ms := f_EUTRA_SetTimerToleranceMin(eutra_Cell1, rrcTimer, 0.480); //@sic R5s240058, R5s230787 sic@ f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger("StartTimer")); //@sic R5s230787 sic@ t_Watchdog.start; //@sic R5s230787 sic@ alt { //@sic R5s230787 sic@ [not(v_FirstReportReceived)] a_EUTRA_ReceiveULInformationTransferMRDC(eutra_Cell1) { v_FirstReportReceived := true; t_TransmissionInterval.start(v_TimerValue_480ms); repeat; } [v_FirstReportReceived] a_EUTRA_ReceiveULInformationTransferMRDC(eutra_Cell1) { if(t_TransmissionInterval.running) { f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 14. Measurement report is received while transmission interval has not expired"); t_TransmissionInterval.stop; } t_TransmissionInterval.start(v_TimerValue_480ms); repeat; } [] t_Watchdog.timeout { t_TransmissionInterval.stop; //continue TC execution } [] t_TransmissionInterval.timeout //@sic R5s240058 sic@ { repeat; } } //@sic R5s230787 sic@ v_DefaultRef := activate(a_EUTRA_AddDefault_PeriodicalMeasurmentReport(eutra_Cell1, cr_ULInformationTransferMRDC)); //@sic R5s240058 sic@ //@siclog "Step 15-17" siclog@ //EXCEPTION: Steps 15 to 17 shall be repeated for k=1 to 11 (increment=1) v_RRCConnectionReconfiguration := cs_508_RRCConnectionReconfiguration_ConditionMEAS(v_RRC_TI, cs_MeasConfig(omit, omit, omit, omit, omit, omit, omit, cs_MeasGapConfig_setup_gp1(9), //@sic R5-203403 sic@ omit, omit, omit, omit, true, true)); v_Counter := 1; while (v_Counter <= 11) { if(v_Counter != 1) { if (bit2int(v_SupportedGaps and4b v_GapMask) == 1) { deactivate(v_DefaultRef); //@sic R5s240058 sic@ fl_ReconfigFr1GapAndGetMeasResults_steps15_17(v_Counter, v_RRCConnectionReconfiguration); //@sic R5s230787 sic@ } v_SupportedGaps := v_SupportedGaps @> 1; } if(v_Counter == 2){v_Counter := v_Counter + 1; } v_Counter := v_Counter + 1; } //@siclog "Step 18-19" siclog@ //SS transmits an RRCConnectionReconfiguration message including measConfig to release fr1-Gap and measid of periodical measurements. //The UE transmits an RCConnectionReconfigrationComplete message v_NR_SCGConfig1 := f_EUTRA_NR_WaitForCoOrd_Data(NR,"Reconfig"); //@sic R5s230787 sic@ v_RRCConnectionReconfiguration_v890_IEs := f_EUTRA38_508_BuildRRCConnReconfig_NR_Config_Setup (v_NR_SCGConfig1, omit, -, -, omit, v_ENDC_ReleaseAndAdd_r15); //@sic R5s230787 sic@ SRB.send(cas_SRB1_RrcNasPduList_REQ(eutra_Cell1, cs_TimingInfo_Now, cs_RRCConnectionReconfiguration_Common(v_RRC_TI, cs_MeasConfig(omit, omit, omit, omit, cs_MeasIdToRemoveList_1Entry(2), omit, omit, cs_MeasGapConfig_release), omit, omit, omit, v_RRCConnectionReconfiguration_v890_IEs), omit)); //@sic R5s230787 sic@ deactivate(v_DefaultRef); //@sic R5s240058 sic@ t_Watchdog5s.start; //@sic R5s230787 sic@ do { //@sic R5s230787 sic@ alt { [] SRB.receive(car_SRB1_RrcPdu_IND(eutra_Cell1, cr_RRCConnectionReconfigurationComplete_r15))-> value v_ReceivedMsg { f_EUTRA_NR_SendCoOrd(NR, cms_EUTRA_NR_OctetData (v_ReceivedMsg.Signalling.Rrc.Dcch.message_.c1.rrcConnectionReconfigurationComplete.criticalExtensions.rrcConnectionReconfigurationComplete_r8.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.scg_ConfigResponseNR_r15, "Complete")); v_RRC_flag := true; if (v_MeasReport_Flag == true) {t_Watchdog5s.stop; v_Timeout := false; } //rrcCompleteisReceive } [] SRB.receive(car_SRB1_RrcPdu_IND(eutra_Cell1, cr_ULInformationTransferMRDC)) { v_MeasReport_Flag := true; if (v_RRC_flag==true) {t_Watchdog5s.stop; v_Timeout :=false; } //received report } [] t_Watchdog5s.timeout { t_Watchdog5s.stop; v_Timeout := false; //continue TC execution } } } while(v_Timeout); //@sic R5s230787 sic@ //End TestBody }

## 2.4 Change 4

| Function name | fl_EUTRA_NR_ENDC_DeriveSupportedGapsFromPics() |
| --- | --- |
| Reason for change | To modify the implementation |
| Summary of change | Modified and added required PICS. |
| TTCN module | RRCMeasurement_ENDC_NR |
| MCC160 Comment |  |

Before Change

function fl_EUTRA_NR_ENDC_DeriveSupportedGapsFromPics() return B8_Type // @sic R5s240058 sic@ { var B8_Type v_SupportedGapPatterns := f_ConvertBoolToBit(pc_gp11_en_dc) & f_ConvertBoolToBit(pc_gp10_en_dc) & f_ConvertBoolToBit(pc_gp9_en_dc) & f_ConvertBoolToBit(pc_gp8_en_dc) & f_ConvertBoolToBit(pc_gp7_en_dc) & f_ConvertBoolToBit(pc_gp6_en_dc) & f_ConvertBoolToBit(pc_gp5_en_dc) & f_ConvertBoolToBit(pc_gp4_en_dc); return v_SupportedGapPatterns; }

After Change

function fl_EUTRA_NR_ENDC_DeriveSupportedGapsFromPics() return B9_Type // @sic R5s240058 sic@ { var B9_Type v_SupportedGapPatterns := f_ConvertBoolToBit(pc_gp11_en_dc) & f_ConvertBoolToBit(pc_gp10_en_dc) & f_ConvertBoolToBit(pc_gp9_en_dc) & f_ConvertBoolToBit(pc_gp8_en_dc) & f_ConvertBoolToBit(pc_gp7_en_dc) & f_ConvertBoolToBit(pc_gp6_en_dc) & f_ConvertBoolToBit(pc_gp5_en_dc) & f_ConvertBoolToBit(pc_gp4_en_dc) & f_ConvertBoolToBit(pc_gp2_gp3_en_dc); return v_SupportedGapPatterns; }

## 2.5 Change 5

| Function name | fl_TC_8_2_3_11_1_ENDC_NR_TestBody() |
| --- | --- |
| Reason for change | To modify the implementation |
| Summary of change | Corrected variables, modified step15-17 to include counter 11 in loop. |
| TTCN module | RRCMeasurement_ENDC_NR |
| MCC160 Comment |  |

Before Change

function fl_TC_8_2_3_11_1_ENDC_NR_TestBody() runs on ENDC_NR_PTC {//@sic R5-206301, R5-206305 sic@ modified v_MeasConfig to be generated by f_NR_GenerateMeasurementConfigNR var template (value) MeasConfig v_MeasConfig; var template (value) MeasObjectParamsList_Type v_MeasuredObjectList; var template (value) ReportConfigToAddModList v_ReportConfigList; var template (value) MeasIdToAddModList v_MeasIdConfigList; var template (value) ReportConfigNR v_ReportConfig; //@sic R5-206632 sic@ var RSRP_Range v_AbsThreshSS_Rsrp_Cell1and3; var PhysCellId v_PhysCellIdServing; var PhysCellId v_PhysCellIdNeighbour; var NR_AbsoluteCellPower_Type v_CellPower_T0_FR1 := -88; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT0; var template (present) MeasResults v_Expected_MeasResults; var MeasResults v_ReceivedMeasResults; //@sic R5s240058 sic@ var EUTRA_NR_Coordination_MSG v_ReceivedMsg; var bitstring v_ULInfoMRDC; var UL_DCCH_Message v_MeasurementReport; var integer v_Counter; var template (omit) MeasGapConfig v_MeasGapFR1; var B8_Type v_SupportedGaps := substr(fl_EUTRA_NR_ENDC_DeriveSupportedGapsFromPics(), 0, 8); //@sic R5s240058 sic@ var B8_Type v_GapMask := int2bit(1, 8); //@sic R5s240058, R5s230787 sic@ timer t_Watchdog; v_CellPowerList_AtT0 := {cs_NR_CellPower(nr_Cell1, v_CellPower_T0_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE), //FR2 is not applicable cs_NR_CellPower(nr_Cell3, v_CellPower_T0_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE) //FR2 is not applicable }; ///The SS re-adjusts the cell-specific reference signal level according to row "T0" f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA,"ChangePower" ); f_NR_SetCellPowerList(v_CellPowerList_AtT0); //@siclog "Steps 11-12" siclog@ //The SS transmits an RRCConnectionReconfiguration including measConfig to setup fr1-Gap and //nr-Config IE containing measConfig to report periodical measurements for NR serving Cell 1 and //NR neighbor cell 3 on FR1 frequency //The UE transmits an RRCConnectionReconfigurationComplete message to confirm the setup of fr1-Gap //and report periodical measurements for NR serving Cell 1 and NR neighbor cell 3 on FR1 frequency v_AbsThreshSS_Rsrp_Cell1and3 := 51; //Table 8.2.3.11.1.3.3-9 v_MeasuredObjectList := {cs_MeasObjectId1(nr_Cell1, v_AbsThreshSS_Rsrp_Cell1and3), cs_MeasObjectId2(nr_Cell3, v_AbsThreshSS_Rsrp_Cell1and3)}; v_MeasIdConfigList := {cs_NR_MeasId_Config_id1_obj2_conf1}; v_MeasGapFR1 := cds_MeasGapConfig_FR1(cs_NR_GapConfig(9, ms6, ms40, ms0dot5)); //@sic R5-203403 sic@ f_NR_SendCoOrdSysInfoToEUTRA({nr_Cell1}); //@sic R5s230787 sic@ v_ReportConfig := cs_38508_ReportConfigNR_Periodical; //@sic R5-206632 sic@ v_ReportConfig.reportType.periodical.reportQuantityCell := cs_NR_MeasReportQuantity_RsrpRsrq; //@sic R5-206632 sic@ v_ReportConfig.reportType.periodical.maxReportCells := 1; //@sic R5-206632 sic@ v_ReportConfigList := {cs_NR_ReportConfigToAddMod_NR(tsc_NR_IdReportConfigId1, v_ReportConfig)}; //@sic R5-206632 sic@ v_MeasConfig := f_NR_GenerateMeasurementConfigNR(v_MeasuredObjectList, v_ReportConfigList, v_MeasIdConfigList, v_MeasGapFR1); //@sic R5-203403, R5-206301, R5-206305 sic@ f_NR_SendRRCReconfigurationContentsToEUTRA (omit, omit, v_MeasConfig, omit, false); //@sic R5s230787 sic@ // Get Security info from EUTRA, calculate latest keys and configure SS f_ENDC_GetKeysActivateSecurity(nr_Cell1, EUTRA); //@sic R5s230787 sic@ f_EUTRA_NR_WaitForCoOrd_Data(EUTRA, "Complete"); //@sic R5s230787 sic@ //@siclog "Step 14" siclog@ //Wait for 30 s to ensure that the UE performs a periodical inter frequency reporting. //EXCEPTION: In parallel to events described in step 4 the steps specified in table 8.2.3.11.2.3.2-3 shall take place v_PhysCellIdServing := f_NR_CellInfo_GetPhysicalCellId(nr_Cell1); v_PhysCellIdNeighbour := f_NR_CellInfo_GetPhysicalCellId(nr_Cell3); v_Expected_MeasResults := cr_38508_MeasResults_NeighCellsNR(tsc_NR_MeasId1, v_PhysCellIdServing, {cr_NR_MeasResultNR_CellResults(v_PhysCellIdNeighbour)}); //@sic R5s230787 sic@ f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA,"StartTimer" ); t_Watchdog.start(30.0); alt { []EUTRA.receive(cmr_EUTRA_NR_OctetData ("ULInfoTransferMRDC")) -> value v_ReceivedMsg { v_ULInfoMRDC := oct2bit(v_ReceivedMsg.OctetData.Data); v_Counter := decvalue(v_ULInfoMRDC, v_MeasurementReport); if (v_Counter != 0) { FatalError(__FILE__, __LINE__, "Decoding error"); } v_ReceivedMeasResults := v_MeasurementReport.message_.c1.measurementReport.criticalExtensions.measurementReport.measResults; if(match(v_ReceivedMeasResults,v_Expected_MeasResults)) //@sic R5s240058 sic@ { f_NR_PreliminaryPass (__FILE__, __LINE__, "Step 14"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 14 - wrong meas results"); //@sic R5s230787 sic@ } repeat; } [] t_Watchdog.timeout { //do nothing } } //@siclog "Step 15-17" siclog@ //EXCEPTION: Steps 15 to 17 shall be repeated for k=1 to 13 (increment=1) v_Counter := 1; while (v_Counter < 11) //@sic R5s230787 sic@ { if(v_Counter != 1) { if (bit2int(v_SupportedGaps and4b v_GapMask) == 1) { fl_NR_ReconfigGapAndGetMeasResultssteps_15_17(v_Expected_MeasResults); } v_SupportedGaps := v_SupportedGaps @> 1; } //@sic R5s230787 sic@ v_Counter := v_Counter + 1; } //@siclog "Step 18-19" siclog@ //SS transmits an RRCConnectionReconfiguration message including measConfig to release fr1-Gap and measid of periodical measurements. //The UE transmits an RCConnectionReconfigrationComplete message v_MeasConfig := cs_NR_MeasConfigCommon(omit, omit,omit, omit, {tsc_NR_MeasId1},omit, omit, omit, cds_MeasGapRelease_FR1); f_NR_SendRRCReconfigurationContentsToEUTRA (omit, omit, v_MeasConfig, omit, false); //@sic R5s230787 sic@ f_EUTRA_NR_WaitForCoOrd_Data(EUTRA, "Complete"); //@sic R5s230787 sic@ } // fl_TC_8_2_3_11_1_ENDC_NR_TestBody

After Change

function fl_TC_8_2_3_11_1_ENDC_NR_TestBody() runs on ENDC_NR_PTC {//@sic R5-206301, R5-206305 sic@ modified v_MeasConfig to be generated by f_NR_GenerateMeasurementConfigNR var template (value) MeasConfig v_MeasConfig; var template (value) MeasObjectParamsList_Type v_MeasuredObjectList; var template (value) ReportConfigToAddModList v_ReportConfigList; var template (value) MeasIdToAddModList v_MeasIdConfigList; var template (value) ReportConfigNR v_ReportConfig; //@sic R5-206632 sic@ var RSRP_Range v_AbsThreshSS_Rsrp_Cell1and3; var PhysCellId v_PhysCellIdServing; var PhysCellId v_PhysCellIdNeighbour; var NR_AbsoluteCellPower_Type v_CellPower_T0_FR1 := -88; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT0; var template (present) MeasResults v_Expected_MeasResults; var MeasResults v_ReceivedMeasResults; //@sic R5s240058 sic@ var EUTRA_NR_Coordination_MSG v_ReceivedMsg; var bitstring v_ULInfoMRDC; var UL_DCCH_Message v_MeasurementReport; var integer v_Counter; var template (omit) MeasGapConfig v_MeasGapFR1; var B9_Type v_SupportedGaps := substr(fl_EUTRA_NR_ENDC_DeriveSupportedGapsFromPics(), 0, 9); //@sic R5s240058 sic@ var B9_Type v_GapMask := int2bit(1, 9); //@sic R5s240058, R5s230787 sic@ timer t_Watchdog; v_CellPowerList_AtT0 := {cs_NR_CellPower(nr_Cell1, v_CellPower_T0_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE), //FR2 is not applicable cs_NR_CellPower(nr_Cell3, v_CellPower_T0_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE) //FR2 is not applicable }; ///The SS re-adjusts the cell-specific reference signal level according to row "T0" f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA,"ChangePower" ); f_NR_SetCellPowerList(v_CellPowerList_AtT0); //@siclog "Steps 11-12" siclog@ //The SS transmits an RRCConnectionReconfiguration including measConfig to setup fr1-Gap and //nr-Config IE containing measConfig to report periodical measurements for NR serving Cell 1 and //NR neighbor cell 3 on FR1 frequency //The UE transmits an RRCConnectionReconfigurationComplete message to confirm the setup of fr1-Gap //and report periodical measurements for NR serving Cell 1 and NR neighbor cell 3 on FR1 frequency v_AbsThreshSS_Rsrp_Cell1and3 := 51; //Table 8.2.3.11.1.3.3-9 v_MeasuredObjectList := {cs_MeasObjectId1(nr_Cell1, v_AbsThreshSS_Rsrp_Cell1and3), cs_MeasObjectId2(nr_Cell3, v_AbsThreshSS_Rsrp_Cell1and3)}; v_MeasIdConfigList := {cs_NR_MeasId_Config_id1_obj2_conf1}; v_MeasGapFR1 := cds_MeasGapConfig_FR1(cs_NR_GapConfig(9, ms6, ms40, ms0dot5)); //@sic R5-203403 sic@ f_NR_SendCoOrdSysInfoToEUTRA({nr_Cell1}); //@sic R5s230787 sic@ v_ReportConfig := cs_38508_ReportConfigNR_Periodical; //@sic R5-206632 sic@ v_ReportConfig.reportType.periodical.reportQuantityCell := cs_NR_MeasReportQuantity_RsrpRsrq; //@sic R5-206632 sic@ v_ReportConfig.reportType.periodical.maxReportCells := 1; //@sic R5-206632 sic@ v_ReportConfigList := {cs_NR_ReportConfigToAddMod_NR(tsc_NR_IdReportConfigId1, v_ReportConfig)}; //@sic R5-206632 sic@ v_MeasConfig := f_NR_GenerateMeasurementConfigNR(v_MeasuredObjectList, v_ReportConfigList, v_MeasIdConfigList, v_MeasGapFR1); //@sic R5-203403, R5-206301, R5-206305 sic@ f_NR_SendRRCReconfigurationContentsToEUTRA (omit, omit, v_MeasConfig, omit, false); //@sic R5s230787 sic@ // Get Security info from EUTRA, calculate latest keys and configure SS f_ENDC_GetKeysActivateSecurity(nr_Cell1, EUTRA); //@sic R5s230787 sic@ f_EUTRA_NR_WaitForCoOrd_Data(EUTRA, "Complete"); //@sic R5s230787 sic@ //@siclog "Step 14" siclog@ //Wait for 30 s to ensure that the UE performs a periodical inter frequency reporting. //EXCEPTION: In parallel to events described in step 4 the steps specified in table 8.2.3.11.2.3.2-3 shall take place v_PhysCellIdServing := f_NR_CellInfo_GetPhysicalCellId(nr_Cell1); v_PhysCellIdNeighbour := f_NR_CellInfo_GetPhysicalCellId(nr_Cell3); v_Expected_MeasResults := cr_38508_MeasResults_NeighCellsNR(tsc_NR_MeasId1, v_PhysCellIdServing, {cr_NR_MeasResultNR_CellResults(v_PhysCellIdNeighbour)}); //@sic R5s230787 sic@ f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA,"StartTimer" ); t_Watchdog.start(30.0); alt { []EUTRA.receive(cmr_EUTRA_NR_OctetData ("ULInfoTransferMRDC")) -> value v_ReceivedMsg { v_ULInfoMRDC := oct2bit(v_ReceivedMsg.OctetData.Data); v_Counter := decvalue(v_ULInfoMRDC, v_MeasurementReport); if (v_Counter != 0) { FatalError(__FILE__, __LINE__, "Decoding error"); } v_ReceivedMeasResults := v_MeasurementReport.message_.c1.measurementReport.criticalExtensions.measurementReport.measResults; if(match(v_ReceivedMeasResults,v_Expected_MeasResults)) //@sic R5s240058 sic@ { f_NR_PreliminaryPass (__FILE__, __LINE__, "Step 14"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 14 - wrong meas results"); //@sic R5s230787 sic@ } repeat; } [] t_Watchdog.timeout { //do nothing } } //@siclog "Step 15-17" siclog@ //EXCEPTION: Steps 15 to 17 shall be repeated for k=1 to 13 (increment=1) v_Counter := 1; while (v_Counter <= 11) //@sic R5s230787 sic@ { if(v_Counter != 1) { if (bit2int(v_SupportedGaps and4b v_GapMask) == 1) { fl_NR_ReconfigGapAndGetMeasResultssteps_15_17(v_Expected_MeasResults); } v_SupportedGaps := v_SupportedGaps @> 1; } //@sic R5s230787 sic@ if(v_Counter == 2) {v_Counter := v_Counter + 1; } v_Counter := v_Counter + 1; } //@siclog "Step 18-19" siclog@ //SS transmits an RRCConnectionReconfiguration message including measConfig to release fr1-Gap and measid of periodical measurements. //The UE transmits an RCConnectionReconfigrationComplete message v_MeasConfig := cs_NR_MeasConfigCommon(omit, omit,omit, omit, {tsc_NR_MeasId1},omit, omit, omit, cds_MeasGapRelease_FR1); f_NR_SendRRCReconfigurationContentsToEUTRA (omit, omit, v_MeasConfig, omit, false); //@sic R5s230787 sic@ f_EUTRA_NR_WaitForCoOrd_Data(EUTRA, "Complete"); //@sic R5s230787 sic@ } // fl_TC_8_2_3_11_1_ENDC_NR_TestBody