3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240510

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4846 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | LTE_FDD: Addition of LTE Rel-16 RACS Test case 9.2.5.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu EMEA Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, RACS-UEConTest |  |  |  |  |  | Date: |  |  | 2024-07-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | LTE_FDD: Addition of LTE Rel-16 RACS Test case 9.2.5.2 |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to test case 9.2.5.2 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be verified |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

3	Branches executed	3

4	Execution Log Files	3

4.1	Samsung Shannon S5E9945ERD UE	3

5	References	3

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 9.2.5.2 which is part of the LTE_A_PRO test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Kalahasti, Narendra

Narendra.Kalahasti@anritsu.com

## Verification Test Summary

Test Case: 	9.2.5.2

ATS Version:	iwd-TTCN3-B2022-09_D24wk24

System Simulator used:	Anritsu Protocol Conformance Test System ME7834NR

UE used:	Samsung Shannon S5E9945ERD UE

Verification Status:	PASS

# Corrections required

## Change 1

| Testcase name | fl_TC_9_2_5_2_Body |
| --- | --- |
| Reason for change | - At Step4a5, not resetting the downlink NAS COUNT counter will lead to NAS decoding error. - Incorrect ID is used to match against manufacturer-assigned UE Radio Capability ID. - According to Table 9.9.3.58.1, TS 24.301, values other than ‘001’ for availability IE is interpreted as UE radio capability ID not available, hence there is no need for else-if condition for the value ‘000’. - The variable v_NasCountUL will be left uninitialized causing runtime error, when UE omits ueRadioCapIdAvailability IE in the attach request at Step 3. |
| Summary of change | - Reset downlink NAS COUNT counter. - Modify the ID used for matching manufacturer-assigned UE Radio Capability ID, in the if case. - Removed else-if condition used for checking the matching availability IE with the value ‘000’. - Corrected if-else statement, to assign the variable v_NasCountUL when UE omits ueRadioCapIdAvailability IE. |
| TTCN module | NAS_RACS.ttcn |
| MCC160 Comment |  |

Before change:

function fl_TC_9_2_5_2_Body() runs on EUTRA_PTC

{

var NAS_MSG_Indication_Type v_NAS_Ind;

var NasCount_Type v_NasCountUL;

var EUTRA_SecurityParams_Type v_AuthParams;

var template (omit) UERadioCapId v_UERadioCapId;

//@siclog "Step 1" siclog@

//If possible (see ICS) switch off is performed or the USIM is removed.Otherwise the power is removed.

f_EUTRA_UE_Detach_SwitchOffUe(eutra_CellA, RRC_IDLE);

//@siclog "Step 2" siclog@

//The UE is brought back to operation with USIM A inserted.

f_UT_USIM_Insert(UT, "36.523-1 Table 9.2.5.2.3.1-1");

f_UT_AutomaticPLMN_Select_WithSwitchOnOff (UT);

f_EUTRA_SwitchOnUEandStartIP(eutra_CellA);//@sic R5s240218 sic@

// @siclog "Step 3" siclog@

// Void.//@sic R5-241690 sic@

// EXCEPTION: Steps 4a1-4b5 describe behaviour that depends on UE configuration; the lower case letter? identifies a step sequence that takes place depending on whether the UE has a manufacturer-assigned UE Radio Capability ID

// @siclog "Step 4a1" siclog@// @sic R5-241690 sic@

// IF the UE has a manufacturer-assigned UE Radio Capability ID THEN Check: Does the UE indicate UE radio Capability ID available in UE radio capability availability indication IE in ATTACH REQUEST message?

v_NAS_Ind:= f_EUTRA_RRC_ConnEst_DefWithNas(eutra_CellA,

tsc_RRC_TI_Def,

cr_EstablishmentCause_Any,

cr_NAS_IndicationWithPiggybacking((tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected),//@sic R5s240218 sic@

cr_508_ATTACH_REQUEST(f_GetEAttachType(NORMAL), f_GetAdditionalUpdateType(NORMAL)),

cr_508_PDN_CONNECTIVITY_REQUEST(tsc_PdnRequest_InitialAttach,

f_GetPdnType())));

if(ispresent(v_NAS_Ind.Pdu.Msg.aTTACH_REQUEST.ueRadioCapIdAvailability.availability)){

if (match (v_NAS_Ind.Pdu.Msg.aTTACH_REQUEST.ueRadioCapIdAvailability.availability, '001'B)) {

//IF the UE has a manufacturer-assigned UE Radio Capability ID THEN Check: Does the UE indicate UE radio Capability ID available in UE radio capability availability indication IE in TRACKING AREA UPDATE REQUEST message?

// @siclog "Step 4a2-4a3" siclog@// @sic R5-241690 sic@

// Steps 5-6 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed.

f_EUTRA_InitialRegistration_Step5_6(eutra_CellA, v_NAS_Ind);

v_AuthParams := f_EUTRA_Security_Get();

// @siclog "Step 4a4" siclog@// @sic R5-241690 sic@

// The SS transmits a NAS SECURITY MODE COMMAND message requesting UE radio capability ID.

// @siclog "Step 4a5" siclog@// @sic R5-241690 sic@

// Check: Does the UE transmit a NAS SECURITY MODE COMPLETE message including the manufacturer-assigned UE Radio Capability ID?

v_NasCountUL:= f_EUTRA_NAS_ActivateSecurity(eutra_CellA,v_AuthParams,-,omit ,false, v_NAS_Ind.Pdu.Msg.aTTACH_REQUEST.ueRadioCapIdAvailability);

v_UERadioCapId := f_EUTRA_MobileInfo_GetRxdRACS_Id();

if (isvalue(v_UERadioCapId)) {

if (match(valueof(v_UERadioCapId), cs_UERadioCapId('66'O, '00000000000000'O))) {

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "step4b1 check pass");

} else {

f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "step4b1 check fail");

}

} else {

f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "step4b1 check fail");

}

}

else if(match (v_NAS_Ind.Pdu.Msg.aTTACH_REQUEST.ueRadioCapIdAvailability.availability, '000'B)){

// @siclog "Step 4b1" siclog@// @sic R5-241690 sic@

//ELSECheck: Does the UE not indicate a UE radio Capability ID is available in ATTACH REQUEST message?

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "step 4b1 pass");

// @siclog "Step 4b2-b5" siclog@// @sic R5-241690 sic@

v_NasCountUL:=f_EUTRA_InitialRegistration_Step5_8 (eutra_CellA, v_NAS_Ind);

}

else {

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "step 4b1 pass");

// @siclog "Step 4b2-b5" siclog@// @sic R5-241690 sic@

v_NasCountUL:=f_EUTRA_InitialRegistration_Step5_8 (eutra_CellA, v_NAS_Ind);

}

}

// @siclog "Step 5-14" siclog@ // @sic R5-241690 sic@

//the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed.

f_EUTRA_InitialRegistration_Step9_18b1 (eutra_CellA, v_NAS_Ind, v_NasCountUL); // @sic R5-160739 sic@

} //End of fl_TC_9_2_5_2_Body

After change:

function fl_TC_9_2_5_2_Body() runs on EUTRA_PTC

{

var NAS_MSG_Indication_Type v_NAS_Ind;

var NasCount_Type v_NasCountUL;

var EUTRA_SecurityParams_Type v_AuthParams;

var template (omit) UERadioCapId v_UERadioCapId;

//@siclog "Step 1" siclog@

//If possible (see ICS) switch off is performed or the USIM is removed.Otherwise the power is removed.

f_EUTRA_UE_Detach_SwitchOffUe(eutra_CellA, RRC_IDLE);

//@siclog "Step 2" siclog@

//The UE is brought back to operation with USIM A inserted.

f_UT_USIM_Insert(UT, "36.523-1 Table 9.2.5.2.3.1-1");

f_UT_AutomaticPLMN_Select_WithSwitchOnOff (UT);

f_EUTRA_SwitchOnUEandStartIP(eutra_CellA);//@sic R5s240218 sic@

// @siclog "Step 3" siclog@

// Void.//@sic R5-241690 sic@

// EXCEPTION: Steps 4a1-4b5 describe behaviour that depends on UE configuration; the lower case letter? identifies a step sequence that takes place depending on whether the UE has a manufacturer-assigned UE Radio Capability ID

// @siclog "Step 4a1" siclog@// @sic R5-241690 sic@

// IF the UE has a manufacturer-assigned UE Radio Capability ID THEN Check: Does the UE indicate UE radio Capability ID available in UE radio capability availability indication IE in ATTACH REQUEST message?

v_NAS_Ind:= f_EUTRA_RRC_ConnEst_DefWithNas(eutra_CellA,

tsc_RRC_TI_Def,

cr_EstablishmentCause_Any,

cr_NAS_IndicationWithPiggybacking((tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected),//@sic R5s240218 sic@

cr_508_ATTACH_REQUEST(f_GetEAttachType(NORMAL), f_GetAdditionalUpdateType(NORMAL)),

cr_508_PDN_CONNECTIVITY_REQUEST(tsc_PdnRequest_InitialAttach,

f_GetPdnType())));

if(ispresent(v_NAS_Ind.Pdu.Msg.aTTACH_REQUEST.ueRadioCapIdAvailability.availability)){

if (match (v_NAS_Ind.Pdu.Msg.aTTACH_REQUEST.ueRadioCapIdAvailability.availability, '001'B)) {

//IF the UE has a manufacturer-assigned UE Radio Capability ID THEN Check: Does the UE indicate UE radio Capability ID available in UE radio capability availability indication IE in TRACKING AREA UPDATE REQUEST message?

// @siclog "Step 4a2-4a3" siclog@// @sic R5-241690 sic@

// Steps 5-6 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed.

f_EUTRA_InitialRegistration_Step5_6(eutra_CellA, v_NAS_Ind);

v_AuthParams := f_EUTRA_Security_Get();

// @siclog "Step 4a4" siclog@// @sic R5-241690 sic@

// The SS transmits a NAS SECURITY MODE COMMAND message requesting UE radio capability ID.

// @siclog "Step 4a5" siclog@// @sic R5-241690 sic@

// Check: Does the UE transmit a NAS SECURITY MODE COMPLETE message including the manufacturer-assigned UE Radio Capability ID?

v_NasCountUL:= f_EUTRA_NAS_ActivateSecurity(eutra_CellA,v_AuthParams,-,-,false, v_NAS_Ind.Pdu.Msg.aTTACH_REQUEST.ueRadioCapIdAvailability);

v_UERadioCapId := f_EUTRA_MobileInfo_GetRxdRACS_Id();

if (isvalue(v_UERadioCapId)) {

if (match(valueof(v_UERadioCapId), cs_UERadioCapId('66'O, '00?????????'O))) {

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "step4b1 check pass");

} else {

f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "step4b1 check fail");

}

} else {

f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "step4b1 check fail");

}

}

else

// @siclog "Step 4b1" siclog@// @sic R5-241690 sic@

//ELSECheck: Does the UE not indicate a UE radio Capability ID is available in ATTACH REQUEST message?

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "step 4b1 pass");

// @siclog "Step 4b2-b5" siclog@// @sic R5-241690 sic@

v_NasCountUL:=f_EUTRA_InitialRegistration_Step5_8 (eutra_CellA, v_NAS_Ind);

}

}

else {

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "step 4b1 pass");

// @siclog "Step 4b2-b5" siclog@// @sic R5-241690 sic@

v_NasCountUL:=f_EUTRA_InitialRegistration_Step5_8 (eutra_CellA, v_NAS_Ind);

}

// @siclog "Step 5-14" siclog@ // @sic R5-241690 sic@

//the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed.

f_EUTRA_InitialRegistration_Step9_18b1 (eutra_CellA, v_NAS_Ind, v_NasCountUL); // @sic R5-160739 sic@

} //End of fl_TC_9_2_5_2_Body

# Branches executed

The test case implementation was executed with Integrity and ciphering enabled using LTE FDD band 4.

# Execution Log Files

## Samsung Shannon S5E9945ERD UE

The Samsung Shannon S5E9945ERD UE passed this test case on Anritsu Protocol Conformance Test System ME7834NR. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

TC_9_2_5_2_LOG.xml

- PICS/PIXIT parameter file:

TC_9_2_5_2 _PIXIT.txt

# References

R5s240523: LTE_FDD: Supporting information for agreement of TC 9.2.5.2