3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240511

# Online,  , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-5 | CR | 0117 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of MCPTT test case 5.1 to the MCX ATS with MCX-EUTRA model |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, MCPTT-ConTest |  |  |  |  |  | Date: |  |  | 2024-07-26 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To add MCPTT TC 5.1 to the MCX ATS on MCX-EUTRA test model |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to the TTCN code required to obtain a pass using the MCX-EUTRA test model. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The testcase will not be added to MCX ATS with MCX-EUTRA model |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.MCPTT |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 36.579-1, 36.579-5, 36.508 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	4

2.3	Change 3	6

2.4	Change 4	6

2.5	Change 5	7

2.6	Change 6	10

2.7	Change 7	10

2.8	Change 8	11

2.9	Change 9	12

2.10	Change 10	14

2.11	Change 11	14

2.12	Change 12	15

2.13	Change 13	15

2.14	Change 14	15

2.15	Change 15	16

2.16	Change 16	17

2.17	Change 17	18

2.18	Change 18	20

2.19	Change 19	21

2.20	Change 20	22

2.21	Change 21	23

2.22	Change 22	24

2.23	Change 23	25

2.24	Change 24	27

2.25	Change 25	28

2.26	Change 26	29

2.27	Change 27	30

2.28	Change 28	33

2.29	Change 29	35

2.30	Change 30	35

2.31	Change 31	36

2.32	Change 32	36

2.33	Change 33	37

2.34	Change 34	38

2.35	Change 35	38

3	Branches executed	39

4	Execution Log Files	39

4.1	Alea - a Leonardo Company, MCXPTT Platform v. 4.1.0 - Android Client v 1.11.5	39

5	References	39

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 5.1.MCPTT which is part of the MCX_EUTRA test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Mohit Kanchan
	mohit.kanchan@keysight.com

## Verification Test Summary

Test Case: 	5.1.MCPTT

ATS Version:	iwd-TTCN3-B2022-09_D24wk24

System Simulator used:	Keysight S8704A Protocol Conformance Toolset

UE used:	Alea - a Leonardo Company, MCXPTT Platform v. 4.1.0 - Android Client v 1.11.5 on COTS LTE UE

Verification Status:	PASS

# Corrections required

## Change 1

| Function name | f_MCX_EUTRA_IPCAN_PdnIndex() |
| --- | --- |
| Reason for change | In current IPCAN PTC implementation, the inter-PTC port for MCX and (optional) IMS PTC are initialised with index of tsc_Index_IMS1 (0) and tsc_Index_IMS2 (1) respectively (see f_MTC_MCX_InitAndConnect_IP_IMS_HTTP) However, when routing SIP messages etc from IPCAN to MCX/IMS PTC, PDN index is used (see f_PDN_PdnIndex2Integer, common in many ATS suites) to determined which port to use, with PDN_2 (1) and PDN_1 (0) mapped to MCX and IMS, which is done in f_MCX_EUTRA_IPCAN_PdnIndex. This results in the messages are routed to the wrong PTC. |
| Summary of change | It is suggeseted to update f_MCX_EUTRA_IPCAN_PdnIndex, using PDN_1 (0) for MCX, and PDN_2 (1) for IMS, to align with the port indexing |
| TTCN module | IPCAN_MCX_EUTRA.ttcn |
| MCC160 Comment |  |

Before Change:

function f_MCX_EUTRA_IPCAN_PdnIndex(MCX_Registration_PDN_Type p_PDN) return PDN_Index_Type { var PDN_Index_Type v_PdnIndex; select (p_PDN) { case (imsPDN) { v_PdnIndex := PDN_1; } case (mcxPDN) { v_PdnIndex := PDN_2; } case (internetPDN) { v_PdnIndex := PDN_3; } case else { FatalError(__FILE__, __LINE__, "invalid PDN"); } } return v_PdnIndex; }

After Change:

function f_MCX_EUTRA_IPCAN_PdnIndex(MCX_Registration_PDN_Type p_PDN) return PDN_Index_Type { var PDN_Index_Type v_PdnIndex; select (p_PDN) { case (imsPDN) { v_PdnIndex := PDN_2; } case (mcxPDN) { v_PdnIndex := PDN_1; } case (internetPDN) { v_PdnIndex := PDN_3; } case else { FatalError(__FILE__, __LINE__, "invalid PDN"); } } return v_PdnIndex; }

## Change 2

| Function name | f_IMS_PTC_Init() (new) cs_MCX_SecurityInfo_Init |
| --- | --- |
| Reason for change | The current TTCN use the same PCIS setting cs_IMS_SecurityInfo_Init for both MCX, and IMS. While in real world application, this may not be true, e.g. different vendor for both client side as well as server side, and support different set of security settings, and requires different setting in MCX than that for IMS. It is suggeseted to apply independent security setting for MCX |
| Summary of change | 1, Apply independent security setting for MCX NOTE – new PIXIT added in Change 3 2, Due to Change 1, the v_SPI_ValidValueList_PDN1 and v_SPI_ValidValueList_PDN2 need to be swapped |
| TTCN module | IMS_Component.ttcn |
| MCC160 Comment |  |

Before Change:

function f_IMS_PTC_Init(PDN_Index_Type p_PdnIndex := PDN_1, IMS_PTC_Handling_Type p_Handling := testcaseSpecificHandling, template (omit) IMS_SecurityScheme_Type p_SecurityScheme := omit, template (omit) IPCAN_RAN_Type p_RanType := omit, IMS_TestSpec_Type p_TestSpec := TS_34229, NameValueList_Type p_TestsuiteParameterList := {}) runs on IMS_PTC { /* @sic R5s130651: new parameter p_PdnIndex sic@ */ /* @sic R5s130756 additional changes: new parameter p_SecurityScheme sic@ */ /* @sic R5s130756: new parameter p_Pcscf sic@ */ /* @sic R5-144747: removal of px_IMS_Pcscf and px_IMS_Pcscf_Emg => p_Pcscf removed sic@ */ /* @sic R5s190727 change 4.2: p_DigestPasswordForSIP, p_TestSpec; p_UE_Release removed sic@ */ /* @sic R5-196983: p_TestSpec - support of MCX sic@ */ /* @sic R5-196983: p_TestsuiteParameterList - support of MCX: to avoid the need of importing MCX specific PICS/PIXITs sic@ */ /* @sic R5s201657: Removal of FBBA sic@ */ var charstring v_HomeDomainName; var charstring v_PrivateUserId; var CharStringList_Type v_IMPUList; var IMS_SecurityScheme_Type v_SecurityScheme; var boolean v_IsMCX; var IntegerList_Type v_SPI_ValidValueList_PDN1 := fl_IMS_PTC_SPI_ValidValueList_Init(px_IMS_SPI_ValidValueList_PDN1); /* @sic R5-172048 sic@ */ var IntegerList_Type v_SPI_ValidValueList_PDN2 := fl_IMS_PTC_SPI_ValidValueList_Init(px_IMS_SPI_ValidValueList_PDN2); /* @sic R5-172048 sic@ */ var template (omit) integer v_UE_Release := omit; var boolean v_UEwithISIM := px_UEwithISIM; ... ... vc_IMS_Global.PdnIndex := p_PdnIndex; vc_IMS_Global.Ctrl.TestBody := false; vc_IMS_Global.Ctrl.BlockUlDataInDefault := false; /* @sic R5s140780 - MCC160 Implementation sic@ */ vc_IMS_Global.Ctrl.IgnoreReset := true; /* @sic R5s150031: IMS is already reset after initialisation sic@ */ vc_IMS_Global.Ctrl.Default := null; vc_IMS_Global.Security := valueof(cs_IMS_SecurityInfo_Init); vc_IMS_Global.ImsInfo := valueof(cs_IMS_ImsInfo_Init(p_RanType, v_IMPUList, v_UE_Release, v_HomeDomainName, v_PrivateUserId, v_SecurityScheme, v_SPI_ValidValueList_PDN1, v_SPI_ValidValueList_PDN2, v_UEwithISIM)); f_IMS_PTC_ActivateDefault(p_Handling); }

After Change:

function f_IMS_PTC_Init(PDN_Index_Type p_PdnIndex := PDN_1, IMS_PTC_Handling_Type p_Handling := testcaseSpecificHandling, template (omit) IMS_SecurityScheme_Type p_SecurityScheme := omit, template (omit) IPCAN_RAN_Type p_RanType := omit, IMS_TestSpec_Type p_TestSpec := TS_34229, NameValueList_Type p_TestsuiteParameterList := {}) runs on IMS_PTC { /* @sic R5s130651: new parameter p_PdnIndex sic@ */ /* @sic R5s130756 additional changes: new parameter p_SecurityScheme sic@ */ /* @sic R5s130756: new parameter p_Pcscf sic@ */ /* @sic R5-144747: removal of px_IMS_Pcscf and px_IMS_Pcscf_Emg => p_Pcscf removed sic@ */ /* @sic R5s190727 change 4.2: p_DigestPasswordForSIP, p_TestSpec; p_UE_Release removed sic@ */ /* @sic R5-196983: p_TestSpec - support of MCX sic@ */ /* @sic R5-196983: p_TestsuiteParameterList - support of MCX: to avoid the need of importing MCX specific PICS/PIXITs sic@ */ /* @sic R5s201657: Removal of FBBA sic@ */ var charstring v_HomeDomainName; var charstring v_PrivateUserId; var CharStringList_Type v_IMPUList; var IMS_SecurityScheme_Type v_SecurityScheme; var boolean v_IsMCX; var IntegerList_Type v_SPI_ValidValueList_PDN1 := fl_IMS_PTC_SPI_ValidValueList_Init(px_IMS_SPI_ValidValueList_PDN1); /* @sic R5-172048 sic@ */ var IntegerList_Type v_SPI_ValidValueList_PDN2 := fl_IMS_PTC_SPI_ValidValueList_Init(px_IMS_SPI_ValidValueList_PDN2); /* @sic R5-172048 sic@ */ var template (omit) integer v_UE_Release := omit; var boolean v_UEwithISIM := px_UEwithISIM; ... ... vc_IMS_Global.PdnIndex := p_PdnIndex; vc_IMS_Global.Ctrl.TestBody := false; vc_IMS_Global.Ctrl.BlockUlDataInDefault := false; /* @sic R5s140780 - MCC160 Implementation sic@ */ vc_IMS_Global.Ctrl.IgnoreReset := true; /* @sic R5s150031: IMS is already reset after initialisation sic@ */ vc_IMS_Global.Ctrl.Default := null; if (v_IsMCX) { vc_IMS_Global.Security := valueof(cs_MCX_SecurityInfo_Init); vc_IMS_Global.ImsInfo := valueof(cs_IMS_ImsInfo_Init(p_RanType, v_IMPUList, v_UE_Release, v_HomeDomainName, v_PrivateUserId, v_SecurityScheme, v_SPI_ValidValueList_PDN2, v_SPI_ValidValueList_PDN1, v_UEwithISIM)); } else { vc_IMS_Global.Security := valueof(cs_IMS_SecurityInfo_Init); vc_IMS_Global.ImsInfo := valueof(cs_IMS_ImsInfo_Init(p_RanType, v_IMPUList, v_UE_Release, v_HomeDomainName, v_PrivateUserId, v_SecurityScheme, v_SPI_ValidValueList_PDN1, v_SPI_ValidValueList_PDN2, v_UEwithISIM)); } f_IMS_PTC_ActivateDefault(p_Handling); } ... ... // new template template (value) IMS_PTC_SecurityInfo_Type cs_MCX_SecurityInfo_Init(template (omit) IMS_SPIs_Type p_SPIs := omit) := //MCPTT-2024 TTCN CR Raised change 2 { /* @status APPROVED (ENDC, IMS, IMS_IRAT, LTE, LTE_A_IRAT, LTE_A_PRO, LTE_A_R10_R11, LTE_A_R12, LTE_IRAT, MCX, NR5GC, NR5GC_IRAT, POS) */ IntegrityAlgorithm := px_MCX_IPSecAlgorithm, CipheringAlgorithm := px_MCX_CiphAlgo_Def, ProtectedPorts := omit, SPIs := p_SPIs, Nonce := omit, XRES := omit, XRESLength := omit };

## Change 3

| Function name | (new) px_MCX_IPSecAlgorithm (new) px_MCX_CiphAlgo_Def |
| --- | --- |
| Reason for change | As for Change 2 |
| Summary of change | Added two new PIXIS for MCX secutirty setting NOTE – prose on 36.579-5 needed |
| TTCN module | IMS_CommonParameters.TTCN |
| MCC160 Comment |  |

New PIXITs:

modulepar IPsec_IntegrityAlgorithm_Type px_MCX_IPSecAlgorithm := hmac_sha_1_96; // @desc Integrity Algorithm // Supported values: // hmac_md5_96, // hmac_sha_1_96 // Reference to other Spec: modulepar IPsec_CipheringAlgorithm_Type px_MCX_CiphAlgo_Def := aes_cbc; // @desc Ciphering Algorithm // Supported values: // des_ede3_cbc, // aes_cbc, // nociph // Reference to other Spec:

## Change 4

| Function name | f_EUTRA_DelayForUserPlaneSignalling() |
| --- | --- |
| Reason for change | In the function f_MTC_MCX_CreateMapAndConnectPTCs_EUTRA(), the IMS PTC is connected to the IPCAN_EUTRA PTC on port IMS[tsc_Index_IMS2]. But the function f_EUTRA_DelayForUserPlaneSignalling()will always synchronise on port IMS[tsc_Index_IMS1] to control IMS registration following IMS PDN establishment. |
| Summary of change | Add a parameter to this function indicating to use port IMS[tsc_Index_IMS2] for IMS coordination in case of MCX configuration. |
| TTCN module | EUTRA_NASSteps.ttcn |
| MCC160 Comment |  |

Before change

function f_EUTRA_DelayForUserPlaneSignalling(DelayForUserPlaneSignalling_Type p_DelayForUserPlaneSignalling := waitForIMS) runs on EUTRA_PTC

{ /* @sic R5s160133 - use of f_IPCAN_DelayForUserPlaneSignalling sic@ */

var boolean v_LoopbackIsActivated;

var CoOrd_OtherRATState_Type v_CoOrd_OtherRATState_Type := f_EUTRA_MobileInfo_GetCoOrd_OtherRATState();

var DelayForUserPlaneSignalling_Type v_DelayForUserPlaneSignalling := p_DelayForUserPlaneSignalling;

if (p_DelayForUserPlaneSignalling == waitForIMS) {

if (v_CoOrd_OtherRATState_Type.IMS_Registered_WithExtraPDP) { // @sic R5s160740 sic@ send trigger to inform IMS_DefaultHandler that 'OtherIPCAN' shall be used as coordination port during IMS registration

f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1], cms_IPCAN_IMS_IpcanInd);

}

v_LoopbackIsActivated := f_EUTRA_LoopbackIsActivated_Get();  /* @sic R5-153971 sic@ */

if (not pc_IMS or v_LoopbackIsActivated) {

v_DelayForUserPlaneSignalling := dontWaitForIMS;

f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1], cms_IPCAN_IMS_NoRegistration);         // @sic R5-169150 sic@

}

}

f_IPCAN_DelayForUserPlaneSignalling(IMS[tsc_Index_IMS1], v_DelayForUserPlaneSignalling); // @sic R5s160562 sic@

}

After change

function f_EUTRA_DelayForUserPlaneSignalling(DelayForUserPlaneSignalling_Type p_DelayForUserPlaneSignalling := waitForIMS,

boolean p_MCX := false) runs on EUTRA_PTC //MCPTT-2024 TTCN CR Raised change 5

{ /* @sic R5s160133 - use of f_IPCAN_DelayForUserPlaneSignalling sic@ */

var boolean v_LoopbackIsActivated;

var CoOrd_OtherRATState_Type v_CoOrd_OtherRATState_Type := f_EUTRA_MobileInfo_GetCoOrd_OtherRATState();

var DelayForUserPlaneSignalling_Type v_DelayForUserPlaneSignalling := p_DelayForUserPlaneSignalling;

var IMS_IPCAN_CO_ORD_PORT v_Port := IMS[tsc_Index_IMS1];

if (p_MCX)

{

v_Port:= IMS[tsc_Index_IMS2];

}

if (p_DelayForUserPlaneSignalling == waitForIMS) {

if (v_CoOrd_OtherRATState_Type.IMS_Registered_WithExtraPDP) { // @sic R5s160740 sic@ send trigger to inform IMS_DefaultHandler that 'OtherIPCAN' shall be used as coordination port during IMS registration

f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1], cms_IPCAN_IMS_IpcanInd);

}

v_LoopbackIsActivated := f_EUTRA_LoopbackIsActivated_Get();  /* @sic R5-153971 sic@ */

if (not pc_IMS or v_LoopbackIsActivated) {

v_DelayForUserPlaneSignalling := dontWaitForIMS;

f_IMS_IPCAN_SendCoOrdMsg(v_Port, cms_IPCAN_IMS_NoRegistration);         // @sic R5-169150 sic@

}

}

f_IPCAN_DelayForUserPlaneSignalling(v_Port, v_DelayForUserPlaneSignalling); // @sic R5s160562 sic@

}

## Change 5

| Function name | JSON related type declaration |
| --- | --- |
| Reason for change | Variant information needs to be added for JSON typs, so that it may be handled properly by the compiler |
| Summary of change | with {variant "JSON:????" } is added in type declaration |
| TTCN module | JSON.ttcn |
| MCC160 Comment |  |

Before change

//====================Types to define JSON Schemas =================================

// JSON Number type (generic)

type float Number (!-infinity .. !infinity); /* with {

variant "JSON:number"

} */

// Integer type

type integer Integer (-infinity .. infinity); /* with {

variant "JSON:integer"

} */

// String type

type universal charstring String; /* with {

variant "JSON:string"

} */

// Array type

type record of JSON.Values Array; /* with {

variant "JSON:array"

} */

// Subsidiary array types

type record of JSON.String StrArray; /* with {

variant "JSON:array"

} */

type record of JSON.Number NumArray; /* with {

variant "JSON:array"

} */

type record of JSON.Integer IntArray; /* with {

variant "JSON:array"

} */

type record of JSON.Bool BoolArray; /* with {

variant "JSON:array"

} */

type record of JSON.Object ObjArray; /* with {

variant "JSON:array"

} */

// Object member

type record ObjectMember {

JSON.String name,

JSON.Values value_

}; /* with {

variant "JSON:objectMember"

} */

// Generic JSON object type

type record Object {

record length (1..infinity) of JSON.ObjectMember memberList optional

}; /* with {

variant "JSON:object"

} */

type union Values {

JSON.String str,

JSON.Number num,

JSON.Integer int,

JSON.Object object,

JSON.Array array,

JSON.StrArray strArray,

JSON.NumArray numArray,

JSON.IntArray intArray,

JSON.BoolArray boolArray,

JSON.ObjArray objArray,

JSON.Bool bool,

JSON.Null null_

}; /* with {

variant "asValue"

} */

//JSON literals

//When only the true and false literals are allowed

type boolean Bool; /* with { variant "JSON:literal" } */

//When only the null literal is allowed

type enumerated Null { null_ }; /* with { variant "JSON:literal" } */

After change

//====================Types to define JSON Schemas =================================

// JSON Number type (generic)

type float Number (!-infinity .. !infinity) /**/ with {

variant "JSON:number"

} /**/

// Integer type

type integer Integer (-infinity .. infinity) /**/ with {

variant "JSON:integer"

} /**/

// String type

type universal charstring String /**/ with {

variant "JSON:string"

} /**/

// Array type

type record of JSON.Values Array /**/ with {

variant "JSON:array"

} /**/

// Subsidiary array types

type record of JSON.String StrArray /**/ with {

variant "JSON:array"

} /**/

type record of JSON.Number NumArray /**/ with {

variant "JSON:array"

} /**/

type record of JSON.Integer IntArray /**/ with {

variant "JSON:array"

} /**/

type record of JSON.Bool BoolArray /**/ with {

variant "JSON:array"

} /**/

type record of JSON.Object ObjArray /**/ with {

variant "JSON:array"

} /**/

// Object member

type record ObjectMember {

JSON.String name,

JSON.Values value_

} /**/ with {

variant "JSON:objectMember"

} /**/

// Generic JSON object type

type record Object {

record length (1..infinity) of JSON.ObjectMember memberList optional

} /**/ with {

variant "JSON:object"

} /**/

type union Values {

JSON.String str,

JSON.Number num,

JSON.Integer int,

JSON.Object object,

JSON.Array array,

JSON.StrArray strArray,

JSON.NumArray numArray,

JSON.IntArray intArray,

JSON.BoolArray boolArray,

JSON.ObjArray objArray,

JSON.Bool bool,

JSON.Null null_

} /**/ with {

variant "asValue"

} /**/

//JSON literals

//When only the true and false literals are allowed

type boolean Bool /**/ with { variant "JSON:literal" } /**/

//When only the null literal is allowed

type enumerated Null { null_ } /**/ with { variant "JSON:literal" } /**/

## Change 6

| Function name | JSON related type declaration |
| --- | --- |
| Reason for change | Variant information needs to be added for JSON typs, so that it may be handled properly by the compiler |
| Summary of change | with {variant "JSON:????" } is added in type declaration |
| TTCN module | MCX_JSON_Objects.ttcn |
| MCC160 Comment |  |

Before change

group JSON_AliasDefinitions {

type JSON.ObjectMember JSON_ObjectMember_Type;                /* @status    APPROVED (MCX) */

type JSON.Object JSON_Object_Type;                            /* @status    APPROVED (MCX) */

type JSON.Object.memberList JSON_Object_MemberList_Type;      /* @status    APPROVED (MCX) */

} with { encode "JSON" }

After change

group JSON_AliasDefinitions {

type JSON.ObjectMember JSON_ObjectMember_Type with {variant "JSON:objectMember"};                /* @status    APPROVED (MCX) */

type JSON.Object JSON_Object_Type with {variant "JSON:object"};                            /* @status    APPROVED (MCX) */

type JSON.Object.memberList JSON_Object_MemberList_Type with {variant "JSON:array"};      /* @status    APPROVED (MCX) */

} with { encode "JSON" }

## Change 7

| Function name | Template cr_MCX_HttpRequest |
| --- | --- |
| Reason for change | cache-control header in Http Request are optional, and a client may not include cache-control header in HTTP requests |
| Summary of change | Allow cache-control to be omitted NOTE – prose needed on 36.579-1 Table 5.5.4.2/3/4/5-1 |
| TTCN module |  |
| MCC160 Comment | MCX_HTTP_CommonFunctions.ttcn |

Before Change:

template (present) HttpRequest_Type cr_MCX_HttpRequest(template (present) charstring p_Method, template (present) charstring p_Uri := ?, template Authorization p_Authorization := *, template Host p_Host := *, template ContentType p_ContentType := *, template HttpMessageBody p_MessageBody := *) := { /* @status APPROVED (MCX) */ requestLine := cr_HttpRequestLine(p_Method, p_Uri), userAgent := *, authorization := p_Authorization, host := p_Host, contentType := p_ContentType, cacheControl := cr_HttpCacheControl, x3GPPIntendedIdentity := *, messageBody := p_MessageBody };

After Change:

template (present) HttpRequest_Type cr_MCX_HttpRequest(template (present) charstring p_Method, template (present) charstring p_Uri := ?, template Authorization p_Authorization := *, template Host p_Host := *, template ContentType p_ContentType := *, template HttpMessageBody p_MessageBody := *) := { /* @status APPROVED (MCX) */ requestLine := cr_HttpRequestLine(p_Method, p_Uri), userAgent := *, authorization := p_Authorization, host := p_Host, contentType := p_ContentType, cacheControl := cr_HttpCacheControl ifpresent, x3GPPIntendedIdentity := *, messageBody := p_MessageBody };

## Change 8

| Function name | f_MCX_HTTP_UserAuthentication_Step3_4() |
| --- | --- |
| Reason for change | The “content type” header is optional , and may not be included in HTTP GET |
| Summary of change | Accepted content type header not present for HTTP GET NOTE – prose needed on 36.579-1 Table 5.5.4.2-1 |
| TTCN module | MCX_HTTP_CommonFunctions.ttcn |
| MCC160 Comment |  |

Before change

function f_MCX_HTTP_UserAuthentication_Step3_4(MCX_ServiceType_Type p_ServiceType) runs on MCX_HTTP_PTC return MCX_OpenIdConnect_ParameterList_Type

{

var template (present) HttpRequest_Type v_HttpRequestGet := cr_MCX_HttpRequest_GET(?, -, cr_ContentTypeUrlEncoded);

var template (present) HttpRequest_Type v_HttpRequestPost := cr_MCX_HttpRequest_POST(tsc_MCX_IdMS_auth_UriPath, -, -, cr_ContentTypeUrlEncoded);

var template (present) MCX_OpenIdConnect_ParameterList_Type v_OpenIdConnectAuthenticationRequest := cr_MCX_OpenIdConnect_AuthenticationRequest(p_ServiceType,

px_MCX_OAuth_RedirectURI_A,

px_MCX_OAuth_ClientId_A);

var IP_Connection_Type v_HttpRoutingInfo;

var HTTP_DATA_IND v_HTTP_DATA_IND;

var MCX_OpenIdConnect_ParameterList_Type v_OpenIdConnectRequest;

// step 3: HTTP GET/POST

v_HTTP_DATA_IND := f_MCX_HTTP_ReceiveRequest(-, (v_HttpRequestGet, v_HttpRequestPost));

v_HttpRoutingInfo := v_HTTP_DATA_IND.routingInfo;

...

...

}

After change

function f_MCX_HTTP_UserAuthentication_Step3_4(MCX_ServiceType_Type p_ServiceType) runs on MCX_HTTP_PTC return MCX_OpenIdConnect_ParameterList_Type

{

var template (present) HttpRequest_Type v_HttpRequestGet := cr_MCX_HttpRequest_GET(?, -, cr_ContentTypeUrlEncoded ifpresent);

var template (present) HttpRequest_Type v_HttpRequestPost := cr_MCX_HttpRequest_POST(tsc_MCX_IdMS_auth_UriPath, -, -, cr_ContentTypeUrlEncoded);

var template (present) MCX_OpenIdConnect_ParameterList_Type v_OpenIdConnectAuthenticationRequest := cr_MCX_OpenIdConnect_AuthenticationRequest(p_ServiceType,

px_MCX_OAuth_RedirectURI_A,

px_MCX_OAuth_ClientId_A);

var IP_Connection_Type v_HttpRoutingInfo;

var HTTP_DATA_IND v_HTTP_DATA_IND;

var MCX_OpenIdConnect_ParameterList_Type v_OpenIdConnectRequest;

// step 3: HTTP GET/POST

v_HTTP_DATA_IND := f_MCX_HTTP_ReceiveRequest(-, (v_HttpRequestGet, v_HttpRequestPost));

v_HttpRoutingInfo := v_HTTP_DATA_IND.routingInfo;

...

...

}

## Change 9

| Function name | f_MCX_HTTP_ConfigurationGroupManagementSubscription () |
| --- | --- |
| Reason for change | The MCSUEID was defined as per the spec 36.579-1 Table 5.5.4.2-1, where it was with prefix “urn:gsma:imei-“. While as per 3GPP TS 23.003 clause 13.8: An instance-id is a SIP Contact header parameter that uniquely identifies the SIP UA performing a registration. When an IMEI is available, the instance-id shall take the form of a IMEI URN (see RFC 7254 [79]). The format of the instance-id shall take the form "urn:gsma:imei:<imeival>" …: … If no IMEI is available, the instance-id shall take the form of a string representation of a UUID as a URN as defined in IETF RFC 4122 [80]. An example of such an instance-id is as follows: EXAMPLE: urn:uuid:f81d4fae-7dec-11d0-a765-00a0c91e6bf6 The MCX client under test may use MCSUEID in form of “urn:uuid” rather than “urn:gsma”, which will unfairly fail the testcase. |
| Summary of change | Accept MCX client with MCSUEID in form of “urn:uuid” NOTE – new PICS and PIXIT defined in Change 12 and Change 13 NOTE – prose on 36.579-1 Table 5.5.4.2-1 (NOTE 3) needed |
| TTCN module | MCX_HTTP_CommonFunctions.ttcn |
| MCC160 Comment |  |

Before Change:

function f_MCX_HTTP_ConfigurationGroupManagementSubscription(MCX_ServiceType_Type p_ServiceType, MCX_SecurityParameters_Type p_SecurityParameters) runs on MCX_HTTP_PTC { var MCX_Coordination_MSG v_Dummy; var boolean v_AllowCMS := false; var boolean v_AllowGMS := false; var boolean v_UE_Config_Done := false; var boolean v_User_Profile_Done := false; var boolean v_Service_Config_Done := false; var boolean v_Group_Config_Done := false; var charstring v_MCSUEID; var charstring v_UE_Config_URI := f_MCX_InitialConfiguration_BuildURI(p_ServiceType, http_get, UECONFIG, -, v_MCSUEID); var charstring v_User_Profile_URI := f_MCX_InitialConfiguration_BuildURI(p_ServiceType, http_get, UEUSERPROF); var charstring v_Service_Config_URI := f_MCX_InitialConfiguration_BuildURI(p_ServiceType, http_get, UESERVCONFIG); var charstring v_Group_Config_URI := f_MCX_InitialConfiguration_BuildURI(p_ServiceType, http_get, GROUPCONFIG); ... ... }

After Change:

function f_MCX_HTTP_ConfigurationGroupManagementSubscription(MCX_ServiceType_Type p_ServiceType, MCX_SecurityParameters_Type p_SecurityParameters) runs on MCX_HTTP_PTC { var MCX_Coordination_MSG v_Dummy; var boolean v_AllowCMS := false; var boolean v_AllowGMS := false; var boolean v_UE_Config_Done := false; var boolean v_User_Profile_Done := false; var boolean v_Service_Config_Done := false; var boolean v_Group_Config_Done := false; var charstring v_MCSUEID; if (pc_MCX_Support_IMEI) { v_MCSUEID := f_SIP_InstanceId(); } else { v_MCSUEID := f_SIP_InstanceId_MCX(); } var charstring v_UE_Config_URI := f_MCX_InitialConfiguration_BuildURI(p_ServiceType, http_get, UECONFIG, -, v_MCSUEID); var charstring v_User_Profile_URI := f_MCX_InitialConfiguration_BuildURI(p_ServiceType, http_get, UEUSERPROF); var charstring v_Service_Config_URI := f_MCX_InitialConfiguration_BuildURI(p_ServiceType, http_get, UESERVCONFIG); var charstring v_Group_Config_URI := f_MCX_InitialConfiguration_BuildURI(p_ServiceType, http_get, GROUPCONFIG); ... ... }

## Change 10

| Function name | f_MCX_IMS_InitialServiceSettingsAndSubscriptions () |
| --- | --- |
| Reason for change | Same as Change 9 |
| Summary of change | Accept MCX client with MCSUEID in form of “urn:uuid” NOTE – new PICS and PIXIT defined in Change 12 and Change 13 NOTE – prose on 36.579-1 Table 5.5.4.2-1 (NOTE 3) needed, same as for Change 9 |
| TTCN module | MCX_SIP_Registration.ttcn |
| MCC160 Comment |  |

Before Change:

function f_MCX_IMS_InitialServiceSettingsAndSubscriptions (MCX_ServiceType_Type p_ServiceType) runs on MCX_HTTP_PTC { var InternetProtocol_Type v_Protocol := f_IMS_PTC_ImsInfo_GetTransportProtocol(); var charstring v_GroupId := f_MCX_MCService_GroupID_Group_A(p_ServiceType); var MCX_SecurityParameters_Type v_SecurityParameters; ...... ...... }

After Change:

function f_MCX_IMS_InitialServiceSettingsAndSubscriptions (MCX_ServiceType_Type p_ServiceType) runs on MCX_HTTP_PTC { var InternetProtocol_Type v_Protocol := f_IMS_PTC_ImsInfo_GetTransportProtocol(); var charstring v_MCSUEID; if (pc_MCX_Support_IMEI) { v_MCSUEID := f_SIP_InstanceId(); } else { v_MCSUEID := f_SIP_InstanceId_MCX(); } var charstring v_GroupId := f_MCX_MCService_GroupID_Group_A(p_ServiceType); var MCX_SecurityParameters_Type v_SecurityParameters; ...... ...... }

## Change 11

| Function name | (new) f_SIP_InstanceId_MCX |
| --- | --- |
| Reason for change | As per Change 9 |
| Summary of change | New function returen URN UUID |
| TTCN module | IMS_SIP_Templates.ttcn |
| MCC160 Comment |  |

New:

function f_SIP_InstanceId_MCX(charstring p_URN_UUID := px_URN_UUID_Value) return charstring { var charstring v_UrnUuidString := p_URN_UUID; return v_UrnUuidString; }

## Change 12

| Function name | (new) pc_MCX_Support_IMEI |
| --- | --- |
| Reason for change | As per Change 9 |
| Summary of change | New PICS pc_MCX_Support_IMEI NOTE –prose on 36.579-5 needed |
| TTCN module | MCX_Parameters.ttcn |
| MCC160 Comment |  |

New:

modulepar boolean pc_MCX_Support_IMEI := TRUE;

## Change 13

| Function name | (new) px_URN_UUID_Value |
| --- | --- |
| Reason for change | As per Change 9 |
| Summary of change | New PIXIT pc_MCX_Support_IMEI NOTE –prose on 36.579-5 needed |
| TTCN module | Parameters.ttcn |
| MCC160 Comment |  |

New:

modulepar charstring px_URN_UUID_Value := "urn:uuid:dut-specific-uuid-value";

## Change 14

| Function name | f_MCX_SignedKmsResponse_EncodeAndSign |
| --- | --- |
| Reason for change | As per 3GPP TS 33.180, in Section D.2.2 "The signature is valid, based on the UserUri and the InK or TrK used to sign the message". UserUri should be set as the MCPTT-Id rather than the username. |
| Summary of change | Use MCPTT-Id instead of Username |
| TTCN module | MCX_XML_mcsecKMSInterface.ttcn |
| MCC160 Comment |  |

Before Change:

function f_MCX_SignedKmsResponse_EncodeAndSign(charstring p_ClientReqUrl, template (value) KMSMessage p_KMSMessage, charstring p_MCServiceID, MCX_SecurityKey_Type p_InK) return universal charstring { var charstring v_KeyName := f_Base64Encode(p_InK.Id); // 33.180 clause 9.3.5 var charstring v_KmsResponseId := "kmsResponse"; var charstring v_ReferenceURI := "#" & v_KmsResponseId; var template (value) SignedKmsResponse v_SignedKmsResponse := cs_MCX_XML_SignedKMSResponse(v_KmsResponseId, tsc_MCX_KMS_Hostname, f_UTC_Timestamp(), // !!!! WORKAROUND: Mantis TTCN-3 Ticket 7999 !!!! tsc_MCX_MC_ID_User_A, p_ClientReqUrl, p_KMSMessage, cs_MCX_XML_Signature(-, v_ReferenceURI, v_KeyName)); var bitstring v_Bitstring := encvalue(v_SignedKmsResponse); var universal charstring v_EncodedSignedKmsResponse := f_Bit2Unichar(v_Bitstring); return fl_MCX_XML_SignKmsMessage(v_EncodedSignedKmsResponse, p_MCServiceID, p_InK); }

After Change:

function f_MCX_SignedKmsResponse_EncodeAndSign(charstring p_ClientReqUrl, template (value) KMSMessage p_KMSMessage, charstring p_MCServiceID, MCX_SecurityKey_Type p_InK) return universal charstring { var charstring v_KeyName := f_Base64Encode(p_InK.Id); // 33.180 clause 9.3.5 var charstring v_KmsResponseId := "kmsResponse"; var charstring v_ReferenceURI := "#" & v_KmsResponseId; var template (value) SignedKmsResponse v_SignedKmsResponse := cs_MCX_XML_SignedKMSResponse(v_KmsResponseId, tsc_MCX_KMS_Hostname, f_UTC_Timestamp(), // !!!! WORKAROUND: Mantis TTCN-3 Ticket 7999 !!!! px_MCPTT_ID_User_A, p_ClientReqUrl, p_KMSMessage, cs_MCX_XML_Signature(-, v_ReferenceURI, v_KeyName)); var bitstring v_Bitstring := encvalue(v_SignedKmsResponse); var universal charstring v_EncodedSignedKmsResponse := f_Bit2Unichar(v_Bitstring); return fl_MCX_XML_SignKmsMessage(v_EncodedSignedKmsResponse, p_MCServiceID, p_InK); }

## Change 15

| Function name | cr_MCX_ResourceLists_List_Sequence_Entry |
| --- | --- |
| Reason for change | The client may include additional attributes from extensions of the resourceList XSD defined in RFC 4826, e.g. copyControl attribute defined in RFC 5364 |
| Summary of change | Allow additional attributes in “entry” NOTE – prose update on 36.579-1 Table 5.5.3.3.1A-1 needed |
| TTCN module | MCX_XML_resource_lists.ttcn |
| MCC160 Comment |  |

Before Change:

template (present) ResourceLists_ListSequenceElement_Type cr_MCX_ResourceLists_List_Sequence_Entry(template (present) XSD.AnyURI p_URI) := { /* @status APPROVED (MCX) */ choice := { entry := { uri := p_URI, attr := omit, display_name := omit, elem_list := {} } } };

After Change:

template (present) ResourceLists_ListSequenceElement_Type cr_MCX_ResourceLists_List_Sequence_Entry(template (present) XSD.AnyURI p_URI) := { /* @status APPROVED (MCX) */ choice := { entry := { uri := p_URI, attr := *, display_name := omit, elem_list := {} } } };

## Change 16

| Function name | f_MCX_EUTRA_IPCAN_IdleUpdated() |
| --- | --- |
| Reason for change | - As per 36.579-1 Table 5.4.2.2.3-1, both IMS and MCX registration may happens in parallel to step 16 / 16A. The current implementation does not account for IMS registration independent to MCX registration. - Pass new parameter to f_EUTRA_DelayForUserPlaneSignalling() to indicate MCX port mapping, see Change 4 |
| Summary of change | - Moved the check of v_InitialRegistrationIMS outside the check on v_InitialRegistrationMCX, and allow the call of f_EUTRA_DelayForUserPlaneSignalling for IMS reg handling. - Set new parameter true in call to f_EUTRA_DelayForUserPlaneSignalling() |
| TTCN module | IPCAN_EUTRA.ttcn |
| MCC160 Comment |  |

Before Change:

function f_MCX_EUTRA_IPCAN_IdleUpdated(EUTRA_CellId_Type p_CellId) runs on EUTRA_PTC { var MCX_Registration_PDN_Type v_FirstPDN := tsc_MCX_InitialRegistrationPdnList[0]; var MCX_Registration_PDN_Type v_SecondPDN := tsc_MCX_InitialRegistrationPdnList[1]; var MCX_Registration_PDN_Type v_ThirdPDN := tsc_MCX_InitialRegistrationPdnList[2]; ...... ...... if (not v_InitialRegistrationMCX) { // if MCX is not registered yet ... if (v_InitialRegistrationIMS) { v_WaitForUserplaneSignalling := waitForIMS; } else { v_WaitForUserplaneSignalling := dontWaitForIMS; } // wait for IP address allocation and/or IMS registration being finished f_EUTRA_DelayForUserPlaneSignalling(v_WaitForUserplaneSignalling); // ... wait for IP address allocation and/or IMS registration and ... f_EUTRA_RRC_ConnectionRelease(p_CellId); // ... release the RRC connection // If the UE is not connected to MCX yet, trigger start of MCX and deal with additional PDN connectivity accordingly f_MCX_EUTRA_IPCAN_IdleUpdated_Step18_26(p_CellId); } };

After Change:

function f_MCX_EUTRA_IPCAN_IdleUpdated(EUTRA_CellId_Type p_CellId) runs on EUTRA_PTC { var MCX_Registration_PDN_Type v_FirstPDN := tsc_MCX_InitialRegistrationPdnList[0]; var MCX_Registration_PDN_Type v_SecondPDN := tsc_MCX_InitialRegistrationPdnList[1]; var MCX_Registration_PDN_Type v_ThirdPDN := tsc_MCX_InitialRegistrationPdnList[2]; ...... ...... // REMOVED to later - if (not v_InitialRegistrationMCX) { // if MCX is not registered yet ... if (v_InitialRegistrationIMS) { v_WaitForUserplaneSignalling := waitForIMS; } else { v_WaitForUserplaneSignalling := dontWaitForIMS; } // wait for IP address allocation and/or IMS registration being finished f_EUTRA_DelayForUserPlaneSignalling(v_WaitForUserplaneSignalling, true); // ... wait for IP address allocation and/or IMS registration and ... if (not v_InitialRegistrationMCX) { f_EUTRA_RRC_ConnectionRelease(p_CellId); // ... release the RRC connection // If the UE is not connected to MCX yet, trigger start of MCX and deal with additional PDN connectivity accordingly f_MCX_EUTRA_IPCAN_IdleUpdated_Step18_26(p_CellId); } };

## Change 17

| Function name | f_MCX_OpenID_DecodeAndCheckQueryParameters() |
| --- | --- |
| Reason for change | ‘Scope’ parameters is not mandatory parameters as per Access token request standard required parameters as per defined in 3GPP TS 33.180, clause B.4.2.4 However, RFC 6749 section 3.3 where authorization and token endpoints allow the client to specify the scope of the access request using the "scope" request parameter The current TTCN query list only allows parameter according to 3GPP TS 33.180 |
| Summary of change | Corrected in the function to allow additional (optional) parameters |
| TTCN module | MCX_OpenIdConnect.ttcn |
| MCC160 Comment |  |

Before change

function f_MCX_OpenID_DecodeAndCheckQueryParameters(charstring p_Parameters,

template (present) MCX_OpenIdConnect_ParameterList_Type p_QueryList) runs on MCX_HTTP_PTC return MCX_OpenIdConnect_ParameterList_Type

{

var template (value) MCX_OpenIdConnect_Parameter_Type v_Parameter;

var MCX_OpenIdConnect_ParameterList_Type v_OpenIdParameterList := {};

var Char1List_Type v_Separator := {"&"};

......

......

for (i := 0; i < lengthof(v_ParameterList); i := i + 1) {

v_Separator := {"="};

v_CharStringList := f_StringSplit(v_ParameterList[i], v_Separator);

v_Name := v_CharStringList[0];

if (lengthof(v_CharStringList) != 2) {

f_MCX_HTTP_SetVerdictFailOrInconc(__FILE__, __LINE__, "Invalid OAuth 2.0 request parameter");

}

v_Value := f_URL_Decoding(v_CharStringList[1]);

select (v_Name) {

case ("scope") {

l := 0;

v_Separator := {" "};

v_CharStringList := f_StringSplit(v_Value, v_Separator);

for (k := 0; k < lengthof(v_CharStringList); k := k + 1) {

v_Value := v_CharStringList[k];

if (v_Value != "") {

v_ScopeList[l] := v_Value;

l := l + 1;

}

}

v_Parameter := cs_MCX_OpenIdConnect_NameValueListParameter(v_Name, v_ScopeList);

}

case else {

v_Parameter := cs_MCX_OpenIdConnect_NameValueParameter(v_Name, v_Value);

}

}

v_OpenIdParameterList[n] := valueof(v_Parameter);

n := n + 1;

}

if (not match(v_OpenIdParameterList, p_QueryList)) {

f_MCX_HTTP_SetVerdictFailOrInconc(__FILE__, __LINE__, "Invalid query");

}

return v_OpenIdParameterList;

}

After change

function f_MCX_OpenID_DecodeAndCheckQueryParameters(charstring p_Parameters,

template (present) MCX_OpenIdConnect_ParameterList_Type p_QueryList) runs on MCX_HTTP_PTC return MCX_OpenIdConnect_ParameterList_Type

{

var template (value) MCX_OpenIdConnect_Parameter_Type v_Parameter;

var MCX_OpenIdConnect_ParameterList_Type v_OpenIdParameterList := {};

var Char1List_Type v_Separator := {"&"};

......

......

for (i := 0; i < lengthof(v_ParameterList); i := i + 1) {

v_Separator := {"="};

v_CharStringList := f_StringSplit(v_ParameterList[i], v_Separator);

v_Name := v_CharStringList[0];

if (lengthof(v_CharStringList) != 2) {

f_MCX_HTTP_SetVerdictFailOrInconc(__FILE__, __LINE__, "Invalid OAuth 2.0 request parameter");

}

v_Value := f_URL_Decoding(v_CharStringList[1]);

select (v_Name) {

case ("scope") {

l := 0;

v_Separator := {" "};

v_CharStringList := f_StringSplit(v_Value, v_Separator);

for (k := 0; k < lengthof(v_CharStringList); k := k + 1) {

v_Value := v_CharStringList[k];

if (v_Value != "") {

v_ScopeList[l] := v_Value;

l := l + 1;

}

}

v_Parameter := cs_MCX_OpenIdConnect_NameValueListParameter(v_Name, v_ScopeList);

}

case else {

v_Parameter := cs_MCX_OpenIdConnect_NameValueParameter(v_Name, v_Value);

}

}

v_OpenIdParameterList[n] := valueof(v_Parameter);

n := n + 1;

}

if (not match(v_OpenIdParameterList, superset (all from p_QueryList))) {

f_MCX_HTTP_SetVerdictFailOrInconc(__FILE__, __LINE__, "Invalid query");

}

return v_OpenIdParameterList;

}

## Change 18

| Function name | MIKEY_CryptoSession (new) Null_Type2 |
| --- | --- |
| Reason for change | The current type definition of MIKEY_CryptoSession is union in TTCN, with two of the option (SRTP, EmptyMap) on Null_Type. While as per RFC 3830, the contents type is determined by csIdMap in the parent message, and when SRTP or EmptyMap is selected, the MIKEY_CryptoSession is omit. The above encode rule is incompatible with the moral union + Null_Type type use in current TTCN, and cause decode error with the default handling. |
| Summary of change | It is suggested to add a new type Null_Type2 with TCI encode rule, to distinguish and assist the codec for MIKEY_CryptoSession message |
| TTCN module | MIKEY_TypeDefs.ttcn |
| MCC160 Comment |  |

Before Change:

type record MIKEY_CommonHeader { /* RFC 3830 clause 6.1: Common Header payload (HDR) */ O1_Type version, O1_Type dataType, O1_Type nextPayload, B1_Type v, B7_Type prfFunc, B32_Type csbId, O1_Type numberCS, O1_Type csIdMap, /* CS ID map type: '00'O SRTP-ID (RFC 3830 clause 6.1) '01'O empty map (RFC 4563 clause 5) '02'O GENERIC-ID (RFC 6043 clause 6.1)*/ MIKEY_CryptoSession csIdMapInfo /* CS ID map info depending on CS ID map type */ }; type union MIKEY_CryptoSession { /* crypto session which correspond to the CS ID map value in the Common Header payload (HDR) */ Null_Type SRTP, /* place holder for SRTP ID according to RFC 3830 clause 6.1.1 */ Null_Type EmptyMap, /* 'CS ID Map Info' shall have length '0' (33.180 E.1.2, RFC 4563) */ MIKEY_GenericIdMapType GENERIC /* GENERIC-ID Map Type according to RFC 6043 clause 6.1.1 */ };

After Change:

type boolean Null_Type2 (true) with {encode "TCI"}; ...... ...... type record MIKEY_CommonHeader { /* RFC 3830 clause 6.1: Common Header payload (HDR) */ O1_Type version, O1_Type dataType, O1_Type nextPayload, B1_Type v, B7_Type prfFunc, B32_Type csbId, O1_Type numberCS, O1_Type csIdMap, /* CS ID map type: '00'O SRTP-ID (RFC 3830 clause 6.1) '01'O empty map (RFC 4563 clause 5) '02'O GENERIC-ID (RFC 6043 clause 6.1)*/ MIKEY_CryptoSession csIdMapInfo /* CS ID map info depending on CS ID map type */ }; type union MIKEY_CryptoSession { /* crypto session which correspond to the CS ID map value in the Common Header payload (HDR) */ Null_Type2 SRTP, /* place holder for SRTP ID according to RFC 3830 clause 6.1.1 */ Null_Type2 EmptyMap, /* 'CS ID Map Info' shall have length '0' (33.180 E.1.2, RFC 4563) */ MIKEY_GenericIdMapType GENERIC /* GENERIC-ID Map Type according to RFC 6043 clause 6.1.1 */ };

## Change 19

| Function name | f_IMS_SubscribeRequest_MessageHeaderRX() |
| --- | --- |
| Reason for change | As per RFC 3261, cl. 20.42, A Via header field value contains the transport protocol used to send the message, the client's host name or network address, and possibly the port number at which it wishes to receive responses. For Subscript request, the port to receive the resp is port_us for UDP, and port_uc for TCP, as clearly illustrated in TS 36.523-3, Figure 4.2.5.2.3.1-2 for IPsec Usage of ports and SAs in UDP and TCP transport. The current TTCN implementation only cover the case for UDP |
| Summary of change | In case of TCP, via header field handled to check protected client port at UE applies NOTE – prose update needed on TS 36.579-1 Table 5.5.2.14-1 – via/sent-by/port |
| TTCN module | IMS_Procedures_RegistrationCommon.ttcn |
| MCC160 Comment |  |

Before Change:

function f_IMS_SubscribeRequest_MessageHeaderRX(EventType p_EventType := "reg", template (present) SipUrl p_SipUrlTo := ?, template (present) charstring p_ExpireTime := "600000", template (present) SemicolonParam_List p_MandatoryContactParams := {}) runs on IMS_PTC return template (present) MessageHeader { /* p_EventType: ...... ...... var template (present) MessageHeader v_MessageHeader_Subscribe := cr_MessageHeader_Dummy; var template (present) PAccessNetworkInfo v_PAccessNetworkInfo := cr_PAccessNetworkInfo(f_IMS_PTC_GetRanType()); var template (present) SipUrl v_SipUrlFrom := f_IMS_PTC_ImsInfo_GetSipUriAnyRX(); // @sic R5s211251: per default "any IMPU within the set of IMPUs on ISIM" sic@ var template (present) SipUrl v_SipUrlTo := p_SipUrlTo; var template integer v_Port_ps_Route := f_IMS_PTC_GetPort_ps(); // @sic R5s211251 sic@ var template integer v_Port_us_Contact := f_IMS_PTC_GetPort_us(); // @sic R5s211251 sic@ var template integer v_Port_us_Via := v_Port_us_Contact; var boolean v_IsGiba := f_IMS_PTC_SecurityScheme_IsGiba(); ...... ...... v_MessageHeader_Subscribe.route := cr_Route(cr_RouteBodyList_InitialReq(v_Port_ps_Route)); /* @sic R5s140934 change 1 sic@ P-CSCF address is checked by f_IMS_MessageHeader_Request_CheckRoute */ v_MessageHeader_Subscribe.via := cr_ViaDef(-, cr_HostPort(?, v_Port_us_Via)); /* host address needs to be checked after receiving (f_IMS_MessageHeader_Request_CheckVia) @sic R5s120727 change 6.1 sic@ */ v_MessageHeader_Subscribe.contact := cr_Contact(f_Contact_SipUri_HostPortRX(?, v_Port_us_Contact), p_MandatoryContactParams); /* host address needs to be checked after receiving (f_IMS_MessageHeader_CheckContactAddr) @sic R5s120727 change 6.1 sic@ @sic R5s130925 change 1: UE may use any port as server port sic@ @sic R5s150721, R5-153760: cr_SipUri_HostPort replaced by f_Contact_SipUri_HostPortRX sic@ */ return v_MessageHeader_Subscribe; }

After Change:

function f_IMS_SubscribeRequest_MessageHeaderRX(EventType p_EventType := "reg", template (present) SipUrl p_SipUrlTo := ?, template (present) charstring p_ExpireTime := "600000", template (present) SemicolonParam_List p_MandatoryContactParams := {}) runs on IMS_PTC return template (present) MessageHeader { /* p_EventType: ...... ...... var template (present) MessageHeader v_MessageHeader_Subscribe := cr_MessageHeader_Dummy; var template (present) PAccessNetworkInfo v_PAccessNetworkInfo := cr_PAccessNetworkInfo(f_IMS_PTC_GetRanType()); var template (present) SipUrl v_SipUrlFrom := f_IMS_PTC_ImsInfo_GetSipUriAnyRX(); // @sic R5s211251: per default "any IMPU within the set of IMPUs on ISIM" sic@ var template (present) SipUrl v_SipUrlTo := p_SipUrlTo; var template integer v_Port_ps_Route := f_IMS_PTC_GetPort_ps(); // @sic R5s211251 sic@ var template integer v_Port_us_Contact := f_IMS_PTC_GetPort_us(); // @sic R5s211251 sic@ var template integer v_Port_us_Via := v_Port_us_Contact; var boolean v_IsGiba := f_IMS_PTC_SecurityScheme_IsGiba(); var InternetProtocol_Type v_Protocol := f_IMS_PTC_ImsInfo_GetTransportProtocol(); var IMS_ProtectedPorts_Type v_Protected := f_IMS_PTC_Security_GetProtectedPorts(); ...... ...... v_MessageHeader_Subscribe.route := cr_Route(cr_RouteBodyList_InitialReq(v_Port_ps_Route)); /* @sic R5s140934 change 1 sic@ P-CSCF address is checked by f_IMS_MessageHeader_Request_CheckRoute */ if (v_Protocol == tcp) { // v_MessageHeader_Subscribe.via := cr_ViaDef(-, cr_HostPort(?, v_Protected.Port_uc)); } else { v_MessageHeader_Subscribe.via := cr_ViaDef(-, cr_HostPort(?, v_Port_us_Via)); /* host address needs to be checked after receiving (f_IMS_MessageHeader_Request_CheckVia) @sic R5s120727 change 6.1 sic@ */ } v_MessageHeader_Subscribe.contact := cr_Contact(f_Contact_SipUri_HostPortRX(?, v_Port_us_Contact), p_MandatoryContactParams); /* host address needs to be checked after receiving (f_IMS_MessageHeader_CheckContactAddr) @sic R5s120727 change 6.1 sic@ @sic R5s130925 change 1: UE may use any port as server port sic@ @sic R5s150721, R5-153760: cr_SipUri_HostPort replaced by f_Contact_SipUri_HostPortRX sic@ */ return v_MessageHeader_Subscribe; }

## Change 20

| Function name | f_MCX_PublishRequest_MessageHeaderRX () |
| --- | --- |
| Reason for change | Same as Change 19, the TCP port checking is not covered |
| Summary of change | In case of TCP, via header field handled to check protected client port at UE applies NOTE – prose update needed on TS 36.579-1 Table 5.5.2.11-1– via/sent-by/port |
| TTCN module | MCX_SIP_Registration.ttcn |
| MCC160 Comment |  |

Before Change:

function f_MCX_PublishRequest_MessageHeaderRX(MCX_ServiceType_Type p_ServiceType, template (present) SipUrl p_SipUrlTo, EventType p_EventType, charstring p_ExpiresDeltaSec) runs on IMS_PTC return template (present) MessageHeader { var template (present) MessageHeader v_MessageHeader_Publish := cr_MessageHeader_Dummy; var template (present) SipUrl v_SipUrlFrom := f_MCX_DefaultPublicUserIdRX(); /* @sic R5-212302: port not present sic@ */ var IMS_ProtectedPorts_Type v_Protected := f_IMS_PTC_Security_GetProtectedPorts(); var integer v_Port_ps := v_Protected.Port_ps; var integer v_Port_us := v_Protected.Port_us; v_MessageHeader_Publish.route := cr_Route(cr_RouteBodyList_InitialReq(v_Port_ps)); /* P-SCCF needs to be checked after receiving (f_IMS_MessageHeader_Request_CheckRoute) */ v_MessageHeader_Publish.via := cr_ViaDef(-, cr_HostPort(?, v_Port_us)); /* host address needs to be checked after receiving (f_IMS_MessageHeader_Request_CheckVia) */ ...... ...... }

After Change:

function f_MCX_PublishRequest_MessageHeaderRX(MCX_ServiceType_Type p_ServiceType, template (present) SipUrl p_SipUrlTo, EventType p_EventType, charstring p_ExpiresDeltaSec) runs on IMS_PTC return template (present) MessageHeader { var template (present) MessageHeader v_MessageHeader_Publish := cr_MessageHeader_Dummy; var template (present) SipUrl v_SipUrlFrom := f_MCX_DefaultPublicUserIdRX(); /* @sic R5-212302: port not present sic@ */ var IMS_ProtectedPorts_Type v_Protected := f_IMS_PTC_Security_GetProtectedPorts(); var integer v_Port_ps := v_Protected.Port_ps; var integer v_Port_us := v_Protected.Port_us; var InternetProtocol_Type v_Protocol := f_IMS_PTC_ImsInfo_GetTransportProtocol(); v_MessageHeader_Publish.route := cr_Route(cr_RouteBodyList_InitialReq(v_Port_ps)); /* P-SCCF needs to be checked after receiving (f_IMS_MessageHeader_Request_CheckRoute) */ if (v_Protocol == tcp) { v_MessageHeader_Publish.via := cr_ViaDef(-, cr_HostPort(?, v_Protected.Port_uc)); } else { v_MessageHeader_Publish.via := cr_ViaDef(-, cr_HostPort(?, v_Port_us)); /* host address needs to be checked after receiving (f_IMS_MessageHeader_Request_CheckVia) */ } ...... ...... }

## Change 21

| Function name | fl_MCX_SIP_Registration_McsInfo () |
| --- | --- |
| Reason for change | As per 36.579-1 Table 5.5.3.2.1-1 client ID is not mandatory for SIP SUBSCRIBE and at SIP PUBLISH we are receiving client ID. There TTCN is trying to get the client Id which is not being set before. Resulting is failing with mismatch. |
| Summary of change | Check if the client ID not set before it will accept any value. |
| TTCN module | MCX_SIP_Registration.ttcn |
| MCC160 Comment |  |

Before Change:

function fl_MCX_SIP_Registration_McsInfo(MCX_ServiceType_Type p_ServiceType, IMS_Request_Type p_IMS_Request, boolean p_PublishWithServiceAuthorization := false) runs on MCX_IMS_PTC return template (present) MCX_ServiceInfo_Type { var MCX_ServiceType_Type v_ServiceType := p_ServiceType; var charstring v_AccessToken := f_MCX_IMS_PTC_SecurityParameters_GetAccessToken(); var template (omit) charstring v_CurrentClientId := f_MCX_IMS_PTC_ClientId_Get(); var template (present) charstring v_ExpectedClientId := pattern "urn:uuid:[a-zA-Z0-9]#8-[a-zA-Z0-9]#4-[a-zA-Z0-9]#4-[a-zA-Z0-9]#4-[a-zA-Z0-9]#12"; var template (present) MCPTT_Info_Type v_MCPTT_InfoTemplate := cr_MCPTT_Info_Base; var template (present) MCVideo_Info_Type v_MCVideo_InfoTemplate := cr_MCVideo_Info_Base; var template (present) MCData_Info_Type v_MCData_InfoTemplate := cr_MCData_Info_Base; var template (present) MCX_ServiceInfo_Type v_MCS_Info := cs_MCX_Service_Info_Init; if (isvalue(v_CurrentClientId)) { v_ExpectedClientId := valueof(v_CurrentClientId); } select union (p_IMS_Request) { ...... ......

After Change:

function fl_MCX_SIP_Registration_McsInfo(MCX_ServiceType_Type p_ServiceType, IMS_Request_Type p_IMS_Request, boolean p_PublishWithServiceAuthorization := false) runs on MCX_IMS_PTC return template (present) MCX_ServiceInfo_Type { var MCX_ServiceType_Type v_ServiceType := p_ServiceType; var charstring v_AccessToken := f_MCX_IMS_PTC_SecurityParameters_GetAccessToken(); var template (omit) charstring v_CurrentClientId := f_MCX_IMS_PTC_ClientId_Get(); var template (present) charstring v_ExpectedClientId := pattern "urn:uuid:[a-zA-Z0-9]#8-[a-zA-Z0-9]#4-[a-zA-Z0-9]#4-[a-zA-Z0-9]#4-[a-zA-Z0-9]#12"; var template (present) MCPTT_Info_Type v_MCPTT_InfoTemplate := cr_MCPTT_Info_Base; var template (present) MCVideo_Info_Type v_MCVideo_InfoTemplate := cr_MCVideo_Info_Base; var template (present) MCData_Info_Type v_MCData_InfoTemplate := cr_MCData_Info_Base; var template (present) MCX_ServiceInfo_Type v_MCS_Info := cs_MCX_Service_Info_Init; if (isvalue(v_CurrentClientId)) { v_ExpectedClientId := valueof(v_CurrentClientId); } else { v_ExpectedClientId := ?; } select union (p_IMS_Request) { ...... ......

## Change 22

| Function name | f_MCX_SIP_Registration (new) f_MCX_IMS_PTC_ImsInfo_SetTransportProtocol |
| --- | --- |
| Reason for change | When MCX UE sends SIP message over TCP, it expects the received message over TCP protocol. Current TTCN implementation always send Norify over UDP |
| Summary of change | To allow send the Notify over TCP /UDP based on previous SIP messages, setting the protocol value fetched from MCX SIP Register globally. |
| TTCN module | MCX_SIP_Registration.ttcn |
| MCC160 Comment |  |

Before Change:

function f_MCX_SIP_Registration(MCX_ServiceType_Type p_ServiceType) runs on MCX_IMS_PTC return IMS_DATA_REQ { var IMS_DATA_REQ v_IMS_DATA_REQ_ByRef; var IMS_DATA_REQ v_IMS_DATA_REQ; var REGISTER_Request v_RegisterReq; var MIME_Message v_MIME_Message; var template (present) SemicolonParam_List v_MandatoryContactParams := f_MCX_SIP_Register_ContactParams_RX(p_ServiceType); var template (present) MCX_ServiceInfo_Type v_MCS_Info; var MCX_MIME_XmlBodies_Type v_XmlBodies; // Step 1 v_IMS_DATA_REQ := f_IMS_REGISTER_InitialRequest(-, -, v_MandatoryContactParams); // check REGISTER and init GlobalInfo v_RegisterReq := v_IMS_DATA_REQ.Request.Register; f_IMS_Register_SecurityInit_ImsSecurity(v_RegisterReq); // Step 2 IMS_Server.send(cas_IMS_DATA_RSP(f_IMS_RoutingInfo_ULtoDL(v_IMS_DATA_REQ.RoutingInfo), cs_Response(c_statusLine401, f_IMS_RegisterResponse_401_MessageHeaderTX(v_RegisterReq)))); ...... ...... }

After Change:

function f_MCX_SIP_Registration(MCX_ServiceType_Type p_ServiceType) runs on MCX_IMS_PTC return IMS_DATA_REQ { var IMS_DATA_REQ v_IMS_DATA_REQ_ByRef; var IMS_DATA_REQ v_IMS_DATA_REQ; var REGISTER_Request v_RegisterReq; var MIME_Message v_MIME_Message; var template (present) SemicolonParam_List v_MandatoryContactParams := f_MCX_SIP_Register_ContactParams_RX(p_ServiceType); var template (present) MCX_ServiceInfo_Type v_MCS_Info; var MCX_MIME_XmlBodies_Type v_XmlBodies; // Step 1 v_IMS_DATA_REQ := f_IMS_REGISTER_InitialRequest(-, -, v_MandatoryContactParams); // check REGISTER and init GlobalInfo v_RegisterReq := v_IMS_DATA_REQ.Request.Register; f_IMS_Register_SecurityInit_ImsSecurity(v_RegisterReq); f_MCX_IMS_PTC_ImsInfo_SetTransportProtocol(v_IMS_DATA_REQ.RoutingInfo); // Step 2 IMS_Server.send(cas_IMS_DATA_RSP(f_IMS_RoutingInfo_ULtoDL(v_IMS_DATA_REQ.RoutingInfo), cs_Response(c_statusLine401, f_IMS_RegisterResponse_401_MessageHeaderTX(v_RegisterReq)))); ...... ...... } function f_MCX_IMS_PTC_ImsInfo_SetTransportProtocol(IMS_RoutingInfo_Type p_RoutingInfo_DL)runs on IMS_PTC { var IMS_RoutingInfo_Type v_RoutingInfo := p_RoutingInfo_DL; vc_IMS_Global.ImsInfo.TransportProtocol := v_RoutingInfo.Protocol; }

## Change 23

| Function name | cs_MCPTT_UserProfile |
| --- | --- |
| Reason for change | The per TS 23.280, The MC Service ID shall be a URI. URI shall contain ‘sip:’. As such the px_MCPTT_ID_User_A shall already containing the ‘sip:’ prefix. We need to removed the ‘sip:’ concantenation from the XUI-URI attribute, that results in duplicated prefix |
| Summary of change | Removed the ‘sip:’ concantenation from the XUI-URI attribute NOTE - prose update needed on 36.579-1 Table 5.5.8.3-1 XUI-URI attribute |
| TTCN module | MCX_XML_Configuration_MCPTT.ttcn |
| MCC160 Comment |  |

Before Change:

template (value) @fuzzy MCPTT_UserProfile_Type cs_MCPTT_UserProfile(template (value) MCPTT_UserProfile_OnNetwork_Type p_OnNetwork := cs_MCPTT_UserProfile_OnNetwork) := { /* 36.579-1 Table 5.5.8.3-1: MCPTT User Profile Defaults @status APPROVED (MCX) */ /* !!!! WORKAROUND: @fuzzy to ensure that fl_MCPTT_UserProfile_Extension (calling f_GetTestcaseAttrib_Disallow_automatic_commencement) is evaluated at the time when cs_MCPTT_UserProfile is used !!!! !!!! @fuzzy to be removed when there is a generic solution (supported by Titan) !!!! !!!! to avoid @fuzzy fl_MCPTT_UserProfile_Extension shall be handed over as parameter !!!! */ xUI_URI := "sip:" & px_MCPTT_ID_User_A, // @sic R5-217985: same as the XUI value of the Document URI sic@ user_profile_index := tsc_MCX_ProfileId_MCPTT, attr := omit, choice_list := { { status := true }, { profileName := cs_MCPTT_UserProfile_Name("mcptt-user-profile-" & int2str(tsc_MCX_ProfileId_MCPTT) & ".xml") }, // @sic R5-217985: name of the user profile document; user-profile-index is the value of the user-profile-index attribute sic@ { common := cs_MCPTT_UserProfile_Common }, { offNetwork := cs_MCPTT_UserProfile_OffNetwork }, { onNetwork := p_OnNetwork }, { elem_list := fl_MCPTT_UserProfile_Extension() } } };

After Change:

template (value) @fuzzy MCPTT_UserProfile_Type cs_MCPTT_UserProfile(template (value) MCPTT_UserProfile_OnNetwork_Type p_OnNetwork := cs_MCPTT_UserProfile_OnNetwork) := { /* 36.579-1 Table 5.5.8.3-1: MCPTT User Profile Defaults @status APPROVED (MCX) */ /* !!!! WORKAROUND: @fuzzy to ensure that fl_MCPTT_UserProfile_Extension (calling f_GetTestcaseAttrib_Disallow_automatic_commencement) is evaluated at the time when cs_MCPTT_UserProfile is used !!!! !!!! @fuzzy to be removed when there is a generic solution (supported by Titan) !!!! !!!! to avoid @fuzzy fl_MCPTT_UserProfile_Extension shall be handed over as parameter !!!! */ xUI_URI := /* REMOVED "sip:" & */ px_MCPTT_ID_User_A, // @sic R5-217985: same as the XUI value of the Document URI sic@ user_profile_index := tsc_MCX_ProfileId_MCPTT, attr := omit, choice_list := { { status := true }, { profileName := cs_MCPTT_UserProfile_Name("mcptt-user-profile-" & int2str(tsc_MCX_ProfileId_MCPTT) & ".xml") }, // @sic R5-217985: name of the user profile document; user-profile-index is the value of the user-profile-index attribute sic@ { common := cs_MCPTT_UserProfile_Common }, { offNetwork := cs_MCPTT_UserProfile_OffNetwork }, { onNetwork := p_OnNetwork }, { elem_list := fl_MCPTT_UserProfile_Extension() } } };

## Change 24

| Function name | f_NAS_GetProtocolConfigOptionList |
| --- | --- |
| Reason for change | It is observed the UE may send RRCConnectinRequest immediately following the RRC release, due to DNS query from UE. This will interrupt the test sequence. |
| Summary of change | It is suggested to apply the same solution as agreed R5-231185 for NR, ie. do not incluee DNS server IP address in ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message. NOTE- prose update needed on 36.805-1 Table 4.7.3-6, with similar change in R5-231185 |
| TTCN module | NAS_AuxiliaryDefsAndFunctions |
| MCC160 Comment |  |

Before Change:

function f_NAS_GetProtocolConfigOptionList(NAS_ProtocolConfigOptions_Type p_ConfigOptionsRX, PDN_Index_Type p_PdnIndex := PDN_1, template (value) NAS_ProtocolConfigOptions_Type p_AdditionalProtocolConfigOptions := {}, boolean p_IgnoreIM_CN_SubsystemSignalingFlag := true) return template (value) NAS_ProtocolConfigOptions_Type { /* @sic R5-198996: p_AdditionalProtocolConfigOptions for 5GC options sic@ */ var PDN_AddressInfo_Type v_PDN_AddressInfo := f_PDN_AddressInfo_Get(p_PdnIndex); var template (value) NAS_ProtocolConfigOptions_Type v_ProtocolContainerList := {}; var integer v_PcoCnt := 0; var O2_Type v_ProtocolId; var octetstring v_Contents; var integer i; for (i := 0; i < lengthof(p_ConfigOptionsRX); i := i + 1) { v_ProtocolId := p_ConfigOptionsRX[i].protocolID; v_Contents := ''O; select (v_ProtocolId) { // See 24.008 Table 10.5.154 case ('0001'O) { // P-CSCF IPv6 address if (pc_IMS) { v_Contents := f_Convert_IPv6Addr2OctString(v_PDN_AddressInfo.PCSCF_IPAddressIPv6); } } case ('0002'O) { // IM CN Subsystem Signalling Flag, reply with an empty container if (not p_IgnoreIM_CN_SubsystemSignalingFlag) { v_ProtocolContainerList[v_PcoCnt] := cs_ProtocolContainer_Common(v_ProtocolId); v_PcoCnt := v_PcoCnt + 1; continue; } } case ('0003'O) { // DNS Server IPv6 address v_Contents := f_Convert_IPv6Addr2OctString(v_PDN_AddressInfo.DNS_ServerAddressIPv6); } case ('000C'O) { // P-CSCF IPv4 address if (pc_IMS) { v_Contents := f_Convert_IPv4Addr2OctString(v_PDN_AddressInfo.PCSCF_IPAddressIPv4); } } case ('000D'O) { // DNS Server IPv4 address v_Contents := f_Convert_IPv4Addr2OctString(v_PDN_AddressInfo.DNS_ServerAddressIPv4); } } if (lengthof(v_Contents) > 0) { v_ProtocolContainerList[v_PcoCnt] := cs_ProtocolContainer(v_ProtocolId, v_Contents); v_PcoCnt := v_PcoCnt + 1; } } return v_ProtocolContainerList & p_AdditionalProtocolConfigOptions; }

After Change:

function f_NAS_GetProtocolConfigOptionList(NAS_ProtocolConfigOptions_Type p_ConfigOptionsRX, PDN_Index_Type p_PdnIndex := PDN_1, template (value) NAS_ProtocolConfigOptions_Type p_AdditionalProtocolConfigOptions := {}, boolean p_IgnoreIM_CN_SubsystemSignalingFlag := true) return template (value) NAS_ProtocolConfigOptions_Type { /* @sic R5-198996: p_AdditionalProtocolConfigOptions for 5GC options sic@ */ var PDN_AddressInfo_Type v_PDN_AddressInfo := f_PDN_AddressInfo_Get(p_PdnIndex); var template (value) NAS_ProtocolConfigOptions_Type v_ProtocolContainerList := {}; var integer v_PcoCnt := 0; var O2_Type v_ProtocolId; var octetstring v_Contents; var integer i; for (i := 0; i < lengthof(p_ConfigOptionsRX); i := i + 1) { v_ProtocolId := p_ConfigOptionsRX[i].protocolID; v_Contents := ''O; select (v_ProtocolId) { // See 24.008 Table 10.5.154 case ('0001'O) { // P-CSCF IPv6 address if (pc_IMS) { v_Contents := f_Convert_IPv6Addr2OctString(v_PDN_AddressInfo.PCSCF_IPAddressIPv6); } } case ('0002'O) { // IM CN Subsystem Signalling Flag, reply with an empty container if (not p_IgnoreIM_CN_SubsystemSignalingFlag) { v_ProtocolContainerList[v_PcoCnt] := cs_ProtocolContainer_Common(v_ProtocolId); v_PcoCnt := v_PcoCnt + 1; continue; } } case ('0003'O) { // DNS Server IPv6 address if (f_GetTestcaseAttrib_XCAP(testcasename()) { v_Contents := f_Convert_IPv6Addr2OctString(v_PDN_AddressInfo.DNS_ServerAddressIPv6); } } case ('000C'O) { // P-CSCF IPv4 address if (pc_IMS) { v_Contents := f_Convert_IPv4Addr2OctString(v_PDN_AddressInfo.PCSCF_IPAddressIPv4); } } case ('000D'O) { // DNS Server IPv4 address if (f_GetTestcaseAttrib_XCAP(testcasename()) { v_Contents := f_Convert_IPv4Addr2OctString(v_PDN_AddressInfo.DNS_ServerAddressIPv4); } } } if (lengthof(v_Contents) > 0) { v_ProtocolContainerList[v_PcoCnt] := cs_ProtocolContainer(v_ProtocolId, v_Contents); v_PcoCnt := v_PcoCnt + 1; } } return v_ProtocolContainerList & p_AdditionalProtocolConfigOptions; }

## Change 25

| Function name | (new) f_GetTestcaseAttrib_XCAP |
| --- | --- |
| Reason for change | Same as Change 24 |
| Summary of change | Added support function f_GetTestcaseAttrib_XCAP |
| TTCN module | TestcaseProperties.ttcn |
| MCC160 Comment |  |

New:

function f_GetTestcaseAttrib_XCAP(charstring p_Testcase) return boolean { return false; }

## Change 26

| Function name | f_MCX_MCPTTInfo_DecryptDecode() |
| --- | --- |
| Reason for change | According to 33.180 clause 9.3.2 Confidentiality protection may be applied to the entire XML document or to the following individual identifiers and values: The current implementation only handle the case that the protection is applied to individual indentifiers. |
| Summary of change | Added handling for encryption on “entire XML document” if the UE choose to do so. Kept UE choise in global variable NOTE - prose update needed on TS 36.579-1 Uplink - Table 5.5.3.2.1-1 (mcptt-info) Table 5.5.3.3.1A-1 (resource-list xml) Downlink - Table 5.5.3.12-1 (xcap-diff - SIP NOTIFY ) NOTE – additional changes (for supporting entire XML protection) are listed in the remaining part of section 2. |
| TTCN module | MCX_XML_mcptt_info.ttcn |
| MCC160 Comment |  |

Before Change:

function f_MCX_MCPTTInfo_DecryptDecode(XmlBody_Type p_XmlDocument, charstring p_MCServiceID, template (omit) MCX_SecurityKey_Type p_CSK) runs on IMS_PTC return Mcpttinfo_Type { var universal charstring v_XmlDocument := p_XmlDocument; var XML_Document_DecryptElementContent_Result_Type v_DecryptResult; var charstring v_ElementName; var charstring v_XmlPath; var integer i; if (isvalue(p_CSK) and tsc_MCX_XML_ConfidentialityProtection) { for (i := 0; i < lengthof(tsc_MCPTT_Info_EncryptedElements); i := i + 1) { v_ElementName := tsc_MCPTT_Info_EncryptedElements[i]; v_XmlPath := "//mcpttinfo/mcptt-Params/" & v_ElementName; v_DecryptResult := f_XML_Document_DecryptElementContent(v_XmlDocument, v_XmlPath, p_MCServiceID, valueof(p_CSK)); // v_XmlDocument: inout parameter if (v_DecryptResult == NOT_ENCRYPTED) { f_IMS_SetVerdictFailOrInconc(__FILE__, __LINE__, v_ElementName & " is not encrypted"); } } fx_XML_Log(__FILE__, __LINE__, UPLINK, v_XmlDocument); } return fl_MCX_MCPTTInfo_Decode(v_XmlDocument); } }

After Change:

function f_MCX_MCPTTInfo_DecryptDecode(XmlBody_Type p_XmlDocument, charstring p_MCServiceID, template (omit) MCX_SecurityKey_Type p_CSK) runs on IMS_PTC return Mcpttinfo_Type { var universal charstring v_XmlDocument := p_XmlDocument; var XML_Document_DecryptElementContent_Result_Type v_DecryptResult; var charstring v_ElementName; var charstring v_XmlPath; var universal charstring v_DecryptedContent; var integer i; var boolean v_ElementEncryption := false; //Full Body Decryption if (isvalue(p_CSK) and tsc_MCX_XML_ConfidentialityProtection) { for (i := 0; i < lengthof(tsc_MCPTT_Info_EncryptedElements); i := i + 1) { v_ElementName := tsc_MCPTT_Info_EncryptedElements[i]; v_XmlPath := "//mcpttinfo/mcptt-Params/" & v_ElementName; v_DecryptResult := f_XML_Document_DecryptElementContent(v_XmlDocument, v_XmlPath, p_MCServiceID, valueof(p_CSK)); // v_XmlDocument: inout parameter if(v_DecryptResult == OK and v_ElementEncryption == false) { v_ElementEncryption := true; vc_IMS_Global.ElementEncryption := v_ElementEncryption; } if (v_DecryptResult == NOT_ENCRYPTED) { f_IMS_SetVerdictFailOrInconc(__FILE__, __LINE__, v_ElementName & " is not encrypted"); } } if (v_DecryptResult == NOT_FOUND and v_ElementEncryption == false) { //Full Body Decryption vc_IMS_Global.ElementEncryption := v_ElementEncryption; v_DecryptedContent := fl_XML_EncryptedData_Decrypt(v_XmlDocument, p_MCServiceID, p_CSK); v_XmlDocument := v_DecryptedContent; if (v_DecryptedContent == "") { v_DecryptResult := NOT_ENCRYPTED; } } fx_XML_Log(__FILE__, __LINE__, UPLINK, v_XmlDocument); } return fl_MCX_MCPTTInfo_Decode(v_XmlDocument); } }

## Change 27

| Function name | cr_XMLENC_EncryptedData fl_XML_EncryptedData_Decrypt f_XML_Document_EncryptElementContent (new) tsc_XML_EncryptionElementType |
| --- | --- |
| Reason for change | As per Change 26 |
| Summary of change | Added handling for encryption on “entire XML document”. |
| TTCN module | MCX_XML_Encryption |
| MCC160 Comment |  |

Before Change:

template (present) EncryptedDataType cr_XMLENC_EncryptedData(charstring p_KeyName, charstring) := { /* 36.579-1 Table 5.5.13.2-1 */ /* @status APPROVED (MCX) */ encoding := omit, id := omit, mimeType := omit, type_ := tsc_XML_EncryptionContentType ifpresent, encryptionMethod := cr_XMLENC_EncryptionMethod(tsc_XML_EncryptionAlgorithm_AES128_GCM) ifpresent, keyInfo := cr_XMLDSIG_KeyInfo_KeyName(p_KeyName) ifpresent, cipherData := { choice := { cipherValue := ? } }, encryptionProperties := omit }; ......... ......... function fl_XML_EncryptedData_Decrypt(universal charstring p_Content, charstring p_MCServiceID, MCX_SecurityKey_Type p_CSK) runs on IMS_PTC return universal charstring { /* p_Content can be a complete XML document or any fragment of it */ var bitstring v_CipherKey; var EncryptedData v_EncryptedData; var charstring v_KeyId; var octetstring v_OctStr; var octetstring v_IV; var universal charstring v_DecryptedContent := ""; var bitstring v_Bitstring := f_Unichar2Bit(p_Content); if (decvalue(v_Bitstring, v_EncryptedData) == 0) { // first parameter is inout v_KeyId := f_Base64Encode(p_CSK.Id); // base64encoded as according to 33.180 clause 9.3.4.2 if (match(v_EncryptedData, cr_XMLENC_EncryptedData(v_KeyId))) { v_CipherKey := f_MCX_XML_GenerateProtectionKey(p_CSK, tsc_MCX_FC_Confidentiality, p_MCServiceID); v_OctStr := v_EncryptedData.cipherData.choice.cipherValue; /* ciphered data is base64 encoded acc. to https://www.w3.org/TR/xmlenc-core1/ clause 4.3 and 4.4; nevertheless as cipherValue is of type XSD.Base64Binary, the codec does the base64 decoding (ES 201 873-9 clause 6.2.11) */ v_IV := substr(v_OctStr, 0, 12); // 96 bit Initialization Vector (IV) at the beginning of the cipher text (https://www.w3.org/TR/xmlenc-core1/ clause 5.2.4) v_OctStr := substr(v_OctStr, 12, lengthof(v_OctStr) - 12); // v_OctStr still consists of the encrypted data and the 128 bit Authentication Tag (T) v_OctStr := fx_XML_Decrypt(v_OctStr, AES_128_GCM, v_CipherKey, v_IV); v_DecryptedContent := oct2unichar(v_OctStr); } else { f_IMS_SetVerdictFailOrInconc(__FILE__, __LINE__, "Invalid EncryptedData"); } } return v_DecryptedContent; } ......... ......... function f_XML_Document_EncryptElementContent(inout universal charstring p_XmlDocument, charstring p_XmlPath, charstring p_MCServiceID, template (omit) MCX_SecurityKey_Type p_CSK) { // NOTE: The function can be called independent from whether or not the element addressed by p_XmlPath exists in the given document var universal charstring v_ElementContent; var template (value) EncryptedData v_EncryptedData; var universal charstring v_NewContent; if (isvalue(p_CSK) and tsc_MCX_XML_ConfidentialityProtection) { v_ElementContent := f_XML_Document_GetElementContent(p_XmlDocument, p_XmlPath); if (v_ElementContent != "") { // the element exists and has content v_EncryptedData := fl_XML_EncryptedData_Encrypt(v_ElementContent, p_MCServiceID, valueof(p_CSK)); v_NewContent := f_Bit2Unichar(encvalue(v_EncryptedData)); p_XmlDocument := f_XML_Document_ReplaceElementContent(p_XmlDocument, p_XmlPath, v_NewContent); } } }

After Change:

const charstring tsc_XML_EncryptionElementType := "http://www.w3.org/2001/04/xmlenc#Element"; ......... ......... template (present) EncryptedDataType cr_XMLENC_EncryptedData(charstring p_KeyName, charstring p_EncryptionType) := { /* 36.579-1 Table 5.5.13.2-1 */ /* @status APPROVED (MCX) */ encoding := omit, id := omit, mimeType := omit, type_ := p_EncryptionType ifpresent, encryptionMethod := cr_XMLENC_EncryptionMethod(tsc_XML_EncryptionAlgorithm_AES128_GCM) ifpresent, keyInfo := cr_XMLDSIG_KeyInfo_KeyName(p_KeyName) ifpresent, cipherData := { choice := { cipherValue := ? } }, encryptionProperties := omit }; ......... ......... function fl_XML_EncryptedData_Decrypt(universal charstring p_Content, charstring p_MCServiceID, MCX_SecurityKey_Type p_CSK) runs on IMS_PTC return universal charstring { /* p_Content can be a complete XML document or any fragment of it */ var bitstring v_CipherKey; var EncryptedData v_EncryptedData; var charstring v_KeyId; var octetstring v_OctStr; var octetstring v_IV; var universal charstring v_DecryptedContent := ""; var bitstring v_Bitstring := f_Unichar2Bit(p_Content); if (decvalue(v_Bitstring, v_EncryptedData) == 0) { // first parameter is inout v_KeyId := f_Base64Encode(p_CSK.Id); // base64encoded as according to 33.180 clause 9.3.4.2 if (match(v_EncryptedData, cr_XMLENC_EncryptedData(v_KeyId, tsc_XML_EncryptionContentType)) or match(v_EncryptedData, cr_XMLENC_EncryptedData(v_KeyId, tsc_XML_EncryptionElementType))) { v_CipherKey := f_MCX_XML_GenerateProtectionKey(p_CSK, tsc_MCX_FC_Confidentiality, p_MCServiceID); v_OctStr := v_EncryptedData.cipherData.choice.cipherValue; /* ciphered data is base64 encoded acc. to https://www.w3.org/TR/xmlenc-core1/ clause 4.3 and 4.4; nevertheless as cipherValue is of type XSD.Base64Binary, the codec does the base64 decoding (ES 201 873-9 clause 6.2.11) */ v_IV := substr(v_OctStr, 0, 12); // 96 bit Initialization Vector (IV) at the beginning of the cipher text (https://www.w3.org/TR/xmlenc-core1/ clause 5.2.4) v_OctStr := substr(v_OctStr, 12, lengthof(v_OctStr) - 12); // v_OctStr still consists of the encrypted data and the 128 bit Authentication Tag (T) v_OctStr := fx_XML_Decrypt(v_OctStr, AES_128_GCM, v_CipherKey, v_IV); v_DecryptedContent := oct2unichar(v_OctStr); } else { f_IMS_SetVerdictFailOrInconc(__FILE__, __LINE__, "Invalid EncryptedData"); } } return v_DecryptedContent; } ......... ......... function f_XML_Document_EncryptElementContent(inout universal charstring p_XmlDocument, charstring p_XmlPath, charstring p_MCServiceID, template (omit) MCX_SecurityKey_Type p_CSK) runs on IMS_PTC //New Added { // NOTE: The function can be called independent from whether or not the element addressed by p_XmlPath exists in the given document var universal charstring v_ElementContent; var template (value) EncryptedData v_EncryptedData; var universal charstring v_NewContent; var boolean v_ElementEncryption := f_IMS_PTC_IsElementEncryption(); //New Added if (isvalue(p_CSK) and tsc_MCX_XML_ConfidentialityProtection) { v_ElementContent := f_XML_Document_GetElementContent(p_XmlDocument, p_XmlPath); if (v_ElementContent != "" and v_ElementEncryption) { // the element exists and has content v_EncryptedData := fl_XML_EncryptedData_Encrypt(v_ElementContent, p_MCServiceID, valueof(p_CSK)); v_NewContent := f_Bit2Unichar(encvalue(v_EncryptedData)); p_XmlDocument := f_XML_Document_ReplaceElementContent(p_XmlDocument, p_XmlPath, v_NewContent); } else { //Full Body Encryption v_EncryptedData := fl_XML_EncryptedData_Encrypt(p_XmlDocument, p_MCServiceID, valueof(p_CSK)); p_XmlDocument := f_Bit2Unichar(encvalue(v_EncryptedData)); } } }

## Change 28

| Function name | f_MCX_ResourceLists_EncodeEncrypt fl_MCX_ResourceLists_DecryptElementContent |
| --- | --- |
| Reason for change | As per Change 26 |
| Summary of change | 1, Added handling for encryption on “entire XML document” in fl_MCX_ResourceLists_DecryptElementContent 2, Added f_MCX_ResourceLists_EncodeEncrypt on IMS_PTC for access modular gloabal parameter |
| TTCN module | MCX_XML_resource_lists |
| MCC160 Comment |  |

Before Change:

function f_MCX_ResourceLists_EncodeEncrypt(template (value) Resource_lists p_ResourceLists, charstring p_MCServiceID , template (omit) MCX_SecurityKey_Type p_CSK := omit) return XmlBody_Type { ......... } ......... function fl_MCX_ResourceLists_DecryptElementContent(inout universal charstring p_XmlDocument, charstring p_MCServiceID, MCX_SecurityKey_Type p_CSK) runs on IMS_PTC return boolean { /* function returns true when either resource-lists or all list elements are (content) encrypted; it returns false when neither resource-lists or any list elements are (content) encrypted; it raises fail/inconc if some but not all list elements are (content) encrypted */ var XML_Document_DecryptElementContent_Result_Type v_DecryptResult; var integer v_NoOfListElements := 0; var integer v_NoOfEncryptedListElements := 0; var charstring v_XmlPath; var integer k; v_XmlPath := "//resource-lists"; v_DecryptResult := f_XML_Document_DecryptElementContent(p_XmlDocument, v_XmlPath, p_MCServiceID, p_CSK); if (v_DecryptResult == NOT_ENCRYPTED) { for (k := 1; v_DecryptResult != NOT_FOUND; k := k + 1) { v_XmlPath := "//resource-lists/list[" & int2str(k) & "]"; v_DecryptResult := f_XML_Document_DecryptElementContent(p_XmlDocument, v_XmlPath, p_MCServiceID, p_CSK); if (v_DecryptResult != NOT_FOUND) { v_NoOfListElements := v_NoOfListElements + 1; if (v_DecryptResult == OK) { v_NoOfEncryptedListElements := v_NoOfEncryptedListElements + 1; } } } if (v_NoOfEncryptedListElements != v_NoOfListElements) { if (v_NoOfEncryptedListElements > 0) { f_IMS_SetVerdictFailOrInconc(__FILE__, __LINE__, "Not all list elements are encrypted"); } else { return false; } } } return true; }

After Change:

<<SKIPPED>> function f_MCX_ResourceLists_EncodeEncrypt(template (value) Resource_lists p_ResourceLists, charstring p_MCServiceID , template (omit) MCX_SecurityKey_Type p_CSK := omit) runs on IMS_PTC return XmlBody_Type { ......... } ......... function fl_MCX_ResourceLists_DecryptElementContent(inout universal charstring p_XmlDocument, charstring p_MCServiceID, MCX_SecurityKey_Type p_CSK) runs on IMS_PTC return boolean { /* function returns true when either resource-lists or all list elements are (content) encrypted; it returns false when neither resource-lists or any list elements are (content) encrypted; it raises fail/inconc if some but not all list elements are (content) encrypted */ var XML_Document_DecryptElementContent_Result_Type v_DecryptResult; var integer v_NoOfListElements := 0; var integer v_NoOfEncryptedListElements := 0; var charstring v_XmlPath; var integer k; var universal charstring v_DecryptedContent; v_XmlPath := "//resource-lists"; v_DecryptResult := f_XML_Document_DecryptElementContent(p_XmlDocument, v_XmlPath, p_MCServiceID, p_CSK); if (v_DecryptResult == NOT_FOUND) { v_DecryptedContent := fl_XML_EncryptedData_Decrypt(p_XmlDocument, p_MCServiceID, p_CSK); p_XmlDocument := v_DecryptedContent; if (v_DecryptedContent == "") { v_DecryptResult := NOT_ENCRYPTED; } } if (v_DecryptResult == NOT_ENCRYPTED) { for (k := 1; v_DecryptResult != NOT_FOUND; k := k + 1) { v_XmlPath := "//resource-lists/list[" & int2str(k) & "]"; v_DecryptResult := f_XML_Document_DecryptElementContent(p_XmlDocument, v_XmlPath, p_MCServiceID, p_CSK); if (v_DecryptResult != NOT_FOUND) { v_NoOfListElements := v_NoOfListElements + 1; if (v_DecryptResult == OK) { v_NoOfEncryptedListElements := v_NoOfEncryptedListElements + 1; } } } if (v_NoOfEncryptedListElements != v_NoOfListElements) { if (v_NoOfEncryptedListElements > 0) { f_IMS_SetVerdictFailOrInconc(__FILE__, __LINE__, "Not all list elements are encrypted"); } else { return false; } } } return true; }

## Change 29

| Function name | f_MCX_MCDataInfo_EncodeEncrypt |
| --- | --- |
| Reason for change | As per Change 26 |
| Summary of change | Added f_MCX_MCDataInfo_EncodeEncrypt on IMS_PTC for access modular gloabal parameter |
| TTCN module | MCX_XML_mcdata_info |
| MCC160 Comment |  |

Before Change:

function f_MCX_MCDataInfo_EncodeEncrypt(template (value) Mcdatainfo_Type p_MCData_Info, charstring p_MCServiceID, template (omit) MCX_SecurityKey_Type p_CSK) return XmlBody_Type { ......... }

After Change:

<<SKIPPED>> function f_MCX_MCDataInfo_EncodeEncrypt(template (value) Mcdatainfo_Type p_MCData_Info, charstring p_MCServiceID, template (omit) MCX_SecurityKey_Type p_CSK) runs on IMS_PTC return XmlBody_Type { ......... }

## Change 30

| Function name | f_MCData_LocationInfoReport_EncodeEncrypt |
| --- | --- |
| Reason for change | As per Change 26 |
| Summary of change | Added f_MCData_LocationInfoReport_EncodeEncrypt on IMS_PTC for access modular gloabal parameter |
| TTCN module | MCX_XML_mcdataLocationInfo |
| MCC160 Comment |  |

Before Change:

function f_MCData_LocationInfoReport_EncodeEncrypt(template (value) Location_info p_LocationInfo, charstring p_MCServiceID, template (omit) MCX_SecurityKey_Type p_CSK) return XmlBody_Type { ......... }

After Change:

function f_MCData_LocationInfoReport_EncodeEncrypt(template (value) Location_info p_LocationInfo, charstring p_MCServiceID, template (omit) MCX_SecurityKey_Type p_CSK) runs on IMS_PTC return XmlBody_Type { ......... }

## Change 31

| Function name | f_MCPTT_LocationInfoReport_EncodeEncrypt |
| --- | --- |
| Reason for change | As per Change 26 |
| Summary of change | Added f_ MCPTT_LocationInfoReport_EncodeEncrypt on IMS_PTC for access modular gloabal parameter |
| TTCN module | MCX_XML_mcpttLocationInfo |
| MCC160 Comment |  |

Before Change:

function f_MCPTT_LocationInfoReport_EncodeEncrypt(template (value) Location_info p_LocationInfo, charstring p_MCServiceID, template (omit) MCX_SecurityKey_Type p_CSK) return XmlBody_Type { ......... }

After Change:

function f_MCPTT_LocationInfoReport_EncodeEncrypt(template (value) Location_info p_LocationInfo, charstring p_MCServiceID, template (omit) MCX_SecurityKey_Type p_CSK) runs on IMS_PTC return XmlBody_Type { ......... }

## Change 32

| Function name | fl_MCX_Mcpttregroup_EncryptElementList |
| --- | --- |
| Reason for change | As per Change 26 |
| Summary of change | Added fl_MCX_Mcpttregroup_EncryptElementList on IMS_PTC for access modular gloabal parameter |
| TTCN module | MCX_XML_mcpttregroup |
| MCC160 Comment |  |

Before Change:

function fl_MCX_Mcpttregroup_EncryptElementList(inout universal charstring p_XmlDocument, charstring p_XmlBasePath, integer p_ElementCount, charstring p_MCServiceID, MCX_SecurityKey_Type p_CSK) { ......... }

After Change:

function fl_MCX_Mcpttregroup_EncryptElementList(inout universal charstring p_XmlDocument, charstring p_XmlBasePath, integer p_ElementCount, charstring p_MCServiceID, MCX_SecurityKey_Type p_CSK) runs on IMS_PTC { ......... }

## Change 33

| Function name | f_MCX_MCVideoInfo_EncodeEncrypt |
| --- | --- |
| Reason for change | As per Change 26 |
| Summary of change | Added f_MCX_MCVideoInfo_EncodeEncrypt on IMS_PTC for access modular gloabal parameter |
| TTCN module | MCX_XML_mcvideo_info |
| MCC160 Comment |  |

Before Change:

function f_MCX_MCVideoInfo_EncodeEncrypt(template (value) Mcvideoinfo_Type p_MCVideo_Info, charstring p_MCServiceID, template (omit) MCX_SecurityKey_Type p_CSK) return XmlBody_Type { ......... }

After Change:

function f_MCX_MCVideoInfo_EncodeEncrypt(template (value) Mcvideoinfo_Type p_MCVideo_Info, charstring p_MCServiceID, template (omit) MCX_SecurityKey_Type p_CSK) runs on IMS_PTC return XmlBody_Type { ......... }

## Change 34

| Function name | f_MCVideo_LocationInfoReport_EncodeEncrypt |
| --- | --- |
| Reason for change | As per Change 26 |
| Summary of change | Added f_MCVideo_LocationInfoReport_EncodeEncrypt on IMS_PTC for access modular gloabal parameter |
| TTCN module | MCX_XML_mcvideoLocationInfo |
| MCC160 Comment |  |

Before Change:

function f_MCVideo_LocationInfoReport_EncodeEncrypt(template (value) Location_info p_LocationInfo, charstring p_MCServiceID, template (omit) MCX_SecurityKey_Type p_CSK) return XmlBody_Type { ......... }

After Change:

function f_MCVideo_LocationInfoReport_EncodeEncrypt(template (value) Location_info p_LocationInfo, charstring p_MCServiceID, template (omit) MCX_SecurityKey_Type p_CSK) runs on IMS_PTC return XmlBody_Type { ......... }

## Change 35

| Function name | f_MCX_XcapDiff_EncodeEncrypt |
| --- | --- |
| Reason for change | As per Change 26 |
| Summary of change | Added f_MCX_XcapDiff_EncodeEncrypt on IMS_PTC for access modular gloabal parameter |
| TTCN module | MCX_XML_xcap_diff |
| MCC160 Comment |  |

Before Change:

function f_MCX_XcapDiff_EncodeEncrypt(template (value) Xcap_diff p_Xcap_diff, charstring p_MCServiceID, template (omit) MCX_SecurityKey_Type p_CSK) return XmlBody_Type { ......... }

After Change:

function f_MCX_XcapDiff_EncodeEncrypt(template (value) Xcap_diff p_Xcap_diff, charstring p_MCServiceID, template (omit) MCX_SecurityKey_Type p_CSK) runs on IMS_PTC return XmlBody_Type { ......... }

# Branches executed

The test case was executed with

- TLSv1.2

- IPSec ipsec-3gpp;alg=hmac-sha-1-96;ealg=aes-cbc

- XML integrity and encrpytion according to  http://www.w3.org/2009/xmlenc11#aes128-gcm

- XML signatures according to  http://www.w3.org/2001/04/xmlenc#sha256

- MIKEY SAKKE/ECCSI (RFC 6507, 6508, 6509)

- LTE FDD Band 5

- Ciphering algorithm eea1 and Integrity algorithm eia1

# Execution Log Files

## Alea - a Leonardo Company, MCXPTT Platform v. 4.1.0 - Android Client v 1.11.5

The Alea - a Leonardo Company, MCXPTT Platform v. 4.1.0 - Android Client v 1.11.5 passed this test case on Keysight S8704A Protocol Conformance Toolset. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

TC_5_1_MCPTT_LOG.html

- PICS/PIXIT parameter file:

TC_5_1_MCPTT_PIXIT.xml

# References

| [1] | R5s240512 : Supporting information for Addition of MCPTT test case 5.1 to the MCX ATS with MCX-EUTRA model |
| --- | --- |