3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240522

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3498 | rev | - | Current version: | 17.11.0 |  |
| For HELPon using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed changeaffects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcase 7.1.1.4.2.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Starpoint |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According to TS 38.214, 6.1.2.2.1: In uplink resource allocation of type 0, the resource block assignment information includes a bitmap indicating the Resource Block Groups (RBGs) that are allocated to the scheduled UE where a RBG is a set of consecutive virtual resource blocks defined by higher layer parameter rbg-Size for DCI format 0_1/0_2 or rbg-SizeDCI-0-3for DCI format 0_3 configured in pusch-Configand the size of the bandwidth part as defined in Table 6.1.2.2.1-1. The unit for PUSCH resource assignment is RBG. In the test loop of TC 7.1.1.4.2.3,if one RBG takes 8 RBs(depends on the current configuration), BWP takes 106RBs ,set the number of RB to 2 and the FirstRbIndex to 0 in ULgrant may lead to error . |  |  |  |  |  |  |  |  |
| Summary of change: |  | Calculate the FirstRbIndex instead of using Index0. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail the test cases. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.4.2.3.NR5GC, 7.1.1.4.2.3.ENDC, 7.1.1.4.2.4.ENDC, 7.1.1.4.2.4.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	10

2.3	Change 3	11

2.4	Change 4	15

- Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D24wk24 related to the title of this CR.

Contact:	Ting Gao
	gaoting@starpointcomm.com

- Corrections required

  - Change 1

| Functionname | f_NR_Resourceallocation0_AllowedNPRB |
| --- | --- |
| Reason for change | According to TS 38.214, 6.1.2.2.1: In uplink resource allocation of type 0, the resource block assignment information includes a bitmap indicating the Resource Block Groups (RBGs) that are allocated to the scheduled UE where a RBG is a set of consecutive virtual resource blocks defined by higher layer parameter rbg-Size for DCI format 0_1/0_2 or rbg-SizeDCI-0-3for DCI format 0_3 configured in pusch-Configand the size of the bandwidth part as defined in Table 6.1.2.2.1-1. The unit for PUSCH resource assignment is RBG. In the test loop of TC 7.1.1.4.2.3,if one RBG takes 8 RBs(depends on the current configuration), BWP takes 106RBs ,set the number of RB to 2 and the FirstRbIndex to 0 in ULgrant may lead to error . |
| Summary of change | Calculate the FirstRbIndex instead of using Index0. |
| TTCN module | MAC_TC_Common_NR.ttcn |
| MCC160 Comment |  |

Before Change:

function f_NR_Resourceallocation0_AllowedNPRB(integer p_MaxNPRB_BWP, integer p_N_PRB ) return boolean { // as per table 7.1.1.4.1.3.3.2-2A/7.1.1.4.1.4.3.2-2A/7.1.1.4.2.3.3.2-2A/7.1.1.4.2.4.3.2-2A var boolean v_Result := false; select (p_MaxNPRB_BWP) { case (11, 25, 31) { v_Result := true; } case (18) { select (p_N_PRB) { case (2,4,6,8,10,12,16,18) { v_Result := true; } } } case (24) { select (p_N_PRB) { case (2,4,6,8,10,12,16,18,20,22,24) { v_Result := true; } } } case (32) { select (p_N_PRB) { case (2,4,6,8,10,12,16,18,20,22,24,26,28,30,32) { v_Result := true; } } } case (38) { select (p_N_PRB) { case (2,4,6,8,10,12,16,18,20,22,24,26,28,30,32,34,36,38) { v_Result := true; } } } case (51) { select (p_N_PRB) { case (3,4,7,8,11,12,15,16,19,20,23,24,27,28,31,32,35,36,39,40,43,44,47,48,51) { v_Result := true; } } } case (52) { select (p_N_PRB) { case (4,8,12,16,20,24,28,32,36,40,44,48,52) { v_Result := true; } } } case (65) { select (p_N_PRB) { case (1,4,5,8,9,12,13,16,17,20,21,24,25,28,29,32,33,36,37,40,41,44,45,48,49, 52,53,56,57,60,61,64,65) { v_Result := true; } } } case (66) { select (p_N_PRB) { case (2,4,6,8,10,12,16,18,20,22,24,26,28,30,32,34,36,38,40,42,44,46,48,50,52, 54,56,58,60,62,64,66) { v_Result := true; } } } case (79) { select (p_N_PRB) { case (7,8,15,16,23,24,31,32,39,40,47,48,55,56,63,64,71,72,79) { v_Result := true; } } } case (106) { select (p_N_PRB) { case (2,8,10,16,18,24,26,32,34,40,42,48,50,56,58,64,66,72,74,80,82,88,90,96, 92,104,106) { v_Result := true; } } } case (107) { select (p_N_PRB) { case (3,8,11,16,19,24,27,32,35,40,43,48,51,56,59,64,67,72,75,80,83,88,91,96, 99,104,107) { v_Result := true; } } } case (132) { select (p_N_PRB) { case (4,8,12,16,20,24,28,32,36,40,44,48,52,56,60,64,68,72,76,80,84,88,92,96, 100,104, 108,112,116,120,124,128,132) { v_Result := true; } } } case (133) { select (p_N_PRB) { case (5,8,13,16,21,24,29,32,37,40,45,48,53,56,61,64,69,72,77,80,85,88,93,96, 101,104, 109,112,117,120,125,128,133) { v_Result := true; } } } case (135) { select (p_N_PRB) { case (7,8,15,16,23,24,31,32,39,40,47,48,55,56,63,64,71,72,79,80,87,88,95,96, 103,104, 111,112,119,120,127,128,135) { v_Result := true; } } } case (160) { //@sic R5s200544 sic@ select (p_N_PRB) { case (16,32,48,64,80,96,112,128,144,160) { v_Result := true; } } } case (216) { select (p_N_PRB) { case (8,16,24,32,40,48,56,64,72,80,88,96,104,112,120,128,136,144,152,160, 168, 176,184,192,200,208,216) { v_Result := true; } } } case (217) { select (p_N_PRB) { case (9,16,25,32,41,48,57,64,73,80,89,96,105,112,121,128,137,144,153,160, 169,176,185,192,201,208,217) { v_Result := true; } } } case (264) { select (p_N_PRB) { case (8,16,24,32,40,48,56,64,72,80,88,96,104,112,120,128,136,144,160,168, 176,184,192,200,208,216,224,232,240,248,256,264) { v_Result := true; } } } case (270) { select (p_N_PRB) { case (14,16,30,32,46,44,62,64,78,80,94,96,110,112, 126,128,142,144,158, 160,174, 176,190,192, 206,208,222,224,238,240, 254,256,270) { v_Result := true; } } } case (273) { select (p_N_PRB) { case (1,16,17,32,33,48,49,64,65,80,81,96,97,112,113,128,129,144,145,160, 161,176,171, 192,193, 208,209, 224,225,240,241,256,257,272,273) { v_Result := true; } } } case else {FatalError(__FILE__, __LINE__, "unknown MaxNPRB per BWP")} } // end of select return v_Result; }

After Change:

function f_NR_Resourceallocation0_AllowedNPRB(integer p_MaxNPRB_BWP, integer p_N_PRB, out integer p_StartIndex) return boolean { // as per table 7.1.1.4.1.3.3.2-2A/7.1.1.4.1.4.3.2-2A/7.1.1.4.2.3.3.2-2A/7.1.1.4.2.4.3.2-2A var boolean v_Result := false; p_StartIndex:=0; select (p_MaxNPRB_BWP) { case (11, 25, 31) { v_Result := true; } case (18) { select (p_N_PRB) { case (2,4,6,8,10,12,16,18) { v_Result := true; p_StartIndex := 18-p_N_PRB; } } } case (24) { select (p_N_PRB) { case (2,4,6,8,10,12,16,18,20,22,24) { v_Result := true; p_StartIndex := 24-p_N_PRB; } } } case (32) { select (p_N_PRB) { case (2,4,6,8,10,12,16,18,20,22,24,26,28,30,32) { v_Result := true; p_StartIndex := 32-p_N_PRB; } } } case (38) { select (p_N_PRB) { case (2,4,6,8,10,12,16,18,20,22,24,26,28,30,32,34,36,38) { v_Result := true; p_StartIndex := 38-p_N_PRB; } } } case (51) { select (p_N_PRB) { case (3,4,7,8,11,12,15,16,19,20,23,24,27,28,31,32,35,36,39,40,43,44,47,48,51) { v_Result := true; p_StartIndex := 51-p_N_PRB; } } } case (52) { select (p_N_PRB) { case (4,8,12,16,20,24,28,32,36,40,44,48,52) { v_Result := true; p_StartIndex := 52-p_N_PRB; } } } case (65) { select (p_N_PRB) { case (1,4,5,8,9,12,13,16,17,20,21,24,25,28,29,32,33,36,37,40,41,44,45,48,49, 52,53,56,57,60,61,64,65) { v_Result := true; p_StartIndex := 65-p_N_PRB; } } } case (66) { select (p_N_PRB) { case (2,4,6,8,10,12,16,18,20,22,24,26,28,30,32,34,36,38,40,42,44,46,48,50,52, 54,56,58,60,62,64,66) { v_Result := true; p_StartIndex := 66-p_N_PRB; } } } case (79) { select (p_N_PRB) { case (7,8,15,16,23,24,31,32,39,40,47,48,55,56,63,64,71,72,79) { v_Result := true; p_StartIndex := 79-p_N_PRB; } } } case (106) { select (p_N_PRB) { case (2,8,10,16,18,24,26,32,34,40,42,48,50,56,58,64,66,72,74,80,82,88,90,96, 92,104,106) { v_Result := true; p_StartIndex := 106-p_N_PRB; } } } case (107) { select (p_N_PRB) { case (3,8,11,16,19,24,27,32,35,40,43,48,51,56,59,64,67,72,75,80,83,88,91,96, 99,104,107) { v_Result := true; p_StartIndex := 107-p_N_PRB; } } } case (132) { select (p_N_PRB) { case (4,8,12,16,20,24,28,32,36,40,44,48,52,56,60,64,68,72,76,80,84,88,92,96, 100,104, 108,112,116,120,124,128,132) { v_Result := true; p_StartIndex := 132-p_N_PRB; } } } case (133) { select (p_N_PRB) { case (5,8,13,16,21,24,29,32,37,40,45,48,53,56,61,64,69,72,77,80,85,88,93,96, 101,104, 109,112,117,120,125,128,133) { v_Result := true; p_StartIndex := 133-p_N_PRB; } } } case (135) { select (p_N_PRB) { case (7,8,15,16,23,24,31,32,39,40,47,48,55,56,63,64,71,72,79,80,87,88,95,96, 103,104, 111,112,119,120,127,128,135) { v_Result := true; p_StartIndex := 135-p_N_PRB; } } } case (160) { //@sic R5s200544 sic@ select (p_N_PRB) { case (16,32,48,64,80,96,112,128,144,160) { v_Result := true; } } } case (216) { select (p_N_PRB) { case (8,16,24,32,40,48,56,64,72,80,88,96,104,112,120,128,136,144,152,160, 168, 176,184,192,200,208,216) { v_Result := true; p_StartIndex := 216-p_N_PRB; } } } case (217) { select (p_N_PRB) { case (9,16,25,32,41,48,57,64,73,80,89,96,105,112,121,128,137,144,153,160, 169,176,185,192,201,208,217) { v_Result := true; p_StartIndex := 217-p_N_PRB; } } } case (264) { select (p_N_PRB) { case (8,16,24,32,40,48,56,64,72,80,88,96,104,112,120,128,136,144,160,168, 176,184,192,200,208,216,224,232,240,248,256,264) { v_Result := true; p_StartIndex := 264-p_N_PRB; } } } case (270) { select (p_N_PRB) { case (14,16,30,32,46,44,62,64,78,80,94,96,110,112, 126,128,142,144,158, 160,174, 176,190,192, 206,208,222,224,238,240, 254,256,270) { v_Result := true; p_StartIndex := 270-p_N_PRB; } } } case (273) { select (p_N_PRB) { case (1,16,17,32,33,48,49,64,65,80,81,96,97,112,113,128,129,144,145,160, 161,176,171, 192,193, 208,209, 224,225,240,241,256,257,272,273) { v_Result := true; p_StartIndex := 273-p_N_PRB; } } } case else {FatalError(__FILE__, __LINE__, "unknown MaxNPRB per BWP")} } // end of select return v_Result; }

  - Change 2

| Functionname | f_NR_ULGrantConfiguration_ULTBS |
| --- | --- |
| Reason for change | For change 1. |
| Summary of change | Pass p_StartIndex in template cs_NR_FreqDomainSchedulExplicit for UL grant instand of using 0. |
| TTCN module | MAC_TC_Common_NR.ttcn |
| MCC160 Comment |  |

Before Change:

function f_NR_ULGrantConfiguration_ULTBS(NR_CellId_Type p_NR_CellId, template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now, NR_ImcsValue_Type p_ImcsValue, integer p_N_PRB, bitstring p_Index, template (value) NR_ResourceAllocationType_Type p_NR_ResourceAllocationType := resourceAllocationType0, //@sic R5-221589 sic@ template (omit) NR_DciFormat_0_X_SpecificInfo_Type p_NR_DciFormat_0_X_SpecificInfo := omit) runs on NR_BASE_PTC { var template (value) NR_DciUlInfo_Type v_DciUlInfo := cds_NR_DciUlInfo_ModifyGrant(cs_NR_FreqDomainSchedulExplicit(-, p_N_PRB), cs_NR_DciCommon_TimeDomainResourceAssignment(p_Index), p_ImcsValue, -,//@sic R5-221589 sic@ -, p_NR_DciFormat_0_X_SpecificInfo); v_DciUlInfo.ResoureAssignment.ResourceAllocationType := p_NR_ResourceAllocationType; f_NR_ULGrantConfiguration_Common(p_NR_CellId, p_TimingInfo, v_DciUlInfo, cs_NR_UplinkTimeAlignment_Start, cs_UL_GrantConfig_OnSR); //@sic R5s210884 sic@ }

After Change:

function f_NR_ULGrantConfiguration_ULTBS(NR_CellId_Type p_NR_CellId, template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now, NR_ImcsValue_Type p_ImcsValue, integer p_N_PRB, bitstring p_Index, template (value) NR_ResourceAllocationType_Type p_NR_ResourceAllocationType := resourceAllocationType0, //@sic R5-221589 sic@ template (omit) NR_DciFormat_0_X_SpecificInfo_Type p_NR_DciFormat_0_X_SpecificInfo := omit, integer p_StartIndex:=0) runs on NR_BASE_PTC { var template (value) NR_DciUlInfo_Type v_DciUlInfo := cds_NR_DciUlInfo_ModifyGrant(cs_NR_FreqDomainSchedulExplicit(p_StartIndex, p_N_PRB),//grj add cs_NR_DciCommon_TimeDomainResourceAssignment(p_Index), p_ImcsValue, -,//@sic R5-221589 sic@ -, p_NR_DciFormat_0_X_SpecificInfo); v_DciUlInfo.ResoureAssignment.ResourceAllocationType := p_NR_ResourceAllocationType; f_NR_ULGrantConfiguration_Common(p_NR_CellId, p_TimingInfo, v_DciUlInfo, cs_NR_UplinkTimeAlignment_Start, cs_UL_GrantConfig_OnSR); //@sic R5s210884 sic@ }

  - Change 3

| Functionname | f_TC_7_1_1_4_1_x_2TB_NR_TestBody |
| --- | --- |
| Reason for change | For change1. |
| Summary of change | Add v_StartIndex in function f_TC_7_1_1_4_1_x_2TB_NR_TestBody for compile error. |
| TTCN module | MAC_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_7_1_1_4_1_x_2TB_NR_TestBody(DRB_Identity p_NR_DRB_Id, template (value) NR_TBS_CalculationParameters_Type p_CalculationParameters, //@sic R5-202610 sic@ NR_ResourceAllocationType_Type p_NR_ResourceAllocationType ) runs on NR_BASE_PTC { /* L2/MAC/7.1 * 7.1.1.4.x: DL-SCH Transport Block Size selection */ var integer v_TBSize:=0; // transport block size in bits var integer v_TBSize2:=0; // transport block size in bits var integer v_I_MCS:=0; // Modulation and Coding Scehme Index var integer v_N_PRB:=1; // Number of physical resource Blocks var integer v_Max_nPRB := f_NR_CellInfo_GetCarrierBandwidth_DL(nr_Cell1); // to store the max allowed PRB resources that can be allocated var integer v_TDRA ; // Time Domain resource allocation var NR_ResourceAllocationType_Type v_NR_ResourceAllocationType := p_NR_ResourceAllocationType; //var integer i; var NR_PDCP_SDUList_Type v_NR_PDCP_SDUList := {''O}; var NR_ResourceAllocation_Type v_NR_ResourceAllocation := {v_Max_nPRB, 28}; // Max NPRB and Max MCS var SubcarrierSpacing v_SCS := f_NR_CellInfo_GetSCS(nr_Cell1); var integer v_NoOfSymbols; var SubFrameNumber_Type v_SF := 2; if (v_SCS ==kHz30) {v_SF:= 1;} // Configure SS for UL grant for Max TB size f_NR_ULGrantConfiguration_Start( nr_Cell1, cs_TimingInfo_Now, v_NR_ResourceAllocation.Imcs, cs_NR_FreqDomainSchedulExplicit(-, v_NR_ResourceAllocation.Lrbs)); //Steps 1 to 5 are repeated for values of nPRB from 1 to v_Max_nPRB and iMCS from 0 to 28, time domain resource allocation v_TDRA 0..8 //f_NR_ConfigureULGrant_MaxTBSupported(); for (v_N_PRB := 1; v_N_PRB <= v_Max_nPRB; v_N_PRB := v_N_PRB + 1) { //for nPRB 1 to v_Max_nPRB if (p_NR_ResourceAllocationType == resourceAllocationType0) { if (not f_NR_Resourceallocation0_AllowedNPRB(v_Max_nPRB, v_N_PRB)) { continue; } } for (v_I_MCS := 0; v_I_MCS <= 28; v_I_MCS := v_I_MCS + 1) { //for iMCS 0 to 28 for (v_TDRA := 0; v_TDRA <= 1; v_TDRA := v_TDRA + 1) { //for Time Domain Resource Allocation 0 to 1 if (v_TDRA == 0) { v_NoOfSymbols := 12; } else { v_NoOfSymbols := 7; } //SS looks up iTBS in table 7.1.7.1-1 in TS 36.213 based on the value of iMCS //v_I_TBS := f_NR_Get_iTBS_For_iMCS(v_I_MCS); //The SS calculates or looks up TBS in TS 38.214 [15] based on the value of S, L, and nPRB. v_TBSize := f_NR_TBS_Calculation(p_CalculationParameters, v_NoOfSymbols, v_N_PRB, v_I_MCS,true ); v_TBSize2 := 2 * v_TBSize; if (v_TBSize2 < 192 ) { //this results in zero number of PDCP SDUs or TBSize high coding rate continue; } // skip this nprb, Imcs, TDRA combination. // Steps 2 to 5 are performed if TBsize is More than minimum //@siclog "Step 2" siclog@ // SS creates one or more PDCP SDUs, depending on TBsize, in accordance with Table 7.1.1.4.1.1.3.2-2 //Configure SS for DCI Format x with RA type y and a Resource block assignment (RBA) correspondent //to nPRB as specified in 7.1.6.1 in TS 36.213 and Modulation and coding scheme iMCS f_NR_DLGrantConfiguration(nr_Cell1, cs_TimingInfo_SubFrame(v_SF), v_NR_ResourceAllocationType, cs_NR_FreqDomainResourceAssignmentDL_Explicit(0, v_N_PRB), int2bit(v_TDRA, 4), cs_NR_TransportBlockRetransmissionList_Def(v_I_MCS), -, -, cs_NR_DciFormat_1_0_Params(v_SCS)); /* @sic R5s190442: v_SCS sic@ */ //@siclog "Step 3" siclog@ // SS sends MAC PDU (NxPDCP SDUs) v_NR_PDCP_SDUList := fl_NR_GetPdcpSduList_2TB(v_TBSize); DRB.send(cas_NR_DRB_COMMON_REQ_PDCP_SDUList(nr_Cell1, p_NR_DRB_Id, //crs_RlcBearerRouting_NR(nr_Cell1), -, v_NR_PDCP_SDUList )); //@siclog "Step 4" siclog@ // automatically taken by SS //@siclog "Step 5" siclog@ // SS receives NxPDCP SDUs f_NR_Rx_N_PDCP_SDUs(v_NR_PDCP_SDUList, p_NR_DRB_Id); }//End for TDRA 0 to 8 }//End for iMCS 0 to 28 }//End for nPRB 1 to 110 }//f_NR_7_1_1_4_1_x

After Change:

function f_TC_7_1_1_4_1_x_2TB_NR_TestBody(DRB_Identity p_NR_DRB_Id, template (value) NR_TBS_CalculationParameters_Type p_CalculationParameters, //@sic R5-202610 sic@ NR_ResourceAllocationType_Type p_NR_ResourceAllocationType ) runs on NR_BASE_PTC { /* L2/MAC/7.1 * 7.1.1.4.x: DL-SCH Transport Block Size selection */ var integer v_TBSize:=0; // transport block size in bits var integer v_TBSize2:=0; // transport block size in bits var integer v_I_MCS:=0; // Modulation and Coding Scehme Index var integer v_N_PRB:=1; // Number of physical resource Blocks var integer v_Max_nPRB := f_NR_CellInfo_GetCarrierBandwidth_DL(nr_Cell1); // to store the max allowed PRB resources that can be allocated var integer v_TDRA ; // Time Domain resource allocation var NR_ResourceAllocationType_Type v_NR_ResourceAllocationType := p_NR_ResourceAllocationType; //var integer i; var NR_PDCP_SDUList_Type v_NR_PDCP_SDUList := {''O}; var NR_ResourceAllocation_Type v_NR_ResourceAllocation := {v_Max_nPRB, 28}; // Max NPRB and Max MCS var SubcarrierSpacing v_SCS := f_NR_CellInfo_GetSCS(nr_Cell1); var integer v_NoOfSymbols; var SubFrameNumber_Type v_SF := 2; var integer v_StartIndex:=0 if (v_SCS ==kHz30) {v_SF:= 1;} // Configure SS for UL grant for Max TB size f_NR_ULGrantConfiguration_Start( nr_Cell1, cs_TimingInfo_Now, v_NR_ResourceAllocation.Imcs, cs_NR_FreqDomainSchedulExplicit(-, v_NR_ResourceAllocation.Lrbs)); //Steps 1 to 5 are repeated for values of nPRB from 1 to v_Max_nPRB and iMCS from 0 to 28, time domain resource allocation v_TDRA 0..8 //f_NR_ConfigureULGrant_MaxTBSupported(); for (v_N_PRB := 1; v_N_PRB <= v_Max_nPRB; v_N_PRB := v_N_PRB + 1) { //for nPRB 1 to v_Max_nPRB if (p_NR_ResourceAllocationType == resourceAllocationType0) { if (not f_NR_Resourceallocation0_AllowedNPRB(v_Max_nPRB, v_N_PRB,v_StartIndex)) { continue; } } for (v_I_MCS := 0; v_I_MCS <= 28; v_I_MCS := v_I_MCS + 1) { //for iMCS 0 to 28 for (v_TDRA := 0; v_TDRA <= 1; v_TDRA := v_TDRA + 1) { //for Time Domain Resource Allocation 0 to 1 if (v_TDRA == 0) { v_NoOfSymbols := 12; } else { v_NoOfSymbols := 7; } //SS looks up iTBS in table 7.1.7.1-1 in TS 36.213 based on the value of iMCS //v_I_TBS := f_NR_Get_iTBS_For_iMCS(v_I_MCS); //The SS calculates or looks up TBS in TS 38.214 [15] based on the value of S, L, and nPRB. v_TBSize := f_NR_TBS_Calculation(p_CalculationParameters, v_NoOfSymbols, v_N_PRB, v_I_MCS,true ); v_TBSize2 := 2 * v_TBSize; if (v_TBSize2 < 192 ) { //this results in zero number of PDCP SDUs or TBSize high coding rate continue; } // skip this nprb, Imcs, TDRA combination. // Steps 2 to 5 are performed if TBsize is More than minimum //@siclog "Step 2" siclog@ // SS creates one or more PDCP SDUs, depending on TBsize, in accordance with Table 7.1.1.4.1.1.3.2-2 //Configure SS for DCI Format x with RA type y and a Resource block assignment (RBA) correspondent //to nPRB as specified in 7.1.6.1 in TS 36.213 and Modulation and coding scheme iMCS f_NR_DLGrantConfiguration(nr_Cell1, cs_TimingInfo_SubFrame(v_SF), v_NR_ResourceAllocationType, cs_NR_FreqDomainResourceAssignmentDL_Explicit(0, v_N_PRB), int2bit(v_TDRA, 4), cs_NR_TransportBlockRetransmissionList_Def(v_I_MCS), -, -, cs_NR_DciFormat_1_0_Params(v_SCS)); /* @sic R5s190442: v_SCS sic@ */ //@siclog "Step 3" siclog@ // SS sends MAC PDU (NxPDCP SDUs) v_NR_PDCP_SDUList := fl_NR_GetPdcpSduList_2TB(v_TBSize); DRB.send(cas_NR_DRB_COMMON_REQ_PDCP_SDUList(nr_Cell1, p_NR_DRB_Id, //crs_RlcBearerRouting_NR(nr_Cell1), -, v_NR_PDCP_SDUList )); //@siclog "Step 4" siclog@ // automatically taken by SS //@siclog "Step 5" siclog@ // SS receives NxPDCP SDUs f_NR_Rx_N_PDCP_SDUs(v_NR_PDCP_SDUList, p_NR_DRB_Id); }//End for TDRA 0 to 8 }//End for iMCS 0 to 28 }//End for nPRB 1 to 110 }//f_NR_7_1_1_4_1_x

  - Change 4

| Functionname | f_TC_7_1_1_4_2_x_DCI0_1_NR_TestBody |
| --- | --- |
| Reason for change | For change1. |
| Summary of change | Add v_StartIndex in function f_TC_7_1_1_4_2_x_DCI0_1_NR_TestBody for compile error. |
| TTCN module | MAC_TC_Common_NR.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_7_1_1_4_2_x_DCI0_1_NR_TestBody(DRB_Identity p_NR_DRB_Id, template (value) NR_TBS_CalculationParameters_Type p_CalculationParameters, NR_ResourceAllocationType_Type p_NR_ResourceAllocationType,//@sic R5s211564 sic@ integer p_Max_Imcs := 28) runs on NR_BASE_PTC { /* L2/MAC/7.1 * 7.1.1.4.2.x: UL-SCH Transport Block Size selection DCI 0_1 */ var integer v_TBSize :=0; // transport block size in bits var integer v_I_MCS :=0; // Modulation and Coding Scehme Index var integer v_N_PRB :=1; // Number of physical resource Blocks var integer v_Max_nPRB := f_NR_CellInfo_GetCarrierBandwidth_DL(nr_Cell1); // to store the max allowed PRB resources that can be allocated var integer v_TDRA; // Time Domain resource allocation var NR_ResourceAllocationType_Type v_NR_ResourceAllocationType := p_NR_ResourceAllocationType; //var integer i; var NR_PDCP_SDUList_Type v_NR_PDCP_SDUList := {''O}; var IntegerList_Type v_NoOfSymbols := {4,14}; var SubcarrierSpacing v_SCS := f_NR_CellInfo_GetSCS(nr_Cell1); var SubFrameNumber_Type v_SF := 2; //@sic R5s210884 sic@ var default v_IgnoreNRSR:= null; var integer v_Max_Imcs :=p_Max_Imcs; //@sic R5s211564 sic@ var integer v_TBSizeLow := 136; //for normal UE //var integer v_NumSR_Rcvd :=0; //@sic R5s220320 sic@ timer t_Watchdog := 12.0; if (v_SCS ==kHz30) {v_SF:= 1;} if(pc_supportOfRedCap_r17) { v_TBSizeLow := 120; } //for Redcap lowest allowed TBSize is 120 bits // Configure SS for DL grant for Max TB size f_NR_DLGrantConfiguration(nr_Cell1, cs_TimingInfo_SubFrame(v_SF),//@sic R5s210884 sic@ resourceAllocationType1, cs_NR_FreqDomainResourceAssignmentDL_Explicit(0, v_Max_nPRB), int2bit(0, 4), //@sic R5s211325 R5s230108 sic@ cs_NR_TransportBlockRetransmissionList_Def(28), -, -, cs_NR_DciFormat_1_1_Params(v_SCS)); //Steps 1 to 5 are repeated for values of nPRB from 1 to v_Max_nPRB and iMCS from 0 to 28, time domain resource allocation v_TDRA 0..1 //f_NR_ConfigureULGrant_MaxTBSupported(); for (v_N_PRB := 1; v_N_PRB <= v_Max_nPRB; v_N_PRB := v_N_PRB + 1) { //for nPRB 1 to v_Max_nPRB if (p_NR_ResourceAllocationType == resourceAllocationType0) { if (not f_NR_Resourceallocation0_AllowedNPRB(v_Max_nPRB, v_N_PRB)) { continue; } } //@sic R5s211564 sic@ for (v_I_MCS := 0; v_I_MCS <= v_Max_Imcs ; v_I_MCS := v_I_MCS + 1) { //for iMCS 0 to 28 if ((v_I_MCS >= 27 and v_N_PRB <= 5) or (v_I_MCS == 28 and (v_N_PRB == 10 or v_N_PRB == 14))) { //@sic R5s200481 sic@ continue; } for (v_TDRA := 0; v_TDRA <= 1 ; v_TDRA := v_TDRA + 1) { //for Time Domain Resource Allocation 0 to 1 if(p_NR_ResourceAllocationType == resourceAllocationType1 and v_I_MCS == 5 and v_N_PRB == 123 and v_TDRA == 0) { //@sic R5s200481 sic@ continue; } //SS looks up iTBS in table 7.1.7.1-1 in TS 36.213 based on the value of iMCS //v_I_TBS := f_NR_Get_iTBS_For_iMCS(v_I_MCS); //The SS calculates or looks up TBS in TS 38.214 [15] based on the value of S, L, and nPRB. v_TBSize := f_NR_TBS_Calculation(p_CalculationParameters, v_NoOfSymbols[v_TDRA], v_N_PRB, v_I_MCS); //@sic R5-227168 R5-227167 sic@ if (v_TBSize < v_TBSizeLow) { //this results in zero number of PDCP SDUs continue; } // skip this nprb, Imcs, TDRA combination. // Steps 2 to 5 are performed if TBsize is More than minimum //@siclog "Step 2" siclog@ // SS creates one or more PDCP SDUs, depending on TBsize, in accordance with Table 7.1.1.4.1.1.3.2-2 //@siclog "Step 3" siclog@ // SS sends MAC PDU (NxPDCP SDUs) v_NR_PDCP_SDUList := fl_NR_GetPdcpSduList_1TB(v_TBSize); //calculates v_N, v_PDCPsize and returns f_NR_Build_N_PDCP_SDUList f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_SchedReq(enable)); f_NR_ULGrantConfiguration_Stop(nr_Cell1, -, cs_NR_UplinkTimeAlignment_Keep); //@sic R5s211325 sic@ v_IgnoreNRSR := activate(a_NR5GC_SR_Handler(nr_Cell1)); DRB.send(cas_NR_DRB_COMMON_REQ_PDCP_SDUList(nr_Cell1, p_NR_DRB_Id, //crs_RlcBearerRouting_NR(nr_Cell1), cs_TimingInfo_Now, v_NR_PDCP_SDUList)); //@siclog "Step 4" siclog@ // v_NumSR_Rcvd :=0; t_Watchdog.start; alt { /*[v_NumSR_Rcvd == 0] SYSIND.receive (car_NR_SchedulingReq_IND (nr_Cell1, ?)) { v_NumSR_Rcvd :=1; repeat; } */ [] SYSIND.receive (car_NR_SchedulingReq_IND (nr_Cell1, ?)) { //@sic R5s220095 sic@ f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_SchedReq(disable)); f_NR_ULGrantConfiguration_ULTBS(nr_Cell1, cs_TimingInfo_Now, v_I_MCS, v_N_PRB, int2bit(v_TDRA, 4), v_NR_ResourceAllocationType); //@siclog "Step 5" siclog@ // SS receives NxPDCP SDUs f_NR_Rx_N_PDCP_SDUs_InOneTTI(v_NR_PDCP_SDUList, p_NR_DRB_Id); deactivate(v_IgnoreNRSR); t_Watchdog.stop; } []t_Watchdog.timeout { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 5"); } }; }//End for TDRA 0 to 13 }//End for iMCS 0 to 28 }//End for nPRB 1 to 110 };//f_NR_7_1_1_4_2_x

After Change:

function f_TC_7_1_1_4_2_x_DCI0_1_NR_TestBody(DRB_Identity p_NR_DRB_Id, template (value) NR_TBS_CalculationParameters_Type p_CalculationParameters, NR_ResourceAllocationType_Type p_NR_ResourceAllocationType,//@sic R5s211564 sic@ integer p_Max_Imcs := 28) runs on NR_BASE_PTC { /* L2/MAC/7.1 * 7.1.1.4.2.x: UL-SCH Transport Block Size selection DCI 0_1 */ var integer v_TBSize :=0; // transport block size in bits var integer v_I_MCS :=0; // Modulation and Coding Scehme Index var integer v_N_PRB :=1; // Number of physical resource Blocks var integer v_Max_nPRB := f_NR_CellInfo_GetCarrierBandwidth_DL(nr_Cell1); // to store the max allowed PRB resources that can be allocated var integer v_TDRA; // Time Domain resource allocation var NR_ResourceAllocationType_Type v_NR_ResourceAllocationType := p_NR_ResourceAllocationType; //var integer i; var NR_PDCP_SDUList_Type v_NR_PDCP_SDUList := {''O}; var IntegerList_Type v_NoOfSymbols := {4,14}; var SubcarrierSpacing v_SCS := f_NR_CellInfo_GetSCS(nr_Cell1); var SubFrameNumber_Type v_SF := 2; //@sic R5s210884 sic@ var default v_IgnoreNRSR:= null; var integer v_Max_Imcs :=p_Max_Imcs; //@sic R5s211564 sic@ var integer v_TBSizeLow := 136; //for normal UE //var integer v_NumSR_Rcvd :=0; //@sic R5s220320 sic@ timer t_Watchdog := 12.0; var integer v_StartIndex:=0; if (v_SCS ==kHz30) {v_SF:= 1;} if(pc_supportOfRedCap_r17) { v_TBSizeLow := 120; } //for Redcap lowest allowed TBSize is 120 bits // Configure SS for DL grant for Max TB size f_NR_DLGrantConfiguration(nr_Cell1, cs_TimingInfo_SubFrame(v_SF),//@sic R5s210884 sic@ resourceAllocationType1, cs_NR_FreqDomainResourceAssignmentDL_Explicit(0, v_Max_nPRB), int2bit(0, 4), //@sic R5s211325 R5s230108 sic@ cs_NR_TransportBlockRetransmissionList_Def(28), -, -, cs_NR_DciFormat_1_1_Params(v_SCS)); //Steps 1 to 5 are repeated for values of nPRB from 1 to v_Max_nPRB and iMCS from 0 to 28, time domain resource allocation v_TDRA 0..1 //f_NR_ConfigureULGrant_MaxTBSupported(); for (v_N_PRB := 1; v_N_PRB <= v_Max_nPRB; v_N_PRB := v_N_PRB + 1) { //for nPRB 1 to v_Max_nPRB if (p_NR_ResourceAllocationType == resourceAllocationType0) { if (not f_NR_Resourceallocation0_AllowedNPRB(v_Max_nPRB, v_N_PRB,v_StartIndex)) { continue; } } //@sic R5s211564 sic@ for (v_I_MCS := 0; v_I_MCS <= v_Max_Imcs ; v_I_MCS := v_I_MCS + 1) { //for iMCS 0 to 28 if ((v_I_MCS >= 27 and v_N_PRB <= 5) or (v_I_MCS == 28 and (v_N_PRB == 10 or v_N_PRB == 14))) { //@sic R5s200481 sic@ continue; } for (v_TDRA := 0; v_TDRA <= 1 ; v_TDRA := v_TDRA + 1) { //for Time Domain Resource Allocation 0 to 1 if(p_NR_ResourceAllocationType == resourceAllocationType1 and v_I_MCS == 5 and v_N_PRB == 123 and v_TDRA == 0) { //@sic R5s200481 sic@ continue; } //SS looks up iTBS in table 7.1.7.1-1 in TS 36.213 based on the value of iMCS //v_I_TBS := f_NR_Get_iTBS_For_iMCS(v_I_MCS); //The SS calculates or looks up TBS in TS 38.214 [15] based on the value of S, L, and nPRB. v_TBSize := f_NR_TBS_Calculation(p_CalculationParameters, v_NoOfSymbols[v_TDRA], v_N_PRB, v_I_MCS); //@sic R5-227168 R5-227167 sic@ if (v_TBSize < v_TBSizeLow) { //this results in zero number of PDCP SDUs continue; } // skip this nprb, Imcs, TDRA combination. // Steps 2 to 5 are performed if TBsize is More than minimum //@siclog "Step 2" siclog@ // SS creates one or more PDCP SDUs, depending on TBsize, in accordance with Table 7.1.1.4.1.1.3.2-2 //@siclog "Step 3" siclog@ // SS sends MAC PDU (NxPDCP SDUs) v_NR_PDCP_SDUList := fl_NR_GetPdcpSduList_1TB(v_TBSize); //calculates v_N, v_PDCPsize and returns f_NR_Build_N_PDCP_SDUList f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_SchedReq(enable)); f_NR_ULGrantConfiguration_Stop(nr_Cell1, -, cs_NR_UplinkTimeAlignment_Keep); //@sic R5s211325 sic@ v_IgnoreNRSR := activate(a_NR5GC_SR_Handler(nr_Cell1)); DRB.send(cas_NR_DRB_COMMON_REQ_PDCP_SDUList(nr_Cell1, p_NR_DRB_Id, //crs_RlcBearerRouting_NR(nr_Cell1), cs_TimingInfo_Now, v_NR_PDCP_SDUList)); //@siclog "Step 4" siclog@ // v_NumSR_Rcvd :=0; t_Watchdog.start; alt { /*[v_NumSR_Rcvd == 0] SYSIND.receive (car_NR_SchedulingReq_IND (nr_Cell1, ?)) { v_NumSR_Rcvd :=1; repeat; } */ [] SYSIND.receive (car_NR_SchedulingReq_IND (nr_Cell1, ?)) { //@sic R5s220095 sic@ f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_SchedReq(disable)); f_NR_ULGrantConfiguration_ULTBS(nr_Cell1, cs_TimingInfo_Now, v_I_MCS, v_N_PRB, int2bit(v_TDRA, 4), v_NR_ResourceAllocationType); //@siclog "Step 5" siclog@ // SS receives NxPDCP SDUs f_NR_Rx_N_PDCP_SDUs_InOneTTI(v_NR_PDCP_SDUList, p_NR_DRB_Id); deactivate(v_IgnoreNRSR); t_Watchdog.stop; } []t_Watchdog.timeout { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 5"); } }; }//End for TDRA 0 to 13 }//End for iMCS 0 to 28 }//End for nPRB 1 to 110 };//f_NR_7_1_1_4_2_x