3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240524

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3499 | rev | - | Current version: | 17.11.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function fl_NR_GetPdcpSduList_1TB |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Each PDCP SDU is limited to 1500 octets and the TBS test cases goes beyond 20 PDCP SDUs for certain mcx Nprb values. The function fl_NR_GetPdcpSduList_1TB needs to extended to ensure that >20 PDCP SDUs are still following the 1500 octet limitation Prose change required to 38.523-1 (R5-244415 will be presented in RAN5#104) |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated fl_NR_GetPdcpSduList_1TB to upto 27 PDCP SDUs |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A Conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.4.2.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | R5-244415 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | This impacts NR5GC and ENDC variants. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

2.1	Change to fl_NR_GetPdcpSduList_1TB	2

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D24wk24 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change to fl_NR_GetPdcpSduList_1TB

| Function name | fl_NR_GetPdcpSduList_1TB |
| --- | --- |
| Reason for change | Each PDCP SDU is limited to 1500 octets and the TBS test cases goes beyond 20 PDCP SDUs for certain mcx Nprb values. The function fl_NR_GetPdcpSduList_1TB needs to extended to ensure that >20 PDCP SDUs are still following the 1500 octet limitation Prose change required to 38.523-1 |
| Summary of change | Updated fl_NR_GetPdcpSduList_1TB to upto 27 PDCP SDUs |
| TTCN module | MAC_TC_Common_NR |
| MCC160 Comment |  |

Before Change:

function fl_NR_GetPdcpSduList_1TB(integer p_TBSize) return NR_PDCP_SDUList_Type {//@sic R5-227169 sic@ var integer v_N; var integer v_PdcpSduSize; if (not pc_supportOfRedCap_r17) //Normal UE { if (p_TBSize <= 12128) { v_N := 1; v_PdcpSduSize := (p_TBSize - 128)/8; } … … … } else { v_N := 20; v_PdcpSduSize := (p_TBSize - 1496)/160; } } else //Redcap UE … … …

After Change:

function fl_NR_GetPdcpSduList_1TB(integer p_TBSize) return NR_PDCP_SDUList_Type {//@sic R5-227169 sic@ var integer v_N; var integer v_PdcpSduSize; if (not pc_supportOfRedCap_r17) //Normal UE { if (p_TBSize <= 12128) { v_N := 1; v_PdcpSduSize := (p_TBSize - 128)/8; } … … … }//extended the PDU split else if (p_TBSize <= 241496) { v_N := 20; v_PdcpSduSize := (p_TBSize - 1496)/160; } else if (p_TBSize <= 253568) { v_N := 21; v_PdcpSduSize := (p_TBSize - 1568)/168; } else if (p_TBSize <= 265640) { v_N := 22; v_PdcpSduSize := (p_TBSize - 1640)/176; } else if (p_TBSize <= 277712) { v_N := 23; v_PdcpSduSize := (p_TBSize - 1712)/184; } else if (p_TBSize <= 289784) { v_N := 24; v_PdcpSduSize := (p_TBSize - 1784)/192; } else if (p_TBSize <= 301856) { v_N := 25; v_PdcpSduSize := (p_TBSize - 1856)/200; } else if (p_TBSize <= 313928) { v_N := 26; v_PdcpSduSize := (p_TBSize - 1928)/208; } else if (p_TBSize <= 326000) { v_N := 27; v_PdcpSduSize := (p_TBSize - 2000)/216; } else { FatalError(__FILE__, __LINE__, "invalid TBS size"); } } else //Redcap UE