3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240531

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3506 | rev | - | Current version: | 17.11.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 Jun'24 baseline upgrade for 5GS Test Suites |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18_Test |  |  |  |  |  | Date: |  |  | 2024-07-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In order to develop the Rel-18 test cases in TTCN-3, the current baseline of type definitions shall be upgraded. RAN5 agreed to upgrade the baseline to June 2024 to all RAN5 TTCN-3 test suites. This CR is implementing this decision on the 38.523-3 Test Suites. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The CR lists all changes affecting the messages and IEs type definitions caused by the baseline upgrade to June 2024 Rel-18. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Rel-18 test cases cannot be implemented and verified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | All 5GS Test Suites |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

Overview	3

1	Baseline Moving – 38.331 (RRC ASN.1)	4

1.1	NR_RRC_ASN1_Definitions	4

1.1.1	Type change of nonCriticalExtension	4

1.1.2	Name changes	5

1.1.3	Fields added with extension marker (ellipsis)	7

1.1.4	Further ASN.1 changes	25

1.2	NR_SideLink_Preconf	25

1.2.1	Fields added with extension marker (ellipsis)	25

1.3	NR_PC5_RRC_Definitions	26

1.3.1	Type change of nonCriticalExtension	26

1.3.2	Name changes	27

1.3.3	Fields added with extension marker (ellipsis)	27

1.3.4	Further ASN.1 changes	27

2	Baseline Moving – 38.212 38.214 (PHY)	28

3	Baseline Moving – 38.321 (MAC)	28

4	Baseline Moving – 38.322 (RLC)	28

5	Baseline Moving – 38.323 (PDCP)	28

6	Baseline Moving – 37.324 (SDAP)	29

7	Baseline Moving – 24.501 (NAS)	30

7.1	Modified Message Definitions	30

7.2	Modified Information Elements	31

7.3	New Information Elements	32

8	Baseline Moving – 24.526 (V2X)	32

8.1	Modified Information Elements	32

9	Baseline Moving – 24.587 (V2X)	33

9.1	Modified Message Definitions	33

9.2	Modified Information Elements	33

10	Baseline Moving – 24.588 (V2X)	33

10.1	Modified Information Elements	33

10.2	New Information Elements	34

11	Baseline Moving – 38.509 (TMC)	34

12	Other changes	34

12.1	Function f_NR_RRCRelease	34

12.2	Changes for Audit test cases	34

12.3	Editorial change in NR_SideLink_ASP_TypeDefs	45

12.4	Split the definition of the CAG_Info (9.11.3.18A) and ExtdCAGInfo (9.11.3.86) Types	45

12.5	Correction of the LADN_Info (9.11.3.30) Type	46

12.6	Correction of the NSAG_Info (9.11.3.87) Type	46

12.7	Updating of the SOR_Header (9.11.3.51) Type	47

12.8	Splitting of the TMGI_LV (9.11.4.31) Type	47

12.9	Changes for SOR test cases due to definition of new structured types	48

13	Summary of ASP changes	54

# Overview

This document lists all the changes needed for baseline moving based on iwd-TTCN3-B2022-09_D24wk24.

Contact:	Olivier Genoud
	olivier.genoud@etsi.org

# Baseline Moving – 38.331 (RRC ASN.1)

## NR_RRC_ASN1_Definitions

### Type change of nonCriticalExtension

| ASN.1 type affected | New nonCriticalExtension type |
| --- | --- |
| DLInformationTransfer-v1700-IEs | DLInformationTransfer-v1800-IEs |
| LoggedMeasurementConfiguration-v1700-IEs | LoggedMeasurementConfiguration-v1800-IEs |
| MBSInterestIndication-r17-IEs | MBSInterestIndication-v1800 |
| MeasurementReportAppLayer-r17-IEs | MeasurementReportAppLayer-v1800-IEs |
| Paging-v1700-IEs | Paging-v1800-IEs |
| RRCReestablishmentComplete-v1610-IEs | RRCReestablishmentComplete-v1800-IEs |
| RRCReconfiguration-v1700-IEs | RRCReconfiguration-v1800-IEs |
| RRCReconfigurationComplete-v1720-IEs | RRCReconfigurationComplete-v1800-IEs |
| RRCResume-v1700-IEs | RRCResume-v1800-IEs |
| RRCResumeComplete-v1720-IEs | RRCResumeComplete-v1800-IEs |
| RRCSetupComplete-v1700-IEs | RRCSetupComplete-v1800-IEs |
| SIB1-v1740-IEs | SIB1-v1800-IEs |
| SidelinkUEInformationNR-v1700-IEs | SidelinkUEInformationNR-v1800-IEs |
| UEAssistanceInformation-v1700-IEs | UEAssistanceInformation-v1800-IEs |
| UEInformationRequest-v1700-IEs | UEInformationRequest-v1800-IEs |
| UEInformationResponse-v1700-IEs | UEInformationResponse-v1800-IEs |
| UE-MRDC-Capability-v1730 | UE-MRDC-Capability-v1800 |
| UE-MRDC-Capability-v15g0 | UE-MRDC-Capability-v15n0 |
| UE-NR-Capability-v1750 | UE-NR-Capability-v1800 |

no changes in TTCN

### Name changes

| ASN.1 type affected | Renaming | Impact on TTCN |
| --- | --- | --- |
| UL-DCCH-MessageType | spare6 renamed to indirectPathFailureInformation-r18 | No impact |
| SIB-ReqInfo-r16 | Spare3 renamed to sib23-v1810 | No impact |
| IABOtherInformation-r16 | rc-TransactionIdentifier renamed to dummy | No impact |
| LoggedMeasurementConfiguration-v1700-IEs | areaConfiguration-v1700 renamed to areaConfiguration-r17 | cs_LoggedMeasurementConfiguration_v1700_IEs |
| UEAssistanceInformation-v1700-IEs | Enumerated scgDeactivationPreferred renamed to scg-DeactivationPreferred | No impact |
| RA-Report-r16 | spare8 renamed to lbt-Failure-r18 | No impact |
| SIB-Type-r17 | sibType10-v1610 renamed to sibType10 sibType11-v1610 renamed to sibType11 sibType12-v1610 renamed to sibType12 sibType13-v1610 renamed to sibType13 sibType14-v1610 renamed to sibType14 spare6 renamed to posSIB-v1810 | No impact |
| FeatureCombination-r17 | spare4 renamed to msg1-Repetitions-r18 spare3 renamed to eRedCap-r18 | cs_FeatureCombination_Common cs_FeatureCombination_SDT cs_FeatureCombination_Msg3 cs_FeatureCombination_RedCap cs_FeatureCombination_None cs_FeatureCombination_NSAG |
| NR-DL-PRS-PDC-ResourceSet-r17 | Spare4 renamed to n1-v1800 | No impact |
|  | Spare1 renmaed to mt-SDT-v1810, Spare2 renamed to srs-PosConfigOrActivationReq-v1800 | No impact |
| ServingCellConfigCommonSIB | enhancedMeasurementLEO-r17 renamed to enhancedMeasurementNGSO-r17 | cs_38508_ServingCellConfigCommonSIB |
| SIB-TypeInfo-v1700 | Enumerated values renamed | No impact |
| AccessStratumRelease | Spare5 renamed to rel18 | f_NR_NRFullCapabilityCheck |
| BandNR | maxNumber-LEO-SatellitesPerCarrier-r17 renamed to maxNumber-NGSO-SatellitesPerCarrier-r17 | No impact |
| BandSidelink-r16 | Spare5 renamed to pc5-v1820 | No impact |
| AppLayerMeasConfig-r17 | rrc-SegAllowed-r17 renamed to rrc-SegAllowedSRB4-r17 | No impact |
| AreaConfiguration-v1700 | Type renamed to AreaConfiguration-r17 | No impact |
| SL-RS-Type-r16 | Spare3 renamed to sl-prs | No impact |
| SL-SyncConfig-r16 | sl-filterCoefficient-r16 renamed to sl-FilterCoefficient-r16 | cs_38508_SL_SyncConfig_Def |
| GroupB-ConfiguredTwoStepRA-r16 | ra-MsgA-SizeGroupA renamed to ra-MsgA-SizeGroupA-r16 ra-MsgA-SizeGroupA renamed to ra-MsgA-SizeGroupA-r16 numberOfRA-PreamblesGroupA renamed to numberOfRA-PreamblesGroupA-r16 | cs_GroupB_ConfiguredTwoStepRA_71119 |
| MeasResultNR | firstTriggeredEvent renamed to firstTriggeredEvent-r17 | cr_TriggeredEvent_r17 |

### Fields added with extension marker (ellipsis)

| ASN.1 type affected | New fields | Impact on TTCN |
| --- | --- | --- |
| RAN-VisibleMeasurements-r17 | pdu-SessionIdListExt-v1800 | No impact |
| SuspendConfig | resumeIndication-r18 srs-PosRRC-InactiveEnhanced-r18 ran-ExtendedPagingCycleConfig-r18 multicastConfigInactive-r18 | Fields added as omit in cs_SuspendConfigDef |
| SDT-MAC-PHY-CG-Config-r17 | cg-SDT-ConfigLCH-RestrictionToAddModListExt-v1800 cg-MT-SDT-MaxDurationToNextCG-Occasion-r18 | Fields added as omit in cs_SDT_MAC_PHY_CG_Config |
| SL-TxResourceReq-v1700 | sl-QoS-InfoList-v1800 | No impact |
| SL-TxResourceReqDisc-r17 | ue-TypeU2U-r18 | No impact |
| VictimSystemType-r16 | uwb-r18 | No impact |
| RA-Report-r16 | sdt-Failed-r18 | Added as * in cr_NR_RA_Report_r16 |
| RA-InformationCommon-r16 | usedFeatureCombination-r18 triggeredFeatureCombination-r18 startPreambleForThisPartition-r18 numberOfPreamblesPerSSB-ForThisPartition-r18 attemptedBWP-InfoList-r18 numberOfLBT-Failures-r18 perRAInfoList-v1800 intendedSIBs-r18 | Added as * in cr_RA_InformationCommon_r16 |
| RLF-Report-r16 | pSCellId-r18 mcg-RecoveryFailureCause-r18 scg-FailureCause-r18 elapsedTimeSCG-Failure-r18 voiceFallbackHO-r18 measResultLastServCellRSSI-r18 measResultNeighFreqListRSSI-r18 bwp-Info-r18 elapsedTimeT316-r18 scg-FailedAfterMCG-r18 | Added as * in cr_NR_Rlf_Report, which is renamed to cr_NR_Rlf_Report_Common cr_NR_RLF_Report_r16 renamed to cr_NR_RLF_Report_CellIDs and updated along with cr_NR_RLF_Report_DAPS calling cr_NR_Rlf_Report_Common |
| SuccessHO-Report-r17 | targetCell-PCI-ARFCN-r17 eutra-TargetCellInfo-r18 measResultServCellRSSI-r18 measResultNeighFreqListRSSI-r18 eutra-C-RNTI-r18 timeSinceSHR-r18 | Added as * in cr_SuccessHO_Report_DAPS |
| SIB2 | frequencyBandList-v1760 frequencyBandListSUL-v1760 frequencyBandListAerial-r18 | Added as omit in cs_38508_SIB2_Def |
| SIB4 | interFreqCarrierFreqList-v1760 interFreqCarrierFreqList-v1800 | Added as omit in cs_38508_SIB4_Def |
| SIB5 | carrierFreqListEUTRA-v1800 | Added as omit in cs_38508_SIB5_Def |
| SIB9 | referenceTimeInfo-r16 eventID-TSS-r18 | No impact |
| SIB12-IEs-r16 | sl-FreqInfoListSizeExt-v1800 sl-RLC-BearerConfigListSizeExt-v1800 sl-SyncFreqList-r18 sl-SyncTxMultiFreq-r18 sl-MaxTransPowerCA-r18 sl-DiscConfigCommon-v1800 sl-L2-U2U-Relay-r18 sl-L3-U2U-RelayDiscovery-r18 t400-U2U-r18 | Added as omit in cs_38508_SIB12_IEs |
| SIB19-r17 | movingReferenceLocation-r18 ntnCovEnh-r18 satSwitchWithReSync-r18 | Added as omit in cs_38508_SIB19_Def |
| SIB20-r17 | cfr-ConfigMCCH-MTCH-RedCap-r18 mcch-ConfigRedCap-r18 | No impact |
| PosSI-SchedulingInfo-r16 | posSI-RequestConfigMSG1-Repetition-r18 posSI-RequestConfigSUL-MSG1-Repetition-r18 posSI-RequestConfigRedCap-MSG1-Repetition-r18 | Added as omit in cs_PosSI_SchedulingInfo |
| BWP-DownlinkDedicated | tci-InDCI-r18 | Added as omit in cs_BWP_DownlinkDedicated_Def |
| BWP-UplinkCommon | additionalRACH-perPCI-ToAddModList-r18 additionalRACH-perPCI-ToReleaseList-r18 rsrp-ThresholdMsg1-RepetitionNum2-r18 rsrp-ThresholdMsg1-RepetitionNum4-r18 rsrp-ThresholdMsg1-RepetitionNum8-r18 preambleTransMax-Msg1-Repetition-r18 | Added as omit in cs_BWP_UplinkCommon_Def |
| CellGroupConfig | prioSCellPRACH-OverSP-PeriodicSRS-r17 ncr-FwdConfig-r18 autonomousDenialParameters-r18 nonCollocatedTypeMRDC-r18 nonCollocatedTypeNR-CA-r18 uplinkTxSwitchingMoreBands-r18 | Added as omit in cs_NR_CellGroupConfigDef |
| ReconfigurationWithSync | rach-LessHO-r18 sl-IndirectPathMaintain-r18 | Added as omit in cs_NR_ReconfigurationWithSyncDef |
| CondReconfigToAddMod-r16 | condExecutionCondPSCell-r18 subsequentCondReconfig-r18 securityCellSetId-r18 scpac-ConfigComplete-r18 | Added as omit in cs_CondReconfigToAddMod_Def |
| ConditionalReconfiguration-r16 | scpac-ReferenceConfiguration-r18 servingSecurityCellSetId-r18 sk-CounterConfiguration-r18 | Added as omit in cs_ConditionalReconfiguration_r16 |
| ConfiguredGrantConfig | In rrc-ConfiguredUplinkGrant added srs-ResourceSetId-r18 cg-LTM-Configuration-r18 cg-SDT-PeriodicityExt-r18 timeReferenceHyperSFN-r18 cg-RRC-Configuration-r18 applyIndicatedTCI-State-r18 | Added as omit in cs_RRC_ConfiguredUplinkGrant |
| ConfiguredGrantConfig | disableCG-RetransmissionMonitoring-r18 nrofSlotsInCG-Period-r18 uto-UCI-Config-r18 | Added as omit in cs_ConfiguredGrantConfig |
| ControlResourceSet | applyIndicatedTCI-State-r18 | Added as omit in cs_38508_ControlResourceSet |
| CSI-AperiodicTriggerState | ltm-AssociatedReportConfigInfo-r18 | No impact |
| CSI-AssociatedReportConfigInfo | resourcesForChannelTDCP-r18 applyIndicatedTCI-State-r18 perResource-r18 applyIndicatedTCI-State2-r18 perResource-r18 csi-ReportSubConfigTriggerList-r18 | No impact |
| CSI-MeasConfig | ltm-CSI-ReportConfigToAddModList-r18 ltm-CSI-ReportConfigToReleaseList-r18 | Added as omit in cs_CSI_MeasConfig_Def |
| CSI-ReportConfig | groupBasedBeamReporting-v1800 reportQuantity-r18 codebookConfig-r18 csi-ReportSubConfigToAddModList-r18 csi-ReportSubConfigToReleaseList-r18 | No impact |
| CSI-SemiPersistentOnPUSCH-TriggerState | csi-ReportSubConfigTriggerList-r18 ltm-AssociatedReportConfigInfo-r18 | No impact |
| DMRS-DownlinkConfig | dmrs-TypeEnh-r18 | Added as omit in cs_NR_DMRS_DownlinkConfig_Def |
| DMRS-UplinkConfig | dmrs-TypeEnh-r18 | Added as omit in cs_38508_DMRS_UplinkConfig |
| DownlinkConfigCommonSIB | frequencyInfoDL-v1800 | Added as omit in cs_38508_DownlinkConfigCommonSIB |
| FeatureCombinationPreambles-r17 | msg1-RepetitionNum-r18 msg1-RepetitionTimeOffsetROGroup-r18 | Added as omit in cs_FeatureCombinationPreambles cs_FeatureCombinationPreambles_711113and4 cs_FeatureCombinationPreambles_711111 |
| FrequencyInfoDL | referenceCell-r18 | Added as omit in cs_FrequencyInfoDL |
| FrequencyInfoUL | additionalSpectrumEmission-v1760 additionalSpectrumEmissionAerial-r18 | Added as omit in cs_FrequencyInfoUL |
| FrequencyInfoUL-SIB | frequencyBandListAerial-r18 | Added as omit in cs_38508_FrequencyInfoUL_SIB |
| MAC-CellGroupConfig | drx-ConfigExt2-v1800 additionalBS-TableAllowed-r18 dsr-ConfigToAddModList-r18 dsr-ConfigToReleaseList-r18 tar-Config-r18 | Added as omit in cs_NR_MAC_CellGroupConfig |
| MeasConfig | effectiveMeasWindowConfig-r18 | Added as omit in cs_NR_MeasConfigCommon |
| MeasIdleConfigSIB-r16 | measIdleCarrierListNR-LessThan5MHz-r18 measReselectionCarrierListNR-r18 measReselectionCarrierListNR-LessThan5MHz-r18 measIdleValidityDuration-r18 measReselectionValidityDuration-r18 | Added as omit in cs_38508_SIB11_Def cs_38508_SIB11_EMR_NR cs_EMR_EUTRA_SIB |
| MeasIdleConfigDedicated-r16 | measReselectionCarrierListNR-r18 measIdleValidityDuration-r18 measReselectionValidityDuration-r18 | Added as omit in cs_MeasIdleConfigDedicated_r16 cs_RRCConnectionRelease_measIdleConfig |
| MeasObjectEUTRA | measSequence-r18 | Added as omit in cs_MeasObjectEUTRA_Common |
| MeasObjectNR | measSequence-r18 cellsToAddModListExt-v1800 | Added as omit in cs_NR_MeasObjectNR_Base |
| SSB-ConfigMobility | ssb-ToMeasureAltitudeBasedList-r18 | Added as omit in cs_SSB_ConfigMobility |
| MeasResults | altitudeUE-r18 cellsMetReportOnLeaveList-r18 | Added as omit in cr_38508_MeasResults cr_38508_MeasResultsSFTD_EUTRA cr_38508_MeasResults_CA cr_38508_MeasResults_Beams cr_NR_MeasResults_SL cr_38508_MeasResults_NR_EUTRA cr_38508_MeasResults_RSSI |
| MeasResultNR | firstEntering-r18 | Added as omit in cr_NR_MeasResultNR |
| MeasResultsPerCellIdleNR-r16 | validityStatus-r18 | Added as * in cr_MeasResultIdleNR_r16 |
| MUSIM-GapConfig-r17 | musim-GapToAddModListExt-v1820 musim-GapKeep-r18 | Added as omit in cs_MUSIM_GapConfig |
| NPN-IdentityInfo-r16 | mobileIAB-Support-r18 | Added as omit in cs_NPN_IdentityInfo_Def |
| NR-DL-PRS-Resource-r17 | dl-PRS-ResourceSymbolOffset-v1800 | No impact |
| NZP-CSI-RS-Resource | subcarrierSpacing-r18 absoluteFrequencyPointA-r18 cyclicPrefix-r18 | Added as omit in cs_NZP_CSI_RS_Resource |
| NZP-CSI-RS-ResourceSet | resourceType-r18 | Added as omit in cs_NZP_CSI_RS_ResourceSet |
| PDCCH-Config | pdcch-MonitoringResumptionAfterNack-r18 searchSpacesToAddModListExt-v1800 | Added as omit in cs_38508_PDCCH_Config |
| PDCCH-ConfigCommon | applyIndicatedTCI-State-r18 commonSearchSpaceListExt-r18 searchSpaceMulticastMCCH-r18 searchSpaceMulticastMTCH-r18 | Added as omit in cs_38508_PDCCH_ConfigCommon |
| PDCP-Config | pdu-SetDiscard-r18 discardTimerForLowImportance-r18 primaryPathOnIndirectPath-r18 sn-GapReport-r18 | Added as omit in cs_PDCP_Config_Def cs_PDCP_Config_Def |
| PDSCH-Config | advancedReceiver-MU-MIMO-r18 pdsch-ConfigDCI-1-3-r18 | Added as omit in cs_NR_PDSCH_ConfigDef |
| PHR-Config | phr-AssumedPUSCH-Reporting-r18 dpc-Reporting-FR1-r18 | Added as omit in cs_PHR_Config_Params |
| PhysicalCellGroupConfig | simultaneousPUCCH-PUSCH-SamePriority-r17 simultaneousPUCCH-PUSCH-SamePriority-SecondaryPUCCHgroup-r17 ncr-RNTI-r18 cellDTRX-DCI-config-r18 twoQCL-TypeD-ForMultiDCI-r18 enableType1HARQ-ACK-MuxForDL-AssignmentAfterUL-Grant-r18 enableType2HARQ-ACK-MuxForDL- enableType3HARQ-ACK-MuxForDL- enableDiffPUCCH-Resource-r18 enableDiffCB-Size-r18 | Added as omit in cs_NR_PhysicalCellGroupConfigDef |
| PLMN-IdentityInfo | mobileIAB-Support-r18 | Added as omit in cs_NR_PLMN_IdentityInfo |
| PTRS-UplinkConfig | maxNrofPorts-SDM-r18 | Added as omit in cs_38508_PTRS_UplinkConfig |
| PUCCH-ResourceExt-v1610 | applyIndicatedTCI-State-r18 multipanelSFN-Scheme-r18 dl-DataToUL-ACK-r18 dl-DataToUL-ACK-DCI-1-2-r18 | No impact |
| PUSCH-Config | maxRank-v1810 sTx-2Panel-r18 multipanelSchemeSDM-r18 multipanelSchemeSFN-r18 codebookTypeUL-r18 applyIndicatedTCI-State-r18 dynamicTransformPrecoderFieldPresenceDCI-0-1-r18 dynamicTransformPrecoderFieldPresenceDCI-0-2-r18 pusch-ConfigDCI-0-3-r18 | Added as omit in cs_NR_PUSCH_ConfigDef |
| PUSCH-ServingCellConfig | maxMIMO-Layers-v1810 maxMIMO-LayersforSDM-r18 maxMIMO-LayersforSDM-DCI-0-2-r18 maxMIMO-LayersforSFN-r18 maxMIMO-LayersforSFN-DCI-0-2-r18 | Added as omit in cs_NR_PUSCH_ServingCellConfigDef |
| CFRA | msg1-RepetitionNum-r18 | Added as omit in cs_38508_RACH_ConfigDedicated |
| RadioBearerConfig | srb5-ToAddMod-r18 srb5-ToRelease-r18 | Added as omit in cs_38508_RadioBearerConfigDef |
| SRB-ToAddMod | srb-Identity-v1800 n3c-BearerAssociated-r18 | Added as omit in cs_NR_SRB_ToAddMod |
| DRB-ToAddMod | n3c-BearerAssociated-r18 | Added as omit in cs_NR_DRB_ToAddMod |
| EventTriggerConfigInterRAT | cellIndividualOffsetList-r18 | Added as omit in cs_NR_ReportConfigInterRAT_Trigger_eventB1_EUTRA cs_NR_ReportConfigInterRAT_Trigger_eventB1_EUTRA_Rsrq cs_NR_ReportConfigInterRAT_Trigger_eventB2_EUTRA cs_NR_ReportConfigInterRAT_Trigger_eventB2_EUTRA_Rsrq cs_NR_ReportConfigInterRAT_Trigger_eventB2_UTRA cs_NR_ReportConfigInterRAT_Trigger_eventB1_UTRA |
| CondTriggerConfig-r16 | nesEvent-r18 | Added as omit in cs_38508_CondTriggerConfig_EventA5 cs_CondTriggerConfig_EventA4 cs_38508_CondTriggerConfig Added as * in cr_ChoConfig_r17_condEventA3 cr_ChoConfig_r17_condEventA5 |
| EventTriggerConfig | numberOfTriggeringCells-r18 cellIndividualOffsetList-r18 eventX1-SD-Threshold1-r18 eventX2-SD-Threshold-r18 reportOnBestCellChange-r18 enteringLeavingReport-r18 | Added as omit in cs_38508_ReportConfigNR_EventTriggered |
| SearchSpace | dci-FormatsNCR-r18 dci-FormatsSL-PRS-r18 | Added as omit in cs_NR_SearchSpace_SearchSpaceType_USS cs_NR_SearchSpace_SearchSpaceType_SLSS |
| ServingCellConfig | multiPDSCH-PerSlotType1-CB-r17 lte-CRS-PatternList3-r18 lte-CRS-PatternList4-r18 pdcch-CandidateReceptionWith-CRS-Overlap-r18 cjt-Scheme-PDSCH-r18 tag2-r18 cellDTX-DRX-Config-r18 positionInDCI-cellDTRX-r18 cellDTX-DRX-L1activation-r18 mc-DCI-SetOfCellsToAddModList-r18 mc-DCI-SetOfCellsToReleaseList-r18 | Added as omit in cs_NR_ServingCellConfigDef |
| UplinkConfig | srs-PosTx-Hopping-r18 enablePL-RS-UpdateForType1CG-PUSCH-r18 powerBoostPi2BPSK-r18 powerBoostQPSK-r18 | Added as omit in cs_NR_UplinkConfigDef cs_NR_UplinkConfigSrs |
| ServingCellConfigCommon | featurePriorities-v1800 atg-Config-r18 | Added as omit in cs_NR_ServingCellConfigCommonDef |
| ServingCellConfigCommonSIB | downlinkConfigCommon-v1760 uplinkConfigCommon-v1760 | Added as omit in cs_38508_ServingCellConfigCommonSIB |
| SRS-Config | dci-TriggeringPosResourceSetLink-r18 | Added as omit in cs_SRS_Config_Def |
| SRS-ResourceSet | applyIndicatedTCI-State-r18 | Added as omit in cs_38508_SRS_ResourceSet |
| SRS-Resource | nrofSRS-Ports-n8-r18 combOffsetHopping-r18 cyclicShiftHopping-r18 | Added as omit in cs_38508_SRS_Resource |
| SRS-PosResource-r16 | srs-PosPeriodicConfigHyperSFN-Index-r18 srs-PosPeriodicConfigHyperSFN-Index-r18 txHoppingConfig-r18 | Added as omit in cs_SRS_PosResourceElement |
| TCI-State | tag-Id-ptr-r18 | Added as omit in cs_38508_TCI_State cs_TCI_State |
| TCI-UL-State-r17 | tag-Id-ptr-r18 | No impact |
| AppLayerMeasParameters-r17 | qoe-IdleInactiveMeasReport-r18 qoe-NRDC-MeasReport-r18 qoe-AdditionalMemoryMeasReport-r18 qoe-PriorityBasedDiscarding-r18 srb5-r18 | No impact |
| FeatureSets | featureSetsDownlinkPerCC-v1780 featureSetsUplinkPerCC-v1780 featureSetsDownlink-v1800 featureSetsDownlinkPerCC-v1800 featureSetsUplink-v1800 featureSetsUplinkPerCC-v1800 | Added as * in cr_UE_NR_FeatureSets |
| MAC-ParametersCommon | harq-RTT-TimerDL-ForNTN-MulticastMBS-r17 sr-TriggeredByTA-ReportATG-r18 extendedDRX-CycleInactive-r18 additionalBS-Table-r18 delayStatusReport-r18 cg-RetransmissionMonitoringDisabling-r18 non-IntegerDRX-r18 | Added as * in cr_NR_mac_ParametersCommon |
| MAC-ParametersXDD-Diff | ptm-Retransmission-r18 ptm-RetransmissionInactive-r18 | Added as * in cr_NR_mac_ParametersXDD_Diff |
| MeasAndMobParametersCommon | l3-MeasUnknownSCellActivation-r18 shortMeasInterval-r18 nr-NeedForInterruptionReport-r18 measSequenceConfig-r18 cellIndividualOffsetPerMeasEvent-r18 eventD2-MeasReportTrigger-r18 concurrentMeasGapsPreMG-r18 dynamicCollision-r18 concurrentMeasGapsNCSG-r18 eutra-NoGapMeasurementOutsideBWP-r18 eutra-NoGapMeasurementInsideBWP-r18 eutra-MeasEMW-r18 concurrentMeasCRS-InsideBWP-EUTRA-r18 ltm-InterFreqMeasGap-r18 ltm-FastUE-Processing-r18 rach-LessHandoverInterFreq-r18 enterAndLeaveCellReport-r18 bestCellChangeReport-r18 secondBestCellChangeReport-r18 | Added as * in cr_NR_MeasAndMobParametersCommon cr_NR_MAC_ParametersXDD_NoCheck |
| MIMO-ParametersPerBand | codebookParametersetype2DopplerCSI-r18 codebookParametersfetype2DopplerCSI-r18 codebookParametersetype2CJT-r18 codebookParametersfetype2CJT-r18 codebookComboParametersCJT-r18 codebookParametersHARQ-ACK-PUSCH-r18 tci-JointTCI-UpdateSingleActiveTCI-PerCC-r18 tci-JointTCI-UpdateMultiActiveTCI-PerCC-r18 tci-SelectionDCI-r18 tci-SeparateTCI-UpdateSingleActiveTCI-PerCC-r18 tci-SeparateTCI-UpdateMultiActiveTCI-PerCC-r18 tci-SelectionAperiodicCSI-RS-r18 tci-SelectionAperiodicCSI-RS-M-DCI-r18 twoTCI-StatePDSCH-CJT-TxScheme-r18 tci-JointTCI-UpdateSingleActiveTCI-PerCC-PerCORESET-r18 tci-JointTCI-UpdateMultiActiveTCI-PerCC-PerCORESET-r18 tci-TRP-BFR-r18 tci-SeparateTCI-UpdateSingleActiveTCI-PerCC-PerCORESET-r18 tci-SeparateTCI-UpdateMultiActiveTCI-PerCC-PerCORESET-r18 commonTCI-SingleDCI-r18 commonTCI-MultiDCI-r18 twoPHR-Reporting-r18 spCell-TAG-Ind-r18 interCellCrossTRP-PDCCH-OrderCFRA-r18 intraCellCrossTRP-PDCCH-OrderCFRA-r18 overlapUL-TransReduction-r18 maxPeriodicityCMR-r18 tdcp-Report-r18 tdcp-Resource-r18 timelineRelax-CJT-CSI-r18 jointConfigDMRSPortDynamicSwitching-r18 srs-combOffsetHopping-r18 srs-combOffsetInTime-r18 srs-combOffsetCombinedGroupSequence-r18 srs-combOffsetHoppingWithinSubset-r18 srs-cyclicShiftHopping-r18 srs-cyclicShiftHoppingSmallGranularity-r18 srs-cyclicShiftCombinedGroupSequence-r18 cyclicShiftHoppingWithinSubset-r18 srs-cyclicShiftCombinedCombOffset-r18 pusch-CB-2PTRS-SingleDCI-STx2P-SDM-r18 pusch-NonCB-2PTRS-SingleDCI-STx2P-SDM-r18 pusch-NonCB-SingleDCI-STx2P-SDM-CSI-RS-SRS-r18 twoPUSCH-NonCB-Multi-DCI-STx2P-CSI-RS-Resource-r18 dmrs-PortEntrySingleDCI-SDM-r18 pusch-CB-2PTRS-SingleDCI-STx2P-SFN-r18 pusch-NonCB-2PTRS-SingleDCI-STx2P-SFN-r18 pusch-NonCB-SingleDCI-STx2P-SFN-CSI-RS-SRS-r18 twoPUSCH-CB-MultiDCI-STx2P-FullTimeFullFreqOverlap-r18 twoPUSCH-CB-MultiDCI-STx2P-FullTimePartialFreqOverlap-r18 twoPUSCH-CB-MultiDCI-STx2P-PartialTimeFullFreqOverlap-r18 twoPUSCH-CB-MultiDCI-STx2P-PartialTimePartialFreqOverlap-r18 twoPUSCH-CB-MultiDCI-STx2P-PartialTimeNonFreqOverlap-r18 twoPUSCH-CB-MultiDCI-STx2P-CG-CG-r18 twoPUSCH-CB-MultiDCI-STx2P-CG-DG-r18 twoPUSCH-NonCB-MultiDCI-STx2P-FullTimeFullFreqOverlap-r18 twoPUSCH-NonCB-MultiDCI-STx2P-FullTimePartialFreqOverlap-r18 twoPUSCH-NonCB-MultiDCI-STx2P-PartialTimeFullFreqOverlap-r18 twoPUSCH-NonCB-MultiDCI-STx2P-PartialTimePartialFreqOverlap-r18 twoPUSCH-NonCB-MultiDCI-STx2P-PartialTimeNonFreqOverlap-r18 twoPUSCH-NonCB-MultiDCI-STx2P-CG-CG-r18 twoPUSCH-NonCB-MultiDCI-STx2P-CG-DG-r18 pucch-RepetitionDynamicIndicationSFN-r18 groupBeamReporting-STx2P-r18 | No impact |
| PDCP-Parameters | longSN-NCR-r18 supportOfPDU-SetDiscard-r18 psi-BasedDiscard-r18 supportOfSN-GapReport-r18 | Added as * in cr_NR_PDCP_Parameters |
| Phy-ParametersCommon | multiPDSCH-PerSlotType1-CB-Support-r17 jointPowerSpatialAdaptation-r18 ncr-AperiodicBeamInd-AccessLink-r18 ncr-Semi-PersistentBeamInd-AccessLink-r18 ncr-SimultaneousUL-BackhaulAndC-Link-r18 ncr-BackhaulBeamInd-r18 ncr-AdaptiveBeamBackhaulAndC-Link-r18 nominalRBG-SizeOfConfig-3-FDRA-Type-0-DCI-1-3-r18 nominalRBG-SizeOfConfig-3-FDRA-Type-0-DCI-0-3-r18 configurableType-1A-FieldsForDCI-0-3-And-1-3-r18 fdra-Type-1-Gty-2-4-8-16-RBs-RIV-DCI-1-3-And-0-3-r18 priorityIndicationDL-r18 priorityIndicationUL-r18 dynamicIndicationSchedulingRestriction-r18 priorityIndicationOneSlotHARQ-r18 multiPUSCH-DCI-0-1-r18 multiPUSCH-DCI-0-2-r18 additionalSR-Periodicities-r18 pathlossRS-UpdateForType1CG-PUSCH-r18 bwp-SwitchingMultiDormancyCC-DCI-0-3-And-1-3-r18 | Added as * in cr_Phy_ParametersCommon |
| Phy-ParametersFRX-Diff | multipleCORESET-RedCap-r17 | Added as * in cr_NR_Phy_ParametersFRX_Diff |
| Phy-ParametersFR1 | k1-RangeExtensionATG-r18 maxHARQ-ProcessNumberATG-r18 uplinkPreCompensationATG-r18 uplinkTA-ReportingATG-r18 advReceiver-MU-MIMO-r18 deltaPowerClassReporting-r18 support12PRB-CORESET0-GSCN-41637-r18 support5MHz-ChannelBW-20PRB-CORESET0-r18 | Added as * in cr_NR_Phy_ParametersFR1 |
| Phy-ParametersFR2 | multiRxPreferenceIndication-r18 | Added as * in cr_NR_Phy_ParametersFR2 |
| RF-Parameters | supportedBandCombinationList-v1760 supportedBandCombinationList-UplinkTxSwitch-v1760 dummy1 dummy2 supportedBandCombinationList-v1780 supportedBandCombinationList-UplinkTxSwitch-v1780 supportedBandCombinationList-v1800 supportedBandCombinationList-UplinkTxSwitch-v1800 supportedBandCombinationListSL-U2U-Relay-r18 | Added as * in cr_NR_RF_Parameters |
| BandNR | posUE-TA-AutoAdjustment-r18 posSRS-ValidityAreaRRC-InactiveInitialUL-BWP-r18 posSRS-ValidityAreaRRC-InactiveOutsideInitialUL-BWP-r18 dl-PRS-MeasurementWithRxFH-RRC-ConnectedForRedCap-r18 posSRS-TxFH-RRC-ConnectedForRedCap-r18 posSRS-TxFH-RRC-InactiveForRedCap-r18 posSRS-BWA-RRC-Inactive-r18 posJointTriggerBySingleDCI-RRC-Connected-r18 dl-PRS-MeasurementWithRxFH-RRC-InactiveforRedCap-r18 dl-PRS-MeasurementWithRxFH-RRC-IdleforRedCap-r18 spatialAdaptation-CSI-Feedback-r18 maxNumberTotalCSI-ResourcePerCC-r18 spatialAdaptation-CSI-FeedbackPUSCH-r18 spatialAdaptation-CSI-FeedbackAperiodic-r18 spatialAdaptation-CSI-FeedbackPUCCH-r18 powerAdaptation-CSI-Feedback-r18 powerAdaptation-CSI-FeedbackPUSCH-r18 powerAdaptation-CSI-FeedbackAperiodic-r18 powerAdaptation-CSI-FeedbackPUCCH-r18 nes-CellDTX-DRX-r18 nes-CellDTX-DRX-DCI2-9-r18 simultaneousCSI-SubReportsPerCC-r18 ntn-DMRS-BundlingNGSO-r18 ltm-BeamIndicationJointTCI-r18 ltm-MAC-CE-JointTCI-r18 ltm-BeamIndicationSeparateTCI-r18 ltm-MAC-CE-SeparateTCI-r18 rach-EarlyTA-Measurement-r18 ue-TA-Measurement-r18 ta-IndicationCellSwitch-r18 triggeredHARQ-CodebookRetxDCI-1-3-r18 unifiedJointTCI-MultiMAC-CE-IntraCell-r18 unifiedSeparateTCI-MultiMAC-CE-IntraCell-r18 multiPUSCH-CG-r18 multiPUSCH-ActiveConfiguredGrant-r18 jointReleaseDCI-r18 cg-PUSCH-UTO-UCI-Ind-r18 pdcch-MonitoringResumptionAfterUL-NACK-r18 support3MHz-ChannelBW-Symmetric-r18 support3MHz-ChannelBW-Asymmetric-r18 support12PRB-CORESET0-r18 nr-PDCCH-OverlapLTE-CRS-RE-r18 nr-PDCCH-OverlapLTE-CRS-RE-MultiPatterns-r18 nr-PDCCH-OverlapLTE-CRS-RE-Span-3-4-r18 twoRateMatchingEUTRA-CRS-patterns-3-4-r18 overlapRateMatchingEUTRA-CRS-Patterns-3-4-Diff-CS-Pool-r18 ncd-SSB-BWP-Wor-r18 rlm-BM-BFD-CSI-RS-OutsideActiveBWP-r18 prach-CoverageEnh-r18 prach-Repetition-r18 dynamicWaveformSwitch-r18 dynamicWaveformSwitchPHR-r18 dynamicWaveformSwitchIntraCA-r18 multiPUSCH-SingleDCI-NonConsSlots-r18 pdc-maxNumberPRS-ResourceProcessedPerSlot-r18 intraSlot-PDSCH-MulticastInactive-r18 multicastInactive-r18 thresholdBasedMulticastResume-r18 lowerMSD-r18 lowerMSD-ENDC-r18 enhancedChannelRaster-r18 fastBeamSweepingMultiRx-r18 beamSweepingFactorReduction-r18 simultaneousReceptionTwoQCL-r18 measEnhCAInterFreqFR2-r18 tci-StateSwitchInd-r18 antennaArrayType-r18 locationBasedCondHandoverATG-r18 maxOutputPowerATG-r18 ltm-FastProcessingConfig-r18 measValidationReportEMR-r18 measValidationReportReselectionMeasurements-r18 eventA4BasedCondHandoverNES-r18 nesBasedCondHandoverWithDCI-r18 rach-LessHandoverCG-r18 rach-LessHandoverDG-r18 locationBasedCondHandoverEMC-r18 mt-CG-SDT-r18 posSRS-PreconfigureRRC-InactiveInitialUL-BWP-r18 posSRS-PreconfigureRRC-InactiveOutsideInitialUL-BWP-r18 cg-SDT-PeriodicityExt-r18 supportOf2RxXR-r18 condHandoverWithCandSCG-change-r18 | No impact |
| RF-ParametersMRDC | dummy1 dummy2 supportedBandCombinationList-v1780 supportedBandCombinationListNEDC-Only-v1780 supportedBandCombinationList-UplinkTxSwitch-v1780 supportedBandCombinationList-v1790 supportedBandCombinationList-UplinkTxSwitch-v1790 supportedBandCombinationList-v1800 supportedBandCombinationList-UplinkTxSwitch-v1800 | Added as * in cr_RF_ParametersMRDC |
| RLC-Parameters | am-WithLongSN-NCR-r18 | Added as * in cr_NR_RLC_Parameters |
| SDAP-Parameters | sdap-QOS-NCR-r18 sdap-HeaderNCR-r18 | No impact |
| SidelinkParametersNR-r16 | pdcp-ParametersSidelink-r18 sl-PRS-CommonProcCapabilityPerUE-r18 | Added as * in cr_SidelinkParametersNR_r16 |
| MAC-ParametersSidelinkCommon-r16 | sl-LBT-FailureDectectionRecovery-r18 | No impact |
| BandSidelink-r16 | sharedSpectrumChAccessParamsSidelinkPerBand-r18 sl-PRS-CommonProcCapabilityPerBand-r18 sl-PRS-RxInSharedResourcePool-r18 sl-PRS-RxInDedicatedResourcePool-r18 sl-PRS-TxInSharedResourcePool-r18 sl-PRS-TxScheme1InDedicatedResourcePool-r18 sl-PRS-TxScheme2InDedicatedResourcePool-r18 sl-PRS-CongestionCtrl-r18 sl-PRS-TxRandomSelection-r18 sl-PRS-TxUsingFullSensing-r18 sl-PRS-RxForBandWithSL-CA-r18 sl-PRS-TxForBandWithSL-CA-r18 sl-DynamicSharingTxRx-r18 sl-CA-Communication-r18 sl-ReceptionIntraCarrierGuardBand-r18 | No impact |
| RelayParameters-r17 | relayUE-U2U-OperationL2-r18 remoteUE-U2U-OperationL2-r18 remoteUE-U2N-PathSwitchOperationL2-r18 multipathRemoteUE-PC5L2-r18 multipathRelayUE-N3C-r18 multipathRemoteUE-N3C-r18 remoteUE-IndirectPathAddChangeToIdleInactiveRelay-r18 pdcp-DuplicationMoreThanOneUuRLC-r18 pdcp-CADuplicationDirectpath-DRB-r18 pdcp-CADuplicationDirectpath-SRB-r18 pdcp-DuplicationMP-SplitDRB-r18 pdcp-DuplicationMP-SplitSRB-r18 directpathRLF-RecoveryViaSRB1-r18 splitDRB-WithUL-BothDirectIndirect-r18 | No impact |
| SON-Parameters-r16 | cef-ReportRedCap-r17 rlf-ReportRedCap-r17 spr-Report-r18 successIRAT-HO-Report-r18 | Added as * in cr_UE_NR_Capability_v1610_IEs |
| UE-BasedPerfMeas-Parameters-r16 | loggedMDT-PNI-NPN-r18 loggedMDT-SNPN-r18 | Added as * in cr_UE_NR_Capability_v1610_IEs |
| UE-CapabilityRequestFilterCommon | lowerMSDRequest-r18 | No impact |
| AppLayerMeasConfig-r17 | rrc-SegAllowedSRB5-r18 idleInactiveReportAllowed-r18 | No impact |
| MeasConfigAppLayer-r17 | reportingSRB-r18 appLayerMeasPriority-r18 appLayerIdleInactiveConfig-r18 | No impact |
| RAN-VisibleParameters-r17 | ran-VisibleReportingSRB-r18 | No impact |
| UE-MeasurementsAvailable-r16 | successPSCell-InfoAvailable-r18 | Added as * in cr_UE_MeasurementsAvailable_r16 |
| SL-BWP-Config-r16 | sl-BWP-PoolConfigA2X-r18 sl-BWP-PRS-PoolConfig-r18 | Added as omit in cs_38508_SL_BWP_Config cs_SL_BWP_Config_Exceptional |
| SL-BWP-Generic-r16 | sl-Unlicensed-r18 | Added as omit in cs_SL_BWP_GenericDef |
| SL-BWP-ConfigCommon-r16 | sl-BWP-PoolConfigCommonA2X-r18 | Added as omit in cs_38508_SL_BWP_ConfigCommon |
| SL-ConfigDedicatedNR-r16 | sl-DiscConfig-v1800 | Added as omit in cs_SL_ConfigDedicatedNR_CSI_REPORT cs_SL_ConfigDedicatedNR_Def |
| SL-PHY-MAC-RLC-Config-v1700 | sl-RLC-BearerToAddModListSizeExt-v1800 sl-RLC-BearerToReleaseListSizeExt-v1800 sl-FreqInfoToAddModListExt-v1800 sl-LBT-SchedulingRequestId-r18 sl-SyncFreqList-r18 sl-SyncTxMultiFreq-r18 sl-MaxTransPowerCA-r18 sl-SCCH-CarrierSetConfig-r18 sl-PRS-SchedulingRequestId-r18 | No impact |
| SL-ConfiguredGrantConfig-r16 | sl-StartRBsetCG-Type1-r18 | No impact |
| SL-FreqConfigCommon-r16 | sl-UnlicensedFreqConfigCommon-r18 sl-PosBWP-List-r18 | Added as omit in cs_38508_SL_FreqConfigCommon |
| SL-LogicalChannelConfig-r16 | sl-ChannelAccessPriority-r18 sl-AllowedCarriers-r18 | Added as omit in cs_38508_SL_LogicalChannelConfig_Def |
| SL-L2RemoteUE-Config-r17 | sl-U2U-RelayUE-ToAddModList-r18 sl-U2U-RelayUE-ToReleaseList-r18 | No impact |
| SL-ResourcePool-r16 | sl-CPE-StartingPositionsPSCCH-PSSCH-InitiateCOT-List-r18 sl-CPE-StartingPositionsPSCCH-PSSCH-InitiateCOT-Default-r18 sl-CPE-StartingPositionsPSCCH-PSSCH-WithinCOT-List-r18 sl-CPE-StartingPositionsPSCCH-PSSCH-WithinCOT-Default-r18 sl-Type1-LBT-BlockingOption1-r18 sl-Type1-LBT-BlockingOption2-r18 sl-NumInterlacePerSubchannel-r18 sl-NumReferencePRBs-OfInterlace-r18 sl-TransmissionStructureForPSFCH-r18 sl-NumDedicatedPRBs-ForPSFCH-r18 sl-NumPSFCH-Occasions-r18 sl-PSFCH-CommonInterlaceIndex-r18 sl-CPE-StartingPositionPSFCH-r18 sl-NumRefSymbolLength-r18 sl-PSFCH-RB-SetList-r18 sl-IUC-RB-SetList-r18 sl-PSFCH-PowerOffset-r18 sl-RBSetIndexOfResourcePool-r18 sl-A2X-Service-r18 sl-PRS-ResourcesSharedSL-PRS-RP-r18 numSym-SL-PRS-2ndStageSCI-r18 sl-SCI-based-SL-PRS-Tx-Trigger-SCI2-D-r18 | Added as omit in cs_SL_ResourcePoolDef |
| SL-UE-SelectedConfigRP-r16 | sl-NRPSSCH-EUTRA-ThresRSRP-List-r18 sl-NRPSFCH-EUTRA-ThresRSRP-List-r18 | Added as omit in cs_38508_SL_UE_SelectedConfigRP |
| SL-RLC-BearerConfig-r16 | sl-RLC-BearerConfigIndex-v1800 | Added as omit in cs_38508_SL_RLC_BearerConfig_Def |
| SL-ScheduledConfig-r16 | sl-ConfiguredGrantConfigDedicated-SL-PRS-RP-List-r18 sl-PRS-RNTI-r18 sl-PRS-CS-RNTI-r18 | Added as omit in cs_SL_ScheduledConfig |
| SL-UE-SelectedConfig-r16 | sl-CBR-CommonTxDedicatedSL-PRS-RP-List-r18 | Added as omit cs_38508_SL_UE_SelectedConfig cs_Preconfig_SL_UE_SelectedConfig |

### Further ASN.1 changes

| ASN.1 type affected | Type change | Impact on TTCN |
| --- | --- | --- |
| PosSIB-ReqInfo-r16 | New enumerated values | No impact |
| SystemInformation-IEs | New choice values | No impact |
| PosSystemInformation-r16-IEs | New choice values | No impact |
| GNSS-ID-r16 | New choice values | No impact |
| MeasObjectToAddMod | New choice | No impact |
| MeasResultsSL-r16 | New choice | No impact |
| EventTriggerConfigInterRAT | New choice | No impact |
| ReportConfigNR | New choice | No impact |
| CondTriggerConfig-r16 | New choice | No impact |
| EventTriggerConfig | New choice | No impact |
| CA-BandwidthClassNR | New choice | No impact |

## NR_SideLink_Preconf

### Fields added with extension marker (ellipsis)

| ASN.1 type affected | New fields | Impact on TTCN |
| --- | --- | --- |
| SidelinkPreconfigNR-r16 | sl-PreconfigFreqInfoListSizeExt-v1800 sl-RLC-BearerConfigListSizeExt-v1800 sl-SyncFreqList-r18 sl-SyncTxMultiFreq-r18 sl-PreconfigDiscConfig-v1800 sl-PosPreconfigFreqInfoList-r18 | Fields added as omit in cs_SL_PreconfigurationNR |

## NR_PC5_RRC_Definitions

### Type change of nonCriticalExtension

| ASN.1 type affected | New nonCriticalExtension type |
| --- | --- |
| NotificationMessageSidelink-r17-IEs | NotificationMessageSidelink-v1800-IEs |
| RemoteUEInformationSidelink-r17-IEs | RemoteUEInformationSidelink-v1800-IEs |
| RRCReconfigurationSidelink-v1700-IEs | RRCReconfigurationSidelink-v1800-IEs |
| UECapabilityInformationSidelink-v1700-IEs | UECapabilityInformationSidelink-v1800-IEs |
| UuMessageTransferSidelink-r17-IEs | UuMessageTransferSidelink-v1800-IEs |

no changes in TTCN

### Name changes

| ASN.1 type affected | Renaming | Impact on TTCN |
| --- | --- | --- |
| MeasurementReportSidelink-r16-IEs | sl-measResults-r16 renamed to sl-MeasResults-r16 | cr_MeasurementReportSidelink_Def_TX |
| AccessStratumReleaseSidelink-r16 | spare6 renamed to rel18 | f_NR_SL_FullCapabilityCheck cr_UECapabilityInformationSidelink_r17 cr_UECapabilityInformationSidelink_r18 cr_UECapabilityInformationSidelink_v1700_IEs cr_UECapabilityInformationSidelink_v1800_IEs |

### Fields added with extension marker (ellipsis)

| ASN.1 type affected | New fields | Impact on TTCN |
| --- | --- | --- |
| SL-MeasResult-r16 | sl-Result-SL-PRS-r18 | Field added as * in cr_MeasurementReportSidelink_Def_TX |
| SL-MeasQuantityResult-r16 | sl-RSRP-DedicatedSL-PRS-RP-r18 | Field added as omit in cr_MeasurementReportSidelink_Def_TX |
| SL-MeasResultRelay-r17 | sl-MeasQuantity-r18 sl-RelayIndicationMP-r18 | No impact |
| SL-LogicalChannelConfigPC5-r16 | sl-LogicalChannelIdentity-v1800 | Field added as omit in cs_SL_LogicalChannelConfigPC5 |
| PDCP-ParametersSidelink-r16 | pdcp-DuplicationSRB-sidelink-r18 pdcp-DuplicationDRB-sidelink-r18 | Fields added as omit in cs_UECapabilityInformationSidelink_Def cr_PDCP_ParametersSidelink_r16 cs_38508_UECapabilityInformationSidelink_TwoWayEnquiryAndRx |
| BandSidelinkPC5-r16 | sl-PathlossBasedOLPC-SL-RSRP-Report-r18 sl-UE-COT-Sharing-r18 sl-PSFCH-MultiContiguousRB-r18 sl-PSFCH-MultiNonContiguousRB-r18 sl-CA-Communication-r18 sl-ReceptionIntraCarrierGuardBand-r18 sl-PowerClassUnlicensed-r18 | Fields added as omit in cs_38508_SupportedBandListSidelink_r16 |

### Further ASN.1 changes

| ASN.1 type affected | Type change | Impact on TTCN |
| --- | --- | --- |
| SCCH-MessageType | New choice values added | No impact |
| SL-SIB-ReqInfo-r17 | New enumerated values | No impact |

# Baseline Moving – 38.212 38.214 (PHY)

38.212 is upgraded to 38.212-i30.

38.214 is upgraded to 38.214-i30.

See impact on ASPs in clause 12.

# Baseline Moving – 38.321 (MAC)

38.321 is upgraded to 38.321-i20.

See impact on ASPs in clause 12.

# Baseline Moving – 38.322 (RLC)

38.322 is upgraded to 38.322-i10.

No change is required in TTCN.

# Baseline Moving – 38.323 (PDCP)

38.323 is upgraded to 38.323-i20.

New types required for SN gap report

type B3_Type NR_PDCP_CtrlPduType_Type;   /* PDU type according to TS 38.323 clause 6.3.8:

* 000  PDCP status report

* 001  Interspersed ROHC feedback

* 010  EHC feedback

* 011  UDC feedback

* 100  SN gap report

* 101-111   Reserved */

type record NR_PDCP_Ctrl_PDCP_SN_gap_report_Type { /* TS 38.323 clause 5.16 */

B1_Type                             D_C,         /* 1 bit,  '0'B for Ctrl PDU */

NR_PDCP_CtrlPduType_Type            PDU_Type,    /* 3 bits, '100'B for PDCP SN gap report */

B4_Type                             Reserved,    /* 4 bits reserved */

B32_Type                            FDC,         /* 32 bits according to TS 38.323, clause 6.3.15 */

octetstring                         DiscardBitmap optional  /* Discard Bitmap according to TS 38.323, clause 6.3.16 */

};

type union NR_PDCP_PDU_Type {

NR_PDCP_DataPduSN12Bits_Type                DataPduSN12Bits,

NR_PDCP_DataPduSN18Bits_Type                DataPduSN18Bits,

NR_PDCP_CtrlPduStatus_Type                  CtrlPduStatus,

NR_PDCP_CtrlPduRohcFeedback_Type            CtrlPduRohcFeedback,

NR_PDCP_CtrlPduEhcFeedbackCID7Bits_Type     CtrlPduEhcFeedbackCID7Bits,

NR_PDCP_CtrlPduEhcFeedbackCID15Bits_Type    CtrlPduEhcFeedbackCID15Bits,

NR_PDCP_CtrlPduUdcFeedback_Type             CtrlPduUdcFeedback,

NR_PDCP_DataPduSN12Bits_UDC_Type            DataPduSN12Bits_UDC,

NR_PDCP_DataPduSN18Bits_UDC_Type            DataPduSN18Bits_UDC,

NR_PDCP_Ctrl_PDCP_SN_gap_report_Type        Ctrl_PDCP_SN_gap_report

};

# Baseline Moving – 37.324 (SDAP)

37.324 is upgraded to 37.324-i00.

No change is required in TTCN.

# Baseline Moving – 24.501 (NAS)

24.501 is upgraded to 24.501-i70.

## Modified Message Definitions

| PDU Definition | New optional fields | Ref: 3GPP 24.501 clause | Impact on TTCN |
| --- | --- | --- | --- |
| Authentication Result | AUN3 device security key | 8.2.3 | Added as omit in cs_NG_AUTHENTICATION_RESULT |
| Registration Request | Unavailability information, Non-3GPP path switching information, AUN3 indication | 8.2.6 | Added as * in f_Check_NG_RegistrationReqMsg |
| Registration Accept | Equivalent SNPNs, NID, Registration accept type 6 IE container, RAN timing synchronization, Alternative NSSAI, Maximum time offset, S-NSSAI time validity information, Unavailability configuration, Feature authorization indication, On-demand NSSAI | 8.2.7 | Added as omit in f_Get_NG_RegistrationAcceptMsg |
| Registration Reject | N3IWF identifier, TNAN information, Extended 5GMM Cause | 8.2.9 | Added as omit in cs_NG_REGISTRATION_REJECT |
| UL NAS Transport | Non-3GPP access path switching indication, Alternative S-NSSAI, Payload container information | 8.2.10 | Added as * in f_Check_NG_ULNASTransport |
| Deregistration Request MO | Unavailability information, NAS message container NOTE: Unavailability information is a non-cleartext IE | 8.2.12 | Added checks for initial NAS message in f_Check_NG_DeregistrationReqMOMsg |
| Configuration Update Command | RAN timing synchronization, Extended LADN information, Alternative NSSAI, S-NSSAI location validity information, S-NSSAI time validity information, Maximum time offset, Partially allowed NSSAI, Partially rejected NASSAI, Feature authorization indication, On-demand NSSAI | 8.2.19 | Added as omit in cs_NG_CONFIGURATION_UPDATE_COMMAND |
| Security Mode Command | AUN3 device security key | 8.2.25 | Added as omit in f_Get_NG_SecurityModeCmdMsg |
| PDU Session Establishment Request | URSP rule enforcement reports | 8.3.1 | Added as * in f_Check_NG_PDUSessionEstablishmentReq |
| PDU Session Establishment Accept | N3QAI, Protocol description | 8.3.2 | Added as omit in f_Get_NG_PDUSessionEstablishmentAccept |
| PDU Session Modification Request | Non-3GPP delay budget, URSP rule enforcement reports | 8.3.7 | Added as * in f_Check_NG_PDUSessionModificationReq |
| PDU Session Modification Command | Alternative S-NSSAI, N3QAI, Protocol description | 8.3.9 | Added as omit in f_Get_NG_PDUSessionModificationCommand |
| PDU Session Release Command | Alternative S-NSSAI | 8.3.14 | Added as omit in f_Get_NG_PDUSessionReleaseCommand |
| Manage UE Policy Command | UE policy network classmark, VPS URSP configuration | D.5.1 | Added as omit in cs_MANAGE_UE_POLICY_COMMAND |

## Modified Information Elements

| PDU Definition | Changes Required | Ref: 3GPP 24.501 clause | Impact on TTCN |
| --- | --- | --- | --- |
| 5GMM Capability | New optional fields: Spare, ESI, ECI, RAN timing, LADN-DS, NSR, SBTS, A2XEPC5, A2XNPC5, UN-PER, SBNS, RSLPS, 5G-Prose-L2U2U relay, 5G-Prose-L3U2U relay, 5G-Prose-L2end, 5G-Prose-L3end, RSLP, PNS, LCS-UPP, SUPL, TempNS, SLVI, A2X-Uu, MCSIU, NVL-SATNR, RSLPL, NSUC | 9.11.3.1 | Added as * in cr_NG_GMM_CapAny |
| 5GS Network Feature Support | New optional fields: UN-PER, RSLP, SUPL, LCS-UPP, NAPS | 9.11.3.5 | Added as ‘0’B and omit in cs_NG_NetworkFeatureSupport_Def |
| Ciphering Key Data | New optional fields: PossSIB Type1-9, PossSIB Type1-10, PossSIB Type2-24, PossSIB Type2-25 | 9.11.3.18C |  |
| SOR Transparent Container | New optional field: SOR-SNPN-SI-LS Restructured field: SOR header NOTE: structured types for the following alternatives for ‘data type 0, list type 1 and additional information’ now defined: PLMN ID & Access Technology List, SOR-CMCI, SOR-SNPN-SI, SOR-SNPN-SI-LS | 9.11.3.51 |  |
| Priority Indicator | Renamed field: spare to MCSI | 9.11.3.91 |  |
| 5GSM Capability | Renamed field: spare to SDNAEPC | 9.11.4.1 |  |
| 5GSM Network Feature Support | Renamed field: spare to NAPS | 9.11.4.18 |  |
| Received MBS Container | Renamed field: spare to IPAT | 9.11.4.31 |  |
| UE Policy Classmark | Renamed fields: spare to SupportRURE, SVPSU, EPSURSP | D.6.5 |  |

## New Information Elements

| PDU Definition | Ref: 3GPP 24.501 clause |
| --- | --- |
| SNPN List | 9.11.3.92 |
| N3IWF Identifier | 9.11.3.93 |
| TNAN information | 9.11.3.94 |
| RAN timing synchronisation | 9.11.3.95 |
| Extended LADN information | 9.11.3.96 |
| Alternative NSSAI | 9.11.3.97 |
| Type 6 IE container | 9.11.3.98 |
| Non-3GPP access path switching information | 9.11.3.99 |
| S-NSSAI location validity information | 9.11.3.100 |
| S-NSSAI time validity information | 9.11.3.101 |
| Non-3GPP path switching information | 9.11.3.102 |
| Partial NSSAI | 9.11.3.103 |
| AUN3 indication | 9.11.3.104 |
| Feature authorization indication | 9.11.3.105 |
| Payload container information | 9.11.3.106 |
| AUN3 device security key | 9.11.3.107 |
| On-demand NSSAI | 9.11.3.108 |
| Extended 5GMM Cause | 9.11.3.109 |
| N3QAI | 9.11.4.36 |
| Non-3GPP delay budget | 9.11.4.37 |
| URSP rule enforcement reports | 9.11.4.38 |
| Protocol description | 9.11.4.39 |
| UE policy network classmark | D.6.7 |
| VPS URSP configuration | D.6.8 |

# Baseline Moving – 24.526 (V2X)

24.526 is upgraded to 24.526-i70.

## Modified Information Elements

| PDU Definition | New Optional Field | Ref: 3GPP 24.526 Figure |
| --- | --- | --- |
| URSP Rule | Additional Indications | 5.2.2 |

# Baseline Moving – 24.587 (V2X)

24.587 is upgraded to 24.587-i60.

## Modified Message Definitions

| PDU Definition | New Optional Fields | Ref: 3GPP 24.587 Clause | Impact on TTCN |
| --- | --- | --- | --- |
| Direct Link Establishment Request | RSPP metadata NOTE: 24.587 v i.6.0 specifies: IEI= X1. Have implemented 50 in TTCN TLV-E, length 4-n. Have implemented TLV, length 3-4 in TTCN | 7.3.1 | Added as * in cr_DirectLinkEstablishmentRequestTX And omit in cs_DirectLinkEstablishmentRequestRX |
| Direct Link Establishment Accept | RSPP metadata NOTE: 24.587 v i.6.0 specifies TLV, length 3-4 | 7.3.2 | Added as * in cr_DirectLinkEstablishmentAcceptTX And omit in cs_DirectLinkEstablishmentAcceptRX |

## Modified Information Elements

| PDU Definition | New Fields | Ref: 3GPP 24.587 Clause | Impact on TTCN |
| --- | --- | --- | --- |
| Requested UE Policies | RSPI, 5P2EUI, 5P3EUI, 5P2UURI, 5P3UURI, A2XI, spare | 8.3.2 | Added as * in cr_RequestedUEPolicies_V2XPC5 |

# Baseline Moving – 24.588 (V2X)

24.588 is upgraded to 24.588-i20.

## Modified Information Elements

| PDU Definition | New Optional Fields | Ref: 3GPP 24.588 Figure |
| --- | --- | --- |
| PLMN Info | New fields: VAMCI, V2X AS MBS configuration | 5.4.1.3 |
| V2X Service Identifier Unrelated Info | New fields: VMCI, V2X MBS Configurations | 5.4.1.6 |
| V2X Service Info | New fields: VMCI, V2X MBS Configurations | 5.4.1.11 |

## New Information Elements

| PDU Definition | Ref: 3GPP 24.588 Figure |
| --- | --- |
| V2X MBS Configurations | 5.4.1.8A |
| V2X MBS Configuration | 5.4.1.8B |
| Frequency Information | 5.4.1.8C |
| FSA IDs Information | 5.4.1.8D |
| V2X MBS SDP Body Information | 5.4.1.8E |

# Baseline Moving – 38.509 (TMC)

38.509 has remained as 38.509-h50 (Rel-17 Mar’24).

No change is required in TTCN.

# Other changes

## Function f_NR_RRCRelease

Removed input parameter p_SuspendConfig in function f_NR_RRCRelease.

Impact on functions f_TC_11_7_2_NR5GC_TestBody, fl_TC_8_1_1_1_2_TestBody, f_TC_8_1_1_4_y_NR5GC_TestBody and fl_TC_9_1_12_4_Body to align with f_NR_RRCRelease.

Example of function f_TC_11_7_2_NR5GC_TestBody in module EDRX_NR5GC:

f_NR_RRCRelease_Common(nr_Cell1, cs_38508_RRCRelease (tsc_NR_RRC_TI_Def,v_SuspendConfig)); // @sic R5s240445 sic@

## Changes for Audit test cases

Module NR_CapabilityFunctions

template UE_NR_Capability_v1740 cr_UE_NR_Capability_v1740_IEs := // @sic R5s240445 sic@

{ /* @status    APPROVED (ENDC, NR5GC) */

redCapParameters_v1740 := ?, //redCapParameters_v1740 non optional

nonCriticalExtension   := {

crossCarrierSchedulingConfigurationRelease_r17 := *,

nonCriticalExtension := {

airToGroundNetwork_r18                   := *,

eRedCapParameters_r18                    := *,

ncr_Parameters_r18                       := *, // NCR-Parameters-r18

softSatelliteSwitchResyncNTN_r18         := *,

hardSatelliteSwitchResyncNTN_r18         := *,

mt_SDT_r18                               := *,

mt_SDT_NTN_r18                           := *,

inDeviceCoexIndAutonomousDenial_r18      := *,

inDeviceCoexIndFDM_r18                   := *,

inDeviceCoexIndTDM_r18                   := *,

musim_GapPriorityPreference_r18          := *,

musim_CapabilityRestriction_r18          := *,

multiRx_FR2_Preference_r18               := *,

ra_InsteadCG_SDT_r18                     := *,

resumeAfterSDT_Release_r18               := *,

ul_TrafficInfo_r18                       := *,

aerialParameters_r18                     := *,  // AerialParameters-r18

ntn_VSAT_AntennaType_r18                 := *,

ntn_VSAT_MobilityType_r18                := *,

ntn_Parameters_v1820                     := *, // NTN-Parameters-v1820

nonCriticalExtension                     := * // left for future extensions  > UE_NR_Capability_v180

} ifpresent  // UE_NR_Capability_v180

} ifpresent // UE_NR_Capability_v1750

} ifpresent; // UE_NR_Capability_v1740

template UE_NR_Capability_v1740 cr_UE_NR_Capability_v1740_IEs_RedCap := // @sic R5s240445 sic@

{ /* @status    APPROVED (ENDC, NR5GC) */

redCapParameters_v1740 := { //redCapParameters_v1740 non optional

ncd_SSB_ForRedCapInitialBWP_SDT_r17 := *

},

nonCriticalExtension   := {

crossCarrierSchedulingConfigurationRelease_r17 := *,

nonCriticalExtension := {

airToGroundNetwork_r18                   := *,

eRedCapParameters_r18                    := *, // ERedCapParameters-r18

ncr_Parameters_r18                       := *, // NCR-Parameters-r18

softSatelliteSwitchResyncNTN_r18         := *,

hardSatelliteSwitchResyncNTN_r18         := *,

mt_SDT_r18                               := *,

mt_SDT_NTN_r18                           := *,

inDeviceCoexIndAutonomousDenial_r18      := *,

inDeviceCoexIndFDM_r18                   := *,

inDeviceCoexIndTDM_r18                   := *,

musim_GapPriorityPreference_r18          := *,

musim_CapabilityRestriction_r18          := *,

multiRx_FR2_Preference_r18               := *,

ra_InsteadCG_SDT_r18                     := *,

resumeAfterSDT_Release_r18               := *,

ul_TrafficInfo_r18                       := *,

aerialParameters_r18                     := *,  // AerialParameters-r18

ntn_VSAT_AntennaType_r18                 := *,

ntn_VSAT_MobilityType_r18                := *,

ntn_Parameters_v1820                     := *, // NTN-Parameters-v1820

nonCriticalExtension                     := * // left for future extensions  > UE_NR_Capability_v180

} ifpresent  // UE_NR_Capability_v180

} ifpresent // UE_NR_Capability_v1750

} ifpresent; // UE_NR_Capability_v1740

template UE_NR_Capability_v1640 cr_UE_NR_Capability_v1640_IEs_RedCap (template UE_NR_Capability_v1740 p_UE_NR_Capability_v1740 := *) :=

// @sic R5-231410 R5s211246 R5-225384 R5s221155 R5s221179 R5-227571 R5-227572 R5-227573 R5s230456 R5-233835 R5-240518 R5s240445 sic@

{ /* @status    APPROVED (ENDC, NR5GC) */

redirectAtResumeByNAS_r16                   := *,

phy_ParametersSharedSpectrumChAccess_r16    := cr_V1640_phy_ParametersSharedSpectrumChAccess_r16 ifpresent,

nonCriticalExtension                        := { // UE-NR-Capability-v1650

mpsPriorityIndication_r16     := f_Derive_NR_mpsPriorityIndication_r16() ifpresent,

highSpeedParameters_v1650     := *,

nonCriticalExtension          := {           // UE-NR-Capability-v1690

ul_RRC_Segmentation_r16   := f_Derive_ul_RRC_Segmentation_r16(),

nonCriticalExtension      := {           // UE-NR-Capability-v1700

inactiveStatePO_Determination_r17 := *,

highSpeedParameters_v1700         := *,

powSav_Parameters_v1700           := *,

mac_Parameters_v1700              := {

mac_ParametersFR2_2_r17   := {      // MAC-ParametersFR2-2-r17

directMCG_SCellActivation_r17       := *,  // @sic R5-232320 sic@

directMCG_SCellActivationResume_r17 := *,  // @sic R5-232320 sic@

directSCG_SCellActivation_r17       := *,

directSCG_SCellActivationResume_r17 := *,

drx_Adaptation_r17                  := *

} ifpresent

} ifpresent,

ims_Parameters_v1700              := *,

measAndMobParameters_v1700        := ?, //MeasAndMobParameters_v1700 non optional

appLayerMeasParameters_r17        := *,

redCapParameters_r17              := {

supportOfRedCap_r17         := f_Derive_supportOfRedCap_r17() ifpresent,         // pc_supportOfRedCap_r17 - only present in NR5GC and RedCap

supportOf16DRB_RedCap_r17   := f_Derive_supportOf16DRB_RedCap_r17() ifpresent    // pc_16DRB_RedCap_r17    - only present in NR5GC and RedCap

} ifpresent,

ra_SDT_r17                        := f_Derive_ra_SDT_r17() ifpresent,    // pc_ra_SDT_r17  - only present in NR5GC

srb_SDT_r17                       := f_Derive_srb_SDT_r17() ifpresent,   // pc_srb_SDT_r17 - only present in NR5GC

gNB_SideRTT_BasedPDC_r17          := *,

bh_RLF_DetectionRecovery_Indication_r17  := *,

nrdc_Parameters_v1700                    := *,

bap_Parameters_v1700                     := *,

musim_GapPreference_r17                  := f_Derive_musim_GapPreference_r17() ifpresent, // @sic R5-236307 sic@ - pc_musim_GapPreference_r17

musimLeaveConnected_r17                  := f_Derive_musimLeaveConnected_r17() ifpresent, // @sic R5-236307 sic@ - pc_musimLeaveConnected_r17

mbs_Parameters_r17                       := ?, //MBS_Parameters_r17 non optional

nonTerrestrialNetwork_r17                := *,

ntn_ScenarioSupport_r17                  := *,

sliceInfoforCellReselection_r17          := f_Derive_sliceInfoforCellReselection_r17() ifpresent, // @sic R5-232202 sic@

ue_RadioPagingInfo_r17                   := {

pei_SubgroupingSupportBandList_r17  := * // @sic R5-233835 sic@

} ifpresent,

ul_GapFR2_Pattern_r17                    := *,

ntn_Parameters_r17                       := *,

nonCriticalExtension                     := p_UE_NR_Capability_v1740  // UE_NR_Capability_v1740

} ifpresent         // UE_NR_Capability_v1700 IEs

} ifpresent             // UE_NR_Capability_v1690 IEs

} ifpresent                 // UE_NR_Capability_v1650 IEs

} ifpresent;                  // UE_NR_Capability_v1640 IEs

template UE_NR_Capability_v1640 cr_UE_NR_Capability_v1640_IEs (template UE_NR_Capability_v1740 p_UE_NR_Capability_v1740 := *) := // @sic R5-230111 R5-230112 R5s211246 R5-225384 R5s221155 R5s221179 R5-227571 R5-227572 R5-227573 R5-233835 R5s240445 sic@

{ /* @status    APPROVED (ENDC, NR5GC) */

redirectAtResumeByNAS_r16                   := *,

phy_ParametersSharedSpectrumChAccess_r16    := cr_V1640_phy_ParametersSharedSpectrumChAccess_r16 ifpresent,

nonCriticalExtension                        := { // UE-NR-Capability-v1650

mpsPriorityIndication_r16     := f_Derive_NR_mpsPriorityIndication_r16() ifpresent,

highSpeedParameters_v1650     := *,

nonCriticalExtension          := {           // UE-NR-Capability-v1690

ul_RRC_Segmentation_r16   := f_Derive_ul_RRC_Segmentation_r16(),

nonCriticalExtension      := {           // UE-NR-Capability-v1700

inactiveStatePO_Determination_r17 := *,

highSpeedParameters_v1700         := *,

powSav_Parameters_v1700           := *,

mac_Parameters_v1700              := {

mac_ParametersFR2_2_r17   := {      // MAC-ParametersFR2-2-r17

directMCG_SCellActivation_r17       := *, // @sic R5-232320 sic@

directMCG_SCellActivationResume_r17 := *, // @sic R5-232320 sic@

directSCG_SCellActivation_r17       := *,

directSCG_SCellActivationResume_r17 := *,

drx_Adaptation_r17                  := *

} ifpresent

} ifpresent,

ims_Parameters_v1700              := *,

measAndMobParameters_v1700        := ?, //MeasAndMobParameters_v1700 non optional

appLayerMeasParameters_r17        := *,

redCapParameters_r17              := *,

ra_SDT_r17                        := *,    // pc_ra_SDT_r17  - only present in NR5GC

srb_SDT_r17                       := *,    // pc_srb_SDT_r17 - only present in NR5GC

gNB_SideRTT_BasedPDC_r17          := *,

bh_RLF_DetectionRecovery_Indication_r17  := *,

nrdc_Parameters_v1700                    := *,

bap_Parameters_v1700                     := *,

musim_GapPreference_r17                  := f_Derive_musim_GapPreference_r17() ifpresent, // @sic R5-236307 sic@ - pc_musim_GapPreference_r17

musimLeaveConnected_r17                  := f_Derive_musimLeaveConnected_r17() ifpresent, // @sic R5-236307 sic@ - pc_musimLeaveConnected_r17

mbs_Parameters_r17                       := ?, //MBS_Parameters_r17 non optional

nonTerrestrialNetwork_r17                := *,

ntn_ScenarioSupport_r17                  := *,

sliceInfoforCellReselection_r17          := f_Derive_sliceInfoforCellReselection_r17() ifpresent,

ue_RadioPagingInfo_r17                   := {

pei_SubgroupingSupportBandList_r17  := * // @sic R5-233835 sic@ - Checked in fl_NRCap_CommonChecks_r17\f_PEI_SubgroupingSupportBandList_r17

} ifpresent,

ul_GapFR2_Pattern_r17                    := *,

ntn_Parameters_r17                       := *,

nonCriticalExtension                     := p_UE_NR_Capability_v1740  // UE-NR-Capability-v1740

} ifpresent         // UE_NR_Capability_v1700 IEs

} ifpresent             // UE_NR_Capability_v1690 IEs

} ifpresent                 // UE_NR_Capability_v1650 IEs

} ifpresent;                  // UE_NR_Capability_v1640 IEs

template (present) UE_NR_Capability cr_UeCapabilitiesRAT_Container_NR_r18 := // @sic R5s240445 sic@

/* @status    APPROVED (ENDC, NR5GC) */

cr_UE_NR_Capability(-,-, cr_UE_NR_Capability_v1530_IEs(cr_UE_NR_Capability_v1610_IEs(cr_UE_NR_Capability_v1640_IEs(cr_UE_NR_Capability_v1740_IEs))));

template (present) UE_NR_Capability cr_UeCapabilitiesRAT_Container_NR_r18_RedCap := // @sic R5s240445 sic@

/* @status    APPROVED (ENDC, NR5GC) */

cr_UE_NR_Capability(-,-, cr_UE_NR_Capability_v1530_IEs(cr_UE_NR_Capability_v1610_IEs(cr_UE_NR_Capability_v1640_IEs_RedCap(cr_UE_NR_Capability_v1740_IEs_RedCap))));

function f_NR_NRFullCapabilityCheck (UE_NR_Capability p_NRCapability,

charstring p_OutputText := "PICS for condition on (pc_nrFDD or pc_nrTDD) not set",

charstring p_OutputText2 := "Received UE Capability template does not match with the expected template",

NR_Configuration_Type p_Configuration_Type)

{ //@sic R5-194844 R5-225384 R5-227571 R5-227572 R5-227573 R5-231410 sic@

var UE_NR_Capability v_Received_NRCapability;

var template UE_NR_Capability v_NRCapability;

if ((not (pc_nrFDD or pc_nrTDD))){

FatalError(__FILE__, __LINE__, p_OutputText);

}

v_Received_NRCapability := p_NRCapability;

select (v_Received_NRCapability.accessStratumRelease) {

case (spare4, spare3, spare2, spare1) {  /* @sic R5s201387 BASELINE MOVING 2020 sic@ */

f_SetVerdictInconc (__FILE__, __LINE__, "AccessStratumRelease < rel15");

// a UE implementing relesase older than rel15 is not considered

}

case (rel15) { // @sic R5s200028 R5s200542 sic@

v_NRCapability := cr_UeCapabilitiesRAT_Container_NR_r15; // check on Rel 15 IEs vs. IEs provided

fl_NRCap_CommonChecks_r15(v_Received_NRCapability, p_Configuration_Type);

}

case (rel16) { /* @sic R5s201387 BASELINE MOVING 2020 sic@ */

v_NRCapability := cr_UeCapabilitiesRAT_Container_NR_r16; // check on Rel 16 IEs vs. IEs provided

fl_NRCap_CommonChecks_r16(v_Received_NRCapability, p_Configuration_Type);

}

case (rel17) { /* @sic R5s221179 BASELINE MOVING 2022 sic@ */

if (p_Configuration_Type == NR_SA) {

v_NRCapability := cr_UeCapabilitiesRAT_Container_NR_r17_RedCap; // check on Rel 17 IEs vs. IEs provided in UE-NR-Capability as per Table 8.1.5.1.1.3.3-3

}

else {

v_NRCapability := cr_UeCapabilitiesRAT_Container_NR_r17;        // check on Rel 17 IEs vs. IEs provided in UE-NR-Capability as per Tables 8.2.1.1.1.3.3-6 and Table 8.2.1.1.2.3.3-4

}

fl_NRCap_CommonChecks_r17(v_Received_NRCapability, p_Configuration_Type);

}

case (rel18) { /* @sic R5s240445 BASELINE MOVING 2024 sic@ */

if (p_Configuration_Type == NR_SA) {

v_NRCapability := cr_UeCapabilitiesRAT_Container_NR_r18_RedCap; // check on Rel 18 IEs vs. IEs provided in UE-NR-Capability as per Table 8.1.5.1.1.3.3-3

}

else {

v_NRCapability := cr_UeCapabilitiesRAT_Container_NR_r18;        // check on Rel 18 IEs vs. IEs provided in UE-NR-Capability as per Tables 8.2.1.1.1.3.3-6 and Table 8.2.1.1.2.3.3-4

}

fl_NRCap_CommonChecks_r17(v_Received_NRCapability, p_Configuration_Type);

}

case else  {

f_SetVerdictInconc (__FILE__, __LINE__, "AccessStratumRelease not applicable");

// a UE implementing an unsupported release must not be failed

}

}

// Match the received UE-NR-CapabilityInformation with the expected UE_NR_Capability template

fl_NR_UE_Capability_MatchTemplates(v_Received_NRCapability, v_NRCapability, p_OutputText2);

}

Module NR_CapabilityFunctions_MRDC

template UE_MRDC_Capability_v1700 cr_UE_MRDC_Capability_v1700_IEs(template UE_MRDC_Capability_v1800 p_UE_MRDC_Capability_v1800 := *)   := /* @sic R5s221179 BASELINE MOVING 2022 - R5s230533 BASELINE UPGRADE 2023 R5s240445 BASELINE MOVING 2024 rel-18 sic@ */

{  /* @status    APPROVED (ENDC) */

// @sic R5-233837 sic@

measAndMobParametersMRDC_v1700   := {

measAndMobParametersMRDC_Common_v1700  := {

condPSCellChangeParameters_r17 := {

inter_SN_condPSCellChangeFDD_TDD_NRDC_r17     := *,

inter_SN_condPSCellChangeFR1_FR2_NRDC_r17     := *,

inter_SN_condPSCellChangeFDD_TDD_ENDC_r17     := *,

inter_SN_condPSCellChangeFR1_FR2_ENDC_r17     := *,

mn_InitiatedCondPSCellChange_FR1FDD_ENDC_r17  := f_NR_MRDCCommon_mn_InitiatedCondPSCellChange_FR1FDD_ENDC_r17(),//See fl_Check_MRDCCommon_mn_InitiatedCondPSCellChange_FR1FDD_ENDC_r17

mn_InitiatedCondPSCellChange_FR1TDD_ENDC_r17  := f_NR_MRDCCommon_mn_InitiatedCondPSCellChange_FR1TDD_ENDC_r17(),//See fl_Check_MRDCCommon_mn_InitiatedCondPSCellChange_FR1TDD_ENDC_r17

mn_InitiatedCondPSCellChange_FR2TDD_ENDC_r17  := f_NR_MRDCCommon_mn_InitiatedCondPSCellChange_FR2TDD_ENDC_r17(),//See fl_Check_MRDCCommon_mn_InitiatedCondPSCellChange_FR2TDD_ENDC_r17

sn_InitiatedCondPSCellChange_FR1FDD_ENDC_r17  := *,

sn_InitiatedCondPSCellChange_FR1TDD_ENDC_r17  := *,

sn_InitiatedCondPSCellChange_FR2TDD_ENDC_r17  := *

} ifpresent,

condHandoverWithSCG_ENDC_r17   := *,

condHandoverWithSCG_NEDC_r17   := *

} ifpresent // MeasAndMobParametersMRDC-Common-v1700

},   // MeasAndMobParametersMRDC-v1700

nonCriticalExtension           := {

measAndMobParametersMRDC_v1730 := {

measAndMobParametersMRDC_Common_v1730 := {

independentGapConfig_maxCC_r17 := {

fr1_Only_r17    := *,

fr2_Only_r17    := *,

fr1_AndFR2_r17  := *

}

} ifpresent // MeasAndMobParametersMRDC-Common-v1730

} ifpresent, //MeasAndMobParametersMRDC-v1730

nonCriticalExtension := p_UE_MRDC_Capability_v1800 // UE_MRDC_Capability_v1800

} ifpresent // UE-MRDC-Capability-v1730

} ifpresent; //UE-MRDC-Capability-v1700

template UE_MRDC_Capability_v1800 cr_UE_MRDC_Capability_v1800_IEs   := /* R5s240445 BASELINE MOVING 2024 rel-18 sic@ */

{  /* @status    APPROVED (ENDC) */

requirementTypeIndication_r18         := *,

measAndMobParametersMRDC_v1810        := {

measAndMobParametersMRDC_Common_v1810  :=  {

mn_ConfiguredMN_TriggerSCPAC_r18     := *,

mn_ConfiguredSN_TriggerSCPAC_r18     := *,

sn_ConfiguredSCPAC_r18               := *,

mn_ConfiguredMN_TriggerSCPAC_afterSCG_release_r18 := *,

mn_ConfiguredReferenceConfigSCPAC_r18              := *,

sn_ConfiguredReferenceConfigSCPAC_r18              := *,

condHandoverWithCandSCG_Addition_r18               := *,

condHandoverWithCandSCG_FR1_FR2_Change_r18         := *,

condHandoverWithCandSCG_FDD_TDD_Change_r18         := *

} // MeasAndMobParametersMRDC-Common-v1810

},  // MeasAndMobParametersMRDC-v1810

nonCriticalExtension                  := * // extension for future releases > v1800

}

template (present) UE_MRDC_Capability cr_UeCapabilitiesRAT_Container_MRDC_r18 := /* R5s240445 BASELINE MOVING 2024 rel-18 sic@ */

/* @status    APPROVED (ENDC) */

cr_UE_MRDC_Capability(-, cr_UE_MRDC_Capability_v1560_IEs(cr_UE_MRDC_Capability_v1610_IEs(cr_UE_MRDC_Capability_v1700_IEs(cr_UE_MRDC_Capability_v1800_IEs))));:= *,

function fl_MRDCCap_CommonChecks_r18(UE_MRDC_Capability p_MRDC_Capability, NR_Configuration_Type p_Configuration_Type)

{

var UE_MRDC_Capability v_Received_MRDCCapabilityMsg := p_MRDC_Capability;

fl_MRDCCap_CommonChecks_r17(v_Received_MRDCCapabilityMsg, p_Configuration_Type);

} // fl_MRDCCap_CommonChecks_r18

function f_NR_MRDCFullCapabilityCheck (UE_MRDC_Capability p_MRDC_Capability, UE_NR_Capability.accessStratumRelease p_UE_AccessStratumRelease, NR_Configuration_Type p_Configuration_Type := EN_DC)

{

var UE_MRDC_Capability v_Received_MRDCCapability;

var template UE_MRDC_Capability v_MRDCCapability;

if (p_Configuration_Type == EN_DC) // Option 3

{

if ((not (pc_EN_DC))){

FatalError(__FILE__, __LINE__, "Table 8.2.1.1.1.3.3-7: PICS for condition on pc_EN_DC not set");

}

}

else if (p_Configuration_Type == NE_DC) // Option 4

{

if ((not (pc_NE_DC))){

FatalError(__FILE__, __LINE__, "Table 8.2.1.1.2.3.3-5: PICS for condition on pc_NE_DC not set");

}

}

else {

f_SetVerdictInconc (__FILE__, __LINE__, "Selected NR Configuration Type is not applicable");

}

v_Received_MRDCCapability := p_MRDC_Capability;

select (p_UE_AccessStratumRelease) {

case (rel15) {

v_MRDCCapability := cr_UeCapabilitiesRAT_Container_MRDC_r15;

fl_MRDCCap_CommonChecks_r15(v_Received_MRDCCapability, p_Configuration_Type); // to check some Rel 15 MRDC_Cap contents

}

case (rel16) {

v_MRDCCapability := cr_UeCapabilitiesRAT_Container_MRDC_r16;

fl_MRDCCap_CommonChecks_r16(v_Received_MRDCCapability, p_Configuration_Type);

}

case (rel17) {

v_MRDCCapability := cr_UeCapabilitiesRAT_Container_MRDC_r17;

fl_MRDCCap_CommonChecks_r17(v_Received_MRDCCapability, p_Configuration_Type);

}

case (rel18) { // @sic R5s240445 BASELINE MOVING 2024 rel-18 sic@

v_MRDCCapability := cr_UeCapabilitiesRAT_Container_MRDC_r18;

fl_MRDCCap_CommonChecks_r18(v_Received_MRDCCapability, p_Configuration_Type);

}

case else  {

f_SetVerdictInconc (__FILE__, __LINE__, "AccessStratumRelease not applicable");

// UE with unsupported release is not failed. Anyway it will never reach this point if it is not an expected UE Release

}

}

// Match the received UE-NR-CapabilityInformation with the expected UE_NR_Capability template

fl_EUTRANR_UE_Capability_MatchTemplates(v_Received_MRDCCapability, v_MRDCCapability);

}

Module NR_CommonDefs

function f_NR_CheckReleaseIndicator(AccessStratumRelease p_ReleaseIndicator_UE)

{

var integer v_RelMin;

var integer v_Rel;

var template (omit) integer v_UE_Release := f_NR_ConvertAccessStratumReleaseToInteger(p_ReleaseIndicator_UE);

if (isvalue(v_UE_Release)) {

v_Rel := valueof(v_UE_Release);

//select (p_ReleaseIndicator_UE) {

select (f_GetTestcaseAttrib_NR_Release (testcasename())) {

case ("") { // no attribute is assigned for Rel-15

v_RelMin := 15;  // >= rel15

}

case ("rel_16"){

v_RelMin := 16;  // >= rel16 - @sic R5s201387 BASELINE MOVING 2020 sic@

}

case ("rel_17"){

v_RelMin := 17;  // >= rel17 - @sic R5s221179 BASELINE MOVING 2022 sic@

}

case ("rel_18"){

v_RelMin := 18;  // >= rel18 - @sic R5s240445 BASELINE MOVING 2024 sic@

}

case else {

FatalError(__FILE__, __LINE__, "");

}

}

if (v_Rel < v_RelMin) {

f_SetVerdictInconc (__FILE__, __LINE__, "Wrong Access Stratum release indicator");

}

}

}

}

Module RRC_UECapability_ENDC_EUTRA

template (present) UE_EUTRA_Capability_v1510_IEs cr_UeCapabilitiesRAT_Container_ENDC_r18 := // @sic R5s240445 BASELINE MOVING 2024 rel-18 sic@

/* @status    APPROVED (ENDC) */

cr_UE_EUTRA_Capability_v1510_IEs(cr_UE_EUTRA_Capability_v1550_IEs(cr_UE_EUTRA_Capability_v1630_IEs(cr_UE_EUTRA_Capability_v1730_IEs)));

function f_EUTRA38_TxUECapEnquiry_RxUECapabilityInformation_TP1(EUTRA_CellId_Type         p_CellId,

RRC_TransactionIdentifier p_RRC_TI := tsc_RRC_TI_Def) runs on EUTRA_PTC

{ // @sic R5-211501 R5-216277 sic@

var UE_EUTRA_Capability v_RxEutraCapability;

var template UE_EUTRA_Capability_v1510_IEs v_EutraCapability_Expected;

var UE_EUTRA_Capability_v1510_IEs v_Rx_Decoded_EutraCapability;

var boolean v_Completed_AllSegments := false;

//@siclog "Step 1" siclog@

// The SS transmits a UECapabilityEnquiry message including eutra with rrc-segAllowed-r16 set to enabled

SRB.send(cas_SRB1_RrcPdu_REQ(p_CellId, cs_TimingInfo_Now, cs_508_UeCapabilityEnquiry_MRDC_r15IEs_true_SegAllowed_r16_enabled(p_RRC_TI)));

v_RxEutraCapability := f_EUTRA_GetCaps(p_CellId, p_RRC_TI);  // @sic R5-217903 sic@

v_Rx_Decoded_EutraCapability := v_RxEutraCapability.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension;

select (v_RxEutraCapability.accessStratumRelease) { //As per TS36.523-1 Table 8.5.4.1.3.3/2; details not checked in the scope of the present NR RRC TC 8.2.1.1.1

case (rel8, rel9, rel10, rel11, rel12, rel13, rel14) {

f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "AccessStratumRelease < release15");

}

case (rel15) {

v_EutraCapability_Expected := cr_UeCapabilitiesRAT_Container_ENDC_r15;

}

case (rel16) { // @sic R5s201387 BASELINE MOVING 2020 R5s221179 BASELINE MOVING 2022 sic@

v_EutraCapability_Expected := cr_UeCapabilitiesRAT_Container_ENDC_r16;

fl_EUTRACap_CommonChecks_r16(v_RxEutraCapability);

}

case (rel17) { // @sic R5s221179 BASELINE MOVING 2022 sic@

v_EutraCapability_Expected := cr_UeCapabilitiesRAT_Container_ENDC_r17;

fl_EUTRACap_CommonChecks_r17(v_RxEutraCapability);

}

case (rel18) { // @sic R5s240445 BASELINE MOVING 2024 sic@

v_EutraCapability_Expected := cr_UeCapabilitiesRAT_Container_ENDC_r18;

fl_EUTRACap_CommonChecks_r18(v_RxEutraCapability);

}

case else  {

f_SetVerdictInconc (__FILE__, __LINE__, "AccessStratumRelease not applicable for the purpose of tc8.2.1.1.1");

}

}

// Match the received UE-EUTRA-CapabilityInformation with the expected UE_EUTRA_Capability template

fl_EUTRA_UE_Capability_MatchTemplates(v_Rx_Decoded_EutraCapability, v_EutraCapability_Expected);

if (v_Completed_AllSegments) {

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 2a2");

}

else {

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 2b1");

}

};

function fl_EUTRACap_CommonChecks_r18(UE_EUTRA_Capability p_EUTRACapability)

{

fl_EUTRACap_CommonChecks_r17(p_EUTRACapability);

} // end of fl_EUTRACap_CommonChecks_r18

Module RRC_UECapability_NEDC_NR

function f_TC_8_2_1_1_2_NEDC_NR5GC_TestBody() runs on NR_BASE_PTC

{

var NR_SRB_COMMON_IND v_ReceivedAspForUeCapabilityInfo;

var NR_SRB_COMMON_IND v_Received_DedicatedMessageSegment;

var UE_NR_Capability v_NRCapability;

var UE_MRDC_Capability v_NRCapability_MRDC;

var EUTRA_NR_Coordination_MSG v_ReceivedMsg;

…

f_NR_MobileInfo_NRCap_Set (v_NRCapability);

// Check the UE release

f_NR_CheckReleaseIndicator (v_NRCapability.accessStratumRelease);

select (v_NRCapability.accessStratumRelease) {

case (rel15, rel16, rel17, rel18) { // @sic R5s201387 BASELINE MOVING 2020 R5s221179 BASELINE MOVING 2022 rel-17 R5s240445 BASELINE MOVING 2024 rel-18 sic@

}

case else{

FatalError (__FILE__, __LINE__, "accessStratumRelease not yet considered");

}

}

…

}//end of f_TC_8_2_1_1_2_NEDC_NR5GC_TestBody

## Editorial change in NR_SideLink_ASP_TypeDefs

Unnecessary FFS comment is deleted in the type NR_SL_RadioBearerId

type union NR_SL_RadioBearerId_Type {

NR_SL_SRB_Identity_Type                   SL_Srb,     /* routing for SL-SRB 0-3 for unicast */

integer                                   SL_Drb      /* routing for SL-DRBs */

};

## Split the definition of the CAG_Info (9.11.3.18A) and ExtdCAGInfo (9.11.3.86) Types

The above types have diverged therefore these are now split into 2 distinct definitions.

type record CAGInfoList {                            /* 24.501 cl. 9.11.3.18A */

IEI8_Type                   iei                   optional, /* present in case of TLV; omit in case of LV */

Type6Length_Type            iel,

record of CAG_Info  listOfCAGInfos optional

};

type record CAG_Info {                              /* 24.501 cl. 9.11.3.18A */

Type4Length_Type            iel,

CAG_Info_V                  cagValue               // Sep22 @sic R5s221179 Baseline Moving sic@

};

type record CAG_Info_V {                              /* 24.501 cl. 9.11.3.18A, 9.11.3.86 */

NAS_PlmnId                  plmn,

B7_Type                     spareBits,

B1_Type                     cagOnly,

record of O4_Type  listOfCAGIds optional

};

type record ExtdCAGInfoList {                       /* 24.501 cl. 9.11.3.86 */

IEI8_Type                   iei,

Type6Length_Type            iel,

record of ExtdCAG_Info  listOfCAGInfos optional

};

type record ExtdCAG_Info {                           /* 24.501 cl. 9.11.3.86 */

Type6Length_Type            iel,

NAS_PlmnId                  plmn,

B4_Type                     spareBits,

B1_Type                     caili,

B1_Type                     lci,

B1_Type                     spare,

B1_Type                     cagOnly,

Type6Length_Type            lengthWithoutAddInfoList    optional,

record of O4_Type           listOfCAGIds                optional,

CAGIdsWithAddInfo           listOfCAGIdsWithAddInfo     optional

};

type record CAGIdsWithAddInfo {

Type6Length_Type                iel,

record of CAGIdsWithAddInfo_V   listOfCAGIds

};

type record CAGIdsWithAddInfo_V {

Type6Length_Type            iel,

O4_Type                     cagId,

B1_Type                     spare,

B6_Type                     svii,

B1_Type                     tvii,

TimeValidityInfoList        timeValidityInfo     optional

};

type record TimeValidityInfoList {

O2_Type                numOfTimePeriods,

record of O16_Type     timePeriod

};

## Correction of the LADN_Info (9.11.3.30) Type

type record ListOfLADN {

Type4Length_Type            dnnLength,

octetstring                 dnnValue,

record length (1..16) of NG_ListOfPartialTaiList  listOfPartialTais  // same as TAIList IE from 3rd octet

};

type record LADN_Info {                              /* 24.501 cl. 9.11.3.30 */

IEI8_Type                   iei                   optional, /* present in case of TLV; omit in case of LV */

Type6Length_Type            iel,

record of ListOfLADN  listOfLADN

};

## Correction of the NSAG_Info (9.11.3.87) Type

type record NSAG {                              /* 24.501 cl. 9.11.3.87 */

Type4Length_Type            iel,              //  @sic R5s230533 sic@

O1_Type                     id,

NSSAI_LV                    sNSSAIList,

O1_Type                     priority,

NG_TrackingAreaIdList_LV    taiList		optional

};

type record NSAG_Info {                            /* 24.501 cl. 9.11.3.87 */

IEI8_Type                   iei,

Type6Length_Type            iel,

record of NSAG              nsagList

};

## Updating of the SOR_Header (9.11.3.51) Type

type union SOR_Header {                               // Jun24 @sic R5s240445 Baseline Moving sic@

SOR_HeaderType0     dataType0,

SOR_HeaderType1     dataType1

}

type record SOR_HeaderType1 {

B4_Type                     spare,                  /* Sep22 @sic R5s221179 Baseline Moving sic@ */

B1_Type                     msSSNPNSILS,            // Jun24 @sic R5s240445 Baseline Moving sic@

B1_Type                     msSNPNSI,               /* Sep22 @sic R5s221179 Baseline Moving sic@ */

B1_Type                     msSI,                   /* Sep22 @sic R5s221179 Baseline Moving sic@ */

B1_Type                     dataType

};

type record SOR_HeaderType0 {

B3_Type                     spare,                  /* Sep22 @sic R5s221179 Baseline Moving sic@ */

B1_Type                     ap,                     // Sep22 @sic R5s221179 Baseline Moving sic@

B1_Type                     ack,                    // Jun24 @sic R5s240445 Baseline Moving sic@

B1_Type                     listType,               /* Sep22 @sic R5s221179 Baseline Moving sic@ */

B1_Type                     listInd,                /* Sep22 @sic R5s221179 Baseline Moving sic@ */

B1_Type                     dataType

};

## Splitting of the TMGI_LV (9.11.4.31) Type

type record TMGI_LV {

Type4Length_Type            iel,

TMGI_V               ,

NAS_PlmnId                  plmn        optional

};

type record TMGI_V {

O3_Type                     serviceID,

NAS_PlmnId                  plmn        optional

};

## Changes for SOR test cases due to definition of new structured types

function f_NG_DecodeAndMatchSORTransportContainer (NR_SRB_COMMON_IND p_ReceivedAsp,

template SOR_Header p_SORContainerRX := cr_NG_SOR_Header (?, ?, ?, ?)) runs on NR_BASE_PTC

{

var SORTransparentContainer v_SORContainerRX;

var octetstring v_SORPayloadRxBytes;

var bitstring v_SORPayloadRx;

v_SORPayloadRxBytes := p_ReceivedAsp.Signalling.Nas[0].Pdu.Msg.ul_Nas_Transport.payload.payload;

v_SORPayloadRx := oct2bit(v_SORPayloadRxBytes);

if (decvalue(v_SORPayloadRx, v_SORContainerRX) != 0) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "SOR Container Failed");

}

// If SOR Header template to be tested

if (ispresent(p_SORContainerRX)) {

if (not match (v_SORContainerRX, cr_NG_SORTransparentContainer (p_SORContainerRX))) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "SOR Header Failed");

}

}

}

function fl_TC_6_3_1_2_TestBody() runs on NR5GC_PTC

{

var GMM_MobilityInfo_Type v_GMM_MobilityInfo;//@sic R5-202598 sic@

var NG_NAS_MSG_Indication_Type v_ReceivedMsg;//@sic R5-202598 sic@

var TypeOfRegistration_Type v_TypeOfRegistration;//@sic R5-202598 sic@

var NAS_PlmnId v_PlmnID_2 := f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell11));

var NAS_PlmnId v_PlmnID_13 := f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell12));//@sic R5-202598 sic@

var NAS_PlmnId v_PlmnID_14 := f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell13));//@sic R5-202598 sic@

var O2_Type v_AccessTechId :='0800'O;//@sic R5s200672 sic@

var template PLMN_AccessTechList v_PLMNAccessTech := {cs_SOR_PLMN_AccessTech (v_PlmnID_2, v_AccessTechId),

cs_SOR_PLMN_AccessTech (v_PlmnID_13, v_AccessTechId),

cs_SOR_PLMN_AccessTech (v_PlmnID_14, v_AccessTechId)};

var octetstring v_Data := bit2oct(encvalue(v_PLMNAccessTech));//v_PlmnID_2 & v_AccessTechId & v_PlmnID_13 & v_AccessTechId & v_PlmnID_14 & v_AccessTechId;

//   var octetstring v_Data := v_PlmnID_2 & v_AccessTechId & v_PlmnID_13 & v_AccessTechId & v_PlmnID_14 & v_AccessTechId;

var template (value) SOR_Header v_Header := cs_NG_SOR_HeaderType('0'B,'1'B,'1'B,'0'B);

timer t_Tmax;

timer t_Wait:= 10.0;

//@siclog "Step 1" siclog@

//Power on the UE

f_UT_SwitchOnUE(UT);

//@siclog "Steps 2-14" siclog@

//Steps 1 to 13 of the registration procedure described in TS 38.508-1 subclause 4.5.2.2-2 are performed on NR Cell 13

//@siclog "Step 15" siclog@

//The SS transmits an DLInformationTransfer message and an REGISTRATION ACCEPT message containing

//steering of roaming information indicating list of preferred PLMN/access technology combination provided with acknowledgment requested from the UE for successful reception

f_NR5GC_SorRegistration(nr_Cell13,v_Data,v_Header,-,-,(tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected));//@sic R5s210658 R5s211298 sic@

//@siclog "Step 16" siclog@

//SS starts timer of tmax =(6 minutes + cell selection time) (Note 1, 2 and 3)

t_Tmax.start(f_NR_SetTimerToleranceMax(nr_Cell13, nonProtocolTimer, 660.0));

//@siclog "Step 17" siclog@

//The UE transmits an ULInformationTransfer message and REGISTRATION COMPLETE message without Steering of Roaming Transparent container.

f_NR5GC_Registration_Complete(nr_Cell13);

//EXCEPTION: Steps 18a1-18a5 are performed IF pc_SOR_ACKNotReqLocalRel is False, ELSE step 18b1 is performed.//@sic R5-211380 sic@

if (not pc_SOR_ACKNotReqLocalRel){//@sic R5-211380 sic@//@sic R5s210613 sic@

//EXCEPTION: Step 18a1a1 is performed in pc_noOf_PDUsSameConnection > 0.

if (pc_noOf_PDUsSameConnection > 0 ){

//@siclog "Step 18a1a1" siclog@//@sic R5-211380 sic@

//The generic procedure for UE-requested PDU session establishment, specified in subclause 4.5A.2, takes place performing establishment of UE-requested PDU session(s)

//with ExpectedNumberOfNewPDUSessions = pc_noOf_PDUsSameConnection.

f_NR5GC_PDUSessionEstablishment (nr_Cell13,-,tsc_NR_RbId_SRB1);//@sic R5s210613 sic@

}

//@siclog "Step 18a2" siclog@//@sic R5-211380 sic@

//The SS transmits an RRCRelease message.

f_NR_RRCRelease(nr_Cell13);

//EXCEPTION: Step 18a3a1 is performed if pc_noOf_PDUsNewConnection > 0 and the UE starts PDU session establishment automatically

if (pc_noOf_PDUsNewConnection > 0 ){

//@siclog "Step 18a3a1" siclog@//@sic R5-211380 sic@

//The generic procedure for UE-requested PDU session establishment, specified in subclause 4.5.2.2-4, takes place from step 2 to 9a1 performing establishment of UE-requested PDU session(s)

//with ExpectedNumberOfNewPDUSessions = pc_noOf_PDUsNewConnection and then release the RRC Connection.

t_Wait.start;

alt {

[] SRB.check(receive(car_NR_SRB0_RrcPdu_IND(nr_Cell13, cr_38508_RRCSetupRequest(cr_NR_EstablishmentCause_Any))))

{

t_Wait.stop;

f_NR5GC_ExIdlePDUsession(nr_Cell13);

}

[] SRB.check(receive(car_NR_SRB0_RrcPdu_IND(nr_Cell11, cr_38508_RRCSetupRequest(cr_NR_EstablishmentCause_Any))))

{

t_Wait.stop;

}

[] t_Wait.timeout{}

}

}

//@siclog "Steps 18a4-18a5a1"-Void siclog@//@sic R5-211380 sic@

}

else{

//@siclog "Step 18b1" siclog@//@sic R5-202598 sic@

//SS performs local release

f_NR_RRCRelease_Local(nr_Cell13, f_NR_GetNextSendOccasion_DL(nr_Cell13));

}

//@siclog "Step 19 - Void" siclog@

//@sic R5-202598 sic@

//@siclog "Step 20" siclog@

//Check: Does the UE transmits an RRCSetupRequest on NR Cell 11 before tmax expires?

alt { //@sic R5s201466 sic@

[] SRB.check(receive(car_NR_SRB0_RrcPdu_IND(nr_Cell11, cr_38508_RRCSetupRequest(cr_NR_EstablishmentCause_Any))))

{

t_Tmax.stop;

f_NR_PreliminaryPass (__FILE__, __LINE__, "Step 20 PASS");

}

[] t_Tmax.timeout

{

f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "Step 20 Failed");

}

}

//@siclog "Step 21" siclog@

//The SS transmits an RRCSetup message on NR Cell 11.//@sic R5-202598 sic@

//@siclog "Step 22" siclog@

//The UE transmits an RRCSetupComplete message and a REGISTRATION REQUEST message on NR Cell 11. //@sic R5-202598 sic@

v_TypeOfRegistration := f_NR5GC_InitialOrMobilityRegistrationRequest (v_GMM_MobilityInfo, v_ReceivedMsg, nr_Cell11);

if (v_TypeOfRegistration == Initial_Secure) {

// @siclog "Steps 23a1-23a15a1" siclog@//@sic R5-202598 sic@

// IF 5GS registration type is set as Initial Registration in step 22, THEN steps 5 to 20a1 of the registration procedure described in TS 38.508-1 [4] Table 4.5.2.2-2 are performed on NR Cell 11.

//@sic R5s201044 R5-204408 sic@

f_NR5GC_SorRegistration_Step5_20(nr_Cell11,

v_Data,

v_Header,

-,

-,

Initial_Secure,

v_GMM_MobilityInfo,

v_ReceivedMsg,

false);

}

else if(v_TypeOfRegistration == Mobility){

// @siclog "Steps 23b1-23b2a1" siclog@//@sic R5-202598 sic@

//ELSE IF 5GS registration type is set as Mobility Registration in step 21, THEN steps 5 to 6a1 of the generic test procedure in TS 38.508-1 [4] Table 4.9.5.2.2-1 are performed on NR Cell 11.

f_NR5GC_MobilityRegistration_Steps4_6(nr_Cell11,v_GMM_MobilityInfo);

}

//@siclog "Steps 24-26" siclog@

//Void//@sic R5-202598 sic@

}//End of fl_TC_6_3_1_2_TestBody

...

function fl_TC_6_3_1_8_TestBody() runs on NR5GC_PTC

{

var GMM_MobilityInfo_Type v_GMM_MobilityInfo;

var NG_NAS_MSG_Indication_Type v_ReceivedMsg;

var NR_SRB_COMMON_IND v_ReceivedAsp;

var template (value) NG_NAS_DL_Pdu_Type v_PduToSend;

var octetstring  v_NAS_TxMsgContainer_SOR;

var TypeOfRegistration_Type v_TypeOfRegistration;//@sic R5-202604 sic@

var O16_Type v_Mac;

var B128_Type v_Mac_A17;

var NG_NAS_SecurityParams_Type v_NAS_SecurityParams;

var octetstring v_Data;

var NAS_PlmnId v_PlmnID := f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell11)); //@sic R5s200739 sic@

var O2_Type v_AccessTechId :='0800'O; //@sic R5s200739 sic@

var O2_Type v_Counter :='0001'O;

var template (value) SOR_Header v_Header := cs_NG_SOR_HeaderType0('1'B,'1'B,'1'B,'0'B);

timer t_Tmax;

//@siclog "Step 1" siclog@

//Power on the UE

f_UT_SwitchOnUE(UT);

//@siclog "Steps 2" siclog@

//whole registration procedure described in TS 38.508-1 subclause 4.5.2.2-2 are performed on NR Cell 13 with"connected without release"

v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (nr_Cell13, tsc_NR_RRC_TI_Def, ?, (tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected));// @sic R5s210658 R5s211298 sic@

if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_NoSecurity)) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed");

}

f_NR5GC_RRC_Idle_Steps5_20(v_GMM_MobilityInfo, nr_Cell13, v_ReceivedMsg, TESTMode_OFF, noRrcConnectionRelease, Initial_NoSecurity); // @sic R5s200739 sic@

//@siclog "Step 3" siclog@

//The SS transmits an DLInformationTransfer message containing

//steering of roaming information indicating list of preferred PLMN/access technology combination provided with acknowledgment requested from the UE for successful reception

v_Data := v_PlmnID & v_AccessTechId;

v_NAS_SecurityParams := f_NR5GC_Security_Get();

v_Mac_A17 :=f_NG_Authentication_A17(bit2oct(encvalue(v_Header)),

v_NAS_SecurityParams.KAUSF,

v_Counter,

v_Data,

v_NAS_SecurityParams.KDF);

v_Mac := bit2oct(v_Mac_A17);

v_NAS_TxMsgContainer_SOR := bit2oct(encvalue(cs_NG_SORTransparentContainerValue(v_Header, v_Mac, v_Counter, v_Data, omit, omit))); // @sic R5s200639 sic@

v_PduToSend := f_Get_NG_DLNASTransport (cs_PayloadContainerType(tsc_PayloadContainerSORMsg),

omit,

cs_PayloadContainer(omit, v_NAS_TxMsgContainer_SOR));

SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell13,

tsc_NR_RbId_SRB2,

cs_TimingInfo_Now,

cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered,v_PduToSend.Msg)));

//@siclog "Step 4" siclog@

//SS starts timer of tmax =(6 minutes + cell selection time) (Note 1, 2 and 3)

t_Tmax.start(f_NR_SetTimerToleranceMax(nr_Cell11, nonProtocolTimer, 660.0));

//@siclog "Step 5" siclog@

//The UE transmits an ULInformationTransfer message carrying acknowledgement of successful reception of the steering of roaming information

SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell13, tsc_NR_RbId_SRB2, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; // @sic R5s201387 Baseline Moving sic@

if (not f_Check_NG_ULNASTransport(v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg, cr_PayloadContainerType(tsc_PayloadContainerSORMsg), omit)) { // @sic R5s201387 Baseline Moving sic@

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "UL NAS Transport Message Failed");

}

// SOR data type should be 1 @sic R5s240531 sic@

f_NG_DecodeAndMatchSORTransportContainer (v_ReceivedAsp);

//@siclog "Step 6" siclog@

//The SS transmits an RRCRelease message.

f_NR_RRCRelease(nr_Cell13);

//@siclog "Step 7" siclog@

//Check: Does the UE transmits an RRCSetupRequest on NR Cell 11 before tmax expires?

alt { //@sic R5s201466 sic@

[] SRB.check(receive(car_NR_SRB0_RrcPdu_IND(nr_Cell11, cr_38508_RRCSetupRequest(cr_NR_EstablishmentCause_Any))))

{

t_Tmax.stop;

f_NR_PreliminaryPass (__FILE__, __LINE__, "Step 7 PASS");

}

[] t_Tmax.timeout

{

f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "Step 7 Failed");

}

}

//@siclog "Step 8" siclog@

//Steps 3-4 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed

//@siclog "10a1-10b5a1" siclog@  @sic R5202604 sic@

//NOTE: The UE performs a " REGISTRATION REQUEST procedure with type "mobility registration updating"? and the RRC connection is released

//NOTE: The 5GS registration type shall be only set as Mobility Registration for R16 UEs according to TS 24.501 subclause 5.2.3.2.5 specified in Release 16. The EXCEPTION description applies only to R15 UEs.

v_TypeOfRegistration := f_NR5GC_InitialOrMobilityRegistrationRequest (v_GMM_MobilityInfo, v_ReceivedMsg, nr_Cell11);

if (v_TypeOfRegistration == Initial_Secure){

//@siclog "10a1-10a15a1" siclog@

//IF 5GS registration type is set as Initial Registration in step 9, THEN steps 5 to 20a1 of the registration procedure described in TS 38.508-1 [4] Table 4.5.2.2-2 are performed on NR Cell 11.

f_NR5GC_MobileInfo_Init(); //@sic R5s230085 sic@

f_NR5GC_RRC_Idle_Steps5_20(v_GMM_MobilityInfo, nr_Cell11, v_ReceivedMsg, TESTMode_OFF,-, v_TypeOfRegistration);

}

else if (v_TypeOfRegistration == Mobility){

//@siclog "10b3-10b5a1" siclog@

// IF 5GS registration type is set as Mobility Registration in step 9, THEN steps 2 to 6a1 of the generic test procedure in TS 38.508-1 [4] Table 4.9.5.2.2-1 are performed on NR Cell 11.

f_NR5GC_MobilityRegistration_Steps4_6(nr_Cell11, v_GMM_MobilityInfo);

}

else {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed");

}

}//End of fl_TC_6_3_1_8_TestBody

function fl_TC_6_3_2_1_TestBody() runs on NR5GC_PTC

{

var octetstring v_Data;

var NAS_PlmnId v_PlmnID := f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell11));

var O2_Type v_AccessTechId :='0800'O;

var O1_Type v_SORInfo :='01'O;//SI/SSCMI/SSSI//@sic R5-233456 sic@

var octetstring  v_NAS_TxMsgContainer_SOR_CMCI;

var O16_Type v_Mac;

var B128_Type v_Mac_A17;

var NG_NAS_SecurityParams_Type v_NAS_SecurityParams;

var octetstring v_CriterionType:= '01'O;

var octetstring v_CriterionValue:= f_DomainName_Encode(pc_APN_ID_Internet);

var octetstring v_TsorcmTimer:= '21'O;

var O2_Type v_CounterSOR :='0001'O;

var template (value) SOR_Header v_Header := cs_NG_SOR_HeaderType0('1'B,'1'B,'1'B,'0'B,'1'B);//@sic R5-233456 sic@

var template (value) NG_NAS_DL_Pdu_Type v_PduToSend;

var O1_Type v_Length_PlmnID_AccessTechId := int2oct(lengthof(v_PlmnID & v_AccessTechId),1);//@sic R5s230554 sic@

var template (value) SOR_DataType0WithAddInfo v_SOR_Data;

var template (value) SOR_CMCIRule v_SOR_CMCIRule;

//@siclog "Step 1" siclog@

//Void//@sic R5-233456 sic@

//@siclog "Steps 2-21a1" siclog@

//Void//@sic R5-233456 sic@

//@siclog "Step 22-34b3a1" siclog@//@sic R5s230554 sic@

v_SOR_CMCIRule := cs_SOR_CMCIRule (v_TsorcmTimer, v_CriterionType, v_CriterionValue);

v_SOR_Data := cs_SOR_DataWithAddInfo(v_Length_PlmnID_AccessTechId,

{cs_SOR_PLMN_AccessTech(v_PlmnID, v_AccessTechId)},

'0'B,

'0'B,

'0'B,

'1'B,  // octet O = '01'

cs_SOR_CMCI({v_SOR_CMCIRule}));

v_Data := bit2oct(encvalue(v_SOR_Data));

//    v_Data := v_Length_PlmnID_AccessTechId & v_PlmnID & v_AccessTechId & v_SORInfo & v_Length_SOR_CMCI_contents & v_Length_SOR_CMCI_rule_contents & v_TsorcmTimer & v_CriterionType & v_CriterionValue;

v_NAS_SecurityParams := f_NR5GC_Security_Get();

v_Mac_A17 :=f_NG_Authentication_A17(bit2oct(encvalue(v_Header)),

v_NAS_SecurityParams.KAUSF,

v_CounterSOR,

v_Data,

v_NAS_SecurityParams.KDF);

v_Mac := bit2oct(v_Mac_A17);

v_NAS_TxMsgContainer_SOR_CMCI := bit2oct(encvalue(cs_NG_SORTransparentContainerValue(v_Header, v_Mac, v_CounterSOR, v_Data, omit, omit)));//@sic R5s230749 sic@

v_PduToSend := f_Get_NG_DLNASTransport (cs_PayloadContainerType(tsc_PayloadContainerSORMsg),

omit,

cs_PayloadContainer(omit, v_NAS_TxMsgContainer_SOR_CMCI));

f_NR5GC_TransportSorCmCheckfunction(nr_Cell12,nr_Cell11,v_PduToSend,-,"Step 26 PASS","Step 28 PASS");

}//End of fl_TC_6_3_2_1_TestBody

Similar changes have been made in all SOR test cases.

# Summary of ASP changes

ASP changes are documented in R5s240536, it includes the differences from delivery iwd-TTCN3-B2022-09_D24wk24 in Annex D of TS 38.523-3.