3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240532

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.571-4 | CR | 0205 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 June’24 baseline upgrade for POS Test Suite |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18_Test |  |  |  |  |  | Date: |  |  | 2024-07-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In order to develop the Rel-18 test cases in TTCN-3, the current baseline shall be upgraded. RAN5 agreed to move baseline to June 2024 in Rel-18 to all RAN5 TTCN-3 test suites. This CR is implementing this decision on 37.571-4 Test Suite. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The CR lists all changes affecting the messages and IEs type definitions caused by the baseline moving to the June 2024 Rel-18. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Rel-18 features cannot be implemented and tested. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | POS ATS |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Changes required to ASN.1 messages	3

2.1 ASN.1 Baseline Moving – TS 37.355	3

2.1.1 LPP_PDU_Definitions ASN.1 module	3

2.1.2 	LPP_Broadcast_Definitions ASN.1 module	10

2.2	ASN.1 Baseline Moving - OMA-TS-LPPe	10

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D24wk24 for baseline moving.

Contact:	Olivier Genoud

Olivier.Genoud@etsi.org

# Changes required to ASN.1 messages

## 2.1 ASN.1 Baseline Moving – TS 37.355

### 2.1.1 LPP_PDU_Definitions ASN.1 module

The tables below list the new IEs included in TS 37.355-i20 and their impact on the existing TTCN code.

#### 2.1.1.1	Non-critical extensions with extension marker (ellipsis)

| ANS.1 type affected | New fields | Impact on TTCN |
| --- | --- | --- |
| RequestAssistanceData-r9-IEs | bt-RequestAssistanceData-r18 | Set to “omit” in cs_RequestAssistData and cs_RequestAssistData_Omit Set to “*” in cr_RequestAssistanceData_r9_IEs |
| ProvideAssistanceData-r9-IEs | bt-ProvideAssistanceData-r18 | Set to “omit” in cs_LppProvideAssistData |
| LocationCoordinateTypes | local2dPointWithUncertaintyEllipse-r18 local3dPointWithUncertaintyEllipsoid-r18 | Set to “*” in cr_LocationCoordinateTypesCap |
| CommonIEsRequestCapabilities | remoteUE-IndicationReq-r18 | Set to “omit” in cs_LppReqCap |
| CommonIEsProvideCapabilities | remoteUE-Indication-r18 locationEstimateAndMeasurementReporting-r18 | Set to “*” in cr_CommonIEsProvideCapabilities |
| CommonIEsRequestLocationInformation | periodicalReportingExt-r18 | Set to “omit” in cs_LppRequestLocationInfo |
| NR-DL-PRS-AssistanceData-r16 | nr-DL-PRS-AggregationInfo-r18 | Set to “omit” in cs_DlPrsAssistanceData |
| DL-PRS-BeamInfoElement-r16 | nr-IntegrityBeamInfoBounds-r18 | No impact |
| NR-DL-PRS-Resource-r16 | dl-PRS-ResourceSymbolOffset-v1800 | Set to “omit” in cs_NR_DL_PRS_Resource |
| PRS-ProcessingCapabilityPerBand-r16 | maxNumOfOneSymbolPRS-ResProcessedPerSlot-RRC-Inactive-r18 maxNumOfOneSymbolPRS-ResProcessedPerSlot-RRC-Connected-r18 ppw-maxNumOfOneSymbolPRS-ResProcessedPerSlot-r18 prs-BWA-TwoContiguousIntrabandInMG-RRC-Connected-r18 prs-BWA-ThreeContiguousIntrabandInMG-RRC-Connected-r18 prs-BWA-TwoContiguousIntraband-RRC-IdleAndInactive-r18 prs-BWA-ThreeContiguousIntraband-RRC-IdleAndInactive-r18 reducedNumOfSampleInMeasurementWithPRS-BWA-RRC-Connected-r18 reducedNumOfSampleInMeasurementWithPRS-BWA-RRC-IdleAndInactive-r18 dl-PRS-MeasurementWithRxFH-RRC-Inactive-r18 dl-PRS-MeasurementWithRxFH-RRC-Idle-r18 reducedNumOfSampleForMeasurementWithFH-RRC-Connected-r18 reducedNumOfSampleForMeasurementWithFH-RRC-IdleAndInactive-r18 supportOfPRS-BWA-WithTwoPFL-Combination-r18 dl-PRS-MeasurementWithRxFH-RRC-Connected-r18 | No impact |
| NR-On-Demand-DL-PRS-Configurations-r17 | onDemandDL-PRS-AggregationList-r18 | No impact |
| NR-On-Demand-DL-PRS-Request-r17 | dl-PRS-AggregationID-PrefList-r18 nr-OnDemandDL-PRS-AggregationReqList-r18 | No impact |
| NR-PositionCalculationAssistance-r16 | nr-IntegrityServiceParameters-r18 nr-IntegrityServiceAlert-r18 nr-IntegrityRiskParameters-r18 nr-IntegrityParametersTRP-LocationInfo-r18 nr-IntegrityParametersDL-PRS-BeamInfo-r18 nr-IntegrityParametersRTD-Info-r18 nr-IntegrityParametersTRP-BeamAntennaInfo-r18 nr-PRU-DL-Info-r18 | Set to “omit” in cs_NrPositionCalculationAssist |
| RTD-InfoElement-r16 | nr-IntegrityRTD-InfoBounds-r18 | No impact |
| NR-TimeStamp-r16 | nr-Symbol-r18 | No impact |
| BeamPowerElement-r17 | nr-IntegrityBeamPowerBounds-r18 | No impact |
| TRP-LocationInfoElement-r16 | trp-LocationCartesian-r18 nr-IntegrityTRP-LocationBounds-r18 | Set to “omit” in cs_TrpLocInfoElement |
| DL-PRS-ResourceSets-TRP-Element-r16 | dl-PRS-ResourceSetARP-Cartesian-r18 nr-IntegrityDL-PRS-ResourceSetARP-LocationBounds-r18 | No impact |
| DL-PRS-Resource-ARP-Element-r16 | dl-PRS-Resource-ARP-locationCartesian-r18 nr-IntegrityDL-PRS-ResourceARP-LocationBounds-r18 | No impact |
| SRS-CapabilityPerBand-r16 | posSRS-Preconfigured-RRC-InactiveInitialUL-BWP-r18 posSRS-Preconfigured-RRC-InactiveOutsideInitialUL-BWP-r18 posSRS-ValidityAreaRRC-InactiveInitialUL-BWP-r18 posSRS-ValidityAreaRRC-InactiveOutsideInitialUL-BWP-r18 posSRS-TxFH-RRC-Connected-r18 posSRS-TxFH-RRC-Inactive-r18 posSRS-TxFH-WithTimeWindow-r18 posSRS-BWA-RRC-Inactive-r18 | No impact |
| SRS-PosResourcesPerBand-r16 | posSRS-BWA-RRC-Connected-r18 posSRS-BWA-IndependentCA-RRC-Connected-r18 | No impact |
| OTDOA-ProvideCapabilities | periodicReportingIntervalMsSupport-r18 | No impact |
| GNSS-CommonAssistData | gnss-los-nlos-GridPoints-r18 gnss-SSR-IOD-Update-r18 | Set to “omit” in cs_GNSS_CommonAssistData |
| GNSS-GenericAssistDataElement | gnss-SSR-OrbitCorrectionsSet2-r17 gnss-SSR-ClockCorrectionsSet2-r17 gnss-SSR-URA-Set2-r17 gnss-LOS-NLOS-GriddedIndications-r18 gnss-SSR-SatellitePCVResiduals-r18 | Set to “omit” in cs_GNSS_GenericAssistDataElement |
| GNSS-PeriodicAssistData-r15 | gnss-SSR-PeriodicOrbitCorrectionsSet2-r17 gnss-SSR-PeriodicClockCorrectionsSet2-r17 gnss-SSR-PeriodicURA-Set2-r17 gnss-SSR-PeriodicIOD-Update-r18 | No impact |
| GNSS-CommonAssistDataReq | gnss-SSR-IOD-UpdateReq-r18 | Set to “omit” in cs_GNSS_CommonAssistDataReq |
| GNSS-GenericAssistDataReqElement | gnss-SSR-OrbitCorrectionsSet2Req-r17 gnss-SSR-ClockCorrectionsSet2Req-r17 gnss-SSR-URA-Set2Req-r17 gnss-LOS-NLOS-GriddedIndicationsReq-r18 gnss-SSR-SatellitePCVResidualsReq-r18 | Set to “omit” in cs_GNSS_GenericAssistDataReqElement |
| GNSS-PeriodicAssistDataReq-r15 | gnss-SSR-PeriodicOrbitCorrectionsSet2Req-r17 gnss-SSR-PeriodicClockCorrectionsSet2Req-r17 gnss-SSR-PeriodicURA-Set2Req-r17 gnss-SSR-PeriodicIOD-UpdateReq-r18 | No impact |
| A-GNSS-ProvideCapabilities | periodicReportingIntervalMsSupport-r18 | Set to “*” in cr_A_GNSS_ProvideCapabilities |
| GNSS-CommonAssistanceDataSupport | gnss-SSR-IOD-UpdateSupport-r18 | No impact |
| GNSS-GenericAssistDataSupportElement | gnss-SSR-OrbitCorrectionsSet2Support-r17 gnss-SSR-ClockCorrectionsSet2Support-r17 gnss-SSR-URA-Set2Support-r17 gnss-LOS-NLOS-GriddedIndicationsSupport-r18 gnss-SSR-SatellitePCVResidualsSupport-r18 | No impact |
| GNSS-TargetDeviceErrorCauses | remoteUE-Indication-r18 | No impact |
| ECID-ProvideCapabilities | periodicReportingIntervalMsSupport-r18 | No impact |
| TBS-ProvideCapabilities-r13 | periodicReportingIntervalMsSupport-r18 | No impact |
| Sensor-ProvideCapabilities-r13 | periodicReportingIntervalMsSupport-r18 | No impact |
| WLAN-ProvideCapabilities-r13 | periodicReportingIntervalMsSupport-r18 | No impact |
| BT-ProvideLocationInformation-r13 | bt-AoA-Config-r18 | Set to “*” in cr_Bt_ProvideLocationInformation_NoError |
| BT-MeasurementInformation-r13 | bt-MeasurementList-r18 | No impact |
| BT-RequestLocationInformation-r13 | bt-requestedAoA-Config-r18 bt-suggestedAoA-Config-r18 | Set to “omit” in cs_BTRequestLocationInfoDef |
| BT-ProvideCapabilities-r13 | bt-AoA-r18 periodicReportingIntervalMsSupport-r18 | No impact |
| NR-ECID-ProvideCapabilities-r16 | periodicReportingIntervalMsSupport-r18 | No impact |
| NR-ECID-TargetDeviceErrorCauses-r16 | remoteUE-Indication-r18 | No impact |
| NR-DL-TDOA-ProvideAssistanceData-r16 | nr-PeriodicAssistData-r18 | Set to “omit” in cs_DlTdoaProvideAssistanceData |
| NR-DL-TDOA-RequestAssistanceData-r16 | nr-PeriodicAssistDataReq-r18 nr-IntegrityAssistanceRequest-r18 | No impact |
| NR-DL-TDOA-MeasElement-r16 | nr-MeasBasedOnAggregatedResources-r18 nr-AggregatedDL-PRS-ResourceInfo-List-r18 nr-RSCPD-r18 nr-PhaseQuality-r18 nr-RSCPD-AddMeasurementSamples-r18 nr-ReportDL-PRS-MeasBasedOnSingleOrMultiHopRx-r18 | No impact |
| NR-DL-TDOA-AdditionalMeasurementElement-r16 | nr-MeasBasedOnAggregatedResources-r18 nr-AggregatedDL-PRS-ResourceInfo-List-r18 nr-RSCPD-r18 nr-PhaseQuality-r18 nr-RSCPD-AdditionalMeasurementsAddSamples-r18 nr-ReportDL-PRS-MeasBasedOnSingleOrMultiHopRx-r18 | No impact |
| NR-DL-TDOA-RequestLocationInformation-r16 | nr-DL-PRS-JointMeasurementRequest-r18 nr-DL-PRS-RxHoppingRequest-r18 | Set to “omit” in cs_DlTDOARequestLocationInfoDef |
| NR-DL-TDOA-ReportConfig-r16 | timingReportingGranularityFactorExt-r18 nr-DL-PRS-MeasurementTimeWindowsConfig-r18 | No impact |
| NR-DL-TDOA-ProvideCapabilities-r16 | multiLocationEstimateInSameMeasReport-r17 locationCoordinateTypes-r18 periodicAssistanceData-r18 nr-IntegrityAssistanceSupport-r18 nr-DL-TDOA-OnDemandPRS-ForBWA-Support-r18 periodicReportingIntervalMsSupport-r18 | No impact |
| DL-TDOA-MeasCapabilityPerBand-r17 | supportOfDL-PRS-BWA-RRC-Connected-r18 supportOfDL-PRS-BWA-RRC-Inactive-r18 supportOfDL-PRS-BWA-RRC-Idle-r18 nr-DL-PRS-RSCPD-ReportingRRC-Connected-r18 assocSingleRSTD-WithUpToNsampleRSCPD-r18 nr-DL-PRS-RSCPD-MeasurementRRC-Idle-r18 supportOfUE-basedCarrierPhasePositioning-r18 supportOfSymbolTimeStampForRSCPD-r18 supportOfFinerTimingReportGranularityForPRS-Meas-r18 supportOfMeasurementsInTimeWindow-r18 supportOfPRS-MeasurementRRC-Idle-r18 | No impact |
| NR-DL-TDOA-TargetDeviceErrorCauses-r16 | remoteUE-Indication-r18 | No impact |
| NR-DL-AoD-RequestAssistanceData-r16 | nr-IntegrityAssistanceRequest-r18 | No impact |
| NR-DL-AoD-MeasElement-r16 | nr-ReportDL-PRS-MeasBasedOnSingleOrMultiHopRx-r18 | No impact |
| NR-DL-AoD-AdditionalMeasurementElement-r17 | nr-ReportDL-PRS-MeasBasedOnSingleOrMultiHopRx-r18 | No impact |
| NR-DL-AoD-RequestLocationInformation-r16 | nr-DL-PRS-RxHoppingRequest-r18 | Set to “omit” in cs_DlAoDRequestLocationInfoDef |
| NR-DL-AoD-ReportConfig-r16 | nr-DL-PRS-MeasurementTimeWindowsConfig-r18 | Set to “omit” in cs_DlAoDRequestLocationInfoDef |
| NR-DL-AoD-ProvideCapabilities-r16 | multiLocationEstimateInSameMeasReport-r17 locationCoordinateTypes-r18 nr-IntegrityAssistanceSupport-r18 periodicReportingIntervalMsSupport-r18 | No impact |
| DL-AoD-MeasCapabilityPerBand-r16 | supportOfMeasurementsInTimeWindow-r18 supportOfPRS-MeasurementRRC-Idle-r18 | No impact |
| NR-DL-AoD-TargetDeviceErrorCauses-r16 | remoteUE-Indication-r18 | No impact |
| NR-Multi-RTT-MeasElement-r16 | nr-MeasBasedOnAggregatedResources-r18 nr-AggregatedDL-PRS-ResourceInfo-List-r18 nr-RSCP-r18 nr-PhaseQuality-r18 nr-RSCP-AddSampleMeasurements-r18 nr-ReportDL-PRS-MeasBasedOnSingleOrMultiHopRx-r18 nr-NTN-UE-RxTxMeasurements-r18 | No impact |
| NR-Multi-RTT-AdditionalMeasurementElement-r16 | nr-MeasBasedOnAggregatedResources-r18 nr-AggregatedDL-PRS-ResourceInfo-List-r18 nr-RSCP-r18 nr-PhaseQuality-r18 nr-RSCP-AdditionalMeasurementsAddSample-r18 nr-ReportDL-PRS-MeasBasedOnSingleOrMultiHopRx-r18 nr-NTN-UE-RxTxMeasurements-r18 | No impact |
| NR-Multi-RTT-RequestLocationInformation-r16 | nr-DL-PRS-RxHoppingRequest-r18 timingReportingGranularityFactorExt-r18 nr-DL-PRS-JointMeasurementRequest-r18 nr-DL-PRS-MeasurementTimeWindowsConfig-r18 | Set to “omit” in cs_MultiRTTRequestLocationInfoDef |
| NR-Multi-RTT-ProvideCapabilities-r16 | nr-MultiRTT-OnDemandPRS-ForBWA-Support-r18 periodicReportingIntervalMsSupport-r18 | No impact |
| Multi-RTT-MeasCapabilityPerBand-r17 | supportOfDL-PRS-BWA-RRC-Connected-r18 supportOfDL-PRS-BWA-RRC-Inactive-r18 nr-NTN-MeasAndReport-r18 nr-DL-PRS-RSCP-ReportingRRC-Connected-r18 nr-DL-PRS-RSCP-ReportingRRC-Inactive-r18 assocSingleRx-Tx-WithUpToNsampleRSCP-r18 supportOfSymbolTimeStampForRSCP-r18 supportOfFinerTimingReportGranularityForPRS-Meas-r18 supportOfMeasurementsInTimeWindow-r18 | No impact |
| NR-Multi-RTT-TargetDeviceErrorCauses-r16 | remoteUE-Indication-r18 | No impact |

#### 2.1.1.2	New value in ENUMERATED type

| ANS.1 type affected | New values | Impact on TTCN |
| --- | --- | --- |
| LocationInformationType | locationEstimateAndMeasurementsRequired-r18 | No impact |
| NR-DL-PRS-ResourceSet-r16.dl-PRS-NumSymbols-r16 | n1-v1800 | No impact |
| NR-On-Demand-DL-PRS-PerFreqLayer-r17.dl-prs-ResourceSetPeriodicityReq-r17 | p128-v1760 p256-v1760 p512-v1760 | No impact |
| NR-On-Demand-DL-PRS-PerFreqLayer-r17.dl-prs-NumSymbolsReq-r17 | n1-v1800 | No impact |
| BT-LocationServerErrorCauses-r13.cause-r13 | assistanceDataNotSupportedByServer-v1810 assistanceDataSupportedButCurrentlyNotAvailableByServer-v1810 notAllrequestedAssistanceDataAvailableByServer-v1810 | No impact |
| BT-TargetDeviceErrorCauses-r13. cause-r13 | assistanceDataMissing-v1810 | No impact |

#### 2.1.1.3	New fields in CHOICE type

| ANS.1 type affected | New fields | Impact on TTCN |
| --- | --- | --- |
| LocationCoordinates | local2dPointWithUncertaintyEllipse-v1800 local3dPointWithUncertaintyEllipsoid-v1800 | No impact |
| NR-AdditionalPath-r16.nr-RelativeTimeDifference-r16 | kMinus6-r18 kMinus5-r18 kMinus4-r18 kMinus3-r18 kMinus2-r18 kMinus1-r18 | No impact |
| ReferencePoint-r16. referencePointGeographicLocation-r16 | localOrigin-v1800 | No impact |
| NR-DL-TDOA-MeasElement-r16.nr-RSTD-r16 | kMinus6-r18 kMinus5-r18 kMinus4-r18 kMinus3-r18 kMinus2-r18 kMinus1-r18 | No impact |
| NR-DL-TDOA-AdditionalMeasurementElement-r16.nr-RSTD-ResultDiff-r16 | kMinus6-r18 kMinus5-r18 kMinus4-r18 kMinus3-r18 kMinus2-r18 kMinus1-r18 | No impact |
| NR-Multi-RTT-MeasElement-r16. nr-UE-RxTxTimeDiff-r16 | kMinus6-r18 kMinus5-r18 kMinus4-r18 kMinus3-r18 kMinus2-r18 kMinus1-r18 | No impact |
| NR-Multi-RTT-AdditionalMeasurementElement-r16. nr-UE-RxTxTimeDiffAdditional-r16 | kMinus6-r18 kMinus5-r18 kMinus4-r18 kMinus3-r18 kMinus2-r18 kMinus1-r18 | No impact |

#### 2.1.1.4	New value in BIT STRING type

| ANS.1 type affected | New values | Impact on TTCN |
| --- | --- | --- |
| BT-RequestLocationInformation-r13.requestedMeasurements-r13 | aod-v1800 | No impact |
| BT-ProvideCapabilities-r13.bt-Modes-r13 | ue-based-v1810 | No impact |
| BT-ProvideCapabilities-r13.bt-MeasSupported-r13 | aod-v1800 | No impact |
| NR-DL-TDOA-RequestAssistanceData-r16.nr-PosCalcAssistanceRequest-r17 | nr-IntegrityBounds-r18 pruInfo-r18 | No impact |
| NR-DL-TDOA-RequestLocationInformation-r16. nr-RequestedMeasurements-r16 | dl-PRS-RSCPD-Request-r18 | No impact |
| NR-DL-TDOA-ProvideCapabilities-r16. nr-PosCalcAssistanceSupport-r17 | nr-IntegritySup-r18 pruInfoSup-r18 | No impact |
| NR-DL-AoD-RequestAssistanceData-r16.nr-PosCalcAssistanceRequest-r17 | nr-IntegrityBounds-r18 | No impact |
| NR-DL-AoD-ProvideCapabilities-r16. nr-PosCalcAssistanceSupport-r17 | nr-IntegrityBoundsSup-r18 | No impact |
| NR-Multi-RTT-RequestLocationInformation-r16. nr-RequestedMeasurements-r16 | dl-PRS-RSCP-Request-r18 | No impact |

### 2.1.2 	LPP_Broadcast_Definitions ASN.1 module

#### 2.1.2.1	New types

| New ANS.1 type | Impact on TTCN |
| --- | --- |
| NR-IntegrityParameters-r18 | No impact |

## 2.2	ASN.1 Baseline Moving - OMA-TS-LPPe

No impact. Version renamed to OMA-TS-LPPe-V1_0-20220916-C but there is no change in the technical content.