3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240535

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-5 | CR | 0118 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 Jun'24 baseline upgrade for MCX Test Suites |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18_Test |  |  |  |  |  | Date: |  |  | 2024-07-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In order to develop the Rel-18 test cases in TTCN-3, the current baseline shall be upgraded. RAN5 agreed to move baseline to June 2024 in Rel-18 to all RAN5 TTCN-3 test suites. This CR is implementing this decision on 36.579-5 Test Suites. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The CR lists all changes affecting the messages and IEs type definitions caused by the baseline moving to the June 2024 Rel-18. These changes of the type definitions in tabular format and XSD have been taken from 3GPP specs, RFCs and OMA specifications as listed below. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Rel-18 test cases cannot be implemented and verified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | All 36.579-5 TTCN-3 ATSs |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 36.579-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	4

2	Baseline Moving – Required Changes	4

2.1	SIP/SDP types	4

2.2	XSD types	4

2.2.1	XSD Modules	4

2.2.2	TTCN Changes	9

2.2.2.1	Renamed element names	9

2.2.2.2	New mandatory fields in Fc_timers_countersType	10

2.2.2.3	New optional fields	12

2.3	TTCN type definitions	21

2.3.1	MIKEY Messages (RFC 6509 and RFC 6043)	21

2.3.2	MIKEY general extension payload (33.180 V18.1.0 E.6)	22

2.3.3	MCPTT Floor Control and Preestablished session control (24.380 V18.6.0 clause 8)	22

2.3.3.1	Floor Taken: List of Audio SSRCs of Granted Participants	22

2.3.3.2	Connect: New field Invited MCPTT User Identity	22

2.3.4	MCVideo media plane control (24.581 V.18.4.0 clause 9)	23

2.3.4.1	Renaming of Granted Party's Identity field (24.581 Clause 9.2.3.6)	23

2.3.4.2	Renaming of SSRC field (24.581 Clause 9.2.3.16), new field for video	24

2.3.4.3	Transmission Granted	25

2.3.4.4	Transmission Arbitration Taken	26

2.3.4.5	Media transmission notification	27

2.3.4.6	Receive media request	28

2.3.4.7	Receive media response	29

2.3.4.8	Media reception notification	30

2.3.4.9	Transmission end request	31

2.3.4.10	Transmission end response	32

2.3.4.11	Media reception end request	33

2.3.4.12	Media reception end response	34

2.3.4.13	Transmission end notify	35

2.3.5	MCData messages (24.282 V18.7.0 clause 15)	36

2.3.5.1	MCData_FD_SIGNALLING_PAYLOAD: New field User location	36

2.4	Further changes	38

2.4.1	Use of @fuzzy in TTCN	38

# Overview

This document lists all the changes needed for baseline moving based on iwd-TTCN3-B2022-09_D24wk24.

Contact:	Oliviers Genoud
	olivier.genoud@etsi.org

# Baseline Moving – Required Changes

# SIP/SDP types

No changes.

# XSD types

## XSD Modules

| IANA-resource-lists.xsd |  | RFC 4826 clause 3.2 (no changes) |
| --- | --- | --- |
| RFC4354-poc-settings.xsd |  | RFC 4354 clause 6.1 (no changes) |
| RFC4661-SimpleFilter.xsd |  | RFC 4661 clause 7 (no changes) |
| RFC5261-patch-ops.xsd |  | RFC 5261 clause 8 (no changes) |
| RFC5874-xcap-diff.xsd |  | RFC 5874 clause 4 (no changes) |
| poc_listService-v1_0.xsd |  | OMA - poc_listService-v1_0: "List Service" (no changes) |
| xdm_extensions-v1_0.xsd |  | OMA - xdm_extensions-v1_0: "XDM - XDM2 - Extensions" (no changes) |
| xdm_rsrclst_uriusage-v1_0.xsd |  | OMA - xdm_commonPolicy-V1_0: "XDM - Common Policy" (no changes) |
| xenc-schema.xsd |  | http://www.w3.org/2001/04/xmlenc# (no changes) |
| xmldsig-core-schema.xsd |  | http://www.w3.org/2000/09/xmldsig# (no changes) |
| TS24281_mcvideoinfo.xsd | V18.6.0 | Updated; requires TTCN changes as shown in 2.2.2.3 (optional field added to cs_MCVideo_Info_Base and cr_MCVideo_Info_Base) |
| TS24281_mcvideolocationinfo.xsd | V18.6.0 | Updated; requires TTCN changes as shown in 2.2.2.1 (fields renamed in cs_MCVideo_GeographicalAreaChangeEmpty) and in 2.2.2.3 (optional field added to cs_MCVideo_LocationInfo_Report and cr_MCVideo_LocationInfo_Report); new top-level elements '5GMbsfsaAreaChange', '5GTrackingAreaChange' and '5GMbsfsaArea' do not have valid element names PATCH: renamed to 'NR5G…' NOTE 1: The elements are for extension (anyExt) of TriggeringCriteriaType and tRequestedLocationType and not used in TTCN yet NOTE 2: The changes in cs_MCVideo_GeographicalAreaChangeEmpty seem not to be backward compatible |
| TS24281_mcvideoPresInfo.xsd | V18.6.0 | No changes |
| TS24282_mcdatainfo.xsd | V18.7.0 | Updated; requires TTCN changes as shown in 2.2.2.3 (optional field added to cs_MCData_Info_Base and cr_MCData_Info_Base) |
| TS24282_mcdatalocationinfo.xsd | V18.7.0 | Updated; requires TTCN changes as shown in 2.2.2.1 (fields renamed in cs_MCData_GeographicalAreaChangeEmpty) and 2.2.2.3 (optional field added to cs_MCData_LocationInfo_Report and cr_MCData_LocationInfo_Report); new top-level elements '5GMbsfsaAreaChange', '5GTrackingAreaChange' and '5GMbsfsaArea' do not have valid element names PATCH: renamed to 'NR5G…' NOTE 1: The elements are for extension (anyExt) of TriggeringCriteriaType and tRequestedLocationType and not used in TTCN yet NOTE 2: The changes in cs_MCData_GeographicalAreaChangeEmpty seem not to be backward compatible |
| TS24282_mcdataPresInfo.xsd | V18.7.0 | No changes |
| TS24282_mcdataPresInfoFA.xsd | V18.7.0 | No changes |
| TS24379_mcpttaff.xsd | V18.7.0 | No changes |
| TS24379_mcpttinfo.xsd | V18.7.0 | Updated; requires TTCN changes as shown in 2.2.2.3 (optional field added to cs_MCPTT_Info_Base and cr_MCPTT_Info_Base) |
| TS24379_mcpttlocationinfo.xsd | V18.7.0 | Updated; requires TTCN changes as shown in 2.2.2.1 (fields renamed in cs_MCPTT_GeographicalAreaChangeEmpty) and in 2.2.2.3 (optional field added to cs_MCPTT_LocationInfo_Report and cr_MCPTT_LocationInfo_Report); new top-level elements '5GMbsfsaAreaChange', '5GTrackingAreaChange', '5GMbsfsaArea' and '5GMbsfsaAreaId' do not have valid element names PATCH: renamed to 'NR5G…' NOTE 1: The elements are for extension (anyExt) of TriggeringCriteriaType, tRequestedLocationType and tLocationType and not used in TTCN yet NOTE 2: The changes in cs_MCPTT_GeographicalAreaChangeEmpty seem not to be backward compatible |
| TS24379_mcpttPresInfo.xsd | V18.7.0 | Updated; no impact on TTCN |
| TS24379_mcpttPresInfoFA.xsd | V18.7.0 | Updated; no impact on TTCN |
| TS24379_mcpttsigneddoc.xsd | V18.7.0 | No changes |
| TS24379_poc-settings.xsd | V18.7.0 | Updated; no impact on TTCN New top-level element 'multiplex-support' with 'minOccurs="0" maxOccurs="1"'. This is not allowed for top-level elements. PATCH: minOccurs, maxOccurs removed from 'multiplex-support' |
| TS24379_preconfiguredRegroup.xsd | V18.7.0 | No changes |
| TS24481-mcptt-group.xsd | V18.1.0 | Updated; no impact on TTCN Definition of ListEntryType is syntactically wrong Required corrections: "<xs:element name="entry" type="mcpttgi:EntryType" maxoccurs=”unbounded”/>" to be replaced by "<xs:element name="entry" type="mcpttgi:EntryType" maxOccurs="unbounded"/>" |
| TS24481-mcpttGKTP.xsd | V18.1.0 | Updated; requires TTCN changes as shown in 2.2.2.3 (optional field added to cs_SingleGKTP) |
| TS24481-mcpttGMOP.xsd | V18.1.0 | No changes |
| TS24484-mcdata-service-config.xsd | V18.6.0 | Updated; requires TTCN changes as shown in 2.2.2.3 (optional field added to cs_MCData_ServiceConfiguration_OffNetwork) |
| TS24484-mcdata-ue-config.xsd | V18.6.0 | No changes |
| TS24484-mcdata-user-profile.xsd | V18.6.0 | Updated; no impact on TTCN |
| TS24484-mcptt-service-config.xsd | V18.6.0 | Updated; requires TTCN changes as shown in 2.2.2.2 (mandatory fields added to MCPTT_ServiceConfiguration_OnNetwork_Type) and 2.2.2.3 (optional field added to cs_MCPTT_ServiceConfiguration_OffNetwork) NOTE 1: Changes in MCPTT_ServiceConfiguration_OnNetwork_Type may cause backward compatibility issues |
| TS24484-mcptt-ue-config.xsd | V18.6.0 | No changes |
| TS24484-mcptt-user-profile.xsd | V18.6.0 | Updated; no impact on TTCN |
| TS24484-mcvideo-service-config.xsd | V18.6.0 | Updated; requires TTCN changes as shown in 2.2.2.3 (optional field added to cs_MCVideo_ServiceConfiguration_OffNetwork) New top-level element 'adhoc-group-call' with 'minOccurs="0"'. This is not allowed for top-level elements. PATCH: minOccurs removed from 'adhoc-group-call' |
| TS24484-mcvideo-ue-config.xsd | V18.6.0 | No changes |
| TS24484-mcvideo-user-profile.xsd | V18.6.0 | Updated; no impact on TTCN Imported namespace "urn:3gpp:ns:mcpttUEinitConfig:1.0" does not exist but needs to be changed to "urn:3gpp:mcptt:mcpttUEinitConfig:1.0 In MigratablePartnerMCVideoSystemInfoEntryType the element "AccessInformationForPartnerMCVideoSystem" with type="mcpttiup:mcptt-UE-initial-configuration" is not allowed; ref="mcpttiup:mcptt-UE-initial-configuration" shall be used instead |
| TS24484-ue-init-config.xsd | V18.6.0 | No changes |
| TS33180-mcsecKMSInterface.xsd | V18.1.0 | No changes |
| TS33180-mcsecKMSKRR.xsd | V18.1.0 | No changes |

## TTCN Changes

## Renamed element names

| Reason for change | In the location-info for MCData (urn_3gpp_ns_mcdataLocationInfo_1_0), MCPTT (urn_3gpp_ns_mcpttLocationInfo_1_0) and MCVideo (urn_3gpp_ns_mcvideoLocationInfo_1_0) the elements 'enterSpecificAreaType' and 'exitSpecificAreaType' have been renamed to 'enterSpecificArea' and 'exitSpecificArea' |
| --- | --- |
| Summary of change | Fields renamed in templates accordingly. NOTE: Depending on a codec implementation the changes may not be backward compatible as the codec may not know the new element names !! (For MCPTT the changes have been done in V18.6.0 (2024-03), i.e. the changes even have not been done at the beginning of Rel-18) |
| TTCN modules | MCX_XML_mcdataLocationInfo, MCX_XML_mcpttLocationInfo, MCX_XML_mcvideoLocationInfo |

template (value) TGeographicalAreaChange cs_MCData_GeographicalAreaChangeEmpty := { attr := omit, anyAreaChange := omit, enterSpecificArea := omit, exitSpecificArea := omit, elem_list := {}, anyExt := omit };

template (value) TGeographicalAreaChange cs_MCPTT_GeographicalAreaChangeEmpty := { attr := omit, anyAreaChange := omit, enterSpecificArea := omit, exitSpecificArea := omit, elem_list := {}, anyExt := omit };

template (value) TGeographicalAreaChange cs_MCVideo_GeographicalAreaChangeEmpty := { attr := omit, anyAreaChange := omit, enterSpecificArea := omit, exitSpecificArea := omit, elem_list := {}, anyExt := omit };

## New mandatory fields in Fc_timers_countersType

| Reason for change | There are new mandatory elements in the complex type "fc-timers-countersType" of the MCPTT service configuration document. |
| --- | --- |
| Summary of change | Fields added to cs_MCPTT_ServiceConfiguration_OnNetwork with values taken from example in 24.484 Table A.2.3-12. NOTE 1: Depending on a codec implementation the changes may not be backward compatible as at the UE side the codec may fail decoding the new elements and consider the XML document as not valid !! NOTE 2: As long as the elements are mandatory, they need to be provided with valid values which need to be specified in 36.579-1 Table 5.5.8.4-1 requires prose CR !! |
| TTCN modules | MCX_XML_Configuration_MCPTT |

template (value) MCPTT_ServiceConfiguration_OnNetwork_Type cs_MCPTT_ServiceConfiguration_OnNetwork := { /* on-network service cnfiguration */ attr := omit, emergency_call := { attr := omit, private_cancel_timeout := "PT30M", group_time_limit := "PT20M", anyExt := omit, elem_list := {} }, private_call := { attr := omit, hang_time := "PT30S", max_duration_with_floor_control := "PT30S", max_duration_without_floor_control := "PT20M", anyExt := omit, elem_list := {} }, num_levels_priority_hierarchy := 10, transmit_time := { attr := omit, time_limit := "PT30S", time_warning := "PT20M", anyExt := omit, elem_list := {} }, hang_time_warning := "PT20M", floor_control_queue := { attr := omit, depth := 5, max_user_request_time := "PT20M", anyExt := omit, elem_list := {} }, fc_timers_counters := { attr := omit, t1_end_of_rtp_media := "PT4S", t3_stop_talking_grace:= "PT3S", t7_floor_idle := "PT2S", t8_floor_revoke := "PT1S", t11_end_of_RTP_dual := "PT4S", t12_stop_talking_dual := "PT30S", t15_conversation := "PT30S", t16_map_group_to_bearer := "PT0.5S", t17_unmap_group_to_bearer := "PT0.5S", t20_floor_granted := "PT1S", t25_mbs_conversation := "PT30S", t26_map_group_to_session_stream := "PT0.5S", t27_unmap_group_from_session_stream := "PT0.2S", t55_connect := "PT2S", t56_disconnect := "PT2S", c7_floor_idle := 10, c17_unmap_group_to_bearer := 3, c20_floor_granted := 3, c27_unmap_group_from_session_stream := 3, c55_connect := 3, c56_disconnect := 3, anyExt := omit, elem_list := {} }, signalling_protection := { attr := omit, confidentiality_protection := tsc_MCX_XML_ConfidentialityProtection, integrity_protection := tsc_MCX_XML_IntegrityProtection, anyExt := omit, elem_list := {} }, protection_between_mcptt_servers := { attr := omit, allow_signalling_protection := true, allow_floor_control_protection := true, anyExt := omit, elem_list := {} }, emergency_resource_priority := cs_MCPTT_ResourcePriority(tsc_MCPTT_EmergencyResourcePriorityNamespace, tsc_MCPTT_EmergencyResourcePriorityValue), imminent_peril_resource_priority := cs_MCPTT_ResourcePriority(tsc_MCPTT_ImminentPerilResourcePriorityNamespace, tsc_MCPTT_ImminentPerilResourcePriorityValue), normal_resource_priority := cs_MCPTT_ResourcePriority(tsc_MCPTT_NormalResourcePriorityNamespace, tsc_MCPTT_NormalResourcePriorityValue), anyExt := { elem_list := { f_Bit2XsdString(encvalue(cs_MCPTT_ServiceConfiguration_FunctionalAliasList)) // @sic R5-213589 sic@ } }, elem_list := {} };

## New optional fields

| Reason for change | There are new optional fields in XSD types |
| --- | --- |
| Summary of change | Optional fields added to templates as omit. |
| TTCN module | MCX_XML_Configuration_MCData, MCX_XML_Configuration_MCPTT, MCX_XML_Configuration_MCVideo, MCX_XML_mcdata_info, MCX_XML_mcdataLocationInfo, MCX_XML_mcptt_info, MCX_XML_mcpttLocationInfo, MCX_XML_mcvideo_info, MCX_XML_mcvideoLocationInfo, MCX_XML_xcap_diff, MCData_Location_MCX_IMS, MCPTT_Location_MCX_IMS, MCVideo_Location_MCX_IMS |

template (value) MCData_ServiceConfiguration_OffNetwork_Type cs_MCData_ServiceConfiguration_OffNetwork := { attr := omit, default_prose_per_packet_priority := cs_MCData_ServiceConfiguration_Default_prose_per_packet_priority, default_pqi := omit, anyExt := omit, elem_list := {} };

template (value) MCPTT_ServiceConfiguration_OffNetwork_Type cs_MCPTT_ServiceConfiguration_OffNetwork := { attr := omit, emergency_call := { attr := omit, private_cancel_timeout := "PT5S", group_time_limit := "PT5S", anyExt := omit, elem_list := {} }, private_call := { attr := omit, hang_time := "PT5S", max_duration_with_floor_control := "PT60S", max_duration_without_floor_control := omit, anyExt := omit, elem_list := {} }, num_levels_priority_hierarchy := 4, transmit_time := { attr := omit, time_limit := "PT60S", time_warning := "PT50S", anyExt := omit, elem_list := {} }, hang_time_warning := "PT4S", default_prose_per_packet_priority := { attr := omit, mcptt_private_call_signalling := 1, mcptt_private_call_media := 1, mcptt_emergency_private_call_signalling := 8, mcptt_emergency_private_call_media := 8, anyExt := omit, elem_list := {} }, allow_log_metadata := true, default_pqi := omit, anyExt := omit, elem_list := {} };

template (value) MCVideo_ServiceConfiguration_OffNetwork_Type cs_MCVideo_ServiceConfiguration_OffNetwork := { attr := omit, default_prose_per_packet_priority := cs_MCVideo_ServiceConfiguration_Default_prose_per_packet_priority, private_call := cs_MCVideo_ServiceConfiguration_Private_call, num_levels_priority_hierarchy := 4, default_pqi := omit, anyExt := omit, elem_list := {} };

template (value) MCData_Info_Type cs_MCData_Info_Base(template (omit) charstring p_RequestType := omit, template (omit) charstring p_RequestURI := px_MCData_ID_User_A, template (omit) charstring p_CallingUserId := px_MCData_ID_User_B, template (omit) charstring p_CallingGroupId := omit, template (omit) charstring p_ClientId := omit, template (omit) charstring p_ControllerPSI := omit) := { /* MCData info according to 36.579-1 Table 5.5.3.2.2-3 */ attr := omit, mcdata_Params := { attr := omit, mcdata_access_token := omit, request_type := p_RequestType, mcdata_request_uri := fl_MCData_Content_URI(p_RequestURI), mcdata_calling_user_id := fl_MCData_Content_URI(p_CallingUserId), mcdata_called_party_id := omit, mcdata_calling_group_id := fl_MCData_Content_URI(p_CallingGroupId), alert_ind := omit, originated_by := omit, mcdata_client_id := fl_MCData_Content_String(p_ClientId), mcdata_controller_psi := fl_MCData_Content_URI(p_ControllerPSI), gw_mcdata_usage := omit, elem_list := {}, anyExt := omit }, elem_list := {}, anyExt := omit };

template (present) MCData_Info_Type cr_MCData_Info_Base(template charstring p_RequestType := omit, template ContentType p_RequestURI := omit, template ContentType p_CallingGroupId := omit, template ContentType p_ClientId := omit) := { /* MCData info according to 36.579-1 Table 5.5.3.2.1-3 */ attr := omit, mcdata_Params := { attr := omit, mcdata_access_token := omit, request_type := p_RequestType, mcdata_request_uri := p_RequestURI, mcdata_calling_user_id := omit, mcdata_called_party_id := omit, mcdata_calling_group_id := p_CallingGroupId, alert_ind := omit, originated_by := omit, mcdata_client_id := p_ClientId, mcdata_controller_psi := omit, gw_mcdata_usage := omit, elem_list := {}, anyExt := * }, elem_list := {}, anyExt := * };

template (value) MCData_LocationInfo_Type cs_MCData_LocationInfo_Report(integer p_Longitude, integer p_Latitude) := { /* 36.579-1 Table 5.5.3.4.4-3: Location-info (Report from the SS) for MCData */ attr := omit, choice := { report := { reportID := omit, reportType := emergency, attr := omit, triggerId_list := {}, mcdata_reporting_uri := omit, currentLocation := { attr := omit, currentServingEcgi := omit, neighbouringEcgi_list := {}, mbmsSaId := omit, mbsfnArea := omit, currentCoordinate := cs_MCData_PointCoordinate(p_Longitude, p_Latitude), elem_list := {}, anyExt := omit }, elem_list := {}, anyExt := omit } } };

template (present) MCData_LocationInfo_Type cr_MCData_LocationInfo_Report(template XSD.String p_ReportId := omit, TReportType.reportType p_ReportType := emergency, TReportType.triggerId_list p_TriggerIdList := {}, template (present) TCurrentLocationType p_CurrentLocation := cr_MCData_CurrentLocation_Any) := { /* 36.579-1 Table 5.5.3.4.1-3: Location-info (Report from the UE) */ attr := omit, choice := { report := { reportID := p_ReportId, reportType := p_ReportType, attr := omit, triggerId_list := p_TriggerIdList, mcdata_reporting_uri := omit, currentLocation := p_CurrentLocation, elem_list := {}, anyExt := omit } } };

template (value) Mcpttinfo_Type cs_MCPTT_Info_Base(template (omit) charstring p_McpttRequestURI := px_MCPTT_ID_User_A, template (omit) charstring p_McpttCallingUserId := px_MCPTT_ID_User_B) := { /* MCPTT info according to 36.579-1 Table 5.5.3.2.2-1 */ attr := omit, mcptt_Params := { attr := omit, mcptt_access_token := omit, session_type := omit, mcptt_request_uri := f_MCPTT_Content_URI(p_McpttRequestURI), mcptt_calling_user_id := f_MCPTT_Content_URI(p_McpttCallingUserId), mcptt_called_party_id := omit, mcptt_calling_group_id := omit, required := omit, emergency_ind := omit, alert_ind := omit, imminentperil_ind := omit, broadcast_ind := omit, mc_org := omit, floor_state := omit, associated_group_id := omit, originated_by := omit, mKFC_GKTPs := omit, mcptt_client_id := omit, alert_ind_rcvd := omit, gw_mcptt_usage := omit, elem_list := {}, anyExt := omit }, elem_list := {}, anyExt := omit };

template (present) MCPTT_Info_Type cr_MCPTT_Info_Base := { /* MCPTT info according to 36.579-1 Table 5.5.3.2.1-1 */ attr := omit, mcptt_Params := { attr := omit, mcptt_access_token := omit, session_type := omit, mcptt_request_uri := omit, mcptt_calling_user_id := omit, mcptt_called_party_id := omit, mcptt_calling_group_id := omit, required := omit, emergency_ind := cr_MCPTT_Content_FalseIfPresent, alert_ind := cr_MCPTT_Content_FalseIfPresent, imminentperil_ind := cr_MCPTT_Content_FalseIfPresent, broadcast_ind := false ifpresent, mc_org := omit, floor_state := omit, associated_group_id := omit, originated_by := omit, mKFC_GKTPs := omit, mcptt_client_id := omit, alert_ind_rcvd := omit, gw_mcptt_usage := omit, elem_list := {}, anyExt := * }, elem_list := {}, anyExt := * };

template (value) MCPTT_LocationInfo_Type cs_MCPTT_LocationInfo_Report(integer p_Longitude, integer p_Latitude) := { /* 36.579-1 Table 5.5.3.4.4-1: Location-info (Report from the SS) for MCPTT */ attr := omit, choice := { report := { reportID := omit, reportType := emergency, attr := omit, triggerId_list := {}, // @sic R5-216678: empty list as for test case 6.1.1.16 there should be no triggerId and there are no other cases sic@ mcptt_reporting_uri := omit, currentLocation := { attr := omit, currentServingEcgi := omit, neighbouringEcgi_list := {}, mbmsSaId := omit, mbsfnArea := omit, currentCoordinate := cs_MCPTT_PointCoordinate(p_Longitude, p_Latitude), elem_list := {}, anyExt := omit }, elem_list := {}, anyExt := omit } } };

template (present) MCPTT_LocationInfo_Type cr_MCPTT_LocationInfo_Report(template XSD.String p_ReportId := omit, TReportType.reportType p_ReportType := emergency, TReportType.triggerId_list p_TriggerIdList := {}, template (present) TCurrentLocationType p_CurrentLocation := cr_MCPTT_CurrentLocation_Any) := { /* 36.579-1 Table 5.5.3.4.1-1: Location-info (Report from the UE) */ attr := omit, choice := { report := { reportID := p_ReportId, reportType := p_ReportType, attr := omit, triggerId_list := p_TriggerIdList, mcptt_reporting_uri := omit, currentLocation := p_CurrentLocation, elem_list := {}, anyExt := omit } } };

template (value) MCVideo_Info_Type cs_MCVideo_Info_Base(template (omit) charstring p_McvideoRequestURI := px_MCVideo_ID_User_A, template (omit) charstring p_McvideoCallingUserId := px_MCVideo_ID_User_B) := { /* MCVideo info according to 36.579-1 Table 5.5.3.2.2-2 */ attr := omit, mcvideo_Params := { attr := omit, mcvideo_access_token := omit, session_type := omit, mcvideo_request_uri := f_MCVideo_Content_URI(p_McvideoRequestURI), mcvideo_calling_user_id := f_MCVideo_Content_URI(p_McvideoCallingUserId), mcvideo_called_party_id := omit, mcvideo_calling_group_id := omit, required := omit, emergency_ind := omit, alert_ind := omit, imminentperil_ind := omit, broadcast_ind := omit, mc_org := omit, associated_group_id := omit, originated_by := omit, mKFC_GKTPs := omit, mcvideo_client_id := omit, alert_ind_rcvd := omit, multiple_devices_ind := omit, video_pull_url := omit, gw_mcvideo_usage := omit, elem_list := {}, anyExt := omit }, elem_list := {}, anyExt := omit };

template (present) MCVideo_Info_Type cr_MCVideo_Info_Base := { /* MCVideo info according to 36.579-1 Table 5.5.3.2.1-1 */ attr := omit, mcvideo_Params := { attr := omit, mcvideo_access_token := omit, session_type := omit, mcvideo_request_uri := omit, mcvideo_calling_user_id := omit, mcvideo_called_party_id := omit, mcvideo_calling_group_id := omit, required := omit, emergency_ind := cr_MCVideo_Content_FalseIfPresent, alert_ind := cr_MCVideo_Content_FalseIfPresent, imminentperil_ind := cr_MCVideo_Content_FalseIfPresent, broadcast_ind := false ifpresent, mc_org := omit, associated_group_id := omit, originated_by := omit, mKFC_GKTPs := omit, mcvideo_client_id := omit, alert_ind_rcvd := omit, multiple_devices_ind := omit, video_pull_url := omit, gw_mcvideo_usage := omit, elem_list := {}, anyExt := * }, elem_list := {}, anyExt := * };

template (value) MCVideo_LocationInfo_Type cs_MCVideo_LocationInfo_Report(integer p_Longitude, integer p_Latitude) := { /* 36.579-1 Table 5.5.3.4.4-2: Location-info (Report from the SS) for MCVideo */ attr := omit, choice := { report := { reportID := omit, reportType := emergency, attr := omit, triggerId_list := {}, mcvideo_reporting_uri := omit, currentLocation := { attr := omit, currentServingEcgi := omit, neighbouringEcgi_list := {}, mbmsSaId := omit, mbsfnArea := omit, currentCoordinate := cs_MCVideo_PointCoordinate(p_Longitude, p_Latitude), elem_list := {}, anyExt := omit }, elem_list := {}, anyExt := omit } } };

template (present) MCVideo_LocationInfo_Type cr_MCVideo_LocationInfo_Report(template XSD.String p_ReportId := omit, TReportType.reportType p_ReportType := emergency, TReportType.triggerId_list p_TriggerIdList := {}, template (present) TCurrentLocationType p_CurrentLocation := cr_MCVideo_CurrentLocation_Any) := { /* 36.579-1 Table 5.5.3.4.1-1: Location-info (Report from the UE) */ attr := omit, choice := { report := { reportID := p_ReportId, reportType := p_ReportType, attr := omit, triggerId_list := p_TriggerIdList, mcvideo_reporting_uri := omit, currentLocation := p_CurrentLocation, elem_list := {}, anyExt := omit } } };

template (value) GKTPType cs_SingleGKTP(XSD.String p_Id, bitstring p_EncodedMikeyMsg) := { activationtime := omit, id := p_Id, attr := omit, base := bit2oct(p_EncodedMikeyMsg) };

template (value) MCData_LocationInfo_Type cs_MCData_LocationInfo_ReportingRequest(XSD.String p_RequestId) := { /* Table 5.5.3.4.3-3: Location-info (Request sent by the SS) for MCData */ attr := omit, choice := { request := { requestId := p_RequestId, refresh := omit } } };

template (value) MCPTT_LocationInfo_Type cs_MCPTT_LocationInfo_ReportingRequest(XSD.String p_RequestId) := { /* Table 5.5.3.4.3-1: Location-info (Request sent by the SS) for MCPTT */ attr := omit, choice := { request := { requestId := p_RequestId, refresh := omit } } };

template (value) MCVideo_LocationInfo_Type cs_MCVideo_LocationInfo_ReportingRequest(XSD.String p_RequestId) := { /* Table 5.5.3.4.3-2: Location-info (Request sent by the SS) for MCVideo */ attr := omit, choice := { request := { requestId := p_RequestId, refresh := omit } } };

# TTCN type definitions

## MIKEY Messages (RFC 6509 and RFC 6043)

No changes in MIKEY_TypeDefs.

## MIKEY general extension payload (33.180 V18.1.0 E.6)

No changes in MIKEY_MCX_Extension_TypeDefs.

## MCPTT Floor Control and Preestablished session control (24.380 V18.6.0 clause 8)

## Floor Taken: List of Audio SSRCs of Granted Participants

| Reason for change | In 24.380 clause 8.2.9 (Floor Taken message) the field "List of SSRCs of granted floor participants" has been renamed to "List of Audio SSRCs of Granted Participants" |
| --- | --- |
| Summary of change | Field "ssrcs" renamed to "ssrcsGrantedParticipants" in FloorControl_Taken and templates cs_FloorCtrlMsg_Taken and cds_FloorCtrlMsg_Taken_MultiTalker |
| TTCN module | MCPTT_MediaControl_TypeDefs, MCPTT_MediaControl_Templates |

## Connect: New field Invited MCPTT User Identity

| Reason for change | New field Invited MCPTT User Identity in 24.380 Table 8.3.4-1 (Connect message) |
| --- | --- |
| Summary of change | MCPTT User Identity added to Connect message |
| TTCN module | MCPTT_MediaControl_TypeDefs, MCPTT_MediaControl_Templates |

type record PreestablishedSessionControl_InvitedUserIdentity { O1_Type id, O1_Type lgth, octetstring val, octetstring padding };

type set PreestablishedSessionControl_Connect { PreestablishedSessionControl_CommonHdr commonHdr, PreestablishedSessionControl_SessionIdentity sessionIdentity optional, PreestablishedSessionControl_GroupIdentity groupIdentity optional, PreestablishedSessionControl_MediaStreams mediaStreams optional, PreestablishedSessionControl_WarningText warningText optional, PreestablishedSessionControl_AnswerState answerState optional, PreestablishedSessionControl_InvitingUserIdentity invitingUserIdentity optional, PreestablishedSessionControl_InvitedUserIdentity invitedUserIdentity optional, // Rel-18 PreestablishedSessionControl_PCK_I_Message pckIMessage optional };

template (value) PreestablishedSessionControlMsg_Type cs_PreestablishedSessionControl_Connect(RTP_SSRC p_RTCP_SSRC_SS, template (value) PreestablishedSessionControl_SessionIdentity p_SessionIdentity, template (omit) PreestablishedSessionControl_GroupIdentity p_GroupIdentity, template (value) PreestablishedSessionControl_MediaStreams p_MediaStreams, template (value) PreestablishedSessionControl_AnswerState p_AnswerState, template (value) PreestablishedSessionControl_InvitingUserIdentity p_InvitingUserIdentity, MCX_MediaControl_AckRequest_Type p_AckRequest) := { connectMessage := { commonHdr := cs_PreestablishedSessionControl_CommonHdr(f_MCX_MediaControlSubtype_TX('00000'B, p_AckRequest), p_RTCP_SSRC_SS), sessionIdentity := p_SessionIdentity, groupIdentity := p_GroupIdentity, mediaStreams := p_MediaStreams, warningText := omit, answerState := p_AnswerState, invitingUserIdentity := p_InvitingUserIdentity, invitedUserIdentity := omit, pckIMessage := omit } };

## MCVideo media plane control (24.581 V.18.4.0 clause 9)

## Renaming of Granted Party's Identity field (24.581 Clause 9.2.3.6)

| Reason for change | In 24.581 clause 9.2.3.6 "Granted Party's Identity" has been renamed to "User Id of the Transmitting User" |
| --- | --- |
| Summary of change | - TransmissionControl_GrantedPartysId renamed to TransmissionControl_UserIdTransmittingUser - cs_TransmissionControl_GrantedPartysId renamed to cs_TransmissionControl_UserIdTransmittingUser - fl_TransmissionControl_GrantedPartysIdTX renamed to fl_TransmissionControl_UserIdTransmittingUser_TX |
| TTCN module | MCVideo_TransmissionControl_TypeDefs, MCVideo_TransmissionControl_Templates |

type record TransmissionControl_UserIdTransmittingUser{ O1_Type id, O1_Type lgth, octetstring val, octetstring padding };

template (value) TransmissionControl_UserIdTransmittingUser cs_TransmissionControl_UserIdTransmittingUser(integer p_Length, octetstring p_ServiceId) := { // 24.581 Table 9.2.3.6-1 id := '04'O, // 24.581 Table 9.2.3.1-2 lgth := int2oct(p_Length, 1), val := p_ServiceId, padding := f_MCX_MediaControlField_Padding(p_Length) };

function fl_TransmissionControl_UserIdTransmittingUser_TX(charstring p_ServiceId) return template (value) TransmissionControl_UserIdTransmittingUser { var integer v_Length := lengthof(p_ServiceId); var octetstring v_String := char2oct(p_ServiceId); return cs_TransmissionControl_UserIdTransmittingUser(v_Length, v_String); }

## Renaming of SSRC field (24.581 Clause 9.2.3.16), new field for video

| Reason for change | In 24.581 clause 9.2.3.16 "SSRC" has been renamed to "Audio SSRC of Transmitting Participant"; new field "Video SSRC of Transmitting Participant" added in 9.2.3.23 |
| --- | --- |
| Summary of change | Alias definitions and templates added |
| TTCN module | MCVideo_TransmissionControl_TypeDefs, MCVideo_TransmissionControl_Templates |

type TransmissionControl_SSRC TransmissionControl_AudioSSRC; type TransmissionControl_SSRC TransmissionControl_VideoSSRC;

template (present) TransmissionControl_AudioSSRC cr_TransmissionControl_AudioSSRC(template (present) O4_Type p_Val := ?) := cr_TransmissionControl_SSRC(p_Val); template (value) TransmissionControl_AudioSSRC cs_TransmissionControl_AudioSSRC(O4_Type p_Val) := cs_TransmissionControl_SSRC(p_Val);

## Transmission Granted

| Reason for change | "SSRC of granted transmission participant" replaced by "Audio SSRC of the Transmitting User" and "Video SSRC of the Transmitting User" |
| --- | --- |
| Summary of change | MCVideo_TransmissionControl_TransmissionGranted: - ssrcGrantedParticipant renamed to ssrcTransmittingUserAudio - ssrcTransmittingUserVideo added Templates updated accordingly |
| TTCN module | MCPTT_MediaControl_TypeDefs, MCPTT_MediaControl_Templates |

type set MCVideo_TransmissionControl_TransmissionGranted { TransmissionControl_CommonHdr commonHdr, TransmissionControl_Duration duration optional, TransmissionControl_AudioSSRC ssrcTransmittingUserAudio optional, TransmissionControl_VideoSSRC ssrcTransmittingUserVideo optional, TransmissionControl_TransmissionPriority transmissionPriority optional, TransmissionControl_UserID userId optional, TransmissionControl_QueueSize queueSize optional, TransmissionControl_SSRC ssrcQueuedParticipant optional, TransmissionControl_UserID queuedUserId optional, TransmissionControl_QueueInfo queueInfo optional, TransmissionControl_TransmissionIndicator transmissionIndicator optional };

template (value) MCVideo_TransmissionControlMsg_Type cs_TransmissionControlMsg_TransmissionGranted(RTP_SSRC p_RTCP_SSRC_SS, RTP_SSRC p_MediaSSRC_UE, B16_Type p_TransmissionInd, MCX_MediaControl_AckRequest_Type p_AckRequest := ACK_NOT_REQUIRED) := { // 36.579-1 Table 5.5.11.2.1-1 transmissionGranted := { commonHdr := cs_TransmissionControl_CommonHdr(f_MCX_MediaControlSubtype_TX('00000'B, p_AckRequest), p_RTCP_SSRC_SS, tsc_MCVideo_MediaControl_MCV1), duration := cs_TransmissionControl_Duration('0010'O), ssrcTransmittingUserAudio := cs_TransmissionControl_AudioSSRC(p_MediaSSRC_UE), ssrcTransmittingUserVideo := omit, transmissionPriority := omit, userId := omit, queueSize := omit, ssrcQueuedParticipant := omit, queuedUserId := omit, queueInfo := omit, transmissionIndicator := cs_TransmissionControl_TransmissionIndicator(p_TransmissionInd) } };

## Transmission Arbitration Taken

| Reason for change | "Granted Party's Identity" renamed to "User Id of the Transmitting User"; "SSRC of granted transmission participant" replaced by "Audio SSRC of the Transmitting User" and "Video SSRC of the Transmitting User" |
| --- | --- |
| Summary of change | MCVideo_TransmissionControl_TransmissionArbitrationTaken: - grantedPartysId renamed to userIdTransmittingUser - ssrcGrantedParticipant renamed to ssrcTransmittingUserAudio - ssrcTransmittingUserVideo added Templates updated accordingly |
| TTCN module | MCPTT_MediaControl_TypeDefs, MCPTT_MediaControl_Templates |

type set MCVideo_TransmissionControl_TransmissionArbitrationTaken { TransmissionControl_CommonHdr commonHdr, TransmissionControl_UserIdTransmittingUser userIdTransmittingUser optional, TransmissionControl_PermissionToRequest permissionToRequest optional, TransmissionControl_UserID userId optional, TransmissionControl_MsgSeqNum msgSeqNum optional, TransmissionControl_TransmissionIndicator transmissionIndicator optional, TransmissionControl_AudioSSRC ssrcTransmittingUserAudio optional, TransmissionControl_VideoSSRC ssrcTransmittingUserVideo optional };

template (value) MCVideo_TransmissionControlMsg_Type cs_TransmissionControlMsg_TransmissionArbitrationTaken(RTP_SSRC p_RTCP_SSRC_SS, integer p_MsgSeqNum, B16_Type p_TransmissionInd, MCX_MediaControl_AckRequest_Type p_AckRequest := ACK_NOT_REQUIRED) := { // 36.579-1 Table 5.5.11.2.3-1 transmissionArbitrationTaken := { commonHdr := cs_TransmissionControl_CommonHdr(f_MCX_MediaControlSubtype_TX('00010'B, p_AckRequest), p_RTCP_SSRC_SS, tsc_MCVideo_MediaControl_MCV1), userIdTransmittingUser := fl_TransmissionControl_UserIdTransmittingUser_TX(px_MCVideo_ID_User_A), permissionToRequest := cs_TransmissionControl_PermissionToRequestTransmission('0001'O), userId := omit, msgSeqNum := cs_TransmissionControl_MsgSeqNum(p_MsgSeqNum), transmissionIndicator := cs_TransmissionControl_TransmissionIndicator(p_TransmissionInd), ssrcTransmittingUserAudio := cs_TransmissionControl_AudioSSRC(tsc_MCX_MediaSSRC_ClientB), ssrcTransmittingUserVideo := omit } };

## Media transmission notification

| Reason for change | "User ID" changed to "User Id of the Transmitting User"; "SSRC of granted transmission participant" replaced by "Audio SSRC of the Transmitting User" and "Video SSRC of the Transmitting User" |
| --- | --- |
| Summary of change | MCVideo_TransmissionControl_MediaTransmissionNotification: - userId (TransmissionControl_UserID) changed to userIdTransmittingUser (TransmissionControl_UserIdTransmittingUser) - ssrcTransmitter renamed to ssrcTransmittingUserAudio - ssrcTransmittingUserVideo added Templates updated accordingly NOTE: Change of the "User ID" changed to "User Id of the Transmitting User" is not backward compatible as the fields have different field ids. |
| TTCN module | MCPTT_MediaControl_TypeDefs, MCPTT_MediaControl_Templates |

type set MCVideo_TransmissionControl_MediaTransmissionNotification { TransmissionControl_CommonHdr commonHdr, TransmissionControl_UserIdTransmittingUser userIdTransmittingUser optional, TransmissionControl_AudioSSRC ssrcTransmittingUserAudio optional, TransmissionControl_VideoSSRC ssrcTransmittingUserVideo optional, TransmissionControl_PermissionToRequest permissionToRequest optional, TransmissionControl_TransmissionIndicator transmissionIndicator optional, TransmissionControl_TrackInfo trackInfo optional, TransmissionControl_FunctionalAlias functionalAlias optional, // Rel-17 TransmissionControl_ReceptionMode receptionMode optional };

template (value) MCVideo_TransmissionControlMsg_Type cs_TransmissionControlMsg_MediaTransmissionNotification(RTP_SSRC p_RTCP_SSRC_SS, B16_Type p_TransmissionInd, charstring p_UserID_Transmitter, RTP_SSRC p_SSRC_Transmitter, O2_Type p_PermissionToRequestTransmissionVal, O2_Type p_ReceptionModeVal, MCX_MediaControl_AckRequest_Type p_AckRequest := ACK_NOT_REQUIRED) := { // 36.579-1 Table 5.5.11.2.7-1 mediaTransmissionNotification := { commonHdr := cs_TransmissionControl_CommonHdr(f_MCX_MediaControlSubtype_TX('00110'B, p_AckRequest), p_RTCP_SSRC_SS, tsc_MCVideo_MediaControl_MCV1), userIdTransmittingUser := fl_TransmissionControl_UserIdTransmittingUser_TX(p_UserID_Transmitter), ssrcTransmittingUserAudio := cs_TransmissionControl_AudioSSRC(p_SSRC_Transmitter), ssrcTransmittingUserVideo := omit, permissionToRequest := cs_TransmissionControl_PermissionToRequestTransmission(p_PermissionToRequestTransmissionVal), transmissionIndicator := cs_TransmissionControl_TransmissionIndicator(p_TransmissionInd), trackInfo := omit, functionalAlias := omit, receptionMode := cs_TransmissionControl_ReceptionMode(p_ReceptionModeVal) } };

## Receive media request

| Reason for change | "User Id of the Transmitting User" added; "SSRC of granted transmission participant" replaced by "Audio SSRC of the Transmitting User" and "Video SSRC of the Transmitting User"; clarification is added that the "User ID" is off-network only |
| --- | --- |
| Summary of change | MCVideo_TransmissionControl_ReceiveMediaRequest: - userIdTransmittingUser added - ssrcTransmitter renamed to ssrcTransmittingUserAudio - ssrcTransmittingUserVideo added Templates updated accordingly |
| TTCN module | MCPTT_MediaControl_TypeDefs, MCPTT_MediaControl_Templates |

type set MCVideo_TransmissionControl_ReceiveMediaRequest { TransmissionControl_CommonHdr commonHdr, TransmissionControl_UserID userId optional, // off-network only TransmissionControl_UserIdTransmittingUser userIdTransmittingUser optional, TransmissionControl_AudioSSRC ssrcTransmittingUserAudio optional, TransmissionControl_VideoSSRC ssrcTransmittingUserVideo optional, TransmissionControl_TransmissionIndicator transmissionIndicator optional, TransmissionControl_ReceptionPriority receptionPriority optional, TransmissionControl_TrackInfo trackInfo optional, TransmissionControl_FunctionalAlias functionalAlias optional // Rel-17 };

template (present) MCVideo_TransmissionControlMsg_Type cr_TransmissionControlMsg_ReceiveMediaRequest(template (present) RTP_SSRC p_RTCP_SSRC_UE, RTP_SSRC p_SSRC_Transmitter, template (present) B16_Type p_TransmissionInd) := { // 36.579-1 Table 5.5.11.1.4-1 receiveMediaRequest := { commonHdr := cr_TransmissionControl_CommonHdr('?0100'B, p_RTCP_SSRC_UE, tsc_MCVideo_MediaControl_MCV0), userId := omit, userIdTransmittingUser := *, ssrcTransmittingUserAudio := cr_TransmissionControl_AudioSSRC(p_SSRC_Transmitter), ssrcTransmittingUserVideo := *, transmissionIndicator := cr_TransmissionControl_TransmissionIndicator(p_TransmissionInd), receptionPriority := cr_TransmissionControl_ReceptionPriority ifpresent, trackInfo := omit, functionalAlias := omit } };

## Receive media response

| Reason for change | "User Id of the Transmitting User" added; "SSRC of granted transmission participant" replaced by "Audio SSRC of the Transmitting User" and "Video SSRC of the Transmitting User" |
| --- | --- |
| Summary of change | MCVideo_TransmissionControl_ReceiveMediaResponse: - userIdTransmittingUser added - ssrcTransmitter renamed to ssrcTransmittingUserAudio - ssrcTransmittingUserVideo added Templates updated accordingly |
| TTCN module | MCPTT_MediaControl_TypeDefs, MCPTT_MediaControl_Templates |

type set MCVideo_TransmissionControl_ReceiveMediaResponse { TransmissionControl_CommonHdr commonHdr, TransmissionControl_Result result optional, TransmissionControl_RejectCause rejectCause optional, TransmissionControl_UserIdTransmittingUser userIdTransmittingUser optional, TransmissionControl_AudioSSRC ssrcTransmittingUserAudio optional, TransmissionControl_VideoSSRC ssrcTransmittingUserVideo optional, TransmissionControl_TransmissionIndicator transmissionIndicator optional };

template (value) MCVideo_TransmissionControlMsg_Type cs_TransmissionControlMsg_ReceiveMediaResponse(RTP_SSRC p_RTCP_SSRC_SS, B16_Type p_TransmissionInd, RTP_SSRC p_SSRC_Transmitter, MCX_MediaControl_AckRequest_Type p_AckRequest := ACK_NOT_REQUIRED) := { // 36.579-1 Table 5.5.11.2.8-1 receiveMediaResponse := { commonHdr := cs_TransmissionControl_CommonHdr(f_MCX_MediaControlSubtype_TX('00111'B, p_AckRequest), p_RTCP_SSRC_SS, tsc_MCVideo_MediaControl_MCV1), result := cs_TransmissionControl_Result('0001'O), rejectCause := omit, userIdTransmittingUser := omit, ssrcTransmittingUserAudio := cs_TransmissionControl_AudioSSRC(p_SSRC_Transmitter), ssrcTransmittingUserVideo := omit, transmissionIndicator := cs_TransmissionControl_TransmissionIndicator(p_TransmissionInd) } };

## Media reception notification

| Reason for change | "Audio SSRC of the Transmitting User" and "Video SSRC of the Transmitting User" added |
| --- | --- |
| Summary of change | MCVideo_TransmissionControl_MediaReceptionNotification: - ssrcTransmittingUserAudio added - ssrcTransmittingUserVideo added Templates updated accordingly |
| TTCN module | MCPTT_MediaControl_TypeDefs, MCPTT_MediaControl_Templates |

type set MCVideo_TransmissionControl_MediaReceptionNotification { TransmissionControl_CommonHdr commonHdr, TransmissionControl_UserID userId optional, TransmissionControl_AudioSSRC ssrcTransmittingUserAudio optional, TransmissionControl_VideoSSRC ssrcTransmittingUserVideo optional, TransmissionControl_FunctionalAlias functionalAlias optional // Rel-17 };

template (value) MCVideo_TransmissionControlMsg_Type cs_TransmissionControlMsg_MediaReceptionNotification(RTP_SSRC p_RTCP_SSRC_SS, MCX_MediaControl_AckRequest_Type p_AckRequest := ACK_NOT_REQUIRED, charstring p_UserID_Receiver := px_MCVideo_ID_User_B) := { // 36.579-1 Table 5.5.11.2.9-1 mediaReceptionNotification := { commonHdr := cs_TransmissionControl_CommonHdr(f_MCX_MediaControlSubtype_TX('01000'B, p_AckRequest), p_RTCP_SSRC_SS, tsc_MCVideo_MediaControl_MCV1), userId := fl_TransmissionControl_UserID_TX(p_UserID_Receiver), ssrcTransmittingUserAudio := omit, ssrcTransmittingUserVideo := omit, functionalAlias := omit } };

## Transmission end request

| Reason for change | "User ID" changed to "User Id of the Transmitting User"; "Audio SSRC of the Transmitting User" and "Video SSRC of the Transmitting User" added |
| --- | --- |
| Summary of change | MCVideo_TransmissionControl_TransmissionEndRequest: - userId (TransmissionControl_UserID) changed to userIdTransmittingUser (TransmissionControl_UserIdTransmittingUser) - ssrcTransmittingUserAudio added - ssrcTransmittingUserVideo added Templates updated accordingly NOTE: Change of the "User ID" changed to "User Id of the Transmitting User" is not backward compatible as the fields have different field ids. In UL as possible workaround TTCN may accept both field ids (FFS) |
| TTCN module | MCPTT_MediaControl_TypeDefs, MCPTT_MediaControl_Templates |

type set MCVideo_TransmissionControl_TransmissionEndRequest { TransmissionControl_CommonHdr commonHdr, TransmissionControl_UserIdTransmittingUser userIdTransmittingUser optional, TransmissionControl_AudioSSRC ssrcTransmittingUserAudio optional, TransmissionControl_VideoSSRC ssrcTransmittingUserVideo optional, TransmissionControl_RejectCause rejectCause optional };

template (present) MCVideo_TransmissionControlMsg_Type cr_TransmissionControlMsg_TransmissionEndRequest(template (present) RTP_SSRC p_RTCP_SSRC_UE, charstring p_UserID) := { transmissionEndRequest := { commonHdr := cr_TransmissionControl_CommonHdr('?0000'B, p_RTCP_SSRC_UE, tsc_MCVideo_MediaControl_MCV2), userIdTransmittingUser := fl_TransmissionControl_UserIdTransmittingUser_RX(p_UserID), ssrcTransmittingUserAudio := *, ssrcTransmittingUserVideo := *, rejectCause := omit } };

template (value) MCVideo_TransmissionControlMsg_Type cs_TransmissionControlMsg_TransmissionEndRequest(RTP_SSRC p_RTCP_SSRC_SS, charstring p_UserID, O2_Type p_RejectCause, charstring p_ReasonPhrase, MCX_MediaControl_AckRequest_Type p_AckRequest := ACK_NOT_REQUIRED) := { transmissionEndRequest := { commonHdr := cs_TransmissionControl_CommonHdr(f_MCX_MediaControlSubtype_TX('00000'B, p_AckRequest), p_RTCP_SSRC_SS, tsc_MCVideo_MediaControl_MCV2), userIdTransmittingUser := fl_TransmissionControl_UserIdTransmittingUser_TX(p_UserID), ssrcTransmittingUserAudio := omit, ssrcTransmittingUserVideo := omit, rejectCause := fl_TransmissionControl_RejectCauseTX(p_RejectCause, p_ReasonPhrase) } };

## Transmission end response

| Reason for change | "User ID" changed to "User Id of the Transmitting User"; "SSRC of granted transmission participant" replaced by "Audio SSRC of the Transmitting User" and "Video SSRC of the Transmitting User" |
| --- | --- |
| Summary of change | MCVideo_TransmissionControl_TransmissionEndResponse: - userId (TransmissionControl_UserID) changed to userIdTransmittingUser (TransmissionControl_UserIdTransmittingUser) - ssrcTransmittingUserAudio added - ssrcTransmittingUserVideo added Templates updated accordingly NOTE: Change of the "User ID" changed to "User Id of the Transmitting User" is not backward compatible as the fields have different field ids. In UL as possible workaround TTCN may accept both field ids (FFS) |
| TTCN module | MCPTT_MediaControl_TypeDefs, MCPTT_MediaControl_Templates |

type set MCVideo_TransmissionControl_TransmissionEndResponse { TransmissionControl_CommonHdr commonHdr, TransmissionControl_UserIdTransmittingUser userIdTransmittingUser optional, TransmissionControl_AudioSSRC ssrcTransmittingUserAudio optional, TransmissionControl_VideoSSRC ssrcTransmittingUserVideo optional };

template (present) MCVideo_TransmissionControlMsg_Type cr_TransmissionControlMsg_TransmissionEndResponse(template (present) RTP_SSRC p_RTCP_SSRC_UE, charstring p_UserID) := { transmissionEndResponse := { commonHdr := cr_TransmissionControl_CommonHdr('?0001'B, p_RTCP_SSRC_UE, tsc_MCVideo_MediaControl_MCV2), userIdTransmittingUser := fl_TransmissionControl_UserIdTransmittingUser_RX(p_UserID), ssrcTransmittingUserAudio := *, ssrcTransmittingUserVideo := * } };

template (value) MCVideo_TransmissionControlMsg_Type cs_TransmissionControlMsg_TransmissionEndResponse(RTP_SSRC p_RTCP_SSRC_SS, charstring p_UserID, MCX_MediaControl_AckRequest_Type p_AckRequest := ACK_NOT_REQUIRED) := { transmissionEndResponse := { commonHdr := cs_TransmissionControl_CommonHdr(f_MCX_MediaControlSubtype_TX('00001'B, p_AckRequest), p_RTCP_SSRC_SS, tsc_MCVideo_MediaControl_MCV2), userIdTransmittingUser := fl_TransmissionControl_UserIdTransmittingUser_TX(p_UserID), ssrcTransmittingUserAudio := omit, ssrcTransmittingUserVideo := omit } };

## Media reception end request

| Reason for change | "User Id of the Transmitting User" added; "SSRC of granted transmission participant" replaced by "Audio SSRC of the Transmitting User" and "Video SSRC of the Transmitting User" |
| --- | --- |
| Summary of change | MCVideo_TransmissionControl_ReceiveMediaRequest: - userIdTransmittingUser added - ssrcTransmitter renamed to ssrcTransmittingUserAudio - ssrcTransmittingUserVideo added Templates updated accordingly |
| TTCN module | MCPTT_MediaControl_TypeDefs, MCPTT_MediaControl_Templates |

type set MCVideo_TransmissionControl_MediaReceptionEndRequest { TransmissionControl_CommonHdr commonHdr, TransmissionControl_UserIdTransmittingUser userIdTransmittingUser optional, TransmissionControl_AudioSSRC ssrcTransmittingUserAudio optional, TransmissionControl_VideoSSRC ssrcTransmittingUserVideo optional TransmissionControl_TransmissionIndicator transmissionIndicator optional };

template (present) MCVideo_TransmissionControlMsg_Type cr_TransmissionControlMsg_MediaReceptionEndRequest(template (present) RTP_SSRC p_RTCP_SSRC_UE, template (present) RTP_SSRC p_SSRC_Transmitter, template (present) B16_Type p_TransmissionInd) := { mediaReceptionEndRequest := { commonHdr := cr_TransmissionControl_CommonHdr('?0010'B, p_RTCP_SSRC_UE, tsc_MCVideo_MediaControl_MCV2), userIdTransmittingUser := *, ssrcTransmittingUserAudio := cr_TransmissionControl_AudioSSRC(p_SSRC_Transmitter), ssrcTransmittingUserVideo := *, transmissionIndicator := cr_TransmissionControl_TransmissionIndicator(p_TransmissionInd) } };

template (value) MCVideo_TransmissionControlMsg_Type cs_TransmissionControlMsg_MediaReceptionEndRequest(RTP_SSRC p_RTCP_SSRC_SS, B16_Type p_TransmissionInd, RTP_SSRC p_SSRC_Transmitter, MCX_MediaControl_AckRequest_Type p_AckRequest := ACK_NOT_REQUIRED) := { mediaReceptionEndRequest := { commonHdr := cs_TransmissionControl_CommonHdr(f_MCX_MediaControlSubtype_TX('00010'B, p_AckRequest), p_RTCP_SSRC_SS, tsc_MCVideo_MediaControl_MCV2), userIdTransmittingUser := omit, ssrcTransmittingUserAudio := cr_TransmissionControl_AudioSSRC(p_SSRC_Transmitter), ssrcTransmittingUserVideo := omit, transmissionIndicator := cs_TransmissionControl_TransmissionIndicator(p_TransmissionInd) } };

## Media reception end response

| Reason for change | "User Id of the Transmitting User" added; "SSRC of granted transmission participant" replaced by "Audio SSRC of the Transmitting User" and "Video SSRC of the Transmitting User" |
| --- | --- |
| Summary of change | MCVideo_TransmissionControl_ReceiveMediaRequest: - userIdTransmittingUser added - ssrcTransmitter renamed to ssrcTransmittingUserAudio - ssrcTransmittingUserVideo added Templates updated accordingly |
| TTCN module | MCPTT_MediaControl_TypeDefs, MCPTT_MediaControl_Templates |

type set MCVideo_TransmissionControl_MediaReceptionEndResponse { TransmissionControl_CommonHdr commonHdr, TransmissionControl_UserIdTransmittingUser userIdTransmittingUser optional, TransmissionControl_AudioSSRC ssrcTransmittingUserAudio optional, TransmissionControl_VideoSSRC ssrcTransmittingUserVideo optional };

template (present) MCVideo_TransmissionControlMsg_Type cr_TransmissionControlMsg_MediaReceptionEndResponse(template (present) RTP_SSRC p_RTCP_SSRC_UE, template (present) RTP_SSRC p_SSRC_Transmitter) := { mediaReceptionEndResponse := { commonHdr := cr_TransmissionControl_CommonHdr('?0011'B, p_RTCP_SSRC_UE, tsc_MCVideo_MediaControl_MCV2), userIdTransmittingUser := *, ssrcTransmittingUserAudio := cr_TransmissionControl_AudioSSRC(p_SSRC_Transmitter), ssrcTransmittingUserVideo := * } };

template (value) MCVideo_TransmissionControlMsg_Type cs_TransmissionControlMsg_MediaReceptionEndResponse(RTP_SSRC p_RTCP_SSRC_SS, RTP_SSRC p_SSRC_Transmitter, MCX_MediaControl_AckRequest_Type p_AckRequest := ACK_NOT_REQUIRED) := { mediaReceptionEndResponse := { commonHdr := cs_TransmissionControl_CommonHdr(f_MCX_MediaControlSubtype_TX('00011'B, p_AckRequest), p_RTCP_SSRC_SS, tsc_MCVideo_MediaControl_MCV2), userIdTransmittingUser := omit, ssrcTransmittingUserAudio := cr_TransmissionControl_AudioSSRC(p_SSRC_Transmitter), ssrcTransmittingUserVideo := omit } };

## Transmission end notify

| Reason for change | "User ID" changed to "User Id of the Transmitting User"; "SSRC of granted transmission participant" replaced by "Audio SSRC of the Transmitting User" and "Video SSRC of the Transmitting User" |
| --- | --- |
| Summary of change | MCVideo_TransmissionControl_TransmissionEndNotify : - userId (TransmissionControl_UserID) changed to userIdTransmittingUser (TransmissionControl_UserIdTransmittingUser) - ssrcTransmitter renamed to ssrcTransmittingUserAudio - ssrcTransmittingUserVideo added Templates updated accordingly NOTE: Change of the "User ID" changed to "User Id of the Transmitting User" is not backward compatible as the fields have different field ids. |
| TTCN module | MCPTT_MediaControl_TypeDefs, MCPTT_MediaControl_Templates |

type set MCVideo_TransmissionControl_TransmissionEndNotify { TransmissionControl_CommonHdr commonHdr, TransmissionControl_UserIdTransmittingUser userIdTransmittingUser optional, TransmissionControl_AudioSSRC ssrcTransmittingUserAudio optional, TransmissionControl_VideoSSRC ssrcTransmittingUserVideo optional };

template (value) MCVideo_TransmissionControlMsg_Type cs_TransmissionControlMsg_TransmissionEndNotify(RTP_SSRC p_RTCP_SSRC_SS, RTP_SSRC p_SSRC_Transmitter := tsc_MCX_MediaSSRC_ClientB, MCX_MediaControl_AckRequest_Type p_AckRequest := ACK_NOT_REQUIRED) := { // 36.579-1 Table 5.5.11.2.15-1 transmissionEndNotify := { commonHdr := cs_TransmissionControl_CommonHdr(f_MCX_MediaControlSubtype_TX('01110'B, p_AckRequest), p_RTCP_SSRC_SS, tsc_MCVideo_MediaControl_MCV1), userIdTransmittingUser := fl_TransmissionControl_UserIdTransmittingUser_TX(px_MCVideo_ID_User_A), ssrcTransmittingUserAudio := cs_TransmissionControl_AudioSSRC(p_SSRC_Transmitter), ssrcTransmittingUserVideo := omit } };

## MCData messages (24.282 V18.7.0 clause 15)

## MCData_FD_SIGNALLING_PAYLOAD: New field User location

| Reason for change | Change of 24.282 Table 15.1.3.1-1 |
| --- | --- |
| Summary of change | Update of TTCN Definitions |
| TTCN module | MCDataMessage_TypeDefs, MCData_MCDataMessages |

type record MCData_FD_SIGNALLING_PAYLOAD { // Table 15.1.3.1-1 MCData_MessageType messageType, MCData_DateAndTime dateAndTime, MCData_ConversationID conversationID, MCData_MessageID messageID, MCData_InReplyToMessageID inReplyTo optional, MCData_ApplicationID applicationID optional, MCData_FD_DispositionRequestType dispositionRequestType optional, MCData_MandatoryDownload mandatoryDownload optional, MCData_Payload payload optional, MCData_Metadata metadata optional, MCData_ExtendedApplicationID extendedApplicationID optional, MCData_UserLocation userLocation optional, // Rel-18 MCData_UserID senderUserID optional, MCData_ApplicationMetadata applicationMetadata optional // Rel-17 };

template (value) MCData_Message cs_MCData_FD_SIGNALLING_PAYLOAD(integer p_TimestampSeconds, MCData_ConversationID p_ConversationID, MCData_MessageID p_MessageID, template (omit) MCData_FD_DispositionRequestType p_DispositionRequestType, template (omit) MCData_MandatoryDownload p_MandatoryDownload := omit, template (omit) MCData_Payload p_Payload := omit, template (omit) MCData_Metadata p_Metadata := omit) := { FD_SIGNALLING_PAYLOAD := { messageType := oct2bit('02'O), dateAndTime := int2oct(p_TimestampSeconds, 5), conversationID := p_ConversationID, messageID := p_MessageID, inReplyTo := omit, applicationID := omit, dispositionRequestType := p_DispositionRequestType, mandatoryDownload := p_MandatoryDownload, payload := p_Payload, metadata := p_Metadata, extendedApplicationID := omit, userLocation := omit, senderUserID := omit, applicationMetadata := omit // @sic R5s221241: BASELINE MOVING 2022 09: new field sic@ } };

template (present) MCData_Message cr_MCData_FD_SIGNALLING_PAYLOAD(template MCData_FD_DispositionRequestType p_DispositionRequestType := cr_MCData_FD_DispositionRequestType, template MCData_MandatoryDownload p_MandatoryDownload := omit, template MCData_Payload p_Payload := omit, template MCData_Metadata p_Metadata := omit) := { FD_SIGNALLING_PAYLOAD := { messageType := oct2bit('02'O), dateAndTime := ?, conversationID := ?, messageID := ?, inReplyTo := omit, applicationID := omit, dispositionRequestType := p_DispositionRequestType, mandatoryDownload := p_MandatoryDownload, payload := p_Payload, metadata := p_Metadata, extendedApplicationID := omit, userLocation := *, senderUserID := omit, applicationMetadata := cr_MCData_ApplicationMetadata ifpresent // @sic R5s221241: BASELINE MOVING 2022 09: new field sic@ } };

# Further changes

## Use of @fuzzy in TTCN

| Reason for change | @fuzzy is still not supported by all tools, i.e. even though due to the @fuzzy modifier cs_MCPTT_UserProfile should be evaluated at the time it is used, still some tools evaluate especially the elem_list at initialisation (i.e. depending on the tool implementation before start of a test case) call of testcasename() in fl_MCPTT_UserProfile_Extension may return "" and is not re-evaluated during a test case |
| --- | --- |
| Summary of change | @fuzzy modifier removed from cs_MCPTT_UserProfile, but fl_MCPTT_UserProfile_Extension() handed over as parameter instead so that tools are enforced to evaluate fl_MCPTT_UserProfile_Extension when cs_MCPTT_UserProfile is used in the test case. |
| TTCN module | MCX_XML_Configuration_MCPTT |

template (value) MCPTT_UserProfile_Type cs_MCPTT_UserProfile(template (value) MCPTT_UserProfile_OnNetwork_Type p_OnNetwork := cs_MCPTT_UserProfile_OnNetwork, MCPTT_UserProfile_Extension_Type p_Extension) := { xUI_URI := "sip:" & px_MCPTT_ID_User_A, user_profile_index := tsc_MCX_ProfileId_MCPTT, attr := omit, choice_list := { { status := true }, { profileName := cs_MCPTT_UserProfile_Name("mcptt-user-profile-" & int2str(tsc_MCX_ProfileId_MCPTT) & ".xml") }, { offNetwork := cs_MCPTT_UserProfile_OffNetwork }, { onNetwork := p_OnNetwork }, { elem_list := p_Extension } } };

function f_MCX_HTTP_ConfigurationGroupManagementSubscription(MCX_ServiceType_Type p_ServiceType, MCX_SecurityParameters_Type p_SecurityParameters) runs on MCX_HTTP_PTC { … select (p_ServiceType) { case (MCPTT) { v_MimeTypeUEConfiguration := tsc_MimeType_McpttUEConfig; v_MimeTypeUserProfile := tsc_MimeType_McpttUserProfile; v_MimeTypeServConfiguration := tsc_MimeType_McpttServConfig; v_EncodedUEConfiguration := f_MCPTT_UEConfiguration_Encode(cs_MCPTT_UEConfiguration); v_EncodedUserProfile := f_MCPTT_UserProfile_Encode(cs_MCPTT_UserProfile(-, f_MCPTT_UserProfile_Extension())); v_EncodedServiceConfiguration := f_MCPTT_ServiceConfiguration_Encode(cs_MCPTT_ServiceConfiguration); v_GroupConfiguration := f_MCPTT_GroupConfiguration(); } … } … }