3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240538

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3510 | rev | - | Current version: | 17.11.0 |  |
| For HELPon using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed changeaffects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcase 7.1.1.2.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Starpoint |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In current TTCN implementation, pdsch-AggregationFactor=n4 is configured in UE before SS. When SS send the RLC status report for RRCReconfigurationComplete message, the new configuration has already applied in UE but not in SS. UE will try to access to network again after received the unexpected RLC status report and lead to test case error. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Try to Configure pdsch-AggregationFactor=n4 in UE and SS at the same time. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test cases. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.2.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

- Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2024-06_D24wk32 related to the title of this CR.

Contact:	Ting Gao
	gaoting@starpointcomm.com

- Corrections required

  - Change 1

| Functionname | fl_TC_7_1_1_2_2_NR5GC_TestBody |
| --- | --- |
| Reason for change | In current TTCN implementation, pdsch-AggregationFactor=n4 is configured in UE before SS. When SS send the RLC status report for RRCReconfigurationComplete message, the new configuration has already applied in UE but not in SS. UE will try to access to network again after received the unexpected RLC status report and lead to test case error. |
| Summary of change | Try to Configure pdsch-AggregationFactor=n4 in UE and SS at the same time. |
| TTCN module | MAC_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function fl_TC_7_1_1_2_2_NR5GC_TestBody(DRB_Identity p_NR_DRB_Id) runs on NR_BASE_PTC { //Correct Handling of DL HARQ process PDSCH Aggregation var template (value) RRCReconfiguration_v1530_IEs v_V1530Ext; var template (omit) NR_BWP_DownlinkDedicated_PDSCH_Config_Type v_BWP_DownlinkDedicated_PDSCH_Config; var template (value) PDSCH_Config v_PDSCH_Config; var template (value) NR_DownlinkBWP_List_Type v_NR_DownlinkBWP_List; var template (value) CellGroupConfig v_CellGroupConfig; var template (value) NR_RadioBearerList_Type v_SS_Drb_ConfigList; //@sic R5s221035 sic@ //@siclog "Steps 1-2" siclog@ // SS transmits in the indicated downlink assignment an NR RRCReconfiguration. (Note 1) // UE transmits NR RRCReconfigurationComplete message to the SS. (Note 2) //Configure pdsch-AggregationFactor=n4 in UE and SS v_PDSCH_Config := cs_NR_PDSCH_ConfigDef( -, omit, omit, -, -, -, -, -, omit, -, n4); v_BWP_DownlinkDedicated_PDSCH_Config := {setup := v_PDSCH_Config}; v_CellGroupConfig := cs_38508_CellGroupConfig(tsc_NR_CellGroupId_MCG,-,-,-,-, cs_NR_SpCellConfigDef(omit, -, -, cs_NR_ServingCellConfigDef(cs_BWP_DownlinkDedicated_Def(-, v_BWP_DownlinkDedicated_PDSCH_Config)) ) ); v_V1530Ext := cs_NR_RRCReconfiguration_v1530_IEs(bit2oct(encvalue(v_CellGroupConfig))); v_NR_DownlinkBWP_List := f_NR_CellInfo_GetDownlinkBWP_List(nr_Cell1); v_NR_DownlinkBWP_List[0].Pdsch.ConfigDedicated.R15.pdsch_AggregationFactor := n4; f_NR_SendRRCReconfiguration(nr_Cell1, -,-, v_V1530Ext); f_NR_SS_CommonCellConfig(nr_Cell1, cads_NR_DownlinkBWP_ConfigCommon_REQ(nr_Cell1, -, v_NR_DownlinkBWP_List)); // Configure MAC in TestMode - after signalling on SRBs //@sic R5s221035 sic@ v_SS_Drb_ConfigList := {cs_NR_SS_RadioBearer_MAC_TestMode(p_NR_DRB_Id, cs_NR_MAC_TestModeInfo_NoHeaderManipulationDLOnly)}; f_NR_SS_CommonRadioBearerConfig(nr_Cell1, v_SS_Drb_ConfigList); f_TC_7_1_1_2_2_NR_TestBody(p_NR_DRB_Id); f_NR_ULGrantConfiguration_Start(nr_Cell1); //@sic R5s221035 sic@ }

After Change:

function fl_TC_7_1_1_2_2_NR5GC_TestBody(DRB_Identity p_NR_DRB_Id) runs on NR_BASE_PTC { //Correct Handling of DL HARQ process PDSCH Aggregation var template (value) RRCReconfiguration_v1530_IEs v_V1530Ext; var template (omit) NR_BWP_DownlinkDedicated_PDSCH_Config_Type v_BWP_DownlinkDedicated_PDSCH_Config; var template (value) PDSCH_Config v_PDSCH_Config; var template (value) NR_DownlinkBWP_List_Type v_NR_DownlinkBWP_List; var template (value) CellGroupConfig v_CellGroupConfig; var template (value) NR_RadioBearerList_Type v_SS_Drb_ConfigList; //@sic R5s221035 sic@ var SubFrameTiming_Type v_TimingDL; //@siclog "Steps 1-2" siclog@ // SS transmits in the indicated downlink assignment an NR RRCReconfiguration. (Note 1) // UE transmits NR RRCReconfigurationComplete message to the SS. (Note 2) //Configure pdsch-AggregationFactor=n4 in UE and SS v_PDSCH_Config := cs_NR_PDSCH_ConfigDef( -, omit, omit, -, -, -, -, -, omit, -, n4); v_BWP_DownlinkDedicated_PDSCH_Config := {setup := v_PDSCH_Config}; v_CellGroupConfig := cs_38508_CellGroupConfig(tsc_NR_CellGroupId_MCG,-,-,-,-, cs_NR_SpCellConfigDef(omit, -, -, cs_NR_ServingCellConfigDef(cs_BWP_DownlinkDedicated_Def(-, v_BWP_DownlinkDedicated_PDSCH_Config)) ) ); v_V1530Ext := cs_NR_RRCReconfiguration_v1530_IEs(bit2oct(encvalue(v_CellGroupConfig))); v_NR_DownlinkBWP_List := f_NR_CellInfo_GetDownlinkBWP_List(nr_Cell1); v_NR_DownlinkBWP_List[0].Pdsch.ConfigDedicated.R15.pdsch_AggregationFactor := n4; v_TimingDL := f_NR_GetNextSendOccasion_DL(nr_Cell1); f_NR_SendRRCReconfiguration(nr_Cell1, -,-, v_V1530Ext,-,-,cs_TimingInfo_NR(v_TimingDL),false); v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,10); f_NR_SS_CommonCellConfig(nr_Cell1, cads_NR_DownlinkBWP_ConfigCommon_REQ(nr_Cell1, cs_TimingInfo_NR(v_TimingDL),v_NR_DownlinkBWP_List)); SRB.receive(car_NR_SRB_RrcPdu_IND(nr_Cell1, tsc_NR_RbId_SRB1, cr_38508_RRCReconfigurationComplete)); // Configure MAC in TestMode - after signalling on SRBs //@sic R5s221035 sic@ v_SS_Drb_ConfigList := {cs_NR_SS_RadioBearer_MAC_TestMode(p_NR_DRB_Id, cs_NR_MAC_TestModeInfo_NoHeaderManipulationDLOnly)}; f_NR_SS_CommonRadioBearerConfig(nr_Cell1, v_SS_Drb_ConfigList); f_TC_7_1_1_2_2_NR_TestBody(p_NR_DRB_Id); f_NR_ULGrantConfiguration_Start(nr_Cell1); //@sic R5s221035 sic@ }