3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240540

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3512 | rev | - | Current version: | 17.11.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR5GC test case 9.1.5.1.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD, |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Agreed CR R5-242487 makes the new Step 29 “Cause the UE to request connectivity to an PDU session (Note 1)” a mandatory step, in which case UE will always trigger a PDU Sessions Establishment Request, which in turn needs to be properly processed and responded to by the NW at Step 37. Otherwise, if NW doesn’t respond and moves to Postamble then test will FAIL since UE will retransmit PDU Session Establishment Request. |  |  |  |  |  |  |  |  |
| Summary of change: |  | By default, the function f_NR5GC_PDUSessionEstablishment uses the value of pixit pc_NoOf_PDUsSameConnection to determine expected PDU session establishment messages, but in this case there is a UE initiated PDU session so even if pixit is set to 0, NW needs to process the PDU Session Establishment Procedure. Therefore, the argument value for this function in this test case is modified to 1 in anticipation of the PDU Session Establishment Request as triggered in Step 29. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.5.1.4.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D24wk24 related to the title of this CR.

Contact:	Kalahasti, Narendra

Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | function fl_TC_9_1_5_1_4_Body |
| --- | --- |
| Reason for change | Agreed CR R5-242487 makes the new Step 29 “Cause the UE to request connectivity to an PDU session (Note 1)” a mandatory step, in which case UE will always trigger a PDU Sessions Establishment Request, which in turn needs to be properly processed and responded to by the NW at Step 37. Otherwise, if NW doesn’t respond and moves to Postamble then test will FAIL since UE will retransmit PDU Session Establishment Request. |
| Summary of change | By default, the function f_NR5GC_PDUSessionEstablishment uses the value of pixit pc_NoOf_PDUsSameConnection to determine expected PDU session establishment messages, but in this case there is a UE initiated PDU session so even if pixit is set to 0, NW needs to process the PDU Session Establishment Procedure. Therefore, the argument value for this function in this test case is modified to 1 in anticipation of the PDU Session Establishment Request as triggered in Step 29 |
| TTCN module | Initial_Registration_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change

function fl_TC_9_1_5_1_4_Body() runs on NR5GC_PTC { var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_CellsOnDifferentPLMN); var NR_CellId_Type v_NGC_NASCellC := f_NR5GC_MapNASCell(ngc_CellC, NG_CellsOnDifferentPLMN); var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var PLMN_Identity v_ASN_PLMN := f_NR_CellInfo_GetPLMN (v_NGC_NASCellA); var NAS_PlmnId v_PLMN := f_NR_Asn2Nas_PlmnId(v_ASN_PLMN); var template (value) NG_NAS_DL_Message_Type v_MsgToSend; var float v_Delay := f_NR_SetTimerToleranceMin(v_NGC_NASCellC, nasTimer, 60.0); var NR_SRB_COMMON_IND v_ReceivedAsp; //@siclog "Step 0" siclog@ //The SS configures: //- NGC Cell A as the "Non-suitable "Off" cell".//@sic R5-216484sic@ //- NGC Cell C as the "Non-suitable "Off" cell".//@sic R5-216484sic@ //@sic R5-242487sic@ f_NR_SetCellPower (v_NGC_NASCellA, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE); f_NR_SetCellPower (v_NGC_NASCellC, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE); //@siclog "Step 1" siclog@ //The UE is Switched ON f_UT_SwitchOnUE(UT); //@siclog "Step 2" siclog@ //The user requests enabling of MICO mode by MMI or AT command f_UT_TriggerMICO(UT,true); //@siclog "Step 3" siclog@ //The SS configures:- NGC Cell A as the "Serving cell".- NGC Cell C as the "Non-suitable "Off" cell".- NGC Cell E as the "Non-suitable "Off" cell".//@sic R5-216484sic@ f_NR_SetCellPower (v_NGC_NASCellA, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); //@siclog "Steps 4-6" siclog@ //The UE establishes an RRC connection by executing steps 2-4 of Table 4.5.2.2-2 in TS38.508-1 [4]. v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellA, tsc_NR_RRC_TI_Def, ?, (tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected)); //@siclog "Steps 7" siclog@ //Check : Does the UE transmit a REGISTRATION REQUEST message including IE MICO indication if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_NoSecurity, -,-,-,-,-,-,-,-,-,-,-, crs_MicoInd('0'B,'0'B))) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } else { f_NR_PreliminaryPass (__FILE__, __LINE__, "Step 7 PASS"); } //@siclog "Steps 8-16" siclog@ //Steps 5-13 of Table 4.5.2.2-2 of the generic procedure in TS 38508-1 [4] are performed f_NR5GC_RRC_Idle_Steps5_13 (v_GMM_MobilityInfo, v_NGC_NASCellA, v_ReceivedMsg, v_PLMN, TESTMode_OFF, Initial_NoSecurity); //@siclog "Steps 17 siclog@ //SS transmits a REGISTRATION ACCEPT message that includes IE MICO indication v_MsgToSend := f_Get_NG_RegistrationAcceptMsg (Initial_Secure, v_GMM_MobilityInfo, // @sic R5-206426 sic@ f_NR5GC_GetMobileIdGUTI (v_NGC_NASCellA), omit, -,-,-,-,-, v_GMM_MobilityInfo.SessionStatus, //@sic R5-198509 sic@ -,-,-, crs_MicoInd('0'B,'1'B)); // Registration Accept SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, tsc_NR_RbId_SRB1, // must be on SRB1 cs_TimingInfo_Now, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); //@siclog "Steps 17A siclog@// @sic R5-226023 sic@//@sic R5s240115 sic@ //The UE transmits a REGISTRATION COMPLETE message. f_NR5GC_Registration_Complete(v_NGC_NASCellA); //@siclog "Steps 17Aa1 siclog@//@sic R5-233348 sic@ //Exception // The generic procedure for UE-requested PDU session establishmentspecified in subclause 4.5A.2, //takes place performing establishment of UE-requested PDU session(s) with ExpectedNumberOfNewPDUSessions = pc_noOf_PDUsSameConnection. if (pc_noOf_PDUsSameConnection > 0 ){//@sic R5-227239 sic@ SRB.check(receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, tsc_NR_RbId_SRB1, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp);//@sic R5s240115 sic@ if (f_Check_NG_ULNASTransport(v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg, cr_PayloadContainerType('0001'B), omit,omit,cr_NG_Request_Type('001'B))) { f_NR5GC_PDUSessionEstablishment (v_NGC_NASCellA,-,tsc_NR_RbId_SRB1);//@sic R5s240115 sic@ } } if (pc_noOf_PDUsNewConnection > 0 or pc_noOf_PDUsSameConnection == 0){ //Anritsu TTCN CR R5s240433 f_NR_RRCRelease(v_NGC_NASCellA); f_NR5GC_RRC_Idle_Extension(v_NGC_NASCellA); } //@siclog "Steps 17B siclog@// @sic R5-226023 sic@ //Void//@sic R5-233348 sic@ //@siclog "Steps 18 siclog@ //The SS releases the RRC Connection f_NR_RRCRelease(v_NGC_NASCellA);//@sic R5-233348 sic@ //@siclog "Steps 19 siclog@ //The SS configures:- NGC Cell A as the "Non-suitable "Off" cell".- NGC Cell C as the "Serving cell".- NGC Cell E as the "Non-suitable "Off" cell".//@sic R5-216484sic@ f_NR_SetCellPower (v_NGC_NASCellA, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE); f_NR_SetCellPower (v_NGC_NASCellC, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); //@siclog "Steps 20 siclog@ //Check : Does the UE transmit a RRCSetupRequest on NGC Cell C ? This is checked for 60s f_NR_CheckNoRRCSetupReq(v_NGC_NASCellC, v_Delay, cr_NR_EstablishmentCause_Any, "Step 20: RRCSetupRequest received"); //@sic R5s201387 sic@ //@siclog "Steps 21 siclog@ //Void//@sic R5-242487 sic@ //@siclog "Step 21A-28 siclog@ //Void//@sic R5-242487 sic@ //@siclog "Steps 29 siclog@ //Cause the UE to request connectivity to an PDU session// @sic R5-242487 sic@ f_UT_RequestPDUSession(UT, f_NR5GC_MobileInfo_GetNoOfSessions()+1, tsc_RandomAPN); // @siclog "Steps 30-31" siclog@// @sic R5-242487 sic@ // Steps 2-3 of the generic procedure for NR RRC_Connected specified in TS 38.508-1 [4] Table 4.5.4.2-3 are performed. // @siclog "Step 32" siclog@// @sic R5-242487 sic@ // The UE transmits an RRCSetupComplete message and a SERVICE REQUEST message with service type IE set to "signalling"?. v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellC, tsc_NR_RRC_TI_Def, ?); if (not f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get (),tsc_NasKsi_NativeSecurityContext), f_NG_S_TMSI2MobileIdentity(omit, f_NR5GC_CellInfo_GetGutiParameters (v_NGC_NASCellC)), '0000'B)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message Failed"); } // @siclog "Steps 33-36" siclog@// @sic R5-242487 sic@ // Steps 5-8 of the generic procedure for NR RRC_Connected specified in TS 38.508-1 [4] Table 4.5.4.2-3 are performed. f_NR5GC_RRC_ConnectedState3N_Steps5_8(v_NGC_NASCellC, v_ReceivedMsg); // @siclog "Step 37" siclog@ // The PDU session establishment procedure as specified in TS 38.508-1 [4] subclause 4.5A.2 take place. f_NR5GC_PDUSessionEstablishment (v_NGC_NASCellC,-,tsc_NR_RbId_SRB2);// @sic R5-242487 sic@ } //End of fl_TC_9_1_5_1_4_Body

After Change

function fl_TC_9_1_5_1_4_Body() runs on NR5GC_PTC { var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_CellsOnDifferentPLMN); var NR_CellId_Type v_NGC_NASCellC := f_NR5GC_MapNASCell(ngc_CellC, NG_CellsOnDifferentPLMN); var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var PLMN_Identity v_ASN_PLMN := f_NR_CellInfo_GetPLMN (v_NGC_NASCellA); var NAS_PlmnId v_PLMN := f_NR_Asn2Nas_PlmnId(v_ASN_PLMN); var template (value) NG_NAS_DL_Message_Type v_MsgToSend; var float v_Delay := f_NR_SetTimerToleranceMin(v_NGC_NASCellC, nasTimer, 60.0); var NR_SRB_COMMON_IND v_ReceivedAsp; //@siclog "Step 0" siclog@ //The SS configures: //- NGC Cell A as the "Non-suitable "Off" cell".//@sic R5-216484sic@ //- NGC Cell C as the "Non-suitable "Off" cell".//@sic R5-216484sic@ //@sic R5-242487sic@ f_NR_SetCellPower (v_NGC_NASCellA, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE); f_NR_SetCellPower (v_NGC_NASCellC, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE); //@siclog "Step 1" siclog@ //The UE is Switched ON f_UT_SwitchOnUE(UT); //@siclog "Step 2" siclog@ //The user requests enabling of MICO mode by MMI or AT command f_UT_TriggerMICO(UT,true); //@siclog "Step 3" siclog@ //The SS configures:- NGC Cell A as the "Serving cell".- NGC Cell C as the "Non-suitable "Off" cell".- NGC Cell E as the "Non-suitable "Off" cell".//@sic R5-216484sic@ f_NR_SetCellPower (v_NGC_NASCellA, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); //@siclog "Steps 4-6" siclog@ //The UE establishes an RRC connection by executing steps 2-4 of Table 4.5.2.2-2 in TS38.508-1 [4]. v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellA, tsc_NR_RRC_TI_Def, ?, (tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected)); //@siclog "Steps 7" siclog@ //Check : Does the UE transmit a REGISTRATION REQUEST message including IE MICO indication if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_NoSecurity, -,-,-,-,-,-,-,-,-,-,-, crs_MicoInd('0'B,'0'B))) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } else { f_NR_PreliminaryPass (__FILE__, __LINE__, "Step 7 PASS"); } //@siclog "Steps 8-16" siclog@ //Steps 5-13 of Table 4.5.2.2-2 of the generic procedure in TS 38508-1 [4] are performed f_NR5GC_RRC_Idle_Steps5_13 (v_GMM_MobilityInfo, v_NGC_NASCellA, v_ReceivedMsg, v_PLMN, TESTMode_OFF, Initial_NoSecurity); //@siclog "Steps 17 siclog@ //SS transmits a REGISTRATION ACCEPT message that includes IE MICO indication v_MsgToSend := f_Get_NG_RegistrationAcceptMsg (Initial_Secure, v_GMM_MobilityInfo, // @sic R5-206426 sic@ f_NR5GC_GetMobileIdGUTI (v_NGC_NASCellA), omit, -,-,-,-,-, v_GMM_MobilityInfo.SessionStatus, //@sic R5-198509 sic@ -,-,-, crs_MicoInd('0'B,'1'B)); // Registration Accept SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, tsc_NR_RbId_SRB1, // must be on SRB1 cs_TimingInfo_Now, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); //@siclog "Steps 17A siclog@// @sic R5-226023 sic@//@sic R5s240115 sic@ //The UE transmits a REGISTRATION COMPLETE message. f_NR5GC_Registration_Complete(v_NGC_NASCellA); //@siclog "Steps 17Aa1 siclog@//@sic R5-233348 sic@ //Exception // The generic procedure for UE-requested PDU session establishmentspecified in subclause 4.5A.2, //takes place performing establishment of UE-requested PDU session(s) with ExpectedNumberOfNewPDUSessions = pc_noOf_PDUsSameConnection. if (pc_noOf_PDUsSameConnection > 0 ){//@sic R5-227239 sic@ SRB.check(receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, tsc_NR_RbId_SRB1, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp);//@sic R5s240115 sic@ if (f_Check_NG_ULNASTransport(v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg, cr_PayloadContainerType('0001'B), omit,omit,cr_NG_Request_Type('001'B))) { f_NR5GC_PDUSessionEstablishment (v_NGC_NASCellA,-,tsc_NR_RbId_SRB1);//@sic R5s240115 sic@ } } if (pc_noOf_PDUsNewConnection > 0 or pc_noOf_PDUsSameConnection == 0){ //Anritsu TTCN CR R5s240433 f_NR_RRCRelease(v_NGC_NASCellA); f_NR5GC_RRC_Idle_Extension(v_NGC_NASCellA); } //@siclog "Steps 17B siclog@// @sic R5-226023 sic@ //Void//@sic R5-233348 sic@ //@siclog "Steps 18 siclog@ //The SS releases the RRC Connection f_NR_RRCRelease(v_NGC_NASCellA);//@sic R5-233348 sic@ //@siclog "Steps 19 siclog@ //The SS configures:- NGC Cell A as the "Non-suitable "Off" cell".- NGC Cell C as the "Serving cell".- NGC Cell E as the "Non-suitable "Off" cell".//@sic R5-216484sic@ f_NR_SetCellPower (v_NGC_NASCellA, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE); f_NR_SetCellPower (v_NGC_NASCellC, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); //@siclog "Steps 20 siclog@ //Check : Does the UE transmit a RRCSetupRequest on NGC Cell C ? This is checked for 60s f_NR_CheckNoRRCSetupReq(v_NGC_NASCellC, v_Delay, cr_NR_EstablishmentCause_Any, "Step 20: RRCSetupRequest received"); //@sic R5s201387 sic@ //@siclog "Steps 21 siclog@ //Void//@sic R5-242487 sic@ //@siclog "Step 21A-28 siclog@ //Void//@sic R5-242487 sic@ //@siclog "Steps 29 siclog@ //Cause the UE to request connectivity to an PDU session// @sic R5-242487 sic@ f_UT_RequestPDUSession(UT, f_NR5GC_MobileInfo_GetNoOfSessions()+1, tsc_RandomAPN); // @siclog "Steps 30-31" siclog@// @sic R5-242487 sic@ // Steps 2-3 of the generic procedure for NR RRC_Connected specified in TS 38.508-1 [4] Table 4.5.4.2-3 are performed. // @siclog "Step 32" siclog@// @sic R5-242487 sic@ // The UE transmits an RRCSetupComplete message and a SERVICE REQUEST message with service type IE set to "signalling"?. v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellC, tsc_NR_RRC_TI_Def, ?); if (not f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get (),tsc_NasKsi_NativeSecurityContext), f_NG_S_TMSI2MobileIdentity(omit, f_NR5GC_CellInfo_GetGutiParameters (v_NGC_NASCellC)), '0000'B)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message Failed"); } // @siclog "Steps 33-36" siclog@// @sic R5-242487 sic@ // Steps 5-8 of the generic procedure for NR RRC_Connected specified in TS 38.508-1 [4] Table 4.5.4.2-3 are performed. f_NR5GC_RRC_ConnectedState3N_Steps5_8(v_NGC_NASCellC, v_ReceivedMsg); // @siclog "Step 37" siclog@ // The PDU session establishment procedure as specified in TS 38.508-1 [4] subclause 4.5A.2 take place. f_NR5GC_PDUSessionEstablishment (v_NGC_NASCellC,1,tsc_NR_RbId_SRB2);// @sic R5-242487 sic@ } //End of fl_TC_9_1_5_1_4_Body

# References

| [1] | R5-242487: Corrrection to NR5GC test case 9.1.5.1.4 |
| --- | --- |