3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240541

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3513 | rev | - | Current version: | 17.11.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR MAC testcase 7.1.1.3.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According to Table 7.1.1.3.4.3.2-1, the buffer size checking should be as follows : in step 4, the Buffer Size=’4’ or bigger; in step 8, the Buffer Size=’4’ or bigger; in step 12, the Buffer Size=’5’ or bigger; The current TTCN is checking Buffer Size value is as “4” in step 4 and step 8; as “5” at step 12. This needs to be corrected to accommodate a bigger value as this is also correct. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the TTCN implement to get the buffer size value first and compare its value bigger or equal “4” at step 4 and step 8; bigger or equal “5” at step 12 to apply the buffer size to the According Table 7.1.1.3.4.3.2-1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TTCN implementation not in-line with Prose |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.3.4.NR5GC, 7.1.1.3.4.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	3

2.1	function f_TC_7_1_1_3_4_NR_TestBody	3

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D24wk24 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## function f_TC_7_1_1_3_4_NR_TestBody

| Function name | f_TC_7_1_1_3_4_NR_TestBody |
| --- | --- |
| Reason for change | According to Table 7.1.1.3.4.3.2-1, the buffer size checking should be as follows : in step 4, the Buffer Size=’4’ or bigger; in step 8, the Buffer Size=’4’ or bigger; in step 12, the Buffer Size=’5’ or bigger; The current TTCN is checking Buffer Size value is as “4” in step 4 and step 8; as “5” at step 12. This needs to be corrected to accommodate a bigger value as this is also correct. |
| Summary of change | Correct the TTCN for checking the buffer Size value at step 4, step 8 and step 12, to apply the bigger value also applied. |
| TTCN module | NR_TC_Common\7_1_1\MAC_TC_Common_NR.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_7_1_1_3_4_NR_TestBody(DRB_Identity p_NR_DRB_Id) runs on NR_BASE_PTC { ...... //@siclog "Step 3" siclog@ v_TimingUL := f_NR_GetNextSendOccasion_ULafterDL(nr_Cell1, v_TimingDL, 20); // 20 ms later f_NR_ULGrantConfiguration_Start(nr_Cell1, cs_TimingInfo_NR(v_TimingUL), v_NR_ResourceAllocation[0].Imcs, cs_NR_FreqDomainSchedulExplicit(-, v_NR_ResourceAllocation[0].Lrbs)); //40 bits // @siclog "Step 4" siclog@ t_Watchdog.start; v_CE_SubPDUListUL := { cr_NR_MAC_CE_SubPDU_UL(cr_NR_MAC_ShortBSR_SubHeader, cr_ShortBSR_Payload('010'B, int2bit(4, 5))) }; //LCG 2, value =4 v_MAC_PDUListUL := { cr_NR_UL_MAC_PDU(omit, v_CE_SubPDUListUL, cr_NR_MAC_Padding_SubPDU('??'O)) }; DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, -, v_MAC_PDUListUL)) -> value v_NR_DRB_COMMON_IND; // 2byte Padding header and 1 byte padding payload v_SubFrameTimingTa := v_NR_DRB_COMMON_IND.Common.TimingInfo.SubFrame; f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4"); // @siclog "Step 5-7" siclog@ //Automatically taken by SS and duration check // @siclog "Step 8" siclog@ v_CE_SubPDUListUL := { cr_NR_MAC_CE_SubPDU_UL(cr_NR_MAC_ShortBSR_SubHeader, cr_ShortBSR_Payload('010'B, int2bit(4, 5))) }; //LCG 2, value =4 v_MAC_PDUListUL := { cr_NR_UL_MAC_PDU(omit, v_CE_SubPDUListUL, cr_NR_MAC_Padding_SubPDU('??'O)) }; DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, -, v_MAC_PDUListUL)) -> value v_NR_DRB_COMMON_IND; // 1byte Padding header and 2 byte padding payload v_SubFrameTimingTb := v_NR_DRB_COMMON_IND.Common.TimingInfo.SubFrame; //@sic R5s200511 sic@ v_SBSR := bit2int(v_NR_DRB_COMMON_IND.U_Plane.SlotData.PduSduList.MacPdu[0].UL.CE_SubPDUList[0].ControlElement.ShortBSR.BufferSize); if (v_SBSR >= 4) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 4"); } v_Duration := f_SubFrameTiming_Duration(v_SubFrameTimingTa, v_SubFrameTimingTb); if (f_NR_SubFrameTimingCheckDuration(nr_Cell1, l2Timer, v_Duration, 320)) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 8"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 8"); } // @siclog "Step 9" siclog@ ………….. // @siclog "Step 10-11" siclog@ //Automatically taken by SS // @siclog "Step 12" siclog@ v_CE_SubPDUListUL := { cr_NR_MAC_CE_SubPDU_UL(cr_NR_MAC_ShortBSR_SubHeader, cr_ShortBSR_Payload('010'B, int2bit(4, 5))) }; //LCG 2, value =5 v_MAC_PDUListUL := { cr_NR_UL_MAC_PDU(omit, v_CE_SubPDUListUL, cr_NR_MAC_Padding_SubPDU('??'O)) }; // 1byte Padding header and 2 byte padding payload DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, -, v_MAC_PDUListUL)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 12"); //@siclog "Step 13" siclog@ ..... }

After Change:

function f_TC_7_1_1_3_4_NR_TestBody(DRB_Identity p_NR_DRB_Id) runs on NR_BASE_PTC { ...... //@siclog "Step 3" siclog@ v_TimingUL := f_NR_GetNextSendOccasion_ULafterDL(nr_Cell1, v_TimingDL, 20); // 20 ms later f_NR_ULGrantConfiguration_Start(nr_Cell1, cs_TimingInfo_NR(v_TimingUL), v_NR_ResourceAllocation[0].Imcs, cs_NR_FreqDomainSchedulExplicit(-, v_NR_ResourceAllocation[0].Lrbs)); //40 bits // @siclog "Step 4" siclog@ t_Watchdog.start; v_CE_SubPDUListUL := { cr_NR_MAC_CE_SubPDU_UL(cr_NR_MAC_ShortBSR_SubHeader, cr_ShortBSR_Payload('010'B, ?)) }; //LCG 2, value =4 v_MAC_PDUListUL := { cr_NR_UL_MAC_PDU(omit, v_CE_SubPDUListUL, cr_NR_MAC_Padding_SubPDU('??'O)) }; DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, -, v_MAC_PDUListUL)) -> value v_NR_DRB_COMMON_IND; // 2byte Padding header and 1 byte padding payload v_SubFrameTimingTa := v_NR_DRB_COMMON_IND.Common.TimingInfo.SubFrame; v_SBSR := bit2int(v_NR_DRB_COMMON_IND.U_Plane.SlotData.PduSduList.MacPdu[0].UL.CE_SubPDUList[0].ControlElement.ShortBSR.BufferSize); if (v_SBSR >= 4) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 4"); } //f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4"); // @siclog "Step 5-7" siclog@ //Automatically taken by SS and duration check // @siclog "Step 8" siclog@ v_CE_SubPDUListUL := { cr_NR_MAC_CE_SubPDU_UL(cr_NR_MAC_ShortBSR_SubHeader, cr_ShortBSR_Payload('010'B, ?)) }; //LCG 2, value =4 v_MAC_PDUListUL := { cr_NR_UL_MAC_PDU(omit, v_CE_SubPDUListUL, cr_NR_MAC_Padding_SubPDU('??'O)) }; DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, -, v_MAC_PDUListUL)) -> value v_NR_DRB_COMMON_IND; // 1byte Padding header and 2 byte padding payload v_SubFrameTimingTb := v_NR_DRB_COMMON_IND.Common.TimingInfo.SubFrame; //@sic R5s200511 sic@ v_SBSR := bit2int(v_NR_DRB_COMMON_IND.U_Plane.SlotData.PduSduList.MacPdu[0].UL.CE_SubPDUList[0].ControlElement.ShortBSR.BufferSize); if (v_SBSR >= 4) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 4"); } v_Duration := f_SubFrameTiming_Duration(v_SubFrameTimingTa, v_SubFrameTimingTb); if (f_NR_SubFrameTimingCheckDuration(nr_Cell1, l2Timer, v_Duration, 320)) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 8"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 8"); } // @siclog "Step 9" siclog@ ………….. // @siclog "Step 10-11" siclog@ //Automatically taken by SS // @siclog "Step 12" siclog@ v_CE_SubPDUListUL := { cr_NR_MAC_CE_SubPDU_UL(cr_NR_MAC_ShortBSR_SubHeader, cr_ShortBSR_Payload('010'B, ?)) }; //LCG 2, value =5 v_MAC_PDUListUL := { cr_NR_UL_MAC_PDU(omit, v_CE_SubPDUListUL, cr_NR_MAC_Padding_SubPDU('??'O)) }; // 1byte Padding header and 2 byte padding payload DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, -, v_MAC_PDUListUL)); v_SBSR := bit2int(v_NR_DRB_COMMON_IND.U_Plane.SlotData.PduSduList.MacPdu[0].UL.CE_SubPDUList[0].ControlElement.ShortBSR.BufferSize); if (v_SBSR >= 5) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 12"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 12"); } //f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 12"); //@siclog "Step 13" siclog@ …………s }