3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240546

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3516 | rev | - | Current version: | 17.11.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to 5GC NAS Type Definitions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18_Test |  |  |  |  |  | Date: |  |  | 2024-08-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - R5s240531 introduced some errors in the 5GS NAS type definitions updated for Rel-18 support. - Loopback Test Mode C is required for the 5GC MBS test cases. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Corrected errors in type definitions. - Added Test Loop Mode C to NR_CLOSE_UE_TEST_LOOP. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 5GS test cases cannot be run. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | All 5GS Test Suites |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required.	3

2.1	Change 1	3

2.2	Change 2	3

2.3	Change 3	3

2.4	Change 4	4

2.5	Change 5	4

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2024-06_D24wk32 related to the title of this CR.

Contact:	Olivier Genoud
	olivier.genoud@etsi.org

# Corrections required

## Change 1

| Type name | FeatureAuthInd |
| --- | --- |
| Reason for change | The MBSRAI field should be 2 bits, not 1 |
| TTCN module | NG_NAS_TypeDefs.ttcn |

type record FeatureAuthInd {                         /* 24.501 cl. 9.11.3.105 */

IEI8_Type                   iei,

Type4Length_Type            iel,

B6_Type                     spare,

B2_Type                     mbsrai,

octetstring                 spareBytes  optional

};

## Change 2

| Type name | TMGI_V |
| --- | --- |
| Reason for change | This type should not contain the length field |
| TTCN module | NG_NAS_TypeDefs.ttcn |

type record TMGI_V {

O3_Type                     serviceID,

NAS_PlmnId                  plmn        optional

};

## Change 3

| Type name | Non3GPP_DelayBudgetEntry |
| --- | --- |
| Reason for change | The packet filter list should be list of QoS Rules |
| TTCN module | NG_NAS_TypeDefs.ttcn |

type record Non3GPP_DelayBudgetEntry {

O2_Type                     budgetValue,

B6_Type                     spareBits,

B1_Type                     qfipi,

B1_Type                     pfpi,

O1_Type                     numOfQFIs           optional,

octetstring                 listOfQFI           optional,

Non3GPPQoS_Rules packetFilterList    optional

};

type record Non3GPPQoS_Rule {

IEI8_Type                   identifier,

Type6Length_Type            iel,

B3_Type                     ruleOperationCode,

B1_Type                     dqrBit,

B4_Type                     numOfPacketFilters,

NG_PacketFilterList_Type2    packetFilterList   optional,

O1_Type                     precedence         optional,

B1_Type                     spare              optional,

B1_Type                     segregation        optional,

B6_Type                     flowIdentifier     optional

};

type record of Non3GPPQoS_Rule Non3GPPQoS_RuleList;

type record Non3GPPQoS_Rules {                                        // 24.501 cl. 9.11.4.37

IEI8_Type                            iei                   optional, /* present in case of TLV; omit in case of LV */

Type6Length_Type               iel,

Non3GPPQoS_RuleList        listofQoSRules

};

## Change 4

| Type name | MBSFreqInfo |
| --- | --- |
| Reason for change | The frequency field is the wrong length |
| TTCN module | UE_POLICY_TypeDefs.ttcn |

type record MBSFreqInfo {        // 24.588 cl. 5.4.1.8C

Type6Length_Type            len,

octetstring           freq    // prose specifies o128 - (119+3) = 7, but defined as 3 octets elsewhere

};

## Change 5

| Type name | NR_CLOSE_UE_TEST_LOOP |
| --- | --- |
| Reason for change | Test Loop Mode C is required in NR |
| TTCN module | Common4G5G_LoopBack.ttcn |

type record NR_CLOSE_UE_TEST_LOOP { /* @status    APPROVED (ENDC, IMS, IMS_IRAT, LTE, LTE_A_IRAT, LTE_A_PRO, LTE_A_R10_R11, LTE_A_R12, LTE_IRAT, MCX, NBIOT, NR5GC, NR5GC_IRAT, POS, UTRAN) */

SkipIndicator                       skipIndicator,                          /*    M V 1/2    */

ProtocolDiscriminator               protocolDiscriminator,                  /*    M V 1/2    */

MessageType                         messageType,                            /*    M V 1      */

UE_TestLoopMode_Type                ueTestLoopMode,                         /*    M V 1      */

UE_TestLoopModeA_NR_LB_Setup_Type   ueTestLoopModeA_LB_Setup optional,      /*    C LV 1-25

present if ueTestLoopMode ='00'F */

UE_TestLoopModeB_LB_Setup_Type      ueTestLoopModeB_LB_Setup optional,      /*    C V 1

present if ueTestLoopMode ='01'F/

represents IP PDU delay time 0..255 seconds */

UE_TestLoopModeC_LB_Setup_Type      ueTestLoopModeC_LB_Setup optional,      /*    C V 3

present if ueTestLoopMode ='02'F  */

UE_TestLoopModeE_LB_Setup_Type      ueTestLoopModeE_LB_Setup optional      /*    C LV 1-25

present if ueTestLoopMode ='04'F */

};