3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240557

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3467 | rev | 1 | Current version: | 17.11.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR MAC testcase 7.1.1.6.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-06-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - The expectation that, at Step 11a, the communication of using a different time domain index in ASP NR_SYSTEM_CTRL_REQ at a particular activation time and expecting it to be used in the NR_DRB_COMMON_REQ request at Step 11a2 at the same activation time makes it not possible to adopt the updated time domain value as these are two independent pieces of information communicated in two different ASPs. The SS needs to first finish the local end config and then apply the updated time domain index value for the subsequent DRB send. - As a result of TTCN CR R5s240268, the following changes are required: Since the local end SPS config is released in the SS implementation, SPS config needs to be re-configured in the SS to enable transmission of the new SPS activation pattern implemented at Step 31. Secondly, as this is a activation of a new SPS pattern, UE would react to the PDCCH transmission on CS-RNTI with NDI = 0, as the CS-RNTI is still stored in the UE, and sends a HARQ ACK back to the SS. This HARQ ACK needs to be handled in TTCN and subsequently Data on the PDSCH using the CS-RNTI needs to be transmitted to perform the expected negative test at Step 31. NOTE: A Prose CR needs to be raised at the next RAN5 for this change. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - The local end config setting up the different time domain index is done with immediate activation so that the updated config is ready to be used for subsequent DRB send at Step 11a2 TTCN changes done to re-configure SPS config in SS and add code to handle the HARQ ACK for the SPS activation and have done timing changes to ensure the Data on the PDSCH using CS-RNTI follows 80ms later to perform the negative check. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant shall fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.6.1.NR5GC, 7.1.1.6.1.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 38.523-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	4

2	Corrections required	4

2.1	Change 1	4

2.2	Change 2	5

2.3	Change 3	6

2.4	Change 4	8

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D24wk24 related to the title of this CR.

Contact:  Bhise Parikshit
parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change 1

| Template name | f_TC_7_1_1_6_1_NR_TestBody1 |
| --- | --- |
| Reason for change | The expectation that, at Step 11a, the communication of using a different time domain index in ASP NR_SYSTEM_CTRL_REQ at a particular activation time and expecting it to be used in the NR_DRB_COMMON_REQ request at Step 11a2 at the same activation time makes it not possible to adopt the updated time domain value as these are two independent pieces of information communicated in two different ASPs. The SS needs to first finish the local end config and then apply the updated time domain index value for the subsequent DRB send |
| Summary of change | The local end config setting up the different time domain index is done with immediate activation so that the updated config is ready to be used for subsequent DRB send at Step 11a2 |
| TTCN module | MAC_SPS_TC_Common_NR.ttcn |

Before Change:

function f_TC_7_1_1_6_1_NR_TestBody1(DRB_Identity p_NR_DRB_Id) runs on NR_BASE_PTC return SubFrameTiming_Type { <<SKIPPED CODE>> //Set K1=8 v_DciDlInfo.ResoureAssignment.TimeDomain.Index := int2bit(1, 4); f_NR_SS_CommonCellConfig(nr_Cell1, cas_NR_DciDlInfo_REQ(nr_Cell1, cs_TimingInfo_NR( v_TimingDL_CRNTI), v_DciDlInfo)) <<SKIPPED CODE>>

After Change:

function f_TC_7_1_1_6_1_NR_TestBody1(DRB_Identity p_NR_DRB_Id) runs on NR_BASE_PTC return SubFrameTiming_Type { <<SKIPPED CODE>> f_NR_SS_CommonCellConfig(nr_Cell1, cas_NR_DciDlInfo_REQ(nr_Cell1, cs_TimingInfo_Now, v_DciDlInfo));//WA# <<SKIPPED CODE>> }

## Change 2

| Template name | f_TC_7_1_1_6_1_NR5GC |
| --- | --- |
| Reason for change | As a result of TTCN CR R5s240268, the following changes are required: Since the local end SPS config is released in the SS implementation, SPS config needs to be re-configured in the SS to enable transmission of the new SPS activation pattern implemented at Step 31. Secondly, as this is a activation of a new SPS pattern, UE would react to the PDCCH transmission on CS-RNTI with NDI = 0, as the CS-RNTI is still stored in the UE, and sends a HARQ ACK back to the SS. This HARQ ACK needs to be handled in TTCN and subsequently Data on the PDSCH using the CS-RNTI needs to be transmitted to perform the expected negative test at Step 31. NOTE: A Prose CR needs to be raised at the next RAN5 for this change. |
| Summary of change | TTCN changes done to re-configure SPS config in SS and add code to handle the HARQ ACK for the SPS activation and have done timing changes to ensure the Data on the PDSCH using CS-RNTI follows 80ms later to perform the negative check. |
| TTCN module | MAC_SPS_NR5GC.ttcn |

Before Change:

function f_TC_7_1_1_6_1_NR5GC() runs on NR5GC_PTC { // Correct handling of DL assignment / Semi-persistent case <<SKIPPED CODE>> //@sic R5s240268 sic@ v_TimingDL2 := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,30); //30ms later f_NR_SpsCgDeactivate(nr_Cell1, cs_TimingInfo_NR( v_TimingDL2),cs_NR_SpsCgDeactivate_LclRel); //@siclog steps 29-30 siclog@ v_SS_Drb_ConfigList := {cs_NR_SS_RadioBearer_MAC_TestMode(v_DRBId, cs_NR_MAC_TestModeInfo_NormalMode)}; f_NR_SS_CommonRadioBearerConfig(nr_Cell1, v_SS_Drb_ConfigList); v_SpCellConfig.spCellConfigDedicated.initialDownlinkBWP.sps_Config := {release := NULL}; v_NR_DownlinkBWP_List[0].Sps := omit; v_CellGroupConfig := cs_38508_CellGroupConfig (tsc_NR_CellGroupId_MCG , omit, omit, omit, omit, v_SpCellConfig ); v_V1530Ext := cs_NR_RRCReconfiguration_v1530_IEs(bit2oct(encvalue(v_CellGroupConfig))); f_NR_SendRRCReconfiguration (nr_Cell1, -,-, v_V1530Ext); //@siclog steps 31-32 siclog@ v_SS_Drb_ConfigList := {cs_NR_SS_RadioBearer_MAC_TestMode(v_DRBId, cs_NR_MAC_TestModeInfo_NoHeaderManipulationDL_UL)}; f_NR_SS_CommonRadioBearerConfig(nr_Cell1, v_SS_Drb_ConfigList); f_TC_7_1_1_6_1_NR_TestBody2(v_DRBId,v_TimingDL,10); //10*80 =800 ms is assumed to be sufficient to perfor RRC Reconfiguration //@sic R5s230088 sic@ f_NR_CellInfo_SetDownlinkBWP_List(nr_Cell1, v_NR_DownlinkBWP_List); f_NR_SS_CommonCellConfig(nr_Cell1, cads_NR_DownlinkBWP_ConfigCommon_REQ(nr_Cell1, -, v_NR_DownlinkBWP_List)); f_NR_TestBody_Set(false); <<SKIPPED CODE>>

After Change:

function f_TC_7_1_1_6_1_NR5GC() runs on NR5GC_PTC { // Correct handling of DL assignment / Semi-persistent case <<SKIPPED CODE>> //@sic R5s240268 sic@ v_TimingDL2 := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,30); //30ms later f_NR_SpsCgDeactivate(nr_Cell1, cs_TimingInfo_NR( v_TimingDL2),cs_NR_SpsCgDeactivate_LclRel); //@siclog steps 29-30 siclog@ v_SS_Drb_ConfigList := {cs_NR_SS_RadioBearer_MAC_TestMode(v_DRBId, cs_NR_MAC_TestModeInfo_NormalMode)}; f_NR_SS_CommonRadioBearerConfig(nr_Cell1, v_SS_Drb_ConfigList); v_SpCellConfig.spCellConfigDedicated.initialDownlinkBWP.sps_Config := {release := NULL}; //REMOVED v_NR_DownlinkBWP_List[0].Sps := omit; WA# v_CellGroupConfig := cs_38508_CellGroupConfig (tsc_NR_CellGroupId_MCG , omit, omit, omit, v_PhysicalCellGroupConfig, v_SpCellConfig ); v_V1530Ext := cs_NR_RRCReconfiguration_v1530_IEs(bit2oct(encvalue(v_CellGroupConfig))); f_NR_SendRRCReconfiguration (nr_Cell1, -,-, v_V1530Ext); //WA# f_NR_SS_CommonCellConfig(nr_Cell1, cads_NR_DownlinkBWP_ConfigCommon_REQ(nr_Cell1, -, v_NR_DownlinkBWP_List)); //@siclog steps 31-32 siclog@ v_SS_Drb_ConfigList := {cs_NR_SS_RadioBearer_MAC_TestMode(v_DRBId, cs_NR_MAC_TestModeInfo_NoHeaderManipulationDL_UL)}; f_NR_SS_CommonRadioBearerConfig(nr_Cell1, v_SS_Drb_ConfigList); f_TC_7_1_1_6_1_NR_TestBody2(v_DRBId,v_TimingDL,10); //10*80 =800 ms is assumed to be sufficient to perfor RRC Reconfiguration v_NR_DownlinkBWP_List[0].Sps := omit; //WA# //@sic R5s230088 sic@ f_NR_CellInfo_SetDownlinkBWP_List(nr_Cell1, v_NR_DownlinkBWP_List); f_NR_SS_CommonCellConfig(nr_Cell1, cads_NR_DownlinkBWP_ConfigCommon_REQ(nr_Cell1, -, v_NR_DownlinkBWP_List)); f_NR_TestBody_Set(false); <<SKIPPED CODE>>

## Change 3

| Template name | f_TC_7_1_1_6_1_ENDC_NR |
| --- | --- |
| Reason for change | As a result of TTCN CR R5s240268, the following changes are required: Since the local end SPS config is released in the SS implementation, SPS config needs to be re-configured in the SS to enable transmission of the new SPS activation pattern implemented at Step 31. Secondly, as this is a activation of a new SPS pattern, UE would react to the PDCCH transmission on CS-RNTI with NDI = 0, as the CS-RNTI is still stored in the UE, and sends a HARQ ACK back to the SS. This HARQ ACK needs to be handled in TTCN and subsequently Data on the PDSCH using the CS-RNTI needs to be transmitted to perform the expected negative test at Step 31. NOTE: A Prose CR needs to be raised at the next RAN5 for this change. |
| Summary of change | TTCN changes done to re-configure SPS config in SS and add code to handle the HARQ ACK for the SPS activation and have done timing changes to ensure the Data on the PDSCH using CS-RNTI follows 80ms later to perform the negative check. |
| TTCN module | MAC_SPS_ENDC_NR.ttcn |

Before Change:

function f_TC_7_1_1_6_1_ENDC_NR() runs on ENDC_NR_PTC {// Correct handling of DL assignment / Semi-persistent case <<SKIPPED CODE>> //@sic R5s240268 sic@ v_TimingDL2 := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,30); //30ms later f_NR_SpsCgDeactivate(nr_Cell1, cs_TimingInfo_NR( v_TimingDL2),cs_NR_SpsCgDeactivate_LclRel); //@siclog steps 29-30 siclog@ if (pc_dl_SchedulingOffset_PDSCH_TypeA) { v_SpCellConfig.reconfigurationWithSync := omit; } v_SpCellConfig.spCellConfigDedicated.initialDownlinkBWP.sps_Config := {release := NULL}; v_NR_DownlinkBWP_List[0].Sps := omit; v_CellGroupConfig := cs_38508_CellGroupConfig (tsc_NR_CellGroupId_SCG , //@sic R5s221195 sic@ omit, omit, omit, omit, v_SpCellConfig ); f_NR_SendRRCReconfigurationContentsToEUTRA (omit, v_CellGroupConfig); //@siclog steps 31-32 siclog@ f_TC_7_1_1_6_1_NR_TestBody2(v_DRBId,v_TimingDL,10); //10*80 =800 ms is assumed to be sufficient to perfor RRC Reconfiguration f_NR_CellInfo_SetDownlinkBWP_List(nr_Cell1, v_NR_DownlinkBWP_List); f_NR_SS_CommonCellConfig(nr_Cell1, cads_NR_DownlinkBWP_ConfigCommon_REQ(nr_Cell1, -, v_NR_DownlinkBWP_List)); f_NR_TestBody_Set(false); <<SKIPPED CODE>>

After Change:

function f_TC_7_1_1_6_1_ENDC_NR() runs on ENDC_NR_PTC {// Correct handling of DL assignment / Semi-persistent case <<SKIPPED CODE>> //@sic R5s240268 sic@ v_TimingDL2 := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,30); //30ms later f_NR_SpsCgDeactivate(nr_Cell1, cs_TimingInfo_NR( v_TimingDL2),cs_NR_SpsCgDeactivate_LclRel); //@siclog steps 29-30 siclog@ if (pc_dl_SchedulingOffset_PDSCH_TypeA) { v_SpCellConfig.reconfigurationWithSync := omit; } v_SpCellConfig.spCellConfigDedicated.initialDownlinkBWP.sps_Config := {release := NULL}; //REMOVED v_NR_DownlinkBWP_List[0].Sps := omit; WA# v_CellGroupConfig := cs_38508_CellGroupConfig (tsc_NR_CellGroupId_SCG , //@sic R5s221195 sic@ omit, omit, omit, v_PhysicalCellGroupConfig, v_SpCellConfig ); f_NR_SendRRCReconfigurationContentsToEUTRA (omit, v_CellGroupConfig); //WA# f_NR_SS_CommonCellConfig(nr_Cell1, cads_NR_DownlinkBWP_ConfigCommon_REQ(nr_Cell1, -, v_NR_DownlinkBWP_List)); //@siclog steps 31-32 siclog@ f_TC_7_1_1_6_1_NR_TestBody2(v_DRBId,v_TimingDL,10); //10*80 =800 ms is assumed to be sufficient to perfor RRC Reconfiguration v_NR_DownlinkBWP_List[0].Sps := omit; //WA# f_NR_CellInfo_SetDownlinkBWP_List(nr_Cell1, v_NR_DownlinkBWP_List); f_NR_SS_CommonCellConfig(nr_Cell1, cads_NR_DownlinkBWP_ConfigCommon_REQ(nr_Cell1, -, v_NR_DownlinkBWP_List)); f_NR_TestBody_Set(false); <<SKIPPED CODE>>

## Change 4

| Template name | f_TC_7_1_1_6_1_NR_TestBody2 |
| --- | --- |
| Reason for change | Secondly, as this is a activation of a new SPS pattern, UE would react to the PDCCH transmission on CS-RNTI with NDI = 0, as the CS-RNTI is still stored in the UE, and sends a HARQ ACK back to the SS. This HARQ ACK needs to be handled in TTCN and subsequently Data on the PDSCH using the CS-RNTI needs to be transmitted to perform the expected negative test at Step 31. NOTE: A Prose CR needs to be raised at the next RAN5 for this change. |
| Summary of change | TTCN changes done to re-configure SPS config in SS and add code to handle the HARQ ACK for the SPS activation and have done timing changes to ensure the Data on the PDSCH using CS-RNTI follows 80ms later to perform the negative check. |
| TTCN module | MAC_SPS_TC_Common_NR.ttcn |

Before Change:

function f_TC_7_1_1_6_1_NR_TestBody2(DRB_Identity p_NR_DRB_Id, SubFrameTiming_Type p_TimingDL, integer p_X) runs on NR_BASE_PTC { <<SKIPPED CODE>> // @siclog "Step 31" siclog@ //@sic R5s240268 sic@ f_NR_DL_SpsAssignmentConfiguration(nr_Cell1,cs_TimingInfo_NR( v_TimingDL),v_N_PRB,v_I_MCS); // PDCP SDU =38B +3B PDCP Header+ 1B RLC UM Header + 2B MAC Header =44B=352 bits (an allowed TB size as per 38.523-3 Annex B) v_EncodedRlcPdu1 := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (7, crs_NR_PDCP_SDU_38B, 1)));//@sic R5-227518 sic@ v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1) }; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; DRB.send(cads_NR_DRB_COMMON_REQ_MAC_PDUList_NoCRNTI_Grant(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR( v_TimingDL), v_MAC_PDUListDL)); <<SKIPPED CODE>>

After Change:

function f_TC_7_1_1_6_1_NR_TestBody2(DRB_Identity p_NR_DRB_Id, SubFrameTiming_Type p_TimingDL, integer p_X) runs on NR_BASE_PTC { <<SKIPPED CODE>> // @siclog "Step 31" siclog@ //@sic R5s240268 sic@ f_NR_DL_SpsAssignmentConfiguration(nr_Cell1,cs_TimingInfo_NR( v_TimingDL),v_N_PRB,v_I_MCS); SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND_Timing(nr_Cell1, ack)); //WA# v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period); //WA# // PDCP SDU =38B +3B PDCP Header+ 1B RLC UM Header + 2B MAC Header =44B=352 bits (an allowed TB size as per 38.523-3 Annex B) v_EncodedRlcPdu1 := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (7, crs_NR_PDCP_SDU_38B, 1)));//@sic R5-227518 sic@ v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1) }; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; DRB.send(cads_NR_DRB_COMMON_REQ_MAC_PDUList_NoCRNTI_Grant(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR( v_TimingDL), v_MAC_PDUListDL)); <<SKIPPED CODE>>