3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240560

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4853 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to LTE RRC RACS test case 8.5.5.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, RACS-UEConTest |  |  |  |  |  | Date: |  |  | 2024-09-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In TDD configuration, uplink grant can be allocated in D and S frames depending upon config type. Current TTCN implementation only allows 16 grant cycles which will not be sufficient for TDD configuration with grant period set as 5. It will need 80 grant cycles to allocate 16 UL grants as required by the prose at step 5. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added 80 grant cycles for TDD. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.5.5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D24wk24 related to the title of this CR.

Contact:	Mohit Kanchan
	mohit.kanchan@keysight.com

# Corrections required

## Change 1

| Function name | f_TC_8_5_5_1_EUTRA |
| --- | --- |
| Reason for change | In TDD configuration, uplink grant can be allocated in D and S frames depending upon config type. Current TTCN implementation only allows 16 grant cycles which will not be sufficient for TDD configuration with grant period set as 5. It will need 80 grant cycles to allocate 16 UL grants as required by the prose at step 5. |
| Summary of change | Added 80 grant cycles for TDD. |
| TTCN module | RRC_Others_PRO.ttcn |
| MCC160 Comment |  |

Before change

var SRB_COMMON_IND v_Received_DedicatedMessageSegment;
    var bitstring v_SuppEutraBandsAccToPICS := f_DeriveSuppEutraBandsFromPics(); // The position on this list represents the band number
    var UECapabilityEnquiry_v1180_IEs.requestedFrequencyBands_r11 v_RequestedFrequencyBands := {px_ePrimaryFrequencyBand}; // Initialise with the primary frequency band
    var UECapabilityEnquiry_v1180_IEs.requestedFrequencyBands_r11 v_BandToBeAdded;
    var integer v_Index := 0;
    var SubFrameTiming_Type v_TimingInfoStep4;
    var SubFrameTiming_Type v_TimingInfoStep5;
    var SubFrameTiming_Type v_TimingInfoStep6;
    var integer v_GrantPeriod;
    var EUTRA_FDD_TDD_Mode_Type v_EUTRA_FDD_TDD_Mode;
   
    timer t_Watchdog := 5.0;

    f_EUTRA_Init(c1);
    v_PhysicalCellIdentity_Cell1 := f_EUTRA_CellInfo_GetPhyCellId(eutra_Cell1); //@sic R5s240175 sic@

    //Create and configure cell
    f_EUTRA_CellConfig_Def(eutra_Cell1);
    v_EUTRA_FDD_TDD_Mode := f_EUTRA_CellInfo_GetFDD_TDD(eutra_Cell1);

//@siclog "Step 4" siclog@
    // The SS transmits a UECapabilityEnquiry message to enquire max capability and with rrc-segAllowed-r16 set to enabled
    v_TimingInfoStep4 := f_EUTRA_GetNextSendOccasion(eutra_Cell1);
    SRB.send(cas_SRB1_RrcPdu_REQ(eutra_Cell1, cs_TimingInfo_EUTRA_NB(v_TimingInfoStep4), cs_508_UeCapabilityEnquiry(tsc_RRC_TI_Def, -,-,-,-, enabled, v_RequestedFrequencyBands))); //@sic R5s240175 sic@

//@siclog "Step 5" siclog@
    // The SS allocates 17 UL grants periodically from 15(FDD)/18 (TDD) subframes after successful completion of step 4 to schedule PUSCH till the ULDedicatedMessageSegment at step 6 is received.
    if (v_EUTRA_FDD_TDD_Mode == FDD) {
      v_TimingInfoStep5 := f_EUTRA_TimingInfoAdd(v_TimingInfoStep4, 15); // 15 subframes = 15 ms
      v_GrantPeriod := 1; // No period value especified in the prose. Value chosed arbitrarily.

    }
    else {
      v_TimingInfoStep5 := f_EUTRA_TimingInfoAdd(v_TimingInfoStep4, 18); // 18 subframes = 18 ms
      v_GrantPeriod := 5; // No period value especified in the prose. Value chosed arbitrarily.
    }
    //@sic R5-231397 sic@
    f_EUTRA_PeriodicCyclicULGrantTransmission(eutra_Cell1,
                                              cs_TimingInfo_EUTRA_NB(v_TimingInfoStep5),
                                              9, // Imcs => Acc. 36.523-3, 4008 bits TBS
                                              25, // Nprb => Acc. 36.523-3, 4008 bits TBS
                                              v_GrantPeriod,
                                              16); //@sic R5s240175 sic@
                                             
    //@sic R5s240175 sic@
    v_TimingInfoStep6 := f_EUTRA_TimingInfoAdd(v_TimingInfoStep5,16);
    f_EUTRA_OneULGrantTransmission(eutra_Cell1, cs_TimingInfo_EUTRA_NB(v_TimingInfoStep6), 20, 4); // Imcs  & Nprb => Acc. 36.523-3, 1416 bits TBS

    //@siclog "Step 6" siclog@
    // The UE sends 1st segment of UECapabilityInformation message contained within the rrc-MessageSegmentContainer-r16 IE of the ULDedicatedMessageSegment message
    SRB.receive(car_SRB1_RrcPdu_IND(eutra_Cell1, cr_508_ULDedicatedMessageSegment_r16(cr_ULDedicatedMessageSegment_r16_enabled(0))));
    f_EUTRA_StopULGrantTransmission_PUCCH_Sync(eutra_Cell1, cs_TimingInfo_Now);

    //@siclog "Step 7" siclog@
    // The SS changes Cell 1 parameters according to the row "T1" in table 8.5.5.1.3.2-1 in order that the radio link quality of Cell 1 is degraded.
    f_EUTRA_SetCellPower (eutra_Cell1, tsc_NonSuitableOffCellRS_EPRE);

After change

var SRB_COMMON_IND v_Received_DedicatedMessageSegment;
    var bitstring v_SuppEutraBandsAccToPICS := f_DeriveSuppEutraBandsFromPics(); // The position on this list represents the band number
    var UECapabilityEnquiry_v1180_IEs.requestedFrequencyBands_r11 v_RequestedFrequencyBands := {px_ePrimaryFrequencyBand}; // Initialise with the primary frequency band
    var UECapabilityEnquiry_v1180_IEs.requestedFrequencyBands_r11 v_BandToBeAdded;
    var integer v_Index := 0;
    var SubFrameTiming_Type v_TimingInfoStep4;
    var SubFrameTiming_Type v_TimingInfoStep5;
    var SubFrameTiming_Type v_TimingInfoStep6;
    var integer v_GrantPeriod;
    var EUTRA_FDD_TDD_Mode_Type v_EUTRA_FDD_TDD_Mode;
    var integer v_GrantCycles;

    timer t_Watchdog := 5.0;

    f_EUTRA_Init(c1);
    v_PhysicalCellIdentity_Cell1 := f_EUTRA_CellInfo_GetPhyCellId(eutra_Cell1); //@sic R5s240175 sic@

    //Create and configure cell
    f_EUTRA_CellConfig_Def(eutra_Cell1);
    v_EUTRA_FDD_TDD_Mode := f_EUTRA_CellInfo_GetFDD_TDD(eutra_Cell1);

//@siclog "Step 4" siclog@
    // The SS transmits a UECapabilityEnquiry message to enquire max capability and with rrc-segAllowed-r16 set to enabled
    v_TimingInfoStep4 := f_EUTRA_GetNextSendOccasion(eutra_Cell1);
    SRB.send(cas_SRB1_RrcPdu_REQ(eutra_Cell1, cs_TimingInfo_EUTRA_NB(v_TimingInfoStep4), cs_508_UeCapabilityEnquiry(tsc_RRC_TI_Def, -,-,-,-, enabled, v_RequestedFrequencyBands))); //@sic R5s240175 sic@

//@siclog "Step 5" siclog@
    // The SS allocates 17 UL grants periodically from 15(FDD)/18 (TDD) subframes after successful completion of step 4 to schedule PUSCH till the ULDedicatedMessageSegment at step 6 is received.
    if (v_EUTRA_FDD_TDD_Mode == FDD) {
      v_TimingInfoStep5 := f_EUTRA_TimingInfoAdd(v_TimingInfoStep4, 15); // 15 subframes = 15 ms
      v_GrantPeriod := 1; // No period value especified in the prose. Value chosed arbitrarily.
      v_GrantCycles :=16;
    }
    else {
      v_TimingInfoStep5 := f_EUTRA_TimingInfoAdd(v_TimingInfoStep4, 18); // 18 subframes = 18 ms
      v_GrantPeriod := 5; // No period value especified in the prose. Value chosed arbitrarily.
      v_GrantCycles :=80;
    }
//@sic R5-231397 sic@
    f_EUTRA_PeriodicCyclicULGrantTransmission(eutra_Cell1,
                                              cs_TimingInfo_EUTRA_NB(v_TimingInfoStep5),
                                              9, // Imcs => Acc. 36.523-3, 4008 bits TBS
                                              25, // Nprb => Acc. 36.523-3, 4008 bits TBS
                                              v_GrantPeriod,
                                              v_GrantCycles);                                       
    //@sic R5s240175 sic@
    v_TimingInfoStep6 := f_EUTRA_TimingInfoAdd(v_TimingInfoStep5,v_GrantCycles);
    f_EUTRA_OneULGrantTransmission(eutra_Cell1, cs_TimingInfo_EUTRA_NB(v_TimingInfoStep6), 20, 4); // Imcs  & Nprb => Acc. 36.523-3, 1416 bits TBS

    //@siclog "Step 6" siclog@
    // The UE sends 1st segment of UECapabilityInformation message contained within the rrc-MessageSegmentContainer-r16 IE of the ULDedicatedMessageSegment message
    SRB.receive(car_SRB1_RrcPdu_IND(eutra_Cell1, cr_508_ULDedicatedMessageSegment_r16(cr_ULDedicatedMessageSegment_r16_enabled(0))));
    f_EUTRA_StopULGrantTransmission_PUCCH_Sync(eutra_Cell1, cs_TimingInfo_Now);

    //@siclog "Step 7" siclog@
    // The SS changes Cell 1 parameters according to the row "T1" in table 8.5.5.1.3.2-1 in order that the radio link quality of Cell 1 is degraded.
    f_EUTRA_SetCellPower (eutra_Cell1, tsc_NonSuitableOffCellRS_EPRE);