3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240567

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3527 | rev | - | Current version: | 17.11.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR MAC test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-09-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In TCs where DL assignment has been configured as explicit, TTCN is not putting the DL config back to Automatic before the Postamble is initiated. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Function f_NR_SS_CellConfig_DciDlInfo_Automatic is used to configure the DL assignment back to Automatic. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TTCN implementation is not optimal |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.3.2.NR5GC, 7.1.1.3.2b.NR5GC, 7.1.1.4.1.1.NR5GC, 7.1.1.4.2.1.NR5GC, 7.1.1.4.2.3.NR5GC, 7.1.1.4.2.4.NR5GC, 7.1.1.4.2.5.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	4

2.3	Change 3	4

2.4	Change 4	5

2.5	Change 5	6

2.6	Change 6	7

2.7	Change 7	8

2.8	Change 8	8

2.9	Change 9	10

2.10	Change 10	11

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2024-06_D24wk37 related to the title of this CR.

Contact:  Bhise Parikshit
parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change 1

| Template name | f_TC_7_1_1_3_2_NR5GC |
| --- | --- |
| Reason for change | In TCs where DL assignment has been configured as explicit, TTCN is not putting the DL config back to Automatic before the Postamble is initiated. |
| Summary of change | Function f_NR_SS_CellConfig_DciDlInfo_Automatic is used to configure the DL assignment back to Automatic. |
| TTCN module | MAC_NR5GC.ttcn |

Before Change:

function f_TC_7_1_1_3_2_NR5GC() runs on NR5GC_PTC { // Logical channel prioritization handling <<SKIPPED CODE>> //Postamble f_NR_ULGrantConfiguration_Start(nr_Cell1); // Def UL Grant Config //@sic R5s230130 sic@ //f_NR5GC_OpenUE_TestLoopMode_Deactivate_TestMode(nr_Cell1); f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); };

After Change:

function f_TC_7_1_1_3_2_NR5GC() runs on NR5GC_PTC { // Logical channel prioritization handling <<SKIPPED CODE>> var NR_PhysicalParameters_Type v_NR_PhysicalParameters; <<SKIPPED CODE>> //Postamble f_NR_ULGrantConfiguration_Start(nr_Cell1); // Def UL Grant Config //@sic R5s230130 sic@ //f_NR5GC_OpenUE_TestLoopMode_Deactivate_TestMode(nr_Cell1); v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(nr_Cell1); f_NR_SS_CellConfig_DciDlInfo_Automatic(nr_Cell1, -, cs_NR_HarqProcessConfig_AnyProcess, cs_NR_DciFormat_1_1_Params(v_NR_PhysicalParameters.SubcarrierSpacing), v_NR_PhysicalParameters); //WA# f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); };

## Change 2

| Template name | f_TC_7_1_1_3_2b_NR5GC |
| --- | --- |
| Reason for change | In TCs where DL assignment has been configured as explicit, TTCN is not putting the DL config back to Automatic before the Postamble is initiated. |
| Summary of change | Function f_NR_SS_CellConfig_DciDlInfo_Automatic is used to configure the DL assignment back to Automatic. |
| TTCN module | MAC_NR5GC.ttcn |

Before Change:

function f_TC_7_1_1_3_2b_NR5GC() runs on NR5GC_PTC { // Logical channel prioritization handling with Mapping restrictions <<SKIPPED CODE>> //Postamble f_NR_ULGrantConfiguration_Start(nr_Cell1); // Def UL Grant Config //@sic R5s230130 sic@ //f_NR5GC_OpenUE_TestLoopMode_Deactivate_TestMode(nr_Cell1); f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); };

After Change:

function f_TC_7_1_1_3_2b_NR5GC() runs on NR5GC_PTC { // Logical channel prioritization handling with Mapping restrictions <<SKIPPED CODE>> var NR_PhysicalParameters_Type v_NR_PhysicalParameters; <<SKIPPED CODE>> //Postamble f_NR_ULGrantConfiguration_Start(nr_Cell1); // Def UL Grant Config //@sic R5s230130 sic@ //f_NR5GC_OpenUE_TestLoopMode_Deactivate_TestMode(nr_Cell1); v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(nr_Cell1); f_NR_SS_CellConfig_DciDlInfo_Automatic(nr_Cell1, -, cs_NR_HarqProcessConfig_AnyProcess, cs_NR_DciFormat_1_1_Params(v_NR_PhysicalParameters.SubcarrierSpacing), v_NR_PhysicalParameters); //WA# f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); };

## Change 3

| Template name | f_TC_7_1_1_4_1_1 |
| --- | --- |
| Reason for change | In TCs where DL assignment has been configured as explicit, TTCN is not putting the DL config back to Automatic before the Postamble is initiated. |
| Summary of change | Function f_NR_SS_CellConfig_DciDlInfo_Automatic is used to configure the DL assignment back to Automatic. |
| TTCN module | MAC_NR5GC.ttcn |

Before Change:

function f_TC_7_1_1_4_1_1_NR5GC() runs on NR5GC_PTC { // DL-SCH Transport Block Size selection / DCI format 1_0 <<SKIPPED CODE>> //Postamble f_NR_ULGrantConfiguration_Start(nr_Cell1); // Def UL Grant Config //@sic R5s230130 sic@ //f_NR5GC_OpenUE_TestLoopMode_Deactivate_TestMode(nr_Cell1); f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); };

After Change:

function f_TC_7_1_1_4_1_1_NR5GC() runs on NR5GC_PTC { // DL-SCH Transport Block Size selection / DCI format 1_0 <<SKIPPED CODE>> var NR_PhysicalParameters_Type v_NR_PhysicalParameters; <<SKIPPED CODE>> //Postamble f_NR_ULGrantConfiguration_Start(nr_Cell1); // Def UL Grant Config //@sic R5s230130 sic@ //f_NR5GC_OpenUE_TestLoopMode_Deactivate_TestMode(nr_Cell1); v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(nr_Cell1); f_NR_SS_CellConfig_DciDlInfo_Automatic(nr_Cell1, -, cs_NR_HarqProcessConfig_AnyProcess, cs_NR_DciFormat_1_0_Params(v_NR_PhysicalParameters.SubcarrierSpacing), v_NR_PhysicalParameters); //WA# f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); };

## Change 4

| Template name | f_TC_7_1_1_4_2_1 |
| --- | --- |
| Reason for change | In TCs where DL assignment has been configured as explicit, TTCN is not putting the DL config back to Automatic before the Postamble is initiated. |
| Summary of change | Function f_NR_SS_CellConfig_DciDlInfo_Automatic is used to configure the DL assignment back to Automatic. |
| TTCN module | MAC_NR5GC.ttcn |

Before Change:

function f_TC_7_1_1_4_2_1_NR5GC() runs on NR5GC_PTC { // UL-SCH Transport Block Size selection / DCI format 0_0 / Transform precoding disabled <<SKIPPED CODE>> //Postamble f_NR_ULGrantConfiguration_Start(nr_Cell1); // Def UL Grant Config //@sic R5s230130 sic@ //f_NR5GC_OpenUE_TestLoopMode_Deactivate_TestMode(nr_Cell1); f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); };

After Change:

function f_TC_7_1_1_4_2_1_NR5GC() runs on NR5GC_PTC { // UL-SCH Transport Block Size selection / DCI format 0_0 / Transform precoding disabled <<SKIPPED CODE>> var NR_PhysicalParameters_Type v_NR_PhysicalParameters; <<SKIPPED CODE>> //Postamble f_NR_ULGrantConfiguration_Start(nr_Cell1); // Def UL Grant Config //@sic R5s230130 sic@ //f_NR5GC_OpenUE_TestLoopMode_Deactivate_TestMode(nr_Cell1); v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(nr_Cell1); f_NR_SS_CellConfig_DciDlInfo_Automatic(nr_Cell1, -, cs_NR_HarqProcessConfig_AnyProcess, cs_NR_DciFormat_1_0_Params(v_NR_PhysicalParameters.SubcarrierSpacing), v_NR_PhysicalParameters); //WA# f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); };

## Change 5

| Template name | f_TC_7_1_1_4_2_3 |
| --- | --- |
| Reason for change | In TCs where DL assignment has been configured as explicit, TTCN is not putting the DL config back to Automatic before the Postamble is initiated. |
| Summary of change | Function f_NR_SS_CellConfig_DciDlInfo_Automatic is used to configure the DL assignment back to Automatic. |
| TTCN module | MAC_NR5GC.ttcn |

Before Change:

function f_TC_7_1_1_4_2_3_NR5GC() runs on NR5GC_PTC { // 7.1.1.4.2.3 UL-SCH transport block size selection / DCI format 0_1 / RA type 0/RA Type 1 / Transform precoding disabled <<SKIPPED CODE>> //Postamble f_NR_ULGrantConfiguration_Start(nr_Cell1); // Def UL Grant Config //@sic R5s230130 sic@ //f_NR5GC_OpenUE_TestLoopMode_Deactivate_TestMode(nr_Cell1); f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); };

After Change:

function f_TC_7_1_1_4_2_3_NR5GC() runs on NR5GC_PTC { // 7.1.1.4.2.3 UL-SCH transport block size selection / DCI format 0_1 / RA type 0/RA Type 1 / Transform precoding disabled <<SKIPPED CODE>> var NR_PhysicalParameters_Type v_NR_PhysicalParameters; <<SKIPPED CODE>> //Postamble f_NR_ULGrantConfiguration_Start(nr_Cell1); // Def UL Grant Config //@sic R5s230130 sic@ //f_NR5GC_OpenUE_TestLoopMode_Deactivate_TestMode(nr_Cell1); v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(nr_Cell1); f_NR_SS_CellConfig_DciDlInfo_Automatic(nr_Cell1, -, cs_NR_HarqProcessConfig_AnyProcess, cs_NR_DciFormat_1_1_Params(v_NR_PhysicalParameters.SubcarrierSpacing), v_NR_PhysicalParameters); //WA# f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); };

## Change 6

| Template name | f_TC_7_1_1_4_2_4 |
| --- | --- |
| Reason for change | In TCs where DL assignment has been configured as explicit, TTCN is not putting the DL config back to Automatic before the Postamble is initiated. |
| Summary of change | Function f_NR_SS_CellConfig_DciDlInfo_Automatic is used to configure the DL assignment back to Automatic. |
| TTCN module | MAC_NR5GC.ttcn |

Before Change:

function f_TC_7_1_1_4_2_4_NR5GC() runs on NR5GC_PTC { // 7.1.1.4.2.4 UL-SCH transport block size selection / DCI format 0_1 / RA type 0/RA Type 1 / 256QAM / Transform precoding disabled <<SKIPPED CODE>> //Postamble f_NR_ULGrantConfiguration_Start(nr_Cell1); // Def UL Grant Config //@sic R5s230130 sic@ //f_NR5GC_OpenUE_TestLoopMode_Deactivate_TestMode(nr_Cell1); f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); };

After Change:

function f_TC_7_1_1_4_2_4_NR5GC() runs on NR5GC_PTC { // 7.1.1.4.2.4 UL-SCH transport block size selection / DCI format 0_1 / RA type 0/RA Type 1 / 256QAM / Transform precoding disabled <<SKIPPED CODE>> var NR_PhysicalParameters_Type v_NR_PhysicalParameters; <<SKIPPED CODE>> //Postamble f_NR_ULGrantConfiguration_Start(nr_Cell1); // Def UL Grant Config //@sic R5s230130 sic@ //f_NR5GC_OpenUE_TestLoopMode_Deactivate_TestMode(nr_Cell1); v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(nr_Cell1); f_NR_SS_CellConfig_DciDlInfo_Automatic(nr_Cell1, -, cs_NR_HarqProcessConfig_AnyProcess, cs_NR_DciFormat_1_1_Params(v_NR_PhysicalParameters.SubcarrierSpacing), v_NR_PhysicalParameters); //WA# f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); };

## Change 7

| Template name | f_TC_7_1_1_4_2_5 |
| --- | --- |
| Reason for change | In TCs where DL assignment has been configured as explicit, TTCN is not putting the DL config back to Automatic before the Postamble is initiated. |
| Summary of change | Function f_NR_SS_CellConfig_DciDlInfo_Automatic is used to configure the DL assignment back to Automatic. |
| TTCN module | MAC_NR5GC.ttcn |

Before Change:

function f_TC_7_1_1_4_2_5_NR5GC() runs on NR5GC_PTC { // UL-SCH Transport Block Size selection / DCI format 0_0 / Transform precoding and 64QAM <<SKIPPED CODE>> //Postamble f_NR_ULGrantConfiguration_Start(nr_Cell1); // Def UL Grant Config //@sic R5s230130 sic@ //f_NR5GC_OpenUE_TestLoopMode_Deactivate_TestMode(nr_Cell1); f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); };

After Change:

function f_TC_7_1_1_4_2_5_NR5GC() runs on NR5GC_PTC { // UL-SCH Transport Block Size selection / DCI format 0_0 / Transform precoding and 64QAM <<SKIPPED CODE>> var NR_PhysicalParameters_Type v_NR_PhysicalParameters; <<SKIPPED CODE>> //Postamble f_NR_ULGrantConfiguration_Start(nr_Cell1); // Def UL Grant Config //@sic R5s230130 sic@ //f_NR5GC_OpenUE_TestLoopMode_Deactivate_TestMode(nr_Cell1); v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(nr_Cell1); f_NR_SS_CellConfig_DciDlInfo_Automatic(nr_Cell1, -, cs_NR_HarqProcessConfig_AnyProcess, cs_NR_DciFormat_1_0_Params(v_NR_PhysicalParameters.SubcarrierSpacing), v_NR_PhysicalParameters); //WA# f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); };

## Change 8

| Template name | f_NR_SS_CellConfig_DciDlInfo_Automatic |
| --- | --- |
| Reason for change | In TCs where DL assignment has been configured as explicit, TTCN is not putting the DL config back to Automatic before the Postamble is initiated. Hence, parameterising this function to accept both DCI X_0 and X_1 formats |
| Summary of change | Modified the function to accept p_DciFormatAndParams and p_PhysicalParameter |
| TTCN module | MAC_TC_Common_NR.ttcn |

Before Change:

function f_NR_SS_CellConfig_DciDlInfo_Automatic(NR_CellId_Type p_NR_CellId, template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now, template (omit) NR_HarqProcessConfig_Type p_HarqProcessConfig := omit ) runs on NR_BASE_PTC { var NR_PhysicalParameters_Type v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(p_NR_CellId); var template (value) NR_DciDlInfo_Type v_DciDlInfo := f_NR_DciDlInfo_Init(v_NR_PhysicalParameters, cs_NR_DciFormat_1_1_Params(v_NR_PhysicalParameters.SubcarrierSpacing), cs_NR_RedundancyVersionList_Def); //reconfigure only TransportBlockScheduling to default automatic f_NR_SS_CommonCellConfig(p_NR_CellId, cas_NR_DciDlInfo_REQ(p_NR_CellId, p_TimingInfo, cs_NR_DciDlInfo(cs_NR_DciFormat_1_X_ResourceAssignment(v_DciDlInfo.ResoureAssignment.ResourceAllocationType, v_DciDlInfo.ResoureAssignment.FreqDomain, // @sic R5s220687 sic@ v_DciDlInfo.ResoureAssignment.TimeDomain, v_DciDlInfo.ResoureAssignment.TransportBlockScheduling, p_HarqProcessConfig)))); };

After Change:

function f_NR_SS_CellConfig_DciDlInfo_Automatic(NR_CellId_Type p_NR_CellId, template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now, template (omit) NR_HarqProcessConfig_Type p_HarqProcessConfig := omit, template (value) NR_DciFormat_1_X_SpecificInfo_Type p_DciFormatAndParams, NR_PhysicalParameters_Type p_PhysicalParameters ) runs on NR_BASE_PTC { var template (value) NR_DciDlInfo_Type v_DciDlInfo := f_NR_DciDlInfo_Init(p_PhysicalParameters, p_DciFormatAndParams, cs_NR_RedundancyVersionList_Def); //reconfigure only TransportBlockScheduling to default automatic f_NR_SS_CommonCellConfig(p_NR_CellId, cas_NR_DciDlInfo_REQ(p_NR_CellId, p_TimingInfo, cs_NR_DciDlInfo(cs_NR_DciFormat_1_X_ResourceAssignment(v_DciDlInfo.ResoureAssignment.ResourceAllocationType, v_DciDlInfo.ResoureAssignment.FreqDomain, // @sic R5s220687 sic@ v_DciDlInfo.ResoureAssignment.TimeDomain, v_DciDlInfo.ResoureAssignment.TransportBlockScheduling, p_HarqProcessConfig)))); };

## Change 9

| Template name | f_TC_7_1_1_9_1_Step4Aato9A |
| --- | --- |
| Reason for change | Due to the changes mentioned in Change 8, using the correct parameters in the call to function Hence, parameterising this function f_NR_SS_CellConfig_DciDlInfo_Automatic |
| Summary of change | Used the parameters v_NR_PhysicalParameters and cs_NR_DciFormat_1_1_Params(v_NR_PhysicalParameters.SubcarrierSpacing) |
| TTCN module | MAC_TC_Common_NR.ttcn |

Before Change:

function f_TC_7_1_1_9_1_Step4Aato9A ( DRB_Identity p_NR_DRB_Id, B6_Type p_LCID, template (value) NR_RadioBearerList_Type p_NR_RadioBearerList, boolean p_StopGrant:= true ) runs on NR_BASE_PTC return boolean { <<SKIPPED CODE>> //@siclog "Step 9A" siclog@ // The SS transmits a STATUS PDU. // Restrict the SS to use of the HARQ proccess id used by the test case f_NR_SS_CellConfig_DciDlInfo_Automatic(nr_Cell1, -, v_NR_HarqProcessConfig_DL); //@sic R5s211710 R5s220506 sic@ <<SKIPPED CODE>> };

After Change:

function f_TC_7_1_1_9_1_Step4Aato9A ( DRB_Identity p_NR_DRB_Id, B6_Type p_LCID, template (value) NR_RadioBearerList_Type p_NR_RadioBearerList, boolean p_StopGrant:= true ) runs on NR_BASE_PTC return boolean { <<SKIPPED CODE>> var NR_PhysicalParameters_Type v_NR_PhysicalParameters; <<SKIPPED CODE>> //@siclog "Step 9A" siclog@ // The SS transmits a STATUS PDU. // Restrict the SS to use of the HARQ proccess id used by the test case v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(nr_Cell1); f_NR_SS_CellConfig_DciDlInfo_Automatic(nr_Cell1, -, v_NR_HarqProcessConfig_DL, cs_NR_DciFormat_1_1_Params(v_NR_PhysicalParameters.SubcarrierSpacing), v_NR_PhysicalParameters); //WA# <<SKIPPED CODE>> };

## Change 10

| Template name | f_TC_7_1_1_9_1_Step24to29 |
| --- | --- |
| Reason for change | Due to the changes mentioned in Change 8, using the correct parameters in the call to function Hence, parameterising this function f_NR_SS_CellConfig_DciDlInfo_Automatic |
| Summary of change | Used the parameters v_NR_PhysicalParameters and cs_NR_DciFormat_1_1_Params(v_NR_PhysicalParameters.SubcarrierSpacing) |
| TTCN module | MAC_TC_Common_NR.ttcn |

Before Change:

function f_TC_7_1_1_9_1_Step24to29 ( DRB_Identity p_NR_DRB_Id, B6_Type p_LCID, template (value) NR_RadioBearerList_Type p_NR_RadioBearerList, boolean p_StopGrant:= true ) runs on NR_BASE_PTC return boolean { <<SKIPPED CODE>> // @siclog "Step 29" siclog@ //The SS transmits a STATUS PDU. // Restrict the SS to use of the HARQ proccess id used by the test case f_NR_SS_CellConfig_DciDlInfo_Automatic(nr_Cell1, -, v_NR_HarqProcessConfig_DL); //@sic R5s211710 R5s220506 sic@ <<SKIPPED CODE>> };

After Change:

function f_TC_7_1_1_9_1_Step24to29 ( DRB_Identity p_NR_DRB_Id, B6_Type p_LCID, template (value) NR_RadioBearerList_Type p_NR_RadioBearerList, boolean p_StopGrant:= true ) runs on NR_BASE_PTC return boolean { <<SKIPPED CODE>> var NR_PhysicalParameters_Type v_NR_PhysicalParameters; <<SKIPPED CODE>> // @siclog "Step 29" siclog@ //The SS transmits a STATUS PDU. // Restrict the SS to use of the HARQ proccess id used by the test case v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(nr_Cell1); f_NR_SS_CellConfig_DciDlInfo_Automatic(nr_Cell1, -, v_NR_HarqProcessConfig_DL, cs_NR_DciFormat_1_1_Params(v_NR_PhysicalParameters.SubcarrierSpacing), v_NR_PhysicalParameters); //WA# <<SKIPPED CODE>> };