3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240573

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3532 | rev | - | Current version: | 17.11.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to the function f_EUTRA_RRC_Procedure_Latency |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-09-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the current TTCN implementation of function f_EUTRA_RRC_Procedure_Latency, t_Timer is started with buffer time of 50 ms . To improve the stability of this case and also take care of Harq retransmissions it is proposed to increase the buffer time from 50ms to 150ms . Increasing this timer does not affect the test purpose , as the latency check is in place to make sure the correct delay is used between Harq ACK reception and SRB COMMON IND Uplink message. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to increase the buffer time of the t_Watchdog from 50ms to 150ms. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Testcase will remain unstable and may fail a conformant UE |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.6.2.1.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

2.1	Change 1	2

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2024-06_D24wk37 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change 1

| Function name | f_EUTRA_RRC_Procedure_Latency |
| --- | --- |
| Reason for change | In the current TTCN implementation of function f_EUTRA_RRC_Procedure_Latency, t_Timer is started with buffer time of 50 ms . To improve the stability of this case and also take care of Harq retransmissions it is proposed to increase the buffer time from 50ms to 150ms . Increasing this timer does not affect the test purpose , as the latency check is in place to make sure the correct delay is used between Harq ACK reception and SRB COMMON IND Uplink message. |
| Summary of change | It is proposed to increase the buffer time of the t_Watchdog from 50ms to 150ms. |
| TTCN module | ttcn/develop/Common/EUTRA/EUTRA_CommonProcedures.ttcn |
| MCC160 Comment |  |

Before Change:

function f_EUTRA_RRC_Procedure_Latency( EUTRA_CellId_Type p_CellId, template (value) SRB_COMMON_REQ p_SRB_COMMON_REQ_1, template (omit) SRB_COMMON_REQ p_SRB_COMMON_REQ_2 := omit, template (present) SRB_COMMON_IND p_SRB_COMMON_IND_1, template SRB_COMMON_IND p_SRB_COMMON_IND_2 := omit, boolean p_IsFDD, integer p_N, charstring p_TestcaseStep, charstring p_UE_Cat, //@sic R5-153973 to be set to true for UE capability sic@ boolean p_BigULGrant := false, integer p_DelayGrantToDataAddition := 0) // @sic R5s230223 sic@ runs on EUTRA_PTC return SRB_COMMON_IND { ……………. v_T5 := f_EUTRA_TimingInfoAdd(v_T0, ((v_GrantCycle * (v_NoOfGrants - 1)) + p_N)); // @sic R5s200666 sic@ /* T5: last UL grant is sent out; @sic R5s130951 sic@ */ v_Tnow := f_EUTRA_GetCurrentTiming(p_CellId); t_Timer.start (int2float(f_SubFrameTiming_Duration(v_Tnow, v_T5) + 50) / 1000.0); /* 50ms added to be at the safe side */ …… }

After Change:

function f_EUTRA_RRC_Procedure_Latency( EUTRA_CellId_Type p_CellId, template (value) SRB_COMMON_REQ p_SRB_COMMON_REQ_1, template (omit) SRB_COMMON_REQ p_SRB_COMMON_REQ_2 := omit, template (present) SRB_COMMON_IND p_SRB_COMMON_IND_1, template SRB_COMMON_IND p_SRB_COMMON_IND_2 := omit, boolean p_IsFDD, integer p_N, charstring p_TestcaseStep, charstring p_UE_Cat, //@sic R5-153973 to be set to true for UE capability sic@ boolean p_BigULGrant := false, integer p_DelayGrantToDataAddition := 0) // @sic R5s230223 sic@ runs on EUTRA_PTC return SRB_COMMON_IND { ……………. v_T5 := f_EUTRA_TimingInfoAdd(v_T0, ((v_GrantCycle * (v_NoOfGrants - 1)) + p_N)); // @sic R5s200666 sic@ /* T5: last UL grant is sent out; @sic R5s130951 sic@ */ v_Tnow := f_EUTRA_GetCurrentTiming(p_CellId); t_Timer.start (int2float(f_SubFrameTiming_Duration(v_Tnow, v_T5) + 150) / 1000.0); /* 150ms added to be at the safe side */ //WA#WI=1318859 …… }