3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240586

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3543 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcase 8.1.4.1.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-09-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Some UEs are having the implementation to do Pre-assembled data packet, which means they get the data to be sent ready first and transmit to MAC layer. In the current implementation of the case 8.1.4.1.2, at steps 14-15 to avoid the collision between the final reconfiguration complete and registration request, Handover functions are split. Expectation is that UE sends reconfiguration complete using the default grant tsc_NR_Msg4GrantHO_Def := 144 ; But some UEs are already having the PDCP status report available in MAC layer , by the time they transmit reconfiguration complete , these UE behavior choose to make use of Long BSR (there're 2 LCGs have data ) instead of Short BSR . this requires more grant to be given during Msg4 UL Grant HO. With the current grant allocation, UE is sending only the first segments of reconfiguration complete using Msg4 UL Grant , thus resulting the remaining segment of reconfiguration complete and registration request come together which fails the case . To avoid the issue , it is proposed to make use of the grant size as 160 only for this case . |  |  |  |  |  |  |  |  |
| Summary of change: |  | 160 bits of grant is provided as part of Msg4 UL grant |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE will fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.4.1.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	3

2	Corrections required	3

2.1	Change to the Function fl_TC_8_1_4_1_2_TestBody	3

2.2	Change to the Function f_NR5GC_508RRC_IntraNR_HO_InterCell_Steps1_12	6

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2024-06_D24wk37 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change to the Function fl_TC_8_1_4_1_2_TestBody

| Function name | fl_TC_8_1_4_1_2_TestBody |
| --- | --- |
| Reason for change | Some UEs are having the implementation to do Pre-assembled data packet, which means they get the data to be sent ready first and transmit to MAC layer. In the current implementation of the case 8.1.4.1.2, at steps 14-15 to avoid the collision between the final reconfiguration complete and registration request, Handover functions are split. Expectation is that UE sends reconfiguration complete using the default grant tsc_NR_Msg4GrantHO_Def := 144 ; But some UEs are already having the PDCP status report available in MAC layer , by the time they transmit reconfiguration complete , these UE behavior choose to make use of Long BSR (there're 2 LCGs have data ) instead of Short BSR . this requires more grant to be given during Msg4 UL Grant HO. With the current grant allocation, UE is sending only the first segments of reconfiguration complete using Msg4 UL Grant , thus resulting the remaining segment of reconfiguration complete and registration request come together which fails the case . To avoid the issue , it is proposed to make use of the grant size as 160 only for this case . |
| Summary of change | 160 bits of grant is provided as part of Msg4 UL grant |
| TTCN module | NR5GC\8_1_4\RRC_IntraNR_Handover_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function fl_TC_8_1_4_1_2_TestBody() runs on NR5GC_PTC { …….. //@siclog "Steps 14-15" siclog@ //The SS transmits an RRCReconfiguration including dedicatedSIB1-Delivery containing SIB1 of NR Cell 3 to order the UE to perform handover to NR Cell 3 //Check: Does the UE transmit an RRCReconfigurationComplete message on NR Cell 3? v_SIB1 := f_NR_GetSIB1(nr_Cell3); v_SIB1.cellAccessRelatedInfo.plmn_IdentityInfoList[0].trackingAreaCode := int2bit(2,24); //@sic R5s211526 BASELINE MOVING 2021 sic@ f_NR_CellInfo_SetTAC(nr_Cell3, int2bit(2,24)); //@sic R5s200309 sic@ //@sic R5s201303 R5s210135 sic@ //To avoid a possible collision between the final clean of the target cell and the UE sending a RegistrationRequest, the HO procedure is split f_NR5GC_508RRC_IntraNR_HO_InterCell_Steps1_12(nr_Cell1, nr_Cell3, -, false, // RBConfig_KeyChange := false -, v_SIB1); // SIB1 f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 15"); ………….. }

After Change:

function fl_TC_8_1_4_1_2_TestBody() runs on NR5GC_PTC { ………….. var NR_PhysicalParameters_Type v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(nr_Cell3); //WA#WI=1323666 ……… //@siclog "Steps 14-15" siclog@ //The SS transmits an RRCReconfiguration including dedicatedSIB1-Delivery containing SIB1 of NR Cell 3 to order the UE to perform handover to NR Cell 3 //Check: Does the UE transmit an RRCReconfigurationComplete message on NR Cell 3? v_SIB1 := f_NR_GetSIB1(nr_Cell3); v_SIB1.cellAccessRelatedInfo.plmn_IdentityInfoList[0].trackingAreaCode := int2bit(2,24); //@sic R5s211526 BASELINE MOVING 2021 sic@ f_NR_CellInfo_SetTAC(nr_Cell3, int2bit(2,24)); //@sic R5s200309 sic@ v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(nr_Cell3); //WA#WI=1323666 //@sic R5s201303 R5s210135 sic@ //To avoid a possible collision between the final clean of the target cell and the UE sending a RegistrationRequest, the HO procedure is split f_NR5GC_508RRC_IntraNR_HO_InterCell_Steps1_12(nr_Cell1, nr_Cell3, -, false, // RBConfig_KeyChange := false -, v_SIB1, -, -, -, cs_NR_RachProcedureConfig_CRNTI_HO(v_NR_PhysicalParameters, tsc_C_RNTI_Value3, 160)); // SIB1 //WA#WI=1323666 f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 15"); ………….. }

## Change to the Function f_NR5GC_508RRC_IntraNR_HO_InterCell_Steps1_12

| Function name | f_NR5GC_508RRC_IntraNR_HO_InterCell_Steps1_12 |
| --- | --- |
| Reason for change | To fulfil the above requirement , rach configuration is parameterized . Default value is used when no configuration is given by the case . Special configuration is used when rach configuration is provided by the case as in 8.1.4.1.2.NR5GC |
| Summary of change | rach configuration is parameterized |
| TTCN module | Common\NR5GC\NR5GC_Handover.ttcn |
| MCC160 Comment |  |

Before Change:

function f_NR5GC_508RRC_IntraNR_HO_InterCell_Steps1_12(NR_CellId_Type p_SourceCellId, NR_CellId_Type p_TargetCellId, RNTI_Value p_RNTI_Value := tsc_C_RNTI_Value3, boolean p_RBConfig_KeyChange := true, template (omit) MeasConfig p_MeasConfig := omit, template (omit) SIB1 p_SIB1 := omit, template (omit) SystemInformation p_SysInfo := omit, template (omit) MasterKeyUpdate p_MasterKeyUpdate := omit, template (omit) ReconfigurationWithSync.rach_ConfigDedicated p_RACH_ConfigDedicated := omit ) runs on NR5GC_PTC { //@sic R5s2008190 sic@ var NR_PhysicalParameters_Type v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(p_TargetCellId); var template (value) NR_RachProcedureConfig_Type v_RachProcedureConfig := cs_NR_RachProcedureConfig_CRNTI_HO(v_NR_PhysicalParameters, p_RNTI_Value); //CBRA var template (value) DL_DCCH_Message v_RRCReconfiguration; var template (value) NR_DRBInfoList_Type v_DRBInfoList := f_NR5GC_MobileInfo_GetDRBInfoList(); var NextHopChainingCount v_NCC; if (isvalue(p_RACH_ConfigDedicated.uplink)){ //CFRA v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(p_TargetCellId); v_RachProcedureConfig := cs_NR_RachProcedureConfig_CFRA(v_NR_PhysicalParameters); } ……….. }

After Change:

function f_NR5GC_508RRC_IntraNR_HO_InterCell_Steps1_12(NR_CellId_Type p_SourceCellId, NR_CellId_Type p_TargetCellId, RNTI_Value p_RNTI_Value := tsc_C_RNTI_Value3, boolean p_RBConfig_KeyChange := true, template (omit) MeasConfig p_MeasConfig := omit, template (omit) SIB1 p_SIB1 := omit, template (omit) SystemInformation p_SysInfo := omit, template (omit) MasterKeyUpdate p_MasterKeyUpdate := omit, template (omit) ReconfigurationWithSync.rach_ConfigDedicated p_RACH_ConfigDedicated := omit, template (omit) NR_RachProcedureConfig_Type p_RachProcedureConfig := omit //WA#WI=1323666 ) runs on NR5GC_PTC { //@sic R5s2008190 sic@ var NR_PhysicalParameters_Type v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(p_TargetCellId); var template (value) NR_RachProcedureConfig_Type v_RachProcedureConfig;// := cs_NR_RachProcedureConfig_CRNTI_HO(v_NR_PhysicalParameters, p_RNTI_Value); //CBRA //WA#WI=1323666 var template (value) DL_DCCH_Message v_RRCReconfiguration; var template (value) NR_DRBInfoList_Type v_DRBInfoList := f_NR5GC_MobileInfo_GetDRBInfoList(); var NextHopChainingCount v_NCC; if(not(isvalue(p_RachProcedureConfig))) //WA#WI=1323666 { v_RachProcedureConfig := cs_NR_RachProcedureConfig_CRNTI_HO(v_NR_PhysicalParameters, p_RNTI_Value); } else{ v_RachProcedureConfig := p_RachProcedureConfig; //WA#WI=1323666 } if (isvalue(p_RACH_ConfigDedicated.uplink)){ //CFRA v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(p_TargetCellId); v_RachProcedureConfig := cs_NR_RachProcedureConfig_CFRA(v_NR_PhysicalParameters); } …………. }