3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240605

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4851 | rev | 1 | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for LTE EIEI test case 11.3.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, EIEI-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Current implementation of eCall 11.3.2 test case presents few issues that needs to be addressed and corrected: - Uri formed in f_SipUrl_eCall_RX is incomplete when call to be established is a TEST call. A tel URI if not represented in global notation, must be represented in local which is the case, a phone-context tag must be included. Reference at rfc3966 cl. 5.1. - At Step 2 Paralell behaviour, in alt statement, v_Protocol and v_MessageHeader_Response are uninitialized and later assigned a value after entered one of the parallel behaviours, this can cause an issue because the second option uses them as input parameters with the consecuence of a runtime error. This issue can be sorted initializing the variable before entering the alt statement. Protocol as well as message header contents used by UE while sending 200 OK will be checked in a_IMS_ReceiveResponse200OK function. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Added phone-context to TEST tel uri. - Initialized input parameters on check statement to avoid runtime errors. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	3

2	Corrections required	3

2.1	Correction to function f_SipUrl_eCall_RX	3

2.2	Correction to function f_TC_11_3_2_IMS1	4

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D24wk24 related to the title of this CR.

Contact: 	Juan Garcia Martin

Juan.GarciaMartin@rohde-schwarz.com

# Corrections required

## Correction to function f_SipUrl_eCall_RX

| Function name | f_SipUrl_eCall_RX |
| --- | --- |
| Reason for change | Uri formed in f_SipUrl_eCall_RX is incomplete when call to be established is a TEST call. A tel URI if not represented in global notation, must be represented in local which is the case, a phone-context tag must be included. Reference at rfc3966 cl. 5.1. |
| Summary of change | Added phone-context to TEST tel uri. |
| TTCN module | IMS_eCall_Functions.ttcn |
| MCC160 Comment |  |

Before Change:

function f_SipUrl_eCall_RX(ECall_Type p_ECallType) return template (present) SipUrl { var template (present) SipUrl v_SipUrl; select (p_ECallType) { case (MANUAL) { v_SipUrl := cr_SipUrl_Common(c_urnScheme, cs_UriComponents_eCall("sos.ecall.manual")) } case (AUTOMATIC) { v_SipUrl := cr_SipUrl_Common(c_urnScheme, cs_UriComponents_eCall("sos.ecall.automatic")) } case (TEST) { /* @sic R5-220971 sic@ */ v_SipUrl := cr_SipUrl_Common(c_telScheme, cr_TelUriComponent(tsc_ECallTestService)); } case else { FatalError(__FILE__, __LINE__, ""); } } return v_SipUrl; }

After Change:

function f_SipUrl_eCall_RX(ECall_Type p_ECallType) return template (present) SipUrl { var template (present) SipUrl v_SipUrl; select (p_ECallType) { case (MANUAL) { v_SipUrl := cr_SipUrl_Common(c_urnScheme, cs_UriComponents_eCall("sos.ecall.manual")) } case (AUTOMATIC) { v_SipUrl := cr_SipUrl_Common(c_urnScheme, cs_UriComponents_eCall("sos.ecall.automatic")) } case (TEST) { /* @sic R5-220971 sic@ */ v_SipUrl := cr_SipUrl_Common(c_telScheme, cr_TelUriComponent(tsc_ECallTestService & ";phone-context=" & px_IMS_HomeDomainName)); } case else { FatalError(__FILE__, __LINE__, ""); } } return v_SipUrl; }

## Correction to function f_TC_11_3_2_IMS1

| Function name | f_TC_11_3_2_IMS1 |
| --- | --- |
| Reason for change | At Step 2 Paralell behaviour, in alt statement, v_Protocol and v_MessageHeader_Response are uninitialized and later assigned a value after entered one of the parallel behaviours, this can cause an issue because the second option uses them as input parameters with the consecuence of a runtime error. This issue can be sorted initializing the variable before entering the alt statement. Protocol as well as message header contents used by UE while sending 200 OK will be checked in a_IMS_ReceiveResponse200OK function. |
| Summary of change | Initialized input parameters on check statement to avoid runtime errors. |
| TTCN module | IMS_ECall_IMS.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_11_3_2_IMS1() runs on IMS_PTC { var IMS_DATA_REQ v_IMS_REGISTER_REQ; var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp; var SipUrl v_ContactUrl; var REGISTER_Request v_RegisterRequest; var INVITE_Request v_InviteRequest; var template (value) MessageHeader v_MessageHeader_Notify; var template (present) MessageHeader v_MessageHeader_Response; var InternetProtocol_Type v_Protocol; var IMS_ResponseWithSdp_Type v_ResponseWithSdp; var IMS_Response_Type v_SessionProgress183; var boolean v_FinishedRegistration := false; var boolean v_RxdInvite := false; f_IMS_PTC_Init(PDN_1); // Don't need to wait for trigger as 1st message UL and won't be sent until PDN is in place // @siclog "Step 1 Parallel Behaviour" siclog@ // steps 4-7 of registration procedure v_IMS_REGISTER_REQ := f_IMS_Register(); v_RegisterRequest := v_IMS_REGISTER_REQ.Request.Register; v_ContactUrl := f_MessageHeader_GetContactSipUrl(v_RegisterRequest.msgHeader); // @siclog "Step 2 Parallel Behaviour" siclog@ // receive INVITE in parallel to steps 8-11 of registration procedure alt { // @sic R5-227567 sic@ // Registration steps 8-9 [] IMS_Server.check(receive(car_IMS_Subscribe_Request())) { f_IMS_Register_Subscribe(); // Registration step 10 v_MessageHeader_Notify := f_IMS_NotifyRequest_MessageHeader_A_1_6(cs_SubscriptionState_Active); /* @sic R5-153756, R5-153986 sic@ */ v_Protocol := f_IMS_PTC_ImsInfo_GetTransportProtocol(); // @sic R5s230164 sic@ IMS_Client.send(cas_IMS_Notify_Request(cs_IMS_RoutingInfo(v_Protocol), cs_NOTIFY_Request(v_ContactUrl, v_MessageHeader_Notify, cs_MessageBody_RegInfo(f_IMS_XmlMessageForNotify_Active(v_RegisterRequest))))); v_MessageHeader_Response := f_IMS_OtherResponse_200_MessageHeaderRX(v_MessageHeader_Notify); // @sic R5230388 sic@ repeat; } // Registration step 11 [] IMS_Client.check(receive(car_IMS_DATA_RSP(cr_IMS_RoutingInfo(v_Protocol), cr_Response(c_statusLine200, v_MessageHeader_Response)))) { // @sic R5s230388 sic@ a_IMS_ReceiveResponse200OK(v_MessageHeader_Notify, v_Protocol); v_FinishedRegistration := true; if (not v_RxdInvite){ repeat; } } // INVITE [] IMS_Server.check(receive(car_IMS_Invite_Request())) { v_InviteRequestWithSdp := f_ECall_InitialInvite_AnnexC47(TEST, A_2_1_A4); /* @sic R5-220971, R5-221073, R5s230164 sic@ */ f_IMS_PreliminaryPass(__FILE__, __LINE__, "INVITE Received"); v_RxdInvite := true; if (not v_FinishedRegistration) { repeat; } } // allow the Register message to be repeated [] IMS_Server.receive(cr_IMS_DATA_REQ_Any(v_IMS_REGISTER_REQ.RoutingInfo, v_IMS_REGISTER_REQ.Request)) { repeat; } } // Inform EUTRA registration finished and Invite received f_IMS_IPCAN_SendCoOrdMsg(IPCAN); <<SKIPPED CODE>>

After Change:

function f_TC_11_3_2_IMS1() runs on IMS_PTC { var IMS_DATA_REQ v_IMS_REGISTER_REQ; var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp; var SipUrl v_ContactUrl; var REGISTER_Request v_RegisterRequest; var INVITE_Request v_InviteRequest; var template (value) MessageHeader v_MessageHeader_Notify; var template (present) MessageHeader v_MessageHeader_Response := ?; var InternetProtocol_Type v_Protocol; var IMS_ResponseWithSdp_Type v_ResponseWithSdp; var IMS_Response_Type v_SessionProgress183; var boolean v_FinishedRegistration := false; var boolean v_RxdInvite := false; f_IMS_PTC_Init(PDN_1); // Don't need to wait for trigger as 1st message UL and won't be sent until PDN is in place // @siclog "Step 1 Parallel Behaviour" siclog@ // steps 4-7 of registration procedure v_IMS_REGISTER_REQ := f_IMS_Register(); v_RegisterRequest := v_IMS_REGISTER_REQ.Request.Register; v_ContactUrl := f_MessageHeader_GetContactSipUrl(v_RegisterRequest.msgHeader); // @siclog "Step 2 Parallel Behaviour" siclog@ // receive INVITE in parallel to steps 8-11 of registration procedure alt { // @sic R5-227567 sic@ // Registration steps 8-9 [] IMS_Server.check(receive(car_IMS_Subscribe_Request())) { f_IMS_Register_Subscribe(); // Registration step 10 v_MessageHeader_Notify := f_IMS_NotifyRequest_MessageHeader_A_1_6(cs_SubscriptionState_Active); /* @sic R5-153756, R5-153986 sic@ */ v_Protocol := f_IMS_PTC_ImsInfo_GetTransportProtocol(); // @sic R5s230164 sic@ IMS_Client.send(cas_IMS_Notify_Request(cs_IMS_RoutingInfo(v_Protocol), cs_NOTIFY_Request(v_ContactUrl, v_MessageHeader_Notify, cs_MessageBody_RegInfo(f_IMS_XmlMessageForNotify_Active(v_RegisterRequest))))); v_MessageHeader_Response := f_IMS_OtherResponse_200_MessageHeaderRX(v_MessageHeader_Notify); // @sic R5230388 sic@ repeat; } // Registration step 11 [] IMS_Client.check(receive(car_IMS_DATA_RSP(cr_IMS_RoutingInfo(), cr_Response(c_statusLine200, v_MessageHeader_Response)))) { // @sic R5s230388 sic@ a_IMS_ReceiveResponse200OK(v_MessageHeader_Notify, v_Protocol); v_FinishedRegistration := true; if (not v_RxdInvite){ repeat; } } // INVITE [] IMS_Server.check(receive(car_IMS_Invite_Request())) { v_InviteRequestWithSdp := f_ECall_InitialInvite_AnnexC47(TEST, A_2_1_A4); /* @sic R5-220971, R5-221073, R5s230164 sic@ */ f_IMS_PreliminaryPass(__FILE__, __LINE__, "INVITE Received"); v_RxdInvite := true; if (not v_FinishedRegistration) { repeat; } } // allow the Register message to be repeated [] IMS_Server.receive(cr_IMS_DATA_REQ_Any(v_IMS_REGISTER_REQ.RoutingInfo, v_IMS_REGISTER_REQ.Request)) { repeat; } } // Inform EUTRA registration finished and Invite received f_IMS_IPCAN_SendCoOrdMsg(IPCAN); <<SKIPPED CODE>>