3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240667

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3526 | rev | 1 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR MAC test case 7.1.1.5.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-09-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Step 11 of TS 38.523-1 Table 7.1.1.5.1.3.2-1 states as below: “In the first PDCCH occasion when the Drx-RetransmissionTimerDL for the MAC PDU in Step 9 is started (i.e. after expiry of drx-HARQ-RTT-TimerDL after step 9), the SS indicates the transmission of a DL MAC PDU on the PDCCH.” According to TS 38.321 sec 5.7: When DRX is configured, the MAC entity shall: 1> if a MAC PDU is received in a configured downlink assignment: 2> start the drx-HARQ-RTT-TimerDL for the corresponding HARQ process in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback; 2> stop the drx-RetransmissionTimerDL for the corresponding HARQ process. With respect to the above clause, the drx-HARQ-RTT-TimerDL timer would be started by the UE after the transmission carrying the DL HARQ feedback for the invalid DL MAC PDU transmitted at Step 9. However, TTCN is subtracting 1SF (FDD) / 1 Slot (TDD) when calculating the timing for the transmission of the DL MAC PDU at Step 11 with the consequence that the UE is in inactive time as drx-HARQ-RTT-TimerDL is still running at its end when the transmission on PDCCH occurs and hence, is unable to monitor the PDCCH transmission. Hence, TTCN needs to be corrected to fully comply with the Step 11. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Corrected the calculated timing at Step 11 not to subtract from tsc_DRX_HARQ_RTT_TimerDL |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant shall fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.5.1.NR5GC, 7.1.1.5.1.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	4

2	Corrections required	4

2.1	Change 1	4

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2024-06_D24wk37 related to the title of this CR.

Contact:  Bhise Parikshit
parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change 1

| Template name | f_TC_7_1_1_5_1_NR_TestBody |
| --- | --- |
| Reason for change | Step 11 of TS 38.523-1 Table 7.1.1.5.1.3.2-1 states as below: “In the first PDCCH occasion when the Drx-RetransmissionTimerDL for the MAC PDU in Step 9 is started (i.e. after expiry of drx-HARQ-RTT-TimerDL after step 9), the SS indicates the transmission of a DL MAC PDU on the PDCCH.” According to TS 38.321 sec 5.7: When DRX is configured, the MAC entity shall: 1> if a MAC PDU is received in a configured downlink assignment: 2> start the drx-HARQ-RTT-TimerDL for the corresponding HARQ process in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback; 2> stop the drx-RetransmissionTimerDL for the corresponding HARQ process. With respect to the above clause, the drx-HARQ-RTT-TimerDL timer would be started by the UE after the transmission carrying the DL HARQ feedback for the invalid DL MAC PDU transmitted at Step 9. However, TTCN is subtracting 1SF (FDD) / 1 Slot (TDD) when calculating the timing for the transmission of the DL MAC PDU at Step 11 with the consequence that the UE is in inactive time as drx-HARQ-RTT-TimerDL is still running at its end when the transmission on PDCCH occurs and hence, is unable to monitor the PDCCH transmission. Hence, TTCN needs to be corrected to fully comply with the Step 11 |
| Summary of change | Corrected the calculated timing at Step 11 not to subtract from tsc_DRX_HARQ_RTT_TimerDL |
| TTCN module | MAC_SPS_TC_Common_NR.ttcn |

Before Change:

function f_TC_7_1_1_5_1_NR_TestBody(DRB_Identity p_NR_DRB_Id, float p_WatchDog := 5.0) runs on NR_BASE_PTC { <<SKIPPED CODE>> // set CRC error as normal mode //v_Timing2 := f_NR_SubFrameTiming_AddSlots(v_Timing1, tsc_DRX_HARQ_RTT_TimerDL); //@sic R5s211597 sic@ //@siclog "Step 11" siclog@ //In the first PDCCH occasion when the Drx-RetransmissionTimerDL for the MAC PDU in Step 9 is started(i.e. after expiry of drx-HARQ-RTT-TimerDL after step 9), v_Timing2 := f_NR_SubFrameTiming_AddSlots(v_Timing1,v_NR_SlotOffsetK1 + tsc_DRX_HARQ_RTT_TimerDL - 1); v_Timing2 := f_NR_NextDL_Slot(nr_Cell1,v_Timing2); f_NR_SS_L1_TestModeConfig(nr_Cell1, cas_NR_L1_TestMode_DL_SCH_CRC_REQ(nr_Cell1, Normal,cs_TimingInfo_NR(v_Timing2))); <<SKIPPED CODE>>

After Change:

function f_TC_7_1_1_5_1_NR_TestBody(DRB_Identity p_NR_DRB_Id, float p_WatchDog := 5.0) runs on NR_BASE_PTC { <<SKIPPED CODE>> // set CRC error as normal mode //v_Timing2 := f_NR_SubFrameTiming_AddSlots(v_Timing1, tsc_DRX_HARQ_RTT_TimerDL); //@sic R5s211597 sic@ //@siclog "Step 11" siclog@ //In the first PDCCH occasion when the Drx-RetransmissionTimerDL for the MAC PDU in Step 9 is started(i.e. after expiry of drx-HARQ-RTT-TimerDL after step 9), v_Timing2 := f_NR_SubFrameTiming_AddSlots(v_Timing1,v_NR_SlotOffsetK1 + tsc_DRX_HARQ_RTT_TimerDL); //WA# v_Timing2 := f_NR_NextDL_Slot(nr_Cell1,v_Timing2); f_NR_SS_L1_TestModeConfig(nr_Cell1, cas_NR_L1_TestMode_DL_SCH_CRC_REQ(nr_Cell1, Normal,cs_TimingInfo_NR(v_Timing2))); <<SKIPPED CODE>> }