3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240675

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3586 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to the function f_NR5GC_CheckDataPath_CA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-11-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the current TTCN implementation of the function f_NR5GC_CheckDataPath_CA, At step 1 : SS transmits an Activation MAC Control element to activate Scell At step 2 : IP data is sent using Scell Mac Bearer Routing Step 1 and 2 are done with immediate activation . There is an instability observed, when the data is transmitted even before UE sends HARQ ACK for the Scell Activation MAC CE . It is proposed to add 50ms delay ( as a minimum ) so that , TTCN can ensure that the Scell is activated, before trying to send the IP Data using Scell MAC Bearer Routing |  |  |  |  |  |  |  |  |
| Summary of change: |  | 50ms delay is added between step 1 and step 2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Confirmant UE may fail the case due to timing issues |  |  |  |  |  |  |  |  |
| Clauses affected: |  | All the CA Test cases involving Scell Activation MAC CE |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

2.1	Change to the function f_NR5GC_CheckDataPath_CA	2

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2024-06_D24wk37 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change to the function f_NR5GC_CheckDataPath_CA

| Function name | f_NR5GC_CheckDataPath_CA |
| --- | --- |
| Reason for change | In the current TTCN implementation of the function f_NR5GC_CheckDataPath_CA, At step 1 : SS transmits an Activation MAC Control element to activate Scell At step 2 : IP data is sent using Scell Mac Bearer Routing Step 1 and 2 are done with immediate activation . There is an instability observed, when the data is transmitted even before UE sends HARQ ACK for the Scell Activation MAC CE . It is proposed to add 50ms delay ( as a minimum ) so that , TTCN can ensure that the Scell is activated, before trying to send the IP Data using Scell MAC Bearer Routing |
| Summary of change | 50ms delay is added between step 1 and step 2 |
| TTCN module | develop\Common\NR5GC\NR5GC_CommonFunctions_CA.ttcn |
| MCC160 Comment |  |

Before Change:

function f_NR5GC_CheckDataPath_CA(NR_CellId_Type p_PCell, NR_CellId_Type p_SCell, charstring p_StepNo := "", boolean p_ActivateSCell := true, SCellIndex p_SCellIndex := tsc_NR_SCellIndex_1, boolean p_UL_CA := false ) runs on NR5GC_PTC { var integer v_DRBId; var PDUSessionInfoList_Type v_PDUSessionInfoList; var template (value) MacBearerRouting_Type v_MacBearRoutingDL := crs_MacBearerRouting(p_SCell); //@sic R5-213616 sic@ var template (value) MacBearerRouting_Type v_MacBearRoutingUL := crs_MacBearerRouting(p_PCell); //@sic R5-213616 sic@ // Step 1 : The SS transmits an Activation MAC control element to activate SCell. if (p_ActivateSCell) { f_SS_CellConfig_ActivateSCell_CA(p_PCell, p_SCellIndex); } <<SKIPPED CODE >> …… }

After Change:

function f_NR5GC_CheckDataPath_CA(NR_CellId_Type p_PCell, NR_CellId_Type p_SCell, charstring p_StepNo := "", boolean p_ActivateSCell := true, SCellIndex p_SCellIndex := tsc_NR_SCellIndex_1, boolean p_UL_CA := false ) runs on NR5GC_PTC { var integer v_DRBId; var PDUSessionInfoList_Type v_PDUSessionInfoList; var template (value) MacBearerRouting_Type v_MacBearRoutingDL := crs_MacBearerRouting(p_SCell); //@sic R5-213616 sic@ var template (value) MacBearerRouting_Type v_MacBearRoutingUL := crs_MacBearerRouting(p_PCell); //@sic R5-213616 sic@ // Step 1 : The SS transmits an Activation MAC control element to activate SCell. if (p_ActivateSCell) { f_SS_CellConfig_ActivateSCell_CA(p_PCell, p_SCellIndex); f_Delay(0.05); //adding 50ms delay to ensure scell is activated //WA#WI=1356121 } <<SKIPPED CODE >> …… }