3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240002

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3294 | rev | - | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function f_TC_7_1_1_1_17_NR_TestBody |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2023-12-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - It is not an rrcResume scenario thus UE is not in RRC_INACTIVE state. - At Step 8, 19 RRC setup is not explicitly sent from TTCN but it is preconfigured at the SS to be sent together with the contention resolution as it is not a rrcResume test steps. Thus, the default UL grant configuration is also required at Step 8, 19 as rrcRelease prior to these steps has released all configuration from SS, so we do need to configure it explicitly here. Please refer our common method which handles rrcSetupRequest and configure UL Grant explicitly in all cases after rrcRelease including preamble procedure. i.e., function f_NR_RRC_SetupRequest_Common. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Corrected the state at Step 4, 15 steps code. - Added f_NR_ULGrantConfiguration_Start() at Step 8, 19. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test cases. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.1.17.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	1

1	Overview	1

2	Corrections required	1

2.1	Function f_TC_7_1_1_1_17_NR_TestBody ()	1

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D23wk49 related to the title of this CR.

Contact:	Parikshit Bhise

parikshit.bhise@rohde-schwarz.com

# Corrections required

## Function f_TC_7_1_1_1_17_NR_TestBody ()

| Function name | f_TC_7_1_1_1_17_NR_TestBody |
| --- | --- |
| Reason for change | 1. It is not an rrcResume scenario thus UE is not in RRC_INACTIVE state. 2. At Step 8, 19 RRC setup is not explicitly sent from TTCN but it is preconfigured at the SS to be sent together with the contention resolution as it is not a rrcResume test steps. Thus, the default UL grant configuration is also required at Step 8, 19 as rrcRelease prior to these steps has released all configuration from SS, so we do need to configure it explicitly here. Please refer our common method which handles rrcSetupRequest and configure UL Grant explicitly in all cases after rrcRelease including preamble procedure. i.e., function f_NR_RRC_SetupRequest_Common. |
| Summary of change | 1. Corrected the state at Step 4, 15 steps code. 2. Added f_NR_ULGrantConfiguration_Start() at Step 8, 19. |
| TTCN module | .. Common\MAC_Common_NR5GC.ttcn |
| MCC160 Comment | - Accepted - Accepted in principle. Used cs_NR_UplinkTimeAlignment_Start for PUCCH sync. |

Before Change:

function f_TC_7_1_1_1_17_NR_TestBody(integer p_StartPreamble, integer p_NumberOfPreamble) runs on NR5GC_PTC { var integer v_ModificationPeriodFrames; var float v_ModificationPeriod; var template (omit) NR_SRB_COMMON_IND v_SrbIndDummy := omit; var template (omit) NR_SYSTEM_IND v_SysIndDummy1 := omit; var template (omit) NR_SYSTEM_IND v_SysIndDummy2 := omit; var NG_NAS_MSG_Indication_Type v_ReceivedMsg; //@siclog "Step 2, 13" siclog@ //The SS transmits a Short message on PDCCH using P-RNTI indicating a systemInfoModification. f_NR_ModifySysinfo(nr_Cell1, true); //@sic R5s230656 sic@ //@siclog "Step 3, 14" siclog@ //Wait for 2.1* modification period second for the UE to receive new system information. v_ModificationPeriodFrames := f_NR_CalculateModificationPeriod(nr_Cell1); v_ModificationPeriod := int2float(v_ModificationPeriodFrames)/100.0; //Modification period in seconds f_Delay(2.1 * v_ModificationPeriod); //Configure SS to report UL_SRB0_LCID f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_UL_SRB0_LCID(enable)); // Enable PRACH reception indication f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_RachPreamble(enable)); //@siclog "Step 4, 15 " siclog@ //The SS transmits a Paging message including a matched identity (correct fullI-RNTI). f_NR_Paging_Def(nr_Cell1, RRC_INACTIVE); //@siclog "Step 5-7, 16-18" siclog@ //Check: Does the UE transmit Preamble on PRACH with ra-PreambleIndex range from 8 to 11 // Check: Does the UE transmit a MAC PDU containing an RRCSetupRequest message by setting LCID to 35 a_NR_RacingCond_AwaitRrcMessageAndTwoSysInd(nr_Cell1, SysInd1BeforeMsgBeforeSysInd2, v_SrbIndDummy, v_SysIndDummy1, v_SysIndDummy2, car_NR_SRB0_RrcPdu_IND(nr_Cell1, cr_38508_RRCSetupRequest), car_NR_PRACH_Preamble_IND(nr_Cell1, -, (p_StartPreamble..p_StartPreamble+p_NumberOfPreamble)), cr_NR_SystemIndicationUL_SRB0_LCID_IND(nr_Cell1, cr_TimingInfo_Any, 35) ); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 5, 7, 16, 18"); //Configure SS to stop report UL_SRB0_LCID f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_UL_SRB0_LCID(disable)); // Configure SS to stop PRACH reception indication f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_RachPreamble(disable)); //@siclog "Step 8, 19" siclog@ //The SS transmits an RRCSetup message. f_NR_RRCSetup_Def (nr_Cell1); //@siclog "Step 9, 20" siclog@ //The UE transmit an RRCSetupComplete message including SERVICE REQUEST to confirm the successful completion of the connection establishment. v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell1, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def), tsc_SHT_IntegrityProtected); //@siclog "Step 10, 21" siclog@ //Steps 5 to 8 of the NR RRC_CONNECTED procedure in TS 38.508-1 Table 4.5.4.2-3 are executed to successfully complete the service request procedure. f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell1, v_ReceivedMsg); };

After Change:

function f_TC_7_1_1_1_17_NR_TestBody(integer p_StartPreamble, integer p_NumberOfPreamble) runs on NR5GC_PTC { var integer v_ModificationPeriodFrames; var float v_ModificationPeriod; var template (omit) NR_SRB_COMMON_IND v_SrbIndDummy := omit; var template (omit) NR_SYSTEM_IND v_SysIndDummy1 := omit; var template (omit) NR_SYSTEM_IND v_SysIndDummy2 := omit; var NG_NAS_MSG_Indication_Type v_ReceivedMsg; //@siclog "Step 2, 13" siclog@ //The SS transmits a Short message on PDCCH using P-RNTI indicating a systemInfoModification. f_NR_ModifySysinfo(nr_Cell1, true); //@sic R5s230656 sic@ //@siclog "Step 3, 14" siclog@ //Wait for 2.1* modification period second for the UE to receive new system information. v_ModificationPeriodFrames := f_NR_CalculateModificationPeriod(nr_Cell1); v_ModificationPeriod := int2float(v_ModificationPeriodFrames)/100.0; //Modification period in seconds f_Delay(2.1 * v_ModificationPeriod); //Configure SS to report UL_SRB0_LCID f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_UL_SRB0_LCID(enable)); // Enable PRACH reception indication f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_RachPreamble(enable)); //@siclog "Step 4, 15 " siclog@ //The SS transmits a Paging message including a matched identity. f_NR_Paging_Def(nr_Cell1, RRC_IDLE); // WA#WA=1227815 //@siclog "Step 5-7, 16-18" siclog@ //Check: Does the UE transmit Preamble on PRACH with ra-PreambleIndex range from 8 to 11 // Check: Does the UE transmit a MAC PDU containing an RRCSetupRequest message by setting LCID to 35 a_NR_RacingCond_AwaitRrcMessageAndTwoSysInd(nr_Cell1, SysInd1BeforeMsgBeforeSysInd2, v_SrbIndDummy, v_SysIndDummy1, v_SysIndDummy2, car_NR_SRB0_RrcPdu_IND(nr_Cell1, cr_38508_RRCSetupRequest), car_NR_PRACH_Preamble_IND(nr_Cell1, -, (p_StartPreamble..p_StartPreamble+p_NumberOfPreamble)), cr_NR_SystemIndicationUL_SRB0_LCID_IND(nr_Cell1, cr_TimingInfo_Any, 35) ); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 5, 7, 16, 18"); //Configure SS to stop report UL_SRB0_LCID f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_UL_SRB0_LCID(disable)); // Configure SS to stop PRACH reception indication f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_RachPreamble(disable)); //@siclog "Step 8, 19" siclog@ f_NR_ULGrantConfiguration_Start(nr_Cell1, -, -, -, cs_NR_UplinkTimeAlignment_Stop, -, -, tsc_NoCnfReq ); // WA#WA=1227815 //The SS transmits an RRCSetup message. f_NR_RRCSetup_Def (nr_Cell1); //@siclog "Step 9, 20" siclog@ //The UE transmit an RRCSetupComplete message including SERVICE REQUEST to confirm the successful completion of the connection establishment. v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell1, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def), tsc_SHT_IntegrityProtected); //@siclog "Step 10, 21" siclog@ //Steps 5 to 8 of the NR RRC_CONNECTED procedure in TS 38.508-1 Table 4.5.4.2-3 are executed to successfully complete the service request procedure. f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell1, v_ReceivedMsg); };