3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240006

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4775 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to the NB-NTN testcases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN_plus_EPS-UEConTest |  |  |  |  |  | Date: |  |  | 2023-12-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | When executing Rel-17 NTN test cases, the current function results in execution error with “Invalid sysinfo combination”. The current function does not handle conditions c8 and c9 The parameter passed for calculating the start of the 6th NPDCCH is incorrect as the 1st NPDCCH starts from 0. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated the function f_NBIOT_Init to handle c8 and c9 Updated test case 22.3.1.5a Step 17 v_NpdcchPeriod calculation to use 5 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE will fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | All NBIOT NTN testcases |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

2.1	Correction to f_NBIOT_Init	2

2.2	Correction to f_TC_22_3_1_5a_NBIOT	4

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D23wk49 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Correction to f_NBIOT_Init

| Template name | f_NBIOT_Init |
| --- | --- |
| Reason for change | When executing Rel-17 NTN test cases, the current function results in execution error with “Invalid sysinfo combination”. The current function does not handle conditions c8 and c9 |
| Summary of change | Updated the function to handle c8 and c9 |
| TTCN module | NBIOT_CellInfo.ttcn |
| MCC160 Comment | Accepted. Additionally, mapping of the sys info combination c6 to c12 will be added |

Before Change:

function f_NBIOT_Init(NB_Combination_Type p_SysinfoCombination, IOT_STATE_Type p_AllowAttachWithoutPDN := CONTROL_PLANE, EUTRA_NAS_NumOfPLMN_Type p_CellsOnSamePLMN := LTE_AllCellsOnSamePLMN, EUTRA_NAS_CellSpecialFreq p_CellSpecialFreq := LTE_NAS_NormalFreq, template (omit)NBIOT_CellId_List_Type p_Neighbour_NTN_Cells := omit) runs on NBIOT_PTC { … … … select(p_SysinfoCombination) {//@sic R5-237459, R5-237126 mapping legacy TC SysinfoCombination to NTN-specific SysinfoCombination sic@ case(c1) { p_SysinfoCombination := c8; } case(c2) { p_SysinfoCombination := c10; } case(c3) { p_SysinfoCombination := c11; } case(c4) { p_SysinfoCombination := c13; } case else { FatalError(__FILE__, __LINE__, "Invalid sysinfo combination"); //no corresponding SysInfo Combination for NTN } }

After Change:

function f_NBIOT_Init(NB_Combination_Type p_SysinfoCombination, IOT_STATE_Type p_AllowAttachWithoutPDN := CONTROL_PLANE, EUTRA_NAS_NumOfPLMN_Type p_CellsOnSamePLMN := LTE_AllCellsOnSamePLMN, EUTRA_NAS_CellSpecialFreq p_CellSpecialFreq := LTE_NAS_NormalFreq, template (omit)NBIOT_CellId_List_Type p_Neighbour_NTN_Cells := omit) runs on NBIOT_PTC { … … … select(p_SysinfoCombination) {//@sic R5-237459, R5-237126 mapping legacy TC SysinfoCombination to NTN-specific SysinfoCombination sic@ case(c1) { p_SysinfoCombination := c8; } case(c2) { p_SysinfoCombination := c10; } case(c3) { p_SysinfoCombination := c11; } case(c4) { p_SysinfoCombination := c13; } case (c8,c9) { } //when c8 / c9 is passed no update is required case else { FatalError(__FILE__, __LINE__, "Invalid sysinfo combination"); //no corresponding SysInfo Combination for NTN } }

## Correction to f_TC_22_3_1_5a_NBIOT

| function name | f_TC_22_3_1_5a_NBIOT |
| --- | --- |
| Reason for change | The parameter passed for calculating the start of the 6th NPDCCH is incorrect as the 1st NPDCCH starts from 0. |
| Summary of change | Updated Step 17 v_NpdcchPeriod calculation to use 5 |
| TTCN module | NBIOT_MAC_NTN.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function f_TC_22_3_1_5a_NBIOT() runs on NBIOT_PTC { … … … //@siclog "Step 16" siclog@ //The UE transmits a HARQ NACK for the DL MAC PDU in Step 15. SYSIND.receive(car_NB_UL_HARQ_IND(nbiot_Cell1, nack)) -> value v_SYSTEM_IND; v_TimingInfo := v_SYSTEM_IND.Common.TimingInfo; //@sic R5-237435 sic@ //@siclog "Step 17" siclog@ //Six NPDCCH periods after the step 16 the SS schedules the DL re-transmission of the MAC PDU in the last search space candidate. //Note 1: Six NPDCCH periods is the duration of the drx-RetransmissionTimer-r13. //(Note 5) v_NpdcchPeriod := f_NBIOT_IncrementSearchSpace(valueof(v_TimingInfo.SubFrame), int2float(6), v_SearchSpaceParameters); //@sic R5s230746 Ch2.1.9, R5-237435 sic@ v_TimingInfo := cs_TimingInfo_SubFrameFull(v_NpdcchPeriod); f_NBIOT_MAC_CPMode_SendMacPdu(nbiot_Cell1, v_TimingInfo, v_MAC_PDU, tsc_LastSearchSpaceCandidate); //@siclog "Step 18" siclog@

After Change:

function f_TC_22_3_1_5a_NBIOT() runs on NBIOT_PTC { … … … //@siclog "Step 16" siclog@ //The UE transmits a HARQ NACK for the DL MAC PDU in Step 15. SYSIND.receive(car_NB_UL_HARQ_IND(nbiot_Cell1, nack)) -> value v_SYSTEM_IND; v_TimingInfo := v_SYSTEM_IND.Common.TimingInfo; //@sic R5-237435 sic@ //@siclog "Step 17" siclog@ //Six NPDCCH periods after the step 16 the SS schedules the DL re-transmission of the MAC PDU in the last search space candidate. //Note 1: Six NPDCCH periods is the duration of the drx-RetransmissionTimer-r13. //(Note 5) // starting from 0, 6th NPDCCH will be 5 v_NpdcchPeriod := f_NBIOT_IncrementSearchSpace(valueof(v_TimingInfo.SubFrame), int2float(5), v_SearchSpaceParameters); //@sic R5s230746 Ch2.1.9, R5-237435 sic@ v_TimingInfo := cs_TimingInfo_SubFrameFull(v_NpdcchPeriod); f_NBIOT_MAC_CPMode_SendMacPdu(nbiot_Cell1, v_TimingInfo, v_MAC_PDU, tsc_LastSearchSpaceCandidate); //@siclog "Step 18" siclog@