3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240049

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3322 | rev | - | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcase 8.2.3.14.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-12-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the current TTCN Implementation of the function f_NRDC_RRC_PSCellChange_InterCell_SplitMCGReestablish for the test case 8.2.3.14.2 - As per 38.523-1 it is expected for UE to receive an RRCReconfiguration message to handover from NR PSCell involving SCG change with reconfigurationWithSync to the target NR cell with Split DRB .but in the current TTCN implementation , on the Target Pscell Id , the split bearer configuration is not present when configuring v_SS_RadioBearerDRB_SCG. Instead it is configured as SCG DRB. this needs to be addressed and It is proposed to configure Split DRB . - When configuring v_SS_MCG_RadioBearerList with Split SCG DRB, SDAP configuration is set to omit , but for SA this has to be set as SDAP_Configuration_None. - v_CellGroupConfigMCG is modified to have reconfigurationwithSync on MCG and random access configurations are provided accordingly in the MCG Pcell. v_CellGroupConfigMCG is assigned to have f_NR_GetCellGroupConfigHO instead of cs_38508_CellGroupConfig. This change is to be in line with other cases where TTCN is doing f_NR5GC_GetExisting_BearerToAddModList(true_); //rlc reestablish - due to change(3), it is proposed to add f_NR_SS_C_RNTI_Config() and f_NR_SS_RachProcedureConfigHO() HO on MCG_PcellId along with f_NR_SS_SRBs_DRBs_Release by using tsc_FollowOnFlag - In step 1 , target Pscell Id is configured with radio bearers ,especially as per the test purpose , we need to have Split DRB. but this configuration can lead to both source Spcell and Target Spcell containing LinkToOtherCellGroup Value = NR (1) Value = nr_Cell1 (1) at the same time. For SS , this can lead to two terminating PDCP entities referring to the same LinkToOtherCellGroup. To address this issue it is proposed to do the step “1. NR Target PSCell: Configuration of SRB3 (if necessary) and DRBs” once source Pscell radio bearers are released. This also means we need to PdcpCountHandOver from source to target Pscell & security key update on the Target Pscell once radio bearers are established. To avoid having multiple timing based procedure in the middle of reconfiguration and reconfiguration complete, it is proposed to configure bearers on p_Target_PSCellId, after the reception of reconfiguration complete |  |  |  |  |  |  |  |  |
| Summary of change: |  | - v_SS_RadioBearerDRB_SCG is configured as Split DRB to match the test purpose requirement - v_SS_MCG_RadioBearerList configuration to have Sdap set to None instead of Omit - v_CellGroupConfigMCG:= f_NR_GetCellGroupConfigHO change is made so that TTCN is in line wherever TTCN is doing f_NR5GC_GetExisting_BearerToAddModList(true_); //rlc reestablish - f_NR_SS_C_RNTI_Config() and f_NR_SS_RachProcedureConfigHO() are added on MCG_PcellId along with f_NR_SS_SRBs_DRBs_Release by using tsc_FollowOnFlag, for achieving reconfigurationWithSync procedure - Changes are done to avoid two SCG Entities having the same LinkToOtherCellGroup configured at the same time p_RNTI_Value is added as a parameter to the function f_NR_GetRACHProcedureConfigPSCell |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE may fail the case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.3.14.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 38.523 -1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | An Associated Prose CR to be raised for change (3) Change (5) requires changes in the test model 38.523-3 clause 7.3.5.5.1 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	3

2	Corrections required	3

2.1	Change to the Function f_NRDC_RRC_PSCellChange_InterCell_SplitMCGReestablish	3

2.2	Change to the Function f_NR_GetRACHProcedureConfigPSCell	12

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D23wk49 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change to the Function f_NRDC_RRC_PSCellChange_InterCell_SplitMCGReestablish

| Function name | f_NRDC_RRC_PSCellChange_InterCell_SplitMCGReestablish |
| --- | --- |
| Reason for change | In the current TTCN Implementation of the function f_NRDC_RRC_PSCellChange_InterCell_SplitMCGReestablish for the test case 8.2.3.14.2 - As per 38.523-1 it is expected for UE to receive an RRCReconfiguration message to handover from NR PSCell involving SCG change with reconfigurationWithSync to the target NR cell with Split DRB .but in the current TTCN implementation , on the Target Pscell Id , the split bearer configuration is not present when configuring v_SS_RadioBearerDRB_SCG. Instead it is configured as SCG DRB. this needs to be addressed and It is proposed to configure Split DRB . - When configuring v_SS_MCG_RadioBearerList with Split SCG DRB, SDAP configuration is set to omit , but for SA this has to be set as SDAP_Configuration_None. - v_CellGroupConfigMCG is modified to have reconfigurationwithSync on MCG and random access configurations are provided accordingly in the MCG Pcell. v_CellGroupConfigMCG is assigned to have f_NR_GetCellGroupConfigHO instead of cs_38508_CellGroupConfig. This change is to be in line with other cases where TTCN is doing f_NR5GC_GetExisting_BearerToAddModList(true_); //rlc reestablish - due to change(3), it is proposed to add f_NR_SS_C_RNTI_Config() and f_NR_SS_RachProcedureConfigHO() HO on MCG_PcellId along with f_NR_SS_SRBs_DRBs_Release by using tsc_FollowOnFlag - In step 1 , target Pscell Id is configured with radio bearers ,especially as per the test purpose , we need to have Split DRB. but this configuration can lead to both source Spcell and Target Spcell containing LinkToOtherCellGroup Value = NR (1) Value = nr_Cell1 (1) at the same time For SS , this can lead to two terminating PDCP entitieis referring to the same LinkToOtherCellGroup. To address this issue it is proposed to do the step “1. NR Target PSCell: Configuration of SRB3 (if necessary) and DRBs” once source Pscell radio bearers are released. This also means we need to PdcpCountHandOver from source to target Pscell & security key update on the Target Pscell once radio bearers are established. Also it is proposed to configure bearers on p_Target_PSCellId, after the reception of reconfiguration complete , so that we don’t have too many time based actions during the reconfiguration procedure on MCG Cell. In addition , To do the reconfigurationWithSync rach configuration on the target PScell with newUE_Identity = (16712) , tsc_C_RNTI_Value3 is to be configured . so that it matches with peer reconfiguration configuration. This is done by adding an extra parameter to the function f_NR_GetRACHProcedureConfigPSCell |
| Summary of change | - v_SS_RadioBearerDRB_SCG is configured as Split DRB to match the test purpose requirement - v_SS_MCG_RadioBearerList configuration to have Sdap set to None instead of Omit - v_CellGroupConfigMCG:= f_NR_GetCellGroupConfigHO change is made so that TTCN is in line wherever TTCN is doing f_NR5GC_GetExisting_BearerToAddModList(true_); //rlc reestablish - f_NR_SS_C_RNTI_Config() and f_NR_SS_RachProcedureConfigHO() are added on MCG_PcellId along with f_NR_SS_SRBs_DRBs_Release by using tsc_FollowOnFlag, for achieving reconfigurationWithSync procedure - Changes are done to avoid two SCG Entities having the same LinkToOtherCellGroup configured at the same time |
| TTCN module | develop\Common\NR5GC\NR5GC_RRCSteps_NRDC.ttcn |
| MCC160 Comment | Accepted in principle, implemented as per R5-240547 |

Before Change:

function f_NRDC_RRC_PSCellChange_InterCell_SplitMCGReestablish( NR_CellId_Type p_MCG_PCellId, NR_CellId_Type p_Source_PSCellId, NR_CellId_Type p_Target_PSCellId, boolean p_RBConfig_KeyChange := true, RNTI_Value p_RNTI_Value := tsc_C_RNTI_Value3, template (omit) SK_Counter p_SK_Counter := omit ) runs on NR5GC_PTC { var integer v_SCG_DRB_Id := f_NR_GetDefaultDRB_SCG(); var template (value) SDAP_Config v_SDAP_ConfigSCG; var template (value) SDAP_Configuration_Type v_SDAP_ConfigurationSCG; var template (omit) GSM_MobilityInfo_Type v_GSM_MobilityInfo := f_Get_PDUSessionForDNNType(f_NR5GC_MobileInfo_GetSessionInfoList(), Internet_DNN); var template (value) NR_RadioBearer_Type v_SS_RadioBearerDRB_SCG; var template (value) NR_RadioBearerList_Type v_SS_TargetSCG_RadioBearerList; var template (value) NR_RadioBearerList_Type v_SourceSCG_RadioBearerToReleaseList := {cs_NR_SS_DRB_Release(v_SCG_DRB_Id)}; var template (value) CellGroupConfig v_CellGroupConfigSCG; var template (value) RadioBearerConfig v_RadioBearerConfig2; var template (value) NR_RLC_BearerToAddModList_Type v_RLC_BearerToAddModListMCG:= f_NR5GC_GetExisting_BearerToAddModList(true_); var template (value) CellGroupConfig v_CellGroupConfigMCG:= cs_38508_CellGroupConfig(tsc_NR_CellGroupId_MCG, v_RLC_BearerToAddModListMCG ); var template (value) NR_RachProcedureConfig_Type v_RachProcedureConfig := f_NR_GetRACHProcedureConfigPSCell( p_Target_PSCellId, -); var NR_SecurityParams_Type v_SecurityParams := f_NR_Security_Get(); var template (value) RRCReconfiguration_v1530_IEs v_V1530Ext; var IntegerList_Type v_SS_DrbToReleaseMCG := f_NR_GetActiveDRBs(p_MCG_PCellId); var template (value) NR_DRBInfoList_Type v_MCG_DRBInfoList := f_NR5GC_MobileInfo_GetDRBInfoList(); var template (omit) NR_RadioBearerList_Type v_SS_MCG_RadioBearerList := f_NR5GC_GetRadioBearerList(v_MCG_DRBInfoList, true); var boolean v_IsFR1_SCG := f_NR_CellInfo_GetIsFR1(p_Target_PSCellId); var SubFrameTiming_Type v_Timing; //To include in RRCReconfiguration //Get SDAP - @sic R5s211604 sic@ if (isvalue(v_GSM_MobilityInfo)) { v_SDAP_ConfigSCG := cs_SDAP_Config(oct2int(valueof(v_GSM_MobilityInfo.SessionId)), false, {5}); v_SDAP_ConfigurationSCG:= f_ReturnNR_SDAP_Configuration_FromSDAP_Config(valueof(v_SDAP_ConfigSCG)); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "No internet PDU session found to modify"); } v_SS_RadioBearerDRB_SCG := f_NR_GetRadioBearerConfig_FullAM(p_Target_PSCellId, v_SCG_DRB_Id,-,-,-,v_SDAP_ConfigurationSCG); v_SS_TargetSCG_RadioBearerList := {v_SS_RadioBearerDRB_SCG}; v_CellGroupConfigSCG := f_NR_GetSCGConfigNRDC_SCG(p_Target_PSCellId, v_SCG_DRB_Id, p_RNTI_Value); v_CellGroupConfigSCG.rlc_BearerToAddModList[0].reestablishRLC := true_; //@sic R5s221057 sic@ v_RadioBearerConfig2 := cs_NRDRB_SplitBearer(omit, v_SCG_DRB_Id, cs_NR_PDCP_Config_Split(cs_38508_PDCP_Config_DRB, omit, tsc_NR_CellGroupId_SCG), secondary, f_NR_AS_CipheringAlgorithm_Get(), f_NR_AS_IntegrityAlgorithm_Get(), true_); v_V1530Ext := f_NRDC_BuildRRCReconfiguration_v1530( v_CellGroupConfigSCG, v_CellGroupConfigMCG, v_RadioBearerConfig2, -, p_SK_Counter ); // Add SCG DRB configured in proxy (split SCG DRB) v_SS_MCG_RadioBearerList[lengthof(v_SS_MCG_RadioBearerList)] := cs_NR_SS_DRB_Config(v_SCG_DRB_Id, cs_NR_PDCP_Configuration_Proxy(cs_RlcBearerRouting_NR(p_Target_PSCellId)), cs_NR_RlcBearerConfig_AM(v_IsFR1_SCG, -, f_NR_LogicalChannelId_DRB(v_SCG_DRB_Id))); //1. NR Target PSCell: Configuration of SRB3 (if necessary) and DRBs f_NR_SS_CommonRadioBearerConfig(p_Target_PSCellId, v_SS_TargetSCG_RadioBearerList); // 2. Transfer of the PDCP Count for DRBs and SRB3 (if necessary) from NR source to NR target PSCell: //a) NR Source PSCell: Get PDCP COUNT. //b) NR Target PSCell: Set PDCP COUNT. //NOTE 1: No further sending/receiving of DRB data before the PSCell change is done. //NOTE 2: For AM DRBs the PDCP count is maintained. For SRB3 (if applied) and UM DRBs, the PDCP count is maintained or reset depending on the RRCReconfiguration message content. f_NR_SS_PdcpCount_Handover(p_Source_PSCellId, p_Target_PSCellId, v_SS_TargetSCG_RadioBearerList, p_RBConfig_KeyChange); //3. NR Target PSCell: Inform the SS about the PSCell change and about the source cell id. f_NR_SS_PdcpHandoverCtrl(p_Target_PSCellId, cas_NR_PdcpHandoverInit_REQ(p_Source_PSCellId, p_Target_PSCellId)); //4. NR Target PSCell: Configure RACH procedure either dedicated or C-RNTI based. //NOTE 3: The FollowOnFlag is set to true in the ASP reconfiguring C-RNTI. f_NR_CellInfo_SetRNTI(p_Target_PSCellId, p_RNTI_Value); f_NR_SS_C_RNTI_Config(p_Target_PSCellId, p_RNTI_Value, -, tsc_NoCnfReq, tsc_FollowOnFlag); f_NR_SS_RachProcedureConfigHO(p_Target_PSCellId, v_RachProcedureConfig); //5. NR Target PSCell: Activate security. if (isvalue(p_SK_Counter)) { v_SecurityParams.SK_Counter := valueof(p_SK_Counter); v_SecurityParams := f_NR_Security_InitSCG(v_SecurityParams); f_NR_Security_Set(v_SecurityParams); } v_SecurityParams.AS_Ciphering_S.ActTimeList := f_NR_RRC_CipherActTime_SCG(p_Target_PSCellId, v_SS_TargetSCG_RadioBearerList); f_NR_SS_RRC_EnableIntProt_CiphULandDL(p_Target_PSCellId, v_SecurityParams.AS_Integrity_S, v_SecurityParams.AS_Ciphering_S); //6. NR Target PSCell: Configure UL grant configuration ("OnSR", default grant). //NOTE 3: Unless explicitly specified UL grant configuration keeps configured as per default at the NR source cell. f_NR_ULGrantConfiguration_Start(p_Target_PSCellId, -, -, -, -, cs_UL_GrantConfig_OnSR); //7. NR PCell: Send RRCReconfiguration. //8. NR PCell: Receive RRCReconfigurationComplete. v_Timing := f_NR_GetNextSendOccasion_DL(p_MCG_PCellId); f_NR_SendRRCReconfiguration (p_MCG_PCellId, tsc_NR_RbId_SRB1, -, v_V1530Ext, -, -, cs_TimingInfo_NR(v_Timing), false); //Reset RLC in MCG v_Timing := f_SubFrameTiming_AddMilliSeconds(v_Timing, 5); f_NR_SS_SRBs_DRBs_Release(p_MCG_PCellId, cs_TimingInfo_NR(v_Timing), v_SS_DrbToReleaseMCG); v_Timing := f_SubFrameTiming_AddMilliSeconds(v_Timing, 5); // add 5ms f_NR_SS_SRBs_DRBs_Config(p_MCG_PCellId, cs_TimingInfo_NR(v_Timing), v_SS_MCG_RadioBearerList); SRB.receive(car_NR_SRB_RrcPdu_IND(p_MCG_PCellId, tsc_NR_RbId_SRB1, cr_38508_RRCReconfigurationComplete(cr_RRCReconfigurationComplete_v1530(-, cr_RRCReconfigurationComplete_v1560(?))))); //9. NR Target PSCell: Inform the SS about completion of the PSCell change (e.g. to trigger PDCP STATUS REPORT PDU). f_NR_SS_PdcpHandoverCtrl(p_Target_PSCellId, cas_NR_PdcpHandoverComplete_REQ(p_Target_PSCellId)); //10. NR Source PSCell: Release SRB3 (if necessary) and DRBs. f_NR_SS_CommonRadioBearerConfig(p_Source_PSCellId, v_SourceSCG_RadioBearerToReleaseList); }

After Change:

function f_NRDC_RRC_PSCellChange_InterCell_SplitMCGReestablish( NR_CellId_Type p_MCG_PCellId, NR_CellId_Type p_Source_PSCellId, NR_CellId_Type p_Target_PSCellId, boolean p_RBConfig_KeyChange := true, RNTI_Value p_RNTI_Value := tsc_C_RNTI_Value3, template (omit) SK_Counter p_SK_Counter := omit ) runs on NR5GC_PTC { var integer v_SCG_DRB_Id := f_NR_GetDefaultDRB_SCG(); var template (value) SDAP_Config v_SDAP_ConfigSCG; var template (value) SDAP_Configuration_Type v_SDAP_ConfigurationSCG; var template (omit) GSM_MobilityInfo_Type v_GSM_MobilityInfo := f_Get_PDUSessionForDNNType(f_NR5GC_MobileInfo_GetSessionInfoList(), Internet_DNN); var template (value) NR_RadioBearer_Type v_SS_RadioBearerDRB_SCG; var template (value) NR_RadioBearerList_Type v_SS_TargetSCG_RadioBearerList; var template (value) NR_RadioBearerList_Type v_SourceSCG_RadioBearerToReleaseList := {cs_NR_SS_DRB_Release(v_SCG_DRB_Id)}; var template (value) CellGroupConfig v_CellGroupConfigSCG; var template (value) RadioBearerConfig v_RadioBearerConfig2; var template (value) NR_RLC_BearerToAddModList_Type v_RLC_BearerToAddModListMCG:= f_NR5GC_GetExisting_BearerToAddModList(true_); //var template (value) CellGroupConfig v_CellGroupConfigMCG:= cs_38508_CellGroupConfig(tsc_NR_CellGroupId_MCG, v_RLC_BearerToAddModListMCG ); //WA#WI=970303 var template (value) CellGroupConfig v_CellGroupConfigMCG:= f_NR_GetCellGroupConfigHO (nr_Cell1,v_RLC_BearerToAddModListMCG ); //WA#WI=970303 var template (value) NR_RachProcedureConfig_Type v_RachProcedureConfig := f_NR_GetRACHProcedureConfigPSCell( p_Target_PSCellId, -,p_RNTI_Value);//WA#WI=970303); var NR_SecurityParams_Type v_SecurityParams := f_NR_Security_Get(); var template (value) RRCReconfiguration_v1530_IEs v_V1530Ext; var IntegerList_Type v_SS_DrbToReleaseMCG := f_NR_GetActiveDRBs(p_MCG_PCellId); var template (value) NR_DRBInfoList_Type v_MCG_DRBInfoList := f_NR5GC_MobileInfo_GetDRBInfoList(); var template (omit) NR_RadioBearerList_Type v_SS_MCG_RadioBearerList := f_NR5GC_GetRadioBearerList(v_MCG_DRBInfoList, true); var boolean v_IsFR1_SCG := f_NR_CellInfo_GetIsFR1(p_Target_PSCellId); var SubFrameTiming_Type v_Timing; var NR_PhysicalParameters_Type v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(p_MCG_PCellId); //WA#WI=970303 var template (value) NR_RachProcedureConfig_Type v_RachProcedureConfig_MCG := cs_NR_RachProcedureConfig_CRNTI_HO(v_NR_PhysicalParameters, tsc_C_RNTI_Value3);//WA#WI=970303 var NR_PdcpCountInfoList_Type v_NR_PdcpCountInfoList; //WA#WI=970303 //To include in RRCReconfiguration //Get SDAP - @sic R5s211604 sic@ if (isvalue(v_GSM_MobilityInfo)) { v_SDAP_ConfigSCG := cs_SDAP_Config(oct2int(valueof(v_GSM_MobilityInfo.SessionId)), false, {5}); v_SDAP_ConfigurationSCG:= f_ReturnNR_SDAP_Configuration_FromSDAP_Config(valueof(v_SDAP_ConfigSCG)); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "No internet PDU session found to modify"); } //v_SS_RadioBearerDRB_SCG := f_NR_GetRadioBearerConfig_FullAM(p_Target_PSCellId, v_SCG_DRB_Id,-,-,-,v_SDAP_ConfigurationSCG); //WA#WI=970303 v_SS_RadioBearerDRB_SCG := cs_NR_SS_DRB_Config(v_SCG_DRB_Id, cs_NR_PDCP_Configuration_RBTerminating(cs_NR_PDCP_RbConfig_ParamsDRB, cs_RlcBearerRouting_NR(p_MCG_PCellId)), cs_NR_RlcBearerConfig_AM(v_IsFR1_SCG, -, f_NR_LogicalChannelId_DRB(v_SCG_DRB_Id)), v_SDAP_ConfigurationSCG); //WA#WI=970303 v_SS_TargetSCG_RadioBearerList := {v_SS_RadioBearerDRB_SCG}; v_CellGroupConfigSCG := f_NR_GetSCGConfigNRDC_SCG(p_Target_PSCellId, v_SCG_DRB_Id, p_RNTI_Value); v_CellGroupConfigSCG.rlc_BearerToAddModList[0].reestablishRLC := true_; //@sic R5s221057 sic@ v_RadioBearerConfig2 := cs_NRDRB_SplitBearer(omit, v_SCG_DRB_Id, cs_NR_PDCP_Config_Split(cs_38508_PDCP_Config_DRB, omit, tsc_NR_CellGroupId_SCG), secondary, f_NR_AS_CipheringAlgorithm_Get(), f_NR_AS_IntegrityAlgorithm_Get(), true_); v_V1530Ext := f_NRDC_BuildRRCReconfiguration_v1530( v_CellGroupConfigSCG, v_CellGroupConfigMCG, v_RadioBearerConfig2, -, p_SK_Counter ); // Add SCG DRB configured in proxy (split SCG DRB) v_SS_MCG_RadioBearerList[lengthof(v_SS_MCG_RadioBearerList)] := cs_NR_SS_DRB_Config(v_SCG_DRB_Id, cs_NR_PDCP_Configuration_Proxy(cs_RlcBearerRouting_NR(p_Target_PSCellId)), cs_NR_RlcBearerConfig_AM(v_IsFR1_SCG, -, f_NR_LogicalChannelId_DRB(v_SCG_DRB_Id)),cs_SDAP_Configuration_None); //WA#WI=970303 //1. NR Target PSCell: Configuration of SRB3 (if necessary) and DRBs //f_NR_SS_CommonRadioBearerConfig(p_Target_PSCellId, v_SS_TargetSCG_RadioBearerList); //WA#WI=970303 // 2. Transfer of the PDCP Count for DRBs and SRB3 (if necessary) from NR source to NR target PSCell: //a) NR Source PSCell: Get PDCP COUNT. //b) NR Target PSCell: Set PDCP COUNT. //NOTE 1: No further sending/receiving of DRB data before the PSCell change is done. //NOTE 2: For AM DRBs the PDCP count is maintained. For SRB3 (if applied) and UM DRBs, the PDCP count is maintained or reset depending on the RRCReconfiguration message content. //WA#WI=zzz moved to be configured after split bearer release from source cell / otherwise we will have 2 split proxy mapped to MCG //f_NR_SS_PdcpCount_Handover(p_Source_PSCellId, p_Target_PSCellId, v_SS_TargetSCG_RadioBearerList, p_RBConfig_KeyChange); v_NR_PdcpCountInfoList := f_NR_GetPdcpCountInfoListIntraNR_HO(p_Source_PSCellId, v_SS_TargetSCG_RadioBearerList, p_RBConfig_KeyChange); //WA#WI=970303 //3. NR Target PSCell: Inform the SS about the PSCell change and about the source cell id. f_NR_SS_PdcpHandoverCtrl(p_Target_PSCellId, cas_NR_PdcpHandoverInit_REQ(p_Source_PSCellId, p_Target_PSCellId)); f_NR_SS_CommonRadioBearerConfig(p_Source_PSCellId, v_SourceSCG_RadioBearerToReleaseList); //WA#WI=970303 //4. NR Target PSCell: Configure RACH procedure either dedicated or C-RNTI based. //NOTE 3: The FollowOnFlag is set to true in the ASP reconfiguring C-RNTI. f_NR_CellInfo_SetRNTI(p_Target_PSCellId, p_RNTI_Value); f_NR_SS_C_RNTI_Config(p_Target_PSCellId, p_RNTI_Value, -, tsc_NoCnfReq, tsc_FollowOnFlag); f_NR_SS_RachProcedureConfigHO(p_Target_PSCellId, v_RachProcedureConfig); //5. NR Target PSCell: Activate security. if (isvalue(p_SK_Counter)) { v_SecurityParams.SK_Counter := valueof(p_SK_Counter); v_SecurityParams := f_NR_Security_InitSCG(v_SecurityParams); f_NR_Security_Set(v_SecurityParams); } //v_SecurityParams.AS_Ciphering_S.ActTimeList := f_NR_RRC_CipherActTime_SCG(p_Target_PSCellId, v_SS_TargetSCG_RadioBearerList); /*f_NR_SS_RRC_EnableIntProt_CiphULandDL(p_Target_PSCellId, v_SecurityParams.AS_Integrity_S, v_SecurityParams.AS_Ciphering_S); */ //WA#WI=970303 //6. NR Target PSCell: Configure UL grant configuration ("OnSR", default grant). //NOTE 3: Unless explicitly specified UL grant configuration keeps configured as per default at the NR source cell. f_NR_ULGrantConfiguration_Start(p_Target_PSCellId, -, -, -, -, cs_UL_GrantConfig_OnSR); //7. NR PCell: Send RRCReconfiguration. //8. NR PCell: Receive RRCReconfigurationComplete. v_Timing := f_NR_GetNextSendOccasion_DL(p_MCG_PCellId); f_NR_SendRRCReconfiguration (p_MCG_PCellId, tsc_NR_RbId_SRB1, -, v_V1530Ext, -, -, cs_TimingInfo_NR(v_Timing), false); //Reset RLC in MCG v_Timing := f_SubFrameTiming_AddMilliSeconds(v_Timing, 5); f_NR_SS_SRBs_DRBs_Release(p_MCG_PCellId, cs_TimingInfo_NR(v_Timing), v_SS_DrbToReleaseMCG,-,tsc_FollowOnFlag); //WA#WI=970303 followonFlag added f_NR_SS_C_RNTI_Config(p_MCG_PCellId, tsc_C_RNTI_Value3, cs_TimingInfo_NR(v_Timing), tsc_NoCnfReq, tsc_FollowOnFlag); //WA#WI=970303 c-rnti added f_NR_SS_RachProcedureConfigHO(p_MCG_PCellId, v_RachProcedureConfig_MCG, cs_TimingInfo_NR(v_Timing), omit); //WA#WI=970303 v_Timing := f_SubFrameTiming_AddMilliSeconds(v_Timing, 5); // add 5ms f_NR_SS_SRBs_DRBs_Config(p_MCG_PCellId, cs_TimingInfo_NR(v_Timing), v_SS_MCG_RadioBearerList); //f_NR_SS_PdcpCount_Set(p_MCG_PCellId, v_PdcpCountInfoListMCG, cs_TimingInfo_NR(v_Timing)); //WA#WI=970303 SRB.receive(car_NR_SRB_RrcPdu_IND(p_MCG_PCellId, tsc_NR_RbId_SRB1, cr_38508_RRCReconfigurationComplete(cr_RRCReconfigurationComplete_v1530(-, cr_RRCReconfigurationComplete_v1560(?))))); f_NR_SS_CommonRadioBearerConfig(p_Target_PSCellId, v_SS_TargetSCG_RadioBearerList); //WA#WI=970303 f_NR_SS_PdcpCount_Set(p_Target_PSCellId, v_NR_PdcpCountInfoList); //WA#WI=970303 //9. NR Target PSCell: Inform the SS about completion of the PSCell change (e.g. to trigger PDCP STATUS REPORT PDU). f_NR_SS_PdcpHandoverCtrl(p_Target_PSCellId, cas_NR_PdcpHandoverComplete_REQ(p_Target_PSCellId)); //10. NR Source PSCell: Release SRB3 (if necessary) and DRBs. //f_NR_SS_CommonRadioBearerConfig(p_Source_PSCellId, v_SourceSCG_RadioBearerToReleaseList); //WA#WI=970303 v_SecurityParams.AS_Ciphering_S.ActTimeList := f_NR_RRC_CipherActTime_SCG(p_Target_PSCellId, v_SS_TargetSCG_RadioBearerList); //WA#WI=970303 f_NR_SS_RRC_EnableIntProt_CiphULandDL(p_Target_PSCellId, //WA#WI=970303 v_SecurityParams.AS_Integrity_S, v_SecurityParams.AS_Ciphering_S); }

## Change to the Function f_NR_GetRACHProcedureConfigPSCell

| Function name | f_NR_GetRACHProcedureConfigPSCell |
| --- | --- |
| Reason for change | To do the reconfigurationWithSync rach configuration on the target PScell with newUE_Identity = (16712) , tsc_C_RNTI_Value3 is to be configured . so that it matches with peer reconfiguration configuration. This is done by adding an extra parameter to the function f_NR_GetRACHProcedureConfigPSCell |
| Summary of change | p_RNTI_Value is added as a parameter to the function f_NR_GetRACHProcedureConfigPSCell |
| TTCN module | Common\NR\NR_HandoverCommonFunctions.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function f_NR_GetRACHProcedureConfigPSCell(NR_CellId_Type p_NR_CellId, template (omit) ReconfigurationWithSync.rach_ConfigDedicated p_RACH_ConfigDedicated := omit) runs on NR_BASE_PTC return template (value) NR_RachProcedureConfig_Type { var template (value) NR_RachProcedureConfig_Type v_RachProcedureConfig; var NR_PhysicalParameters_Type v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(p_NR_CellId); if (isvalue(p_RACH_ConfigDedicated.uplink)){ v_RachProcedureConfig := cs_NR_RachProcedureConfig_CFRA(v_NR_PhysicalParameters); } else { //Default CBRA procedure // As per 38.523-3 Table 7.1.2.3.3-1: Default grants @sic R5-198994 sic@ v_RachProcedureConfig := cs_NR_RachProcedureConfig_CBRA_CrntiBased(v_NR_PhysicalParameters, -, -, tsc_NR_ENDC_Msg3Grant_Def); } return v_RachProcedureConfig; }

After Change:

function f_NR_GetRACHProcedureConfigPSCell(NR_CellId_Type p_NR_CellId, template (omit) ReconfigurationWithSync.rach_ConfigDedicated p_RACH_ConfigDedicated := omit, RNTI_Value_Type p_TempC_RNTI_Value := tsc_C_RNTI_Value2) runs on NR_BASE_PTC return template (value) NR_RachProcedureConfig_Type //WA#WI=970303 { var template (value) NR_RachProcedureConfig_Type v_RachProcedureConfig; var NR_PhysicalParameters_Type v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(p_NR_CellId); if (isvalue(p_RACH_ConfigDedicated.uplink)){ v_RachProcedureConfig := cs_NR_RachProcedureConfig_CFRA(v_NR_PhysicalParameters); } else { //Default CBRA procedure // As per 38.523-3 Table 7.1.2.3.3-1: Default grants @sic R5-198994 sic@ v_RachProcedureConfig := cs_NR_RachProcedureConfig_CBRA_CrntiBased(v_NR_PhysicalParameters, -, p_TempC_RNTI_Value, tsc_NR_ENDC_Msg3Grant_Def); //WA#WI=970303 } return v_RachProcedureConfig; }