3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240050

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3323 | rev | - | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of NR5GC eCall over IMS testcase 11.5.5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NR_EIEI-UEConTest |  |  |  |  |  | Date: |  |  | 2023-12-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Several changes are required for correct functioning of this test case: - 1.1. UE will not try to Register until eCall is initiated but current implementation expects UE to be in state 1N-A after waiting 60 seconds since power on. UE should just camp in cell before initiating eCall, reason why a 60 second delay is added after Power On the UE. - 1.2. Coordination message is expected inside function f_NR5GC_EmergencyCallReleaseMT to confirm the call release from IMS component, extra coordination message after mentioned function will block the test case until timeout. It needs to be removed. - 2. eCall in limited service will use NULL integrity algorithm. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - 1.1. Removed function to set UE in idle mode before eCall. - 1.2. Removed f_IMS_IPCAN_WaitForTrigger in test case body. - 2. Allowed NULL integrity algorithm for this test case |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.5.5.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

2.1	Change 1	2

2.2	Change 2	4

- Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D23wk49 related to the title of this CR.

Contact: 	Juan Garcia Martin

Juan.GarciaMartin@rohde-schwarz.com

- Corrections required

  - Change 1

| Function name | fl_TC_11_5_5_TestBody |
| --- | --- |
| Reason for change | - 1.1. UE will not try to Register until eCall is initiated but current implementation expects UE to be in state 1N-A after waiting 60 seconds since power on. UE should just camp in cell before initiating eCall, reason why a 60 second delay is added after Power On the UE. - 1.2. Coordination message is expected inside function f_NR5GC_EmergencyCallReleaseMT to confirm the call release from IMS component, extra coordination message after mentioned function will block the test case until timeout. It needs to be removed. |
| Summary of change | - 1.1. Removed function to set UE in idle mode before eCall. - 1.2. Removed f_IMS_IPCAN_WaitForTrigger in test case body. |
| TTCN module | NR5GC\11.5\ECall_NR5GC.ttcn |
| MCC160 Comment | Accepted. |

Before Change:

<<SKIPPED CODE>> function fl_TC_11_5_5_TestBody() runs on NR5GC_PTC { var float v_Wait:= 120.0; //@siclog "Step 1" siclog@ f_NR5GC_SwitchOnAndResetIMS(); //@siclog "Step 2" siclog@ f_Delay(60.0); f_NR5GC_RRC_IdleState1N_Def(nr_Cell1); // @sic R5-227244 sic@ //@siclog "Step 3" siclog@ f_UT_InitiateECall(UT, MANUAL); //@siclog "Step 4A-4R" siclog@ f_NR5GC_EmergencyCallLimitedService(nr_Cell1); // @sic R5-227244 sic@ //@siclog "Step 5" siclog@ f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS2]); f_NR5GC_EmergencyCallReleaseMT(nr_Cell1); // @sic R5-227244 sic@ f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS2]); //@siclog "Step 7" siclog@ f_NR_RRCRelease(nr_Cell1); // @sic R5-227244 sic@ //@siclog "Step 8" siclog@ f_Delay(5.0); //@siclog "Step 9" siclog@ f_UT_InitiateECall(UT, TEST, -, LOCAL_CNF_REQUIRED); // @sic R5s230789 sic@ //@siclog "Step 10" siclog@ //Check: Does UE send an RRCConnectionRequest on NGC Cell A within 120 seconds? f_NR_CheckNoRRCSetupReq(nr_Cell1, v_Wait, -, "Step 10: RRCSetupRequest received"); // @sic R5-227244 sic@ } <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> function fl_TC_11_5_5_TestBody() runs on NR5GC_PTC { var float v_Wait:= 120.0; //@siclog "Step 1" siclog@ f_NR5GC_SwitchOnAndResetIMS(); //@siclog "Step 2" siclog@ f_Delay(60.0); <<REMOVED CODE>> //f_NR5GC_RRC_IdleState1N_Def(nr_Cell1); // @sic R5-227244 sic@ Change 1.1 <<REMOVED CODE>> //@siclog "Step 3" siclog@ f_UT_InitiateECall(UT, MANUAL); //@siclog "Step 4A-4R" siclog@ f_NR5GC_EmergencyCallLimitedService(nr_Cell1); // @sic R5-227244 sic@ //@siclog "Step 5" siclog@ f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS2]); f_NR5GC_EmergencyCallReleaseMT(nr_Cell1); // @sic R5-227244 sic@ <<REMOVED CODE>> //f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS2]); Change 1.2 <<REMOVED CODE>> //@siclog "Step 7" siclog@ f_NR_RRCRelease(nr_Cell1); // @sic R5-227244 sic@ //@siclog "Step 8" siclog@ f_Delay(5.0); //@siclog "Step 9" siclog@ f_UT_InitiateECall(UT, TEST, -, LOCAL_CNF_REQUIRED); // @sic R5s230789 sic@ //@siclog "Step 10" siclog@ //Check: Does UE send an RRCConnectionRequest on NGC Cell A within 120 seconds? f_NR_CheckNoRRCSetupReq(nr_Cell1, v_Wait, -, "Step 10: RRCSetupRequest received"); // @sic R5-227244 sic@ }<<SKIPPED CODE>>

  - Change 2

| Function name | f_GetTestcaseAttrib_NullIntegrityAlgorithmAllowed |
| --- | --- |
| Reason for change | eCall in limited service will use NULL integrity algorithm. |
| Summary of change | Allowed NULL integrity algorithm for this test case |
| TTCN module | NR5GC\Common\TestcaseProperties.ttcn |
| MCC160 Comment | Accepted. |

Before Change:

<<SKIPPED CODE>> function f_GetTestcaseAttrib_NullIntegrityAlgorithmAllowed(charstring p_Testcase) return boolean { select (p_Testcase) { case ("TC_11_4_2_NR5GC") { return true; } case ("TC_11_4_3_NR5GC") { return true; } case ("TC_11_4_4_NR5GC") { return true; } case ("TC_11_4_5_NR5GC") { return true; } case ("TC_11_4_9_NR5GC") { return true; } } return false; } <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> function f_GetTestcaseAttrib_NullIntegrityAlgorithmAllowed(charstring p_Testcase) return boolean { select (p_Testcase) { case ("TC_11_4_2_NR5GC") { return true; } case ("TC_11_4_3_NR5GC") { return true; } case ("TC_11_4_4_NR5GC") { return true; } case ("TC_11_4_5_NR5GC") { return true; } case ("TC_11_4_9_NR5GC") { return true; } case ("TC_11_5_5_NR5GC") { return true; } } return false; } <<SKIPPED CODE>>