3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240055

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3326 | rev | - | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NR5GC FR1 : Addition of Rel-17 NR Slice test case 6.1.2.24 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, NR_slice-UEConTest |  |  |  |  |  | Date: |  |  | 2023-12-19 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To add test case 6.1.2.24 to the 5G/NR5GC ATS for FR1 path. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to test case 6.1.2.24 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.2.24.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR ... CR…38.523-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	7

2.3	Change 3	10

2.4	Change 4	11

2.5	Change 5	12

3	Branches executed	18

4	Execution Log Files	18

4.1	Mediatek MT6815	18

5	References	18

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 6.1.2.24 which is part of the 5G NR5GC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Kalahasti, Narendra

Narendra.Kalahasti@anritsu.com

## Verification Test Summary

Test Case: 	6.1.2.24

ATS Version:	iwd-TTCN3-B2022-09_D23wk49

System Simulator used:	Anritsu Protocol Conformance Test System ME7834NR

UE used:	Mediatek MT6815

Verification Status:	PASS

# Corrections required

## Change 1

| Function name | function f_TC_6_1_2_24_NR5GC () |
| --- | --- |
| Reason for change | - Updated SIB16, dl-ImplicitCarrierFreq-r17 for all cells according to prose R5-237336 - According to 38.508 Table 6.3.1.2-1: Inter-frequency carrier frequency lists for NR cells cell 6 configuration will be on the second element of SIB4 of cell3, So to represent Cell6 frequency on SIB16 index should be set to value 2 instead 1 - SIB4 configuration was missing on current TTCN for all 3 Cells. |
| Summary of change | - Updated SIB16, dl-ImplicitCarrierFreq-r17 for all cells. - Updated dl-ImplicitCarrierFreq-r17 index for cell6 frequency on Table 6.1.2.24.3.3.3-3 - Added SIB4 configuration with 2 interfrequency cells with same reselection priority for cell 1, cell 3 and cell6 |
| TTCN module | Idle_CellReSelection_NR5GC |
| MCC160 Comment | - Accepted, prose CR reference is R5-237335 - Accepted, due to prose CR agreement of R5-241497 at RAN5#102 - Accepted in principle but implemented differently. See Change 4 |

Before Change

function f_TC_6_1_2_24_NR5GC() runs on NR5GC_PTC { // Slice-based cell reselection / Reseletion priorities provided by SIB16 // @sic R5-234721 sic@*/ var integer v_T0_Cell1_RS_EPRE_FR1 := tsc_NR_ServingCellSSS_EPRE_FR1; var integer v_T0_Cell1_RS_EPRE_FR2 := tsc_NR_SuitableInterFreqCellSSS_EPRE_FR2; var integer v_T0_CellOff := tsc_NR_NonSuitableOffCellSSS_EPRE; var template(value) NR_CellPowerList_Type v_CellPowerList_AtT0; var ARFCN_ValueNR v_SSB_Freq_Cell1; var ARFCN_ValueNR v_SSB_Freq_Cell3; var ARFCN_ValueNR v_SSB_Freq_Cell6; v_CellPowerList_AtT0 :={ cs_NR_CellPower(nr_Cell1, v_T0_Cell1_RS_EPRE_FR1, v_T0_Cell1_RS_EPRE_FR2), cs_NR_CellPower(nr_Cell3, v_T0_CellOff, v_T0_CellOff), cs_NR_CellPower(nr_Cell6, v_T0_CellOff, v_T0_CellOff) }; f_NR5GC_Init(NR_30); // System information combination NR-30 as defined in TS 38.508-1 clause 4.4.3.1.2 is used in NR cells v_SSB_Freq_Cell1 := f_NR_CellInfo_GetFrequencySSB(nr_Cell1); v_SSB_Freq_Cell3 := f_NR_CellInfo_GetFrequencySSB(nr_Cell3); v_SSB_Freq_Cell6 := f_NR_CellInfo_GetFrequencySSB(nr_Cell6); // Table 6.1.2.24.3.3.3-2: SIB16 for NR Cell 1 f_NR_CellInfo_SetSIB16_2Entries(nr_Cell1,v_SSB_Freq_Cell1, '00000001'B, 5, v_SSB_Freq_Cell3, '00000010'B); // Table 6.1.2.24.3.3.3-3: SIB16 for NR Cell 3 f_NR_CellInfo_SetSIB16_2Entries(nr_Cell3,v_SSB_Freq_Cell3, '00000010'B, 4, v_SSB_Freq_Cell6, '00000010'B, 6); // Table 6.1.2.24.3.3.3-4: SIB16 for NR Cell 6 f_NR_CellInfo_SetSIB16_FreqPriorityListSlicing_r17(nr_Cell6, {cs_FreqPrioritySlicing_r17_Def(v_SSB_Freq_Cell6, {cs_SliceInfo_r17_1Entry('00000010'B,6)})}); // Table 6.1.2.24.3.2-1/2: : Time instances of cell power level and parameter changes for FR1/FR2 f_NR_CellInfo_InitMaxReferencePower(nr_Cell3, -78, tsc_NR_ServingCellSSS_EPRE_FR2); f_NR_CellInfo_InitMaxReferencePower(nr_Cell6, -78, tsc_NR_ServingCellSSS_EPRE_FR2); // Create and configure cell f_NR_CellConfig_Def(nr_Cell1); f_NR_CellConfig_Def(nr_Cell3); f_NR_CellConfig_Def(nr_Cell6); // Preamble: UE is in state Switched OFF (state 0N-B) f_NR5GC_Preamble(nr_Cell1, STATE_OFF_0B); f_NR_TestBody_Set(true); // Table 6.1.2.24.3.2-1: Time instances of cell power level and parameter changes f_NR_SetCellPowerList(v_CellPowerList_AtT0); f_TC_6_1_2_24_NR5GC_TestBody(); f_NR_TestBody_Set(false); // Switch/Power off UE f_NR_Postamble(nr_Cell6, STATE_IDLE_1A); } //end of f_TC_6_1_2_24_NR5GC

After Change

function f_TC_6_1_2_24_NR5GC() runs on NR5GC_PTC { // Slice-based cell reselection / Reseletion priorities provided by SIB16 // @sic R5-234721 sic@*/ var integer v_T0_Cell1_RS_EPRE_FR1 := tsc_NR_ServingCellSSS_EPRE_FR1; var integer v_T0_Cell1_RS_EPRE_FR2 := tsc_NR_SuitableInterFreqCellSSS_EPRE_FR2; var integer v_T0_CellOff := tsc_NR_NonSuitableOffCellSSS_EPRE; var template(value) NR_CellPowerList_Type v_CellPowerList_AtT0; var ARFCN_ValueNR v_SSB_Freq_Cell1; var ARFCN_ValueNR v_SSB_Freq_Cell3; var ARFCN_ValueNR v_SSB_Freq_Cell6; v_CellPowerList_AtT0 :={ cs_NR_CellPower(nr_Cell1, v_T0_Cell1_RS_EPRE_FR1, v_T0_Cell1_RS_EPRE_FR2), cs_NR_CellPower(nr_Cell3, v_T0_CellOff, v_T0_CellOff), cs_NR_CellPower(nr_Cell6, v_T0_CellOff, v_T0_CellOff) }; f_NR5GC_Init(NR_30); // System information combination NR-30 as defined in TS 38.508-1 clause 4.4.3.1.2 is used in NR cells v_SSB_Freq_Cell1 := f_NR_CellInfo_GetFrequencySSB(nr_Cell1); v_SSB_Freq_Cell3 := f_NR_CellInfo_GetFrequencySSB(nr_Cell3); v_SSB_Freq_Cell6 := f_NR_CellInfo_GetFrequencySSB(nr_Cell6); f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreq_DL_CarrierFreq(nr_Cell1, 0, v_SSB_Freq_Cell3); f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreq_DL_CarrierFreq(nr_Cell1, 1, v_SSB_Freq_Cell6); f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreq_CellReselectionPriority(nr_Cell1, 0, 4); f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreq_CellReselectionPriority(nr_Cell1, 1, 4); f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreq_DL_CarrierFreq(nr_Cell3, 0, v_SSB_Freq_Cell1); f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreq_DL_CarrierFreq(nr_Cell3, 1, v_SSB_Freq_Cell6); f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreq_CellReselectionPriority(nr_Cell3, 0, 4); f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreq_CellReselectionPriority(nr_Cell3, 1, 4); f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreq_DL_CarrierFreq(nr_Cell6, 0, v_SSB_Freq_Cell1); f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreq_DL_CarrierFreq(nr_Cell6, 1, v_SSB_Freq_Cell3); f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreq_CellReselectionPriority(nr_Cell6, 0, 4); f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreq_CellReselectionPriority(nr_Cell6, 1, 4); // Table 6.1.2.24.3.3.3-2: SIB16 for NR Cell 1 f_NR_CellInfo_SetSIB16_2Entries(nr_Cell1,0, '00000001'B, 5, 1, '00000010'B); // Table 6.1.2.24.3.3.3-3: SIB16 for NR Cell 3 f_NR_CellInfo_SetSIB16_2Entries(nr_Cell3,0, '00000010'B, 4, 2, '00000010'B, 6); // Table 6.1.2.24.3.3.3-4: SIB16 for NR Cell 6 f_NR_CellInfo_SetSIB16_FreqPriorityListSlicing_r17(nr_Cell6, {cs_FreqPrioritySlicing_r17_Def(0, {cs_SliceInfo_r17_1Entry('00000010'B,6)})}); // Table 6.1.2.24.3.2-1/2: : Time instances of cell power level and parameter changes for FR1/FR2 f_NR_CellInfo_InitMaxReferencePower(nr_Cell3, -78, tsc_NR_ServingCellSSS_EPRE_FR2); f_NR_CellInfo_InitMaxReferencePower(nr_Cell6, -78, tsc_NR_ServingCellSSS_EPRE_FR2); // Create and configure cell f_NR_CellConfig_Def(nr_Cell1); f_NR_CellConfig_Def(nr_Cell3); f_NR_CellConfig_Def(nr_Cell6); // Preamble: UE is in state Switched OFF (state 0N-B) f_NR5GC_Preamble(nr_Cell1, STATE_OFF_0B); f_NR_TestBody_Set(true); // Table 6.1.2.24.3.2-1: Time instances of cell power level and parameter changes f_NR_SetCellPowerList(v_CellPowerList_AtT0); f_TC_6_1_2_24_NR5GC_TestBody(); f_NR_TestBody_Set(false); // Switch/Power off UE f_NR_Postamble(nr_Cell6, STATE_IDLE_1A); } //end of f_TC_6_1_2_24_NR5GC

## Change 2

| Function name | function f_TC_6_1_2_24_NR5GC_TestBody () |
| --- | --- |
| Reason for change | - At step23, as per TP 1 UE must reselect cell 3 based on priority of NSAG. But on current prose NSAG ID 2 has lower prioriy 2 than serving NSAG ID 1 which has higher priority 3 as per Table 6.1.2.24.3.3.3-1: REGISTRATION ACCEPT. Priority of NSAG ID 2 should be set to higher than NSAG ID 1 to achive test purpose 1 of this case. |
| Summary of change | - Updated NSAG ID 2 priorty to 3 and NSAG ID 1 priority to 2 at step 14 |
| TTCN module | Idle_CellReSelection_NR5GC |
| MCC160 Comment | Rejected due to prose CR agreement of R5-241497 at RAN5#102 |

Before Change

function f_TC_6_1_2_24_NR5GC_TestBody() runs on NR5GC_PTC { ---- skipped code----- //@siclog "Step 14" siclog@ // The SS transmits a REGISTRATION ACCEPT message v_ASN_PLMN := f_NR_CellInfo_GetPLMN (nr_Cell1); v_PLMN := f_NR_Asn2Nas_PlmnId(v_ASN_PLMN); v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters (nr_Cell1); v_GUTIToSend := f_NG_GutiParameters2MobileIdentity (v_PLMN, v_GutiParams); v_TAC := f_NR_CellInfo_GetTAC (nr_Cell1); v_TAIList := cs_NG_TAIListNonConsecutive(v_PLMN, {bit2oct(v_TAC)}); v_NsagInfo := cs_NSAG_2Elements(cs_NSAG_InfoElement('01'O, v_NSSAI_1, '03'O, v_TAIList.lvPart), cs_NSAG_InfoElement('02'O, v_NSSAI_2, '02'O, v_TAIList.lvPart)); v_MsgToSend := f_Get_NG_RegistrationAcceptMsg (Initial_Secure, v_GMM_MobilityInfo, v_GUTIToSend, v_TAIList, omit, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, v_NsagInfo); SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell1, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); //@siclog "Step 15-20a1" siclog@ -----skipped code----- }//end of f_TC_6_1_2_24_NR5GC_TestBody

After Change

function f_TC_6_1_2_24_NR5GC_TestBody() runs on NR5GC_PTC { ---- skipped code----- //@siclog "Step 14" siclog@ // The SS transmits a REGISTRATION ACCEPT message v_ASN_PLMN := f_NR_CellInfo_GetPLMN (nr_Cell1); v_PLMN := f_NR_Asn2Nas_PlmnId(v_ASN_PLMN); v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters (nr_Cell1); v_GUTIToSend := f_NG_GutiParameters2MobileIdentity (v_PLMN, v_GutiParams); v_TAC := f_NR_CellInfo_GetTAC (nr_Cell1); v_TAIList := cs_NG_TAIListNonConsecutive(v_PLMN, {bit2oct(v_TAC)}); v_NsagInfo := cs_NSAG_2Elements(cs_NSAG_InfoElement('01'O, v_NSSAI_1, '02'O, v_TAIList.lvPart), cs_NSAG_InfoElement('02'O, v_NSSAI_2, '03'O, v_TAIList.lvPart)); v_MsgToSend := f_Get_NG_RegistrationAcceptMsg (Initial_Secure, v_GMM_MobilityInfo, v_GUTIToSend, v_TAIList, omit, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, v_NsagInfo); SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell1, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); //@siclog "Step 15-20a1" siclog@ -----skipped code----- }//end of f_TC_6_1_2_24_NR5GC_TestBody

## Change 3

| Function name | function f_NR_CellInfo_SetSIB16_FreqPriorityListSlicing_r17() |
| --- | --- |
| Reason for change | - SI index for SIB16 is 2 because SIB2, SIB4 and SIB16 are configured on SI, but on current TTCN it was set to 1 |
| Summary of change | - Modified SI index to 2 for SIB16. |
| TTCN module | NR_CellInfo |
| MCC160 Comment | Already accepted in R5s240053 Ch. 4 |

Before Change

function f_NR_CellInfo_SetSIB16_FreqPriorityListSlicing_r17 (NR_CellId_Type p_NR_CellId, template (value) FreqPriorityListSlicing_r17 p_FreqPriorityListSlicing_r17) runs on NR_BASE_PTC { var NR_CellInfo_Type v_NR_CellInfo := f_NR_CellInfo_Get(p_NR_CellId); select (v_NR_CellInfo.Sysinfo.NR_SysinfoCombination) { case (NR_30) { v_NR_CellInfo.Sysinfo.BcchInfo.SIs[1].message_.c1.systemInformation.criticalExtensions.systemInformation.sib_TypeAndInfo[0].sib16_v1700.freqPriorityListSlicing_r17 := valueof(p_FreqPriorityListSlicing_r17); f_NR_CellInfo_Set(p_NR_CellId, v_NR_CellInfo); } case else { FatalError(__FILE__, __LINE__, "Invalid System Combination"); } } }

After Change

function f_NR_CellInfo_SetSIB16_FreqPriorityListSlicing_r17 (NR_CellId_Type p_NR_CellId, template (value) FreqPriorityListSlicing_r17 p_FreqPriorityListSlicing_r17) runs on NR_BASE_PTC { var NR_CellInfo_Type v_NR_CellInfo := f_NR_CellInfo_Get(p_NR_CellId); select (v_NR_CellInfo.Sysinfo.NR_SysinfoCombination) { case (NR_30) { v_NR_CellInfo.Sysinfo.BcchInfo.SIs[2].message_.c1.systemInformation.criticalExtensions.systemInformation.sib_TypeAndInfo[0].sib16_v1700.freqPriorityListSlicing_r17 := valueof(p_FreqPriorityListSlicing_r17); f_NR_CellInfo_Set(p_NR_CellId, v_NR_CellInfo); } case else { FatalError(__FILE__, __LINE__, "Invalid System Combination"); } } }

## Change 4

| Function name | function f_NR_InitSysinfoFrequencyList () |
| --- | --- |
| Reason for change | - Initialise frequencyList in SYSINFO for SysInfoCombination NR-30 to Add InterFreqCarrierFreqList in SIB4 for all cells |
| Summary of change | - Added NR_30 as case condition for SysInfoCombination NR-30. |
| TTCN module | NR_CellInfoInit |
| MCC160 Comment | Already accepted in R5s240053 Ch. 5 |

Before Change

function f_NR_InitSysinfoFrequencyList (NR_SysinfoCombination_Type p_NR_SysinfoCombination ) runs on NR_BASE_PTC { //@sic R5-190659 sic@ select (p_NR_SysinfoCombination) { case(NR_4, NR_5, NR_7, NR_14, NR_18){ //Add InterFreqCarrierFreqList in SIB4 for all cells fl_NR_InitSIB4_FreqInfoList(); } case else {} //case else do nothing - no SIB3/SIB4 included } }

After Change

function f_NR_InitSysinfoFrequencyList (NR_SysinfoCombination_Type p_NR_SysinfoCombination ) runs on NR_BASE_PTC { //@sic R5-190659 sic@ select (p_NR_SysinfoCombination) { case(NR_4, NR_5, NR_7, NR_14, NR_18, NR_30){ //Add InterFreqCarrierFreqList in SIB4 for all cells fl_NR_InitSIB4_FreqInfoList(); } case else {} //case else do nothing - no SIB3/SIB4 included } }

## Change 5

| Function name | fl_NR_InitialiseSiScheduling () |
| --- | --- |
| Reason for change | - On schedulingInfoList2_r17 on SI_SchedulingInfo_v1700 si_WindowPosition_r17 is set to 3 and current interger var value of i is set to 2 when TTCN evaluating rel 17 SI broadcast value. So by doing i+1 SI rel 17 breadcast setting can be done. - Corrected index of schedulingInfoList2_r17 with integer variable with j (=0) instead i (=2). Also valueof not needed to convert SI periodicity for v_SI_SchedulingInfo_v1700 - Variable v_SI_Brodacasted will be true for SI-SchedulingInfo-v1700 as well and periodicity for SI-SchedulingInfo-v1700 is already set. So TTCN may through error for it. So added extra condition if ( i < v_LengthofSchedulingInfoList) to update SI periodicity correctly |
| Summary of change | - In if condition modified for it with i+1. - Removed valueof and updated to j instead i. - Added if ( i < v_LengthofSchedulingInfoList) condition to update SI periodicity. |
| TTCN module | NR_ConfigurationSteps |
| MCC160 Comment | Already accepted in R5s240053 Ch. 6 |

Before Change

function fl_NR_InitialiseSiScheduling(NR_CellId_Type p_NR_CellId, boolean p_BroadcastAllSI := false) runs on NR_BASE_PTC return template (omit) NR_AllOtherSiSchedul_Type { var template (omit) NR_AllOtherSiSchedul_Type v_AllOtherSiSchedul := omit; var NR_CellSysInfo_Type v_NR_CellSysInfo := f_NR_CellInfo_GetSysinfo(p_NR_CellId); var SI_SchedulingInfo.si_WindowLength v_SiWindowLength; var integer v_NoOfSIs; var integer v_BroadcastIndex; var integer i; var template (omit) NR_OtherSiSchedulList_Type v_SiSchedulList := omit; //@sic R5s210361 sic@ var template (omit) NR_OtherSiSchedulList_Type v_SegmentedSiSchedulList := omit; var template (omit) PosSiSchedulInfo_Type v_PosSiSchedulInfo := omit; var SI_SchedulingInfo v_SI_SchedulingInfo; var template (omit) SI_SchedulingInfo_v1700 v_SI_SchedulingInfo_v1700; //@sic R5-233379 sic@ var integer v_NoOfSIsSegmented; var NR_RedundancyVersion_Type v_RV := 0; var NR_PhysicalParameters_Type v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(p_NR_CellId); var integer v_SegmentedSiPosistionInSIB1; var integer v_LengthofSchedulingInfoList; //@sic R5-233379 sic@ var integer j :=0; //@sic R5-233379 sic@ var boolean v_SI_Brodacasted:= true; //@sic R5-233379 sic@ var integer v_SI_SlotOffset := f_NR_GetSI_SlotOffset(p_NR_CellId); select (v_NR_CellSysInfo.NR_SysinfoCombination) { case (MibOnly, NR_1) {} //do nothing - no SI case else { //Include SI and/or Segmented SI v_SI_SchedulingInfo := f_NR_GetSIB1_SiSchedulingInfo(p_NR_CellId); v_SiWindowLength := v_SI_SchedulingInfo.si_WindowLength; v_BroadcastIndex := 0; v_SI_SchedulingInfo_v1700 := f_NR_GetSIB1_SiSchedulingInfo_v1700(p_NR_CellId); if (isvalue(v_NR_CellSysInfo.BcchInfo.SIs)) { // SI included in the NR sysinfo combination @sic R5s210361 sic@ v_NoOfSIs := lengthof(v_NR_CellSysInfo.BcchInfo.SIs); v_LengthofSchedulingInfoList := lengthof(v_SI_SchedulingInfo.schedulingInfoList); for (i := 0; i < v_NoOfSIs; i:=i+1) { if ( i < v_LengthofSchedulingInfoList) { //@sic R5-233379 sic@ if (v_SI_SchedulingInfo.schedulingInfoList[i].si_BroadcastStatus== broadcasting) { v_SI_Brodacasted := true; } else { v_SI_Brodacasted := false; } } else { //SI added in the SI-SchedulingInfo-v1700 for (j:=0; j<lengthof(v_SI_SchedulingInfo_v1700.schedulingInfoList2_r17); j:=j+1) { if (valueof(v_SI_SchedulingInfo_v1700.schedulingInfoList2_r17[j].si_WindowPosition_r17) == i ) { v_SiSchedulList[v_BroadcastIndex].Periodicity := fl_ConvertSiPeriodicity(valueof(v_SI_SchedulingInfo_v1700.schedulingInfoList2_r17[i].si_Periodicity_r17)); if (valueof(v_SI_SchedulingInfo_v1700.schedulingInfoList2_r17[j].si_BroadcastStatus_r17) == broadcasting) { v_SI_Brodacasted := true; } else { v_SI_Brodacasted := false; } } } } if (v_SI_Brodacasted or p_BroadcastAllSI) { //@sic R5-201006 sic@ v_SiSchedulList[v_BroadcastIndex].Periodicity := v_SI_SchedulingInfo.schedulingInfoList[i].si_Periodicity; //@sic R5s230496 sic@ //@sic R5-237429 sic@ v_SiSchedulList[v_BroadcastIndex].Window[0].SearchSpaceAndDci := cs_NR_SearchSpaceDlDciAssignment_CSSType0A(v_NR_PhysicalParameters, v_RV); //Only one repetition within the SI-window v_SiSchedulList[v_BroadcastIndex].Window[0].SlotOffset := v_SI_SlotOffset; v_BroadcastIndex := v_BroadcastIndex + 1; //@sic R5-201006 sic@ } } } if(isvalue(v_NR_CellSysInfo.BcchInfo.SegmentedSIs)){ //@sic R5s190425 sic@ v_NoOfSIsSegmented := lengthof(v_NR_CellSysInfo.BcchInfo.SegmentedSIs); for (i := 0; i < v_NoOfSIsSegmented; i:=i+1) { v_SegmentedSiPosistionInSIB1 := i + v_BroadcastIndex; v_SegmentedSiSchedulList[i].Periodicity := v_SI_SchedulingInfo.schedulingInfoList[v_SegmentedSiPosistionInSIB1].si_Periodicity; //@sic R5-237429 sic@ v_SegmentedSiSchedulList[i].Window[0].SearchSpaceAndDci := cs_NR_SearchSpaceDlDciAssignment_CSSType0A(v_NR_PhysicalParameters, v_RV); v_SegmentedSiSchedulList[i].Window[0].SlotOffset := v_SI_SlotOffset; } } if(isvalue(v_NR_CellSysInfo.BcchInfo.PosSIs)){ //@sic R5-222377 sic@ // p_BroadcastAllSI not handled inside the function fl_InitSS_PosSI_Scheduling as it is not needed so far: in all PosSI test case all posSIs are broadcasted v_PosSiSchedulInfo := fl_InitSS_PosSI_Scheduling(p_NR_CellId); } v_AllOtherSiSchedul := cs_NR_SiSchedulList_Def(v_SiWindowLength, v_SiSchedulList, v_SegmentedSiSchedulList, v_PosSiSchedulInfo); } } return v_AllOtherSiSchedul; }

After Change

function fl_NR_InitialiseSiScheduling(NR_CellId_Type p_NR_CellId, boolean p_BroadcastAllSI := false) runs on NR_BASE_PTC return template (omit) NR_AllOtherSiSchedul_Type { var template (omit) NR_AllOtherSiSchedul_Type v_AllOtherSiSchedul := omit; var NR_CellSysInfo_Type v_NR_CellSysInfo := f_NR_CellInfo_GetSysinfo(p_NR_CellId); var SI_SchedulingInfo.si_WindowLength v_SiWindowLength; var integer v_NoOfSIs; var integer v_BroadcastIndex; var integer i; var template (omit) NR_OtherSiSchedulList_Type v_SiSchedulList := omit; //@sic R5s210361 sic@ var template (omit) NR_OtherSiSchedulList_Type v_SegmentedSiSchedulList := omit; var template (omit) PosSiSchedulInfo_Type v_PosSiSchedulInfo := omit; var SI_SchedulingInfo v_SI_SchedulingInfo; var template (omit) SI_SchedulingInfo_v1700 v_SI_SchedulingInfo_v1700; //@sic R5-233379 sic@ var integer v_NoOfSIsSegmented; var NR_RedundancyVersion_Type v_RV := 0; var NR_PhysicalParameters_Type v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(p_NR_CellId); var integer v_SegmentedSiPosistionInSIB1; var integer v_LengthofSchedulingInfoList; //@sic R5-233379 sic@ var integer j :=0; //@sic R5-233379 sic@ var boolean v_SI_Brodacasted:= true; //@sic R5-233379 sic@ var integer v_SI_SlotOffset := f_NR_GetSI_SlotOffset(p_NR_CellId); select (v_NR_CellSysInfo.NR_SysinfoCombination) { case (MibOnly, NR_1) {} //do nothing - no SI case else { //Include SI and/or Segmented SI v_SI_SchedulingInfo := f_NR_GetSIB1_SiSchedulingInfo(p_NR_CellId); v_SiWindowLength := v_SI_SchedulingInfo.si_WindowLength; v_BroadcastIndex := 0; v_SI_SchedulingInfo_v1700 := f_NR_GetSIB1_SiSchedulingInfo_v1700(p_NR_CellId); if (isvalue(v_NR_CellSysInfo.BcchInfo.SIs)) { // SI included in the NR sysinfo combination @sic R5s210361 sic@ v_NoOfSIs := lengthof(v_NR_CellSysInfo.BcchInfo.SIs); v_LengthofSchedulingInfoList := lengthof(v_SI_SchedulingInfo.schedulingInfoList); for (i := 0; i < v_NoOfSIs; i:=i+1) { if ( i < v_LengthofSchedulingInfoList) { //@sic R5-233379 sic@ if (v_SI_SchedulingInfo.schedulingInfoList[i].si_BroadcastStatus== broadcasting) { v_SI_Brodacasted := true; } else { v_SI_Brodacasted := false; } } else { //SI added in the SI-SchedulingInfo-v1700 for (j:=0; j<lengthof(v_SI_SchedulingInfo_v1700.schedulingInfoList2_r17); j:=j+1) { if (valueof(v_SI_SchedulingInfo_v1700.schedulingInfoList2_r17[j].si_WindowPosition_r17) == i+1 ) { v_SiSchedulList[v_BroadcastIndex].Periodicity := fl_ConvertSiPeriodicity(v_SI_SchedulingInfo_v1700.schedulingInfoList2_r17[j].si_Periodicity_r17); if (valueof(v_SI_SchedulingInfo_v1700.schedulingInfoList2_r17[j].si_BroadcastStatus_r17) == broadcasting) { v_SI_Brodacasted := true; } else { v_SI_Brodacasted := false; } } } } if (v_SI_Brodacasted or p_BroadcastAllSI) { //@sic R5-201006 sic@ if ( i < v_LengthofSchedulingInfoList) { v_SiSchedulList[v_BroadcastIndex].Periodicity := v_SI_SchedulingInfo.schedulingInfoList[i].si_Periodicity; //@sic R5s230496 sic@ }//@sic R5-237429 sic@ v_SiSchedulList[v_BroadcastIndex].Window[0].SearchSpaceAndDci := cs_NR_SearchSpaceDlDciAssignment_CSSType0A(v_NR_PhysicalParameters, v_RV); //Only one repetition within the SI-window v_SiSchedulList[v_BroadcastIndex].Window[0].SlotOffset := v_SI_SlotOffset; v_BroadcastIndex := v_BroadcastIndex + 1; //@sic R5-201006 sic@ } } } if(isvalue(v_NR_CellSysInfo.BcchInfo.SegmentedSIs)){ //@sic R5s190425 sic@ v_NoOfSIsSegmented := lengthof(v_NR_CellSysInfo.BcchInfo.SegmentedSIs); for (i := 0; i < v_NoOfSIsSegmented; i:=i+1) { v_SegmentedSiPosistionInSIB1 := i + v_BroadcastIndex; v_SegmentedSiSchedulList[i].Periodicity := v_SI_SchedulingInfo.schedulingInfoList[v_SegmentedSiPosistionInSIB1].si_Periodicity; //@sic R5-237429 sic@ v_SegmentedSiSchedulList[i].Window[0].SearchSpaceAndDci := cs_NR_SearchSpaceDlDciAssignment_CSSType0A(v_NR_PhysicalParameters, v_RV); v_SegmentedSiSchedulList[i].Window[0].SlotOffset := v_SI_SlotOffset; } } if(isvalue(v_NR_CellSysInfo.BcchInfo.PosSIs)){ //@sic R5-222377 sic@ // p_BroadcastAllSI not handled inside the function fl_InitSS_PosSI_Scheduling as it is not needed so far: in all PosSI test case all posSIs are broadcasted v_PosSiSchedulInfo := fl_InitSS_PosSI_Scheduling(p_NR_CellId); } v_AllOtherSiSchedul := cs_NR_SiSchedulList_Def(v_SiWindowLength, v_SiSchedulList, v_SegmentedSiSchedulList, v_PosSiSchedulInfo); } } return v_AllOtherSiSchedul; }

# Branches executed

The test case was executed on NR band n28 using NR ciphering nea2 and integrity algorithm nia2

# Execution Log Files

## Mediatek MT6815

The Mediatek MT6815 UE passed this test case on Anritsu Protocol Conformance Test System ME7834NR. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

TC_6_1_2_24_LOG.xml

- PICS/PIXIT parameter file:

TC_6_1_2_24_PIXIT.txt

# References

| [1] | R5s240056: NR5GC FR1: Supporting information for addition of Rel-17 NR Slice test case 6.1.2.24 |
| --- | --- |