3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240065

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4777 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to the NB-NTN testcase 22.3.2.7a |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN_plus_EPS-UEConTest |  |  |  |  |  | Date: |  |  | 2024-01-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | t-ReorderingExt-r17 is set to ms3200 and this timer is started at the UE when the UE receives the NPDSCH data from Step 7. At Step 8 TTCN triggers a periodic uplink grant in the next NPDCCH bundle from Step 7. This results in a grant provided to the UE just before the t-reordering timer is expires and as a result the UE uses the next uplink grant to send the Step 9 Status PDU. The next uplink grant is too late and results in test case failure as only 1*K-offset (v_NTN_Delay) is applied in f_NBIOT_NTN_SubFrameTimingCheckDuration. In order to avoid this situation it is proposed to shift the start of uplink grant by 1 NPDCCH period (64msec), so that there will be an uplink grant provided to the UE just after t-reordering timer expires. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated v_SearchSpaceIncr[5] to 1+2*v_SearchSpaceInc |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.3.2.7a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections to test case 22.3.2.7a	2

2.1	Correction to function fl_TC_22_3_2_7_and7a_TestBody	2

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D23wk49 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections to test case 22.3.2.7a

## Correction to function fl_TC_22_3_2_7_and7a_TestBody

| Template name | function fl_TC_22_3_2_7_and7a_TestBody |
| --- | --- |
| Reason for change | t-ReorderingExt-r17 is set to ms3200 and this timer is started at the UE when the UE receives the NPDSCH data from Step 7. At Step 8 TTCN triggers a periodic uplink grant in the next NPDCCH bundle from Step 7. This results in a grant provided to the UE just before the t-reordering timer is expires and as a result the UE uses the next uplink grant to send the Step 9 Status PDU. The next uplink grant is too late and results in test case failure as only 1*K-offset (v_NTN_Delay) is applied in f_NBIOT_NTN_SubFrameTimingCheckDuration. In order to avoid this situation it is proposed to shift the start of uplink grant by 1 NPDCCH period (64msec), so that there will be an uplink grant provided to the UE just after t-reordering timer expires. |
| Summary of change | Updated v_SearchSpaceIncr[5] to 1+2*v_SearchSpaceInc |
| TTCN module | NBIOT_RLC_Common |
| MCC160 Comment | Accepted |

Before Change:

function fl_TC_22_3_2_7_and7a_TestBody(inout RLC_SS_State p_RLC_Rec, boolean p_NTN := false) runs on NBIOT_PTC { var integer v_RLC_DL_SDU_Length; var integer v_RLC_UL_SDU_Length := 36; var integer v_L; var integer v_Duration; var template (value) TimingInfo_Type v_TimingInfo; var template (value) TimingInfo_Type v_TimingInfo1; var template (value) TimingInfo_Type v_TimingInfo2; var template (value) TimingInfo_Type v_TimingInfo3; var template (value) TimingInfo_Type v_TimingInfo4; var template (value) TimingInfo_Type v_TimingInfo5; var template (value) TimingInfo_Type v_TimingInfo6; var template (value) TimingInfo_Type v_TimingInfo7; var template (present) RLC_Status_NACK_List_Type v_NACK_List; var template (present) RLC_Status_Padding_Type v_Padding_Any := ?; var template (present) RLC_Status_Padding_Type v_Padding_3bits_Any := '???'B; var template (value) NB_DciUlInfo_Type v_DciUlInfo; var RLC_DataField_Type v_RLC_Data1; var integer v_VTS_Offset := p_RLC_Rec.AM_VTS; var SubFrameTiming_Type v_TimingStart; var SubFrameTiming_Type v_SubFrameTimingTa; var SubFrameTiming_Type v_SubFrameTimingTb; var boolean v_NTN := p_NTN; var IntegerList_Type v_SearchSpaceIncr; var IntegerList_Type v_SearchSpaceIncr_NTN; var integer v_NTN_delay := 0; var integer v_SearchSpaceInc := 0; var float v_Watch_T_Reordering := f_NBIOT_SetTimerToleranceMax(l2Timer, p_RLC_Rec.t_Reordering_r14); timer t_Watch_T_Reordering; // The SS does not respond to PRACH preambles transmitted by UE for Uplink transmission, but instead allocates the UL C-RNTI grant on NPDCCH when specified in the test sequence. // Note: In steps 1 to 25 the size of each RLC UL SDU #1 - #6 is PRBS of 288 bits (36 octets). The size of the RLC SDUs used in downlink is L. // establish RLC SDUs (DL and UL) and obtain the length of the DL RLC SDU // generate SDUs for SDU#1-6 with PRBS size 36 octets v_RLC_DL_SDU_Length := f_NBIOT_Generate_RLC_SDUs (p_RLC_Rec, 6, v_RLC_UL_SDU_Length, 0); // @sic R5s230535 sic@ v_L := v_RLC_DL_SDU_Length - v_RLC_UL_SDU_Length; // RLC DL SDU length - RLC UL SDU length, Length difference is the NAS and RRC overhead // values of Search Space Increment for use in chosen TC --- for GSO only if (v_NTN) { v_NTN_delay := f_NBIOT_NTN_Get_T_NTNoffset(nbiot_Cell1); v_SearchSpaceInc := ((v_NTN_delay / 64) + 2); // NPDCCH cycle is 64 ms } // values of Search Space Increments in TC 22_3_2_7 in the order of use v_SearchSpaceIncr := {1, // step 1 2, // step 1 3, // step 1 5, // step 2 1, // step 7 2, // step 8 6, // step 11 9, // step 12 10}; // step 14 // values of Search Space Increments in TC 22_3_2_7a in the order of use (steps 7 and 8 differing !) // as per @sic R5s230535 sic@ resp @sic R5-235470 sic@ v_SearchSpaceIncr_NTN := {1, // step 1 1 + v_SearchSpaceInc, // step 1 1 + 2*v_SearchSpaceInc, // step 1 1 + 4*v_SearchSpaceInc, // step 2 aliged to the prose not the TTCN CR ! v_SearchSpaceInc,// step 7 2*v_SearchSpaceInc,// step 8 @sic R5s230710 sic@ 1, // step 11 1 + v_SearchSpaceInc, // step 12 1 + 2*v_SearchSpaceInc}; // step 14 if (v_NTN) {v_SearchSpaceIncr := v_SearchSpaceIncr_NTN;}; //@siclog "Step 1" siclog@

After Change:

function fl_TC_22_3_2_7_and7a_TestBody(inout RLC_SS_State p_RLC_Rec, boolean p_NTN := false) runs on NBIOT_PTC { var integer v_RLC_DL_SDU_Length; var integer v_RLC_UL_SDU_Length := 36; var integer v_L; var integer v_Duration; var template (value) TimingInfo_Type v_TimingInfo; var template (value) TimingInfo_Type v_TimingInfo1; var template (value) TimingInfo_Type v_TimingInfo2; var template (value) TimingInfo_Type v_TimingInfo3; var template (value) TimingInfo_Type v_TimingInfo4; var template (value) TimingInfo_Type v_TimingInfo5; var template (value) TimingInfo_Type v_TimingInfo6; var template (value) TimingInfo_Type v_TimingInfo7; var template (present) RLC_Status_NACK_List_Type v_NACK_List; var template (present) RLC_Status_Padding_Type v_Padding_Any := ?; var template (present) RLC_Status_Padding_Type v_Padding_3bits_Any := '???'B; var template (value) NB_DciUlInfo_Type v_DciUlInfo; var RLC_DataField_Type v_RLC_Data1; var integer v_VTS_Offset := p_RLC_Rec.AM_VTS; var SubFrameTiming_Type v_TimingStart; var SubFrameTiming_Type v_SubFrameTimingTa; var SubFrameTiming_Type v_SubFrameTimingTb; var boolean v_NTN := p_NTN; var IntegerList_Type v_SearchSpaceIncr; var IntegerList_Type v_SearchSpaceIncr_NTN; var integer v_NTN_delay := 0; var integer v_SearchSpaceInc := 0; var float v_Watch_T_Reordering := f_NBIOT_SetTimerToleranceMax(l2Timer, p_RLC_Rec.t_Reordering_r14); timer t_Watch_T_Reordering; // The SS does not respond to PRACH preambles transmitted by UE for Uplink transmission, but instead allocates the UL C-RNTI grant on NPDCCH when specified in the test sequence. // Note: In steps 1 to 25 the size of each RLC UL SDU #1 - #6 is PRBS of 288 bits (36 octets). The size of the RLC SDUs used in downlink is L. // establish RLC SDUs (DL and UL) and obtain the length of the DL RLC SDU // generate SDUs for SDU#1-6 with PRBS size 36 octets v_RLC_DL_SDU_Length := f_NBIOT_Generate_RLC_SDUs (p_RLC_Rec, 6, v_RLC_UL_SDU_Length, 0); // @sic R5s230535 sic@ v_L := v_RLC_DL_SDU_Length - v_RLC_UL_SDU_Length; // RLC DL SDU length - RLC UL SDU length, Length difference is the NAS and RRC overhead // values of Search Space Increment for use in chosen TC --- for GSO only if (v_NTN) { v_NTN_delay := f_NBIOT_NTN_Get_T_NTNoffset(nbiot_Cell1); v_SearchSpaceInc := ((v_NTN_delay / 64) + 2); // NPDCCH cycle is 64 ms } // values of Search Space Increments in TC 22_3_2_7 in the order of use v_SearchSpaceIncr := {1, // step 1 2, // step 1 3, // step 1 5, // step 2 1, // step 7 2, // step 8 6, // step 11 9, // step 12 10}; // step 14 // values of Search Space Increments in TC 22_3_2_7a in the order of use (steps 7 and 8 differing !) // as per @sic R5s230535 sic@ resp @sic R5-235470 sic@ v_SearchSpaceIncr_NTN := {1, // step 1 1 + v_SearchSpaceInc, // step 1 1 + 2*v_SearchSpaceInc, // step 1 1 + 4*v_SearchSpaceInc, // step 2 aliged to the prose not the TTCN CR ! v_SearchSpaceInc,// step 7 1+2*v_SearchSpaceInc,// step 8 @sic R5s230710 sic@ // updated by 1 NPDCCH period 1, // step 11 1 + v_SearchSpaceInc, // step 12 1 + 2*v_SearchSpaceInc}; // step 14 if (v_NTN) {v_SearchSpaceIncr := v_SearchSpaceIncr_NTN;}; //@siclog "Step 1" siclog@

## Additional Change from MCC160

The preamble functions have been updated in the latest TTCN in such a way that they can be used instead of some existing TTCN. This is shown hereafter:

After Change:

function f_TC_22_3_2_7a_NBIOT() runs on NBIOT_PTC { /* NB-IoT / NTN / AM RLC / Receiver status triggers / extended t-Reordering configured */ var RLC_SS_State v_RLC_Rec; var NB_RLC_CountsInfoList_Type v_RLC_CountsInfoList; var template (value) DL_CCCH_Message_NB v_RrcConnSetup; var T_PollRetransmit_NB_r13 v_T_PollRetransmit_NB_r13; var RLC_Config_NB_v1430.t_Reordering_r14 v_T_Reordering_r14; var RLC_Config_NB_v1700.t_ReorderingExt_r17.setup v_T_Reordering_r17; var UL_AM_RLC_NB_r13.maxRetxThreshold_r13 v_MaxRetxThreshold_r13; var template (value) SRB_ToAddModList_NB_r13 v_SRB_ToAddModList_NB_r13; var integer v_HARQ_ProcessNumber := 0; //RLC global initialisation f_InitRLC_Record(v_RLC_Rec, AM_Mode); //Initialise acc. Table 22.3.2.7a.3.3-1 v_T_PollRetransmit_NB_r13 := ms4000; v_RLC_Rec.t_PollRetransmit_r13 := 4.0; // t_PollRetransmit_r13 set to 4000 ms v_MaxRetxThreshold_r13 := t2; v_RLC_Rec.maxRetxThreshold_r13 := 2; // maxRetxThreshold_r13 set to 2 (=t2) v_RLC_Rec.enableStatusReportSN_Gap_r13 := true; // enableStatusReportSN-Gap-r13 set to true v_T_Reordering_r14 := ms1600_v1310; v_RLC_Rec.t_Reordering_r14 := 1.6; // t_Reordering_r14 set to 1600 ms v_T_Reordering_r17 := ms3200; v_RLC_Rec.t_Reordering_r17 :=3.2; //t_Reordering_r17 set to 3200 ms //UE RLC settings are as initialized v_SRB_ToAddModList_NB_r13 := {cs_SRB_ToAddMod_NB_RLC_Config (cs_UL_AM_RLC_NB_r13 (v_T_PollRetransmit_NB_r13, v_MaxRetxThreshold_r13), cs_DL_AM_RLC_NB_r13_GapReport, cs_RLC_Config_NB_v1430(v_T_Reordering_r14))}; v_SRB_ToAddModList_NB_r13[0].rlc_Config_v1700.t_ReorderingExt_r17.setup := v_T_Reordering_r17; //acc to Table 22.3.2.7a.3.3-1 // RRCConnectionSetup-NB with twoHARQ-ProcessesConfig-r14 configured v_RrcConnSetup := f_NBIOT_508_RRCConnectionSetup(-, v_SRB_ToAddModList_NB_r13, -, -, true_); // perform NBIOT specific preamble for RLC if(pc_NB_ntn_Connectivity_EPC) { f_PreconfigureUE_ForNTN(UT); } f_NBIOT_RLC_Preamble_State2B_NB(v_RLC_Rec, nbiot_Cell1, STATE2A_NB_TESTLOOP_ModeG, v_RrcConnSetup, v_HARQ_ProcessNumber); f_NBIOT_TestBody_Set(true, layer2); f_NBIOT_HarqErrorHandling_Set(raiseInconc); fl_TC_22_3_2_7a_TestBody (v_RLC_Rec); f_NBIOT_HarqErrorHandling_Set(ignore); f_NBIOT_TestBody_Set(false); f_NBIOT_L2_Postamble_State2B_NB(nbiot_Cell1, v_RLC_Rec.AM_VTS, v_RLC_Rec.AM_VRR); }