3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240079

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3335 | rev | - | Current version: | 17.9.0 |  |
| For HELPon using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcase 8.1.1.4.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Starpoint |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-01-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In NRcell1, radiolinkMonitoringconfig is configured to monitor ssb_indx 1. Then UE goto NRcell 2 radiolinkMonitoringconfig is set to omit, UE still monitor ssb_index 1. But in SIB1 of NRcell 2,only ssb_idx 0 exist, this will lead to radio link failure in test procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Set radiolinkMonitoringconfig for NRcell2 at step17. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.1.4.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

2.1	fl_TC_8_1_1_4_1_TestBody	2

- Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D23wk49 related to the title of this CR.

Contact:	Ting Gao

gaoting@starpointcomm.com

- Corrections required

  - fl_TC_8_1_1_4_1_TestBody

| Functionname | fl_TC_8_1_1_4_1_TestBody |
| --- | --- |
| Reason for change | In NRcell1, radiolinkMonitoringconfig is configured to monitor ssb_indx 1. Then UE goto NRcell 2 radiolinkMonitoringconfig is set to omit, UE still monitor ssb_index 1. But in SIB1 of NRcell 2,only ssb_idx 0 exist, this will lead to radio link failure in test procedure. |
| Summary of change | Set radiolinkMonitoringconfig for NRcell2 at step17. |
| TTCN module | RRC_Resume_NR5GC.ttcn |
| MCC160 Comment | Accepted conditional to prose CR agreement at RAN5#102 |

Before Change:

function fl_TC_8_1_1_4_1_TestBody() runs on NR5GC_PTC { //@sic R5-197053, R5-198912, R5s191051 sic@ var PDUSessionInfoList_Type v_PDUSessionInfo; // @sic R5-227538 sic@ var integer v_DRBId; var integer v_ModificationPeriodFrames; var float v_ModificationPeriod; var template (value) CellGroupConfig v_CellGroupConfig; var NR_DownlinkBWP_Type v_NR_DownlinkBWP_Cell2; // @sic R5-204436 sic@ var PDSCH_Config v_PDSCH_Config; // @sic R5-204436 sic@ var template (omit) NR_BWP_DownlinkDedicated_PDSCH_Config_Type v_BWP_DownlinkDedicated_PDSCH_Config; // @sic R5-204436 sic@ var integer v_PrachConfigurationIndex; // @sic R5-213465 sic@ var NR_PhysicalParameters_Type v_PhysicalParameters; // @sic R5-213465 sic@ var template (omit) NR_SRB_COMMON_IND v_SrbIndByRef := omit; // not used (needed when other information of the RRC message needs to be access) // @sic R5-213465 sic@ var template (omit) NR_SYSTEM_IND v_SysIndByRef := omit; // to store SYSTEM_IND if it takes over the RRC message // @sic R5-213465 sic@ var template (present) TimingInfo_Type v_TimingInfo; // @sic R5-213465 sic@ timer t_WaitTime := 60.0; … //@siclog "Step 17" siclog@ //The SS transmits an RRCResume message //@sic R5-201646, R5-204436 sic@ v_NR_DownlinkBWP_Cell2 := f_NR_CellInfo_GetDownlinkBWP(nr_Cell2, tsc_NR_BWP_Id); v_PDSCH_Config := v_NR_DownlinkBWP_Cell2.Pdsch.ConfigDedicated.R15; v_PDSCH_Config.dmrs_DownlinkForPDSCH_MappingTypeA := omit; v_BWP_DownlinkDedicated_PDSCH_Config := {setup := v_PDSCH_Config}; v_CellGroupConfig := f_NR_GetSCGConfigResume(nr_Cell2); v_CellGroupConfig.spCellConfig.spCellConfigDedicated.initialDownlinkBWP := cs_BWP_DownlinkDedicated_Def(-, v_BWP_DownlinkDedicated_PDSCH_Config); f_NR_SendRRCResume_Def(nr_Cell2, -, -, v_CellGroupConfig); //@siclog "Step 18" siclog@ //The UE transmit an RRCResumeComplete message to complete the resume procedure f_NR_RRCResumeComplete_Def(nr_Cell2); … }

After Change:

function fl_TC_8_1_1_4_1_TestBody() runs on NR5GC_PTC { //@sic R5-197053, R5-198912, R5s191051 sic@ var PDUSessionInfoList_Type v_PDUSessionInfo; // @sic R5-227538 sic@ var integer v_DRBId; var integer v_ModificationPeriodFrames; var float v_ModificationPeriod; var template (value) CellGroupConfig v_CellGroupConfig; var NR_DownlinkBWP_Type v_NR_DownlinkBWP_Cell2; // @sic R5-204436 sic@ var PDSCH_Config v_PDSCH_Config; // @sic R5-204436 sic@ var template (omit) NR_BWP_DownlinkDedicated_PDSCH_Config_Type v_BWP_DownlinkDedicated_PDSCH_Config; // @sic R5-204436 sic@ var integer v_PrachConfigurationIndex; // @sic R5-213465 sic@ var NR_PhysicalParameters_Type v_PhysicalParameters; // @sic R5-213465 sic@ var template (omit) NR_SRB_COMMON_IND v_SrbIndByRef := omit; // not used (needed when other information of the RRC message needs to be access) // @sic R5-213465 sic@ var template (omit) NR_SYSTEM_IND v_SysIndByRef := omit; // to store SYSTEM_IND if it takes over the RRC message // @sic R5-213465 sic@ var template (present) TimingInfo_Type v_TimingInfo; // @sic R5-213465 sic@ timer t_WaitTime := 60.0; var template (omit) NR_BWP_DownlinkDedicated_RadioLinkMonitoringConfig_Type v_RadioLinkMonitoringConfig; … //@siclog "Step 17" siclog@ //The SS transmits an RRCResume message //@sic R5-201646, R5-204436 sic@ v_NR_DownlinkBWP_Cell2 := f_NR_CellInfo_GetDownlinkBWP(nr_Cell2, tsc_NR_BWP_Id); v_PDSCH_Config := v_NR_DownlinkBWP_Cell2.Pdsch.ConfigDedicated.R15; v_PDSCH_Config.dmrs_DownlinkForPDSCH_MappingTypeA := omit; v_BWP_DownlinkDedicated_PDSCH_Config := {setup := v_PDSCH_Config}; v_RadioLinkMonitoringConfig := cs_38508_RadioLinkMonitoringConfigDef(f_NR_CellInfo_GetSSB_Index(nr_Cell2)); f_NR_CellInfo_SetRadioLinkMonitoringConfig(nr_Cell2, v_RadioLinkMonitoringConfig); v_CellGroupConfig := f_NR_GetSCGConfigResume(nr_Cell2); v_CellGroupConfig.spCellConfig.spCellConfigDedicated.initialDownlinkBWP := cs_BWP_DownlinkDedicated_Def(-, v_BWP_DownlinkDedicated_PDSCH_Config,-,v_RadioLinkMonitoringConfig); f_NR_SendRRCResume_Def(nr_Cell2, -, -, v_CellGroupConfig); //@siclog "Step 18" siclog@ //The UE transmit an RRCResumeComplete message to complete the resume procedure f_NR_RRCResumeComplete_Def(nr_Cell2); … }