3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240080

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3336 | rev | - | Current version: | 17.9.0 |  |
| For HELPon using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcase 9.1.10.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Starpoint |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, eNS-UEConTest |  |  |  |  |  | Date: |  |  | 2024-01-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In preamble, PDU has already been activeded with CID1,still use CID1 in step16 and step52,UE will not transmit a PDU SESSION ESTABLISHMENT REQUEST message. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Instead of using CID1,using CID2 in step16 and step52. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.10.3.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

2.1	fl_TC_9_1_10_3_TestBody	2

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D23wk49 related to the title of this CR.

Contact:	Ting Gao

gaoting@starpointcomm.com

# Corrections required

## fl_TC_9_1_10_3_TestBody

| Functionname | fl_TC_9_1_10_3_TestBody |
| --- | --- |
| Reason for change | In preamble, PDU has already been activeded with CID1,still use CID1 in step16 and step52,UE will not transmit a PDU SESSION ESTABLISHMENT REQUEST message. |
| Summary of change | Instead of using CID1,using CID2 in step16 and step52. |
| TTCN module | NetworkSlicingEnh_NR5GC.ttcn |
| MCC160 Comment | Rejected. At Step 16 it is negative check, so the UE should never use the cid. Also at Step 18 UE is power-cycled. Again at Step38a1 the USIM is removed, or in Step 38b1 UE is power-cycled. Thus the UE should clear up everything by Step 52. |

Before Change:

function fl_TC_9_1_10_3_TestBody() runs on NR5GC_PTC { … //@siclog "Step 16" siclog@ //Cause the UE to request establishment of PDU session with S-NSSAI=1 f_UT_RequestPDUSession(UT, 1, -, -, -, v_NSSAI, -, -, -, -, -, -, LOCAL_CNF_REQUIRED); // @sic R5s230789 sic@ //@siclog "Step 17" siclog@ //Check: Does the UE transmit a RRCSetupRequest message within 5 seconds? f_NR_CheckNoRRCSetupReq(v_NGC_NASCellA, 5.0); //@sic R5-232339 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 17"); … //@siclog "Step 52" siclog@ f_UT_RequestPDUSession(UT, 1, -, -, -, v_NSSAI); //@siclog "Step 53" siclog@ SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, v_SRB, //@sic R5s221174 sic@ cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; //@sic R5-222008 sic@ if (not f_Check_NG_PDUSessionEstablishmentReq(v_GSM_MobilityInfo, v_ReceivedAsp.Signalling.Nas[0].Pdu, ?,?,-,-, -,-,-,-, -,-,-,-, cs_S_NSSAI_WithIEI('22'O, cs_S_NSSAI('01'O, '01'O, omit)) //@sic R5s220948, R5s221179 Baseline Moving sic@ )) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Establishment Message Failed"); } v_GSMMsg := v_ReceivedAsp.Signalling.Nas[0].Pdu.PiggybackedPduList[0].Msg; v_GSM_MobilityInfo_s.SessionId := v_GSMMsg.pdu_Session_Establishment_Request.pduSessionId.sessionId; v_PTI := v_ReceivedAsp.Signalling.Nas[0].Pdu.PiggybackedPduList[0].Msg.pdu_Session_Establishment_Request.procedureTransactionIdentifier; //@sic R5s220535 sic@ //@siclog "Step 54" siclog@ v_MToSend := f_Get_NG_PDUSessionEstablishmentReject(v_GSM_MobilityInfo_s.SessionId , v_PTI, cs_GMM_GSM_Cause (omit, '00011010'B)); SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, v_SRB, //@sic R5s220948 sic@ -, cs_NG_NAS_RequestWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered, v_MToSend))); //@sic R5-222008 sic@ //@siclog "Step 55" siclog@ f_NR_RRCRelease(v_NGC_NASCellA); //@sic R5-222008 sic@ } //End of fl_TC_9_1_10_3_Body

After Change:

function fl_TC_9_1_10_3_TestBody() runs on NR5GC_PTC { … //@siclog "Step 16" siclog@ //Cause the UE to request establishment of PDU session with S-NSSAI=1 f_UT_RequestPDUSession(UT, 2, -, -, -, v_NSSAI, -, -, -, -, -, -, LOCAL_CNF_REQUIRED); // @sic R5s230789 sic@ //@siclog "Step 17" siclog@ //Check: Does the UE transmit a RRCSetupRequest message within 5 seconds? f_NR_CheckNoRRCSetupReq(v_NGC_NASCellA, 5.0); //@sic R5-232339 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 17"); … //@siclog "Step 52" siclog@ f_UT_RequestPDUSession(UT, 2, -, -, -, v_NSSAI); //@siclog "Step 53" siclog@ SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, v_SRB, //@sic R5s221174 sic@ cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; //@sic R5-222008 sic@ if (not f_Check_NG_PDUSessionEstablishmentReq(v_GSM_MobilityInfo, v_ReceivedAsp.Signalling.Nas[0].Pdu, ?,?,-,-, -,-,-,-, -,-,-,-, cs_S_NSSAI_WithIEI('22'O, cs_S_NSSAI('01'O, '01'O, omit)) //@sic R5s220948, R5s221179 Baseline Moving sic@ )) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Establishment Message Failed"); } v_GSMMsg := v_ReceivedAsp.Signalling.Nas[0].Pdu.PiggybackedPduList[0].Msg; v_GSM_MobilityInfo_s.SessionId := v_GSMMsg.pdu_Session_Establishment_Request.pduSessionId.sessionId; v_PTI := v_ReceivedAsp.Signalling.Nas[0].Pdu.PiggybackedPduList[0].Msg.pdu_Session_Establishment_Request.procedureTransactionIdentifier; //@sic R5s220535 sic@ //@siclog "Step 54" siclog@ v_MToSend := f_Get_NG_PDUSessionEstablishmentReject(v_GSM_MobilityInfo_s.SessionId , v_PTI, cs_GMM_GSM_Cause (omit, '00011010'B)); SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, v_SRB, //@sic R5s220948 sic@ -, cs_NG_NAS_RequestWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered, v_MToSend))); //@sic R5-222008 sic@ //@siclog "Step 55" siclog@ f_NR_RRCRelease(v_NGC_NASCellA); //@sic R5-222008 sic@ } //End of fl_TC_9_1_10_3_Body