3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240100

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3342 | rev | - | Current version: | 17.9.0 |  |
| For HELPon using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcase 7.1.3.4.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Starpoint |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NR_Mob_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2024-01-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In current ttcn implementation,step1 use function f_NR5GC_DAPS_HO_RRCReconfiguration_SSConfig_PDCP to set PDCP for NeighbourCell.But in this function, f_NR_SS_PdcpCount_Set in step4 has an effective time,but step6 f_NR_SS_AS_ActivateSecurity_NoRefresh get current parameters. So the PdcpCount for NeighbourCell still not correct. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Set PdcpCount after step3 for NeighbourCell in test body. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.3.4.3.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

2.1	fl_TC_7_1_3_4_3and4_NR5GC_TestBody	2

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D23wk49 related to the title of this CR.

Contact:	Ting Gao

gaoting@starpointcomm.com

# Corrections required

## fl_TC_7_1_3_4_3and4_NR5GC_TestBody

| Functionname | fl_TC_7_1_3_4_3and4_NR5GC_TestBody |
| --- | --- |
| Reason for change | In current ttcn implementation,step1 use function f_NR5GC_DAPS_HO_RRCReconfiguration_SSConfig_PDCP to set PDCP for NeighbourCell.But in this function, f_NR_SS_PdcpCount_Set in step4 has an effective time,but step6 f_NR_SS_AS_ActivateSecurity_NoRefresh get current parameters. So the PdcpCount for NeighbourCell still not correct. |
| Summary of change | Set PdcpCount after step3 for NeighbourCell in test body. |
| TTCN module | PDCP_NR5GC.ttcn |
| MCC160 Comment | Accepted in principle. MCC160 Implementation see below. |

Before Change:

function fl_TC_7_1_3_4_3and4_NR5GC_TestBody(NR_PDCP_SS_State_Type p_NR_PDCP_State, NR_CellId_Type p_NR_NeighbourCellId) runs on NR5GC_PTC { var octetstring v_IPData := f_IPv4IPv6_IcmpEchoReply(f_LoopbackModeB_IP_Address_UE()); var boolean v_IsFR1 := f_NR_CellInfo_GetIsFR1(nr_Cell1); var SubFrameTiming_Type v_Timing; var SubFrameTiming_Type v_Timing2; var template (present) NR_PDCP_PDU_Type v_ExpectedPdu; var NR_CellId_Type v_NR_NeighbourCellId := p_NR_NeighbourCellId; var template (value) RACH_ConfigGeneric v_RACH_ConfigGeneric := f_NR_CellInfo_GetRACH_ConfigGeneric(nr_Cell1, tsc_NR_BWP_Id); //@sic R5s230148, R5s230150 sic@ //@siclog "Step 1" siclog@ // The SS transmits an RRCReconfiguration message containing reconfigurationWithSync to order UE to perform DAPS handover to NR Cell 2/3. // DRB#n of the first PDU session is configured as DAPS bearer. As per TS 38.523-3, 7.3.5.3.5, Steps 1-8. v_Timing := f_NR_GetNextSendOccasion_DL(nr_Cell1, 700); // 700 ms as used for the HO_CA procedure f_NR5GC_DAPS_HO_RRCReconfiguration_SSConfig_PDCP(nr_Cell1, v_NR_NeighbourCellId, v_Timing); //TS 38.523-3, 7.3.5.3.5, Steps 1-8 // EXCEPTION: In parallel with step 2-3, parallel behaviour defined in table 7.1.3.4.3.3.2-2 is executed repeatedly. //@siclog "Step 2" siclog@ // The SS sends one IP Packet to the UE via DRB#n of the first PDU session in NR Cell 1 and v_Timing2 := f_SubFrameTiming_AddMilliSeconds (v_Timing, 15); f_NR_PDCP_Send(p_NR_PDCP_State, nr_Cell1, v_IPData, v_Timing2); //@siclog "Step 2A" siclog@ // The SS stops sending RLC acknowledgements. f_NR_SS_CommonRadioBearerConfig(nr_Cell1, {cs_NR_SS_ReConfig_NotACK_NextRLC_PDU_DRB(cs_NR_RadioBearerId_DRB(p_NR_PDCP_State.DRB_Id),v_IsFR1) }, cs_TimingInfo_Now); //@siclog "Step 3" siclog@ // Check: Does the UE loop back the IP packet received at step 2 in NR Cell 1? f_NR_PDCP_Recv(p_NR_PDCP_State, nr_Cell1, v_IPData, cr_RlcBearerRouting_NR(nr_Cell1)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 3"); //@siclog "Step 4" siclog@ // The SS transmits Random Access Response to respond to the latest preamble in NR Cell 2/3. f_NR5GC_DAPS_HO_RACHProcedure(v_NR_NeighbourCellId, tsc_C_RNTI_Value3, cs_NR_RACH_ConfigDedicated_Uplink(cs_38508_RACH_ConfigDedicated(v_RACH_ConfigGeneric, -, f_NR_CellInfo_GetSSB_Index(v_NR_NeighbourCellId)))); //@sic R5s230148, R5s230150 sic@ // Specify expected PDU which is to be received in step 6. Same SN as in step 3. v_ExpectedPdu := f_NR_PDCP_PDU_Get(p_NR_PDCP_State.PDCP_SN_Size, 0, v_IPData ); // SN to be received is 0 // EXCEPTION: Steps 5-7 can occur in any order. interleave { //@siclog "Step 5" siclog@ // The UE transmits an RRCReconfigurationComplete message in NR Cell 2/3. // Target Cell: Receive RRCReconfigurationComplete [] SRB.receive(car_NR_SRB1_RrcPdu_IND(v_NR_NeighbourCellId, cr_38508_RRCReconfigurationComplete)) {} //@siclog "Step 6" siclog@ // Check: Does the UE retransmit the IP packet received at step 2 in NR Cell 2/3? // Step 10. Target Cell: Receive loopbacked PDCP data packet. [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(v_NR_NeighbourCellId, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(v_NR_NeighbourCellId), -, {v_ExpectedPdu})) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 6"); } //@siclog "Step 7" siclog@ // Check: Does the UE send PDCP status report in NR Cell 2/3? // Target Cell: Receive PDCP STATUS REPORT PDU. [] DRB.receive (car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(v_NR_NeighbourCellId, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(v_NR_NeighbourCellId), -, {cr_NR_PDCP_STATUS_REPORT(1)})) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 7"); } } // Target Cell: Re-configure RACH procedure as for initial access f_NR_SS_RachProcedureConfig_Def(v_NR_NeighbourCellId); //@siclog "Step 8" siclog@ // The SS transmits an RRCReconfiguration message with condition DAPS_HO_ReleaseSource in NR Cell 2/3. // Target Cell: Send RRCReconfiguration (daps_SourceRelesase) to release the Source Cell SRB.send(cas_NR_SRB1_RrcPdu_REQ(v_NR_NeighbourCellId, cs_TimingInfo_Now, cs_38508_RRCReconfiguration_DAPS_HO_ReleaseSource)); // EXCEPTION: Steps 9-10 can occur in any order. interleave { //@siclog "Step 9" siclog@ // The UE transmits an RRCReconfigurationComplete message in NR Cell 2/3. // Target Cell: Receive RRCReconfigurationComplete) [] SRB.receive(car_NR_SRB1_RrcPdu_IND(v_NR_NeighbourCellId, cr_38508_RRCReconfigurationComplete)) {} //@siclog "Step 10" siclog@ // Check: Does the UE send PDCP status report in NR Cell 2/3? // Target Cell: Receive PDCP STATUS REPORT PDU. [] DRB.receive (car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(v_NR_NeighbourCellId, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(v_NR_NeighbourCellId), -, {cr_NR_PDCP_STATUS_REPORT(1)})) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 10"); } } // Source Cell: Stop periodic TA // Source Cell: Reset SRBs and release DRBs. f_NR5GC_DAPS_HO_SourceRelease_SSConfig(nr_Cell1); }

After Change:

import from NR_SecuritySteps all; function fl_TC_7_1_3_4_3and4_NR5GC_TestBody(NR_PDCP_SS_State_Type p_NR_PDCP_State, NR_CellId_Type p_NR_NeighbourCellId) runs on NR5GC_PTC { var octetstring v_IPData := f_IPv4IPv6_IcmpEchoReply(f_LoopbackModeB_IP_Address_UE()); var boolean v_IsFR1 := f_NR_CellInfo_GetIsFR1(nr_Cell1); var SubFrameTiming_Type v_Timing; var SubFrameTiming_Type v_Timing2; var template (present) NR_PDCP_PDU_Type v_ExpectedPdu; var NR_CellId_Type v_NR_NeighbourCellId := p_NR_NeighbourCellId; var template (value) RACH_ConfigGeneric v_RACH_ConfigGeneric := f_NR_CellInfo_GetRACH_ConfigGeneric(nr_Cell1, tsc_NR_BWP_Id); //@sic R5s230148, R5s230150 sic@ var NR_PdcpCountInfoList_Type v_NR_PdcpCountInfoList //@siclog "Step 1" siclog@ // The SS transmits an RRCReconfiguration message containing reconfigurationWithSync to order UE to perform DAPS handover to NR Cell 2/3. // DRB#n of the first PDU session is configured as DAPS bearer. As per TS 38.523-3, 7.3.5.3.5, Steps 1-8. v_Timing := f_NR_GetNextSendOccasion_DL(nr_Cell1, 700); // 700 ms as used for the HO_CA procedure f_NR5GC_DAPS_HO_RRCReconfiguration_SSConfig_PDCP(nr_Cell1, v_NR_NeighbourCellId, v_Timing); //TS 38.523-3, 7.3.5.3.5, Steps 1-8 // EXCEPTION: In parallel with step 2-3, parallel behaviour defined in table 7.1.3.4.3.3.2-2 is executed repeatedly. //@siclog "Step 2" siclog@ // The SS sends one IP Packet to the UE via DRB#n of the first PDU session in NR Cell 1 and v_Timing2 := f_SubFrameTiming_AddMilliSeconds (v_Timing, 15); f_NR_PDCP_Send(p_NR_PDCP_State, nr_Cell1, v_IPData, v_Timing2); //@siclog "Step 2A" siclog@ // The SS stops sending RLC acknowledgements. f_NR_SS_CommonRadioBearerConfig(nr_Cell1, {cs_NR_SS_ReConfig_NotACK_NextRLC_PDU_DRB(cs_NR_RadioBearerId_DRB(p_NR_PDCP_State.DRB_Id),v_IsFR1) }, cs_TimingInfo_Now); //@siclog "Step 3" siclog@ // Check: Does the UE loop back the IP packet received at step 2 in NR Cell 1? f_NR_PDCP_Recv(p_NR_PDCP_State, nr_Cell1, v_IPData, cr_RlcBearerRouting_NR(nr_Cell1)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 3"); v_NR_PdcpCountInfoList:=f_NR_SS_PdcpCount_Get(nr_Cell1); f_NR_SS_PdcpCount_Set(v_NR_NeighbourCellId, v_NR_PdcpCountInfoList); f_NR_SS_AS_ActivateSecurity_NoRefresh(v_NR_NeighbourCellId, cs_TimingInfo_Now); //@siclog "Step 4" siclog@ // The SS transmits Random Access Response to respond to the latest preamble in NR Cell 2/3. f_NR5GC_DAPS_HO_RACHProcedure(v_NR_NeighbourCellId, tsc_C_RNTI_Value3, cs_NR_RACH_ConfigDedicated_Uplink(cs_38508_RACH_ConfigDedicated(v_RACH_ConfigGeneric, -, f_NR_CellInfo_GetSSB_Index(v_NR_NeighbourCellId)))); //@sic R5s230148, R5s230150 sic@ // Specify expected PDU which is to be received in step 6. Same SN as in step 3. v_ExpectedPdu := f_NR_PDCP_PDU_Get(p_NR_PDCP_State.PDCP_SN_Size, 0, v_IPData ); // SN to be received is 0 // EXCEPTION: Steps 5-7 can occur in any order. interleave { //@siclog "Step 5" siclog@ // The UE transmits an RRCReconfigurationComplete message in NR Cell 2/3. // Target Cell: Receive RRCReconfigurationComplete [] SRB.receive(car_NR_SRB1_RrcPdu_IND(v_NR_NeighbourCellId, cr_38508_RRCReconfigurationComplete)) {} //@siclog "Step 6" siclog@ // Check: Does the UE retransmit the IP packet received at step 2 in NR Cell 2/3? // Step 10. Target Cell: Receive loopbacked PDCP data packet. [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(v_NR_NeighbourCellId, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(v_NR_NeighbourCellId), -, {v_ExpectedPdu})) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 6"); } //@siclog "Step 7" siclog@ // Check: Does the UE send PDCP status report in NR Cell 2/3? // Target Cell: Receive PDCP STATUS REPORT PDU. [] DRB.receive (car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(v_NR_NeighbourCellId, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(v_NR_NeighbourCellId), -, {cr_NR_PDCP_STATUS_REPORT(1)})) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 7"); } } // Target Cell: Re-configure RACH procedure as for initial access f_NR_SS_RachProcedureConfig_Def(v_NR_NeighbourCellId); //@siclog "Step 8" siclog@ // The SS transmits an RRCReconfiguration message with condition DAPS_HO_ReleaseSource in NR Cell 2/3. // Target Cell: Send RRCReconfiguration (daps_SourceRelesase) to release the Source Cell SRB.send(cas_NR_SRB1_RrcPdu_REQ(v_NR_NeighbourCellId, cs_TimingInfo_Now, cs_38508_RRCReconfiguration_DAPS_HO_ReleaseSource)); // EXCEPTION: Steps 9-10 can occur in any order. interleave { //@siclog "Step 9" siclog@ // The UE transmits an RRCReconfigurationComplete message in NR Cell 2/3. // Target Cell: Receive RRCReconfigurationComplete) [] SRB.receive(car_NR_SRB1_RrcPdu_IND(v_NR_NeighbourCellId, cr_38508_RRCReconfigurationComplete)) {} //@siclog "Step 10" siclog@ // Check: Does the UE send PDCP status report in NR Cell 2/3? // Target Cell: Receive PDCP STATUS REPORT PDU. [] DRB.receive (car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(v_NR_NeighbourCellId, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(v_NR_NeighbourCellId), -, {cr_NR_PDCP_STATUS_REPORT(1)})) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 10"); } } // Source Cell: Stop periodic TA // Source Cell: Reset SRBs and release DRBs. f_NR5GC_DAPS_HO_SourceRelease_SSConfig(nr_Cell1); }

MCC160 Implementation
follows what has been implemented in TC 8.1.3.4.1
- retrieves PDCP counts after steps 1-8 of the DAPS handover procedure

- sets these counts after step 10 of the DAPS handover procedure

1. f_NR5GC_DAPS_HO_RRCReconfiguration_SSConfig_PDCP

function f_NR5GC_DAPS_HO_RRCReconfiguration_SSConfig_PDCP(NR_CellId_Type p_SourceCellId, NR_CellId_Type p_TargetCellId, SubFrameTiming_Type p_Timing) runs on NR5GC_PTC return NR_PdcpCountInfoList_Type //@sic R5s240100 sic@ { var template (value) RACH_ConfigGeneric v_RACH_ConfigGeneric; var template (value) NR_DRBInfoList_Type v_DRBInfoList := f_NR5GC_MobileInfo_GetDRBInfoList(); var template (value) NR_RadioBearerList_Type v_RadioBearerList := f_NR5GC_GetRadioBearerList(v_DRBInfoList); var template (value) DL_DCCH_Message v_RRCReconfiguration; var SubFrameTiming_Type v_Timing1; var SubFrameTiming_Type v_Timing2; var template (value) TimingInfo_Type v_TimingInfo; //@sic R5s230023 sic@ var NR_PdcpCountInfoList_Type v_NR_PdcpCountInfoList; //@sic R5-230116 sic@ var NR_PdcpCountInfoList_Type v_NR_PdcpCountInfoList_SRBs; //@sic R5-230116 sic@ var NR_PdcpCountInfoList_Type v_NR_PdcpCountInfoList_DRB; //@sic R5s240100 sic@ var boolean v_RBConfig_KeyChange := false; // without key change @sic R5s230334 sic@ v_Timing1 := f_SubFrameTiming_AddMilliSeconds (p_Timing, 5); // time for sending RRCReconfiguration v_Timing2 := f_SubFrameTiming_AddMilliSeconds (p_Timing, 10); // time for sending RRCReconfiguration //Acc. Table 7.1.3.4.3.3.3-1 v_RACH_ConfigGeneric := f_NR_CellInfo_GetRACH_ConfigGeneric(p_SourceCellId, tsc_NR_BWP_Id); //@sic R5s230148 sic@ v_RRCReconfiguration := f_NR5GC_RRCReconfiguration_IntraNR_HO_DAPS(p_TargetCellId, -, -, // no master key update cs_NR_RACH_ConfigDedicated_Uplink(cs_38508_RACH_ConfigDedicated(v_RACH_ConfigGeneric, -, f_NR_CellInfo_GetSSB_Index(p_TargetCellId))), -, v_RBConfig_KeyChange, tsc_NR_DRB2); // without key change @sic R5s230148 sic@ // Step 0. Source Cell: Get PDCP COUNT for the DAPS DRB and SRBs v_NR_PdcpCountInfoList := f_NR_GetPdcpCountInfoListIntraNR_HO(p_SourceCellId, v_RadioBearerList, v_RBConfig_KeyChange); // without key change @sic R5s230334 sic@ //Calculate T in the TargetCell v_TimingInfo := f_NR_TimingOtherCell(p_SourceCellId, cs_TimingInfo_NR(p_Timing), p_TargetCellId); //@sic R5s230023 sic@ // Step 1. Target Cell: Configuration of DRBs f_NR_SS_CommonRadioBearerConfig(p_TargetCellId, v_RadioBearerList, v_TimingInfo); //@sic R5s230148 sic@ // Step 2. Target Cell: Configuration of Target Cell for no RACH response transmission f_NR_SS_RachProcedureConfig(p_TargetCellId, cs_NR_RachProcedureConfig_NoResponse, v_TimingInfo); // Step 3. Source Cell: Send RRCReconfiguration SRB.send(cas_NR_SRB1_RrcPdu_REQ(p_SourceCellId, cs_TimingInfo_NR(v_Timing1), v_RRCReconfiguration)); //Calculate T + 10 in the TargetCell v_TimingInfo := f_NR_TimingOtherCell(p_SourceCellId, cs_TimingInfo_NR(v_Timing2), p_TargetCellId); //@sic R5s230023 sic@ // Step 4. set PDCP count v_NR_PdcpCountInfoList_SRBs := substr(v_NR_PdcpCountInfoList,0,2); f_NR_SS_PdcpCount_Set(p_TargetCellId, v_NR_PdcpCountInfoList_SRBs, v_TimingInfo); //Set count for SRB1 and SRB2 @sic R5s230023 sic@ // Step 5. not needed for PDCP // Step 6. Activate security (no refresh) f_NR_SS_AS_ActivateSecurity_NoRefresh(p_TargetCellId, v_TimingInfo); // Step 7. Target Cell: Configure UL grant configuration ("OnSR", periodic TA is not started f_NR_ULGrantConfiguration_Common(p_TargetCellId, v_TimingInfo, -, -, cs_UL_GrantConfig_OnSR); // Step 8. Target Cell: Start periodic TA f_NR_ULGrantConfiguration_Common(p_TargetCellId, v_TimingInfo, -, cs_NR_UplinkTimeAlignment_Start, -); //return the DRB count to be used later in the procedure v_NR_PdcpCountInfoList_DRB := substr(v_NR_PdcpCountInfoList,2,1); return v_NR_PdcpCountInfoList_DRB; //@sic R5s240100 sic@ }

2. fl_TC_7_1_3_4_3and4_NR5GC_TestBody

function fl_TC_7_1_3_4_3and4_NR5GC_TestBody(NR_PDCP_SS_State_Type p_NR_PDCP_State, NR_CellId_Type p_NR_NeighbourCellId) runs on NR5GC_PTC { var octetstring v_IPData := f_IPv4IPv6_IcmpEchoReply(f_LoopbackModeB_IP_Address_UE()); var boolean v_IsFR1 := f_NR_CellInfo_GetIsFR1(nr_Cell1); var SubFrameTiming_Type v_Timing; var SubFrameTiming_Type v_Timing2; var template (present) NR_PDCP_PDU_Type v_ExpectedPdu; var NR_CellId_Type v_NR_NeighbourCellId := p_NR_NeighbourCellId; var template (value) RACH_ConfigGeneric v_RACH_ConfigGeneric := f_NR_CellInfo_GetRACH_ConfigGeneric(nr_Cell1, tsc_NR_BWP_Id); //@sic R5s230148, R5s230150 sic@ var NR_PdcpCountInfoList_Type v_NR_PdcpCountInfoList_DRB; //@sic R5s240100 sic@ //@siclog "Step 1" siclog@ // The SS transmits an RRCReconfiguration message containing reconfigurationWithSync to order UE to perform DAPS handover to NR Cell 2/3. // DRB#n of the first PDU session is configured as DAPS bearer. As per TS 38.523-3, 7.3.5.3.5, Steps 1-8. v_Timing := f_NR_GetNextSendOccasion_DL(nr_Cell1, 700); // 700 ms as used for the HO_CA procedure v_NR_PdcpCountInfoList_DRB := f_NR5GC_DAPS_HO_RRCReconfiguration_SSConfig_PDCP(nr_Cell1, v_NR_NeighbourCellId, v_Timing); //TS 38.523-3, 7.3.5.3.5, Steps 1-8 @sic R5s240100 sic@ // EXCEPTION: In parallel with step 2-3, parallel behaviour defined in table 7.1.3.4.3.3.2-2 is executed repeatedly. //@siclog "Step 2" siclog@ // The SS sends one IP Packet to the UE via DRB#n of the first PDU session in NR Cell 1 and v_Timing2 := f_SubFrameTiming_AddMilliSeconds (v_Timing, 15); f_NR_PDCP_Send(p_NR_PDCP_State, nr_Cell1, v_IPData, v_Timing2); //@siclog "Step 2A" siclog@ // The SS stops sending RLC acknowledgements. f_NR_SS_CommonRadioBearerConfig(nr_Cell1, {cs_NR_SS_ReConfig_NotACK_NextRLC_PDU_DRB(cs_NR_RadioBearerId_DRB(p_NR_PDCP_State.DRB_Id),v_IsFR1) }, cs_TimingInfo_Now); //@siclog "Step 3" siclog@ // Check: Does the UE loop back the IP packet received at step 2 in NR Cell 1? f_NR_PDCP_Recv(p_NR_PDCP_State, nr_Cell1, v_IPData, cr_RlcBearerRouting_NR(nr_Cell1)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 3"); //After data exchange, transfer count to the target cell v_NR_PdcpCountInfoList_DRB[0].DL.Value := int2bit((bit2int(v_NR_PdcpCountInfoList_DRB[0].DL.Value) + 1), 32); v_NR_PdcpCountInfoList_DRB[0].UL.Value := int2bit((bit2int(v_NR_PdcpCountInfoList_DRB[0].UL.Value) + 1), 32); f_NR_SS_PdcpCount_Set(v_NR_NeighbourCellId, v_NR_PdcpCountInfoList_DRB, cs_TimingInfo_Now); //Set count for DAPS DRB; TS 38.523-3, 7.3.5.3.5, Step 10A @sic R5s240100 sic@ //@siclog "Step 4" siclog@ // The SS transmits Random Access Response to respond to the latest preamble in NR Cell 2/3. f_NR5GC_DAPS_HO_RACHProcedure(v_NR_NeighbourCellId, tsc_C_RNTI_Value3, cs_NR_RACH_ConfigDedicated_Uplink(cs_38508_RACH_ConfigDedicated(v_RACH_ConfigGeneric, -, f_NR_CellInfo_GetSSB_Index(v_NR_NeighbourCellId)))); //@sic R5s230148, R5s230150 sic@ <<SKIPPED>> }