3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240109

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3345 | rev | - | Current version: | 17.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR5GC testcase 8.1.1.1a.3 in FR1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_UE_pow_sav_enh_plus_CT-UEConTest |  |  |  |  |  | Date: |  |  | 2024-01-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To add NR5GC test case 8.1.1.1a.3 to the IWD_23wk49 ATS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to test case 8.1.1.1a.3 required for approval. See detailed change description for further information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.1.1a.3.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required to test case 8.1.1.1a.3	4

2.1	f_TC_8_1_1_1a_3_NR5GC	4

2.2	f_GetTestcaseAttrib_EarlyContentionResolution	5

3	Branches executed	7

4	Execution Log Files	7

4.1	Hisilicon Balong 5000	7

5	References	7

# Overview

This document lists all the essencial changes needed to correct problems in the TTCN implementation of NR5GC test case 8.1.1.1a.3 which is part of the NR5GC test suite in iWD_TTCN3-B2022-09_D23wk49.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log file(s) is (are) provided as evidence.

Contact:	Parikshit Bhise

parikshit.bhise@rohde-schwarz.com

## Verification Test Summary

Test Case: 	8.1.1.1a.3.NR5GC

ATS Version:	iwd-TTCN3-B2022-09_D23wk49

System Simulator used:	R&S® 5G Protocol Conformance Test platform

UE used:	Hisilicon Balong 5000

Verification Status:	PASS

# Corrections required to test case 8.1.1.1a.3

## f_TC_8_1_1_1a_3_NR5GC

| Function name | f_TC_11_7_2_NR5GC_TestBody |
| --- | --- |
| Reason for change | In the current TTCN implementation the function f_NR_CellInfo_SetSIB1_PEI_Config_r17(nr_Cell1); is called to initialize the SIB1, but this does not set the IE subgroupsNumForUEID-r17 as OMIT. This is not inline with 38.523-1 Table 8.1.1.1a.3.3.3-3. This needs to be corrected |
| Summary of change | Instead of calling f_NR_CellInfo_SetSIB1_PEI_Config_r17(nr_Cell1);, called f_NR_CellInfo_SetSIB1_PEI_Config_r17(nr_Cell1, cs_SubgroupConfig_r17(-, omit)); |
| TTCN module | RRC_PEI_NR5GC.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function f_TC_8_1_1_1a_3_NR5GC() runs on NR5GC_PTC { //Paging Early Indication without Subgrouping / RRC_IDLE f_NR5GC_Init(NR_1); //SIB1 will not include subgroupsNumForUEID-r17 as indicated in Table 8.1.1.1a.3.3.3-3 f_NR_CellInfo_SetSIB1_PEI_Config_r17(nr_Cell1); //Create and configure cell f_NR_CellConfig_Def(nr_Cell1); f_NR5GC_Preamble(nr_Cell1, STATE_IDLE_1A); f_NR_TestBody_Set(true); fl_TC_8_1_1_1a_3_TestBody(); f_NR_TestBody_Set(false); f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); }

After Change:

function f_TC_8_1_1_1a_3_NR5GC() runs on NR5GC_PTC { //Paging Early Indication without Subgrouping / RRC_IDLE f_NR5GC_Init(NR_1); //SIB1 will not include subgroupsNumForUEID-r17 as indicated in Table 8.1.1.1a.3.3.3-3 f_NR_CellInfo_SetSIB1_PEI_Config_r17(nr_Cell1, cs_SubgroupConfig_r17(-, omit)); //WA#8_1_1_1a_3 //Create and configure cell f_NR_CellConfig_Def(nr_Cell1); f_NR5GC_Preamble(nr_Cell1, STATE_IDLE_1A); f_NR_TestBody_Set(true); fl_TC_8_1_1_1a_3_TestBody(); f_NR_TestBody_Set(false); f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); }

## f_GetTestcaseAttrib_EarlyContentionResolution

| Function name | f_GetTestcaseAttrib_EarlyContentionResolution |
| --- | --- |
| Reason for change | In the function fl_TC_8_1_1_1a_3_TestBody at Step 2 upon reception of the rrcSetupREQ, a preconfigured rrcSetup will be transmitted. This is incorrect, and needs to be addressed. Hence it is proposed that the testcase 8.1.1.1a.3 be incuded in this function, after which the preconfigured rrcSetup will not be transmitted. |
| Summary of change | Included testcase 8.1.1.1a.3 in function f_GetTestcaseAttrib_EarlyContentionResolution |
| TTCN module | TestcaseProperties.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function f_GetTestcaseAttrib_EarlyContentionResolution(charstring p_Testcase) return boolean { select (p_Testcase) { case ("TC_11_3_2_NR5GC") { return true; } case ("TC_11_3_6_NR5GC") { return true; } case ("TC_11_3_6a_NR5GC") { return true; } case ("TC_11_7_1_NR5GC") { return true; } //WA#11_7_1 case ("TC_11_7_2_NR5GC") { return true; } //WA#11_7_2 case ("TC_11_7_3_NR5GC") { return true; } //WA#11_7_3 case ("TC_6_4_1_1_NR5GC") { return true; } case ("TC_6_4_2_2_NR5GC") { return true; } case ("TC_7_1_1_1_3_NR5GC") { return true; } case ("TC_7_1_1_1_5_NR5GC") { return true; } case ("TC_7_1_1_8_4_NR5GC") { return true; } case ("TC_8_1_1_1_1_NR5GC") { return true; } case ("TC_8_1_1_2_1_NR5GC") { return true; } case ("TC_8_1_1_2_3_NR5GC") { return true; } case ("TC_8_1_1_4_1_NR5GC") { return true; } <<SKIPPED CODE>>

After Change:

function f_GetTestcaseAttrib_EarlyContentionResolution(charstring p_Testcase) return boolean { select (p_Testcase) { case ("TC_11_3_2_NR5GC") { return true; } case ("TC_11_3_6_NR5GC") { return true; } case ("TC_11_3_6a_NR5GC") { return true; } case ("TC_11_7_1_NR5GC") { return true; } //WA#11_7_1 case ("TC_11_7_2_NR5GC") { return true; } //WA#11_7_2 case ("TC_11_7_3_NR5GC") { return true; } //WA#11_7_3 case ("TC_6_4_1_1_NR5GC") { return true; } case ("TC_6_4_2_2_NR5GC") { return true; } case ("TC_7_1_1_1_3_NR5GC") { return true; } case ("TC_7_1_1_1_5_NR5GC") { return true; } case ("TC_7_1_1_8_4_NR5GC") { return true; } case ("TC_8_1_1_1_1_NR5GC") { return true; } case ("TC_8_1_1_1a_3_NR5GC") { return true; } //WA#8_1_1_1a_3 case ("TC_8_1_1_2_1_NR5GC") { return true; } case ("TC_8_1_1_2_3_NR5GC") { return true; } case ("TC_8_1_1_4_1_NR5GC") { return true; } <<SKIPPED CODE>>

# Branches executed

## This NR5GC TC was executed in 5GS Stand Alone mode (Option 2) on NR Primary Band “n41” using NIA1 Integrity and NEA1 ciphering algorithms.

# Execution Log Files

## Hisilicon Balong 5000

The Hisilicon Balong 5000 UE passed this test case on R&S® 5G Protocol Conformance Test platform. The documentation below is enclosed as evidence of the successful test case run [1]:

Test case execution log file:

tc_ 8_1_1_1a_3_Hisilicon.log
(Note: PICS/PIXIT settings are captured at the beginning of the TLI log)

In the log file the complete test case execution as well as the PICS/PIXIT parameter settings can be seen. All TLI events are presented and message contents are fully decoded and can be verified. Preliminary verdicts and the final test case verdict can be seen in the log file.

# References

| [1] | R5s240110: Supporting information for addition of NR5GC testcase 8.1.1.1a.3 in FR1 |
| --- | --- |