3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240122

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3354 | rev | - | Current version: | 17.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC test cases 11.3.6 and 11.3.6a |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-01-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Preamble has Test_LoopModeB_On configured, that would set the delay time to 0s, and there is no Open Loop called before the next Close Loop configuration with delay of 2s at steps 1AJ-1AK and 6E-6F In this case some UEs may ignore the second instance, because there is no Open UE Test Loop Mode B command between those two instances. This causes testcase failure because if a device ignores the second instance, then it will set the IP PDU Delay to 0s, whereas the testcase requirement is 2s in test body. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added additional steps to call f_NR5GC_OpenUE_TestLoopMode just before f_NR5GC_CloseUE_TestLoopModeB so that UE should not ignore the configured IP PDU delay and use latest configured delay. Please note that a draft prose CR is associated with this change. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant may fail these test cases |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.3.6.NR5GC, 11.3.6a.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.523-1 CR 4156 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required.	3

2.1	Change 1	3

- Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2022-09_D23wk49 related to the title of this CR.

Contact:	Mohit Kanchan
	mohit.kanchan@keysight.com

- Corrections required.

  - Change 1

| Function name | fl_TC_11_3_6and6a_TestBody |
| --- | --- |
| Reason for change | Preamble has Test_LoopModeB_On configured, that would set the delay time to 0s, and there is no Open Loop called before the next Close Loop configuration with delay of 2s at steps 1AJ-1AK and 6E-6F In this case some UEs may ignore the second instance, because there is no Open UE Test Loop Mode B command between those two instances. This causes testcase failure because if a device ignores the second instance, then it will set the IP PDU Delay to 0s, whereas the testcase requirement is 2s in test body. |
| Summary of change | Added additional steps to call f_NR5GC_OpenUE_TestLoopMode just before f_NR5GC_CloseUE_TestLoopModeB so that UE should not ignore the configured IP PDU delay and use latest configured delay. Please note that a draft prose CR is associated with this change. |
| TTCN module | UAC_NR5GC.ttcn |
| MCC160 Comment | Accepted in principle, but implemented differently as shown below. No prose CR is needed and it’s related CR R5-240022 (final Tdoc: R5-241527) has been Withdrawn at RAN5#102. |

Before Change:

function fl_TC_11_3_6and6a_TestBody() runs on NR5GC_PTC { var Octet_Type v_IP_PDU_delay := '02'O; //@sic R5s210669, R5-216194 sic@ var NG_NAS_GutiParameters_Type v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters(nr_Cell1); var template (present) NG_MobileIdentity v_S_TMSI := f_NG_S_TMSI2MobileIdentity(omit, v_GutiParams); var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var NR_SRB_COMMON_IND v_ReceivedAsp; var octetstring v_IPData := f_IPv4IPv6_IcmpEchoReply(f_LoopbackModeB_IP_Address_UE()); var integer v_DRBId; var template (value) QosFlow_Identification_Type v_QoS_Id; var template (value) NR_CellPowerList_Type v_CellPowerList_T1; var template (value) NR_CellPowerList_Type v_CellPowerList_T2; var float v_TimerValue := 21.0; //@sic R5-213506, R5-223437 sic@ var float v_TimerValue1 := 10.0; var float v_TimerValue2 := 30.0; //@sic R5-201227 sic@ timer t_WaitTime; …… …… //@siclog "Steps 1AF - 1AI" siclog@ //@sic R5-223437 sic@ //Steps 5 to 8 of the NR RRC_CONNECTED procedure in TS 38.508-1 Table 4.5.4.2-3 are performed f_NR_RRC_ActivateSecurity (nr_Cell12, valueof(v_ReceivedMsg.SecurityProtection.NasCount)); f_NR_RRCReconfigExistingDRBs (nr_Cell12, omit); //@siclog "Steps 1AJ - 1AK" siclog@ //@sic R5-223437 sic@ //The SS transmits a CLOSE UE TEST LOOP message. //The UE transmits a CLOSE UE TEST LOOP COMPLETE message. f_NR5GC_CloseUE_TestLoopModeB (nr_Cell12, v_IP_PDU_delay); …… …… //@siclog "Step 6A - 6D" siclog@ //@sic R5-201227 sic@ //Steps 5-8 as defined in TS 38.508-1 [4] Table Table 4.5.4.2-3. f_NR_RRC_ActivateSecurity (nr_Cell12, valueof(v_ReceivedMsg.SecurityProtection.NasCount)); f_NR_RRCReconfigExistingDRBs (nr_Cell12, omit); //@siclog "Step 6E - 6F" siclog@ //@sic R5s210669, R5-216194 sic@ //The SS transmits a CLOSE UE TEST LOOP message. //The UE transmits a CLOSE UE TEST LOOP COMPLETE message. f_NR5GC_CloseUE_TestLoopModeB (nr_Cell12, v_IP_PDU_delay); …... …… }

After Change:

function fl_TC_11_3_6and6a_TestBody() runs on NR5GC_PTC { var Octet_Type v_IP_PDU_delay := '02'O; //@sic R5s210669, R5-216194 sic@ var NG_NAS_GutiParameters_Type v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters(nr_Cell1); var template (present) NG_MobileIdentity v_S_TMSI := f_NG_S_TMSI2MobileIdentity(omit, v_GutiParams); var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var NR_SRB_COMMON_IND v_ReceivedAsp; var octetstring v_IPData := f_IPv4IPv6_IcmpEchoReply(f_LoopbackModeB_IP_Address_UE()); var integer v_DRBId; var template (value) QosFlow_Identification_Type v_QoS_Id; var template (value) NR_CellPowerList_Type v_CellPowerList_T1; var template (value) NR_CellPowerList_Type v_CellPowerList_T2; var float v_TimerValue := 21.0; //@sic R5-213506, R5-223437 sic@ var float v_TimerValue1 := 10.0; var float v_TimerValue2 := 30.0; //@sic R5-201227 sic@ timer t_WaitTime; …… …… //@siclog "Steps 1AF - 1AI" siclog@ //@sic R5-223437 sic@ //Steps 5 to 8 of the NR RRC_CONNECTED procedure in TS 38.508-1 Table 4.5.4.2-3 are performed f_NR_RRC_ActivateSecurity (nr_Cell12, valueof(v_ReceivedMsg.SecurityProtection.NasCount)); f_NR_RRCReconfigExistingDRBs (nr_Cell12, omit); //@siclog "Steps 1AF1 - 1AI1" siclog@ f_NR5GC_OpenUE_TestLoopMode(nr_Cell12); //@siclog "Steps 1AJ - 1AK" siclog@ //@sic R5-223437 sic@ //The SS transmits a CLOSE UE TEST LOOP message. //The UE transmits a CLOSE UE TEST LOOP COMPLETE message. f_NR5GC_CloseUE_TestLoopModeB (nr_Cell12, v_IP_PDU_delay); …… …… //@siclog "Step 6A - 6D" siclog@ //@sic R5-201227 sic@ //Steps 5-8 as defined in TS 38.508-1 [4] Table Table 4.5.4.2-3. f_NR_RRC_ActivateSecurity (nr_Cell12, valueof(v_ReceivedMsg.SecurityProtection.NasCount)); f_NR_RRCReconfigExistingDRBs (nr_Cell12, omit); //@siclog "Steps 6A1 - 6D1" siclog@ f_NR5GC_OpenUE_TestLoopMode(nr_Cell12); //@siclog "Step 6E - 6F" siclog@ //@sic R5s210669, R5-216194 sic@ //The SS transmits a CLOSE UE TEST LOOP message. //The UE transmits a CLOSE UE TEST LOOP COMPLETE message. f_NR5GC_CloseUE_TestLoopModeB (nr_Cell12, v_IP_PDU_delay); …… …… }

MCC160 Implementation:

function f_TC_11_3_6_NR5GC() runs on NR5GC_PTC

{ // UAC / Access Identity 2 / New cell not in the country of its HPLMN/EHPLMN 0% access probability / MCS indicator / HPLMN/0%/100% accessibility AC7 / RRC_INACTIVE

var NR_CellId_Type v_NR_CellId := nr_Cell1; // @sic R5s210669 sic@

f_NR5GC_Init(NR_1);

f_NR_CellInfo_SetSIB1_UacBarringInfo(nr_Cell12,

cs_SIB1_UacBarringInfo(cs_UAC_BarringPerCatList_1Entry(7,1), -, cs_UAC_BarringInfoSetList_1Entry(p00, s16, '1011111'B))); //@sic R5-213506, R5-223437 sic@

//Set maximum cell power level for the Cells to be used

f_NR_CellInfo_InitMaxReferencePower(nr_Cell1, -80, -80);  // Max Power level for FR2 is FFS

f_NR_CellInfo_InitMaxReferencePower(nr_Cell12, -80, -80);  // Max Power level for FR2 is FFS

//Create and configure cell

f_NR_CellConfig_Def(nr_Cell1);

f_NR_CellConfig_Def(nr_Cell12);

//The UE is equipped with a USIM configuration as defined in TS 38.508-1 [4] Table 6.4.1-19

f_UT_USIM_Insert (UT, "USIM Configuration 19 specified in Table 6.4.1-19 TS 38.508-1");

//The UE is in state 1N-A on NR Cell 1(serving cell) according to TS 38.508-1 [4] Table 4.4A.2-1 and using the message condition UE TEST LOOP MODE B prepared

f_NR5GC_Preamble(nr_Cell1, STATE_IDLE_1A, TESTModeB_ON, -, -, -, '0101'B); // @sic R5-216194, R5s211714 sic@

f_NR_TestBody_Set(true);

fl_TC_11_3_6and6a_TestBody(); // @sic R5-227452 sic@

f_NR_TestBody_Set(false);

//@sic R5-213506, R5s210669, R5-216194 sic@

if(not pc_inactiveState){

v_NR_CellId := nr_Cell12;

}

//f_NR5GC_OpenUE_TestLoopMode_Deactivate_TestMode(v_NR_CellId);@sic R5s230130 sic@

f_NR_Postamble(v_NR_CellId, STATE_CONNECTED_3A);

}

function fl_TC_11_3_6and6a_TestBody() runs on NR5GC_PTC

{

//@siclog "Steps 1N - 1O" siclog@

//@sic R5-223437 sic@

//The UE is Switched ON. The generic test procedure in TS 38.508-1 [4] Table 4.5.2.2-2 indicate that the UE performs registration on NR Cell 1 with condition Test Mode = On.

//EXCEPTION: Step 1O is performed in p pc_noOf_PDUsNewConnection > 0.

//The generic procedure in TS 38.508-1 [4] Table 4.5.2.2-4 for UE-requested PDU session establishment performs registration on NR Cell 1 and then release the RRC Connection.

f_NR5GC_SwitchOnAndResetIMS();

f_NR5GC_RRC_IdleState1N_Def (nr_Cell1, TESTModeB_ON, -, -, -, '0101'B);

//@siclog "Step 1P" siclog@

//@sic R5-223437 sic@

//The SS adjusts the NR Cells power levels according to row "T1" in table 11.3.6.3.2-1/2.

f_NR_SetCellPowerList(v_CellPowerList_T1);

}

function f_TC_11_3_6a_NR5GC() runs on NR5GC_PTC

{ // UAC / Access Identity 2 / MCS indicator / SNPN / 0% / 100% accessibility AC7 / RRC_INACTIVE

var NR_CellId_Type v_NR_CellId := nr_Cell1;

f_NR5GC_Init(NR_12);

f_NR_CellInfo_SetSIB1_UacBarringInfo(nr_Cell12,

cs_SIB1_UacBarringInfo(cs_UAC_BarringPerCatList_1Entry(7,1), -, cs_UAC_BarringInfoSetList_1Entry(p00, s16, '1011111'B)));

//Set maximum cell power level for the Cells to be used

f_NR_CellInfo_InitMaxReferencePower(nr_Cell1, -80, -80);  // Max Power level for FR2 is FFS

f_NR_CellInfo_InitMaxReferencePower(nr_Cell12, -80, -80);  // Max Power level for FR2 is FFS

//Create and configure cell

f_NR_CellConfig_Def(nr_Cell1);

f_NR_CellConfig_Def(nr_Cell12);

//The UE is provisioned with a "list of subscriber data" to allow access to SNPN identified by NR Cell 1 and NR Cell 12

//The "list of subscriber data" indicates that the UE is configured for access identitiy 2 for the SNPN identified by NR cell 1

f_UT_ProvideSubscriberDataList(UT, f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell1)), 1, "2");

f_UT_ProvideSubscriberDataList(UT, f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell12)), 12);

//The UE is in state 1N-A on NR Cell 1(serving cell) according to TS 38.508-1 [4] Table 4.4A.2-1 and using the message condition UE TEST LOOP MODE B prepared

f_NR5GC_Preamble(nr_Cell1, STATE_IDLE_1A, TESTModeB_ON, -, -, -, '0101'B);

f_NR_TestBody_Set(true);

fl_TC_11_3_6and6a_TestBody(); // Same as test case 11.3.6

f_NR_TestBody_Set(false);

if(not pc_inactiveState){

v_NR_CellId := nr_Cell12;

}

//f_NR5GC_OpenUE_TestLoopMode_Deactivate_TestMode(v_NR_CellId);@sic R5s230130 sic@

f_NR_Postamble(v_NR_CellId, STATE_CONNECTED_3A);

}