3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240129

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3357 | rev | - | Current version: | 17.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NR5GC FR1 : Addition of Rel-17 NR Slice test case 6.1.2.25 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, NR_slice-UEConTest |  |  |  |  |  | Date: |  |  | 2024-01-22 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To add test case 6.1.2.25 to the 5G/NR5GC ATS for FR1 path. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to test case 6.1.2.25 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.2.25.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.523-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	5

2.2	Change 2	7

3	Branches executed	10

4	Execution Log Files	11

4.1	Mediatek MT6815	11

5	References	11

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 6.1.2.25 which is part of the 5G NR5GC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Kalahasti, Narendra

Narendra.Kalahasti@anritsu.com

## Verification Test Summary

Test Case: 	6.1.2.25

ATS Version:	iwd-TTCN3-B2022-09_D23wk49

System Simulator used:	Anritsu Protocol Conformance Test System ME7834NR

UE used:	Mediatek MT6815

Verification Status:	PASS

# Corrections required

## Change 1

| Function name | function f_TC_6_1_2_25_NR5GC() |
| --- | --- |
| Reason for change | - To enable the slice-based cell reselection between two cells, the specific message contents for SIB16 needs to be updated to two freqPriorityListSlicing entities. - To update the incorrect value of dl-ImplicitCarrierFreq IE for TC 6.1.2.25 Slice-based cell reselection / Reselection priorities provided by SIB16. # a draft proseCR will be raised for RAN5#102 meeting |
| Summary of change | - The specific message contents of SIB16 for NR cell 1 were updated to 3 freqPriorityListSlicing entities. - The value of dl-ImplicitCarrierFreq IE for NR CELL3/6 were updated. |
| TTCN module | Idle_CellReSelection_NR5GC |
| MCC160 Comment | Accepted conditional to prose CR agreement at RAN5#102 |

Before Change

function f_TC_6_1_2_25_NR5GC() runs on NR5GC_PTC { // Slice-based cell reselection / Re-derive reselection priority for frequency var integer v_T0_Cell1_RS_EPRE_FR1 := tsc_NR_ServingCellSSS_EPRE_FR1; var integer v_T0_Cell1_RS_EPRE_FR2 := tsc_NR_SuitableInterFreqCellSSS_EPRE_FR2; var integer v_T0_CellOff := tsc_NR_NonSuitableOffCellSSS_EPRE; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT0; var PhysCellId v_PhysCellId; v_CellPowerList_AtT0 :={ cs_NR_CellPower(nr_Cell1, v_T0_Cell1_RS_EPRE_FR1, v_T0_Cell1_RS_EPRE_FR2), cs_NR_CellPower(nr_Cell3, v_T0_CellOff, v_T0_CellOff), cs_NR_CellPower(nr_Cell6, v_T0_CellOff, v_T0_CellOff) }; f_NR5GC_Init(NR_30); // System information combination NR-30 as defined in TS 38.508-1 clause 4.4.3.1.2 is used in NR cells // Table 6.1.2.25.3.3.3-2: SIB16 for NR Cell 1 f_NR_CellInfo_SetSIB16_FreqPriorityListSlicing_r17(nr_Cell1, {cs_FreqPrioritySlicing_r17_Def(1, {cs_SliceInfo_r17_1Entry('00000001'B,5)})}); // Table 6.1.2.25.3.3.3-3: SIB16 for NR Cell 3 v_PhysCellId := f_NR_CellInfo_GetPhysicalCellId(nr_Cell3); f_NR_CellInfo_SetSIB16_FreqPriorityListSlicing_r17(nr_Cell3, {cs_FreqPrioritySlicing_r17_Def(1, cs_SliceInfoList_r17_2Entries_2ndExcluded('00000001'B, 6, '00000010'B, 4, cs_SliceListNR_r17_sliceExcludedCellListNR_1Entry(v_PhysCellId)))}); // Table 6.1.2.25.3.3.3-4: SIB16 for NR Cell 6 v_PhysCellId := f_NR_CellInfo_GetPhysicalCellId(nr_Cell6); f_NR_CellInfo_SetSIB16_FreqPriorityListSlicing_r17(nr_Cell6, {cs_FreqPrioritySlicing_r17_Def(1, cs_SliceInfoList_r17_2Entries_2ndExcluded('00000001'B, 4, '00000010'B, 4, cs_SliceListNR_r17_sliceExcludedCellListNR_1Entry(v_PhysCellId)))}); // Table 6.1.2.25.3.2-1/2: : Time instances of cell power level and parameter changes for FR1/FR2 f_NR_CellInfo_InitMaxReferencePower(nr_Cell3, -78, tsc_NR_ServingCellSSS_EPRE_FR2); f_NR_CellInfo_InitMaxReferencePower(nr_Cell6, -78, tsc_NR_ServingCellSSS_EPRE_FR2); // Create and configure cell f_NR_CellConfig_Def(nr_Cell1); f_NR_CellConfig_Def(nr_Cell3); f_NR_CellConfig_Def(nr_Cell6); // Preamble: UE is in state Switched OFF (state 0N-B) f_NR5GC_Preamble(nr_Cell1, STATE_OFF_0B); f_NR_TestBody_Set(true); // Table 6.1.2.25.3.2-1: Time instances of cell power level and parameter changes f_NR_SetCellPowerList(v_CellPowerList_AtT0); f_TC_6_1_2_25_NR5GC_TestBody(); f_NR_TestBody_Set(false); // Switch/Power off UE f_NR_Postamble(nr_Cell6, STATE_IDLE_1A); } //end of f_TC_6_1_2_25_NR5GC

After Change

function f_TC_6_1_2_25_NR5GC() runs on NR5GC_PTC { // Slice-based cell reselection / Re-derive reselection priority for frequency var integer v_T0_Cell1_RS_EPRE_FR1 := tsc_NR_ServingCellSSS_EPRE_FR1; var integer v_T0_Cell1_RS_EPRE_FR2 := tsc_NR_SuitableInterFreqCellSSS_EPRE_FR2; var integer v_T0_CellOff := tsc_NR_NonSuitableOffCellSSS_EPRE; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT0; var PhysCellId v_PhysCellId_3; var PhysCellId v_PhysCellId_6; v_CellPowerList_AtT0 :={ cs_NR_CellPower(nr_Cell1, v_T0_Cell1_RS_EPRE_FR1, v_T0_Cell1_RS_EPRE_FR2), cs_NR_CellPower(nr_Cell3, v_T0_CellOff, v_T0_CellOff), cs_NR_CellPower(nr_Cell6, v_T0_CellOff, v_T0_CellOff) }; f_NR5GC_Init(NR_30); // System information combination NR-30 as defined in TS 38.508-1 clause 4.4.3.1.2 is used in NR cells v_PhysCellId_3 := f_NR_CellInfo_GetPhysicalCellId(nr_Cell3); v_PhysCellId_6 := f_NR_CellInfo_GetPhysicalCellId(nr_Cell6); // Table 6.1.2.25.3.3.3-2: SIB16 for NR Cell 1 f_NR_CellInfo_SetSIB16_FreqPriorityListSlicing_r17(nr_Cell1, {cs_FreqPrioritySlicing_r17_Def(0, {cs_SliceInfo_r17_1Entry('00000001'B,5)}), cs_FreqPrioritySlicing_r17_Def(1, {cs_SliceInfo_r17_1Entry_sliceExcludedCellListNR('00000010'B,4, cs_SliceListNR_r17_sliceExcludedCellListNR_1Entry(v_PhysCellId_3))}), cs_FreqPrioritySlicing_r17_Def(2, cs_SliceInfoList_r17_2Entries_2ndExcluded('00000001'B,5,'00000010'B, 4, cs_SliceListNR_r17_sliceExcludedCellListNR_1Entry(v_PhysCellId_6)))}); // Table 6.1.2.25.3.3.3-3: SIB16 for NR Cell 3 f_NR_CellInfo_SetSIB16_FreqPriorityListSlicing_r17(nr_Cell3, {cs_FreqPrioritySlicing_r17_Def(0, cs_SliceInfoList_r17_2Entries_2ndExcluded('00000001'B, 6, '00000010'B, 4, cs_SliceListNR_r17_sliceExcludedCellListNR_1Entry(v_PhysCellId_3)))}); // Table 6.1.2.25.3.3.3-4: SIB16 for NR Cell 6 f_NR_CellInfo_SetSIB16_FreqPriorityListSlicing_r17(nr_Cell6, {cs_FreqPrioritySlicing_r17_Def(0, cs_SliceInfoList_r17_2Entries_2ndExcluded('00000001'B, 4, '00000010'B, 4, cs_SliceListNR_r17_sliceExcludedCellListNR_1Entry(v_PhysCellId_6)))}); // Table 6.1.2.25.3.2-1/2: : Time instances of cell power level and parameter changes for FR1/FR2 f_NR_CellInfo_InitMaxReferencePower(nr_Cell3, -78, tsc_NR_ServingCellSSS_EPRE_FR2); f_NR_CellInfo_InitMaxReferencePower(nr_Cell6, -78, tsc_NR_ServingCellSSS_EPRE_FR2); // Create and configure cell f_NR_CellConfig_Def(nr_Cell1); f_NR_CellConfig_Def(nr_Cell3); f_NR_CellConfig_Def(nr_Cell6); // Preamble: UE is in state Switched OFF (state 0N-B) f_NR5GC_Preamble(nr_Cell1, STATE_OFF_0B); f_NR_TestBody_Set(true); // Table 6.1.2.25.3.2-1: Time instances of cell power level and parameter changes f_NR_SetCellPowerList(v_CellPowerList_AtT0); f_TC_6_1_2_25_NR5GC_TestBody(); f_NR_TestBody_Set(false); // Switch/Power off UE f_NR_Postamble(nr_Cell6, STATE_IDLE_1A); } //end of f_TC_6_1_2_25_NR5GC

## Change 2

| Function name | function f_TC_6_1_2_25_NR5GC_TestBody() |
| --- | --- |
| Reason for change | At step 24, UE should have send RRCsetuprequest on cell3 within 60 sec. But t_watchdog was not started for it after step22. So added t_Watchdog.start before step 23. |
| Summary of change | Added t_Watchdog.start before step23. |
| TTCN module | Idle_CellReSelection_NR5GC |
| MCC160 Comment | Accepted |

Before Change

function f_TC_6_1_2_25_NR5GC_TestBody() runs on NR5GC_PTC { /* @sic R5-237332 sic@ */ ----Skiped code-- //@siclog "Step 22" siclog@ // Wait for 66[FR1]/258[FR2] seconds to allow UE to recognise the change. if (f_NR_IsBandFR1(v_FreqBandIndicatorNR)) { // FR1 f_Delay(66.0); } else { // FR2 f_Delay(258.0); } //t_Watchdog is not started here //@siclog "Step 23-24" siclog@ // Step 1 and 2 of the generic test procedure in TS 38.508-1 Table 4.9.4.2.2-1 indicate that the UE is camped on NR Cell 3 within 60s? f_NR_Paging_Def(nr_Cell3, RRC_IDLE); // Step 23 - Page UE if (f_NR_RRC_SetupRequest_Common(nr_Cell3, t_Watchdog)) { // Step 24 - Check: Does the UE transmit an RRCSetupRequest message on the cell specified in the test case within 60s? f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 24"); }----Skiped code-- }//end of f_TC_6_1_2_25_NR5GC_TestBody

After Change

function f_TC_6_1_2_25_NR5GC_TestBody() runs on NR5GC_PTC { /* @sic R5-237332 sic@ */ ----Skiped code-- //@siclog "Step 22" siclog@ // Wait for 66[FR1]/258[FR2] seconds to allow UE to recognise the change. if (f_NR_IsBandFR1(v_FreqBandIndicatorNR)) { // FR1 f_Delay(66.0); } else { // FR2 f_Delay(258.0); } t_Watchdog.start; //@siclog "Step 23-24" siclog@ // Step 1 and 2 of the generic test procedure in TS 38.508-1 Table 4.9.4.2.2-1 indicate that the UE is camped on NR Cell 3 within 60s? f_NR_Paging_Def(nr_Cell3, RRC_IDLE); // Step 23 - Page UE if (f_NR_RRC_SetupRequest_Common(nr_Cell3, t_Watchdog)) { // Step 24 - Check: Does the UE transmit an RRCSetupRequest message on the cell specified in the test case within 60s? f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 24"); }----Skiped code-- }//end of f_TC_6_1_2_25_NR5GC_TestBody

# Branches executed

The test case was executed on NR band n78 using NR ciphering nea2 and integrity algorithm nia2

# Execution Log Files

## Mediatek MT6815

The Mediatek MT6815 UE passed this test case on Anritsu Protocol Conformance Test System ME7834NR. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

TC_6_1_2_25_LOG.html

- PICS/PIXIT parameter file:

TC_6_1_2_25_PIXIT.xml

# References

| [1] | R5s240140: NR5GC FR1: Supporting information for addition of Rel-17 NR Slice test case 6.1.2.25 |
| --- | --- |