3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240132

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3358 | rev | - | Current version: | 17.9.0 |  |
| For HELPon using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NR5GC FR1 : Addition of Rel-17 NR Slice test case 8.1.1.3.9 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, NR_slice-UEConTest |  |  |  |  |  | Date: |  |  | 2024-01-22 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To add test case 8.1.1.3.9 to the 5G/NR5GC ATS for FR1 path. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to test case 8.1.1.3.9 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.1.3.9.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.523-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	5

3	Branches executed	10

4	Execution Log Files	11

4.1	Mediatek MT6815	11

5	References	11

- Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 8.1.1.3.9which is part of the 5G NR5GC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Kalahasti, Narendra

Narendra.Kalahasti@anritsu.com

  - Verification Test Summary

Test Case: 	8.1.1.3.9

ATS Version:	iwd-TTCN3-B2022-09_D23wk49

System Simulator used:	Anritsu Protocol Conformance Test System ME7834NR

UE used:	Mediatek MT6815

Verification Status:	PASS

- Corrections required

  - Change 1

| Function name | function f_TC_8_1_1_3_9_NR5GC () |
| --- | --- |
| Reason for change | - For Cell6 SIB16 should be assigned with NSAG ID 2 else UE will reselect cell 3 on postamble step (after RRCrelease at step33) which has NSAG priority over the cell6 |
| Summary of change | - Added function f_NR_CellInfo_SetSIB16_FreqPriorityListSlicing_r17to update cell6 SIB16. |
| TTCN module | GSM_NR5GC |
| MCC160 Comment | Accepted conditional to prose CR agreement at RAN5#102 |

Before Change

function f_TC_8_1_1_3_9_NR5GC() runs on NR5GC_PTC { //RRC connection release / Success / With slice specific cell reselection information f_NR5GC_Init(NR_30); //Preconfigure SIB16 for NR Cell 3: Table 8.1.1.3.9.3.3-2 f_NR_CellInfo_SetSIB16_FreqPriorityListSlicing_r17(nr_Cell3, {cs_FreqPrioritySlicing_r17_Def(0, {cs_SliceInfo_r17_1Entry('00000010'B,5)}), cs_FreqPrioritySlicing_r17_Def(2, {cs_SliceInfo_r17_1Entry('00000010'B,4)})}); f_NR_CellInfo_InitMaxReferencePower(nr_Cell3, -78, tsc_NR_ServingCellSSS_EPRE_FR2); f_NR_CellInfo_InitMaxReferencePower(nr_Cell6, -78, tsc_NR_ServingCellSSS_EPRE_FR2); f_NR_CellConfig_Def(nr_Cell1); f_NR_CellConfig_Def(nr_Cell3); f_NR_CellConfig_Def(nr_Cell6); //The UE is in state Switched OFF (state 0N-B) according to TS 38.508-1 [4]. f_NR5GC_Preamble (nr_Cell1, STATE_OFF_0B); f_NR_TestBody_Set(true); fl_TC_8_1_1_3_9_TestBody(); f_NR_TestBody_Set(false); f_NR_Postamble(nr_Cell6, STATE_IDLE_1A); }

After Change

function f_TC_8_1_1_3_9_NR5GC() runs on NR5GC_PTC { //RRC connection release / Success / With slice specific cell reselection information f_NR5GC_Init(NR_30); //Preconfigure SIB16 for NR Cell 3: Table 8.1.1.3.9.3.3-2 f_NR_CellInfo_SetSIB16_FreqPriorityListSlicing_r17(nr_Cell3, {cs_FreqPrioritySlicing_r17_Def(0, {cs_SliceInfo_r17_1Entry('00000010'B,5)}), cs_FreqPrioritySlicing_r17_Def(2, {cs_SliceInfo_r17_1Entry('00000010'B,4)})}); f_NR_CellInfo_SetSIB16_FreqPriorityListSlicing_r17(nr_Cell6, {cs_FreqPrioritySlicing_r17_Def(0, {cs_SliceInfo_r17_1Entry('00000010'B,5)})}); f_NR_CellInfo_InitMaxReferencePower(nr_Cell3, -78, tsc_NR_ServingCellSSS_EPRE_FR2); f_NR_CellInfo_InitMaxReferencePower(nr_Cell6, -78, tsc_NR_ServingCellSSS_EPRE_FR2); f_NR_CellConfig_Def(nr_Cell1); f_NR_CellConfig_Def(nr_Cell3); f_NR_CellConfig_Def(nr_Cell6); //The UE is in state Switched OFF (state 0N-B) according to TS 38.508-1 [4]. f_NR5GC_Preamble (nr_Cell1, STATE_OFF_0B); f_NR_TestBody_Set(true); fl_TC_8_1_1_3_9_TestBody(); f_NR_TestBody_Set(false); f_NR_Postamble(nr_Cell6, STATE_IDLE_1A); }

- Branches executed

The test case was executed on NR band n78using NR ciphering nea2 and integrity algorithm nia2

- Execution Log Files

  - Mediatek MT6815

The MediatekMT6815UE passed this test case on Anritsu Protocol Conformance Test System ME7834NR. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

TC_8_1_1_3_9_LOG.html

- PICS/PIXIT parameter file:

TC_8_1_1_3_9_PIXIT.xml

- References

| [1] | R5s240133: NR5GC FR1: Supporting information for addition of Rel-17 NR Slice test case 8.1.1.3.9 |
| --- | --- |