3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240144

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.571-4 | CR | 0195 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to f_POS_NR_Set_DlPrsAssistanceDataPerTRP |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | R5-237422 (affecting 3GPP 38.508-1) introduced a new parameter called Physical Cell Id for SIG, which is now used for cell configuration of th NR cells. However, the LPP assistance data from 37.571-2 still uses the old PCIs, and does not point to 38.508-1 specification to update its value. The mismatch between the PCI for SIG used for cell configuration and the PCI used in the assistance data causes UE to fail any DL-PRS test case. The assistance data must be aligned and use the same PCI as the cell configuration. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In the function f_POS_NR_Set_DlPrsAssistanceDataPerTRP, instead of manually setting the value of nr-PhysCellID-r16, such value should come from f_NR_CellInfo_GetPhysicalCellId (the one used for cell configuration) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.4.1, 9.3.4.2, 9.3.4.3, 9.3.4.4, 9.3.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 37.571-2 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

2.1	Change 1	2

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D23wk49 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change 1

| Function name | f_POS_NR_Set_DlPrsAssistanceDataPerTRP |
| --- | --- |
| Reason for change | R5-237422 (affecting 3GPP 38.508-1) introduced a new parameter called Physical Cell Id for SIG, which is now used for cell configuration of th NR cells. However, the LPP assistance data from 37.571-2 still uses the old PCIs, and does not point to 38.508-1 specification to update its value. The mismatch between the PCI for SIG used for cell configuration and the PCI used in the assistance data causes UE to fail any DL-PRS test case. The assistance data must be aligned and use the same PCI as the cell configuration. |
| Summary of change | Change the hardcoded values 0, 2 and 3 to the ones coming from f_NR_CellInfo_GetPhysicalCellId(p_CellId) |
| TTCN module | NR5GC_Positioning_Functions.ttcn |
| MCC160 Comment | Accepted conditional to approval of corresponding prose CR at RAN5#102. |

Before

<<SKIPPED CODE>>

function f_POS_NR_Set_DlPrsAssistanceDataPerTRP(NR_CellId_Type p_CellId,

integer p_ExpRSTD_Uncert) runs on NR_BASE_PTC return template (value) NR_DL_PRS_AssistanceDataPerTRP_r16

{

var template (value) NR_DL_PRS_AssistanceDataPerTRP_r16 v_DlPrs_AssistanceDataPerTRP;

var NCGI_r15 v_CellGlobalId := f_POS_Get_CellGlobalId(p_CellId);

var ARFCN_ValueNR v_FrequencySSB := f_NR_CellInfo_GetFrequencySSB(p_CellId);

var NR_DL_PRS_Periodicity_and_ResourceSetSlotOffset_r16 v_PeriodicityAndOffset := f_POS_NR_Set_DlPrsPeriodicityAndOffset_11ms(p_CellId);

select(p_CellId){

case (nr_Cell1) {

v_DlPrs_AssistanceDataPerTRP := cs_DlPrsAssistanceDataPerTRP(0, 0, v_CellGlobalId, v_FrequencySSB, cs_NR_DL_PRS_Info_Def(v_PeriodicityAndOffset, 0, 0), p_ExpRSTD_Uncert); //@sic R5s221043, R5s230101 sic@

}

case (nr_Cell2) {

v_DlPrs_AssistanceDataPerTRP := cs_DlPrsAssistanceDataPerTRP(1, 2, v_CellGlobalId, v_FrequencySSB, cs_NR_DL_PRS_Info_Def(v_PeriodicityAndOffset, 1, 1), p_ExpRSTD_Uncert); //@sic R5s221043, R5s230101 sic@

}

case (nr_Cell3) {

v_DlPrs_AssistanceDataPerTRP := cs_DlPrsAssistanceDataPerTRP(2, 3, v_CellGlobalId, v_FrequencySSB, cs_NR_DL_PRS_Info_Def(v_PeriodicityAndOffset, 2, 0), p_ExpRSTD_Uncert); //@sic R5s221043, R5s230101 sic@

}

case else {

FatalError (__FILE__, __LINE__, "Invalid Cell ID");

}

}

return v_DlPrs_AssistanceDataPerTRP;

}

<<SKIPPED CODE>>

After

<<SKIPPED CODE>>

function f_POS_NR_Set_DlPrsAssistanceDataPerTRP(NR_CellId_Type p_CellId,

integer p_ExpRSTD_Uncert) runs on NR_BASE_PTC return template (value) NR_DL_PRS_AssistanceDataPerTRP_r16

{

var template (value) NR_DL_PRS_AssistanceDataPerTRP_r16 v_DlPrs_AssistanceDataPerTRP;

var NCGI_r15 v_CellGlobalId := f_POS_Get_CellGlobalId(p_CellId);

var ARFCN_ValueNR v_FrequencySSB := f_NR_CellInfo_GetFrequencySSB(p_CellId);

var PhysCellId v_PhysCellId := f_NR_CellInfo_GetPhysicalCellId(p_CellId);

var NR_DL_PRS_Periodicity_and_ResourceSetSlotOffset_r16 v_PeriodicityAndOffset := f_POS_NR_Set_DlPrsPeriodicityAndOffset_11ms(p_CellId);

select(p_CellId){

case (nr_Cell1) {

v_DlPrs_AssistanceDataPerTRP := cs_DlPrsAssistanceDataPerTRP(0, v_PhysCellId, v_CellGlobalId, v_FrequencySSB, cs_NR_DL_PRS_Info_Def(v_PeriodicityAndOffset, 0, 0), p_ExpRSTD_Uncert); //@sic R5s221043, R5s230101 sic@

}

case (nr_Cell2) {

v_DlPrs_AssistanceDataPerTRP := cs_DlPrsAssistanceDataPerTRP(1, v_PhysCellId, v_CellGlobalId, v_FrequencySSB, cs_NR_DL_PRS_Info_Def(v_PeriodicityAndOffset, 1, 1), p_ExpRSTD_Uncert); //@sic R5s221043, R5s230101 sic@

}

case (nr_Cell3) {

v_DlPrs_AssistanceDataPerTRP := cs_DlPrsAssistanceDataPerTRP(2, v_PhysCellId, v_CellGlobalId, v_FrequencySSB, cs_NR_DL_PRS_Info_Def(v_PeriodicityAndOffset, 2, 0), p_ExpRSTD_Uncert); //@sic R5s221043, R5s230101 sic@

}

case else {

FatalError (__FILE__, __LINE__, "Invalid Cell ID");

}

}

return v_DlPrs_AssistanceDataPerTRP;

}

<<SKIPPED CODE>>